3GPP TSG RAN WG1 #119	               	           R1-2409493

Orlando, US, November 18th – 22nd, 2024

Source: 	CMCC

Title:	Discussion on RAN4 LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

Agenda item:	5

Document for:	Discussion & Decision

- Introduction

RAN4 sends an LS to respectfully ask RAN1 to provide clarification on the target scenarios for L1-based UE-to-UE CLI measurement [1]. In this contribution, we will discuss this LS.

- Discussion

RAN4#112bis meeting discussed the requirements for UE-to-UE L1 CLI measurement, e.g., L1 SRS-RSRP, in SBFD operation and it was identified that the time difference between the victim UE’s DL reference timing in the serving cell and aggressor UE’s SRS arrival timing depends on the applicable scenarios. Regarding the intra-cell UE-to-UE L1 CLI measurement, as both aggressor UE and victim UE belong to the same cell, cell phase synchronization error of 3us does not need to be considered. However, for inter-cell UE-to-UE L1 CLI measurement, the cell phase synchronization error of 3us needs to be considered since aggressor UE and victim UE belong to different cells. RAN4 sends the LS [1] to RAN1 and asks RAN1 to clarify the scenarios.

# 1 Overall description When RAN4 discussed the requirements for UE-to-UE L1 CLI measurement, e.g., L1 SRS-RSRP, in SBFD operation, it was identified that the time difference between the victim UE’s DL reference timing in the serving cell and aggressor UE’s SRS arrival timing depends on the applicable scenarios: - Intra-cell UE-to-UE L1 CLI measurement, where both aggressor UE and victim UE belong to the same cell and thus cell phase synchronization error of 3us does not need to be considered - Inter-cell UE-to-UE L1 CLI measurement, where aggressor UE and victim UE belong to different cells and thus cell phase synchronization error of 3us needs to be considered To define the appropriate requirements, RAN4 needs to understand from RAN1 perspective if both scenarios are considered or just intra-cell scenario is considered. Any other clarification or info from RAN1 is appreciated too.

During the study phase, the following applicable and relevant deployment scenarios were identified for SBFD, all of which are inter-cell scenarios, including:

-	SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration.

-	SBFD Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy static TDD operation while the others use SBFD operation with the same SBFD subband configuration.

-	Deployment Case 3-2: 2-layer is considered. Layer 1 uses legacy static TDD operation, Layer 2 uses SBFD operation. All the gNBs in Layer 2 use the same SBFD subband configuration.

-	SBFD Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy static TDD operation while the other operator uses SBFD operation with the same SBFD subband configuration.

During the study of SBFD system evaluation and inter-UE CLI in the Rel-18 and Rel-19 study phase, no distinction was made between intra-cell and inter-cell scenarios. The aggressor UE and victim UE can be from the same cell or different cells in UE-UE co-channel inter-subband CLI modelling used in system level simulation in Rel-18 (see Annex A.2.4 in TR 38.858 [2]), and the studied L1 inter-UE CLI measurement and reporting solutions are applicable to both intra-cell and inter-cell scenarios.

Last, in realistic deployment scenarios, it is highly probable that one UE suffers both intra-cell inter-UE CLI and inter-cell inter-UE CLI. For example, a victim UE at the cell edge will suffer both intra-cell inter-UE CLI and inter-cell inter-UE CLI and the interference level is similar.

Thus, from RAN1’s perspective, to define the appropriate requirements, both scenarios of intra-cell UE-to-UE L1 CLI measurement and inter-cell UE-to-UE L1 CLI measurement should be considered.

Proposal 1. Reply to RAN4 that from RAN1’s perspective, the target scenarios for UE-to-UE L1 CLI measurement include both intra-cell UE-to-UE L1 CLI measurement and inter-cell UE-to-UE L1 CLI measurement.

- Conclusion

In this contribution, we discuss the LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation from RAN4, and the following proposal is made.

Proposal 1. Reply to RAN4 that from RAN1’s perspective, the target scenarios for UE-to-UE L1 CLI measurement include both intra-cell UE-to-UE L1 CLI measurement and inter-cell UE-to-UE L1 CLI measurement.

- Reference

- R1-2409352(R4-2416919), LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation, RAN4, Apple.

- 3GPP TR 38.858, Study on Evolution of NR Duplex Operation (Release 18).