3GPP TSG RAN WG1 #119			R1-2409501

Orlando, US, November 18th – 22nd, 2024

Source: 	CMCC

Title:	Discussion on AI/ML for CSI prediction

Agenda item:	9.1.3

Document for:	Discussion & Decision

# Introduction

Rel-19 NR AI/ML for Air Interface WI was revised in [1], we will discuss the following objectives for CSI prediction in this contribution.

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

# Discussion on specification impacts on AI/ML based CSI prediction

## Consistency of NW-side additional condition across training and inference

Based on the WID for CSI prediction, the consistency of training and inference still needs to be studied. And this topic actually has already been discussed in the agenda of beam enhancement and other aspects of AI/ML model and data in previous meetings [4] [5] [6]. Here we copied them as reference:

Agreement @RAN1#116bis Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. Agreement @RAN1#116bis From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. Working Assumption @RAN1#117 Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study) Agreement @RAN1#118 Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study) Agreement @RAN1#118 For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

It should be noted both AI based beam reporting, including BM-Case 1 and Case 2, and AI based CSI prediction are some kinds of enhancement of CSI reporting. And the current discussion on consistency of training/inference is also based on the assumption of UE-sided model.

So, the basic method to guarantee consistency between training and inference for the case of beam management can be reused here, at least for further study:

- Option 1: Based on associated ID

- Option 2: Based on performance monitoring

Proposal 1: For the consistency of NW-side additional condition across training and inference for AI based CSI prediction, the following two options can be further studied:

- Option 1: Based on associated ID

- Option 2: Based on performance monitoring

For Option 1, the logic and procedure behind it is very similar to the case of beam management:

- Step 1: NW transmit the data collection related configurations and their associated ID

- Step 2: UE collects the data corresponding to the associated ID

- Step 3: AI/ML models are trained at UE side based on the collected data corresponding to the associated ID

Thus, the associated ID of a functionality is related to NW side additional conditions, and these additional conditions may include some NW side proprietary information which may affect the model generalization/scalability.

For AI based beam management, the properties of a DL Tx beam or beam set/list, e.g. 3dB beamwidth, beam boresight direction are discussed to refer the NW side additional conditions. While for AI based CSI prediction, it is not easy to figure out what UE assumption is behind a same associated ID.

Based on TR 38.843 [2], we have evaluated AI based CSI prediction from the perspective of generalization over various scenarios/configurations, including various UE speed, deployment scenario, carrier frequency.

However, for the deployment scenario, it is usually much related to channel condition, which involves gNB site location, the characteristics of surrounding buildings. In addition, even for gNB itself, it is still hard to directly indicate which deployment scenario it belongs to. As for the example of carrier frequency, it already can be indicated to UE via current RRC configurations, which seems it does not belong to the range of NW side additional conditions.

In RAN1#118bis meeting [7], we have the following agreements on how to identify NW-side additional conditions that may impact on UE assumption for CSI prediction using UE-sided model:

Agreement @RAN1#118bis For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered. Agreement @RAN1#118bis For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

Here are two aspects of NW-sided additional conditions proposed to be evaluated. For gNB electronic tilt angle, it is possible that the tilt angle may change from time to time. And for TXRU mapping, at least for the case of DPS and NES type-2 scenarios, the TXRU virtualization will also change adequately.

For Option 2, the consistency guarantee is achieved after model deployment, if the performance is worse during performance monitoring, then it might be due to consistency issue between training and inference. However, it should be noted there are still some other possibilities which lead to performance loss. It is obvious that performance monitoring-based solutions are not very efficient to make consistency between training and inference.

Meanwhile, it is not excluded that both associated ID based solution and performance monitoring-based solution can be used together. Before performance monitoring, gNB can preliminarily guarantee consistency of NW-side additional condition without exposing proprietary information based on associated ID to reduce measurement and reporting overhead of monitoring.

If the gNB can provide the same additional conditions behind associated ID of a model, there are still other NW-side additional conditions not guaranteed by associated ID, then performance monitoring can be used additionally to guarantee the performance during inference.

Proposal 2: It is proposed that the associated ID based solution can be used together with performance monitoring.

Following the agreement in agenda of other aspects of model and data, the application range of associated ID should at least be a cell:

Agreement @RAN1#118 Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Proposal 3: If associated ID is supported for AI based CSI prediction, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell.

In Rel-18 Doppler codebook enhancement, the major spec impacts mainly target at the CSI reporting framework, and there is also a working assumption that the associated ID for BM-Case1 and Case2 can at least be configured within CSI framework. So, it seems a natural way to configure associated ID within CSI framework.

As for the details of associated ID, e.g. configuration granularity of associated ID within CSI framework, it can be discussed after the UE assumption behind a same associated ID become clear.

Proposal 4: If associated ID is supported for AI based CSI prediction, the associated ID at least can be configured within CSI framework.

## Monitoring methods

Based on TR 38.843 [2], only functionality-based LCM is agreed for performance monitoring of AI/ML based CSI prediction and three types of monitoring methods are identified.

For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

And in RAN1#117 meeting [5], the following agreement about performance monitoring Type 1 and Type 3 have been achieved:

Agreement For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

Between Type 1 and Type 3, the common part between them is that:

- UE need to calculate the performance metric

- UE need to report performance metric or performance output

- NW make LCM decision based on the received performance metric or performance output.

While Type 1 might need two additional steps comparing to Type 3:

- NW need to configure threshold criterion for UE

- UE will convert the performance metric to performance output based on configured threshold.

As for the definition of performance metric in Type 3, based on the TR 38.843, generally it can be divided into two major types: intermediate KPI and eventual KPI. Eventual KPI, like throughput, BLER, although can be considered for RAN4 testing, however, this kind of KPI not only is related with the performance of AI model itself, but also related to other factors, e.g., network scheduling strategy, UE mobility, traffic characteristics, etc. So, we think eventual KPI is not appropriate for the performance monitoring of AI functionality. The intermediate KPI, e.g. SGCS, NMSE, can show the performance of model itself at current time, no matter real time or near real time. We think it can be considered as starting point for Type 3 performance monitoring.

As for the definition of performance output in Type 1, based on our understanding, it is derived from the performance metric calculated at UE side and NW will used it for LCM decision. There might be two possibilities on the performance output:

- It can be the calculation or comparison outcome based on the performance metric and the threshold criterion configured by NW. For example, the threshold criterion can be a configured intermediate KPI value, and UE could report the comparison result between calculated intermediate KPI and configured intermediate KPI, or the event occurrence based on the calculated intermediate KPI and the configured intermediate KPI.

- It can be the recommended LCM decision reported to NW. For example, after UE calculate the intermediate KPI, based on some additional NW configured threshold and LCM decision rules, UE might prefer to do some specific LCM decision, including fallback to legacy CSI reporting. Then UE will report the preferred LCM operation accordingly and NW may make final LCM decision considering the reported performance output.

As for Type 2, both the predicted CSI and ground-truth CSI are a kind of PMI, which is legacy CSI report quantity. And UE also do not have to calculate performance metric or output, calculation overhead at UE side can be reduced. Considering specification simplicity and UE calculation overhead, Type 2 can also be considered for further discussion.

Proposal 5: For the definition of performance metric in Type 3 performance monitoring, intermediate KPI can be considered as starting point.

Proposal 6: For the definition of performance metric in Type 1 performance monitoring, there might be two alternatives:

- Alt1: UE calculates the performance metric or determines the comparison outcome based on the performance metric, and the threshold criterion is configured by NW.

- Alt2: UE recommends the decision of the LCM to NW.

## Data collection

Data collection can be used for training, monitoring, and inference. In this subsection, we mainly discuss data collection for training.

Both the use case of CSI prediction and beam management adopt single-sided model and both of them are kinds of CSI report. Besides, AI/ML based beam management will be specified at the beginning of Rel-19. So, we think the data collection mechanism of AI/ML based beam management can be reused as much as possible for CSI prediction, e.g., data collection configuration, triggering/requesting mechanism, potential RS and reporting overhead reduction.

Proposal 7: For data collection of AI/ML based CSI prediction, data collection mechanism of AI/ML based beam management can be reused as much as possible.

## Inference related aspects

In RAN1#116bis meeting [4], the following agreements have been achieved:

Agreement For AI/ML based CSI prediction, at least for inference, legacy CSI-RS configuration can be a starting point. Further study on whether there is a need for specification enhancement. Agreement - At least for inference, for UE-sided model based CSI prediction, legacy feedback mechanism using codebook type set to “typeII-Doppler-r18” is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects: - CSI processing criteria and timeline

Regarding the spec impact of AI based CSI prediction during inference phase, the CSI codebook for high/medium UE velocities in Rel-18 MIMO have already enhanced the CSI measurement configuration and reporting configuration to facilitate CSI prediction, including some concepts related to measurement window, reporting window, etc.

In this sense, legacy feedback mechanism using codebook type set to “typeII-Doppler-r18” can be reused here. However, some CSI related parameters agreed in Rel-18 MIMO are appropriate for non-AI-enabled CSI feedback and most of them are decided based on the simulation results without assuming AI/ML algorithm involved. If these parameters are reused for AI-enabled CSI prediction, we might need to revisit some of them to adapt AI/ML based scheme, to harvest the maximum gain of AI/ML based approach.

Proposal 8: For CSI prediction, the CSI related parameters in Rel-18 can be reused with modification to adapt AI/ML-enabled CSI prediction, e.g. the parameters about measurement window, reporting window.

# Conclusions

In this contribution, we discussed AI/ML based CSI prediction, and the following proposal are made.

Proposal 1: For the consistency of NW-side additional condition across training and inference for AI based CSI prediction, the following two options can be further studied:

- Option 1: Based on associated ID

- Option 2: Based on performance monitoring

Proposal 2: It is proposed that the associated ID based solution can be used together with performance monitoring.

Proposal 3: If associated ID is supported for AI based CSI prediction, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell.

Proposal 4: If associated ID is supported for AI based CSI prediction, the associated ID at least can be configured within CSI framework.

Proposal 5: For the definition of performance metric in Type 3 performance monitoring, intermediate KPI can be considered as starting point.

Proposal 6: For the definition of performance metric in Type 1 performance monitoring, there might be two alternatives:

- Alt1: UE calculates the performance metric or determines the comparison outcome based on the performance metric, and the threshold criterion is configured by NW.

- Alt2: UE recommends the decision of the LCM to NW.

Proposal 7: For data collection of AI/ML based CSI prediction, data collection mechanism of AI/ML based beam management can be reused as much as possible.

Proposal 8: For CSI prediction, the CSI related parameters in Rel-18 can be reused with modification to adapt AI/ML-enabled CSI prediction, e.g. the parameters about measurement window, reporting window.

# References

- RP- 242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- RP-233133, TR 38.843 v2.0.0, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface, Qualcomm

- Chairman's Notes RAN1#116, Athens, Greece, February 26th - March 1st, 2024.

- Chairman's Notes RAN1#116bis, Changsha, Hunan Province, China, April 15th – 19th, 2024.

- Chairman's Notes RAN1#117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

- Chairman's Notes RAN1#118, Maastricht, NL, August 19th – 23rd, 2024.

- Chairman's Notes RAN1#118bis, Hefei, China, October 14th – 18th, 2024.