3GPP TSG RAN WG1 meeting #119                                   R1-2409552

Orlando, US, November 18th – 22nd, 2024

Source: 	ZTE Corporation, Sanechips

Title:	Discussion on general aspects of physical layer design for Ambient IoT

Agenda Item:	9.4.2.1

Document for:	Discussion and decision

- Introduction

In RAN1 #118bis meeting, agreements and TPs on general aspects of physical layer design for Ambient IoT were reached in [1].

In this contribution, based on the agreements and TPs in last RAN1 meeting, the Ambient IoT physical layer design including waveform, modulation, coding, multiple access and bandwidths is further discussed.

- Discussion on A-IoT R2D

  - R2D waveform

In the last RAN1 meeting [1], TPs on Method Type 1 and Method Type 2 were agreed for R2D CP handling for OFDM based OOK waveform.

Agreement in RAN1#118bis Adopt the TPs below. #### 6.1.1.x R2D waveform, modulation and numerology …(unchanged parts omitted)… For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. For Method 1, two ways that CP location/length can be determined are studied: Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration For CP handling Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP. Alt M1-1 and/or Alt M1-2 (if both are supported) can be up to device implementation which may depend on M values. - Some sources [Futurwei][Nokia][Ericsson][EURECOM][Sharp][InterDigital][Qualcomm][LGE][Lenovo] [Spreadtrum][Samsung][Docomo] report device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location - Some sources [vivo][Huawei][CMCC][Futurewei] report the device implementation may depend on M values For CP handling Alt M1-1, the potential impacts are discussed as follows: - Some sources [Futurewei][Huawei][Spreadtrum][vivo][InterDigital][Qualcomm] report that CP handling of Alt M1-1 can be used for both small and large M values for OOK-4, while [vivo] reports that for large M values Alt M1-1 is used in combination with Alt M1-2. - Some sources [Ericsson][ZTE] report that CP handling of Alt M1-1 is challenging to be used for large M values for OOK-4 considering large SFO and [vivo][Apple] report that CP handling of Alt M1-1 may not completely remove CP samples due SFO impact. - Among of them, [Huawei] show that the performance loss of PRDCH carrying 20 bits due CP handling is negligible at 10% BLER even for large M values (e.g. M=24) under large SFO (e.g. 10^4-10^5 ppm). Sources [vivo][ZTE] show some performance loss due CP handling for both small (M=4) and large M values (M=24) under large SFO (e.g. 10^4-10^5 ppm ) while [ZTE] shows [1~2 dB] loss compare to no CP case for M<24, and an error floor at BLER=10% for M=24. - Some source [CMCC][Apple] report that the device needs additional complexity to handle CP, while other sources [Huawei][InterDigital] reports that it is feasible in terms of implementation complexity based on transition edge detection. For CP handling Alt M1-2, the potential impacts are discussed as follows: - Some sources [Futurewei][Nokia][CMCC][Ericsson][Huawei][Spreadtrum][vivo][ZTE][Apple][InterDigital][Docomo] report that CP handling of Alt M1-2 cannot be used for large M values, e.g. M>8, while [vivo] reports that for large M values Alt M1-2 is used in combination with Alt M1-1. - One source [LGE] report that CP handling of Alt M1-2 can be used for both small and large M values (e.g. M>8) if with the knowledge of OFDM symbol boundaries. - Among of them, [vivo] show that the performance of Alt M1-2 is not applicable for large M values (e.g. M=24) under large SFO (e.g. 10^4 ppm). For Method 2, two approaches regarding subcarrier orthogonality are studied: …(unchanged parts omitted)…. #### 6.1.1.x R2D waveform, modulation and numerology …(unchanged parts omitted)… For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. …(unchanged parts omitted)… For Method 2, two approaches regarding subcarrier orthogonality are studied: Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Alt M2-2: Method Type 2 does not retain subcarrier orthogonality. For CP handling Method Type 2, depends on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by, - M, and the length of OFDM symbol with CP - M, and the length of OFDM symbol without CP - Depending on detailed solutions, chip duration may or may not be constant. - Some source [Qualcomm] report that non-constant OOK chip duration may impact performance, while some other source [ZTE] report that non-constant OOK chip duration does not impact performance. For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple][Ericsson] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). - Some sources [Qualcomm][CMCC][ZTE] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. - Some sources [Ericsson][Huawei][CATT][ZTE][LGE][Nokia][DoCoMo] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [InterDigital][Futurewei][CMCC] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. For CP handling Alt M2-2, the solutions and potential impacts are discussed as follows, - [vivo][OPPO][CATT][Samsung] report solutions for Alt M2-2 (e.g. CP is copied from the start of OFDM symbol or do not insert CP to OFDM symbol). - [Ericsson][Huawei][Spreadtrum][Xiaomi][ZTE][InterDigital][Qualcomm][Nokia][CMCC][LGE][DOCOMO] report that CP handling of Alt M2-2 would cause interference to NR, while [vivo] reports single PRB guard band would be sufficient to handle interference. - Sources [Huawei][InterDigital][Qualcomm][Lenovo][CMCC][LGE][DOCOMO] report that CP handling of Alt M2-2 would increase the transmitter complexity..

For Method Type 1, specific schemes were described for two alternatives, i.e. Alt M1-1 and Alt M1-2 in the above TP. For Method Type 2, specific schemes were also mentioned for Alt M2-1-1 and Alt M2-2. But for Alt M2-1-2, there is only a purpose of this alternatives, i.e. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Actually, there were many schemes proposed by some sources, i.e. [ZTE], [CMCC] and [Samsung] in last RAN1 meeting[2][3][4]. If there is not any clarifications for the different schemes of Alt M2-1-2, observation and evaluations of Alt M2-1-2 from different sources will be considered as the same. In order to avoid misunderstanding and make the SI’s content much clearer, we propose to capture the schemes of Alt M2-1-2 from sources of [ZTE], [CMCC] and [Samsung].

For both R2D CP handling Method Type 1 and Method Type 2, alternatives except for  Alt M2-1-2 are described with specific schemes in the TP. If there is not any clarifications for the different schemes of Alt M2-1-2, observation and evaluations of Alt M2-1-2 from different sources will be considered as the same.

In order to avoid misunderstanding and make the SI’s content much clearer, we propose to capture the schemes of Alt M2-1-2 from sources of [ZTE], [CMCC] and [Samsung].

#### 6.1.1.x R2D waveform, modulation and numerology ***unchanged parts omitted*** For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Samsung][CMCC][ZTE] report different schemes of CP handling of Alt M2-1-2. - [Samsung] reports Alt M2-1-2: Ensure the last OOK chip within an OFDM symbol is not shorter than the CP. - [CMCC] reports Alt M2-1-2: Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1.The first OFDM symbol carries odd value M chips, and the following OFDM symbols carry even value M chips. where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1. - [ZTE] reports Alt M2-1-2: PRDCH transmission is started from the even number-th chip in the first OFDM symbol. The first odd number of chip(s) in the first OFDM symbol can be used for preamble transmission. - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple][Ericsson] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). - Some sources [Qualcomm][CMCC][ZTE] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. - Some sources [Ericsson][Huawei][CATT][ZTE][LGE][Nokia][DoCoMo] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [InterDigital][Futurewei][CMCC] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. - ***unchanged parts omitted***

According to the above analysis, the methods of Alt M2-1-2 are different from different sources. The observation of Alt M2-1 is based on the evaluation of Alt M2-1-1 and Alt M2-1-2 reported by [ZTE] in [2]. Therefore, we propose to capture the Alt M2-1-2 description under the associated observation of performance reported by [ZTE] in the TP.

The observation of Alt M2-1 is based on the evaluation of Alt M2-1-1 and Alt M2-1-2 reported by [ZTE] in [2].

Capture the Alt M2-1-2 description under the associated observation of performance reported by [ZTE] in the following TP.

#### 6.1.1.x R2D waveform, modulation and numerology ***unchanged parts omitted*** For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple][Ericsson] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). Source [ZTE] reports Alt M2-1-2 for evaluation is that PRDCH transmission is started from the even number-th chip in the first OFDM symbol. The first odd number of chip(s) in the first OFDM symbol can be used for preamble transmission. ***unchanged parts omitted***

  - R2D repetition

In the last RAN1 meeting [1], TPs on R2D repetitions were agreed. In this section, block-level repetition is discussed.

Agreement in RAN1#118bis For R2D repetitions, the following observations are captured in TR 38.769: - It is reported by sources [CATT] (only for R2D control, if supported), [OPPO], [ZTE], [NEC], [Samsung], [ETRI], [QC] and [IITK, IITM] that R2D repetitions should be supported. The following are observations from companies regarding the different types of repetition that should be supported: - Bit-level repetition Positive observations - Source [Ericsson] state that bit level repetition can be studied if coverage enhancement of the R2D link is required. - Source [OPPO] state that bit level repetition where every input bit repeated for 8 times before Manchester coding could have ~4dB gain when compared with no repetition. They claim using Manchester codes with repetitions require a simple structure and consumes extremely low power. - Source [QC] state that bit level repetitions with scrambling is required since the former would improve the link budget and the latter would add extra randomness to the information bits, providing gain by suppressing the interference. They also claim that repetitions can be used in devices that cannot soft combine the repetitions, and majority-based detection would offer gain for these devices. Negative observations - Source [CMCC] state that since envelope detection is used for R2D reception, bit level repetition may not provide expected gain for the reception. - Source [Vivo] state that though it may be feasible, it increases the device’s processing complexity for reception, e.g., combination, repetition parameters determination. - Block-level repetition Positive observations - Source [ZTE] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides the time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. will be received with sufficient quality to correctly decode the information gain. Negative observations - Source [Vivo] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded. - Chip-level repetition Positive observations - Source [CMCC] state that it may be useful for R2D transmission coverage and can be considered to generate a lower data rate than 7kbps. - Source [IITK, IITM] state that chip-level repetition increases the chip duration, improving the edge detection at the receiver, thereby having a ~2dB performance increase when compared to bit level repetitions. Negative observations - Sources [Ericsson][vivo] and [CATT] state that chip-level repetition is equivalent to long chip transmission, i.e., by using a smaller modulation index, and therefore, there is no need to support this option.

For R2D block-level repetition, during a long transmission duration, the channel state changes numerously. Therefore, device only performing the block-wise detection without chase combination of the repeated blocks can achieve a better performance. That is to say, block-level repetition may not need additional buffer and increase the complexity and cost. Therefore, we propose to capture the above observations of block-level repetition in TR

Device can perform the block-wise detection without chase combination of the repeated blocks so that block-level repetition may not need additional buffer and increase the complexity and cost.

Capture the observation on block-level repetition in TR.

***unchanged parts omitted*** - Block-level repetition Positive observations - Source [ZTE] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides the time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. will be received with sufficient quality to correctly decode the information gain.Device can perform the block-wise detection without chase combination of the repeated blocks so that block-level repetition may not need additional buffer and increase the complexity and cost. Negative observations - Source [Vivo] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded. ***unchanged parts omitted***

  - R2D time-domain definition

- Subcarrier spacing

According to the agreements in RAN1#116bis meeting [5], R2D study includes subcarrier spacing of 15 kHz, from the reader perspective. The inclusion of subcarrier spacing of 30 kHz is FFS. In our views, the inclusion of subcarrier spacing of 30 kHz depends on the operating band for A-IoT R2D transmission.

The inclusion of subcarrier spacing of 30 kHz depends on the operating band for A-IoT R2D transmission.

Agreement in RAN1#116bis meeting R2D study includes subcarrier spacing of 15 kHz, from the reader perspective, for OFDM-based waveform. - Inclusion in the study of subcarrier spacing of 30 kHz is FFS.

- Time unit(s)

According to the discussion in the last RAN1 meeting [6], proposals for the definition of time unit(s) are shown as followings.

- Proposal 2.7.2b(I): Capture in the TR that the following chip duration calculations have been studied, and FFS: whether/how to couple them with CP handling method(s): - Calculation 1: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is constant in one OFDM symbol. [Ericsson][Huawei][IDCC][Sharp][Spreadtrum][vivo] ][Futurewei] ][LGE] - Calculation 2: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is floating with e.g. T0−∆T0≤Tchip≤T0+∆T0, and FFS ∆T0T0. [CMCC] - Calculation 3: Chip duration = (1/M) × {OFDM symbol duration including CP part}. [Xiaomi] - Calculation 4: Chip duration = (1/M) × {OFDM symbol duration excluding CP part} for CP handling method type 1, while Chip duration = (1/M) × {OFDM symbol duration including CP part} with different M values in the odd and even OFDM symbols for CP handling method type 2. [DOCOMO] - Calculation 5: the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation [ZTE] - for the case of larger M values (e.g. M≥24), dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length; - for other cases, - if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part}; - if the number of IFFT points of an OFDM symbol can not be evenly divided by M, chip duration = Nf,refM∙Ts or Nf,refM∙Ts, where Ts/Tc=64.

Advantages and disadvantages for Calculations of chip duration are analyzed as followings:

- Calculation 1: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is constant in one OFDM symbol.

  - Advantages: chip duration is calculated as a constant value.

  - Disadvantages: chip duration can not aligned with the actual chip duration value if the number of IFFT points of an OFDM symbol can not be evenly divided by M. Moreover, if all chip duration is determined by ceiling operation, the total length of M chips may larger than an OFDM symbol duration and lead to non-orthogonality issue.

- Calculation 2: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is floating with e.g. T0−∆T0≤Tchip≤T0+∆T0, and FFS ∆T0T0.

  - Advantages: defining a floating range for chip duration.

  - Disadvantages: it can not ensure that the total length of M chip durations is equal to OFDM symbol duration.

- Calculation 3: Chip duration = (1/M) × {OFDM symbol duration including CP part}.

  - Advantages: chip duration is calculated as a constant value before CP removal.

  - Disadvantages: 1) chip duration can not aligned with the actual chip duration value if the total number of IFFT points and points of CP of an OFDM symbol can not be evenly divided by M; 2) chip duration is not a constant value under the cases of different CP lengths; 3) it will increase R2D signal generation complexity and wast timing resource if CP is used as a part of chip duration because of CP is normally generated by the end of the OFDM symbol duration.

- Calculation 4: Chip duration = (1/M) × {OFDM symbol duration excluding CP part} for CP handling method type 1, while Chip duration = (1/M) × {OFDM symbol duration including CP part} with different M values in the odd and even OFDM symbols for CP handling method type 2. [DOCOMO]

  - Advantages: considering the calculation of chip duration for both CP handling M1 and M2.

  - Disadvantages: it merges the disadvantages of both Calculation 1 and Calculation 3.

- Calculation 5: the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation [ZTE]

  - for the case of larger M values (e.g. M≥24), dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length;

  - for other cases,

    - if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part};

    - if the number of IFFT points of an OFDM symbol can not be evenly divided by M, chip duration = Nf,refM∙Ts or Nf,refM∙Ts, where Ts/Tc=64.

  - Advantages: 1) considering the calculation of chip duration for both CP handling M1 and M2. 2) defining the specific points of each chip in an OFDM symbol which is aligned with the actual chip duration. 3) the total length of M chip(s) should be equal to the duration of OFDM symbol. 4) ensure no additional transition edge during CP under the cases of large M values.

  - Disadvantages: the Chip duration is not constant for some CP handling methods.

In our views, according to the analysis on advantages and disadvantages of potential Calculations, Calculation 5 should be the correct way for determining the chip duration and should be captured in TR 38.769.

Calculation 5 should be captured in TR 38.769.

Moreover, the chip duration calculation is related to the details of CP handling, which is usually to be completed in the normative works. Because the potential alternatives for CP handling were captured in TR we think that it is OK to summarize and capture the potential calculation methods for chip duration, which correspond to different CP handling methods. Therefore, we propose to capture the above advantages and disadvantages for the potential Calculations of chip duration in TR 38.769.

Because the potential alternatives for CP handling were captured in TR we think that it is OK to summarize and capture the potential calculation methods for time unit(s), which correspond to different CP handling methods.

Capture the following calculation method for time unit(s) in TR.

For R2D time-domain definition, the following observations are captured in TR 38.769: - Calculation 1: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is constant in one OFDM symbol. - Advantages: chip duration is calculated as a constant value. - Disadvantages: chip duration can not aligned with the actual chip duration value if the number of IFFT points of an OFDM symbol can not be evenly divided by M. Moreover, if all chip duration is determined by ceiling operation, the total length of M chips may larger than an OFDM symbol duration and lead to non-orthogonality issue. - Calculation 2: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is floating with e.g. T0−∆T0≤Tchip≤T0+∆T0, and FFS ∆T0T0. - Advantages: defining a floating range for chip duration. - Disadvantages: it can not ensure that the total length of M chip durations is equal to OFDM symbol duration. - Calculation 3: Chip duration = (1/M) × {OFDM symbol duration including CP part}. - Advantages: chip duration is calculated as a constant value before CP removal. - Disadvantages: 1) chip duration can not aligned with the actual chip duration value if the total number of IFFT points and points of CP of an OFDM symbol can not be evenly divided by M; 2) chip duration is not a constant value under the cases of different CP lengths; 3) it will increase R2D signal generation complexity and wast timing resource if CP is used as a part of chip duration because of CP is normally generated by the end of the OFDM symbol duration. - Calculation 4: Chip duration = (1/M) × {OFDM symbol duration excluding CP part} for CP handling method type 1, while Chip duration = (1/M) × {OFDM symbol duration including CP part} with different M values in the odd and even OFDM symbols for CP handling method type 2. [DOCOMO] - Advantages: considering the calculation of chip duration for both CP handling M1 and M2. - Disadvantages: it merges the disadvantages of both Calculation 1 and Calculation 3. - Calculation 5: the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation [ZTE] - for the case of larger M values (e.g. M≥24), dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length; - for other cases, - if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part}; - if the number of IFFT points of an OFDM symbol can not be evenly divided by M, chip duration = Nf,refM∙Ts or Nf,refM∙Ts, where Ts/Tc=64. - Advantages: 1) considering the calculation of chip duration for both CP handling M1 and M2. 2) defining the specific points of each chip in an OFDM symbol which is aligned with the actual chip duration. 3) the total length of M chip(s) should be equal to the duration of OFDM symbol. 4) ensure no additional transition edge during CP under the cases of large M values. - Disadvantages: the Chip duration is not constant for some CP handling methods.

- Discussion on A-IoT D2R

  - D2R coding

    - Small frequency shift

In last RAN1 meeting [1], the agreements of small frequency shift were agreed as below.

Agreement in RAN1#118bis For small frequency shifts in D2R using Manchester line codes by repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length). Agreement in RAN1#118bis For small frequency shifts in D2R using Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2), the multiplication is performed according to the following options: - One option is that the multiplication operation is an XOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. - Another option is that the multiplication operation is an XNOR operation between Manchester codeword corresponding to the information bit and the square wave for the small frequency shift. Agreement Regarding small frequency shift factor, capture in the TR that: - For study purposes - For BPSK and OOK, the small-frequency-shift factor is defined as the ratio of bit length to two times the chip length - The value of small-frequency-shift factor depends on details of design, which according to proposals considered could be e.g. - The repetition number of Manchester codeword within one information bit duration - The M value of Miller in Figure 6-12 of UHF RFID standard - The number of square-wave periods within one information bit duration Agreement For small frequency shifts in D2R, the following observations are captured in TR 38.769: - Sources [Futurewei], [Vivo] and [Qualcomm] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is equivalent to bit-level repetitions within the same duration prior to Manchester encoding. - Sources [Vivo], [ZTE] and [NEC] state that the output waveform for Manchester line codes by multiplying the codeword with a square wave corresponding to the small frequency shift (Option 2) introduces a phase reversal of the output waveform in the middle of the time duration corresponding to an information bit as compared to Manchester line codes by repetition of the codewords within the same time duration. - Sources [Futurewei], [Ericsson] [CMCC] [QC] [IDC] [LG] state that the output waveform using a square-wave corresponding to the small frequency-shift and no line codes is equivalent to using Manchester line codes by repetition of the codewords within the same time duration corresponding to an information bit. - Source [ZTE] shows that small frequency shift (Option 1) is more sensitive to SFO and the time difference between devices than small frequency shift (Option 2), and small frequency shift via Miller code has the worst performance compared with small frequency shift (Option 1) and small frequency shift (Option 2).

The amount of small frequency shift for option 1 has been agreed as the above agreement (highlighted in yellow). In our views, in order to make a clear description of the amount of small frequency shift, the calculation of amount of small frequency shift for option 2 should be also presented in TR 38.769.

The amount of small frequency shift for option 1 has been agreed in RAN1#118bis meeting, while the amount of small frequency shift for option 2 has no clear description.

Capture the following calculation method (Option 2) for the amount of small frequency shift in Hz in TR 38.769.

- For small frequency shifts in D2R using Manchester line codes by multiplying the codeword with a square wave within the same time duration Tb corresponding to an information bit (Option 2), each information bit includes square wave number M (i.e. the ratio of bit length to two times the chip length), where M = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is M/Tb = 1/(2 * chip length).

    - Repetition

In last RAN1 meeting [1], the agreements of D2R repetitions were agreed as below.

Agreement in RAN1#118bis For D2R repetitions, the following observations are captured in TR 38.769: - It is reported by sources [Huawei], [Nokia], [TCL], [Vivo], [CMCC], [Xiaomi], [CATT], [ZTE], [Samsung] that block level repetitions should be supported. General observations - Source [Huawei] state that detection of D2R can obtain combining gain and time diversity gain by the block repetition - Sources [Huawei] and [Vivo] state that it can enable BS to terminate reception early if it successfully decodes the block. - Source [CATT] state that with small D2R information block size, memory size of A-IoT devices is comparable for bit level repetition and block level repetition, but compared to block level repetition, bit level repetition would degrade power and BLER performance of D2R transmission. - Source [Nokia] and [TCL] state that block level repetition would be a TB level repetition, while [Nokia] goes on to state that the transmission of a PDRCH multiple times may go through multiple channels showing the diversity gain/ channel averaging impact of a specific binary data. Its implementation/specification impact would be minimum. - Source [Vivo], ZTE state that it may require larger buffer than bit level repetition at device side, depending on device implementation. Performance comparisons - Source [Huawei] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding. - Source [Xiaomi] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1. - Source [ZTE] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions and the performance between block level repetition and bit level repetition type 2 is the same. - Source [Vivo] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping. - Sources [CATT] and [ZTE] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel. Agreement in RAN1#118bis For D2R repetitions, the following observations are captured in TR 38.769: - It is reported by sources [ZTE], and [Samsung] that bit level repetitions should be supported. - Source [Huawei] state that for bit level type 1, the repetition data bit is decoded after FEC detection, and hence, the digital data bit cannot be combined with combining gain. - Source [ZTE] state that the performance of bit level type 1 repetition is degraded by ~3.5dB @ 10% BLER under the cases of AWGN and TDL-A channel when compared to bit level type 2. - Source [Huawei] state that since the bit level type 2 repetition is performed after FEC at the transmitter, at the receiver side, each bit would have a log-likelihood ratio (LLR) that can be combined and provided as the input for FEC decoding, thereby providing a better performance as compared to bit level type 1.

For block-based repetition, A-IoT devices need to buffer the entire coded bits or repeatedly encode (FEC and line coding), which increases complexity.

Here, we propose a repetition method in the context of convolutional coding, as shown in Figure 1 where the repetition is 1 time. The k-th bit of the information sequence is input into the CC encoder, and output 4 coded bits(,, and ) , which are then repeated on a block basis. For example, the input information sequence with a length of K bits is , and the sequence after CC encoding and repetition operation is . Since the encoding, repetition and data transmission are carried out simultaneously, A-IoT devices do not require buffering all the coded bits, resulting in simpler hardware implementation.

As shown in Figure 2, it illustrates a performance comparison between using block level repetition and the proposed repetition. It can be observed that the proposed repetition has the same performance as block level repetition under the case of TDL-A channel.

Simulation assumptions

|  | Configurations | Notes |
| --- | --- | --- |
| Information block length | 96 bits | BLER is computed per information block. |
| CRC length | 16 bits |  |
| Modulation | BPSK (TBCC) OOK (Manchester) |  |
| Manchester | R=1/2 |  |
| TBCC | k=1, n=4; polynomials with g0=1338, g1=1718, g0=1658 , and g1=1178; mother code rate = 1/4; constrain length = 7. | Soft input Viterbi decoder |
| Repetition | The Repetition denotes twice of repetition for each output 4 coded bits of TBCC coding for each input information bit. Decoder performs soft combinations. |  |

Repeat on a block (n coded bits of the convolutional coding, n is the number of polynomials) basis, A-IoT devices do not require buffering coded bits, resulting in simpler hardware.

Figure 1 Repetition after TBCC coding

Figure 21 BLER performance comparison between block level repetition and proposed repetition

It is proposed to be captured in TR 38.769.

For D2R repetitions, the following observations are captured in TR 38.769: - It is reported by sources [Huawei], [Nokia], [TCL], [Vivo], [CMCC], [Xiaomi], [CATT], [ZTE], [Samsung] that block level repetitions should be supported. General observations - Source [Huawei] state that detection of D2R can obtain combining gain and time diversity gain by the block repetition - Sources [Huawei] and [Vivo] state that it can enable BS to terminate reception early if it successfully decodes the block. - Source [CATT] state that with small D2R information block size, memory size of A-IoT devices is comparable for bit level repetition and block level repetition, but compared to block level repetition, bit level repetition would degrade power and BLER performance of D2R transmission. - Source [Nokia] and [TCL] state that block level repetition would be a TB level repetition, while [Nokia] goes on to state that the transmission of a PDRCH multiple times may go through multiple channels showing the diversity gain/ channel averaging impact of a specific binary data. Its implementation/specification impact would be minimum. - Source [Vivo], ZTE state that it may require larger buffer than bit level repetition at device side, depending on device implementation. [ZTE] reports an enhanced block-level repetition scheme with no need for device to buffer all coded bits. The enhanced block-level repetition scheme is that n coded bits of the convolutional coding are repeated for each input information bit, n is the number of polynomials, such as the output sequence after repetition is , where are the coded bits for the k-th bit of the information sequence. Performance comparisons - Source [Huawei] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding. - Source [Xiaomi] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1. - Source [ZTE] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions and the performance between block level repetition and bit level repetition type 2 is the same. Moreover, the enhanced block-level repetition scheme has the same performance as block level repetition under the case of TDL-A channel. - Source [Vivo] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping. - Sources [CATT] and [ZTE] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel.

  - D2R multiple access

In last RAN1 meeting [1], agreements on D2R CDMA were reached as following. In this section, observations on CDMA schemes are discussed.

Agreement in RAN1#118bis Add the following TP to the TR section on “D2R multiple access” for CDMA: 6.1.2.x D2R multiple access (…) According to sources [CATT][IIT][OPPO][ZTE], CDMA can improve the resource utilization efficiency without increasing the device complexity significantly. Sources [DOCOMO] thinks CDMA would help the multiplexing among readers and it can alleviate the cross-link interference. Source [OPPO] thinks CDMA is also beneficial for the latency reduction and success rate improvement of access procedure. However, sources [Apple, ASUSTek, Ericsson, FUTUREWEI, Huawei, Spreadtrum, vivo] show concerns on the necessity and feasibility of CDMA, especially considering the limited capability (e.g., large SFO/CFO) of Ambient IoT devices and the cost (additional memories to store a set of codewords at device) versus benefits. In detail, the observations are as follows. The impact of SFO on the performance of CDMA is studied as follows. - In the case of large SFO (e.g., X = 5), - Source [Spreadtrum] think that the orthogonality between different codes/sequences will be severely disrupted, as the large SFO will accumulate an additional sampling error of 10 points of 100 points. - Source [vivo] think that the increased inter-device interference would materially degrade D2R performance, e.g., increased false alarm rate and miss detection probability, which in turn reduce spectrum efficiency even lower than the case of simple TDMA. - Source [FUTUREWEI] think that the accurate timing and power control required by CDMA are far-fetched for Ambient IoT devices, referring to the IS-95 CDMA system. - Source [ZTE] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 4-length orthogonal code is feasible for D2R transmission carrying 20 information bits. - Source [Qualcomm] state that quite large number of SFO hypotheses is necessary at reader to achieve reasonable false-alarm and miss detection probabilities with the SFO of [0.1 – 1] * 105 ppm. - Source [Huawei] state that correlation properties of sequences are severely damaged with the SFO of 105 ppm. Besides, D2R receiver fails to estimate the SFO of each of the parallel D2R transmissions for the SFO of 105 ppm. - Source [OPPO] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). For 1% missed-detection rate, simulation results showed that m-sequences and Gold sequences are able to achieve this performance level when SNR is about -24dB and -23dB, respectively. - In the case of relatively smaller SFO (e.g., X = 4), - Source [Qualcomm] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm with reasonable number of SFO hypotheses at reader. The impact of CFO on the performance of CDMA is studied for Device 2b as follows. - Source [Huawei] state that codeword detection at D2R receiver fails considering non-coherent demodulation has to be used due to the quick phase rotation caused by the residual CFO of e.g. 10s or 100s of Hz after CFO estimation and correction. The impact of timing offset between CDMed D2R transmissions on the performance of CDMA is studied for Device 2b as follows. - Source [Lenovo] state that binary modulated orthogonal sequence such as Golay sequence can tolerate timing error by selecting a suitable cyclic shift spacing. - Source [OPPO] state that it is possible to detect multiple transmitters with timing difference and power difference among devices. - Source [MTK] think that poor synchronization performance in time and frequency domain of device would degrade the code orthogonality and thus results in a bad cross-correlation performance. - Source [Samsung] think that the different propagation delays from devices may also degrades decoding performance. - Source [ZTE] state that the negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced synchronization sequence and enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission. The impact of power variation between devices on the performance of CDMA is studied as follows. - Source [Qualcomm] state that the larger power variation severely degrades the capacity of CDMA for msg-1. - Source [ZTE] state that the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - Source [Ericsson] think that using spreading sequence can lead to transmitting a larger number of bits which can be extremely inefficient considering that the devices are extremely power inefficient. - Source [Ericsson] think that CDMA might be too complex to implement in A-IoT devices, which might involve complexities with generating orthogonal sequences. - Source [MTK] think that a large device density (e.g., 150 devices per 100 m2 for indoor scenarios per TR) requires a long code sequence, which is challenging for the device with limited buffer size. - Source [TCL] think that CDMA leads to higher power consumption and lower data rate. - Source [vivo] think that the usable number of binary sequences would be much smaller due to impairment such as timing/frequency error and interference. - Source [OPPO] state that in comparison to RN16, when Msg. 1 is transmitted using RACH preamble m-sequences or Gold sequences, the number of usable binary sequences that can be used is large since the base sequence design from LTE and NR can be reused. - Source [OPPO] state RN16 cannot tolerate collision for any one of its bits. Once collided, the bit sequence is changed and became non-detectable. On the other hand, m-sequences and Gold sequences are able to tolerate transmission overlap. - Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 16-length or 64-length orthogonal code improve the D2R transmission performance and multiplexing capacity compared with using 4-length orthogonal code for mapping. - Source [ZTE] state that BPSK modulation for the CDMA improve the D2R transmission performance and multiplexing capacity compared with OOK based modulation..

In the last RAN1 meeting, we illustrated the performances of CDMA for D2R transmission with different device numbers under the assumption of different SFO, TO, TBS, FEC schemes, modulation schemes and lengths of spreading code, and the performances were compared with the corresponding performances of the repetition scheme with single device and FDMA scheme with multiple devices in [2]. The performance differences between CDMA scheme with multiple devices and repetition scheme with single device can be further observed. Besides, the performance difference between CDMA scheme and FDMA scheme when with the same device number can be also observed. We suggest to capture these additional observations in TP, which are detailed as follows:

The impact of SFO on the performance of CDMA is studied as follows.

- In the case of large SFO (e.g., X = 5),

*** unchanged are omitted ***

  - Source [ZTE] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 4-length orthogonal code is feasible for D2R transmission carrying 20 information bits.

  - Source [ZTE] state that convolutional codes and BPSK modulation for the CDMA improve the D2R transmission performance with multiple multiplexing devices. Source [ZTE] state that for D2R transmission with TBS16+CRC0, the performance difference between the CDMA scheme using 4-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is less than 0.5dB, and the CDMA scheme has 1~2dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers. While for D2R transmission with TBS96+CRC16, the performance difference between the CDMA scheme with 4-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is also less than 0.5dB, and the CDMA scheme has 1~2.5dB SNR gain at 10%BLER compared with FDMA scheme at the same multiplexing device numbers.

*** unchanged are omitted ***

Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions:

*** unchanged are omitted ***

- Source [ZTE] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 16-length or 64-length orthogonal code improve the D2R transmission performance and multiplexing capacity compared with using 4-length orthogonal code for mapping. Source [ZTE] state that for D2R transmission with TBS16+CRC0 under the assumption of SFO being 1E4~1E5, the performance difference between the CDMA scheme with 64-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is about 1dB, and the CDMA scheme has about 2.5dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers.

- Source [ZTE] state that BPSK modulation and convolutional code for the CDMA further improve the D2R transmission performance with multiple multiplexing devices and multiplexing capacity compared with OOK based modulation. Source [ZTE] state that for D2R transmission with TBS16+CRC0 under the assumption of SFO being 1E4~1E5 and using convolutional codes and BPSK modulation, the performance difference between the CDMA scheme using 16-length or 64-length orthogonal code for 6 devices and the repetition scheme for single device at 1%BLER is less than 1dB, and the CDMA scheme has about 9dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers.

  - D2R time-domain definition

According to the discussions in the last RAN1 meeting [6], proposal on D2R chip duration calculation is as following.

Proposal 3.7.1b(I): Capture in the TR that the following chip duration calculations have been studied: - Method 1: Calculated according to the transmission bandwidth, and depending on small frequency shift factor. [CMCC] [Ericsson][Huawei][Spreadtrum][vivo][Xiaomi][ZTE] - FL NOTE: The small frequency-shift factor is in Section 3.3.2, and FL would arrange the TR text suitably after agreements. - Method 2: One of a pre-defined set of pulse time durations. [Lenovo][vivo] - E.g., D2R chip duration is an integer multiple (or fraction) of R2D chip duration, where a reference SCS (e.g. the R2D SCS) is used as the basis for the value of D2R chip duration. [Sharp]

In our views, because D2R transmission is based on non-OFDM waveform, the chip duration is related to the bandwidth under the case of D2R transmission without frequency shift, i.e. a chip duration=1/(Btx,D2R/2) for double sideband modulation. For the case of D2R transmission with frequency shift, chip duration calculations should consider both transmission bandwidth and small frequency shift factor. Therefore, Method 1 should be captured in TR 38.769.

Capture Method 1 in TR 38.769.

- Method 1: Calculated according to the transmission bandwidth, and depending on small frequency shift factor.

  - D2R bandwidths

In last RAN1 meeting [1], agreements on D2R bandwidth were reached as followings. In this section, the values of transmission bandwidths Btx,D2R that can be supported for A-IoT D2R are discussed.

Agreement in RAN1#118bis State in the TR that: - Bocc,D2R of the D2R transmission associated with one/each single-tone of a carrier wave would be impacted by at least the following aspects: - A possible guard band Bguard,D2R due to at least SFO - A possible guard band Bguard,D2R due to CFO for Device 2b Agreement in RAN1#118bis For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 1, and if no D2R line-code is used (i.e., by using a square-wave corresponding to the small frequency-shift) in the TR, where: - Fc is the carrier-wave frequency, at least when externally generated - FSx depict examples of amount of small frequency shift - For 2SB transmission, +FSx and -FSx parts are included. - For 1SB transmission, FFS which part is included. Agreement in RAN1#118bis For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 1, and if no D2R line-code is used (i.e., by using a square-wave corresponding to the small frequency-shift) in the TR, where: - Fc is the carrier-wave frequency, at least when externally generated - FSx depict examples of amount of small frequency shift - For 1SB transmission, one of the following figures applies as appropriate. Agreement For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 2, and by Miller line-code in the TR: Agreement For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier-wave, capture the following figure for small frequency-shift by: Manchester with option 2, and by Miller line-code in the TR, where: - Fc is the carrier-wave frequency, at least when externally generated - FSx depict examples of amount of small frequency shift - For 1SB transmission, one of the following figures applies as appropriate.

For A-IoT D2R transmission based on single-tone and multi-tone waveform, the Btx,D2R is associated with the data rate. In RFID 18000-C specification, the data rate is in a range of 20 to 640kbps, while A-IoT D2R can be designed to support a same or larger range of data rate than RFID. Therefore, the potential values of Btx,D2R and chip duration corresponding to different data rate for the case of double sideband modulation without frequency shift are described as the following three options.

- option1: chip length{133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.16μs, 2.08μs, 1.04μs, 0.52μs}; DSB bandwidth{15, 30, 60, 120, 240, 480, 960, 1920, 3840}kHz

- option2: chip length{133.33μs, 66.67μs, 26.67μs, 13.34μs, 6.67μs, 3.33μs, 1.67μs, 0.84μs}; DSB bandwidth{15, 30, 75, 150, 300, 600, 1200, 2400}kHz

- option3: chip length{133.33μs, 66.67μs, 44.44μs, 22.22μs, 11.11μs, 5.56μs, 2.78μs, 1.39μs, 0.69μs}; DSB bandwidth{15, 30, 45, 90, 180, 360, 720, 1440, 2880}kHz

Though it shows that A-IoT D2R transmission can support the data rate in the range of 3.75 to 960kbps based on the three options, the highest bandwidth of option 1 is much larger than that of RFID. Therefore, we propose that the highest bandwidth can be selected between 2400kHz and 2880kHz for option 1, that the multiple between the highest LF (2400kHz or 2880kHz) and the second highest LF (1920kHz) can be less than 2, which is similar to the case of RFID link frequency {40, 95, 107, 160, 256, 320, 465}kHz with DR=8.

A-IoT D2R transmission can support the data rate in the range of 3.75 to 960kbps which covers the data rate supported by RFID reader-to-tag transmission.

The highest bandwidth can be selected between 2400kHz and 2880kHz for option 1, that the multiple between the highest LF (2400kHz or 2880kHz) and the second highest LF (1920kHz) can be less than 2, which is similar to the case of RFID link frequency {40, 95, 107, 160, 256, 320, 465}kHz with DR=8.

Capture the following three options for combinations of Btx,D2R and chip duration for D2R can be used for D2R study in TR 38.769.

- option1: chip length{133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.16μs, 2.08μs, 1.04μs, 0.52μs}; DSB bandwidth{15, 30, 60, 120, 240, 480, 960, 1920, 3840}kHz

- option2: chip length{133.33μs, 66.67μs, 26.67μs, 13.34μs, 6.67μs, 3.33μs, 1.67μs, 0.84μs}; DSB bandwidth{15, 30, 75, 150, 300, 600, 1200,2400}kHz

- option3: chip length{133.33μs, 66.67μs, 44.44μs, 22.22μs, 11.11μs, 5.56μs, 2.78μs, 1.39μs, 0.69μs}; DSB bandwidth{15, 30, 45, 90, 180, 360, 720, 1440, 2880}kHz

- Conclusion

In this contribution, we discuss the Ambient IoT physical layer design and have the following observations and proposals.

Observation 1: For both R2D CP handling Method Type 1 and Method Type 2, alternatives except for  Alt M2-1-2 are described with specific schemes in the TP. If there is not any clarifications for the different schemes of Alt M2-1-2, observation and evaluations of Alt M2-1-2 from different sources will be considered as the same.

Observation 2: The observation of Alt M2-1 is based on the evaluation of Alt M2-1-1 and Alt M2-1-2 reported by [ZTE] in [2].

Observation 3: Device can perform the block-wise detection without chase combination of the repeated blocks so that block-level repetition may not need additional buffer and increase the complexity and cost.

Observation 4: Because the potential alternatives for CP handling were captured in TR we think that it is OK to summarize and capture the potential calculation methods for time unit(s), which correspond to different CP handling methods.

Observation 5: The amount of small frequency shift for option 1 has been agreed in RAN1#118bis meeting, while the amount of small frequency shift for option 2 has no clear description.

Observation 6: Repeat on a block (n coded bits of the convolutional coding, n is the number of polynomials) basis, A-IoT devices do not require buffering coded bits, resulting in simpler hardware.

Observation 7: A-IoT D2R transmission can support the data rate in the range of 3.75 to 960kbps which covers the data rate supported by RFID reader-to-tag transmission.

Observation 8: The highest bandwidth can be selected between 2400kHz and 2880kHz for option 1, that the multiple between the highest LF (2400kHz or 2880kHz) and the second highest LF (1920kHz) can be less than 2, which is similar to the case of RFID link frequency {40, 95, 107, 160, 256, 320, 465}kHz with DR=8.

Proposal 1: In order to avoid misunderstanding and make the SI’s content much clearer, we propose to capture the schemes of Alt M2-1-2 from sources of [ZTE], [CMCC] and [Samsung].

#### 6.1.1.x R2D waveform, modulation and numerology ***unchanged parts omitted*** For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Samsung][CMCC][ZTE] report different schemes of CP handling of Alt M2-1-2. - [Samsung] reports Alt M2-1-2: Ensure the last OOK chip within an OFDM symbol is not shorter than the CP. - [CMCC] reports Alt M2-1-2: Change a=b≠c case to a=b=c case by changing OOK-4 M value to adjacent M+1.The first OFDM symbol carries odd value M chips, and the following OFDM symbols carry even value M chips. where a, b and c is the last chip of symbol n, first chip of symbol n+1 and last chip of symbol n+1. - [ZTE] reports Alt M2-1-2: PRDCH transmission is started from the even number-th chip in the first OFDM symbol. The first odd number of chip(s) in the first OFDM symbol can be used for preamble transmission. - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple][Ericsson] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). - Some sources [Qualcomm][CMCC][ZTE] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. - Some sources [Ericsson][Huawei][CATT][ZTE][LGE][Nokia][DoCoMo] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [InterDigital][Futurewei][CMCC] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. - ***unchanged parts omitted***

Proposal 2: Capture the Alt M2-1-2 description under the associated observation of performance reported by [ZTE] in the following TP.

#### 6.1.1.x R2D waveform, modulation and numerology ***unchanged parts omitted*** For CP handling Alt M2-1, the potential impacts are discussed as follows, - Some sources [Huawei][CMCC][vivo][Fujitsu]Samsung][CATT][Apple][Ericsson] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). - Some sources [Futurewei][Spreadtrum][Qualcomm][ZTE] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [Spreadtrum][ZTE] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 10^5 ppm). Source [ZTE] reports Alt M2-1-2 for evaluation is that PRDCH transmission is started from the even number-th chip in the first OFDM symbol. The first odd number of chip(s) in the first OFDM symbol can be used for preamble transmission. ***unchanged parts omitted***

Proposal 3: Capture the observation on block-level repetition in TR.

***unchanged parts omitted*** - Block-level repetition Positive observations - Source [ZTE] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides the time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. will be received with sufficient quality to correctly decode the information gain.Device can perform the block-wise detection without chase combination of the repeated blocks so that block-level repetition may not need additional buffer and increase the complexity and cost. Negative observations - Source [Vivo] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded. ***unchanged parts omitted***

Proposal 4: The inclusion of subcarrier spacing of 30 kHz depends on the operating band for A-IoT R2D transmission.

Proposal 5: Calculation 5 should be captured in TR 38.769.

Proposal 6: Capture the following calculation method for time unit(s) in TR.

For R2D time-domain definition, the following observations are captured in TR 38.769: - Calculation 1: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is constant in one OFDM symbol. - Advantages: chip duration is calculated as a constant value. - Disadvantages: chip duration can not aligned with the actual chip duration value if the number of IFFT points of an OFDM symbol can not be evenly divided by M. Moreover, if all chip duration is determined by ceiling operation, the total length of M chips may larger than an OFDM symbol duration and lead to non-orthogonality issue. - Calculation 2: Chip duration = (1/M) × {OFDM symbol duration excluding CP part}, where chip duration is floating with e.g. T0−∆T0≤Tchip≤T0+∆T0, and FFS ∆T0T0. - Advantages: defining a floating range for chip duration. - Disadvantages: it can not ensure that the total length of M chip durations is equal to OFDM symbol duration. - Calculation 3: Chip duration = (1/M) × {OFDM symbol duration including CP part}. - Advantages: chip duration is calculated as a constant value before CP removal. - Disadvantages: 1) chip duration can not aligned with the actual chip duration value if the total number of IFFT points and points of CP of an OFDM symbol can not be evenly divided by M; 2) chip duration is not a constant value under the cases of different CP lengths; 3) it will increase R2D signal generation complexity and wast timing resource if CP is used as a part of chip duration because of CP is normally generated by the end of the OFDM symbol duration. - Calculation 4: Chip duration = (1/M) × {OFDM symbol duration excluding CP part} for CP handling method type 1, while Chip duration = (1/M) × {OFDM symbol duration including CP part} with different M values in the odd and even OFDM symbols for CP handling method type 2. [DOCOMO] - Advantages: considering the calculation of chip duration for both CP handling M1 and M2. - Disadvantages: it merges the disadvantages of both Calculation 1 and Calculation 3. - Calculation 5: the total length of M chip(s) should be equal to the duration of OFDM symbol, i.e. the smallest time unit of resource allocation [ZTE] - for the case of larger M values (e.g. M≥24), dedicated chip duration is introduced to ensure no additional transition edge introduced by CP, i.e. chip duration is equal to CP length; - for other cases, - if the number of IFFT points of an OFDM symbol can be evenly divided by M, chip duration = (1/M) × {OFDM symbol duration excluding CP part}; - if the number of IFFT points of an OFDM symbol can not be evenly divided by M, chip duration = Nf,refM∙Ts or Nf,refM∙Ts, where Ts/Tc=64. - Advantages: 1) considering the calculation of chip duration for both CP handling M1 and M2. 2) defining the specific points of each chip in an OFDM symbol which is aligned with the actual chip duration. 3) the total length of M chip(s) should be equal to the duration of OFDM symbol. 4) ensure no additional transition edge during CP under the cases of large M values. - Disadvantages: the Chip duration is not constant for some CP handling methods.

Proposal 7: Capture the following calculation method (Option 2) for the amount of small frequency shift in Hz in TR 38.769.

- For small frequency shifts in D2R using Manchester line codes by multiplying the codeword with a square wave within the same time duration Tb corresponding to an information bit (Option 2), each information bit includes square wave number M (i.e. the ratio of bit length to two times the chip length), where M = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is M/Tb = 1/(2 * chip length).

Proposal 8: It is proposed to be captured in TR 38.769.

For D2R repetitions, the following observations are captured in TR 38.769: - It is reported by sources [Huawei], [Nokia], [TCL], [Vivo], [CMCC], [Xiaomi], [CATT], [ZTE], [Samsung] that block level repetitions should be supported. General observations - Source [Huawei] state that detection of D2R can obtain combining gain and time diversity gain by the block repetition - Sources [Huawei] and [Vivo] state that it can enable BS to terminate reception early if it successfully decodes the block. - Source [CATT] state that with small D2R information block size, memory size of A-IoT devices is comparable for bit level repetition and block level repetition, but compared to block level repetition, bit level repetition would degrade power and BLER performance of D2R transmission. - Source [Nokia] and [TCL] state that block level repetition would be a TB level repetition, while [Nokia] goes on to state that the transmission of a PDRCH multiple times may go through multiple channels showing the diversity gain/ channel averaging impact of a specific binary data. Its implementation/specification impact would be minimum. - Source [Vivo], ZTE state that it may require larger buffer than bit level repetition at device side, depending on device implementation. [ZTE] reports an enhanced block-level repetition scheme with no need for device to buffer all coded bits. The enhanced block-level repetition scheme is that n coded bits of the convolutional coding are repeated for each input information bit, n is the number of polynomials, such as the output sequence after repetition is , where are the coded bits for the k-th bit of the information sequence. Performance comparisons - Source [Huawei] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding. - Source [Xiaomi] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1. - Source [ZTE] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions and the performance between block level repetition and bit level repetition type 2 is the same. Moreover, the enhanced block-level repetition scheme has the same performance as block level repetition under the case of TDL-A channel. - Source [Vivo] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping. - Sources [CATT] and [ZTE] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel.

Proposal 9: Capture Method 1 in TR 38.769.

 Method 1: Calculated according to the transmission bandwidth, and depending on small frequency shift factor.

Proposal 10: Capture the following three options for combinations of Btx,D2R and chip duration for D2R can be used for D2R study in TR 38.769.

- option1: chip length{133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.16μs, 2.08μs, 1.04μs, 0.52μs}; DSB bandwidth{15, 30, 60, 120, 240, 480, 960, 1920, 3840}kHz

- option2: chip length{133.33μs, 66.67μs, 26.67μs, 13.34μs, 6.67μs, 3.33μs, 1.67μs, 0.84μs}; DSB bandwidth{15, 30, 75, 150, 300, 600, 1200,2400}kHz

- option3: chip length{133.33μs, 66.67μs, 44.44μs, 22.22μs, 11.11μs, 5.56μs, 2.78μs, 1.39μs, 0.69μs}; DSB bandwidth{15, 30, 45, 90, 180, 360, 720, 1440, 2880}kHz

- References

Draft_Minutes_report_RAN1#118bis_v020.

R1-2408067 Discussion on general aspects of physical layer design for Ambient IoT,	ZTE Corporation, Sanechips.

R1-2407906 Discussion on general aspects of A-IoT physical layer design, Samsung.

R1-2408647 Considerations for general aspects of Ambient IoT, CMCC.

Draft Report of 3GPP TSG RAN WG1 #116bis v0.3.0.

R1-2409259	Feature Lead Summary #4 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design” Moderator (Huawei)

- Annex

Table A.1: Simulation assumptions

| Parameters |  | Assumptions |
| --- | --- | --- |
| R2D/D2R common parameters |  |  |
| Carrier frequency |  | 900MHz |
| SCS |  | 15 kHz |
| Block structure |  | 20 info. bits + 6bits CRC 96 info. Bits + 16 bits CRC 400 info. Bits + 16 bits CRC |
| Channel model |  | TDL-A, TDL-D |
| Delay spread |  | 30 ns |
| Device velocity |  | 3 km/h |
| Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| BS | Number of antenna elements | 2 |
|  | Number of TXRUs | 2 |
| Intermediate UE | Number of antenna elements | 1 |
|  | Number of TXRUs | 1 |
| BLER target |  | 10%, 1% |
| Sampling frequency |  | 1.92MHz |
| R2D specific parameters |  |  |
| Transmission bandwidth |  | 180/360/540/720 kHz |
| RF-ED bandwidth |  | 180*n kHz |
| BB LPF |  | 3-order Butterworth filter with cutoff frequency at 180/360/540/720 kHz |
| Waveform |  | OOK waveform generated by OFDM modulator and N=128, N'=128, X=12 |
| Modulation |  | OOK-4 with M=1, 2, 4, 6, 8, 12, 16, 24, 32 chips per OFDM symbol |
| Line code |  | 1/2 Manchester: 0-->{01}, 1-->{10} PIE: 0-->{10}, 1-->{1110} |
| FEC |  | No FEC |
| ADC bit width |  | - bit for device 1; 4-bit for device 2 |
| D2R specific parameters |  |  |
| Transmission bandwidth (w.r.t. D2R data rate) |  | 15 kHz as baseline |
| Waveform (CW) |  | unmodulated single tone, multi-tone |
| Modulation |  | OOK, BPSK |
| Line code |  | Miller, FM0, Manchester |
| FEC |  | CC, No FEC |
| ADC bit width |  | 11-bit |