3GPP TSG RAN WG1 #119			R1-2409566

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.3.1

Source:	InterDigital, Inc.

Title:	Discussion on SBFD TX_RX_measurement procedures

Document for:	Discussion and Decision

# Introduction

In Rel-19, a work item on evolution of NR duplex operation for Sub-band full duplex (SBFD) was approved and updated [1], where the objectives identified for the work item are as follows:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC-CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]: - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In RAN1#118bis meeting [2], the following agreements were made:

Conclusion Nominal RBG size for PDSCH/PUSCH in SBFD symbols is determined based on the size of DL/UL BWP as legacy. Nominal CSI reporting subband size in SBFD symbols is determined based on the size of DL BWP as legacy. Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 6 (Dynamic or semi-static DL vs. valid RO), reuse the existing collision handling rules for HD-FDD RedCap UEs. - UE does not expect collision between PRACH triggered by PDCCH order and dynamically scheduled DL reception. - For collision between PRACH triggered by PDCCH order and semi-statically configured DL reception, UE does not receive DL channel/signal. - For collision between PRACH triggered by higher layer and DL channel/signal, leave it to UE implementation whether to receive the DL or transmit PRACH. Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. Agreement The number of configured FH offsets of FH offset list for SBFD symbols is the same as the number of FH offsets of FH offset list for non-SBFD symbols. - The number of configured FH offsets of FH offset list is determined based on the UL BWP size as legacy. The number of bits used to indicate FH offset in DCI is determined as legacy. Agreement Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options. - Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS applicable usages - Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource. - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured. - FFS separate configurations of other parameters - FFS applicable usages Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability. - UE capability reports the maximum number of supported partial PRGs in SBFD symbols. Agreement - For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs. Agreement For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configuration 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2. Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, - The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy. - The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. - Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ) - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded. Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband Agreement If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, support Option 1 for determining UL/DL usable PRBs. - Option 1: For a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols. Agreement For a PDSCH/PUSCH with assigned PRBs overlapping with SBFD subband boundary and only the PRBs within DL/UL usable PRBs are valid for PDSCH/PUSCH reception/transmission, legacy DMRS sequence mapping is applied to assigned PRBs within DL/UL usable PRBs only (effectively, this is same as the case when the DMRS sequence mapped to the RBs outside the DL/UL usable PRBs are punctured). Agreement For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SPS PDSCH, CG PUSCH, PDSCH/PUSCH repetition, multi-PDSCH/PUSCH scheduled by a single DCI and TBoMS is SBFD symbols, - When an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively. - SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs. - FFS: The above applies to SP-CSI on PUSCH when applicable For configuration 1 and frequency resource allocation Type 1, if the valid symbol type for SPS PDSCH, PDSCH repetition and multi-PDSCH scheduled by a single DCI is SBFD symbols, then - Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. - Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused. - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only.

In this contribution, we discuss issues on SBFD Tx, Rx, measurement procedures, including separate TCI/power control across different symbol types, UL/DL resource allocations in SBFD/non-SBFD symbols, slot collision due to timing advance for one UE perspective, and SBFD operation in SSB symbols.

# Discussion on SBFD TX, RX, measurement procedures

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-18, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-19, enhancements to support full duplex (FD) at least for gNB were proposed and endorsed as the work item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on SBFD configurations including DL/UL subbands

## Separate TCI/power control across different symbol types (SBFD or non-SBFD)

In RAN1#118 meeting, for a single TRP scenario, the following three options were identified for down-selection on separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework:

Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters. - Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively - FFS: No RRC impact - FFS: New MAC CE or reuse existing MAC CE - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively - FFS: Impact on MAC CE - Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS - FFS: Same offset or different offsets for all channels - FFS: RRC or MAC CE introduced to convey power offset - FFS whether the maximum number of closed power control loops is increased to be more than 2. - FFS: Whether/How to handle the case where UL power control is not associated within TCI state

According to this agreement, RAN1 has to complete the necessary UE behaviors on how to at least separately maintain different UL power control (PC) across SBFD and non-SBFD symbols. In the current specifications (up to Rel-18), when the unified TCI framework is enabled (i.e., when dl-OrJointTCIStateToAddModList is configured or ul-TCI-ToAddModList is configured), each configured TCI-state (or TCI-UL-state) has its own associated PC parameter set which includes P0, alpha, PL-RS, etc. for open-loop PC and a closed-loop(CL)-index (l = 1 or 2) for the closed-loop PC, as shown in Figure 3 and Figure 4 with highlighted in yellow. This means UL transmission power determination is done according to an indicated TCI-state to be used for a UL transmission (e.g., PUSCH, PUCCH, SRS) associated with the indicated TCI-state, which is in line with the approach suggested by Option 1 (One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively). It means when Option 1 is adopted, the existing RRC parameters (as shown in Figures 3 and 4) does not have to be updated (e.g., no RRC impact), but just in a MAC-CE level, new MAC-CE can be introduced based on largely reusing mTRP-extended TCI-activation commands (introduced in Rel-18).

TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ..., [[ additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI ]], [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] }

Figure 3. TCI-state IE (when ‘dl-OrJointTCIStateToAddModList’ is configured)

TCI-UL-State-r17 ::= SEQUENCE { tci-UL-StateId-r17 TCI-UL-StateId-r17, servingCellId-r17 ServCellIndex OPTIONAL, -- Need R bwp-Id-r17 BWP-Id OPTIONAL, -- Cond CSI-RSorSRS-Indicated referenceSignal-r17 CHOICE { ssb-Index-r17 SSB-Index, csi-RS-Index-r17 NZP-CSI-RS-ResourceId, srs-r17 SRS-ResourceId }, additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond Mandatory ..., [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] }

Figure 4. TCI-state IE (when ‘ul-TCI-ToAddModList’ is configured)

Observation 1. In the current specifications (up to Rel-18), when the unified TCI framework is enabled, each configured TCI-state (or TCI-UL-state) has its own associated PC parameter set which includes P0, alpha, PL-RS, and a closed-loop(CL)-index (l = 1 or 2), which is in line with the approach suggested by Option 1.

Proposal 1. Support Option 1 (One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively) where the existing RRC parameters on TCI-state can be reused and in a MAC-CE level, new MAC-CE should be introduced based on largely reusing mTRP-extended TCI-activation commands (introduced in Rel-18).

### Case A: When only PC (not TCI) is separated across different symbol types

According to the agreement in RAN1#117, this Case is already agreed at least for FR1. We don’t see any reason not to support this for FR2 as well, as the unified TCI framework is applicable for both FR1 and FR2 with the applicability of QCL-typeD limited for FR2.

Observation 2. Unified TCI framework is applicable for both FR1 and FR2 with the applicability of QCL-typeD limited for FR2.

Proposal 2. Separating UL PC across different symbol types should also be supported in FR2, as the unified TCI framework is applicable for both FR1 and FR2. To support SBFD, the legacy spatial-relation-info framework is not needed to be enhanced together to simplify specifications and avoid the duplication.

Case A means that the network does not intend to change a TCI-state across different symbol type, but the UL PC should be able to be controlled differently per symbol type. As shown in Figures 3 and 4, since the current specifications do not support different PC parameters for one TCI-state, one simple extension for separating PC would be adding a second PC parameter set (e.g., “-SBFD” added in its original parameter) inside each TCI-state, as illustrated in Figure 5 and 6, which is in line with the approach including the FFS point suggested by Option 2 (a new Uplink-powerControlId is introduced for SBFD symbols).

TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ..., [[ additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI pathlossReferenceRS-SBFD-Id PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-SBFD Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI ]], [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] }

Figure 5. Separating PC in TCI-state IE (when ‘dl-OrJointTCIStateToAddModList’ is configured)

TCI-UL-State-r17 ::= SEQUENCE { tci-UL-StateId-r17 TCI-UL-StateId-r17, servingCellId-r17 ServCellIndex OPTIONAL, -- Need R bwp-Id-r17 BWP-Id OPTIONAL, -- Cond CSI-RSorSRS-Indicated referenceSignal-r17 CHOICE { ssb-Index-r17 SSB-Index, csi-RS-Index-r17 NZP-CSI-RS-ResourceId, srs-r17 SRS-ResourceId }, additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond Mandatory ul-powerControl-SBFD Uplink-powerControlId-r17 OPTIONAL, -- Need R pathlossReferenceRS-SBFD-Id PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond Mandatory ..., [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]] }

Figure 6. Separating PC in TCI-state IE (when ‘ul-TCI-ToAddModList’ is configured)

The Figures 5 and 6 show that all the PC parameters of P0, alpha, PL-RS, and CL-index can be separated by adding such second parameters ending ‘-SBFD’, but RAN1 needs to discuss whether all the parameters are to be separated or some parameters may still be good to be common across different symbol types. One identified issue in RAN1 was whether the CL-PC needs to be separated or not. In our view, the separation of CL-PC is important, as the CLI nature is not so predictable so that the gNB anyway should be able to adjust the UL power level based on such closed-loop adjustment by separated TPC commands.

Observation 3. Adding second PC parameters in addition to first PC parameters in a TCI-state implies all the PC parameters of P0, alpha, PL-RS, and CL-index can be separated across different symbol types, but it needs to be checked on whether all the parameters are to be separated or some parameters may still be good to be common across different symbol types.

Proposal 3. Support Option 2 (a new Uplink-powerControlId is introduced for SBFD symbols) as a second possible approach, and it needs to be discussed on whether all the PC parameters are to be separated. Support separating closed-loop PC across different symbol types, as the CLI nature is not so predictable in practice and gNB should be able to adjust the UL power level by explicit TPC commands.

More specifically, the current specifications support up to two closed-loop adjustment index (i.e., l = 1 or 2) introduced in Rel-15, e.g., for supporting different data traffic types each associated with different value of l, and this limited upper bound of configurable CL-indexes was carefully made considering UE complexity in maintaining up to two parallel closed-loop TPC accumulators if accumulated TPC is enabled. Therefore, simply increasing the value range of l, e.g., up to 4 independent TPC accumulators, merely due to different symbol types for SBFD operation is not desirable. It is because an additional CL-index for SBFD should work as a secondary and dependent TPC accumulator, meaning by default the additional CL-index can work commonly as the main CL-index already associated with a TCI-state and can have a separated TPC indication only when necessary by an explicit indication from gNB, which effectively maintains UE complexity similar to the legacy CL-PC adjustments.

Observation 4. Simply increasing the value range of CL-index l, e.g., up to 4 TPC accumulators all equally independent may not be necessary, in that an additional CL-index for SBFD should work as a secondary TPC accumulator that can have a separated TPC indication by gNB only when necessary.

Proposal 4. Support an additional CL-index dedicated for SBFD symbols for a TCI-state, where it can be configured by default to work commonly along with the CL-index already associated with the TCI-state to avoid excessive TPC commands unnecessarily, while it can be separately controlled only when necessary.

### Case B: When both PC and TCI are separated across different symbol types

It has been discussed in RAN1 that especially in FR2 the network may intend to control separate TCI-states, each to be applied in different symbol types (SBFD or non-SBFD). For example, a first TCI-state is to be used on non-SBFD symbols, and a second TCI-state is to be used on SBFD symbols. This can be automatically achieved when RAN1 adopts Option 1 (One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively), unless any restriction is additionally introduced.

As a second possible approach, the same functionality can also be achieved by Option 2 (a new Uplink-powerControlId is introduced for SBFD symbols) with additional enhancements as follows. In this release, RAN1 focuses on a single TRP scenario in that Rel-19 SBFD should properly operate first in the single TRP scenario where the single TRP employs a type of split Tx/Rx panels for SBFD operation. When a UE is configured with the unified TCI framework in a single TRP scenario (as in Rel-17), the UE may receive a TCI-activation MAC-CE activating up to 8 codepoints in the TCI field of a DL-DCI, where each codepoint is mapped to one TCI-state, also in line with the Option 2. It means that up to 8 activate TCI-states are ready for use for the UE, in terms of QCL tracking on configured RSs of the activated TCI-states, and the UE can apply a single indicated TCI-state (out of the activated TCI-states) at a time when the single indicated TCI-state has passed at least a beam application time (BAT) after being received. This makes sense in that one single indicated TCI-state can be used for most of DL and/or UL channels as a one-unified beam/TCI reference for such a single TRP scenario. However, in Rel-19 SBFD scenario where still a single TRP (but employing the split Tx/Rx panels) is considered, an enhanced requirement for a Rel-19 UE to apply/use two separated indicated TCI-states, at a time, out of the up to 8 activated TCI-states can be considered. Note this is different from Rel-18 unified TCI extension feature for mTRP, in that a TCI field codepoint can be mapped to one or two TCI-states (assuming joint TCI mode) for two TRPs in Rel-18, which involves many MIMO-specific details (e.g., further separated multi-DCI-based or single-DCI-based schemes with scheme-dependent behaviors) not relevant for the Rel-19 SBFD scenario with a single TRP employing the split Tx/Rx panels in the same TRP site. Furthermore, directly reusing existing multi-TRP schemes (in Rel-18) by re-interpretating the TRP domain into the symbol type domain should be avoided, as the NR-Duplex operation can co-exist with multi-TRP operations in future releases as a practical deployment scenario.

Observation 5. For separate TCI control across different symbol types, directly reusing existing multi-TRP schemes (up to Rel-18) by re-interpretating the TRP domain into the symbol type domain is not a preferred approach, as the NR-Duplex operation can co-exist with multi-TRP operations in future releases as a practical deployment scenario.

Proposal 5. Support either Option 1 or Option 2 with an addition enhancement that Rel-19 SBFD-aware UE can apply up to two separately indicated TCI-states, of the same set of activated TCI-states by the TCI-activation MAC-CE (Rel-17), across time-domain (not across TRP-domain, to avoid any impacts to Rel-18 multi-TRP features).

It can be further considered in RAN1 to make the Case A and Case B be configurable by RRC based on the network implementation choice, meaning Case B may not need to be enabled in a deployment scenario, e.g., in FR1, also depending on UE capability. However, in our view, both Case A and Case B are needed for properly supporting various deployment cases and it is desired to let the network configure preferred TCI/PC separated behaviors across symbol types, along with employed CLI management techniques.

In addition, Option 3 (A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS) allows a low-complexity version of power control parameter separation which is based on purely semi-static power offset across different symbol types. RAN1 has to make a down-selection of at least one among the identified 3 options based on previous agreements supporting at least separate power control across different symbol types.

### On separate SRS configurations

In RAN1#118bis meeting, it was agreed to support separate SRS configurations for SBFD and non-SBFD symbols, where one from the following Option 1 or 2 needs to be down-selected:

Agreement Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options. - Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS applicable usages - Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource. - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured. - FFS separate configurations of other parameters - FFS applicable usages

In our view, Option 2 may create many subsequent issues at least for clarification in relation with the legacy behaviors in UL transmission cases, e.g., SRS resource indicator (SRI) field interpretation mapped to the UL-DCI. Since in the legacy operation, e.g., for codebook-based UL, the 1-bit SRI field values in a UL DCI are automatically mapped from all the SRS resources configured in the SRS resource set with the ‘usage’ parameter set to ‘codebook’. According to the Option 2, an SRI may point to the same SRS resource that has been used for SRS transmissions on SBFD symbols as well as non-SBFD symbols. A UL grant scheduling a PUSCH may indicate an SRI based on the Option 2, then there may exist an ambiguity in that the SRI points to which time moment of one of transmitted SRSs because the same SRS resource being used for both symbol types is indicated by the SRI.

Option 1, however, intends to separate the whole considered SRS resource set for only one symbol type, i.e., either an SBFD symbol type or a non-SBFD symbol type, which can simplify the specification and minimize any unexpected issue in relation with the legacy behaviors in UL transmission cases.

Regarding the FFS points on “applicable usages”, RAN1 needs to discuss for which ‘usage’ (among CB, NCB, BM, antenna switching(AS)) is applicable for the separated SRS resource set for SBFD symbols. The baseline should be at least codebook-based(CB) and non-codebook-based(NCB) applicable for the separated SRS resource set for SBFD symbols, as these are the supported UL transmission schemes. For example, assuming “Option 1: One TCI codepoint is mapped to separate TCI states for SBFD and non-SBFD symbols” is adopted regarding the separate TCI/PC issue, when a TCI codepoint associated with a first TCI-state (for non-SBFD symbols) and a second TCI-state (for SBFD symbols) is indicated, the UE transmits an SRS (if its usage parameter set to CB or NCB) by using the first TCI-state when a transmission occasion is on non-SBFD symbols, or using the second TCI-state when a transmission occasion is on SBFD symbols. However, the UE transmits an SRS (if its usage parameter set to BM or AS) by using the first TCI-state regardless of a symbol type of a transmission occasion, where this default behavior should be applicable to other UL transmission cases, e.g., PUCCH, PRACH, etc. as well, since the indicated TCI codepoint is in general applied to other physical channels or signals altogether, under the unified TCI framework.

Observation 6. Option 2 (Separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource) may create many subsequent issues at least for clarification in relation with the legacy behaviors in UL transmission cases, e.g., SRS resource indicator (SRI) field interpretation mapped to the UL-DCI, etc.

Proposal 6. Support Option 1 (Separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage), and at least CB and NCB for the ‘usage’ parameter should be applicable for the separated SRS resource set for SBFD symbols.

## UL and DL resource allocation in SBFD and Non-SBFD symbols

[Transmission/reception across SBFD symbols and non-SBFD symbols in different slots]

During the study item on NR-Duplex in Rel-18, the UL/DL across SBFD and non-SBFD symbols in different slots were discussed, where the UL and DL include PDSCH, PUSCH, PUCCH repetitions, SPS PDSCH, configured grant PUSCH, TB processing over multi-slot PUSCH (TBoMS), multi-PUSCH or PDSCH scheduled with a single DCI, periodic, semi-persistent SRS, CSI-RS, PUCCH, PDCCH and so forth.

The common problem among the mentioned UL and DL signals and channels is that they could span over more than one slot, where the different slots may be configured to have SBFD or non-SBFD symbols. That is, the scheduled resources that are configured based on non-SBFD symbols for UL transmission or DL reception could fall outside of UL and DL subbands, respectively, in SBFD configuration.

In the RAN1#118bis meeting, it was agreed that SBFD-aware UE will be configured via RRC with either Configuration 1 (i.e., transmission/reception across multiple slots only on SBFD or only on non-SBFD symbols) or Configuration 2 (i.e., transmission/reception across multiple slots can be across SBFD and non-SBFD symbols):

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configuration 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2.

Three options regarding the granularity of the configuration were left for selection. Option 1 allows the configuration per UE and per cell, allowing the network to support different UE capabilities under the same cell. Some UEs may not be capable of supporting Configuration 2 and other UEs can. With Option 1, the two capabilities can coexist in the same cell while Option 3 forces all the UEs under the cell to be capable of Configuration 2. Option 2 adds more UE complexity as the UE can have different configuration for different channels/signals. Such complexity has no clear benefit. It is thus preferred to support Option 1.

Proposal 7. SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis.

Based on the definition of Configuration 2 in the RAN#117 meeting, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols, but it is not restricted to always span SBFD symbols and non-SBFD symbols. For example, the UE can be configured with Configuration 2 and receives a DCI scheduling a PDSCH repetitions that is repeated across only SBFD or only non-SBFD. Therefore, even with configuration 2, all the assigned PRBs for PDSCH can be available for transmission. The TBS determination should depend on whether there are SBFD symbols in the repeated slots.

Proposal 8. In case of Configuration 2, the number of PRBs for TBS determination for PDSCH repetitions is based on assigned PRBs within DL usable PRBs only if at least one PDSCH repetition overlaps with SBFD symbols.

In the RAN1#118 meeting, with Configuration2, it was agreed that single resource configuration/indication for non-SBFD symbols and RB offset to determine the frequency resource for SBFD symbols. Furthermore, in the RAN1#118bis meeting, the following options were agreed for consideration:

- Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ) - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded.

With Option 1, the UE is configured with one or multiple RB offset which will be used in the determination of the start of the allocated RB in SBFD symbols. Four equations were captured in the agreements regarding the calculation of the RB start. Equation 1-A provides simple and clean solution by allowing negative values of the RB offset. Option 2 rather relies on an equation or a certain pre-defined formula to determine the new RB start in the SBFD symbols. In our view, relying on such a pre-defined formula limits the scheduler flexibility to indicate different possibilities on allocating eventually different resources across UEs. In other words, such flexibility is needed when the gNB is also scheduling another UE within the UL subband in the SBFD symbols. By having different RB offsets UE-specifically, the gNB can multiplex other UEs in the same SBFD symbol with less constraint.

Proposal 9. When PUSCH frequency hopping is not enabled, for transmission across SBFD and non-SBFD symbols, one or multiple RBoffsetSBFD are configured (as Option 1) and the starting PRB for PUSCH in SBFD symbol is determined according to the following equation:

    - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD and Negative values of RBoffsetSBFD are not precluded.

[Uplink transmission across SBFD and non-SBFD symbols in the same slot]

One of the main motivations to support NR duplex is to solve the issue of limited available uplink resources. This limited availability of uplink resources can impact the coverage of uplink transmissions. By having more available resources for uplink, the gNB can schedule longer transmission duration which will help enhancing the decoding performance for UEs in coverage limited scenario. When a slot is configured with both SBFD and non-SBFD symbols, it can be beneficial to use both SBFD and non-SBFD symbols for uplink transmission. The gNB can schedule a UE in limited coverage with both SBFD and non-SBFD symbols to achieve longer transmission duration and thus better performance.

To obtain such benefits, the usage of both SBFD and non-SBFD can at least be conditioned on, for example, the number of available symbols for SBFD and/or non-SBFD in the slot. When the slot consists of small number of SBFD or small number of non-SBFD symbols, using both SBFD and non-SBFD symbols for the transmission may not bring significant gain because the additional transmission duration from SBFD or non-SBFD symbols will not be significant. For example, adding one or two symbols to the transmission duration will not significantly increase the coverage of the transmission. When the number of SBFD and non-SBFD symbols is high in the slot, it can be beneficial to transmit on both SBFD and non-SBFD symbols since longer duration can be achieved from using SBFD and non-SBFD symbols. To minimize the impact of supporting transmission in both SBFD and non-SBFD symbols, the gNB can configure the UE to use the same frequency domain resource allocation for the entire transmission.

Observation 7. Supporting uplink transmission occupying both SBFD and non-SBFD symbols in a slot can increase the coverage of uplink transmission.

Proposal 10. Support uplink transmission across SBFD and non-SBFD symbols in the same slot.

## Slot collision due to timing advance for one UE perspective

In conventional TDD systems, the UL and DL slots are considered separately in time domain. Therefore, subsequent slots are split in DL-only, UL-only, and flexible slots. The symbols in flexible slots can be scheduled to be used as DL or UL based on received configurations. The flexible symbols can also be used as guard period for timing alignment requirements (e.g., DL/UL switching, UL timing advance (TA)).

In SBFD schemes with one or more subbands allocated for UL transmission in DL slots, a non-zero timing advance or switching time in SBFD could result in slot collision cases. As an example, shown in Figure 7, where the UL signals in UL SBs and DL signals in DL SBs/slots are collided by each other due to timing advance. Also, another example on slot-boundary collision case is shown in Figure 8 that is affecting the SBFD slots due to required switching time for UE between DL and UL that is also a factor for the timing alignment requirements.

For example, especially for a semi-statically scheduled resource (e.g., configured grant PUSCH), every time when UE receives a TA command which affects all related UL transmission including the CG-PUSCH, the UE has to check whether the adjusted/accumulated TA and the switching time requirement result in overlap with DL symbol in a prior DL slot, if scheduled, for SBFD operation. And if any slot-boundary collision happens, the UE does not know which selected one of a DL reception and a UL transmission the UE should perform as scheduled and how to perform the other scheduled one that is currently overlapped in symbols, since the UE performs either DL only or UL only on a given set of symbols as a half-duplex UE. This does not happen in legacy TDD system as there is the flexible slot for this reason at least, while in SBFD operation, it is always possible to face a back-to-back scheduling between DL and UL for one UE perspective.

Figure 7. Slot collision issue due to timing advance for one UE perspective

Figure 8. Slot collision issue due to switching time between DL and UL

While the gNB might address the required timing advance/switching time based on resource allocations and scheduling (e.g., as part of gNB implementation), this requires significantly increased scheduling complexity at the gNB side and also increases the signaling overhead, especially for taking such semi-statically scheduled resources for both DL and UL into account. In another aspect, in case a scheduled (dynamic) grant for UL or DL with high priority is received and if the timing alignment requirement including required Rx/Tx or Tx/Rx switching time is not met, e.g., resulting in some DL/UL overlapped symbols being faced at the UE, this could again result in a severe performance degradation or dropping corresponding slots.

Observation 8. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in slot collision issues for one UE perspective and possible dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

In case a UL transmission in SBFD UL subband overrides the symbols from a previous slot (e.g., a legacy ‘D’ slot) due to the non-zero timing advance, the DL scheduled in those symbols results in the slot collision case and may experience inter-subband CLI as well as inter-slot interference. The slot collision issue due to non-zero TA becomes critical, as SBFD framework in Rel-19 has no ‘Special’ slot in-between the legacy D slot and the SBFD slot back-to-back. The slot collision case with the inter-slot interference is more severe for the legacy UEs scheduled in the previous slot for DL reception, as such a timing misalignment results in intra-subband (directly colliding) and inter-slot interference on the symbols close to the end of the DL slot. The legacy UEs may be scheduled to receive SSB, CORESET#0, or DMRS in PDSCH in the symbols that are overridden by the misaligned UL transmission in the UL subband in the right next slot. As such, the performance of legacy UEs may be affected due to the non-zero TA.

Therefore, there should be a means for UE to detect if the allocated UL/DL timing alignment is accurate and enough. In case UE has detected such timing discrepancies, the UE could report the issues along with requesting for adjustment or at least should be able to determine which overlapped portion of either DL symbol(s) or UL symbol(s) can be dropped or punctured. For example, the UE could also determine to include the time required for the timing alignment inside respective UL/DL SB in SBFD slot, where the scheduled UL transmission on the SBFD slot can have a rate-matched or punctured symbol(s) in the front symbol positions of the UL transmission, or alternatively DL symbol(s) can be punctured, which should depend on gNB’s flexible configurations for such UE behaviours.

Observation 9. The slot collision issue due to non-zero TA from UL transmission in UL subband in SBFD configuration could increase the inter-slot and inter-subband interference including other UEs, specifically on the DL symbols scheduled in the previous slot.

Observation 10. The slot collision issue and the time misalignment due to non-zero TA for an UL transmission in UL subband in an SBFD configuration could affect the legacy UE that is configured to receive critical DL signals such as SSB, CORESET#0, or DMRS close to the end of a preceding DL slot.

Proposal 11. Support methods to handle the slot collision and time misalignment issues due to non-zero TA for a scheduled UL Tx in an SBFD slot, right after the legacy ‘D’ slot back-to-back, e.g., including which channel direction the UE can prioritize and UE reporting on a minimum PUSCH starting symbol to aid gNB’s scheduling.

## SBFD operation in SSB symbols

During the study phase, the support for the SBFD operation in SSB symbols was discussed. Based on the discussions, it was clear that not supporting the SBFD operation in SSB symbols may degrade the performance and objectives of the SBFD operation.

Contiguous UL subband resources in time domain

Considering the UL repetition, the contiguous UL subband resources should be provided in time domain. However, in case the SBFD operation is not allowed on SSB symbols, this requires switching to DL-only symbols each time there is an SSB transmission. As such, switching between SBFD symbols and DL-only symbols will affect the resources for UL repetition in SBFD symbols and will degrade the coverage enhancement purposes.

Latency in UL transmission

In addition to coverage enhancement, one of the objectives in SBFD operation is to reduce latency in UL transmissions. However, in case the SSB symbols are not transmitted in SBFD resources, this will change the TDD model back to DDDSU each time there is an SSB burst transmission. On the other hand, considering SSB transmission in SBFD symbols may improve the latency even in initial access or cell reselection procedures due to availability of resources for sending PRACH preamble and random-access procedures.

Observation 11. In case the SBFD operation is not supported in SSB symbols, the SBFD objectives such as UL coverage enhancement and latency reduction are affected and degraded.

In RAN1 #117, following agreement was made:

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

Based on this agreement, in case an SSB transmission overlaps with an SBFD symbol, the SBFD symbol is converted to a full DL slot. Alternatively, the UE expects to only receive DL within DL usable PRBs and no UL is allowed for the SBFD-aware UE in the corresponding SBFD symbol. One of the main reasons for not transmitting UL in the overlapping SBFD symbols is handling the collision between the UL transmission and SSB reception, as the potential CLI caused by UEs transmitting UL could degrade SSB reception for the other UEs.

The CLI due to UL transmission in UL subbands of SBFD symbols that overlap in time with SSB transmission could also be caused by UL transmission by SBFD-aware UEs in an adjacent cell with SBFD configuration in the same symbol (although the entire network has a common “DXXXU” pattern for all the cells for example). That is, while the SBFD-aware UEs in the serving cell are refrained from UL transmission in the SBFD symbol overlapping with SSB transmission, the UL transmission from SBFD-aware UEs in an adjacent cell could affect SSB reception performance. As an example, see Figure 5, where the UEs in both Cell 1 and Cell 2 are configured commonly with DXXXU SBFD configuration. In Figure 9, Cell1 is configured with SSB transmission in SBFD Slot2, Cell2 is configured with SSB transmission in SBFD Slot 3, and so the UEs in Cell2 are refrained from UL transmission in Slot 3. On the other hand, the UEs in Cell1 may perform UL transmissions in Slot 3, as there are no SSB transmission from their serving Cell1 in Slot3, which may cause CLI and degradation in SSB reception for UEs in Cell2.

Figure 9. Issue on neighbor-cell’s SSB slot to be considered for the ‘D’ slot fallback on UEs in serving-cell

Observation 12. The UL transmission from SBFD-aware UEs in an adjacent cell in an SBFD symbol that coincides with SSB transmission in a serving cell degrades the SSB reception performance for the UEs in the serving cell.

To mitigate the CLI and potential degradation in SSB reception in a first cell, caused by UL transmission from the SBFD-aware UEs in an adjacent second cell, the respective UEs can explicitly receive information on additional neighbor-cell(s) with their SSB transmission slot information to additionally apply the ‘D’ slot fallback, or could verify some conditions before UL transmission in the SBFD symbols that overlap in time with SSBs from another cell. For example, in Figure 5, UE1 that is closer to the center of Cell1 and far from Cell2 can still transmit UL in the SBFD symbols that overlap in time with Slot3, where SSB is transmitted in Cell2. On the other hand, in Figure 5, UE2 that is far from the center of Cell1 and close to Cell2 cannot transmit UL in Slot3, where SSB is transmitted in Cell2.

Proposal 12. Support mechanisms for collision handling between coincidence of UL transmission in a serving-cell cell and SSB transmission in a neighbour cell, when information on the SSBs of the neighbour cell is configured.

# Conclusion

In this contribution, we discussed issues on SBFD Tx, Rx, measurement procedures, including separate TCI/power control across different symbol types, UL/DL resource allocations in SBFD/non-SBFD symbols, slot collision due to timing advance for one UE perspective, and SBFD operation in SSB symbols. From the discussions, we made following observations and proposals:

[2.1	Separate TCI/power control across different symbol types (SBFD or non-SBFD)]

Observation 1. In the current specifications (up to Rel-18), when the unified TCI framework is enabled, each configured TCI-state (or TCI-UL-state) has its own associated PC parameter set which includes P0, alpha, PL-RS, and a closed-loop(CL)-index (l = 1 or 2), which is in line with the approach suggested by Option 1.

Proposal 1. Support Option 1 (One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively) where the existing RRC parameters on TCI-state can be reused and in a MAC-CE level, new MAC-CE should be introduced based on largely reusing mTRP-extended TCI-activation commands (introduced in Rel-18).

[2.1.1	Case A: When only PC (not TCI) is separated across different symbol types]

Observation 2. Unified TCI framework is applicable for both FR1 and FR2 with the applicability of QCL-typeD limited for FR2.

Proposal 2. Separating UL PC across different symbol types should also be supported in FR2, as the unified TCI framework is applicable for both FR1 and FR2. To support SBFD, the legacy spatial-relation-info framework is not needed to be enhanced together to simplify specifications and avoid the duplication.

Observation 3. Adding second PC parameters in addition to first PC parameters in a TCI-state implies all the PC parameters of P0, alpha, PL-RS, and CL-index can be separated across different symbol types, but it needs to be checked on whether all the parameters are to be separated or some parameters may still be good to be common across different symbol types.

Proposal 3. Support Option 2 (a new Uplink-powerControlId is introduced for SBFD symbols) as a second possible approach, and it needs to be discussed on whether all the PC parameters are to be separated. Support separating closed-loop PC across different symbol types, as the CLI nature is not so predictable in practice and gNB should be able to adjust the UL power level by explicit TPC commands.

Observation 4. Simply increasing the value range of CL-index l, e.g., up to 4 TPC accumulators all equally independent may not be necessary, in that an additional CL-index for SBFD should work as a secondary TPC accumulator that can have a separated TPC indication by gNB only when necessary.

Proposal 4. Support an additional CL-index dedicated for SBFD symbols for a TCI-state, where it can be configured by default to work commonly along with the CL-index already associated with the TCI-state to avoid excessive TPC commands unnecessarily, while it can be separately controlled only when necessary.

[2.1.2	Case B: When both PC and TCI are separated across different symbol types]

Observation 5. For separate TCI control across different symbol types, directly reusing existing multi-TRP schemes (up to Rel-18) by re-interpretating the TRP domain into the symbol type domain is not a preferred approach, as the NR-Duplex operation can co-exist with multi-TRP operations in future releases as a practical deployment scenario.

Proposal 5. Support either Option 1 or Option 2 with an addition enhancement that Rel-19 SBFD-aware UE can apply up to two separately indicated TCI-states, of the same set of activated TCI-states by the TCI-activation MAC-CE (Rel-17), across time-domain (not across TRP-domain, to avoid any impacts to Rel-18 multi-TRP features).

[2.1.3	On separate SRS configurations]

Observation 6. Option 2 (Separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource) may create many subsequent issues at least for clarification in relation with the legacy behaviors in UL transmission cases, e.g., SRS resource indicator (SRI) field interpretation mapped to the UL-DCI, etc.

Proposal 6. Support Option 1 (Separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage), and at least CB and NCB for the ‘usage’ parameter should be applicable for the separated SRS resource set for SBFD symbols.

[2.2	UL and DL resource allocation in SBFD and Non-SBFD symbols]

Proposal 7. SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis.

Proposal 8. In case of Configuration 2, the number of PRBs for TBS determination for PDSCH repetitions is based on assigned PRBs within DL usable PRBs only if at least one PDSCH repetition overlaps with SBFD symbols.

Proposal 9. When PUSCH frequency hopping is not enabled, for transmission across SBFD and non-SBFD symbols, one or multiple RBoffsetSBFD are configured (as Option 1) and the starting PRB for PUSCH in SBFD symbol is determined according to the following equation:

    - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD and Negative values of RBoffsetSBFD are not precluded.

Observation 7. Supporting uplink transmission occupying both SBFD and non-SBFD symbols in a slot can increase the coverage of uplink transmission.

Proposal 10. Support uplink transmission across SBFD and non-SBFD symbols in the same slot.

[2.3	Slot collision due to timing advance for one UE perspective]

Observation 8. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in slot collision issues for one UE perspective and possible dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

Observation 9. The slot collision issue due to non-zero TA from UL transmission in UL subband in SBFD configuration could increase the inter-slot and inter-subband interference including other UEs, specifically on the DL symbols scheduled in the previous slot.

Observation 10. The slot collision issue and the time misalignment due to non-zero TA for an UL transmission in UL subband in an SBFD configuration could affect the legacy UE that is configured to receive critical DL signals such as SSB, CORESET#0, or DMRS close to the end of a preceding DL slot.

Proposal 11. Support methods to handle the slot collision and time misalignment issues due to non-zero TA for a scheduled UL Tx in an SBFD slot, right after the legacy ‘D’ slot back-to-back, e.g., including which channel direction the UE can prioritize and UE reporting on a minimum PUSCH starting symbol to aid gNB’s scheduling.

[2.4	SBFD operation in SSB symbols]

Observation 11. In case the SBFD operation is not supported in SSB symbols, the SBFD objectives such as UL coverage enhancement and latency reduction are affected and degraded.

Observation 12. The UL transmission from SBFD-aware UEs in an adjacent cell in an SBFD symbol that coincides with SSB transmission in a serving cell degrades the SSB reception performance for the UEs in the serving cell.

Proposal 12. Support mechanisms for collision handling between coincidence of UL transmission in a serving-cell cell and SSB transmission in a neighbour cell, when information on the SSBs of the neighbour cell is configured.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei.

“Report of 3GPP TSG RAN WG1 #118bis”, 3GPP RAN1 Meeting #118, October, 2024.