3GPP TSG RAN WG1 #119			R1-2409581

Orlando, US, November 18th – 22nd, 2024

Agenda item:	9.1.1

Source:	Samsung

Title:	Discussion for supporting AI/ML based beam management

Document for:	Discussion and Decision

- Introduction

In RAN#102, Rel-19 work item on “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. The objective of the work item is as follows.

Provide specification support for the following aspects: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we will provide our view on the specification support for AI/ML beam management, including the aspects for NW-side AI/ML model and UE-side AI/ML model, respectively.

- NW-side AI/ML model

In this section, specification impact with respect to NW-side AI/ML model for beam management is discussed. In particular, different LCM operations (e.g., data collection, model inference, model monitoring, etc) associated with the NW-side AI/ML and the corresponding signalling/mechanism to facilitate the LCM operations are further discussed as follows.

2.0 Typical procedure

Beam management specific LCM procedure for NW-side AI/ML model is illustrated in Figure 1.

Figure 1. LCM procedure for NW-side AI/ML model.

For data collection, NW configures RS resources (e.g., SSB resources or CSI-RS resources) for L1-RSRP measurement. For example, RS resources for Set A and/or Set B can be configured for transmission. UE performs measurements based on the configured RS resources and report the corresponding collected data based on NW control. From NW perspective, the collected data can be used for model training, model inference or performance monitoring. For model training, the collected data can be further delivered or process for offline training or fine tuning. For model inference, NW can use the collected data as model input and generate the corresponding output, where the output (e.g., predicted beam) can be further used for beam indication. The collected data can also be used to monitor the performance of the model, and NW can decide whether to activate/deactivate/fallback the AI/ML operation based on the performance monitoring.

2.1 Data collection for Model training

For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold when the data generation and termination entities differ. For instance, for: - Model Training: - For gNB-side models, training data can be generated by the gNB or UE, while the termination point for training data may include the gNB, or OAM. - Note: RAN2 identified the case in which OTT server and Core Network may be used for gNB-side model training. However, no study was conducted since this is beyond the scope of this Working Group.

The procedure of data collection (e.g., data collection for model training) for NW-side AI/ML model was studied and the corresponding outcome are captured in TR 38.843 as above. From RAN1 perspective, the training data generated by UE needs to be looked into. The contents of collected data and the corresponding container/signalling to convey the collected data will be further discussed as follows.

2.1.1 Training Data collection content

Data collection content for training is highly dependent on the type of AI/ML model. In general, there are two types of AI/ML models for beam management: (i) classification-based model; (ii) regression-based model.

- Classification-based model is capable to provide the ranking information of Set A beams (as model output). In terms of data collection for training, classification-based model requires data of L1-RSRPs of (all/subset) Set B beams as model input. Also, the labels corresponding L1-RSRPs of (all/subset) Set B beams are necessary for training. Here, the labels for classification-based model should be beam ID of Set A beams (typically, Top-1 beam ID of Set A beams).

- Regression-based model is capable to provide the predicted L1-RSRP information of Set A beams (as model output). In terms of data collection for training or fine-tuning, similar to classification-based model, regression-based model requires data of L1-RSRPs of (all/subset) Set B beams as model input. Also, the labels corresponding L1-RSRPs of (all/subset) Set B beams are necessary for training. Here, the labels for regression-based model should be L1-RSRPs of Set A beams (e.g., L1-RSRP for all/subset of Set A beams).

The discussion above is equally applicable to BM-Case1 and BM-Case2. Moreover, to facilitate data collection for BM-Case2, time domain information (e.g., timestamps for data and label) is additionally required. Also, a further issue is how to convey the timestamps, e.g., whether it is implicit or explicit.

An example for training data collection content for classification-based model and regression-based model are provided in Table 1.

|  | Classification-based model | Regression-based model |
| --- | --- | --- |
| Data collection content | L1-RSRPs for all Set B beams Top-1 Beam ID for Set A Related timestamps (BM-Case2) | L1-RSRPs for all Set B beams L1-RSRPs for all/subset Set A beams Related timestamps (BM-Case2) |

Table 1. Exemplary training data collection content

Apart from timestamp, the assistance information corresponding data, label, timestamps to facilitate model training, e.g., model generalization for different scenarios need to be considered. Hence, for the content of data collection, we have the following proposal.

Proposal 1. For NW-side AI/ML model data collection for training, support at least the following as data collection content:

- L1-RSRP(s) for all beam(s) of Set B

- L1-RSRP(s) for all beam(s) of Set A

- Top-K Beam ID(s) for Set A

- Related timestamp

- The information to facilitate model training (FFS details)

2.1.2 Signalling/Container for training data collection

Apart from the content of data collection, another aspect is the signalling/container to convey the content of collected data for training. Basically, three types of signaling (L1 signaling, MAC-CE, RRC) can be used. For model training purpose, the latency requirement to convey the collected data is less stringent. Hence, high layer signaling (e.g., MAC-CE, RRC) can be naturally used for this purpose as baseline. Hence, potential enhancement to use high layer signal to convey the data collection content should be supported. For L1 signaling, the existing L1 beam reporting framework can be largely reused for the collection of model training data. Whether it is necessary to enhance L1 signaling for model training purpose should be further discussed.

Proposal 2. For NW-side AI/ML model data collection for training, at least support the enhancement to use high layer signaling to convey data collection content.

2.2 Data collection for model inference

In general, there are two aspects for beam report enhancement: spatial domain (e.g., to support one reporting instance with measurements of more than 4 beams) and time domain (e.g., to support one reporting instance with beam measurements for multiple time instances). Details are discussed as follows. Note that the methods for data collection for model inference can be reused for performance monitoring.

2.2.1 Spatial domain related enhancement (> 4 beams)

Agreement (RAN1#116) For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement (RAN1#116bis) For network-sided AI/ML model for BM-Case1 and BM-Case2, - support using existing CSI framework for configuration of Set A as the starting point - support using existing CSI framework for configuration of Set B as the starting point - Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications Agreement (RAN1#117) At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range Agreement (RAN1#117) For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information

Based on the agreement in RAN1#117, a beam reporting with more than 4 beams is supported with more details. Due to the increased number of reported beams in comparison with that of legacy beam reporting, payload size reduction techniques for L1 beam report should be considered. One FFS point is to report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP. This method is intended for more efficient use of UL resource with variable UCI payload since the number of reported beam (i.e., within X dB gap to the largest measured beam) may be different at different reporting instance. To achieve this, two-part CSI is needed where the first part of the CSI is of fixed size and the second part of the CSI is variable depending on the number of beams indicated by the first part.

Moreover, the quantization of L1-RSRP measurement should be discussed. Based on the study outcome of SI phase, AI/ML model is not very sensitive to quantization method (e.g., sparser quantization of L1-RSRP measurement does not impact the performance AI/ML model inference). This means that L1-RSRP reporting with sparser quantization step can be considered to reduce beam reporting overhead.

Proposal 3. For NW-side AI/ML model inference, for CSI-ReportConfig with the measurements for more than 4 beams in one reporting instance

- Support differential L1-RSRP reporting with larger quantization step(s) and/or smaller range(s)

- Consider to reduce the overhead of beam report using two-part CSI

Another follow-up issue is which report quantities is applicable to beam report with more than 4 beams. Basically, there are for report quantities associated to L1-RSRP based beam reporting, i.e., 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP-Index', 'ssb-Index-RSRP-Index'. In our view, unless there is a strong reason, these four L1-RSRP based report quantities should be supported.

Proposal 4. For NW-side AI/ML model inference, for CSI-ReportConfig with the measurements for more than 4 beams in one reporting instance, consider the applicability of the following report quantity:

- 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP-Index', 'ssb-Index-RSRP-Index'.

2.2.2 Time domain related enhancement (multiple past time instances)

For BM-Case2 with NW AI/ML model inference, NW requires L1-RSRPs for multiple past time instances as model input. For L1 signaling, there are two existing methods to obtain L1-RSRP measurement for a particular time instance: 1) CSI-ReportConfig configured with timeRestrictionForChannelMeasurements; 2) aperiodic CSI reporting using aperiodic CSI-RS. However, those methods can only provide measurement for a single time instance in the corresponding CSI reporting and the time instance is couple with the timing of the CSI reporting (e.g., the latest measurement no later than the CSI reference for the CSI reporting). This is very restrictive if more than one measurement results are desired for model inference from NW perspective. Hence, it is preferable to support L1 beam reporting (i.e., CSI-ReportConfig) with measurements for each of multiple past time instances in one reporting instance. NW can directly use the measurement results as model input for future beam prediction. One follow-up issue is how to identify the multiple past time instances so that NW and UE can have a common understanding on the exact time domain position of those time instances. Also, another issue for further study is which type of CSI reporting (e.g., P/SP/AP reporting) should be supported.

Proposal 5. For NW-side AI/ML model inference, support CSI-ReportConfig with measurements for each of multiple past time instances in one reporting instance.

- FFS: How to identify the multiple past time instances

- FFS: The support of P/SP/AP reporting

2.3 Beam indication based on model inference output

Agreement (RAN1#116bis) For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework - FFS on whether/how potential enhancement is needed

For NW-side AI/ML model, the output of model inference can be used for beam indication to UE. For BM-Case2, it is beneficial to support predictive beam indication. For the existing beam indication framework, only one (future) time instance can be indicated at a time. This may not work well in high-speed scenario since the beam switch is very frequent in this scenario and the single indicated beam may be out-dated very quickly. Hence, to ensure the robustness of beam indication and reduce signalling overhead, single beam indication for multiple future time instances based on the model inference output is preferred. Also, for the beam indication of multiple future time instances, unified TCI framework should be used as baseline since it is already time instance dependent beam indication.

Proposal 6. Support single beam indication for multiple future time instances using unified TCI framework.

- UE-side AI/ML model

In this section, specification impact with respect to UE-side AI/ML model for beam management is discussed. In particular, different LCM operations (e.g., data collection, model inference, model monitoring, etc) associated with the UE-side AI/ML and the corresponding signalling/mechanism(s) to facilitate the LCM operations are further discussed as follows.

3.0 Typical procedure

Beam management specific LCM procedure for UE-side AI/ML model is illustrated in Figure 2.

Figure 2. LCM procedure for UE-side AI/ML model.

For data collection, NW can configure or trigger the CSI-RS/SSB transmission for L1-RSRP measurement. The RS configuration may depend on the request from UE. For example, the RS resources for Set A and/or Set B can be configured by NW based on the request from UE. UE performs measurements based on the configured/triggered RS resources and performs data collection correspondingly. From UE perspective, the collected data can be used for model training, model inference or performance monitoring. For model training, the collected data can be further delivered or process for offline training or fine tuning. For model inference, UE can use the collected data as model input and generate the corresponding output, where the output (e.g., predicted beam and/or predicted L1-RSRP) should be reported to NW for predictive beam indication. Moreover, the collected data can also be used to monitor the performance of the model. For type 1 performance monitoring, UE can provide measurement result or associated performance metric to NW and NW can further indicate the activation/deactivation/fallback of AI/ML operation via DL signalling. For type 2 performance monitoring, UE can decide whether to activate/deactivate/fallback the AI/ML operation based on the collected data.

Data collection for UE-side Model training is not discussed in detail in the paper since it is still under study in the parallel section. Instead, this paper will focus on the procedure of model inference and the procedure of performance monitoring as follows.

3.1 Model inference

3.1.1 Procedure for model inference

Conclusion (RAN1#116) For UE sided model at least for inference, for measurement, the configuration of Set B, - take the current CSI framework as the starting point Agreement (RAN1#116bis) For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded. Agreement (RAN1#118) For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A

For the purpose of UE-side model inference, NW can configure Set A for prediction and Set B for inference. Base on the conclusion in RAN1#116, existing CSI framework is used for the configuration of Set B, i.e., CSI-ResourceConfigId in CSI-ReportConfig. Note that, for BM-Case2, the CSI-RS/SSB resources configuration should facilitate UE to collect measurements for multiple past time instance. The design principle for doppler domain CSI prediction introduced in Rel-18 can be considered. For the configuration of Set A, one discussion point is whether Set A beams are mapped with reference signals as legacy. That is, whether the identifiers for representing Set A beams should be always associated with RS resources as legacy. In our view, this should not be mandated since one of the key purposes of AI/ML beam predication is to save the overhead of RS measurement. Hence, we are supportive to the configurability of two alternatives (Alt 1 and Alt 3) formulated in RAN1#116bis. For Alt-1, for simplicity, the size of Set A can be explicitly configured by the CSI-ReportConfig. The beam information in Set A can be further determined based on associated ID and/or the association information for Set A and/or Set B.

Proposal 7. For UE-sided model at least for BM Case-1, for the configuration of inference results reporting using CSI-ReportConfig, support the configurability between Alt 1 and Alt 3.

- Alt 1: one CSI-ResourceConfigId is configured for Set B

  - the size of Set A is configured by the CSI-ReportConfig.

- Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

- The resources configured in Set B can be periodic, semi-persistent or aperiodic.

  - FFS: For BM-Case2, the configuration of aperiodic resources for Set B to facilitate measurements for multiple past time instance.

One FFS point for the measurement report configuration of UE-side model is the association of Set A and Set B. In our understanding, DL Tx IDs should be introduced for the representation or identification of downlink spatial domain transmission filter since the existing resource ID cannot ensure the use of same downlink spatial domain transmission filters. The downlink spatial domain transmission filters corresponding to Set A and Set B should be consistent across LCM procedure, otherwise it is impossible for UE to generate accurate inference result. For a given gNB setting, a set of beams (i.e., downlink spatial domain transmission filters) can be used by gNB for DL transmission. As baseline, each beam of Set A and each beam of Set B should be associated with one DL Tx ID which ensures the spatial association between Set A and Set B. From UE perspective, UE shall assume the beams corresponding to the same DL Tx ID shares the same downlink spatial domain transmission filter. Also, it is worth considering the relationship between DL Tx ID and associated ID. For example, the above description is only applicable to the case that Set A/B are related to the same associated ID.

Proposal 8. For UE-sided model at least for BM Case-1, for the association between Set A and Set B, introduce DL Tx IDs for the identification of downlink spatial domain transmission filter.

- Each beam in Set A is associated with an DL Tx ID

- Each beam in Set B is associated with an DL Tx ID

- Note: UE assumes the beams corresponding to the same DL Tx ID shares the same downlink spatial domain transmission filter.

- FFS: the relationship between DL Tx ID and associated ID.

Agreement (RAN1#116) For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

In terms of the reporting of UE-side model inference result, two options (i.e., option 1 and option 2) were agreed in RAN1#116. One remaining issue is how the top K predicted beam is reported. This issue is associated with the configuration of Set A. If a Set A consists of several SSB/CSI-RS resources, predicted SSBRI/CRI should be used as the predicted beam information for reporting. If a Set A is defined by the size of Set A (K1), predicted beam indicator can be used as the predicted beam information for reporting.

Proposal 9. For UE-sided model, at least for BM-Case1, for the definition of the beam information on predicted Top K beam(s) among Set A, consider the following options:

- Option 1. The beam information is predicted SSBRI/CRI.

- Option 2. The beam information is predicted beam indicator.

Agreement (RAN1#116bis) For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Working Assumption (RAN1#116bis) For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

Another remaining issue for UE-side model inference reporting is the definition of reported L1-RSRP. Based on the agreement in RAN1#116bis, two options are formulated (Option A and Option B). In our view, both Option A and Option B should be supported since they have both pros and cons. For option A, since only predicted RSRP is reported, it is easier for UE to use the inference result for model monitoring. For example, another CSI-ReportConfig can be configured for Set A measurement and the comparison between measured L1-RSRP and predicted L1-RSRP can be done at NW-side. For option B, the report of measured L1-RSRP if applicable can provide better accuracy of reported L1-RSRP corresponding to the predicted beam.

Proposal 10. For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, support both Option A and Option B.

AI/ML model output is dependent on the type of the model. For classification-based model, each of the Set A beams corresponds to a probability. Typically, top-K beams are the beams with the highest probability values. For regression-based model, each of the Set A beams corresponds to a predicted L1-RSRP. Typically, top-K beams are determined based on the value of the corresponding predicted L1-RSRPs.

For regression-based model, one remaining issue from the RAN1#116bis is whether probability information is reported. In our view, the probability information could also be useful since the probability can reflect beam prediction accuracy in some extend. Hence, apart from the report of K predicted beam, it is also preferable to report: the probabilities correspond to the K predicted beams.

Proposal 11. For UE-sided model, at least for BM-Case1, for content in the report of inference results, support:

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

  - FFS on the quantization method of probability information

  - Probability information is the probability of the beam to be the Top 1 or Top K beam

In case of K>1, one issue is the convey of the ranking information of the reported K beam. In legacy, for L1-RSRP based beam reporting, the ranking information is conveyed implicitly based on the reported L1-RSRP. For example, NW can assume that the beam corresponding to the highest L1-RSRP is the beam that has the highest ranking. For beam reporting for UE-side model inference, since new reporting contents/combinations are introduced, the method to convey the ranking information of the predicted Top K beams should be further studied.

Proposal 12. For UE-sided model, at least for BM-Case1, for content in the report of inference results, further study the method to convey the ranking information of the predicted Top K beams in case of K > 1.

For UE-side model inference, one remaining issue is the configuration and the maximum value of the number of predicted beam (i.e., K). In our view, K is configured by the corresponding CSI-ReportConfig and the maximum value of K is subject to UE capability.

Proposal 13. For UE-sided model inference, for the reporting of predicted Top K beam(s) among Set A,

- K is configured by CSI-ReportConfig and the maximum configurable value of K is subject to UE capability.

Agreement (RAN1#116bis) For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N (N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Agreement (RAN1#118) For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted. Agreement (RAN1#118bis) For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

For BM-Case2, it was agreed in RAN1#116bis that the output of model inference can be associated with N (N≥1) future time instances. The key remaining issues is the determination of the N future time instances. The length and the time domain position for each of the N future time instances should be considered. The length for each of the N future time instance should be the same. Also, each of the time instances should be slot based which is similar to the existing definition of CSI reference resource in time domain. For the time domain position, three options for the reference time of the earliest time instance from the N further time instance(s)was agreed to considered in RAN1#118bis. For the down-selection, all three methods can work with marginal difference and option 1 is slightly preferred since it is consistent with the design principle for Rel-18 CSI prediction in doppler domain. With the information of reference time, the earliest time instance can be further determined based on additional offset. In case of N >1, for the determination of the remaining time instances other than the earliest one, one solution is to assume that the time domain separation between two adjacent time instance(s) from the N future time instances are the same. Moreover, how the time domain separation is determined can be further discussed. Another issue is defining measurement window(s) for the determination of the report. With the measurement window(s), the averaging of L1-RSRP across different measurement windows can be avoided which facilitate the model inference of BM-Case2.

Proposal 14. For BM-Case 2 of UE-side model, for the determination of N future time instance(s), support the following:

- Each of the N time instance(s) is consist of P (P≥1) consecutive slots

- For the reference time of the earliest time instance for the predicted results, support option 1 (Based on the uplink slot for the report)

  - An offset between the reference time slot and the first slot of the earliest time instance is D1

- For N>1, the offset between the first slots of two adjacent time instances is D2

- FFS: the determination and the values for P , D1 and D2

Agreement (RAN1#118) For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details

In RAN1#118, it differential L1-RSRP reporting is supported for the report of inference result. An FFS point is the detail of BM-Case2. The existing design principle for differential L1-RSRP should be reused, i.e., only one absolute L1-RSRP in one report instance while others are differential L1-RSRPs. In case of multiple time instances for beam prediction, how to identified time instance for the largest measured L1-RSRP value requires further study. One potential solution is to report a time instance indicator for this purpose.

Proposal 15. For UE-side AI/ML model inference and BM-Case2, support the following:

- The differential L1-RSRP value is with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance.

- FFS: How to identify the largest measured L1-RSRP value if there are multiple time instance.

3.1.2 On the consistency between training and inference

Agreement (RAN1#116bis) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. Agreement (RAN1#118) For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

In RAN1#116bis, two options were agreed for the consistency of NW-side additional condition across training and inference for UE-sided model. For Option 1, on the top of the configuration of Set A and Set B, associated ID is used for UE to identify the proper model for inference and generate meaningful beam prediction results. The supported/preferred associated ID can be configured by NW and the associated ID can be reported to NW before configuring the beam report for model inference, e.g., via UE reporting or UE capability signaling. Along with the report of association ID, other information can be also considered to be transmitted. For example, cell information and/or information on DL Tx IDs for Set A/B corresponding to the cell.

Proposal 16. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, support the following:

- UE to report the information on the supported/preferred associated ID.

- FFS: Other information along with the report of the association ID.

Proposal 17. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, support the following:

- NW configuration of associated ID in a CSI-ReportConfig

In RAN1 118, it was agreed that UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID. A remaining FFS point is whether/how to define “similar properties” of a DL Tx beam or beam set/list. Similar to the UE assumption of P3 UE side beam sweeping, the “similar properties” of a DL Tx beam or beam set/list can be the same set of downlink spatial domain transmission filters. Also, the “similar properties” of a DL Tx beam or beam set/list can include a consistent mapping, e.g., consistent order of measurement resources. Moreover, as previously discussed, additional information can be provided Set A and Set B, e.g., DL TX ID which facilitates UE to identify the Tx beams of Set A and/or Set B under a particular associated ID. This can ensure the consistency of model input and model output across LCM procedure.

Proposal 18. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, and for the definition of similar properties of a DL Tx beam or beam set/list

- At least in a cell level, the UE assumes consistent physical beams characteristics on DL Tx beams (e.g., same downlink spatial domain transmission filters) of Set A and Set B, as well as a consistent mapping, e.g., consistent order of measurement resources

3.1.3 Beam indication based on the predicted beam

Agreement (RAN1#116bis) For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework - FFS on whether/how potential enhancement is needed

The beam indication aspect associated with UE-side AI/ML model was discussed in RAN1#116bis and unified TCI state framework is considered for further enhancement.

After the reception of the reported predicted beams from UE, NW can provide beam indication based on the predicted beams. As there is a common understanding between UE and NW on Set A beams, the indication of Set A beams is possible. This can provide finer beam granularity in terms of beam indication.

Proposal 19. For UE-side AI/ML model, support beam indication for Set A beams.

3.2 Performance monitoring

Agreement (RAN1#117) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement (RAN1#118) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

In RAN1#117, Type 1 performance monitoring is supported for BM-Case1 and BM-Case2 with a UE-side AI/ML model. Also, the supported Type 1 performance monitoring includes two options: Option 1 (NW-side performance monitoring) and Option 2 (UE-assisted performance monitoring). In the following, discussions are provided from the perspective of different types of performance monitoring.

3.2.1 Type 1 performance monitoring

For the option 1 of Type 1 performance monitoring, the existing CSI framework and the CSI reporting mechanism for UE-side model inference can be used as baseline. For example, a CSI-ReportConfig for UE-side model inference can be used for the collection of prediction results of Set A and another existing CSI-ReportConfig can be used for the measurement for Set A. In this way, NW can obtain the beam prediction accuracy of Set A. Also, further enhancement can be considered. For example, the use of a single CSI-ReportConfig for the report for both prediction results and measurement results for Set A.

Proposal 20. For the option 1 of Type 1 performance monitoring, the existing CSI reporting mechanism and the CSI reporting mechanism for UE-side model inference are used as baseline.

- FFS: whether to use single CSI-ReportConfig for the report for both prediction results and measurement results for Set A.

Agreement (RAN1#118bis) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement(RAN1#118bis) At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

For the option 2 of Type 1 performance monitoring, UE calculates the performance metrics to save reporting overhead. As agreed in RAN1#118bis, Alt-1 (beam prediction accuracy) is supported for UE-assisted performance monitoring. The metric of beam prediction accuracy is to deliver the beam accuracy of inference results to gNB. The metric should be defined based on the reported inference results. In this way, gNB has a clear understanding on the meaning of the metric which is the beam prediction accuracy subject to the previous inference reports.

Observation 1. To ensure gNB has a clear understanding on the meaning of the metric for beam prediction accuracy, the metric should be derived based on the reported inference results.

For BM-Case1, to obtain the percentage of beam accuracy, a metric can be defined by the comparisons between the Top 1/K predicted beam per report and the Top 1/K measured beam from a resource set for monitoring. In particular, the metric is determined based on the comparisons of the inference result for each of X (X≥1) CSI reports of a given CSI-ReportConfig and the associated measurement(s). For example, a new report quantity for beam accuracy, namely, beam accuracy indicator (BAI) can be introduced in CSI report. The value of BAI k (0≤k≤X) is the number of CSI reports with comparison results as “true” from the X CSI reports. The prediction accuracy associated with the BAI is kX and the size of CSI field associated with the BAI is log2X. In terms of the definition of “true”, several options listed in TR38.843  (highlighted) can be considered. Here, an FFS point is how to determine the X CSI reports. For example, the X CSI reports should be at least no later than the report of the associated BAI. Also, for the purpose of comparison, the measurement associated with each of the X CSI report should be defined/clarified.

TR38.843 - Beam prediction accuracy (%): - Top-1 (%): the percentage of "the Top-1 genie-aided beam is Top-1 predicted beam" - Top-K/1 (%): the percentage of "the Top-1 genie-aided beam is one of the Top-K predicted beams" - Top-1/K (%) (Optional): the percentage of "the Top-1 predicted beam is one of the Top-K genie-aided beams" - Where K >1 and values can be reported - CDF of L1-RSRP difference for Top-1 predicted beam - Beam prediction accuracy (%) with 1dB margin for Top-1 beam - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam"

Proposal 21. For the monitoring Type 1 Option 2 of UE-side model monitoring, for BM-Case1, introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring

- X (X≥1) CSI reports for inference of a given CSI-ReportConfig are used for the determination of BAI

- The value of BAI k (0≤k≤X) is the number of CSI reports with comparison results as “true” from the X CSI reports

  - FFS: the definition of “true”:

    - Option 1 (Top-1): the Top-1 beam of the resource set for monitoring is Top-1 predicted beam

    - Option 2 (Top-K/1): the Top-1 beam of the resource set for monitoring is one of the Top-K predicted beams

    - Option 3 (Top-1/K): the Top-1 predicted beam is one of the Top-K beams of the resource set for monitoring

    - Option 4 (1dB margin for Top-1 beam): the Top-1 predicted beam with measured L1-RSRP is within 1dB of the measured L1-RSRP of the Top-1 beam of the resource set for monitoring

- The accuracy corresponds to the BAI is kX

- The size of CSI field associated with the BAI is log2X

- FFS: how to determine the X CSI reports for inference

- FFS: for the purpose of comparison, how to determine the associated measurement from a resource set for monitoring for each of the X CSI reports

Agreement(RAN1#118bis) At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results.

Another discussion point is the framework for the configuration of the CSI report for monitoring. In RAN1#118bis, two alternatives are agreed to be further considered. Option 1 is to configure the function of inference and monitoring using the same CSI-ReportConfig. Option 2 is to configure the function of inference and monitoring, respectively, using the separate CSI-ReportConfig with linkage. In our view, option 2 is preferred since it is cleaner and is consistent with the Rel-19 MIMO design on the CSI report for CJT calibration. Also, for option 1, a CSI-ReportConfig is associated with different types of in different report instances which has no precedent and will be cumbersome in terms of the configuration/triggering of the associated reporting, especially when different types of time domain behaviours are taken into account. Also, for the time domain behavior of the CSI report for monitoring, AP/SP/P CSI report are supported.

Proposal 22. At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support option 2,

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

- UE measures the resource set(s) for monitoring

- Note: AP/SP/P CSI report for monitoring are supported

Another FFS point is how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams. The connection of the RS is to facilitate the comparison to generation metric for beam prediction accuracy. In our view, if the resource set for monitoring is of the same size of Set A, then a predefine mapping rule can be used based on the ordering of resources within those sets. For example, the i-th resource within Set A is mapped to the i-th resource in the resource set for monitoring. The resource in Set A can be mapped to a resource in resource set for monitoring so that the comparison of predicted beam and the beam for monitoring is possible.

To save the measurement overhead, all the resource within Set A is configured in the resource set in monitoring is not desired. In such case, a subset of Set A is more preferred to be configured for performance monitoring. It is possible that a beam predicted in Set A is not in the set for monitoring. To make the comparison possible, it is necessary to have the linkage between Set A resources and the resources in the monitoring set. To make comparison meaning, it is reasonable to assume that the subset of Set is a down-sampled version of Set A. That is, several beams in similar directions are treated together and only one of them is used in the subset. In this way, the mapping of resource in Set A and resource in monitoring set are comparable. Note that similar framework can be used for full set of set A and subset of set A. For example, the resource mapping is done in predefine manner, where the i-th resource within Set A is mapped to the iD-th resource in the resource set for monitoring. Here, D is the down-sampling factor.

Proposal 23. At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the calculation the metric for beam prediction accuracy, support predefined mapping between the resources within Set A and the resources within the set for monitoring

- the i-th resource within Set A is mapped to the iD-th resource in the resource set for monitoring

- D is down-sampling factor

- FFS: the determination and the value of D

For the option 2 of Type 1 performance monitoring, as part of FFS point, event-based reporting can be considered to save the overhead of UE reporting. Basically, three events can be considered as starting point: 1) Event-1: The measured Top-K beam(s) of Set A and the predicted Top-K beam(s) of Set A are different; 2) Event-2: The L1-RSRP difference between the measured Top-K beam(s) of Set A and predicted Top-K beam(s) of Set A are larger than a threshold value; 3) Event-3: The probability information of Top-1 beam of Set A is lower than a threshold. Event-1, Event-2 and Event-3 are corresponding to metric Opt 1, Opt 2 and Opt 3, respectively. Based on the event(s), UE can report the corresponding notification/report to NW. The reporting/notification content and the Set A / Set B configuration related to the events can be further studied.

Proposal 24. For the option 2 of Type 1 performance monitoring, at least for BM-Case1, consider the following events to trigger UE reporting/notification:

- Event-1: The measured Top-K beam(s) of Set A and the predicted Top-K beam(s) of Set A are different

- Event-2: The L1-RSRP difference between the measured Top-K beam(s) of Set A and predicted Top-K beam(s) of Set A are larger than a threshold value

- Event-3: The probability information of Top-1 beam of Set A is lower than a threshold

- FFS: The content of the reporting/notification

- FFS: The configuration of Set A and or Set B

3.2.2 Type 2 performance monitoring

For type 2 performance monitoring, LCM operation is mostly up to UE implementation. For the signaling exchange between UE and NW (e.g., for requesting or configuring performance monitoring) the request, measurement and/or reporting mechanism for UE-side AI/ML model inference can be reused as much as possible.

Proposal 25. For UE-side AI/ML model, support Type 2 performance monitoring.

For type 2 performance monitoring, UE can be configured with Set A and Set B for performance monitoring and no corresponding report is needed. This can be done by the extension of the CSI reporting mechanism, e.g., CSI-ReportConfig with reportQuantity set to ‘none’. Also, Set A and Set B can be configured in the same CSI-ReportConfig for measurement or in different CSI-ReportConfig, respectively.

Proposal 26. For the support of Type 2 performance monitoring of UE-side AI/ML model, consider the extension of the CSI reporting mechanism, e.g., CSI-ReportConfig with reportQuantity set to ‘none’.

3.3 CPU and timeline

Agreement (RAN1#118bis) For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2 Agreement (RAN1#118bis) For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

In RAN1#118bis, it was agreed that the existing CPU mechanism is used as the starting point for the AI/ML-based CSI reporting for UE-side model. The legacy CPU mechanism includes two aspects: the number of occupied computing resource (i.e., OCPU) and the corresponding occupation time. OCPU is defined for the number of computing units that occupied by a CSI reporting in a given symbol. For occupation time, starting symbol and ending symbol of CPU occupation for a given CSI reporting is defined which is subject to the time domain behavior of the CSI reporting.

For a given AI/ML CSI reporting, OCPU and the CPU occupation time needs to defined as well. For the existing CSI reporting for beam management, OCPU =1 since only L1-RSRP calculation of measurement results are needed. For the CSI reporting for inference, this value may not be sufficient due to the complexity of model inference. Considering the complexity of model inference is UE-specific, it is safe to assume that OCPU is at least depending on UE capability. An FFS point is the granularity of the UE capability. For example, the UE capability is use case specific (e.g., BM-Case1 and BM-Case2 are of different values).

Proposal 27. For the CSI report for inference for UE-side model, OCPU is depending on UE capability.

- FFS: the granularity of the UE capability, e.g., whether it is use case specific

One of FFS point is whether the OCPU should be shared or separately counted across non-AI/ML based CSI reports and AI/ML based CSI reports. Although they share some common prospects, some aspects of AI/ML based CSI report are different from legacy non-AI/ML based CSI report. For example, in some UE implementation, all AI/ML based CSI report (e.g., for BM and/or CSI) could share same dedicated NPU architecture, which is different from the legacy computational unit. During activation of specific AI/ML functionality, model of the functionality, including the parameters of network and software instructions for execution particular model, needed to be loaded to the NPU. Moreover, simultaneously active functionalities will be constraint by either total computation units of NPU or memory associated with the NPU architecture. Therefore, for a more flexible management of AI/ML computing resources, it is preferred to have a separated UE capability can be defined for AI/ML based CSI reports with separated CPU counting from legacy non-AI/ML CSI report.

Proposal 28. Consider separately CPU counting between legacy non-AI/ML based CSI report and AI/ML based CSI report.

For CPU occupation time, legacy definition can be reused as much as possible. For BM-Case1, the legacy definition of Rel-15 P/SP/AP CSI can be reused. For BM-Case2, the legacy definition of CSI for Rel-18 doppler domain CSI prediction can be reused.

Proposal 29. For CPU occupation time, legacy definition is reused

- For BM-Case1, the legacy definition of Rel-15 P/SP/AP CSI is reused

- For BM-Case2, the legacy definition of CSI for Rel-18 doppler domain CSI prediction is reused

If the legacy definition CPU occupation time is reused, the minimum CPU occupation time for a particular CSI report is subject to Z timeline. For legacy CSI report for beam management, the Z timeline is Z3/Z3’. However, for the generation of inference results, more time may be needed. The extension of CSI timeline can be the scaling of Z3/Z3’. Similar to the discussion of OCPU, the complexity of model inference is UE-specific. Then, it is safe to assume that the scaling factor is at least depending on UE capability. An FFS point is the granularity of the UE capability. For example, the UE capability is use case specific (e.g., BM-Case1 and BM-Case2 are of different values).

Proposal 30. For the CSI report for inference for UE-side model, support the scaling of legacy Z timeline (i.e., Z3/Z3’) for beam management with a scaling factor

- The scaling factor is depending on UE capability

- FFS: the granularity of the UE capability, e.g., whether it is use case specific

- Draft reply to R2-2407848

4.1 Introduction

RAN2 sent LS for applicable functionality reporting for beam management with the following question [1].

On General - Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? On NW-side additional condition and configuration - Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? - Q3: Is NW-side additional condition functionality specific? - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition? - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities? - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc? - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities? - Q5: What is the content of applicable functionality reporting in Step 4? - Q6: What is the content of inference configuration in Step 5? On Functionality Activation - Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? - Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5? - Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? - Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

In addition, in RAN1#118bis, the following agreements related to RAN2 LS were agreed as follows.

Agreement Answer to Q2 in R1-2407604 as below: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.. Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration. Agreement For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report. Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing Agreement Answer to Q1 in R1-2407604 as below, In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI..

The following part discusses RAN2’s questions and provides draft responses.

4.2 Discussion

Question 1 (Agreed)

- Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?

[Response] In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Question 2 (Agreed)

- Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

[Response] RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Question 3

- Is NW-side additional condition functionality specific?

[Draft response] RAN 1 did not have any agreement on whether NW-side additional condition is functionality specific or not. RAN 1 will further discuss whether/how to the relation between associated ID and functionality.

Question 4

Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5):

- Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

- Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities?

- Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

- Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

[Draft response] For Q4, the following replies are provided:

- Q4-1: NW-side additional condition can be optional. If without NW-side additional condition, it is up to UE implementation to decide applicable functionalities. However, this will introduce additional burden and potential performance loss to UE.

- Q4-2: It is feasible but not required. To provide functionality related information in Step 3 may be helpful but to provide inference configuration in step 3 could be less motivated since UE cannot use the configuration until applicable functionalities are reported. RAN 1 agreed to use CSI-ReportConfig to configure resources for measurement (for Set A and Set B of beams) and the inference report configuration in CSI framework (FFS to configure the associated ID in other procedure). RAN 1 also agreed a working assumption to configure the associated ID at least within CSI framework for NW-side additional condition. From RAN 1 point of view, the above configuration, i.e., the CSI-ReportConfig belongs to Step 5 other than Step 3.

- Q4-3: Not applicable.

- Q4-4: Not applicable.

Question 5

- Q5: What is the content of applicable functionality reporting in Step 4?

[Draft response] This is depending on the outcome of Q1. UE is expected to provide applicable functionality reporting to NW in the same granularity as in Step 1.

Question 6

- Q6: What is the content of inference configuration in Step 5?

[Draft response] From RAN 1 point of view, the above inference configuration, i.e., CSI-ReportConfig can be configured in Step 5.

Question 7

- Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3?

[Draft response] From RAN 1 perspective, CSI framework are used to configure/activate a CSI report carrying inference results. Whether the activation of functionality is coupled with the configuration of CSI report (e.g., activate the functionality immediately upon receiving configuration) will be further discussed in RAN1.

Question 8

- Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5?

[Draft response] Please see the reply to Q7.

Question 9

- If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated?

[Draft response] Yes, it is possible. For example, if use case level granularity is supported, these two functionalities can be activated at the same time. Also, different use cases such as BM and CSI prediction can be activated at the same time.

Question 10

- Is L1/L2 signaling for functionality activation/deactivation needed?

[Draft response] Yes. L1/L2 signalling is better for UE to have a clear timeline to check the applicability of L1 reporting for beam prediction. RAN1 will further discuss whether the signaling is a new signaling or the same/variation of existing signaling to trigger CSI report.

- Conclusion

The observations and proposals made in this contribution are summarized below.

Observation 1. To ensure gNB has a clear understanding on the meaning of the metric for beam prediction accuracy, the metric should be derived based on the reported inference results.

Proposal 1. For NW-side AI/ML model data collection for training, support at least the following as data collection content:

- L1-RSRP(s) for all beam(s) of Set B

- L1-RSRP(s) for all beam(s) of Set A

- Top-K Beam ID(s) for Set A

- Related timestamp

- The information to facilitate model training (FFS details)

Proposal 2. For NW-side AI/ML model data collection for training, at least support the enhancement to use high layer signaling to convey data collection content.

Proposal 3. For NW-side AI/ML model inference, for CSI-ReportConfig with the measurements for more than 4 beams in one reporting instance

- Support differential L1-RSRP reporting with larger quantization step(s) and/or smaller range(s)

- Consider to reduce the overhead of beam report using two-part CSI

Proposal 4. For NW-side AI/ML model inference, for CSI-ReportConfig with the measurements for more than 4 beams in one reporting instance, consider the applicability of the following report quantity:

- 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP-Index', 'ssb-Index-RSRP-Index'.

Proposal 5. For NW-side AI/ML model inference, support CSI-ReportConfig with measurements for each of multiple past time instances in one reporting instance.

- FFS: How to identify the multiple past time instances

- FFS: The support of P/SP/AP reporting

Proposal 6. Support single beam indication for multiple future time instances using unified TCI framework.

Proposal 7. For UE-sided model at least for BM Case-1, for the configuration of inference results reporting using CSI-ReportConfig, support the configurability between Alt 1 and Alt 3.

- Alt 1: one CSI-ResourceConfigId is configured for Set B

  - the size of Set A is configured by the CSI-ReportConfig.

- Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

- The resources configured in Set B can be periodic, semi-persistent or aperiodic.

  - FFS: For BM-Case2, the configuration of aperiodic resources for Set B to facilitate measurements for multiple past time instance.

Proposal 8. For UE-sided model at least for BM Case-1, for the association between Set A and Set B, introduce DL Tx IDs for the identification of downlink spatial domain transmission filter.

- Each beam in Set A is associated with an DL Tx ID

- Each beam in Set B is associated with an DL Tx ID

- Note: UE assumes the beams corresponding to the same DL Tx ID shares the same downlink spatial domain transmission filter.

- FFS: the relationship between DL Tx ID and associated ID.

Proposal 9. For UE-sided model, at least for BM-Case1, for the definition of the beam information on predicted Top K beam(s) among Set A, consider the following options:

- Option 1. The beam information is predicted SSBRI/CRI.

- Option 2. The beam information is predicted beam indicator.

Proposal 10. For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, support both Option A and Option B.

Proposal 11. For UE-sided model, at least for BM-Case1, for content in the report of inference results, support:

- Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

  - FFS on the quantization method of probability information

  - Probability information is the probability of the beam to be the Top 1 or Top K beam

Proposal 12. For UE-sided model, at least for BM-Case1, for content in the report of inference results, further study the method to convey the ranking information of the predicted Top K beams in case of K > 1.

Proposal 13. For UE-sided model inference, for the reporting of predicted Top K beam(s) among Set A,

- K is configured by CSI-ReportConfig and the maximum configurable value of K is subject to UE capability.

Proposal 14. For BM-Case 2 of UE-side model, for the determination of N future time instance(s), support the following:

- Each of the N time instance(s) is consist of P (P≥1) consecutive slots

- For the reference time of the earliest time instance for the predicted results, support option 1 (Based on the uplink slot for the report)

  - An offset between the reference time slot and the first slot of the earliest time instance is D1

- For N>1, the offset between the first slots of two adjacent time instances is D2

- FFS: the determination and the values for P , D1 and D2

Proposal 15. For UE-side AI/ML model inference and BM-Case2, support the following:

- The differential L1-RSRP value is with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance.

- FFS: How to identify the largest measured L1-RSRP value if there are multiple time instance.

Proposal 16. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, support the following:

- UE to report the information on the supported/preferred associated ID.

- FFS: Other information along with the report of the association ID.

Proposal 17. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, support the following:

- NW configuration of associated ID in a CSI-ReportConfig

Proposal 18. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, and for the definition of similar properties of a DL Tx beam or beam set/list

- At least in a cell level, the UE assumes consistent physical beams characteristics on DL Tx beams (e.g., same downlink spatial domain transmission filters) of Set A and Set B, as well as a consistent mapping, e.g., consistent order of measurement resources

Proposal 19. For UE-side AI/ML model, support beam indication for Set A beams.

Proposal 20. For the option 1 of Type 1 performance monitoring, the existing CSI reporting mechanism and the CSI reporting mechanism for UE-side model inference are used as baseline.

- FFS: whether to use single CSI-ReportConfig for the report for both prediction results and measurement results for Set A.

Proposal 21. For the monitoring Type 1 Option 2 of UE-side model monitoring, for BM-Case1, introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring

- X (X≥1) CSI reports for inference of a given CSI-ReportConfig are used for the determination of BAI

- The value of BAI k (0≤k≤X) is the number of CSI reports with comparison results as “true” from the X CSI reports

  - FFS: the definition of “true”:

    - Option 1 (Top-1): the Top-1 beam of the resource set for monitoring is Top-1 predicted beam

    - Option 2 (Top-K/1): the Top-1 beam of the resource set for monitoring is one of the Top-K predicted beams

    - Option 3 (Top-1/K): the Top-1 predicted beam is one of the Top-K beams of the resource set for monitoring

    - Option 4 (1dB margin for Top-1 beam): the Top-1 predicted beam with measured L1-RSRP is within 1dB of the measured L1-RSRP of the Top-1 beam of the resource set for monitoring

- The accuracy corresponds to the BAI is kX

- The size of CSI field associated with the BAI is log2X

- FFS: how to determine the X CSI reports for inference

- FFS: for the purpose of comparison, how to determine the associated measurement from a resource set for monitoring for each of the X CSI reports

Proposal 22. At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support option 2,

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

- UE measures the resource set(s) for monitoring

- Note: AP/SP/P CSI report for monitoring are supported

Proposal 23. At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the calculation the metric for beam prediction accuracy, support predefined mapping between the resources within Set A and the resources within the set for monitoring

- the i-th resource within Set A is mapped to the iD-th resource in the resource set for monitoring

- D is down-sampling factor

- FFS: the determination and the value of D

Proposal 24. For the option 2 of Type 1 performance monitoring, at least for BM-Case1, consider the following events to trigger UE reporting/notification:

- Event-1: The measured Top-K beam(s) of Set A and the predicted Top-K beam(s) of Set A are different

- Event-2: The L1-RSRP difference between the measured Top-K beam(s) of Set A and predicted Top-K beam(s) of Set A are larger than a threshold value

- Event-3: The probability information of Top-1 beam of Set A is lower than a threshold

- FFS: The content of the reporting/notification

- FFS: The configuration of Set A and or Set B

Proposal 25. For UE-side AI/ML model, support Type 2 performance monitoring.

Proposal 26. For the support of Type 2 performance monitoring of UE-side AI/ML model, consider the extension of the CSI reporting mechanism, e.g., CSI-ReportConfig with reportQuantity set to ‘none’.

Proposal 27. For the CSI report for inference for UE-side model, OCPU is depending on UE capability.

- FFS: the granularity of the UE capability, e.g., whether it is use case specific

Proposal 28. Consider separately CPU counting between legacy non-AI/ML based CSI report and AI/ML based CSI report.

Proposal 29. For CPU occupation time, legacy definition is reused

- For BM-Case1, the legacy definition of Rel-15 P/SP/AP CSI is reused

- For BM-Case2, the legacy definition of CSI for Rel-18 doppler domain CSI prediction is reused

Proposal 30. For the CSI report for inference for UE-side model, support the scaling of legacy Z timeline (i.e., Z3/Z3’) for beam management with a scaling factor

- The scaling factor is depending on UE capability

- FFS: the granularity of the UE capability, e.g., whether it is use case specific

References

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm.