3GPP TSG-RAN WG1 Meeting #119	  R1-2409623

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	5

Source:	Spreadtrum, UNISOC

Title:	Discussion on LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

Work Item:	NR_duplex_evo-Core

Document for:	Discussion, Decision

# 1	Introduction

RAN4 sent an LS to RAN1 with the title “LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation” [1]. An excerpt is shown below:

1. Overall Description: When RAN4 discussed the requirements for UE-to-UE L1 CLI measurement, e.g., L1 SRS-RSRP, in SBFD operation, it was identified that the time difference between the victim UE’s DL reference timing in the serving cell and aggressor UE’s SRS arrival timing depends on the applicable scenarios: - Intra-cell UE-to-UE L1 CLI measurement, where both aggressor UE and victim UE belong to the same cell and thus cell phase synchronization error of 3us does not need to be considered - Inter-cell UE-to-UE L1 CLI measurement, where aggressor UE and victim UE belong to different cells and thus cell phase synchronization error of 3us needs to be considered To define the appropriate requirements, RAN4 needs to understand from RAN1 perspective if both scenarios are considered or just intra-cell scenario is considered. Any other clarification or info from RAN1 is appreciated too. 2. Actions: To RAN ACTION: RAN4 respectfully asks RAN1 to provide clarification on the target scenarios for UE-to-UE L1 CLI measurement.

# 2	Discussion

According to the WID of Evolution of NR duplex operation [2], the notes of L1 based UE-to-UE CLI measurement and reporting shows that there is no SRS configuration information exchange between gNBs. Therefore, it’s impossible for the victim UE to do inter-cell UE-to-UE L1 CLI measurement without knowing the location of SRS transmitted from aggressor UE belong to different cells. So only intra-cell scenario is considered for UE-to-UE CLI measurement and reporting in SBFD operation.

- Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

Proposal 1: Only intra-cell scenario is considered for UE-to-UE L1 CLI measurement and reporting in SBFD operation.

# 3	Conclusion

Based on the discussion we propose the following:

Proposal 1: Only intra-cell scenario is considered for UE-to-UE L1 CLI measurement and reporting in SBFD operation.

# References

R1-2409352, “LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation”, RAN1#119, Orlando, US, November 18th – 22nd, 2024

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#104, Shanghai, China, June 17th-20th, 2024