3GPP TSG RAN WG1 #119			R1-2409624

Orlando, US, November 18th – 22nd, 2024

Agenda item:	7

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on PDCCH candidate with larger AL than the REGs in CORESET

Document for:	Discussion and decision

Introduction

This contribution discusses counting and overbooking of PDCCH candidate with larger AL than the REGs in CORESET for Rel-15.

Discussion

The issue is happened when a PDCCH candidate with an aggregation level is larger than the number of REGs in the CORESET. For example, a candidate of aggregation level configured for a search space set is 16, and the CORESET associated with the search space has only 48 PRB with 1 symbol duration.

There was an agreement in RAN1#92 to allow this type of configuration, and UE is not required to monitor this candidate.

Agreements RAN1#92: - To adopt the TP for TS38.213 Section 10.1 - Also add one sentence in the spec saying “when the number of REGs is not sufficient for a given aggregation level, the UE is not required to monitor candidates of the given aggregation level” - Up to spec editor for final wording === Start === Table 10.1-1: CCE aggregation levels and max number of candidates per CCE aggregation level for Type0/Type0A/Type2-PDCCH common search space CCE Aggregation Level; Number of Candidates. 4; 4. 8; 2. 16; 1. === End ===

There are some other PDCCH candidates which UE is not required to monitor and they are still counted, such as when PDCCH candidates overlapping with SSB or rate matching pattern of PDSCH, as shown below.

38.213 clause 10.1 For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SSB-MTCAdditionalPCI for a serving cell, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block, after puncturing if applicable, corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SSB-MTCAdditionalPCI with same physical cell identity as the one associated with a RS having same quasi-collocation properties as a CORESET for the PDCCH candidate, the UE is not required to monitor the PDCCH candidate.

If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception scheduled by a DCI format as described in [6, TS 38.214]. If a PDCCH candidate that provides a DCI format is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate.

For a PDCCH candidate with an aggregation level which is larger than the number of REGs in the CORESET, UE is not required to monitor this candidate according to the agreement; however, it is still not clear whether the candidate should be counted or not. It is necessary to clarify the UE behaviour for these PDCCH candidates as it not only will affect BD counting of this search space set, but also will affect other search space sets monitoring if overbooking is configured on the PCell.

There would have two understandings from implementation aspect:

- Understanding 1: PDCCH candidates are counted. Because the counting of PDCCH is based on the configuration of search space, same rule also applied when PDCCH candidates are overlapping with SSB/rate matching pattern.

- Understanding 2: PDCCH candidates are not counted. These PDCCH candidates are not exist, which can be excluded from the configuration of search space and CORESET. It requires UE to set the number of PDCCH candidates to 0 or ignore these PDCCH candidates.

Observation 1: When the PDCCH candidates with an aggregation level are larger than the REGs in CORESET, UE is not required to monitor these candidates.

- Clarify the UE behaviour for the counting of these PDCCH candidates with one of the following understandings.

- Understanding 1: PDCCH candidates are counted.

- Understanding 2: PDCCH candidates are not counted.

Conclusion

In this contribution, we made the following observation and proposal.

Observation 1: When the PDCCH candidates with an aggregation level are larger than the REGs in CORESET, UE is not required to monitor these candidates.

- Clarify the UE behaviour for the counting of these PDCCH candidates with one of the following understandings.

- Understanding 1: PDCCH candidates are counted.

Understanding 2: PDCCH candidates are not counted.