3GPP TSG RAN WG1 #119																			R1-2409656

Orlando, US, November 18th – 22nd, 2024

Title:	Draft LS Reply on UE RF issues for Rel-19 MIMO enhancement

Response to:	R1-2409353 (R4-2417117)

Release:	Rel-19

Work Item:	NR_MIMO_Ph5

Source:	[RAN1]

To:	RAN4

Cc:

Contact person:

1	Overall description

RAN1 thanks RAN4 for the questions, please find RAN1 answers on questions 1, 2, 3 below.

Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID.

Heterogeneous Network can be deployed to improve UL throughput. Since the macro gNB and micro nodes differ in power rating, a UE may receive DL transmission from the macro gNB, but transmit UL to either the macro gNB or non-co-located micro nodes in order to maximize UL throughput. As an option to further reduce energy consumption, the micro nodes can, for instance, reduce or even turn off DL transmissions. To support such deployment scenario, enhancements on UL power control (PC) are needed at least. First, when pathloss RS is transmitted from the macro gNB and the UE transmits UL to the micro nodes, the pathloss measured from the pathloss RS from the macro gNB is not accurate. Therefore, it is necessary to configure the UE with pathloss offset to facilitate accurate calculation of the pathloss associated with the micro nodes. Second, an additional SRS closed-loop PC for DL CSI acquisition to the macro gNB (for DL transmission), separate from that for the SRS to the micro nodes (for UL mTRP reception) should be introduced. Therefore, there is a need for supporting two closed-loop PC adjustment states for SRS, both separate from PUSCH. Third, since neither the macro gNB nor the micro nodes can be configured with a coresetPoolIndex, the Rel-18 2TA (designed for multi-DCI) is deficient to ensure proper UL reception. Therefore, there is a need for extending the Rel-18 2TA specification for multi-DCI-based multi-TRP by removing the restriction that coresetPoolIndex needs to be configured (with legacy PRACH resources).

Answer 1: the text below is an excerpt of justification of revised WID for NR MIMO phase 5, yellow highlight sentence clearly mentions that the micro nodes can reduce or even turn off DL transmission. Reduced DL transmission from micro nodes hence it can be understood as the micro nodes can transmit SSB. The term “UL TRP(s)” could cause misunderstanding as it may indicate the scenario where the DL transmission from micro node is turned off only. Instead, the question should be addressing the micro nodes with reduced DL transmission. From RAN1 perspective, it is confirmed that the micro nodes transmitting SSBs is within the scope of NR MIMO Phase 5 WID. From RAN1 perspective, considering spec impact, it is applicable for the scenario of Rel-17 intercell beam management.

Following 2 questions are addressing same issue,

Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers.

Question 3:  RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks.

Answer 2, 3: Below is the excerpt of TS 38.213V16.7.0 section 7.1, the yellow highlighted points are relevant to UL full power transmission mode 0. It is very clear from the description that when UL full power transmission mode 0 is configured the scaling factor s=1 and the UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power. For non-coherent codebook rank=1 precoder there is only 1 antenna port on which the UE transmits PUSCH with non-zero power, for rank=2 precoder there are 2 antenna ports on which the UE transmits PUSCH with non-zero power and for rank=3 precoder there are 3 antenna ports on which the UE transmits PUSCH with non-zero power. There shouldn’t be any mis-interpretation of the current specification regarding the maximum output power for any rank values with UL full power transmission mode 0.

7.1 Physical uplink shared channel

For a PUSCH transmission on active UL BWP  b, as described in clause 12, of carrier f of serving cell c, a UE first calculates a linear value PPUSCH,b,f,c(i,j,qd,l) of the transmit power PPUSCH,b,f,c(i,j,qd,l), with parameters as defined in clause 7.1.1. For a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook',

-	if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where:

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2,

-	s=1 for full power TPMIs reported by the UE [16, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with an SRS resource indicated by an SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook',

-	s=1, if an SRS resource with a single port is indicated by an SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1

-	else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource.

The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.

RAN1 would like ask RAN4 to take note of the answers above.

2	Actions

To RAN4

ACTION: 	RAN1 respectfully asks RAN4 to take above answers in their work.

3	Dates of next TSG RAN WG 1 meetings

3GPP RAN1#120       		        17 – 21 February 2025                     Athens, Greece

3GPP RAN1#120bis       		  07 – 11 April 2025                           China