3GPP TSG-RAN WG1 Meeting #119  	 R1-2409660

Orlando, US, November 18th– 22th, 2024

Agenda item:       5

Source:	vivo

Title:	Discussion on applicable functionality reporting for AI/ML beam management UE-sided model

Document for:	Discussion and Decision

- Introduction

RAN1 received an LS from RAN2 regarding applicable functionality reporting for a UE-sided beam prediction model (R2-2407848/R1-2407604). In the LS, RAN2 mentioned the following,

RAN2 agreed the following understandings on terminologies: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference. RAN2 further has made following agreements and signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. RAN2 also agreed the applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality. The above agreements were made based on the following assumptions: NW-side additional condition is assumed as associated ID in RAN2 (which is assumed by majority of companies).

- Discussion on inference procedure options on Question 4

RAN2 is asking the following questions to RAN1:

On NW-side additional condition and configuration

- Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5):

  - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

  - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities?

  - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

  - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

In RAN#118bis meeting, the following agreement was achieved,

Agreement (RAN1#118bis)

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

Considering interaction of the Associated ID in RAN2 LS [1], we will initially analyze above 3 potential interaction processes of the Associated ID combining RAN1 and RAN2 agreements.

Figure 1: interactive processing of associated ID for option 1

For option 1:

- Step 1: NW sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- Step 3:  Following configurations are provided from NW to UE:

  - NW configures one or more CSI-ReportConfig incluing assossicated ID to UE as inference configuration

- Step 4: UE reports applicable functionality:

  - UE reports one or more supported CSI-ReportConfig based on Step 3

- Step 5:

  - If Step4 is carried by RRC reconfiguration complete signaling, Step 5 is not necessary.

  - If Step4 is carried by UAI, Step 5 is needed.

Based on the aforementioned process, it is evident that option 1 involves configuring multiple inference configurations based on the CSI framework in Step 3, with the UE providing feedback on the supported inference configurations in Step 4. However, even with a periodic CSI report configured in Step 3, resource measurement and report feedback cannot be executed after receving the configuration which is contrary to the existing CSI framework.

Additionally, the lack of prior associated ID interaction with direct inference configuration in step3 from NW may result in amount of invalid inference configurations and leading to a siginicant wastage of NW signalling, communication resources and UE storage resources. For example, in situation 1, the UE receives the configuration of a local associated ID, but the cell is not a known cell; In situation 2, even if the UE can confirm that the cell is a known cell, the received local associated ID is not trained by the UE during training phase. For both of these situations, the network's configuration of the inference configuration in step 3 will be futile. In addition, UE needs to waste its precious on-chip storage resources to store all these configuration parameters which may not be used at all.

- Option 1 has much specification impact on CSI framework compared to other options. Even with a periodic CSI report configured in Step 3, resource measurement and report feedback cannot be executed after receving the configuration which is contrary to the existing CSI framework.

- Due to the lack of prior associated ID interaction resulting in amount of invalid inference configurations, option 1 may lead to a significant wastage of NW signaling, communication resources and UE storage resources.

Figure 2: interactive processing of associated ID for option 2

For option 2:

- Step 1: NW sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- Step 3:  Following configurations are provided from NW to UE:

  - NW enables UE to report its supported associated IDs in UAI

  - NW indicates supported/available associated ID(s) to UE

  - NW indicates supported/preferred Set B pattern(s)

- Step 4: UE reports applicable functionality:

  - UE reports supported/available associated ID(s) based on Step 3

- Step 5: inference configuration from NW to UE together with the associated ID to be used:

  - E.g. Report configuration, resource configuration, associated configuration

Figure 3: interactive processing of associated ID for option 3

For option 3:

- Step 1: NW sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- Step 3:  Following configurations are provided from NW to UE:

  - NW enables UE to report its supported associated IDs in UAI

  - Other filtering indications, e.g. beam prediction with L1-RSRP

- Step 4: UE reports applicable functionality:

  - UE reports supported/available associated ID(s) + assistance information

- Step 5: inference configuration from NW to UE together with the associated ID to be used:

  - E.g. Report configuration, resource configuration, associated configuration

The issues mentioned in Option 1 do not exist in Option 2 and Option 3. For options 2 and 3, if it is periodic CSI report, UE can direct perform inference after receiving the CSI report configuration in step 5, which is aligned with the current CSI framework. Further, the interaction of applicable inference configuration parameters and applicable associated IDs in step 3 and step 4 allows NW to have full understanding of what models and configurations for AI can be supported by UE currently. This will avoid NW make a useless CSI report configuration. Hence we support to prioritize option 2 and option 3.

Prioritize Option 2 and Option 3 in applicable functionality determination procedure.

Comparing option 2 and option 3, the difference is whether gNB firstly provides its candidate inference configuration parameter and associated ID list in Step 3. Option 2 provides such information in step 3, while Option 3 does not have it. The possible motivation of this information in step 3 is to reduce the signaling overhead for UE UAI reporting in Step 4, where the number of associated IDs which has been trained and ready to be used by UE can be large.

Based on the processes outlined in the aforementioned options, the associated ID must undergo interaction prior to the inference configuration. Therefore, the Associated ID not only needs to be configured within the CSI framework, but also requires configuration before the inference configuration to address the consistency issue. We, thus, propose,

RAN1 should answer questions of Q4 as follows.

- Answer to Q4-1: It is not feasible for UE to decide the applicable functionalities without NW-side additional condition. RAN1 has determined that ensuring the consistency of NW-side additional conditions across training and inference for UE-sided models is crucial. Failure to do so may result in unpredictable and significant performance degradation. Whether the functionality at UE side is applicable or not is dependent at least on the provided associated ID. Therefore, it is not feasible for the UE to determine the applicable functionalities without NW-side providing associated IDs first.

- Answer to Q4-2: There can be some other information to be transmitted in step 3 to facilitate the whole procedure, but it is not inference configuration. The detailed information is up to RAN1 discussion, e.g., the interested/preferred set B patterns and/or other inference parameters from NW perspective.

- Answer to Q4-3: Associated IDs in step 3 are separate from inference configuration.

- Answer to Q4-4: The detailed information is still under  RAN1 discussion, e.g., the interested/preferred set B patterns and/or other inference parameters from NW perspective can be configured, but it is not inference configuration.

- Conclusions

In this contribution, we have the following proposals:

- Prioritize Option 2 and Option 3 in applicable functionality determination procedure.

RAN1 should answer questions of Q4 as follows.

- Answer to Q4-1: It is not feasible for UE to decide the applicable functionalities without NW-side additional condition. RAN1 has determined that ensuring the consistency of NW-side additional conditions across training and inference for UE-sided models is crucial. Failure to do so may result in unpredictable and significant performance degradation. Whether the functionality at UE side is applicable or not is dependent at least on the provided associated ID. Therefore, it is not feasible for the UE to determine the applicable functionalities without NW-side providing associated IDs first.

- Answer to Q4-2: There can be some other information to be transmitted in step 3 to facilitate the whole procedure, but it is not inference configuration. The detailed information is up to RAN1 discussion, e.g., the interested/preferred set B patterns and/or other inference parameters from NW perspective.

- Answer to Q4-3: Associated IDs in step 3 are separate from inference configuration.

- Answer to Q4-4: The detailed information is still under  RAN1 discussion, e.g., the interested/preferred set B patterns and/or other inference parameters from NW perspective can be configured, but it is not inference configuration.

- References

- Moderator (Intel), R1-2407604, “LS on applicable functionality reporting for beam management UE-sided model”, Aug. 2024