3GPP TSG RAN WG1 #119	                                           R1-2409697

Orlando, Florida, USA, 18th – 22nd November, 2024

Source:	vivo

Title:	Discussion on enabling data transmissions for XR during RRM measurements

Agenda Item:	9.10.1

Document for:	Discussion and Decision

- Introduction

In RAN1#118 meeting, great process was achieved regarding enabling transmission/reception in gaps/restrictions that are caused by RRM measurements in RAN1 and the following agreements and working assumption were made [1].

Agreement For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select one among the following options: Option 1: Support Alt. 1-1: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. Option 2: Support Alt. 3-1: - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

In RAN1#118bis meeting, the following agreements were made [2].

Agreement At least for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2. - FFS: Cross carrier scheduling. - FFS: DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. Agreement In case of cross carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell. Agreement Clarify the note from previous agreement as follows: - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. Agreement From RAN1 perspective, the case where an occasion(s) of gap/restrictions that is caused by RRM measurements is cancelled/skipped partially is not supported in Release 19.

In this contribution, we will discuss some remaining issues about the enhancements to enable transmissions/receptions in gaps/restrictions.

- Discussion

  - For different alternatives to enable transmissions/receptions in gaps/restrictions, it was concluded as working assumption in RAN1#118 meeting that Alt.1 is selected with an LS sent to RAN4 to inform them of the working assumption and ask them if there is any issue with it.

  - Interpretation on the indication of value ‘1’ and ‘0’ in a DCI

  - One important open issue for Alt 1 is how to interpret the indication of value ‘1’ and ‘0’ in a DCI. This issue was extensively discussed during the last meeting. Possible options are listed as following.

For explicit indication by 1 bit in a DCI to skip a particular gap/restriction occasion:

- The bit equal to 1 indicates the corresponding gap/restriction occasion is to be “skipped”.

- Option 1: The bit equal to 0 indicates that UE behaviour for the corresponding gap/restriction occasion is the same as legacy, i.e., the corresponding gap/restriction occasion is “not skipped”.

- Option 2: When the bit is equal to 0, the UE ignores the indication of this bit.

  - For Option 1, it can be further divided into following Option 1-1 and Option 1-2

- Option 1-1: Once a UE receives ‘1’ in a DCI for a gap/restriction occasion, the UE can receive ‘0’ in a later DCI for the same gap/restriction occasion and the UE follows the indication in the latest DCI for the gap/restriction occasion.

- Option 1-2: Once a UE receives ‘1’ in a DCI for a gap/restriction occasion, the UE does not expect to receive ‘0’ in a later DCI for the same gap/restriction occasion, that is, later DCI(s) for the same gap/restriction occasion, if any, should always indicate ‘1’.

Figure 1 Example for different options of interpretation on indication of ‘1’ and ’0’

Figure 1 shows the differences among these options. For Option 1-2, once the UE receives a DCI indicating ‘1’ for a gap/restriction occasion, the UE does not expect to receive ‘0’ in a later DCI for the same gap/restriction occasion. That means, the gNB can’t change its indication from ‘1’ to ‘0’ freely. For Option 1-1 and Option 2, there is no restriction on gNB’s indication, e.g., the gNB can indicate ‘0’ after indicating ‘1’ for the same gap/restriction occasion. The later ‘0’ is useful to change status of the gap/restriction occasion from ‘skipped’ to ‘not skipped’ for Option 1-1, but is meaningless for Option 2 since the UE will ignore such indication.

Option 1-1 can afford the most flexibility for the gNB. However, whether this flexibility is necessary or beneficial is debatable. For skipping a gap/restriction occasion, the typical use case is that the gNB knows there is data to be transmitted within the gap/restriction occasion based arrival of packet(s) or gNB’s predication based on statistical characteristics of XR traffic. The gNB would not indicate the UE to skip a gap/restriction occasion if the gap/restriction occasion is very far away from the DCI since the gNB cannot make a proper decision at this time. In contrast, generally the gNB will indicate the UE to skip a gap/restriction occasion at a time near the gap/restriction occasion, when the gNB has full knowledge to make a relative reliable decision on indicating the UE to skip the gap/restriction occasion. In this case, the gNB may very probably have no time to change its decision after the first indication. In addition, changing ‘1’ to ‘0’ means that the gNB thought some data would be transmitted within the gap/restriction occasion, but later, the gNB finds that there is no data to be transmitted. This case needs further discussion.

Once main concern for Option 1-1 is feasibility and complexity in UE implementation. Some companies think that the UE always needs to wait for the latest DCI for a gap/restriction occasion, and then acts based on the latest DCI. But this is not aligned with conventional UE implementation in practical. Generally, when a UE receives a DCI, the UE may not wait until the deadline to start its processing corresponding to this DCI. On the contrary, the UE can start the processing at any time after the UE receives this DCI. The timelines defined in NR specification is to make sure that the UE have enough time to finish the processing. However, it does not mean that the UE needs to check and wait until the timeline is exactly met. Generally, a UE’s processing is in time order. Once the UE receives a DCI indicating ‘1’, the UE will check which gap/restriction occasion is indicated based on the reception time and time offset, then acts as the indicated gap/restriction occasion is ‘skipped’. For example, for the RRM module, the UE can re-arrange its schedule of RRM measurement. For the data transmission module, if there are configured transmissions such as SPS HARQ-ACK PUCCH/CG PUSCH in the gap/restriction occasion, the UE can start the preparations for such PUCCH/PUSCH. If the UE receives a DCI indicating ‘0’ for the same gap/restriction occasion later, it means for the RRM module, the UE needs to re-arrange RRM measurement schedule and may need to prepare to perform RRM measurement in the gap/restriction occasion (note that in the worst case, if the UE does not perform RRM measurement in the gap/restriction occasion, the UE may not be able to meet the corresponding requirements). Obviously, it is not friendly for the UE to frequently re-arrange its schedule of RRM measurement. For the data module, the UE can’t transmit PUCCH/PUSCH in the gap/restriction occasion, so the UE needs to cancel its preparation for the PUCCH/PUSCH transmission. This is similar to cancelling a PUSCH via UL CI whose timeline is longer than PUSCH preparation timeline of Tproc, 2 and is based on UE capability. This is why the UE can easily handle the indications of ‘0’, ‘0’ for a same gap/restriction occasion in two consecutive DCIs, but may be difficult to handle the indications from ‘1’ to ‘0’ in these two DCIs. In addition, since the UE needs to follow the latest DCI among the DCIs indicating the same gap/restriction occasion, one issue is how to figure out the latest DCI.

Option 1-2 and Option 2 are similar. Both are simpler from UE implementation perspective. Option 1-2 may be beneficial to improving the robustness of DCI indication, with the restriction of consistent indications. On the contrary, Option 2 has no restriction on gNB’s indication, however, new interpretation behavior would be introduced, i.e. ignoring the indication when its value is ‘0’, which is somewhat strange and not consistent with conventional behavior with DCI indication.

Table 1 Comparison among different options

|  |  | Option 1-1 | Option 1-2 | Option 2 |
| --- | --- | --- | --- | --- |
| gNB indication |  | No any restriction. Can change from ‘1’ to ‘0’ | Once the gNB indicates ‘1’ for a gap/restriction occasion, it is not allowed to indicate ‘0’ for the same occasion | No any restriction. Can change from ‘1’ to ‘0’ |
| UE implementation | Valid DCI | Follow the latest DCI. How to determine the latest DCI needs to be defined | Follow the DCI indicating ‘1’ for a gap/restriction occasion | Follow the DCI indicating ‘1’ for a gap/restriction occasion |
|  | Case ‘00’ | Same as legacy. |  |  |
|  | Case ‘01’ | After receiving ‘0’, no action compared to legacy; after receiving ‘1’, the UE may start to prepare UL transmission and re-arrange its schedule for RRM measurement. |  |  |
|  | Case ‘10’ | After receiving ‘1’, the UE may start to prepare UL transmission and re-arrange its schedule for RRM measurement. After receiving ‘0’, the UE may need to cancel its ongoing preparation of UL transmission and re-arrange its schedule for RRM measurement again. | After receiving ‘1’, the UE may start to prepare UL transmission and re-arrange its schedule for RRM measurement. Since no ‘0’ is expected to be received by the UE, this case is invalid and the UE just continues its processing. | After receiving ‘1’, the UE may start to prepare UL transmission and re-arrange its schedule for RRM measurement. After receiving ‘0’, the UE ignores the indication so that the UE can continue its processing. |

Based on the above analysis, the following proposal is made.

Proposal 1: For explicit indication by a DCI to skip a particular gap/restriction occasion, the following option can be supported.

- Bit equal to 1 indicates the corresponding gap/restriction occasion is to be “skipped”.

- Bit equal to 0 indicates that UE behaviour for the corresponding gap/restriction occasion is the same as legacy.

- Once the UE receives ‘1’ in a DCI for a gap/restriction occasion, the UE does not expect to receive ‘0’ in a later DCI for the same gap/restriction occasion.

Support of DCI formats 0_3/1_3

For Alt. 1, the indication is included as part of a scheduling DCI and the bit-field size is one bit. One remaining issue regarding DCI format is whether DCI formats 0_3/1_3 are supported or not. In our view, the necessity to support DCI format x_3 is not clear. In addition, to support DCI format x_3 with 1-bit indication, there are some remaining issues that need to be discussed. For DCI formats 0_1/1_1 and 0_2/1_2, it was agreed that one-bit indication included as a part of a DCI format corresponds to a scheduled cell. However, for DCI format x_3, there are multiple scheduled cells, thus one issue arises, i.e. which cell the 1-bit indication corresponds to. Note that gap/scheduling configurations on different serving cells may be different. As shown in following Figure 2, where gap1 and gap2 are per FR gaps, gap1 applies to serving cell 1 in FR1, and gap2 applies to serving cell 2 in FR2, if the 1-bit indication corresponds to all scheduled cells as some companies proposed during the last meeting, if the gNB wants to skip gap1 corresponding to serving cell 1, the gNB can send a DCI indicating ‘to be skipped’ by the field, but unfortunately gap2 corresponding to serving cell 2 is also indicated as ‘to be skipped’ due to gap 1 and gap 2 share the same indication filed. Note that gap 2 can be very far away from the DCI and gNB may be unable to make accurate decision regarding whether UE needs to skip gap 2. If consistent indication is adopted, gNB would have no chance to change its indication for gap 2 as ‘not to be skipped’ any more. Support DCI format x_3 with such 1-bit indication will complicate the design and is not preferred.

Figure 2 example for gap/scheduling occasion skipping with DCI format x_3

Proposal 2: To enable Tx/Rx in gaps/restrictions that are caused by RRM measurements with Alt. 1, DCI formats 0_3 and 1_3 are not supported.

  - Considering more than one DCI format is supported with such one-bit indication, it was agreed that explicit indication can be configured for each DCI format individually by higher layer. New RRC parameter e.g., kipGapRestrictionDCI-1-1 and kipGapRestrictionDCI-1-2 can be added in PDSCH-Config for DCI format 1_1 and 1_2, respectively. For DCI format 0_1 or DCI 0_2, corresponding parameters can be added in PUSCH-Config. When the parameter is set to ‘enabled’, it means there is one-bit indication for enabling transmissions/receptions in gaps/restrictions in the corresponding DCI format.

Proposal 3: To enable Tx/Rx in gaps/restrictions that are caused by RRM measurements with Alt. 1, new RRC parameter to configure the presence of the one-bit indication in different DCI formats are added in IE PDSCH-Config and PUSCH-Config.

Whether a scheduling DCI with indication has to schedule a PDSCH reception or not

  - Another issue is whether the scheduling DCI has to schedule a PDSCH reception or not. Considering that the main use case for this feature of enabling transmissions/receptions in gaps/restrictions is when the gNB is aware of emergency demand for traffic delivery, it means that typically there is traffic to be transmitted when the gNB sends a DCI to indicate the UE to skip a particular gap/restriction occasion. Currently, for several cases, e.g. SCell dormancy indication and Type-3 HARQ-ACK codebook triggering, a DL scheduling DCI with invalid FDRA is supported and HARQ-ACK for the DCI without scheduling PDSCH reception are specified. Considering the marginal specification impact and more flexibility, it is slightly preferred that the DL scheduling DCI can indicate invalid FDRA.

Proposal 4: For a DCI indicating to skip a particular gap/restriction occasion, the DCI can indicate invalid FDRA without scheduling a PDSCH reception.

  - Clarification on UE behaviour for skipping a gap/restriction occasion

In the last meeting, it was agreed that for self-scheduling case, for explicit indication by DCI to skip a particular gap/restriction occasion, one bit indication is included as part of DCI formats 0_1/1_1 and 0_2/1_2 and in case of cross-carrier scheduling, for explicit indication by DCI to skip a particular gap/restriction occasion, one bit indication included as a part of DCI formats 0_1/1_1 and 0_2/1_2 corresponds to a scheduled cell. In addition, in the first meeting of the XR Rel-19 WI, it was agreed that from RAN1 perspective, when an occasion(s) of gaps/restrictions that are caused by RRM measurements is cancelled/skipped fully, the UE is assumed to receive/transmit in the gaps/restrictions that are caused by RRM measurements as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements.

Agreement (in RAN1#116)

From RAN1 perspective, when an occasion(s) of gaps/restrictions that are caused by RRM measurements are cancelled/skipped fully, UE is assumed to receive/transmit in the gaps/restrictions that are caused by RRM measurements as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements.

- FFS: Whether or not/How to support of the case where an occasion(s) of gap/restrictions that are caused by RRM measurements are cancelled/skipped partially

  - Note that measurement gaps can be configured per UE or per UE per frequency range, and serving cells for which there are restrictions on reception/transmission in a gap occasion are defined in TS 38.133 which is cited as below. For example, for SA scenario, for per-UE measurement gap, there are restrictions on reception/transmission on all serving cells of the UE in a per-UE gap occasion. For per-FR gap, e.g. measurement gap for FR1, there are restrictions on reception/transmission on all FR1 serving cells of the UE in a per-FR gap occasion.

  - For scheduling restrictions due to L1/L3 measurements, scheduling restrictions are introduced for a given serving cell (or a given serving cell frequency layer), however it may also apply to other serving cells (or other serving cell frequency layers), based on specification of TS 38.133.

TS 38.133 v18.7.0 2024-09 ### 9.1.2 Measurement gap During the per-UE measurement gaps the UE: - is not required to conduct reception/transmission from/to the corresponding E-UTRAN PCell, E-UTRAN SCell(s) and NR serving cells for E-UTRA-NR dual connectivity except the reception of signals used for RRM measurement(s) and the signals used for random access procedure according to TS38.321 [7]. - is not required to conduct reception/transmission from/to the corresponding NR serving cells for SA (with single carrier or CA configured) except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7]. - is not required to conduct reception/transmission from/to the corresponding PCell, SCell(s) and E-UTRAN serving cells for NR-E-UTRA dual connectivity except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7]. - is not required to conduct reception/transmission from/to the corresponding NR serving cells for NR-DC except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7]. During the per-FR measurement gaps the UE: - is not required to conduct reception/transmission from/to the corresponding E-UTRAN PCell, E-UTRAN SCell(s) and NR serving cells in the corresponding frequency range for E-UTRA-NR dual connectivity except the reception of signals used for RRM measurement(s) and the signals used for random access procedure according to TS38.321 [7]. - is not required to conduct reception/transmission from/to the corresponding NR serving cells in the corresponding frequency range for SA (with single carrier or CA configured) except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to TS38.321 [7]. - is not required to conduct reception/transmission from/to the corresponding PCell, SCell(s) and E-UTRAN serving cells in the corresponding frequency range for NR-E-UTRA dual connectivity except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to TS38.321 [7]. - is not required to conduct reception/transmission from/to the corresponding NR serving cells in the corresponding frequency range for NR-DC except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to TS38.321 [7].

Figure 3 Example for different types of gaps and affected serving cells

  - As shown in Figure 3, following legacy specification without gap skipping the UE does not need to conduct reception/transmission on all serving cells in gap1 and does not need to conduct reception/transmission on serving cell 0 and serving cell 1 in gap2. Following RAN1 latest agreement, the DCI on serving cell 0 and scheduling a PDSCH on serving cell 1 can be used to indicate the UE to skip a gap/restriction occasion corresponding to the scheduled cell, i.e., serving cell 1, that is, gap 1 is indicated to be skipped by the DCI in Figure 1. If the DCI indicates ‘1’, it means the UE will fully skip gap1. As a result, the UE is assumed to receive/transmit in gap1 as it would without gap1. In other words, the UE shall conduct reception/transmission in gap1 on all serving cells affected by gap1, i.e. serving cell 0~2 as there is actually no gap.

  - Proposal 5: Clarify that once a gap/restriction occasion is indicated as to be skipped by a DCI, the UE shall conduct reception/transmission in the gap/restriction occasion on all serving cells affected by the gap/restriction occasion as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements.

- The serving cells affected by the gap/restriction occasion is defined in TS 38.133 as legacy.

Multiple concurrent measurement gap configurations

  - Currently, for a UE, it can be configured with multiple concurrent measurement gap configurations. When the UE is configured with concurrent measurement gaps, two measurement gap occasions are considered colliding if at least one of the following conditions is met:

- the two occasions are fully or partially overlapping in time domain, or

- the distance between the two occasions is equal to or smaller than [4]ms.

In case of collision between two occasions based on two measurement gap configurations with different priorities, the UE shall perform measurements in the occasion corresponding to the measurement gap configuration with higher priority, and the occasion corresponding to the measurement gap configuration with lower priority is considered to be dropped.

In a case with multiple concurrent measurement gap configurations, the UE can handle collision among multiple measurement gap occasions as legacy firstly, and then determine the occasion indicated by a DCI as shown in Figure 1. In other words, the bit in the DCI is used to indicate a survived measurement gap occasion after collision handling.

Figure 4 Example for DCI indication in a case with concurrent measurement gap configurations

Proposal 6: When a UE is configured with multiple concurrent measurement gap configurations, if there are collided measurement gap occasions, the UE first solves collision among different measurement gap occasions as legacy, and DCI indication to enable Tx/Rx in a particular gap/restriction occasion that are caused by RRM measurements is applied to a survived measurement gap occasion after collision handling.

Clarification of applicable gap/restriction occasions

In addition, in RAN1#116 meeting, it was agreed as WA that RAN1 aims to develop/identify solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements agnostic in RAN1 normative work to types of gaps/restrictions that are caused by RRM measurements. RAN4 is discussing which types of gaps/restrictions can be skipped and whether UAI is supported to indicate which gap/restriction occasions can be skipped. It can be understood that if RAN4 decides that only some particular type(s) of gaps/restrictions can support this feature of enabling transmissions/receptions in gaps/restrictions, e.g. only measurement gaps used for L3 measurement, the DCI indication should only apply to gaps/restrictions of these particular type(s). For a particular gap/restriction configuration, if only a subset of occasions for the configuration can be skipped, e.g. a pattern of protected/non-skippable gap/restriction occasions is configurated by the gNB, it can be understood that the DCI should only apply to the measurement gap/restriction occasions that can be skipped. Our understanding on the operation of DCI indicating to skip a particular gap/restriction occasion is illustrated in Figure 2.

Figure 5 Example for the operation of DCI indicating to skip a gap/restriction occasion

Proposal 7: For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements, the bit in a DCI is used to indicate whether to skip the first skippable gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

- How to determine whether an occasion is skippable or not is up to RAN4

- Conclusion

In this contribution, some remaining issues about enhancements to enable transmissions/receptions in gaps/restrictions are discussed, with the following proposals:

Proposal 1: For explicit indication by a DCI to skip a particular gap/restriction occasion, the following option can be supported.

- Bit equal to 1 indicates the corresponding gap/restriction occasion is to be “skipped”.

- Bit equal to 0 indicates that UE behaviour for the corresponding gap/restriction occasion is the same as legacy.

- Once the UE receives ‘1’ in a DCI for a gap/restriction occasion, the UE does not expect to receive ‘0’ in a later DCI for the same gap/restriction occasion.

Proposal 2: To enable Tx/Rx in gaps/restrictions that are caused by RRM measurements with Alt. 1, DCI formats 0_3 and 1_3 are not supported.

Proposal 3: To enable Tx/Rx in gaps/restrictions that are caused by RRM measurements with Alt. 1, new RRC parameter to configure the presence of the one-bit indication in different DCI formats are added in IE PDSCH-Config and PUSCH-Config.

Proposal 4: For a DCI indicating to skip a particular gap/restriction occasion, the DCI can indicate invalid FDRA without scheduling a PDSCH reception.

  - Proposal 5: Clarify that once a gap/restriction occasion is indicated as to be skipped by a DCI, the UE shall conduct reception/transmission in the gap/restriction occasion on all serving cells affected by the gap/restriction occasion as it would without any (measurement etc. related) gaps/restrictions that are caused by RRM measurements.

- The serving cells affected by the gap/restriction occasion is defined in TS 38.133 as legacy.

Proposal 6: When a UE is configured with multiple concurrent measurement gap configurations, if there are collided measurement gap occasions, the UE first solves collision among different measurement gap occasions as legacy, and DCI indication to enable Tx/Rx in a particular gap/restriction occasion that are caused by RRM measurements is applied to a survived measurement gap occasion after collision handling.

Proposal 7: For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements, the bit in a DCI is used to indicate whether to skip the first skippable gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

- How to determine whether an occasion is skippable or not is up to RAN4

# References

- Chair’s notes RAN1#118 meeting, Maastricht, Netherland, August 19 – 23, 2024

- Chair’s notes RAN1#118bis meeting, Hefei, China, October 14 – 18, 2024