3GPP TSG RAN WG1 #119                                                                                  R1-2409705

Orlando, US, November 18th – 22nd, 2024

Source:	vivo

Title:	Discussion on E-UTRAN measurement in IoT-NTN

Agenda Item:	8.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the following:

- Clarifications on the reference point for E-UTRAN measurement abilities for IoT-NTN

- Discussion

TS 36.108 SAN type 1-H: Satellite Access Node operating at FR1 with a requirement set consisting of conducted requirements defined at individual TAB connectors and OTA requirements defined at RIB. SAN type 1-O: Satellite Access Node operating at FR1 with a requirement set consisting only of OTA requirements defined at the RIB. ====omitted==== 4.3 Requirement reference points - 4.3.1 SAN type 1-H For SAN type 1-H, the requirements are defined for two points of reference, signified by radiated requirements and conducted requirements. Figure 4.3.1-1: Radiated and conducted reference points for SAN type 1-H Radiated characteristics are defined over the air (OTA), where the radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. ====omitted==== How a conducted requirement is applied to the transceiver array boundary is detailed in the respective requirement clause. - 4.3.2 SAN type 1-O For SAN type 1-O, the radiated characteristics are defined over the air (OTA), where the operating band specific radiated interface is referred to as the Radiated Interface Boundary (RIB). Radiated requirements are also referred to as OTA requirements. The (spatial) characteristics in which the OTA requirements apply are detailed for each requirement. Figure 4.3.2-1: Radiated reference points for SAN type 1-O

According to the specification in RAN4, the access node used to provide E-UTRA user plane and control plane protocol terminations towards IoT NTN Satellite capable UE is called SAN, instead of BS. The related functions and requirements for SAN are defined in a separate specification, i.e., TS 36.108. In other words, TS 36.104 and TS 37.105 addresses the functions and requirements for base stations under TN case only.

For TN, the reference point for E-UTRAN measurement (including DL RS TX power, Received Interference Power, Thermal noise power, eNB Rx – Tx time difference, E-UTRAN GNSS Timing of Cell Frames for UE positioning, and TUL-RTOA) is the TX or RX antenna connector, whereas "antenna connector" refers to the "BS antenna connector" test port A and test port B of non-AAS base station as described in TS 36.104; the reference point for E-UTRAN measurement (including eNB Rx – Tx time difference) is TX/Rx antenna and the TX/Rx Transceiver Array Boundary connector for OTA AAS base station and Hybrid AAS base station as described in TS 37.105 respectively.

TS 36.214 [10] 3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception ". 5.2 E-UTRAN measurement abilities The structure of the table defining a E-UTRAN measurement quantity is shown below. Column field; Comment. Definition; Contains the definition of the measurement.. The term "antenna connector" used in this clause to define the reference point for the E-UTRAN measurements refers to the "BS antenna connector" test port A and test port B as described in [10]. The term "antenna connector" refers to Rx or Tx antenna connector as described in the respective measurement definitions. - 5.2.1 DL RS TX power Definition; Downlink reference signal transmit power is determined for a considered cell as the linear average over the power contributions (in [W]) of the resource elements that carry cell-specific reference signals which are transmitted by the eNode B within its operating system bandwidth. For DL RS TX power determination the cell-specific reference signals R0 and if available R1 according TS 36.211 [3] can be used. The reference point for the DL RS TX power measurement shall be the TX antenna connector.. - 5.2.5 eNB Rx – Tx time difference Definition; The eNB Rx – Tx time difference is defined as T eNB-RX – TeNB-TX Where: T eNB-RX is the eNB received timing of uplink radio frame #i, defined by the first detected path in time. The reference point for TeNB-RX shall be: - for non-AAS base station [10,TS 36.104]: the Rx antenna connector, - for OTA AAS base station [19,TS 37.105]: the Rx antenna, - for Hybrid AAS base station [19,TS 37.105]: the Rx Transceiver Array Boundary connector. T eNB-TX is the eNB transmit timing of downlink radio frame #i. The reference point for TeNB-TX shall be: - for non-AAS base station [10,TS 36.104]: the Tx antenna connector, - for OTA AAS base station [19,TS 37.105]: the Tx antenna, - for Hybrid AAS base station [19,TS 37.105]: the Tx Transceiver Array Boundary connector.. TS 36.104 6.1 General Unless otherwise stated, the requirements in clause 6 are expressed for a single transmitter antenna connector. In case of multi-carrier transmission with one or multiple transmitter antenna connectors, transmit diversity or MIMO transmission, the requirements apply for each transmitter antenna connector. Unless otherwise stated, the transmitter characteristics are specified at the BS antenna connector (test port A) with a full complement of transceivers for the configuration in normal operating conditions. If any external apparatus such as a TX amplifier, a filter or the combination of such devices is used, requirements apply at the far end antenna connector (port B). Unless otherwise stated the requirements in clause 6 applies at all times, i.e. during the Transmitter ON period, the Transmitter OFF period and the Transmitter transient period. Unless otherwise stated the requirements for NB-IoT in clause 6 applies for all operation modes (In-band operation, Guard-band operation and Stand-alone operation). Figure 6.1-1: Transmitter test ports TS 37.105 4.3 Conducted and radiated requirement reference points AAS BS requirements are defined for two points of reference, signified by radiated requirements and conducted requirements. Figure 4.3-1: Radiated and conducted points of reference of hybrid AAS BS Figure 4.3-2: Radiated points of reference of OTA AAS BS

However, the definition of the reference point of E-UTRAN measurement in TS 36.214 does not reference TS 36.108 when the E-UTRAN measurement is conducted for IoT NTN. Consequently, how the E-UTRAN measurement is defined for IoT NTN is not complete. For example, the determination of DL RS TX power in IoT NTN is not clearly described as there is no text on TX antenna connector for SAN.

In addition, according to Figure 4.3.1-1 and Figure 4.3.2-1 of the SAN structure in TS 36.104, it is unclear if test port A and test port B still exist in SAN. Although SAN type 1-O and 1-H look similar to OTA AAS base station and Hybrid AAS base station, respectively, it is unclear whether the TX/Rx antenna and the TX/Rx Transceiver Array Boundary connector can be defined as the reference point for SAN type 1-O and 1-H for IoT NTN case.

Given this situation, RAN1 may need to first clarify which measurement metric defined in 36.215 is supported for IoT NTN(including eMTC NTN and NB IoT NTN). As how to define the reference point is a RAN4 issue, we suggest sending a LS to RAN4 to directly request clarification on the reference point for IoT NTN E-UTRAN measurement.

Observation 1: Requirements for SAN providing E-UTRAN user plane and control plane protocol terminations towards NTN satellite capable UE is defined in 36.108. In the E-UTRAN measurement abilities definition in 36.214, the reference point for SAN performing E-UTRAN measurement in the IoT NTN case is not defined.

Observation 2: Reference point for E-UTRAN measurement(including DL RS TX power, Received Interference Power, Thermal noise power, eNB Rx – Tx time difference, E-UTRAN GNSS Timing of Cell Frames for UE positioning, and TUL-RTOA) in TN is the TX or RX antenna connector for non-AAS base station, whereas "antenna connector" refers to the "BS antenna connector" test port A and test port B as described in TS 36.104, however, whether test port A and test port B exist in SAN type 1-O and 1-H for IoT NTN case is not clear.

Observation 3: Reference point for E-UTRAN measurement(i.e., eNB Rx – Tx time difference)  in TN is TX/Rx antenna and the TX/Rx Transceiver Array Boundary connector for OTA AAS base station and Hybrid AAS base station as described in TS 37.105 respectively. However, whether the TX/Rx antenna and the TX/Rx Transceiver Array Boundary connector can be defined as the reference point for SAN type 1-O and 1-H for IoT NTN case is not clear.

Observation 4: It is also unclear which E-UTRAN measurement abilities defined in 36.214 is supported for IoT NTN and how to define the reference point for E-UTRAN measurement for IoT NTN.

Proposal 1: Clarify which E-UTRAN measurement metric in 36.214 is supported for IoT NTN (including eMTC NTN and NB IoT NTN).

Proposal 2: Send a LS to RAN4 to directly request clarification on the reference point for E-UTRAN measurement for IoT NTN in 36.214.

TS 36.307 3A.4 Other release independent features This clause covers requirements for a UE coming from all other release independent features that are not covered under clause 3A.1, 3A.2 and 3A.3, e.g. generic baseband requirements or requirements that are not band/CA configuration specific. Table 3A.4-1: Additional requirements of other release independent features Feature; Release independent from; Requirements to be fulfilled (see 36.307 of the REL when the feature was introduced); Further information. RF, RRM and demodulation requirements for NB-IoT standalone operation over NTN; Rel-17; Table F.1-1 for UE RF requirements, Table F.2-1 for RRM requirements Table F.3-1 for UE demodulation requirements; Rel-18 WI LTE_NBIoT_eMTC_NTN_req introduced RF, RRM and demodulation requirements for NB-IoT standalone operation over NTN with bands specified in Rel-18 TS 36.102. See tables F.1-1, F.2-1, F.3-1.. RF, RRM and demodulation requirements for eMTC operation over NTN; Rel-17; Table F.1-2 for UE RF requirements, Table F.2-2 for RRM requirements Table F.3-2 for UE demodulation requirements; Rel-18 WI LTE_NBIoT_eMTC_NTN_req introduced RF, RRM and demodulation requirements for eMTC operation over NTN with bands specified in Rel-18 TS 36.102. See tables F.1-2, F.2-2, F.3-2..

It is noted that although TS 36.108/102 are introduced in R18, they are Release independent from R17.

Observation 5: TS 36.108/102 are introduced in R18, but they are Release independent from R17, therefore changes or clarifications to R17 IoT NTN may also be necessary.

Proposal 3: Clarify if changes or clarifications on reference points for E-UTRAN measurement for R17 IoT NTN are also necessary.

- Conclusion

In this contribution, we provide our views on the clarification on E-UTRAN measurement in IoT NTN, and have the following observations and proposals. The draft CRs are in section 5, 6 and 7.

Observation 1: Requirements for SAN providing E-UTRAN user plane and control plane protocol terminations towards NTN satellite capable UE is defined in 36.108. In the E-UTRAN measurement abilities definition in 36.214, the reference point for SAN performing E-UTRAN measurement in the IoT NTN case is not defined.

Observation 2: Reference point for E-UTRAN measurement(including DL RS TX power, Received Interference Power, Thermal noise power, eNB Rx – Tx time difference, E-UTRAN GNSS Timing of Cell Frames for UE positioning, and TUL-RTOA) in TN is the TX or RX antenna connector for non-AAS base station, whereas "antenna connector" refers to the "BS antenna connector" test port A and test port B as described in TS 36.104, however, whether test port A and test port B exist in SAN type 1-O and 1-H for IoT NTN case is not clear.

Observation 3: Reference point for E-UTRAN measurement(i.e., eNB Rx – Tx time difference)  in TN is TX/Rx antenna and the TX/Rx Transceiver Array Boundary connector for OTA AAS base station and Hybrid AAS base station as described in TS 37.105 respectively. However, whether the TX/Rx antenna and the TX/Rx Transceiver Array Boundary connector can be defined as the reference point for SAN type 1-O and 1-H for IoT NTN case is not clear.

Observation 4: It is also unclear which E-UTRAN measurement abilities defined in 36.214 is supported for IoT NTN and how to define the reference point for E-UTRAN measurement for IoT NTN.

Observation 5: TS 36.108/102 are introduced in R18, but they are Release independent from R17, therefore changes or clarifications to R17 IoT NTN may also be necessary.

Proposal 1: Clarify which E-UTRAN measurement metric in 36.214 is supported for IoT NTN (including eMTC NTN and NB IoT NTN).

Proposal 2: Send a LS to RAN4 to directly request clarification on the reference point for E-UTRAN measurement for IoT NTN in 36.214.

Proposal 3: Clarify if changes or clarifications on reference points for E-UTRAN measurement for R17 IoT NTN are also necessary.