3GPP TSG RAN WG1 #119											R1-2409855

Orlando, US, November 18th – 22nd, 2024

Agenda item:	9.1.1

Source:	NEC

Title: 	Discussion on specification support for beam management

Document for:	Discussion and Decision

- Introduction

Rel-19 Work Item (WI) is targeting to provide specification support for Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model [2]. In the last meeting [3], the following agreements have been achieved. In this contribution, we provide our views on AI/ML beam management use case. More specifically, in Section 2, the discussion is on beam management specific issues on lifecycle management (LCM), and in Section 3, the discussion is on how to utilize AI/ML results to improve beam management.

Agreement Answer to Q2 in R1-2407604 as below: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.. Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2 Agreement For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - Aperiodic (AP) CSI-RS For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - FFS: Aperiodic (AP) CSI-RS Note: above CSI-RS resource refers to that used for beam management. Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration. Agreement For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report. Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Agreement Incorporating below text into the general part of the LS In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs - The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing Agreement Answer to Q1 in R1-2407604 as below, In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI..

- Discussion on Beam Management Specific Issues on AI/ML LCM

  - Functionality, UE Capability, and Conditions

RAN2 has developed signalling procedure and sent LS [4] to ask RAN1 about the granularity of functionality. Base on their agreed procedure, in ‘Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side’, it seems the granularity of functionality can only be a sub-use case or even finer, because it is possible that UE supports BM-Case1 but not supports BM-Case2, it is difficult to report supported functionality if the granularity is a use case. However, RAN1 has reached an agreement [3] that the concept/terminology ‘functionality’ of ‘Supported functionalities’ in step 2 may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs. Therefore, we need to understand what will be included in the UECapablityInformation message. As discussed in TR[1] and previous RAN 1 meeting, the possible contents may include ‘conditions’, such as information regarding model inference, Set A / Set B configuration, performance monitoring, data collection, assistance information. For example, assuming that the Rel-19 AI/ML-specific FG is a sub-use case, the capability message may comprise signalling of detailed ‘conditions’ information such as Set A and Set B, and others (output type, e.g., whether predicted RSRP is supported, and prediction accuracy, e.g., confidence of predicted RSRP) as in the following table for an example of supporting BM-Case1. Meanwhile, for support BM-Case2, more information may be needed, such as information related to prediction window, observation window, etc.

Table 1 Definitions for parameters included in the UECapablityInformation message

spatialDomainBeamPrediction Defines support of spatial domain beam prediction (BM-Case 1). The capability comprises signalling of - Information of Set A. - Information of Set B. - Information of Set A Set B association. - Output type - Prediction accuracy

temporalDomainBeamPrediction Defines support of temporal domain beam prediction (BM-Case 2). The capability comprises signalling of - Information of Set A. - Information of Set B. - Information of Set A Set B association. - Observation/prediction window - Output type - Prediction accuracy

The UECapablityInformation message comprises signalling of detailed information of the conditions.

The information of Set A/B in above table also includes the time information of Set B measurements and Set A predictions. While Set A and Set B configurations are considered as part of the conditions associated with a functionality/model, the details need to be further investigated. For instance, timing information/periodicity of Set B measurements and Set A predictions are equally important in determining the model which shall be used for beam prediction. As an example, UE may have multiple models trained with different periodicity of Set B beams e.g., 20ms and 40ms, in such a case when the model is being configured to the UE for inference, the UE can determine which model to use based on the Set B periodicity configured by the gNB. Similarly, multiple models may be trained at UE side for different periodicity of prediction output (e.g., 20ms/40ms/80ms) and again the model configured to UE by the network should indicate that which model to use for the inference operation. The timing relation becomes even more crucial when considering temporal beam prediction.

The timing information of Set B measurements and Set A predictions is crucial for UE to determine the model to be used for inference.

The timing information of Set B measurements and Set A predictions should be specified as conditions for BM-Case1 and BM-Case2 and comprised in UE capability reporting.

The prediction accuracy in above table is referred to confidence of model inference, which is a property related to the AI/ML model itself. For regression-based model (or when the model can predict RSRP), in addition to the predicted RSRP, the confidence corresponding to the model can also be obtained. Generally, the confidence represents the degree of certainly the model has in its predictions, or it indicates the uncertainty or reliability of the predicted results. Specifically, a higher confidence level means that the predicted results are more reliable. Based on the confidence, NW can comprehensively assess the predicted results obtained during model inference, for instance, trade-off between the predicted RSRP and the confidence. Therefore, as one of information regarding model inference, during the RAN2 agreed procedure, confidence information can also be included in the UECapablityInformation message.

To our understanding, the confidence is usually related to the range of prediction errors. Specifically, in case of beam prediction, the confidence should be defined as a confidence interval or prediction interval associated with predicted RSRPs at a specific confidence level (e.g., 95%). Specifically, the confidence interval or prediction interval is a range of predicted RSRP, i.e., [lower bound, upper bound]. In fact, the confidence interval or prediction interval output by the AI/ML model is essentially obtained by adding & subtracting an interval error from the predicted RSRP, e.g., lower bound = predicted RSRP – interval error, upper bound = predicted RSRP + interval error. This means that the confidence interval or prediction interval can be represented by the interval error. Therefore, reporting the confidence interval or prediction interval can be achieved by reporting the interval error.

Support confidence comprised in UE capability reporting or as a condition for functionality.

During RAN1 discussion on the procedure, regarding step 3-5, 3 different options are for further study, where Option 1 is NW provides one or multiple candidate inference configurations in step 3 for UE to confirm in step 4, option 2 is NW provides one or multiple sets of inference related parameters for UE to confirm in step 4, Option 3 is NW provides configuration in step 3 to allow UE to report applicable parameters in step 4. Consider that UE has the best knowledge of its supported parameters of UE-sided model, it may be easier for UE to initiate this process, e.g., using Option 3, which can also alleviate the issue that NW may provide too many useless inference configurations as Option 2 or parameter combinations as Option 3. Therefore we have the following proposal.

Support Option 3 for applicability for inference for UE-side model.

  - Additional Conditions and Ensure the Consistency

Further, already discussed during evaluation phase, different antenna structure and cell layout can have a major impact on the performance of AI/ML model. As signalling antenna configuration information may lead to disclosure of proprietary information, it is proposed to signal this information between UE and network (e.g., during model identification, UE capability transfer or model configuration) as an abstract identifier like pattern ID which allows UE and network to have common understanding in terms of which models to enable. For instance, UE may indicate pattern ID for each model it supports during model identification or UE capability transfer, where different pattern IDs may indicate that models are trained with different antenna configurations/cell layout. The network can then configure the pattern ID value during model configuration to allow UE to select the appropriate model for inference.

Note that RAN1#118 had discussion for this issue, as shown in the introduction section. Further, a following agreement was reached for associated ID in RAN1#118.

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

If the scope of additional conditions is only restricted within a single cell, then this implies that beam management training is performed per cell which can lead to significant complications as UE may need to train numerous models for beam management operation catering to different antenna patterns, cell parameters etc. Hence, the scope of additional conditions to should be applicable to multiple cells for handling consistency between training and inference.

For the consistency of NW-side additional conditions across training and inference for UE-sided model for BM-Case 1 and BM-Case 2 based on associated ID, where the NW-side additional conditions may at least impact UE assumption on beams of Set A/Set B.

Associated ID shall at least indicate the site/cell specific variables used for model training like antenna configuration.

NW-side additional conditions with the same associated ID can be consistent across multiple cells. FFS for which cells NW-side additional conditions can be assumed consistent.

It could be quite difficult to always have this consistency because different NW/UE vendors have different implementations and algorithms for beamforming weight design. Even for one NW/UE device, it may or may not apply the same beamforming all the time. One simple solution is to restrict the association by fixing the IDs of Set B beams and Set A beams, however, different devices may have a different mapping between the beam IDs and real beamforming weights, as in the following figure, even the beam IDs of Set B beam are the same for two devices, they may correspond to different actual beams. Therefore, a reference beam pattern may be useful and it may contain the reference antenna information, beamforming weights and the mapping relationship among beamforming weights and beam IDs (or resource IDs) of Set B beams and corresponding Set A beams.

Figure 1 different beamforming weight arrangement, with same Set A/B beam IDs

Support a reference beam pattern as NW sided additional condition to ensure the consistency.

In addition to the consistency between model training and model inference, NW should provide proper configurations also for performance monitoring. The consistency/association of Set B beams and Set A beams across model training, model inference and performance monitoring is also needed. In particular, when CSI-RS resources are used as the measurement resources for model training, model inference and performance monitoring respectively, an explicit association may be needed between different CSI-RS resource configurations and/or different report configurations based on those CSI-RS resources. If the associated ID is introduced, it can be a good choice to indicate the association and it can be configured in the data collection for performance monitoring in addition to the data collection for model training and model inference. Therefore, we have the following proposal.

Support to configure the associated ID(s) also for AI/ML performance monitoring.

  - Data Collection

Typically, a well-trained model requires model testing based on field data before use, but extensive field data may not always be necessary for model testing, and the same situation may also exist for performance monitoring. For model testing and performance monitoring for NW-side model, UE needs to calculate and report the measured L1-RSRPs of beam measurement resources configured by NW. We believe that leveraging existing CSI report framework, without the need for large quantities of field data, may be advantageous. For example, collecting L1-RSRP can align as closely as possible with existing mechanism based on CSI report for reporting L1-RSRP. Furthermore, NW can more promptly access field data to evaluate model performance. Additionally, and most importantly, NW can dynamically determine, based on the field data already acquired, whether further data collection is warranted in real-time, thus avoiding unnecessary data collection. For instance, in scenarios where data collection is based on periodic or semi-persistent CSI report, NW can release or deactivate such CSI report once it determines that sufficient field data has been collected. Therefore, in addition to the data collection discussed/agreed in RAN2, we have the following proposal.

For data collection for NW-sided model, also support L1 report leveraging the existing CSI report framework.

  - Model Inference

    - Model inference at UE side

It is agreed to configure two resource sets for Set A and Set B. In the legacy PDSCH rate matching, UE assumed that the corresponding resource elements (REs) for P/SP NZP-CSI-RS are not used for PDSCH, while the corresponding REs for an aperiodic NZP-CSI-RS may be used for PDSCH under certain conditions. When P/SP NZP-CSI-RS are configured and active, the available REs for PDSCH transmission are reduced. This limitation might decrease the overall system throughput.

If Set A resources are configured within one CSI-ResourceConfigId, it means that the physical resources (time resource, frequency resource, sequence resource) are provided for Set A beams, which may not be used in model inference and can only be useful during other procedures, e.g., model training. The configured resources for Set A should be available for other channel/signal (e.g., PDSCH) during model inference.

P/SP NZP-CSI-RS are configured and active for model inference, it might impact the overall system throughput.

The configured P/SP NZP-CSI-RS resources for Set A should be available for other channel/signal (e.g., PDSCH) at least during model inference.

In legacy spec, for reporting measured beams, the reported measured beam(s) are selected based on measured L1-RSRPs, e.g., UE may report top beam(s) with the largest L1-RSRP to NW, or based on UE selection. Similarly, for reporting predicted beams, some pre-defined rules are needed. In last meeting, it is FFS on the definition of predicted Top K beam(s). Specifically, for classification based model, the reported predicted beams can be selected based on probability of being Top 1 or Top K beam higher than a threshold, e.g., the pre-defined rule can be: a sum probability of being Top 1 or Top K beam higher than a threshold. For regression based model, the reported predicted beams can be selected based on predicted L1-RSRP, e.g., the pre-defined rule can be: predicted L1-RSRP higher than a threshold.

Support selecting Top-K beam(s) according to some pre-defined rules (e.g., a sum probability of being Top 1 or Top K beam higher than a threshold, predicted L1-RSRP higher than a threshold) as the reported predicted beams.

For reporting measured beams, the value of K (i.e., the number of beams to report) is explicitly configured by NW, in other words, K is determined by NW. However, for reporting predicted beams, in some cases, the number of beams to report configured by NW may not be sufficient to find the real best beam. For instance, in AI/ML model output, when multiple predicted beams exhibit similar or close predicted L1-RSRPs or probabilities (of being Top 1 beam), theoretically, these beams all need to be reported to the NW, and then the real best beam needs to be discovered through further beam measurement and reporting. Furthermore, variations in model performance may also impact the value of K. For example, when model performance deteriorates, NW may require UE to report more predicted beams to search for the real best beam. Therefore, for model inference at UE side, the value of K (i.e., the number of predicted beams to report) should be determined and provided by to NW by UE. To provide the value of K, consideration may be given a flexible beam reporting mechanism, e.g., two parts, or it can be separately provided to NW by UE to assist NW in configuration of K.

For BM-Case1 and BM-Case2, the value of K (i.e., the number of predicted beams to report) should be determined and provided to NW by UE.

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, the following two options are agreed to further study in the last meeting:

•	Option A: Predicted RSRP

•	Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement

To our understanding, due to the possibility that Set B may be a subset of Set A, the predicted beam may simultaneously serves as a measured beam. For instance, for classification based model, the predicted Top-1 or Top K beam can be output by the AI/ML model. In this case, if the predicted beam is a measured beam, that is, it is one of the measured beams used as AI/ML model input, whether its corresponding measured L1-RSRP is reported may need to be considered. Otherwise, additional unnecessary beam measurement and reporting may be required to further obtain measured L1-RSRP of the predicted Top-1 or Top K beam. Therefore, Option B should be supported to avoid additional unnecessary beam measurement and reporting.

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, Option B should be supported, i.e., Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.

In addition, for the predicted RSRP, the assumed Tx power may need to be defined or provided. Since RSRP is reflecting the receive power, which is related to Tx power and may not be known if the value is obtained from prediction. For example, in the legacy CSI framework, powerControlOffsetSS is configured per resource and ss-PBCH-BlockPower is provide per cell, while for AI/ML BM cases, the set A resources may or may not be configured. The simplest solution is to mandate the same transmit power for all beams in Set B/Set A, however, the obvious issue of this approach is the loss of flexibility. Thus, some other methods may be needed, for example, for predicted RSRP, the Tx power is assumed based on the configured powerControlOffsetSS of the resource corresponding to the predicted beam if Set A resources are configured and the Tx power is assumed based on setting powerControlOffsetSS to 0 if Set A resources are not configured.

For predicted RSRP, the Tx power is assumed based on the configured powerControlOffsetSS of the resource corresponding to the predicted beam if Set A resources are configured and the Tx power is assumed based on setting powerControlOffsetSS to 0 if Set A resources are not configured.

For P/SP beam report, UE has to perform beam measurement and reporting frequently. Leveraging BM-Case 2 can significantly reduce the overhead associated with beam measurement and reporting. Specifically, the measured beams in multiple historical time instances within the observation (or measurement) window are be used to predict the beam(s) in one or more future time instance(s) within the prediction window. Therefore, within the observation window, UE needs to perform beam measurement and reporting; and within the prediction window (or outside the observation window), UE does not need to perform beam measurement and reporting. To achieve this, for instance, NW can configure a SP beam report, then NW activates or deactivates it before and after the observation window (or after and before the prediction window). However, it would evidently result in significant signaling overhead. To address this issue, measurement window and prediction window should be configured for at least for P/SP beam report.

For BM-Case2, at least for P/SP beam report, observation window and prediction window should be configured.

In BM-Case 2, the time interval between adjacent time instances should be consistent, whether for the observation window or the prediction window. Therefore, at least for P/SP beam report, the time interval between adjacent time instances within the predicted time window should be the periodicity of the P/SP beam report. Accordingly, the duration of the N time instance(s) that can be predicted should be a multiple of the periodicity of the P/SP beam report, e.g., N times the periodicity.

For BM-Case2, at least for P/SP beam report, the duration of the N time instance(s) that can be predicted should be a multiple of the periodicity of the P/SP beam report.

In RAN1 #118b, among the three options for determining the reference time of predicted time instance in BM-Case2, Option 1 and 3 are preferred over Option 2. For Option 1, the reference time is based on the uplink slot of the report. This method is straightforward and aligns well with the approach used in Rel-18 MIMO predictive CSI. Using the uplink reporting slot plus a configurable offset allows flexibility in setting the earliest prediction instance and facilitates easy management of future prediction intervals. Option 2, which based on CSI reference resource is not preferred because it indirectly ties the prediction timing to the Option 1. For Option 3, the approach is also straightforward. The earliest time instance can be set bead on the latest transmission occasion of the CSI-RS/SSB resource in Set B for the report, with a time offset applied between this reference resource and the earliest prediction instance.

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, Option 1 and Option 3 are preferred.

The inference results reported by UE are not always entirely accurate. In other words, the predicted best beam may not be the genie-aided (or measured, actual) best beam, and the predicted L1-RSRP may have a certain error compared to the measured L1-RSRP. This is also reflected in the performance evaluation results, for instance, the Top-K beam prediction accuracy can only reach a higher value when K is larger than 1. Therefore, it is necessary to perform measurement and reporting for the predicted Top-K beams reported by UE, namely (predicted) Top-K beam measurement and reporting, before the beam(s) can be used for further communication. Accordingly, potential enhancements on Top-K beam measurement and reporting should be considered.

For BM-Case 1/2 with UE-sided model, support further measurement and reporting among (predicted) Top-K beams.

    - Mode inference at NW side

It is agreed to support the report of more than 4 beam related information in L1 signaling and it is still FFS on the max number of reported beam related information in one report. Even not considering multiple models or different LCM stages, for NW-sided model, the max number of reported beam should be not less than the number of resources in the Set B. If all measurement results of Set B needs to be reported, the overhead may still be too high, and if measurement results of multiple historical time instances are in one report, the payload size of one report is even higher. Therefore some overhead reduction mechanisms can be further studied, for example, the CRI/SSBRI information may be omitted as agreed in the last meeting. Moreover, at least for BM-Case 2, considering that the earlier historical results may contribute less to the overall prediction, those results can be quantized with a larger step size to reduce some bits in report. As a specific example, if 8 historical results (e.g., obtained at t1, t2, …, t8 and |t1-t0| > |t2-t0| >…>|t8-t0|) are needed as model inputs (e.g., for model inference at t0), the first 4 (i.e., results obtained at t1-t4) can be quantized with a larger step size than the later 4 (i.e., results obtained at t5-t8).

For overhead reduction for BM-Case2, support a larger RSRP quantization step size for the historical results obtained earlier, e.g., with a longer time from measurement to model inference.

Set B may be fixed or variable across training and inference. And for variable Set B, it may be selected from a set of pre-configured Set B patterns, or be a subset of measured beams Set C and based on some criterions/thresholds. For the former case, multiple candidate Set B patterns could be configured by NW explicitly. In addition to L1-RSRP, UE also needs to report indication related to the selected Set B, e.g., index of a group of beams. As for the latter case, criterions/thresholds for determining the Set B need to be defined, e.g., beams with top-M measurements of Set C, beams with the reported measurements within a given gap to the best beam in Set C.

For a variable Set B selected from a set of pre-configured Set B patterns, indication related to the selected Set B (e.g., index of a group of beams) needs to be reported. For a variable Set B that is a subset of measured beams Set C, criterions/thresholds for determining the Set B need to be defined.

  - Performance Monitoring

    - General aspects

In RAN1 #118b [3], it was agreed that the resource sets and report configuration for monitoring could either be integrated within the CSI report configuration for inference (Option1) or configure as a dedicated CSI report configuration used for monitoring (Option2). In Option 1, the monitoring and inference results would be reported within the same CSI report, which may simplify configuration by combining both results into a single report. This option could be beneficial if the monitoring requirements are aligned closely with inference, reducing the need for separate configurations. However, this option cannot handle that monitoring and inference requires different report quantity. Also, a combined configuration could also lead to challenges in meeting distinct needs if the monitoring requires a different reporting frequency, specific metrics, etc. In Option 2, monitoring is handled in a dedicated report configuration separate from inference, so it allows to configure different report quantity for monitoring and inference. Also, this separation allows the reporting intervals, resource allocation and configurations to be adjusted specifically to monitoring needs. Option 2 provides a better flexibility, especially when monitoring requirements differ from those of inference, for example, in the case that monitoring needs to be event-triggered independently or has a longer reporting interval.

For the monitoring RS resources, configuring the whole Set A for UE may result in excessive signaling overhead. Also, in some scenarios, it is not necessary to configure the whole Set A. Therefore, adopting a strategy that involves configuring a subset of Set A might be needed, including how to identify and select the beams for efficient performance monitoring.

For the configuration of monitoring Type 1 Option 2 of UE-side model monitoring,

Option 2 can be a baseline and further study Option 1.

The resource set(s) for monitoring can be a full set of Set A or a subset of Set A.

As we mentioned before, it is feasible to use L1 signalling (e.g., CSI report) for data collection for performance monitoring, hence, the RS for performance monitoring can naturally be configured based on CSI framework. On the other hand, considering that performance monitoring may introduce new report content, and the calculation of performance metric typically relies on statistical outcomes, along with the potential requirement for extensive resource configuration, independently configuration of RS for performance monitoring can be considered, for instance, similar as BFD-RSs. Additionally, to save the overhead of configuring RS for performance monitoring, it can also be considered to implicitly obtain the RS for performance monitoring, for example, based on measurement RSs associated with the already CSI report(s).

For determination of RS for performance monitoring, the following methods can be considered:

Explicit configuration based on CSI framework.

Explicit independent configuration, e.g., similar as BFD-RSs.

Implicit obtainment, e.g., based on RSs associated with the already CSI report(s).

For Option 1, on the other hand, configuration for model inference and configuration for performance monitoring can be seen as two different CSI report configurations in one CSI-ReportConfig. Current spec provides one possible solution, which is to adopt the sub configurations of CSI-ReportConfig, with one sub configuration for model inference and one sub configuration for performance monitoring if needed. The two sub configurations are naturally connected since they belong to the same CSI-ReportConfig.

For Option1, study sub configuration of CSI report, one sub configuration may correspond to model inference and the other sub configuration may corresponding to performance monitoring.

From the system perspective, only when physical layer problem is detected, NW or UE needs to investigate if the cause of the problem is the bad performance of AI/ML model inference. At least for UE-side monitoring, legacy procedures like BFD, RLM to detect physical layer problem can be used as the triggering condition of AI/ML model performance monitoring. For example, if the MAC entity received a (or N consecutive) beam failure instance indication(s) or an (or N consecutive) out-of-sync indication(s), it may trigger the AI/ML performance monitoring procedure.

For both BM-Case1 and BM-Case2, performance monitoring is only needed when the legacy procedures like BFD, RLM detected a physical layer problem.

Support UE to initiate performance monitoring of current AI/ML model if beam failure instance indication or out-of-sync indication is received.

For performance monitoring of UE-side model, it is essential to assess the performance for multiple (different) Set Bs, even for the same AI/ML model. For instance, as depicted in Figure 2, suppose Set B is a subset of Set A. The Set B-0 and Set B-1 are selected uniformly. The Set B-0 is 1/8 of the Set A, and the Set B-1 is 1/4 of the Set A. During the performance monitoring, the performance corresponding to these two Set Bs, namely the performance when each Set B is utilized as input of the AI/ML model, should both be subject to evaluation, thereby balancing the beam measurement overhead and performance metrics (e.g., beam prediction accuracy, L1-RSRP difference). The performance monitoring results for multiple Set Bs will exert an influence on NW’s configuration of Set B in model inference, such as the number of configured beam measurement resources.

Figure 2 Different Set Bs

For performance monitoring of UE-side model, support to assess the performance for multiple Set Bs to balance beam measurement overhead and performance metrics.

If the current AI/ML model cannot reach the performance requirement and there are other AI/ML models available (which may be no activated yet), it may be worth trying other models before fallback to non-AI operations. For both type1 and type 2 performance monitoring of UE-sided model, UE could activate (the inactive) candidate models and initiate performance monitoring of the candidate models before model switching. During this procedure, UE can configure a MAC CE carrying the candidate models related information and send a Scheduling Request (SR) if no PUSCH is configured. Once the UL grant is received, the UE will provide the information about the candidate models to NW. Based on this information, NW will configure the related RS for the performance monitoring of the candidate models. Candidate model related information may also be provided through other means, such as UE capability transfer or UE Assistance Information (UAI). If none of AI/ML models can provide satisfactory performance, fallback maybe inevitable.  Additionally, according to TR [1], model selection requires selecting one AI/ML model as the current used AI/ML model from multiple AI/ML models with the same functionality, which means that the performance of these AI/ML models need to be fully evaluated before model selection. Therefore, performance monitoring of candidate models should be performed before model switching or selection. Furthermore, in order to reduce the latency of model switching or model selection, simultaneous performance monitoring for multiple candidate models should be supported.

Study simultaneous performance monitoring for multiple candidate models, including how to inform the NW the inactive candidate models and how to request resources and configurations for performance monitoring of the inactive candidate models.

    - Performance metrics

When we consider Option-2 (UE-assisted performance monitoring), there is a need to define how UE determines performance metric and reports the results to the network. We can consider the following method for this.

- Step-0: gNB configures UE for performance monitoring and reporting (FFS whether gNB configures the performance metric)

- Step-1: UE determines the performance metric based on the received configuration

- Step-2: UE reports the results of performance metric to the gNB based on the reporting configuration

- Step-3: gNB decides what model management decision to take based on the report received from UE.

For Option-2 (UE-assisted performance monitoring), following methodology is used.

Step-0: gNB configures UE for performance monitoring and reporting (FFS whether gNB configures the performance metric)

Step-1: UE determines the performance metric based on the received configuration

Step-2: UE reports the results of performance metric to the gNB based on the reporting configuration

Step-3: gNB decides what model management decision to take based on the report received from UE.

This raises the second question that how to standardize the performance metric. An agreement as follows was reached in RAN1#118 for Option 2 (UE-assisted performance reporting). Among them, Alt 1 was agreed in RAN1#118bis.

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

We deem Alt 1 to be beneficial for the performance monitoring for classification-based model (i.e., the AI/ML model output is only beam ID). For regression-based model (or the model can predict RSRP), typically, the model output should be RSRPs of all beams of Set A. To evaluate the performance of the model, it is natural to compare predicted RSRPs and measured RSRPs of beams of Set A. Therefore, at least for regression-based model, Alt 3 (The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring) should be supported as a performance metric.

Furthermore, Alt 3 is an effective solution for identifying the partial inaccurate predicted beams (in Set A). In performance monitoring, the degradation in performance monitoring may be confined solely to partial beams rather all beams. For example as shown in Figure 3, some beams (e.g., beam-12) may be affected or blocked due to unexpected or unpredictable obstacle. In this case, in the set of predicted beams (i.e., Set A), only beam-12 may not be accurately predicted by the AI/ML model. Specifically, for classification-based AI/ML model, the AI/ML model cannot accurately identify beam-12 as the top-1 beam. In other words, only when the AI/ML model outputs beam-12 as the top-1 beam can it be considered that this predicted result is inaccurate; for regression-based AI/ML model, the predicted L1-RSRP of beam-12 is inaccurate. If the performance degradation of the model is solely due to a few partial inaccurate predicted beams, then the overall performance of the model should not be considered ‘poor’, nor should the model be hastily deactivated. Instead, we simply need to ignore or drop the predicted results corresponding to these beams. To identify the inaccurate predicted beams, performance monitoring based on Alt 3 might be an effective solution. When the AI/ML model output is only beam ID, to evaluate the performance of the model, typically we can compare the beam ID’s of the predicted beams with the beam ID’s of the subset of Set A from monitoring resources/ resource set. In this case, Alt 2 can be supported for performance monitoring for partial inaccurate predicted beams.

Figure 3 Partial inaccurate predicted beams in Set A

The model performance degradation may be caused by a few partial inaccurate predicted beams (in Set A), which may be blocked due to unexpected or unpredictable obstacle in reality. However, for the majority of other beams (in Set A), the corresponding predicted results can still be considered accurate.

Performance monitoring based on Alt 3 is an effective solution for identifying the partial inaccurate predicted beams (in Set A).

At least for regression-based model (or the model can predict RSRP), support Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring.

Probability can reflect the model's uncertainty regarding the predicted results, which is crucial for assessing the reliability of prediction results. Specifically, for classification based model, UE reports probability for being Top 1 or Top K beam can assist NW in making reliable evaluations of the predicted beams reported by UE. Consequently, Alt 4 (The probability information of the predicted beam(s) to be the Top 1 or Top K beam) should be supported as a performance metric.

When the model can output probability, support Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam.

From our point of view, the probability (e.g., 95%, 90%, 80%, 60%) can be quantized or/and reported based on some pre-defined interval or threshold/criterion. For example, similar to report the measured L1-RSRP, the measured probability can be transformed into the reported probability by defining at least one specific probability interval/range. If UE does not explicitly report probability, it should be at least support incorporating probability as one of the determining factors when reporting predicted beams, especially for classification based model.

Support UE to report probability(ies) of predicted Top K beam(s) based on some pre-defined interval or threshold/criterion. Otherwise, if the probability is not reported, the probability should be used as one of the determining factors when reporting predicted beams at least for classification model.

For Alt 1, to our understanding, using full set of Set A for measurement is needed to determine the ground-truth, i.e., genie-aided Top 1/K beam(s). This is because relying on a subset of set A may fail to identify the precise genie-aided Top 1/K beam(s), specifically, the actual or real genie-aided Top 1/K beam(s) might not be within the subset of Set A that is used for measurement (or monitoring). For Alt 3, since the model can predict RSRPs for all beams of Set A, therefore, to obtain the RSRP difference between predicted RSRP and measured RSRP for each beam of Set A, it is necessary to measure the full set of Set A. Consequently, for Alt 1, it is necessary to measure full set of Set A.

For Alt 1 and Alt 3, UE may need to measure full set of Set A.

When Alt 1 is used as the performance metric, if the performance metric is calculated per sample, the calculated performance metric should be an indication (or status), e.g., whether the predicted Top 1 beam is one of the genie-aided Top 1 or Top K beam(s). If the performance metric is calculated per set of samples (or within a time window), the calculated performance metric should be a percentage, e.g., the percentage of ‘the Top-1 measured beam is one of the Top 1 or Top K predicted beam(s)’. When Alt 3 is used as the performance metric, if the performance metric is calculated per sample, the calculated performance metric should be: L1-RSRP difference per beam (in this case, multiple L1-RSRP differences may be obtained), or one statistical L1-RSRP difference (e.g., mean of all L1-RSRP difference per beam). If the performance metric is calculated per set of samples (or within a time window), the calculated performance metric should be: statistical L1-RSRP difference per beam (in this case, multiple statistical L1-RSRP differences may be obtained), or one statistical L1-RSRP difference.

From our point of view, the performance metric can be calculated either per sample or per set of samples (or window), each with its own merits. Specifically, for the performance metric calculated per sample, NW can obtain performance in real time; for the performance metric calculated per set of samples (or window), unnecessary overhead for reporting the performance metric can be reduced.

For Alt 1 and Alt 3, the performance metric can be calculated either per sample or per set of samples (or within a time window).

For performance metric calculations supporting Alt 1 and Alt 3, it is observed that the UE needs to measure the full set of Set A beams. If we consider full set of Set A in monitoring resources/resource set, Alt 1 and Alt2 may be similar in terms of effectiveness to evaluate the performance of an AI/ML model. On the other hand, if we consider the overhead and only configure a subset of Set A in monitoring resources/resource set, Alt 2 might give a better performance metric than Alt 1. So, depending on whether full set or subset of Set A RS is configured as monitoring RS, we may need different performance matrices. Therefore, we have the following observation and proposal.

Depending on whether full set or a subset of Set A RS can be configured as monitoring RS, different performance matrices may be needed.

Study the calculation of performance metric when a subset of Set A is configured as monitoring RS, if different performance matrices will be supported for full Set A as monitoring RS and a subset of Set A as monitoring RS.

  - Others

Priority rule need to be defined to handle the collision of CSI reports. Specifically, the priority value corresponding to each CSI report should be determined and compared with each other. The existing priority rule is only for non-AI/ML based CSI report. If the AI/ML based CSI report (e.g., carrying predicted beams/L1-RSRPs) is introduced, both AI/ML based CSI report vs. non-AI/ML based CSI report and AI/ML based CSI report vs. AI/ML based CSI report (e.g., different types of AI/ML based report) should be considered.

If the AI/ML based CSI report is introduced, define the priority rule at least in following cases:

Non-AI/ML based CSI report vs. AI/ML based CSI report

AI/ML based CSI report vs. AI/ML based CSI report

For model management, an important point to consider and discuss is how the overall beam prediction framework co-work with carrier aggregation and bandwidth part functionality. For carrier aggregation where cells are operating in different RF bands (e.g. one cell in FR1 and other cell in FR2), whether multiple beam prediction models can run simultaneously at UE corresponding to different RF bands? If yes, then how does UE indicate its capability on how many models can be simultaneously run? Furthermore, in this case, the signalling for management of the model (e.g. activation/deactivation) needs to consider this association of AI/ML model with the appropriate serving cell or RF band.

Multiple beam prediction models may run simultaneously at UE corresponding to different RF bands in CA.

When we consider the bandwidth part functionality, there can be a single model which can run for different bandwidth parts of a serving cell as all bandwidth parts share the same set of beams. However, there would still be a need to provide BWP specific configuration parameters for beam prediction. For instance, when UE switches from one BWP to another, the Set B association may not be applicable anymore as reference signals configurations are provided per BWP. This creates the need to have some of the beam prediction configuration (like Set B association to RS resources) to be BWP specific.

Although the signalling framework for the above issues may ultimately be designed by RAN2, but RAN1 still needs to provide relevant guidelines to RAN2 to allow developing such framework and hence further discussions are necessary on this topic.

Some of the beam prediction configuration parameters may be BWP specific when UE is configured with multiple BWPs

RAN1 to discuss the impact of CA and BWP operation on the beam prediction framework.

As discussed, if none of AI/ML models can provide satisfactory performance, fallback to non-AI operations maybe inevitable. For BM related sub use cases, the non-AI operation may be the NW configured/triggered beam report introduced in previous releases, and all supported beam report methods require proper NW configurations. It may be too late for the NW to configure legacy beam report via RRC reconfiguration after determining that none of AI/ML model can work properly, therefore the legacy beam report may be configured simultaneously with the AI/ML based BM-Case1 and/or BM-Case2. For example, the association between the measurement/report configuration IDs of the legacy BM and the AI/ML based BM can be explicitly configured.

If legacy BM is configured simultaneously with the AI/ML based BM, the fallback can be determined by the legacy BM.

Support to provide associated legacy BM configuration together with the configuration of AI/ML based BM.

- Discussion on Impacts of AI/ML on Beam Management

The introduction of AI/ML to enhance beam management would inevitably impact those legacy signalling and procedures, for example, on beam measure measurement and report, beam indication and beam failure recovery. In this section, the discussion is on the possible enhancements making full use of the predicted beam obtained by AI/ML model inference.

  - Necessary enhancements on beam indication

It is agreed in the last meeting that for NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework and FFS on whether/how potential enhancement is needed. Since the legacy TCI framework is providing a QCL relationship between one or two downlink reference signals and the target (ports) of channels/signals (e.g., DM-RS ports of the PDSCH/PDCCH or the CSI-RS port(s) of a CSI-RS resource), with either spatial-domain or temporal beam prediction, when the predicated beam is obtained, there may be no actually measured reference signals, TCI framework needs to be extended to provide beam indication based on the prediction for DL/UL channels/signals.

Legacy TCI framework needs to be enhanced to indicate the predicted beam(s) since there may be no actually measured QCL reference signals.

As discussed above, one of the key problems to be solved is which reference signal needs to be configured in the TCI state. The problem becomes tricky if the predicted beam is not in the Set B. One possible solution is that NW configures the reference signal in associated Set A which represents the predicted beam as the QCL reference signal in the TCI state. However, there are still open issues, for example, it may need RRC reconfiguration, which is time consuming and causes significant signalling overhead.

To enhance unified TCI state to indicate the predicted beam, support to configure RS in associated Set A as the QCL reference signal in the TCI state.

For BM-Case2, regardless of whether based on UE-side model or NW-side model, NW possesses foresight of predictive beam(s) at one or multiple future time instance(s). Consequently, to reduce unnecessary signaling overhead of beam indication, we should extent the legacy TCI framework to support to use one MAC CE or DCI to activate/indicate multiple (future) TCI states. Furthermore, the time period (i.e., beam application/dwelling time) corresponding to each (future) TCI state can be indicated.

For BM-Case 2, support to use one MAC CE or DCI to activate/indicate multiple (future) TCI states, and corresponding time period.

Meanwhile, if the QCL reference signal(s) contained in an indicated TCI state is not measured or reported, according to current spec, it is an unknown TCI state. In this case, if a TCI state refers to the predicted beam which is not in the measured Set B, it might be an unknown TCI state and a longer time duration is required for UE to determine suitable Rx beam. In this case, network would configure/activate UE to measure the top-1 (or top-K) predicted beam(s) to allow UE to identify the suitable Rx beam for the predicted beam(s). However, this procedure results in extra overhead and delay where UE measures on the predicted beam(s) before actual data transmission which may somewhat diminish the benefits of the AI/ML operation.

Additional measurements of Top-K UE predicted beams can be performed to allow UE to determine the suitable Rx beam for the predicted beams. However, this operation results in higher signaling overhead and diminishing the AI/ML benefits.

It is possible that above mentioned procedure (i.e. additional measurements of predicted beams) can be eliminated for the case when UE has the capability to predict beam pair (both Rx and Tx beam) rather than only Tx beam. Note that even for the case of beam pair prediction, UE may report only best DL Tx beams to the network and the predicted Rx beam may only be used for UE’s internal consumption. When UE is aware of the best Rx beam for a predicted Tx beam then this additional step of UE measuring predicted beam can be avoided and improve the overall system performance. However, for network to optimise this procedure, the gNB needs to be aware of the UE capability whether UE can perform only Tx beam prediction or beam pair prediction.

If UE has the capability to predict Tx-Rx beam pair, then additional procedure of measuring Top-1 UE predicted beams can be avoided.

Discuss how to reduce the signaling overhead of UE performing measurements of predicted beams to determine the suitable Rx beam.

  - Necessary enhancements on beam failure recovery

Beam failure recovery has been introduced to combat the FR2 blockage issue. It has some major steps such as beam failure detection, candidate beam identification, beam failure recovery request and the NW’s response to the request. With AI/ML model inference at NW side, it is possible to have a timelier beam switching, which may depends on NW implementation and needs less specification efforts. With AI/ML model inference at UE side, however, at least the following enhancements can be expected.

- For beam failure detection: at least with BM-Case 2, it is possible to have predicted beam quality of the BFD-RS, which may be used to trigger early switching or to avoid false alarm of BFD. However, some problems still require further investigations, for example, the legacy threshold is a hypothetical PDCCH BLER, which may not be the typical model output (the widely discussed metric may be L1-RSRP).

- For new beam identification: for both BM-Case1 and BM-Case2, it is possible to have a predicted beam not in the configured candidate beam set, which may be used to reduce the time and complexity required to find the new beam (q_new) after BFD.

Either spatial-domain beam prediction or time-domain beam prediction can improve BFD and BFR.

At least for BM-Case 2, support beam failure detection based on time-domain prediction.

Support predicted beam as the newly identified beam q_new.

- Conclusion

In this contribution, we provided our views on Rel-19 AI/ML for air-interface work on beam management enhancement use case, and we have the following observations and proposals:

Observation 1:	The timing information of Set B measurements and Set A predictions is crucial for UE to determine the model to be used for inference.

Observation 2:	P/SP NZP-CSI-RS are configured and active for model inference, it might impact the overall system throughput.

Observation 3:	For both BM-Case1 and BM-Case2, performance monitoring is only needed when the legacy procedures like BFD, RLM detected a physical layer problem.

Observation 4:	The model performance degradation may be caused by a few partial inaccurate predicted beams (in Set A), which may be blocked due to unexpected or unpredictable obstacle in reality. However, for the majority of other beams (in Set A), the corresponding predicted results can still be considered accurate.

Observation 5:	Performance monitoring based on Alt 3 is an effective solution for identifying the partial inaccurate predicted beams (in Set A).

Observation 6:	For Alt 1 and Alt 3, UE may need to measure full set of Set A.

Observation 7:	Depending on whether full set or a subset of Set A RS can be configured as monitoring RS, different performance matrices may be needed.

Observation 8:	Multiple beam prediction models may run simultaneously at UE corresponding to different RF bands in CA.

Observation 9:	Some of the beam prediction configuration parameters may be BWP specific when UE is configured with multiple BWPs

Observation 10:	If legacy BM is configured simultaneously with the AI/ML based BM, the fallback can be determined by the legacy BM.

Observation 11:	Legacy TCI framework needs to be enhanced to indicate the predicted beam(s) since there may be no actually measured QCL reference signals.

Observation 12:	Additional measurements of Top-K UE predicted beams can be performed to allow UE to determine the suitable Rx beam for the predicted beams. However, this operation results in higher signaling overhead and diminishing the AI/ML benefits.

Observation 13:	If UE has the capability to predict Tx-Rx beam pair, then additional procedure of measuring Top-1 UE predicted beams can be avoided.

Observation 14:	Either spatial-domain beam prediction or time-domain beam prediction can improve BFD and BFR.

Proposal 1:	The UECapablityInformation message comprises signalling of detailed information of the conditions.

Proposal 2:	The timing information of Set B measurements and Set A predictions should be specified as conditions for BM-Case1 and BM-Case2 and comprised in UE capability reporting.

Proposal 3:	Support confidence comprised in UE capability reporting or as a condition for functionality.

Proposal 4:	Support Option 3 for applicability for inference for UE-side model.

Proposal 5:	For the consistency of NW-side additional conditions across training and inference for UE-sided model for BM-Case 1 and BM-Case 2 based on associated ID, where the NW-side additional conditions may at least impact UE assumption on beams of Set A/Set B.

−	Associated ID shall at least indicate the site/cell specific variables used for model training like antenna configuration.

−	NW-side additional conditions with the same associated ID can be consistent across multiple cells. FFS for which cells NW-side additional conditions can be assumed consistent.

Proposal 6:	Support a reference beam pattern as NW sided additional condition to ensure the consistency.

Proposal 7:	Support to configure the associated ID(s) also for AI/ML performance monitoring.

Proposal 8:	For data collection for NW-sided model, also support L1 report leveraging the existing CSI report framework.

Proposal 9:	The configured P/SP NZP-CSI-RS resources for Set A should be available for other channel/signal (e.g., PDSCH) at least during model inference.

Proposal 10:	Support selecting Top-K beam(s) according to some pre-defined rules (e.g., a sum probability of being Top 1 or Top K beam higher than a threshold, predicted L1-RSRP higher than a threshold) as the reported predicted beams.

Proposal 11:	For BM-Case1 and BM-Case2, the value of K (i.e., the number of predicted beams to report) should be determined and provided to NW by UE.

Proposal 12:	For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, Option B should be supported, i.e., Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.

Proposal 13:	For predicted RSRP, the Tx power is assumed based on the configured powerControlOffsetSS of the resource corresponding to the predicted beam if Set A resources are configured and the Tx power is assumed based on setting powerControlOffsetSS to 0 if Set A resources are not configured.

Proposal 14:	For BM-Case2, at least for P/SP beam report, observation window and prediction window should be configured.

Proposal 15:	For BM-Case2, at least for P/SP beam report, the duration of the N time instance(s) that can be predicted should be a multiple of the periodicity of the P/SP beam report.

Proposal 16:	For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, Option 1 and Option 3 are preferred.

Proposal 17:	For BM-Case 1/2 with UE-sided model, support further measurement and reporting among (predicted) Top-K beams.

Proposal 18:	For overhead reduction for BM-Case2, support a larger RSRP quantization step size for the historical results obtained earlier, e.g., with a longer time from measurement to model inference.

Proposal 19:	For a variable Set B selected from a set of pre-configured Set B patterns, indication related to the selected Set B (e.g., index of a group of beams) needs to be reported. For a variable Set B that is a subset of measured beams Set C, criterions/thresholds for determining the Set B need to be defined.

Proposal 20:	For the configuration of monitoring Type 1 Option 2 of UE-side model monitoring,

−	Option 2 can be a baseline and further study Option 1.

−	The resource set(s) for monitoring can be a full set of Set A or a subset of Set A.

Proposal 21:	For determination of RS for performance monitoring, the following methods can be considered:

−	Explicit configuration based on CSI framework.

−	Explicit independent configuration, e.g., similar as BFD-RSs.

−	Implicit obtainment, e.g., based on RSs associated with the already CSI report(s).

Proposal 22:	For Option1, study sub configuration of CSI report, one sub configuration may correspond to model inference and the other sub configuration may corresponding to performance monitoring.

Proposal 23:	Support UE to initiate performance monitoring of current AI/ML model if beam failure instance indication or out-of-sync indication is received.

Proposal 24:	For performance monitoring of UE-side model, support to assess the performance for multiple Set Bs to balance beam measurement overhead and performance metrics.

Proposal 25:	Study simultaneous performance monitoring for multiple candidate models, including how to inform the NW the inactive candidate models and how to request resources and configurations for performance monitoring of the inactive candidate models.

Proposal 26:	For Option-2 (UE-assisted performance monitoring), following methodology is used.

−	Step-0: gNB configures UE for performance monitoring and reporting (FFS whether gNB configures the performance metric)

−	Step-1: UE determines the performance metric based on the received configuration

−	Step-2: UE reports the results of performance metric to the gNB based on the reporting configuration

−	Step-3: gNB decides what model management decision to take based on the report received from UE.

Proposal 27:	At least for regression-based model (or the model can predict RSRP), support Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring.

Proposal 28:	When the model can output probability, support Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam.

Proposal 29:	Support UE to report probability(ies) of predicted Top K beam(s) based on some pre-defined interval or threshold/criterion. Otherwise, if the probability is not reported, the probability should be used as one of the determining factors when reporting predicted beams at least for classification model.

Proposal 30:	For Alt 1 and Alt 3, the performance metric can be calculated either per sample or per set of samples (or within a time window).

Proposal 31:	Study the calculation of performance metric when a subset of Set A is configured as monitoring RS, if different performance matrices will be supported for full Set A as monitoring RS and a subset of Set A as monitoring RS.

Proposal 32:	If the AI/ML based CSI report is introduced, define the priority rule at least in following cases:

−	Non-AI/ML based CSI report vs. AI/ML based CSI report

−	AI/ML based CSI report vs. AI/ML based CSI report

Proposal 33:	RAN1 to discuss the impact of CA and BWP operation on the beam prediction framework.

Proposal 34:	Support to provide associated legacy BM configuration together with the configuration of AI/ML based BM.

Proposal 35:	To enhance unified TCI state to indicate the predicted beam, support to configure RS in associated Set A as the QCL reference signal in the TCI state.

Proposal 36:	For BM-Case 2, support to use one MAC CE or DCI to activate/indicate multiple (future) TCI states, and corresponding time period.

Proposal 37:	Discuss how to reduce the signaling overhead of UE performing measurements of predicted beams to determine the suitable Rx beam.

Proposal 38:	At least for BM-Case 2, support beam failure detection based on time-domain prediction.

Proposal 39:	Support predicted beam as the newly identified beam q_new.

- References

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Chair’s Notes, 3GPP TSG RAN WG1 #118bis, Hefei, China, October 14th – 18th, 2024

- R1-2407604(R2-2407848), LS on applicable functionality reporting for beam management UE-sided model