3GPP TSG RAN WG1 #119			R1-2409892

Orlando, US, November 18th – 22nd, 2024

Source: 	     Xiaomi

Title:                  Discussion on reception and transmission procedure for SBFD operation

Agenda item:    9.3.1

Document for:  Discussion

Introduction

In RAN1#118bis meeting, SBFD TX/RX/measurement procedure was heatedly discussed. Very good progress were achieved. The following agreements were drawn:[1]

Conclusion Nominal RBG size for PDSCH/PUSCH in SBFD symbols is determined based on the size of DL/UL BWP as legacy. Nominal CSI reporting subband size in SBFD symbols is determined based on the size of DL BWP as legacy. Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 6 (Dynamic or semi-static DL vs. valid RO), reuse the existing collision handling rules for HD-FDD RedCap UEs. - UE does not expect collision between PRACH triggered by PDCCH order and dynamically scheduled DL reception. - For collision between PRACH triggered by PDCCH order and semi-statically configured DL reception, UE does not receive DL channel/signal. - For collision between PRACH triggered by higher layer and DL channel/signal, leave it to UE implementation whether to receive the DL or transmit PRACH. Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. Agreement The number of configured FH offsets of FH offset list for SBFD symbols is the same as the number of FH offsets of FH offset list for non-SBFD symbols. - The number of configured FH offsets of FH offset list is determined based on the UL BWP size as legacy. The number of bits used to indicate FH offset in DCI is determined as legacy. Agreement Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options. - Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS applicable usages - Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource. - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured. - FFS separate configurations of other parameters - FFS applicable usages Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability - UE capability reports the maximum number of supported partial PRGs in SBFD symbols. Agreement - For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs. Agreement: For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations Agreement: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configurat 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2. Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, - The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy. - The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. - Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ) - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded. Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband Agreement If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, support Option 1 for determining UL/DL usable PRBs. - Option 1: For a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols. Agreement For a PDSCH/PUSCH with assigned PRBs overlapping with SBFD subband boundary and only the PRBs within DL/UL usable PRBs are valid for PDSCH/PUSCH reception/transmission, legacy DMRS sequence mapping is applied to assigned PRBs within DL/UL usable PRBs only (effectively, this is same as the case when the DMRS sequence mapped to the RBs outside the DL/UL usable PRBs are punctured). Agreement For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SPS PDSCH, CG PUSCH, PDSCH/PUSCH repetition, multi-PDSCH/PUSCH scheduled by a single DCI and TBoMS is SBFD symbols, - When an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively. - SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs. - FFS: The above applies to SP-CSI on PUSCH when applicable For configuration 1 and frequency resource allocation Type 1, if the valid symbol type for SPS PDSCH, PDSCH repetition and multi-PDSCH scheduled by a single DCI is SBFD symbols, then - Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. - Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused. - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only.

In this contribution, we first provide our views on the remaining issues on subband configuration. In the follow-up section, we discuss the detail SBFD TX/RX/measurement procedure.

Discussion

Time domain resource allocation for UL subband

In RAN1#118 meeting, a TDD UL-DL configuration like mechanism was adopted to provide the time domain location of SBFD subband. To be specific, the following parameters are configured within a TDD-UL-DL pattern period:

- A starting slot index

- A starting symbol index within the starting slot

- An ending slot index

- An ending symbol index within the ending slot

According to the nature of the aforementioned mechanism, it can only indicate a SBFD subband with consecutive OFDM symbols locating on semi-static flexible symbol and downlink symbol. One example is shown in Figure 1 as below. In theory, guard period is needed between DL transmission and UL transmission in order to avoid inter-slot interference. Otherwise, UE-to-UE CLI will jeopardize the DL performance for both SBFD aware UE and non-SBFD aware UE which receives DL transmission on the previous DL slot. The Rx-to-Tx switching at UE side and Rx-to-Tx switching at gNB side also need guard period. Therefore, it needs attention on how to define or indicate guard period between DL symbol and the first OFDM symbol occupied by UL subband.

Figure 1 Example of SBFD subband time location based on starting position indication and ending position indication

Proposal 1: Guard period between DL symbol and first symbol occupied by UL subband can be determined implicitly or explicitly.

It is obvious that SBFD subband can be configured on semi-static flexible symbol configured by tdd-UL-DL-ConfigurationCommon. On the other hand, it is well known that gNB can adjust the transmission direction of semi-static flexible symbol according to traffic load at least. Assuming the TDD UL-DL configuration for carrier is DFFFU configured via tdd-UL-DL-ConfigurationCommon, gNB configures SBFD subband time location via a cell-specific signalling. Here we assume the UL subband occupies slot#1, slot#2 and slot#3 in time domain, which is shown in Figure 2. As slot#1, slot#2 and slot#3 are semi-static flexible slots configured via cell-specific signalling, gNB can further indicate the slot direction via UE-dedicated signalling or SFI. To be specific, the flexible symbols can be either indicated as DL symbol or UL symbol. It can be summarized as below:

- If semi-flexible symbol is indicated as DL symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

- If semi-flexible symbol is indicated as UL symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

- If semi-flexible symbol is indicated as SFI flexible symbol on which UL subband is configured, what is the expected behaviour for SBFD aware UE?

Figure 2 Example of SBFD subband locates on semi-static flexible slots

In RAN1#116 meeting, it was agreed that RAN1 would discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0. It was mentioned that DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol. From our understanding, non-SBFD symbol means a OFDM symbol without SBFD subband on it. In RAN1#116bis meeting, it was agreed that SBFD/non-SBFD symbol configured by cell-specific signalling cannot be reverted to non-SBFD/SBFD symbol. The intention is actually to preclude UE-dedicated configuration for SBFD time location. As clarified in online session, the definition of SBFD symbol and non-SBFD symbol only relates to whether there is SBFD subband on the dedicated symbol respectively. There is nothing about UE behaviour on transmission/reception in such symbols. It should be a separate discussion on the interaction between legacy direction indication, i.e., SFI and UE-dedicated TDD-UL-DL configuration, and semi-static flexible symbol with SBFD subband configuration. Hence, we have the following proposal to further clarify UE behaviour once semi-static symbol with SBFD subband is further indicated with certain direction via legacy signalling.

Proposal 2: If UL subband is configured on semi-static flexible symbol and gNB further adjusts direction for the symbols, clarify behaviour of SBFD aware UE:

- Option 1: UE ignores UE-dedicated TDD-UL-DL configuration and SFI on SBFD symbols

- Option 2: UE determines symbol type according to direction indicated by UE-dedicated TDD-UL-DL configuraiton or SFI. The following can be starting point:

  - If the SBFD symbol is additionally indicated as a UL symbol, UE ignores SBFD subband configuraiton on this symbol and transmits uplink within active BWP.

  - Otherwise, UE treats the symbols as SBFD symbol.

One concern is that frequent switching between SBFD and non-SBFD symbols may increase the implementation complexity and interruptions of transmissions/receptions during transition. It is true that UE may need to adjust its RF parameters, power parameters and spatial parameters for the transmission on SBFD symbol and non-SBFD symbols. In order to provide sufficient period for accommodating operation on different symbol type, one or several symbols in-between may be needed. However, switching between SBFD symbols and non-SBFD symbols is a generic issue which needs to be studied for SBFD operation. In the other words, it is not specific for a slot consisting of both SBFD symbols and non-SBFD symbols. Therefore, we should consider the switching point not only for SBFD symbol and non-SBFD symbol, but also for SBFD symbol and SBFD symbol with different transmission direction.

Observation 1: The maximum number of transition point between DL/UL to UL/DL is a generic issue for SBFD operation.

It would be quite helpful if the maximum number of transition point can be limited within a window for both gNB and UE. Directly, the maximum number can be applicable to the configuration period of UL subband.

Proposal 3: The maximum number of transition point between DL/UL and UL/DL within consecutive SBFD slots is determined per time window.

For a slot consisting of both SBFD symbols and non-SBFD symbols, it can directly reuse the mechanisms as self-contained slot structure, i.e., only one switching point is allowed within the slot. The flexible symbols between DL symbols and UL symbols can be used as switching period for UE transition operation between SBFD symbols and non-SBFD symbols. Regarding which scenarios a guard period between SBFD and non-SBFD symbols is required/not required and how to determine the length of guard period, we think it is irrelevant to SBFD subband time domain configuration.

Proposal 4: Only one transition point is allowed within a slot consisting of both SBFD symbols and non-SBFD symbols.

- Flexible symbols between DL symbol and UL symbol can be used as guard period for switching between SBFD symbol and non-SBFD symbol.

Frequency domain resource allocation for UL subband

In RAN1#118 meeting, a working assumption was agreed that SLIV-based mechanism is adopted for SBFD DL subband configuration and UL SBFD UL subband configuration, respectively.  However, it was also worried about that the overhead on frequency location indication.

On the other hand, it may be unnecessary to configure a UL subband with different sizes from that of active UL BWP. For example, the frequency location of UL subband can be exactly same as one of UL BWP. It can significantly reduce complexity of RRC signalling on frequency location configuration, i.e., gNB doesn’t need to additionally indicate the frequency location of a UL subband and DL subband. However, gNB has to guarantee all  SBFD aware UEs have same UL BWP in order to achieve a common UL subband configuration with same frequency location. Considering UE supports up to 4 UL BWPs within a carrier, the UL BWP with the smallest index can be used to determine the frequency location of UL subband if gNB doesn’t provide frequency location configuration for UL subband. Furthermore, the frequency location of initial UL BWP can also be used for UL subband determination in frequency domain by default.

Proposal 5: For configuring frequency location of UL subband, the following default mechanism can be considered:

- For a SBFD symbol indicated by time domain indication, a default BWP is used to determine the frequency location of the UL subband if gNB doesn’t provide relevant configuration.

DL resource allocation in frequency domain on SBFD symbols

First of all, there is no restriction on frequency location of UL subband. Therefore, either DUD or DU are possible UL subband configuration. In Figure 3, we provide four cases considering the location of UL subband and whether there is guard band. No matter which case is configured by gNB, the available DL RB for DL transmission on SBFD slot is different from that of non-SBFD slot.

Figure 3 Example of frequency resources allocated to subband in SBFD slot

However, FDRA indication and frequency granularity are determined according to active DL BWP. Therefore, there are two issues needs to be resolved:

- Issue#1: Following current specification, resource allocation for downlink channel and signal may be across DL subband and other unavailable subband. For example, the current DL resource allocation mechanism may lead to the DL channel or DL signal across two DL subbands.

- Issue#2: For the frequency unit at the boundary of DL subband, it may not be aligned with DL subband boundary so that partial RB within the frequency unit overlaps with guard band and/or UL subband.

Resource allocation in frequency domain for PDSCH/PUSCH

In the existing specification, the frequency allocation for PDSCH and PUSCH is pretty flexible. Taking PDSCH as example,

- There are two RA types, i.e., RA type 0 and RA type 1. RA type 0 indicates RBG used for PDSCH transmission with a bitmap indication. Hence, indicated RBG can be contiguous or discrete.  It should be noted that the RBG is determined according to active DL BWP. RA type 1 indicates contiguous VRB or VRB group for PDSCH with a SLIV indication. The mapping between VRB to PRB can be either non-interleaved mapping or interleaved mapping. Despite of RA type 0 and RA type 1, it is sufficient to indicate frequency resources for PDSCH transmission on SBFD symbols.

As analysed above, existing mechanisms of allocating frequency resources for PDSCH and PUSCH can be used to indicate target resources in SBFD symbols. On the other hand, the indicated frequency resources may overlap with unavailable RBs.

For RA type 0, it was agreed that only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

For PDSCH RA type 1, the following agreement was achieved in RAN1#117 meeting:

Agreement For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4}, - Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only - FFS: DMRS sequence mapping - FFS: when PRG is determined as wideband - FFS: partial PRG

Basically, current mechanism/procedure of frequency resource allocation is almost reused. The key difference is SBFD operation has to consider there may be a number of unavailable RBs outside DL subband which are assigned for PDSCH transmission.

For DMRS sequence mapping, it was agreed that legacy DMRS sequence mapping is applied to assigned PRBs within DL/UL usable PRBs only.

Regarding partial PRG overlapping with DL subband boundary, same principle as partial RBG issue should be applied, i.e., precoder is only applied to the usable RBs within a PRG.

Proposal 6:  For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS, precoder is only applied to the usable RBs within a PRG across DL subband boundary.

On the other hand, PDSCH can also be scheduled by DCI format 1_0 which is transmitted in CSS. Basically, DCI format 1_0 can be used to schedule either unicast PDSCH or common PDSCH.  For common PDSCH, option 1-1 is not a good solution as it will degrade reception performance for legacy UE. For unicast PDSCH, there is no barrier to reuse option 1-1.

Proposal 7: For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled by DCI format 1_0 in CSS, option 1-1 is adopted if the scheduled PDSCH is a unicast PDSCH.

In order to accommodate option 1-1, specification needs trivial modification that only RBs contained in DL subband are available for PDSCH mapping.

Proposal 8: Capture it in TS38.214 that only RBs contained in DL subband are available for PDSCH.

Physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots

In RAN1#112 meeting, we clarified that each transmission/reception within a slot has either all SBFD or all non-SBFD symbols.  Basically, two directions were identified, i.e., whether transmissions/receptions are allowed to be across different types of symbols, which are addressed by the following proposal.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH

In RAN1#118 meeting, transmission/reception across different slot types issue was extensively discussed under the umbrella of configuration 1 and configuration 2. Basically, it was agreed that DL reception following FDRA and only mapped to usable RBs while UL transmission following FDRA and a frequency offset in SBFD symbol respectively.

In RAN1#118bis meeting, the granularity of configurability for configuration 1 and configuration 2 was discussed and the following three options were raised for further down-selection:

- Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis

- Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis

- Option 3: Configuration 1 or Configuration 2 is on a per cell basis.

Given that the time domain location and frequency domain location of SBFD subband are configured in a cell-specific manner, we fail to see the necessity of configuration other than cell basis.

Proposal 9: Configuration 1 and configuration 2 is provided per cell basis, i.e., option 3.

Remaining issues on configuration 1

The nature of configuration 1 is that UE can only transmit/receive in symbols with a specific type, i.e., SBFD symbol or non-SBFD symbol. One of key issues is how to determine symbol type for transmission and reception.

For semi-statically transmissions/receptions without activation DCI, it was agreed that the valid symbol type is explicitly configured by RRC. For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. On the other hand, it is still open for transmissions/receptions with semi-persistent characteristic. Basically, there are two options on the table:

- Option 1: - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig. - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig. - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config. - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource]. - Option 2: - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation. - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

In brief, option 1 means the applicable symbol type is semi-statically configured while option 2 provides flexibility on symbol type determination to some extent.

For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, the time-frequency resources occupied for transmission/reception cannot be changed once activated. It is well known that semi-persistent transmission/reception has more similarity with semi-static ones other than that of scheduled by dynamic signalling. Hence, option 1 is sufficient.

Option2 can provide more flexibility as gNB can determine valid symbol type while L1 activation signalling. However, it may be a kind of over optimization as semi-static determination on symbol type is sufficient. More importantly, it brings more UE complexity as UE has to determine the valid symbol type in a dynamic manner.

Furthermore, it was agreed that the symbol type used for SRS transmission is configured by RRC signalling. Although final solution on how to provide the aforementioned indication depends on further discussion, i.e., per SRS resource set basis or per SRS resource basis, the spirit is quite aligned with option 1.

Based on the above analyses, we have the following proposal.

Proposal 10: For configuration 1, the following mechanism (option 1) is used to determine valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS:

- The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig.

- The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig.

- The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config.

- The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource].

Regarding to frequency allocation and TBS determination, it was agreed that usable PRB is used. There is an FFS point on whether the following rules can be applied to SP-CSI on PUSCH:

- When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- The number of PRBs for TBS determination is based on the assigned PRBs within UL usable PRBs only for PDSCH.

- SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

SP CSI on PUSCH is activated/released by DCI format 0_1 or DCI format 0_2 with CRC scrambled with SP-CSI-RNTI. From resource allocation perspective, there is no difference with Type 2 CG PUSCH.  Furthermore, gNB has full power to control whether the PRBs occupied by PUSCH carrying SP CSI exceed UL subband or not.

Proposal 11: For configuration 1, the following applies to SP-CSI on PUSCH when applicable:

- When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- The number of PRBs for TBS determination is based on the assigned PRBs within UL usable PRBs only for PDSCH.

- SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

Remaining issues on configuration 2

For PDSCH reception across different symbol types, only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.  Given the available DL resources are different across SBFD symbols and non-SBFD symbols, how to determine the TBS on each symbol type need further discussion.  In previous meeting, this issue is discussed per PDSCH type, i.e., SPS PDSCH, PDSCH with repetition and multi-PDSCH scheduled by a single DCI.

For SPS PDSCH, it is transmitted periodically with same FDRA across different symbol types. UE can independently receive and decode each PDSCH occasion. For TBS, it can be determined either by assigned PRBs or assigned PRBs within DL PRBs only. There is no additional UE complexity as a SBFD-aware UE supports both mechanisms anyway.

For PDSCH repetitions, there are three options as shown below:

- Option 1: Based on assigned PRBs

- Option 2: Based on assigned PRBs within DL usable PRBs only

- Option 3: Based on assigned PRBs within DL usable PRBs of the first repetition

Considering combination of different repetitions at UE side is fundamental, it is critical to make sure same TBS is applied to each repetition. From this point of view, both option 1 and option 2 may lead to different TBS across different repetitions. In the end, the reception performance for PDSCH degrades.

For multi-PDSCH scheduled by a single DCI, TBS determination can be based on assigned PRBs or assigned PRBs within DL PRBs only due to the nature of UE independently decodes each PDSCH occasion.

Proposal 12: For configuration 2, whether option 1 or option 2 is used to determine TBS is indicated by gNB.

For PUSCH transmission across different symbol types, single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols.  In general, there are two key points:

- Key point#1: same FDRA is applied to different symbol types.

- Key point#2: RB offset for SBFD symbol is introduced.

For point#1, we believe there is no ambiguity according to the agreement and discussion happened during RAN1#118.

For point#2, further clarification on how to derive the RB offset(s) from UE side is necessary. From our understanding, the RB offset should be provided by gNB and only applied to SBFD symbol. Furthermore, it was agreed that the number of PRBs are the same for PUSCH transmissions in SBFD symbols and PUSCH transmissions in non-SBFD symbols. For sure it can be guaranteed by gNB that frequency resource allocation is fully confined within UL subband on SBFD symbols. On the other hand, the price of such kind of restriction should also be further discussed. It should be noticed that PUSCH scheduling on non-SBFD symbols is inevitably impacted.

In RAN1#118bis meeting, two options on how to determine the starting PRB of PUSCH transmission in SBFD symbols were identified for further discussion, which are shown as below:

- Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to predefined equation.

- Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to predefined equation.

The major difference between option 1 and option 2 is whether RBoffsetSBFD is explicitly configured.  Given that gNB schedules or configures a PUSCH transmission across SBFD symbols and non-SBFD symbols with single signalling, it knows the starting PRB index of PUSCH transmission in non-SBFD symbols. It is sufficient for both gNB and UE to determine the starting PRB index of PUSCH transmission in SBFD symbols with a single RB offset provided by gNB.

Proposal 13: For configuration 2, single RB offset is provided by gNB to determine the starting PRB index of PUSCH transmission in SBFD symbols.

Under the umbrella of option 1, there are four equations need further down-selection:

- Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD

  - Negative values of RBoffsetSBFD are not precluded.

- Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize

- Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

- Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize

Equation 1-B employs NBWPsize for mod operation. In this case, it is highly possible that the RBstartSBFD is still out of UL subband as it is actually determined on BWP basis while BWP is typically larger than UL subband. From this perspective, we don’t think equation 1-B is sufficient.

Regarding to equation 1-C and equation 1-D, we fail to see the difference as the key step is mod operation based on UL subband. Compared with equation 1-A, they are much more complex. We think equation 1-A is sufficient as both RBstartnon-SBFD and RBoffsetSBFD are provided by gNB directly. With equation 1-A, gNB can make a wise decision on the frequency location of PUSCH transmission across SBFD symbols and non-SBFD symbols. It is also much easier for UE implementation.

Proposal 14: For configuration 2, single RB offset is provided by gNB and the following equation is used to to determine the starting PRB index of PUSCH transmission in SBFD symbols:

- Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD

  - Negative values of RBoffsetSBFD are not precluded.

Separate configuration for SBFD symbols and non-SBFD symbols

In general, it is common understanding that parameters for frequency hopping, parameter for uplink power control and parameter for PUCCH configuration should be separately determined for SBFD symbols and non-SBFD symbols respectively.

Separate configurations of FH offsets for SBFD symbols and non-SBFD symbols

For a SBFD-aware UE, it applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions. In current specification, the bit length of FH offset indication is determined by the length of FH offset list. The same procedure can be reused to determine the number of bits used to indicate FH offset in DCI.

Regarding to the case that FH offset/FH offset list is not provided for SBFD symbols, we prefer to apply FH offset/FH offset list for non-SBFD symbols to non-SBFD symbols. The reason is that gNB can fully take the responsibility to guarantee each frequency hop of PUSCH are confined within UL subband. Otherwise, gNB should provide FH offset/FH offset list for SBFD symbols.

Proposal 15: If FH offset/FH offset list is not provided for SBFD symbols, FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols.

As agreed in RAN1#118 meeting, an RB offset is introduced to determine the FDRA on SBFD symbols on top of same FDRA indication.  In short, the allocated frequency resource for each PUSCH transmission across different symbol types can be separately determined with the same number of PRBs. Based on this, if frequency hopping is enabled, how to determine the FH offset between two adjacent PUSCH repetitions located on SBFD symbol and non-SBFD symbols respectively is unclear. To simplify the discussion, we assume PUSCH repetition with inter-slot frequency hopping is employed. As shown in Figure 4, the FH offset#1 and FH offset#2 are clear which are determined by the FH offset list for non-SBFD symbol and SBFD symbol respectively. However, for the applicable FH offset between repetition#2 and repetition#3, it is ambiguous.

In order to address this issue, we propose that frequency hopping is conducted within SBFD symbol and non-SBFD symbol separately. Consequently, FH offset for non-SBFD symbol is only applied to PUSCH repetitions on non-SBFD symbols while FH offset for SBFD symbol is only applied to PUSCH repetitions on SBFD symbols.

Figure 4 Example of inter-slot frequency hopping across different symbol types

In RAN1#118bis meeting, the following mechanism is agreed for inter-slot frequency hopping:

For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations

The intention of above agreement is to separately apply inter-slot frequency hoping across SBFD symbols from that of non-SBFD symbols. From this point of view, alt 1 is more align with the intention and the procedure of determining second hop in non-SBFD symbols.

Proposal 16: Adopt the following modification on inter-slot frequency hopping in SBFD symbols:

For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications

Separate configurations of PUCCH resource for SBFD symbols and non-SBFD symbols

Similar as PUSCH, we have the following two proposals on PUCCH.

Proposal 17: If no separate configuration is provided for SBFD symbols, configurations for non-SBFD symbols are applied for SBFD symbols.

Proposal 18: Separate startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols are applied to SBFD symbols and non-SBFD symbols respectively.

intraSlotFrequencyHopping is used to enable intra-frequency hopping for a specific PUCCH resource. Despite of configuration 1 and configuration 2, separate configurations of intraSlotFrequencyHopping are needed as long as UE supports PUCCH transmission on different OFDM symbols, especially for configuration 2.

Proposal 19: Support separate configurations of intraSlotFrequencyHopping for both Configuration 1 and 2.

Separate configurations of PC parameter for SBFD symbols and non-SBFD symbols

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, three options were identified during RAN1#118 meeting:

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively.

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols.

- Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols.

Technically speaking, we believe any one of the above options is workable. Hence, we should pursue to minimize specification impacts.

If unified TCI is configured, each TCI state points to a set of uplink power control parameters which is identified by a unique ID, i.e., Uplink-powerControlId. To be specific, UE specific power control parameter for PUSCH, PUCCH and SRS provided by Uplink-powerControl is shown as below.

Uplink-powerControl-r17 ::= SEQUENCE { ul-powercontrolId-r17 Uplink-powerControlId-r17, p0AlphaSetforPUSCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforPUCCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforSRS-r17 P0AlphaSet-r17 OPTIONAL -- Need R }

If option 1 or option 3 is adopted, we believe there is no impact on the above IE. For option 3, an additional power control offset is needed between SBFD symbol and non-SBFD symbol. The modification on RRC signalling is trivial. For option 1, existing RRC signalling can be fully reused. The key point is how to determine the specific TCI state for each symbol type. Considering that different spatial domain information will be configured for different symbol type, i.e., separate SRS resource sets are provided to SBFD symbol and non-SBFD symbol respectively, there is a one-to-one mapping between TCI state ID and SBFD symbol type in nature. From this point of view, existing MAC CE mechanism to update TCI list can be fully reused.

Proposal 20: For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, option 1 is preffered with the following assumption.

- One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively.

- There is no RRC impact and no MAC CE impact.

DL-to-UL switching period between SBFD symbol and non-SBFD symbol

For unpaired spectrum, guard period is needed between DL region and UL region. Basically, the guard period is expected to provide sufficient time between DL region and UL region for DL-to-UL switching and ICI interference avoidance. For example, UE needs time to conduct DL-to-UL switching and TA adjustment. Currently the guard period is covered by flexible slot and the length of guard period is up to implementation, i.e. a value not smaller than NTA+NTA,offset. Similarly, guard period is also needed between the last OFDM symbol in DL region and the first OFDM symbol in UL subband. One example is shown in Figure 9, wherein DL-to-UL switching point between DL slot and UL subband is needed in addition to the legacy guard period. The additional DL-to-UL switching point can be configured explicitly or implicitly. However, the overhead should also be carefully evaluated as the guard period cannot be used for transmission and reception. Additional DL-to-UL switching point would reduce the spectrum utilization.

Figure 9 Examples of DL to UL switching point between DL and UL subband

For legacy TDD, uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE side. The purpose is to compensate propagation delay and provide sufficient time for Rx-to-Tx switching at gNB side.[5] The Uplink-downlink time relation can be explained by Figure 10. As there is always flexible symbol between DL symbol and UL symbol, inter-slot interference caused by time advance at UE side can be avoided and Rx-to-Tx switching gap NRx-TxTc can be guaranteed.

Figure 10 Uplink-downlink timing relation

For SBFD aware UE, UL subband is configured in DL slots. An example is shown in Figure 11, wherein assuming UL subband is configured in DL slot#2 to DL slot#5. There is a back-to-back transmission between DL slot#1 and UL subband in slot#2. As a timing advance is applied to UE, i.e., TTA defined in TS38.211, UE needs to transmit uplink in advance with TTA relative to its DL timing. As there is no guard period between DL symbol and UL subband, the following issues are observed:

- Issue#1: A SBFD aware UE needs to transmit uplink on the resources potentially scheduled with downlink, i.e., there is collision between DL and UL for the UE. Considering UE only half duplex capability, gNB has to avoid such kind of collision via scheduling. In the other words, some resources are unavailable for DL reception or UL transmission for this UE, which degrades its performance.

- Issue#2: If gNB schedules uplink transmission on the conflicting symbols, gNB suffers intra-subband self-interference as gNB needs to schedule DL for the other UEs. This case is out of scope as Rel-18 only study SBFD operation, instead of SSFD operation.

- Issue#3: The uplink transmission on the conflicting resources introduces intra-subband UE-UE CLI for the other UEs belonging to the same serving cell. It will significantly degrade legacy UE’s DL performance, which is not preferred.

Figure 11 Timing alignment due to back-to-back DL and UL

Currently, flexible symbols need to be configured between DL region and UL region. Thanks to the flexible symbols in between, NTA,offset and NTA can be employed to guarantee aligned timing at gNB side and provide sufficient gap for Rx-to-Tx switching. Hence the key point is a guard period must be provided between the last DL symbol in DL slot and the first UL symbol in UL subband. At least the following mechanisms can be considered in the further study:

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFDM symbol of UL subband is always adjacent UL symbol or flexible symbol.

Option 3 is the simplest mechanism and can fully enjoy the bonus of semi-static flexible OFDM symbols. No or trivial standard impact is expected. One example is shown in Figure 12. UL subband is configured right behand semi-static UL symbol, i.e., UL region is contiguous from SBFD aware UE point of view. The flexible symbols can be used for the guard period between DL region and UL region. Unified TA value, i.e., determined by the current mechanism, is applied to SBFD symbol and UL symbol. As the legacy gap period can be fully reused, we don’t need to introduce or define additional gap period between DL symbol and SBFD symbol. The spectrum efficiency is higher compared with option 1 and option 2. Hence, we slightly prefer option 3.

Figure 12 UL subband is adjacent to UL symbol

Proposal 21: Guard period between DL region and UL subband is needed and at least the following mechanism can be considered.

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFDM symbol of UL subband is always adjacent UL symbol or flexible symbol.

Conclusion

In this contribution, we provide our views on subband non-overlapping full duplex. We have the following observations:

Observation 1: The maximum number of transition point between DL/UL to UL/DL is a generic issue for SBFD operation.

Based on the aforementioned discussion and observation, we have the following proposals:

Proposal 1: Guard period between DL symbol and first symbol occupied by UL subband can be determined implicitly or explicitly.

Proposal 2: If UL subband is configured on semi-static flexible symbol and gNB further adjusts direction for the symbols, clarify behaviour of SBFD aware UE:

- Option 1: UE ignores UE-dedicated TDD-UL-DL configuration and SFI on SBFD symbols

- Option 2: UE determines symbol type according to direction indicated by UE-dedicated TDD-UL-DL configuraiton or SFI. The following can be starting point:

  - If the SBFD symbol is additionally indicated as a UL symbol, UE ignores SBFD subband configuraiton on this symbol and transmits uplink within active BWP.

  - Otherwise, UE treats the symbols as SBFD symbol.

Proposal 3: The maximum number of transition point between DL/UL and UL/DL within consecutive SBFD slots is determined per time window.

Proposal 4: Only one transition point is allowed within a slot consisting of both SBFD symbols and non-SBFD symbols.

- Flexible symbols between DL symbol and UL symbol can be used as guard period for switching between SBFD symbol and non-SBFD symbol.

Proposal 5: For configuring frequency location of UL subband, the following default mechanism can be considered:

- For a SBFD symbol indicated by time domain indication, a default BWP is used to determine the frequency location of the UL subband if gNB doesn’t provide relevant configuration.

Proposal 6:  For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS, precoder is only applied to the usable RBs within a PRG across DL subband boundary.

Proposal 7: For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled by DCI format 1_0 in CSS, option 1-1 is adopted if the scheduled PDSCH is a unicast PDSCH.

Proposal 8: Capture it in TS38.214 that only RBs contained in DL subband are available for PDSCH.

Proposal 9: Configuration 1 and configuration 2 is provided per cell basis, i.e., option 3.

Proposal 10: For configuration 1, the following mechanism (option 1) is used to determine valid symbol type for SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS:

- The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig.

- The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig.

- The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config.

- The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource].

Proposal 11: For configuration 1, the following applies to SP-CSI on PUSCH when applicable:

- When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- The number of PRBs for TBS determination is based on the assigned PRBs within UL usable PRBs only for PDSCH.

- SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

Proposal 12: For configuration 2, whether option 1 or option 2 is used to determine TBS is indicated by gNB.

Proposal 13: For configuration 2, single RB offset is provided by gNB to determine the starting PRB index of PUSCH transmission in SBFD symbols.

Proposal 14: For configuration 2, single RB offset is provided by gNB and the following equation is used to to determine the starting PRB index of PUSCH transmission in SBFD symbols:

- Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD

  - Negative values of RBoffsetSBFD are not precluded.

Proposal 15: If FH offset/FH offset list is not provided for SBFD symbols, FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols.

Proposal 16: Adopt the following modification on inter-slot frequency hopping in SBFD symbols:

For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications

Proposal 17: If no separate configuration is provided for SBFD symbols, configurations for non-SBFD symbols are applied for SBFD symbols.

Proposal 18: Separate startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols are applied to SBFD symbols and non-SBFD symbols respectively.

Proposal 19: Support separate configurations of intraSlotFrequencyHopping for both Configuration 1 and 2.

Proposal 20: For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, option 1 is preffered with the following assumption.

- One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively.

- There is no RRC impact and no MAC CE impact.

Proposal 21: Guard period between DL region and UL subband is needed and at least the following mechanism can be considered.

- Option 1: The guard period is configured by gNB for SBFD aware UE.

- Option 2: UE does not expect to transmit in the uplink on the first G symbols within UL subband.

- Option 3: The first OFDM symbol of UL subband is always adjacent UL symbol or flexible symbol.

Reference

  - RAN1#118bis meeting chairman notes

  - R4-2214376, Reply LS on interference modelling for duplex evolution, RAN WG4

  - TS38.214, v17.3.0

  - R4-1804119, On NR NTA offset, Intel Corporation

  - R1- 2301411, Feasibility and techniques for subband non-overlapping full duplex, Qualcomm Incorporated

  - R1-2304792, Sub-band non-overlapping full duplex, Ericsson