3GPP TSG RAN WG1 #119                                             R1-2409931

Orlando, US, November 18th – 22nd, 2024

Source:         CATT

Title:            Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI

Agenda Item:    9.12.1

Document for:   Discussion and Decision

Introduction

In RAN#109-e meeting, a new Rel-19 WI on multi-cell PUSCH/PDSCH scheduling with a single DCI was approved as follows[1].

1. Specify the support of the following for multi-cell PUSCH/PDSCH scheduling with a single DCI [RAN1] - Different SCS/carrier type among co-scheduled cells by the single DCI. - One or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI. - The maximum number of PUSCHs/PDSCHs per scheduled cell is [4 or 8]. - Note: Type-1 HARQ-ACK codebook is not enhanced for Rel-19 multi-cell scheduling. - Note: The maximum number of sub-codebooks for Type-2 HARQ-ACK codebook is not increased for Rel-19 multi-cell scheduling. - Note: UE does not expect to be configured with both single-cell multi-PUSCH/PDSCH scheduling and multi-cell multi-PUSCH/PDSCH scheduling on the same or different cells within a same PUCCH group. - Note: No new DCI format is introduced.

In this contribution, we discuss the potential specification impacts for multi-cell PUSCH/PDSCH with a single DCI.

Scenarios and general aspects

In RAN1#118bis meeting, a proposal on supported FR and SCS for multi-cell multi-PUSCH/PDSCH was discussed as followings [2].

#### Proposal 1-3 rev2: - Multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to cells with 120kHz SCS in Rel-19. - Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR1 cells and FR2-1 cells.

In existing specification, the UE capabilities on the multi-PUSCH scheduling by a single DCI can be listed as Table 1. It can be observed that the multi-PUSCH scheduling has been applicable to FR1, 120 kHz in FR2-1 and 120kHz/480kHz/960kHz in FR2-2.

Table 1: The UE capability on the multi-PUSCH scheduling by a single DCI

| Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] |
| --- | --- | --- | --- | --- |
| 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 |  | multiPUSCH-UL-grant-r16 |
| 24-1e | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS | 24-1a | multiPUSCH-SingleDCI-FR2-2-SCS-120kHz-r17 |
| 24-1g | Multiple PUSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PUSCH scheduling by single DCI for the operation with 120 kHz SCS with non-contiguous allocation |  | multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 |
| 24-4a | 480KHz SCS support for UL | 1. PRACH with 480KHz and length 139 2. 480KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS | 24-1a, 24-4 | ul-FR2-2-SCS-480kHz-r17 |
| 24-5a | 960KHz SCS support for UL | 1. PRACH with 960KHz and length 139 2. 960KHz SCS for UL data and control channels and reference signal transmission in FR2-2 3. Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS | 24-1a, 24-5 | ul-FR2-2-SCS-960kHz-r17 |
| 55-3 | Multiple PUSCHs scheduling by single DCI for non-consecutive slots in FR1 | 1. Multi-PUSCH scheduling by single DCI format 0_1 for the operation with non-contiguous allocation | 10-17 |  |

In existing specification, the UE capabilities on the multi-PDSCH scheduling by a single DCI can be listed as Table 2. It can be observed that the multi-PDSCH scheduling has been applicable to 120kHz in FR2-1 and 120kHz/480kHz/960kHz in FR2-2.

Table 2: The UE capability on the multi-PDSCH scheduling by a single DCI

| Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] |
| --- | --- | --- | --- | --- |
| 24-1d | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-2 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI | 24-1 | multiPDSCH-SingleDCI-FR2-2-SCS-120kHz-r17 |
| 24-1f | Multiple PDSCH scheduling by single DCI for 120kHz in FR2-1 | 1. Multi-PDSCH scheduling by single DCI for the operation with 120 kHz SCS 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI |  | multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 |
| 24-4 | 480KHz SCS support for DL | …. 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements … | 24-1 | dl-FR2-2-SCS-480kHz-r17 |
| 24-5 | 960KHz SCS support for DL | … 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements … | 24-1 | dl-FR2-2-SCS-960kHz-r17 |

Based on above FGs on multi-PUSCH/PDSCH scheduling, multi-cell single-PUSCH/PDSCH scheduling in Rel-18 can be extended to multi-cell multi-PUSCH/PDSCH scheduling in Rel-19. The above applicable FR and SCS can be extended to multi-cell multi-PUSCH/PDSCH scheduling in Rel-19. At the same time, it’s still a controversial part whether to support FR2-2 for DCI 0_3/1_3, thus multi-cell multi-PUSCH scheduling can be supported at least for FR1 and 120kHz in FR2-1, and multi-cell multi-PDSCH scheduling can be supported at least for 120kHz in FR2-1.

Proposal 1: For supported FR and SCS for multi-cell multi-PUSCH/PDSCH, support at least follows:

- Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR1 and 120 kHz SCS in FR2-1.

- Multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to 120 kHz SCS in FR2-1.

In Rel-18, the maximum number of scheduled cell for multi-cell scheduling is 4. One pending issue is whether the maximum number of PUSCHs/PDSCH per scheduled cell for multi-cell multi-PUSCH/PDSCH scheduling is 4 or 8. As the number of PUSCHs/PDSCHs per cell, the payload size of enhanced DCI format 0_3/1_3 will increase as well. Since the payload size of DCI format is limited by 164 bits, one important factor is how many bits are required for a cell supporting multi-PUSCHs/PDSCHs. Based on current DCI format 0_3/1_3, for a given cell, at least NDI field and RV field require more bits for the cell to support multi-PUSCHs/PDSCHs scheduling. Similar to DCI format 0_1 and DCI format 1_1 for FR 2-2, 1TB is considered for multi-PUSCH scheduled by DCI format 0_3, 2TB is considered for multi-PDSCH scheduled by DCI format 1_3. The required bits of NDI field and RV field for supporting multi-PUSCH/PDSCH per cell are analyzed in Table 1.

Table 3: Total required bits of NDI field and RV field for supporting multi-PUSCH/PDSCH per cell

| Max Num. of PUSCH/PDSCH | 4-PUSCH, 1TB(bit) | 8-PUSCH, 1TB(bit) | 4-PDSCH, 2TB(bit) | 8-PDSCH, 2TB(bit) |
| --- | --- | --- | --- | --- |
| NDI | 4 | 8 | 8 | 16 |
| RV | 4 | 8 | 8 | 16 |
| Total bit | 8 | 16 | 16 | 32 |
| Additional bit | 6 | 14 | 12 | 28 |

The total bit refers to total required bit of NDI field and RV field for supporting multi-PUSCH/PDSCH per cell in DCI format 0_3/1_3. And the additional bit are the additional required bit of NDI field and RV field for supporting multi-PUSCH/PDSCH per cell on top of current DCI format 0_3/1_3. According to above table, it showed when the maximum number of PUSCHs per scheduled cell is 4 and 8, the additional bits is 6 bits and 14 bits per cell, respectively. When the maximum number of PDSCHs per scheduled cell is 4 and 8, the additional bits is 12 bits and 28 bits per cell, respectively. Also, there is still another aspect having impact on the payload size of DCI format 0_3/1_3, which is the maximum number of cell supporting multi-PUSCHs/PDSCHs. Our view on the maximum number of PUSCHs per scheduled cell and the maximum number of cell supporting multi-PUSCHs/PDSCHs for supporting multi-cell multi-PUSCH/PDSCH scheduling can be summarized as follows:

- If the maximum number of PUSCHs per scheduled cell is defined as 4, the maximum number of cell supporting multi-PUSCHs scheduling can be defined as 4. In this case, the maximum of additional bits in DCI 0_3 is about 24 bits.

- If the maximum number of PUSCHs per scheduled cell is defined as 8, the maximum number of cell supporting multi-PUSCHs scheduling can be defined as 1. In this case, the maximum of additional bits in DCI 0_3 is about 28 bits.

- If the maximum number of PDSCHs per scheduled cell is defined as 4, the maximum number of cell supporting multi-PDSCHs scheduling can be defined as 2. In this case, the maximum of additional bits in DCI 1_3 is about 24 bits.

- If the maximum number of PDSCHs per scheduled cell is defined as 8, the maximum number of cell supporting multi- PDSCHs scheduling can be defined as 1. In this case, the maximum of additional bits in DCI 1_3 is about 28 bits.

Table 4: Additional bits for DCI format 0_3/1_3

|  | Max. number of PUSCHs/PDSCH per cell | Max. number of cell supporting multi-PUSCH/PDSCH | Additional bit |
| --- | --- | --- | --- |
| Multi-cell multi-PUSCH(1TB) | 4 | 4 | 24 bits |
|  | 8 | 2 | 28 bits |
| Multi-cell multi-PDSCH(2TB) | 4 | 2 | 24 bits |
|  | 8 | 1 | 28 bits |

Proposal 2: For multi-cell/multi-PUSCH scheduling, consider following options for the maximum number of PUSCHs per scheduled cell with 1TB and the maximum number of cell supporting multi-PUSCH scheduling in a cell set:

- Option 1: the maximum number of PUSCHs per scheduled cell is 4, and the maximum number of cell supporting multi-PUSCHs scheduling is 4.

- Option 2: the maximum number of PUSCHs per scheduled cell is 8, and the maximum number of cell supporting multi-PUSCHs scheduling is 2.

Proposal 3: For multi-cell multi-PDSCH scheduling, consider following options for maximum number of PDSCHs per scheduled cell with 2TBs and the maximum number of cell supporting multi-PDSCH scheduling in a cell set:

- Option 1: the maximum number of PDSCHs per scheduled cell is 4, and the maximum number of cell supporting multi-PDSCHs scheduling is 2.

- Option 2: the maximum number of PDSCH per scheduled cell is 8, and the maximum number of cell supporting multi-PDSCHs scheduling is 1.

DCI field

In RAN1#118bis, it was achieved two agreements on the NDI field and RV field in DCI format 0_3/1_3 for supporting multi-cell multi-PUSCH/PDSCH separately [3].

Agreement - In DCI format 0_3/1_3, for each block of NDI field, consider the following options: - Option 1: the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3. - Option 2: the number of bits is equal to the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3. - Option 3: if the number of scheduled PUSCH/PDSCH is 1, then one bit NDI is applied; otherwise, option 1 is applied. Agreement - In DCI format 0_3/1_3, for each block of RV field, consider the following options: - Option 1: the number of bits is determined based on the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the corresponding cell. - Option 2: the number of bits is determined based on the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the corresponding cell. - Option 3: if the number of scheduled PUSCH/PDSCH is 1, then option 2 is applied; otherwise, option 1 is applied.

We can start with the discussion on NDI field. For option 1, the number of bits for each block of NDI field is equal to the maximum number of scheduled PUSCH/PDSCH on each cell, thus the number of bits for entire NDI field is the sum of the maximum number of scheduled PUSCH/PDSCH of all cells. For option 2, the number of bits for each block of NDI field is equal to the actual number of scheduled PUSCHs/PDSCHs for each cell, and the number of bits for entire NDI field is the maximum number of the actual number of scheduled PUSCH/PDSCH by a DCI format. And for option 3, the bits determination for each block of NDI field depends on whether the number of scheduled PUSCH/PDSCH for the cell is 1 or not. If the number of scheduled PUSCH/PDSCH is 1, then option 2 is applied; otherwise, option 1 is applied.

From the perspective of the total bits required for NDI field, option 1 requires the most bits for NDI field and option 2 requires least bits among the above options. For example, there is a joint TDRA table for a DCI format 1_3 associated a cell set, each row index in the table implicitly indicates the number of scheduled PDSCHs for each cell as Table 5. The total bits required for NDI field in DCI format 1_3 for three options are as following:

- Option1: 16 bits

- Option 2: 9 bits

- Option 3: 13 bits

Table 5: The number of PDSCHs for each cell

| Row index | CC1 | CC2 | CC3 | CC4 |
| --- | --- | --- | --- | --- |
| 1 | 4 PDSCHs | 2 PDSCHs | 2 PDSCHs | 1 PDSCHs |
| 2 | 1 PDSCHs | 4 PDSCHs | 2 PDSCHs | 2 PDSCHs |
| 3 | 2 PDSCHs | 1 PDSCHs | 4 PDSCHs | 2PDSCHs |
| 4 | 2 PDSCHs | 2 PDSCHs | 1 PDSCHs | 4 PDSCHs |

To support multi-cell multi-PUSCH/PDSCH scheduling by a single DCI, it’s better to reduce the bits of each DCI field as much as possible to accommodate more scheduling information in DCI format 0_3/1_3. Therefore, option 2 for each block of NDI field should be supported for DCI format 0_3/1_3 in Rel-19.

Proposal 4: In DCI format 0_3/1_3, for each block of NDI field, option 2 can be supported, i.e. the number of bits is equal to the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

Also, for each block of RV field, the similar principle as NDI field should be considered to reduce the number of bits for RV field in DCI format 0_3/1_3. Therefore, option 2 for each block of RV field can be supported for DCI format 0_3/1_3 in Rel-19.

Proposal 5: In DCI format 0_3/1_3, for each block of RV field, option 2 can be supported, i.e. the number of bits is determined based on the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and the number of bits for RV configured for the corresponding cell.

HARQ-ACK enhancement

In Rel-17, a new procedure is introduced in the section of Type-2 HARQ-ACK codebook generation to support HARQ-ACK feedback for multi-PDSCHs scheduling by a DCI format 1_1. To further optimize HARQ-ACK feedback, a time domain HARQ-ACK bundling mechanism is supported for both Type-1 and Type-2 HARQ-ACK codebooks.

In RAN1#118bis meeting, it was agreed that time domain HARQ-ACK bundling is supported for HARQ-ACK feedback of multi-cell PUSCH/PDSCH scheduling with a DCI format 1_3. There are two potential alternatives on time domain HARQ-ACK bundling mechanism as followings:

- Alt-1: nrofHARQ-BundlingGroups is configured based on a serving cell

For Alt-1, each serving cell in cell set can be configured with the parameter nrofHARQ-BundlingGroups separately. And the candidate range can be as {n1, n2, n4}, aligning with as legacy operations. For each serving cell, UE generates Type-2 HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions, and the number of TBGs is provided by the nrofHARQ-BundlingGroups of the corresponding serving cell.

The benefit of Alt-1 is that it allows for flexible configuration of nrofHARQ-BundlingGroups for each cell, which can be matched with channel condition of each cell, and can be matched with the maximum number of NPDSCHmax PDSCH receptions of each cell.

- Alt-2: nrofHARQ-BundlingGroups is configured based on a cell set.

For Alt-2, the parameter of nrofHARQ-BundlingGroups is configured commonly for one cell set. Each cell in the cell set has same value to determine the maximum number of TBGs NHARQ-ACK,cTBG,max.

The benefit of Alt-2 is that the procedure is simple when UE generates Type-2 HARQ-ACK information for PDSCHs receptions scheduled by DCI format 1_3, since the number of TBGs  is same among the cells within a cell set.

Proposal 6: For the second Type-2 HARQ-ACK information for PDSCH scheduled by a multi-cell multi-PDSCH DCI, the following alternatives can be considered for the time domain HARQ-ACK bundling mechanism:

- Alt-1: nrofHARQ-BundlingGroups is configured based on serving cell

- Alt-2: nrofHARQ-BundlingGroups is configured based on cell set

For support HARQ information feedback, UE will generate corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where the value of k is indicated by DCI and the value of n is the last UL slot overlapping with the DL slot nD for the reference PDSCH reception. In Rel-18, the reference PDSCH is the latest PDSCH ending among the PDSCHs scheduled by a DCI format 1_3.

In Rel-19, when the PDSCHs within different SCS are scheduled by a DCI format 1_3, and if two or more scheduled PDSCHs have same ending time, there is more than one reference PDSCH for timing of PUCCH determination. It will result in ambiguity UL slot n .

To solve the ambiguity issue, the following alternatives can be considered:

  - Alt.1: The reference PDSCH is the PDSCH with the smallest serving cell index among the same latest PDSCHs ending.

  - Alt.2: The reference PDSCH is the PDSCH with the smallest SCS among the same latest PDSCHs ending.

  - Alt.3: The reference PDSCH is the PDSCH with the largest SCS among the same latest PDSCHs ending.

In our view, Alt.1 can be supported. Since using the PDSCH with smallest serving cell index as reference PDSCH is a legacy method for last DCI format determination and DAI counting for DCI format 1_3 in Rel-18.

Proposal 7: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of PDSCHs scheduled with different SCS by a DCI, the reference PDSCH is the PDSCH with the smallest serving cell index among the same latest PDSCHs ending.

Conclusion

In this contribution, the potential spec impacts for multi-cell PUSCH/PDSCH scheduling with a single DCI are discussed. The proposals are summarized as follows:

Proposal 1: For supported FR and SCS for multi-cell multi-PUSCH/PDSCH, support at least follows:

- Multi-cell multi-PUSCH scheduling by DCI format 0_3 is applicable to FR1 and 120 kHz SCS in FR2-1.

- Multi-cell multi-PDSCH scheduling by DCI format 1_3 is applicable to 120 kHz SCS in FR2-1.

Proposal 2: For multi-cell/multi-PUSCH scheduling, consider following options for the maximum number of PUSCHs per scheduled cell with 1TB and the maximum number of cell supporting multi-PUSCH scheduling in a cell set:

- Option 1: the maximum number of PUSCHs per scheduled cell is 4, and the maximum number of cell supporting multi-PUSCHs scheduling is 4.

- Option 2: the maximum number of PUSCHs per scheduled cell is 8, and the maximum number of cell supporting multi-PUSCHs scheduling is 2.

Proposal 3: For multi-cell multi-PDSCH scheduling, consider following options for maximum number of PDSCHs per scheduled cell with 2TBs and the maximum number of cell supporting multi-PDSCH scheduling in a cell set:

- Option 1: the maximum number of PDSCHs per scheduled cell is 4, and the maximum number of cell supporting multi-PDSCHs scheduling is 2.

- Option 2: the maximum number of PDSCH per scheduled cell is 8, and the maximum number of cell supporting multi-PDSCHs scheduling is 1.

Proposal 4: In DCI format 0_3/1_3, for each block of NDI field, option 2 can be supported, i.e. the number of bits is equal to the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

Proposal 5: In DCI format 0_3/1_3, for each block of RV field, option 2 can be supported, i.e. the number of bits is determined based on the actual number of scheduled PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the corresponding cell.

Proposal 6: For the second Type-2 HARQ-ACK information for PDSCH scheduled by a multi-cell multi-PDSCH DCI, the following alternatives can be considered for the time domain HARQ-ACK bundling mechanism:

- Alt-1: nrofHARQ-BundlingGroups is configured based on serving cell

- Alt-2: nrofHARQ-BundlingGroups is configured based on cell set

Proposal 7: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of PDSCHs scheduled with different SCS by a DCI, the reference PDSCH is the PDSCH with the smallest serving cell index among the same latest PDSCHs ending.

References

- RP-242408, New WID: Multi-carrier enhancements for NR Phase 2, Lenovo, RAN#109-e meeting

- R1-2409113, Feature lead summary #3 on multi-cell scheduling with a single DCI, Moderator (Lenovo), October 14th – 18th, 2024

- RAN1 Chair’s Notes RAN1 #118bis, Hefei, China, October 14th – 18th, 2024