3GPP TSG RAN WG1 #119			R1-2410006

Orlando, US, November 18th – 22nd, 2024

Agenda item:		9.7.1

Source:	China Telecom

Title:	Discussion on ISAC deployment scenarios

Document for:		Discussion

Introduction

In RAN# 102, a new Rel-19 study item on channel modelling for Integrated Sensing And Communication (ISAC) for NR was approved [1]. The objectives are listed as follows.

Objective: The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency.

Based on the scope of the study item, in this contribution, we provide our views on the issues of ISAC deployment scenarios [2].

Deployment scenario parameters/principles

Scenarios for AGV sensing targets

In RAN1#118-bis meeting, a preliminary table regarding AGV sensing target scenarios was formed and parameters was discussed [2]. Despite notable progress, there remains several issues to be discussed.

For the target 3D mobility, the velocity of AGV is not expected to have only two fixed values, means that Option 2 may not be consistent with the practical scenario. So, we prefer Option 1, which is also aligned with the definition of AGV mobility in InF in TR38.901 [3].

Additionally, considering the safety of factory operation, there should be a safe distance between sensing targets, and which is relevant to the speed of vehicles.

The updated parameters for scenarios for AGV sensing targets are provided in the following Table 1.

Table 1. Evaluation parameters for Automated Guided Vehicles

| Parameters |  | Values |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | InF (TR38.901 including Table 7.8-7) |
| Sensing transmitters and receivers properties NOTE2 |  | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h |
|  | 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane |
|  | Orientation | Horizontal plane only |
|  | Physical characteristics (e.g., size) | Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: Min distances based on min. TRP/UE distances defined in TR38.901 |
| Minimum 3D distance between sensing targets |  | Option B: Fixed value, [x] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS |

Proposal 1:  Adopt the evaluation parameters for AGV sensing scenarios in Table 1.

Scenarios for automotive sensing

TR 37.885 has described the evaluation methodology to be used in evaluating technical solutions to support the full set of 5G V2X use cases, including simulation assumptions and channel models [4]. Therefore, it can be a reference in the definition of automotive sensing target scenarios. In last meeting, it has been concluded that the highway and urban grid for communication evaluations incorporating road configuration is going to be studied in the autonomous sensing target scenarios definition.

Despite progress, there remains for us to exhibit the parameters of sensing targets continually. First, regarding the mobility modelling, it is clear that the speed of different vehicles differs significantly, for example, vehicles travel at much higher speeds on highways than on urban roads, so it is preferred to define the 3D mobility separately at least for these two scenarios. As referred in TR 37.885 [4], the vehicle speed in highway scenario is from 20-140 km/h along the lane, and that in urban grid scenario is from 15-60 km/h in all lanes. Specifically, for highway scenario, several kinds of typical vehicle speeds in lane including {20,30,40,60,70,80,100,140} km/h were defined, while for urban grid, vehicle speed includes {15,20,50,60}km/h were defined. These existed values can be reused.

Additionally, TR 37.885 defines three types of vehicles with different size as follows:

-	Type 1 (passenger vehicle with lower antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 0.75 meters

-	Type 2 (passenger vehicle with higher antenna position): length 5 meters, width 2.0 meters, height 1.6 meters, antenna height 1.6 meters

-	Type 3 (truck/bus): length 13 meters, width 2.6 meters, height 3 meters, antenna height 3 meters

Note that since Type 1 and Type 2 vehicles have the same sizes, only the antenna heights are different, which is irrelevant for sensing purposes, hence the two types can be combined into type 1, with the original Type 3 renamed Type 2. That is, redefine

- vehicle Type A with size of length 5 meters, width 2.0 meters, height 1.6 meters,

- vehicle Type B with size of length 13 meters, width 2.6 meters, height 3 meters.

Furthermore, since in meeting RAN1#118-bis, it was agreed to define multiple types of targets including passenger vehicle and truck/bus, even e-scooter/motorcycle/bike, the distribution of these types of vehicles will be an inevitable issue, and it is noted that vehicle type distribution per TR 37.885 as a starting point. Review that for the target distribution, TR 37.885 ruled three vehicle type distributions. Specifically, distribution of 100% vehicle Type 2; or 20% vehicle Type 1, 60% vehicle Type 2, 20% vehicle Type 3; or 0% vehicle Type 1, 67% vehicle Type 2, 33% vehicle Type 3. Three distributions apply for highway and urban grid scenarios separately, which can be a reference for automotive sensing target scenarios. Based on our redefinition of vehicle types, these three distributions are then updated accordingly as

- Option 1: 100% vehicle Type A;

- Option 2: 80% vehicle Type A, 20% vehicle Type B;

- Option 3: 67% vehicle Type A, 33% vehicle Type B;

Additionally, the specific distribution can be determined with an additional total number of vehicles.

As for the minimum distances between targets, TR 37.885 ruled that the distance between the rear bumper of a vehicle and the front bumper of the following vehicle in the same lane is max {2 meter, an exponential random variable with the average of the speed * 2 sec}. This definition reflects a fundamental understanding that vehicle spacing is directly related to average speeds. Since the average speeds of vehicles on highways and urban roads are significantly different, the definition of the minimum 3D distance between sensing targets should be based on this reasonable consideration as well. Therefore, it is acceptable to define a fixed value based on the vehicle speed for each of the two scenarios. Additionally, it is difficult to define sufficiently convincing minimum distance based on Option 1 since the exact value of near-far field breaking point is unknown to us. Therefore, Option 2 in current agreement seems more appealing to us.

The overall proposals on parameter values for scenarios with autonomous vehicles as sensing targets incorporating the above considerations can be found in in Table 2.

Table 2. Evaluation parameters for automotive sensing scenarios

| Parameters |  | Values |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Highway | Urban Grid |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Option 2: ISD between TRPs of Urban Grid is 250 meters |  |
| Sensing target | LOS/NLOS | LOS and NLOS (including NLOSv) |  |
|  | Outdoor/indoor | Outdoor |  |
|  | Mobility (horizontal plane only) | {20,30,40,60,70,80,100,140}km/h along lane(s), 0km/h otherwise; | {15,20,50,60}km/h along lane(s), 0km/h otherwise |
|  | Distribution (horizontal) | Option 1: 100% vehicle type A. Option 2: 80% vehicle type A, 20% vehicle type B Option 3: 67% vehicle type 1, 33% vehicle type B | Option 1: 100% vehicle type A. Option 2: 80% vehicle type A, 20% vehicles type B |
|  | Orientation | Lane direction in horizontal plane |  |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - passenger vehicle (type A): 5m x 2m x 1.6m - truck/bus (type B): 13m x 2.6m x 3m FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike |  |
|  | Number of targets | FFS | FFS |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |  |
| Minimum 3D distance between sensing targets |  | Fixed value, [x1] m. value of x1 is FFS | Fixed value, [x2] m. value of x2 is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS | EO Type 2 for Urban Grid FFS: details, e.g. 4 walls (as EO type 2) per building of size [413mx230mx20m] |

Proposal 2: Evaluation parameters for autonomous vehicles as sensing targets in highway and urban grid scenarios, including Mobility, Distribution, Minimum 3D distance between sensing targets, as well as Environment Objects, needs to be defined separately.

Proposal 3: Adopt the evaluation parameters for automotive target sensing scenarios as in Table 2.

Scenarios for human sensing targets

In RAN1#118-bis meeting, the evaluation parameters for sensing target scenarios including both and indoor and outdoor was discussed. Part of the necessary sections as well as the related values was defined. However, there are still a few gaps to go before final completion. Here we provide our views on these parameters.

For 3D mobility of targets, we prefer a uniform distribution with velocity below a maximum value. In addition, we think the value of 3km/h in indoor scenarios similar to the UE mobility defined in TR 38.901 [3] can be adopted. The same applies to outdoor scenarios with a larger maximum speed value, and which is recognized as 10km/h. therefore, Option 3 in the 3D mobility row can be confirmed.

The Minimum 3D distance between sensing targets is reasonably assumed to be at least larger than the physical size of a sensing target rather than a fixed value since the occurrence of individuals standing in close proximity to one another is a frequent phenomenon, whether in indoor or outdoor scenarios. Due to the differences in size between adults and children, this approach is more aligned with practical considerations than defining a fixed value.

Therefore, regarding the evaluation parameter for human sensing scenarios, we make the following updates:

Table 3. Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Indoor office, indoor factory [TR38.901] Indoor room [TR38.808] | UMi, Uma, RMa [TR38.901] |
| Sensing transmitters and receivers properties | Rx/Tx Locations NOTE 2 | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario |
|  | Rx/Tx Mobility for UEs | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr |
| Sensing target | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory) | Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory) |
|  | 3D distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment FFS: Value of N | Uniform in horizontal plane |
|  | Orientation | Random over the horizontal area | Random over the horizontal area |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 as a starting point | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 as a starting point |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target | Option 1: At least larger than the physical size of a sensing target |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS, based on outcome for AI 9.7.2 | FFS, based on outcome for AI 9.7.2 |

Proposal 4: Adopt the evaluation parameters for human target sensing scenarios as in Table 3.

Conclusions

In this contribution, we discuss ISAC deployment scenarios related issues and have following proposals:

Proposal 1: Agree on the evaluation parameters for AGV sensing scenarios as in Table 1.

Proposal 2: Evaluation parameters for autonomous vehicles as sensing targets in highway and urban grid scenarios, including Mobility, Distribution, Minimum 3D distance between sensing targets, as well as Environment Objects, needs to be defined separately.

Proposal 3: Adopt the evaluation parameters for automotive target sensing scenarios as in Table 2.

Proposal 4: Agree on the evaluation parameters for human target sensing scenarios as in Table 3.

References

- RP-234069, “Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T, Dec. 11 - 15, 2023.

- 3GPP RAN1, Chair’s Notes RAN1#118-bis, RAN1#118-bis, Hefei, China, October 14th – 18th, 2024.

- 3GPP TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz”.

- 3GPP TR 37.885 “Study on evaluation methodology of new vehicle-to-everything (V2X) use cases for LTE and NR (Release 15)”.