3GPP TSG RAN WG1 #119	R1-2410016

Orlando, USA, Nov. 18th – 22nd, 2024

Source:	Lenovo

Title:	Channel model extension for 7-24 GHz

Agenda Item:	9.8.2

Document for:	Discussion and Decision

# Introduction

In RAN1#118bis [1], the following has been agreed as part of the channel model extension study for 7-24 GHz, including near-field channel modeling and spatial non-stationarity modeling:

Conclusion The spatial non-stationarity characteristics at UE side (e.g., due to the impact of hand(s) and/or head), i.e., the antenna element-wise power variation, is supported in the channel modelling in TR 38.901 with potential updates. Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE, - Angular domain parameters - FFS: Doppler shift, Delay with Option-2 “Determined by the antenna element locations of both TRP and UE”. Note: Angular domain parameters can be optionally modelled. Conclusion For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE: - Amplitude Agreement For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE is modelled by: - For the non-direct paths, the antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster, i.e., - d1 is used to determine the 1st point associated with cluster to calculate the parameter at least for BS side; - FFS: d2 is used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side; - FFS: whether one point is sufficient - FFS: the details to generate the d1, d2,e.g.,if the d2 is needed, both d1 and d2 will be generated pair wise. - FFS: The ratio of non-direct path which show the phenomenon with spherical wavefront - FFS: The determination of the point associated with cluster is conducted in Cluster-level Agreement For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter. - Angular domain parameters, Delay (both can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. - FFS: Doppler shift Agreement For near-field channel, if necessary, to model the following antenna element-wise channel parameter of direct path between TRP and UE, - Delay (can be optionally modelled) - Working Assumption - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay. with Option-2 “Determined by the antenna element locations of both TRP and UE”. Conclusion RAN1 confirms that the modelling of near-field propagation characteristics (i.e., characteristics of spherical wavefront) is taken as an additional modelling component, and the relevant progress (e.g., the agreement(s) with/without “if necessary”) in AI 9.8.2 will be captured in TR 38.901. Agreement For modelling spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Billboard; Cartesian: w=2.4m; h=3.6m; Stationary. Outdoor; Street lamp; Cartesian: w=0.4m; h=0.8m; Stationary. Outdoor; Building edge; Cartesian: w=X m; h=Y m; Stationary. Indoor; Pillar; Cartesian: w=0.3m; h=3m; Stationary. - FFS: the value of X and Y for the blocker dimensions of building edge is needed. - FFS: the details related to the user hand/head: Indoor;Outdoor; FFS: User hand; Cartesian: w=[0.2]m; h=[0.1]m; Stationary. Indoor;Outdoor; FFS: User head; Cartesian: w=[0.24]m; h=[0.20] m; Stationary. - FFS: The details of blockage model B to implement the impact of user hand and head. - FFS: The location of the user hand/head Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B. Agreement For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120: - Option-1: generated by leveraging the existing blockage Model-B with potential updates. - At most 2 hand type blockers and one head type blocker for a specific UE is assumed. - Option-2: generated by leveraging the existing blockage Model-A with potential updates - Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity - FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

In this document we discuss further remaining aspects on channel extension for 7-24 GHz that are expected to be captured in TR 38.901 [2].

# Channel model extension for 7-24 GHz

In RAN1#118bis, modeling of near-field channels for both the direct path between the TRP and the UE as well as the corresponding non-direct path(s) has been discussed, where it has been agreed to model the angular domain parameters in an antenna element-wise fashion for the direct path and non-direct path(s), as well as supporting antenna element-wise delay modeling as an optional component. No conclusion was reached on Doppler shift handling. Furthermore, for delay modeling, a working assumption has been made corresponding to adopting the same the model in Clause 7.6.2 of TR 38.901 for large bandwidth and large antenna array. Under this model, the delay values are modeled to vary across different rays per cluster, and additionally, the number of rays per cluster would depend on the antenna aperture size. The additional modeling component does not however model delay values to be per antenna element; it models the delay values to vary based on the overall antenna array dimension. In our opinion, further clarity is needed on the applicability of the additional modeling component in Clause 7.6.2 in TR 38.901 to antenna element-wise modeling for the direct and non-direct path(s) of the near field channel model.

For antenna element-wise modeling of delay for the near-field channel for the direct and non-direct path(s), further clarification is needed on the working assumption regarding the applicability of the additional modeling component in Clause 7.6.2 of TR 38.901 to the near-field channel with antenna element-wise delay modeling.

Furthermore, no conclusion was reached on whether Doppler shift is to be modeled as an antenna element-wise parameter for the near-field channel of either of the direct path or non-direct path(s). In our opinion, the near-field region is expected to model crowded areas, e.g., malls or stadiums with a gNB placed within the crowd area, where most devices are handheld devices by pedestrians, spectators or FWA nodes, in which Doppler shift values are relatively small. On the other hand, Doppler shift values become notable for high-speed train scenarios and/or highways and vehicle roads, wherein for these scenarios it is reasonable to model the cells to be far enough from the main road to cover a larger area, i.e., the near-field region based on the Fraunhofer distance would be small with respect to the cell coverage area. Hence, our preference is to reuse the legacy Doppler modeling for the near-field channel without introducing additional modeling steps.

For antenna element-wise modeling of Doppler shift for the near-field channel for direct and non-direct path(s), reuse the legacy modeling in TR 38.901 with antenna-common Doppler shift, i.e., without introducing any additional modeling steps.

In general, the applicability of the near-field region depends on two factors: the gNB-UE distance as well as the gNB antenna aperture size. Large gNB antenna arrays are expected to be packed with a massive number of antenna elements, e.g., >1000 antenna elements, and hence element-wise modeling of non-angular parameters may incur significant complexity in simulations requiring to model channels for multiple UEs with different TRPs each, as well as modeling each gNB-UE channel with multiple clusters and multiple rays within, with further modeling of channel parameters to be different across antenna elements. While accurate channel modeling is important, the support of reasonably complex models is instrumental to enable more engaged simulation activities and ensure that simulation runtime is not exponentially larger, leading to limiting the simulation activity. Therefore, antenna common parameters or at least antenna-group common parameter design is preferred, e.g., an antenna array with K antenna elements is decomposed to G groups of M antenna elements each, wherein the M antenna elements within the same antenna group share the same parameter values, i.e., K= G.M antenna elements per gNB antenna array with G groups of non-angular parameter values are supported.

For non-angular parameters in near-field channels, the parameters are modeled to either have antenna-element common parameter values or antenna group-wise parameters, i.e., antenna elements of a sub-array of the gNB antenna array comprising multiple antenna elements are modeled to have the same parameter values.

It was also agreed in RAN1#118bis to model non-direct path(s) between a UE and a gNB for near-field channels to be based on one or two reference distance positions d1,m and d2,m associated with the BS side and the UE side, respectively, e.g., corresponding to a distance from the BS or UE, respectively, to a reflecting point for each cluster. To the best of our knowledge, no robust heuristic models or sufficiently reliable field tests have been pursued to provide a geometric method for such approach. Our preference is to use a stochastic method, where the values of d1 and d2 are generated in a pairwise fashion so as to guarantee d1,m+d2,m>dD for each cluster m, where dD is the 3D Euclidean distance between the gNB and the UE. The underlying distribution(s) for generating d1,m and d2,m should be further studied.

For modeling the non-direct path(s) between a UE and a gNB for near-field channels, the two reference distance positions d1,m and d2,m associated with the BS side and the UE side, respectively are generated in a pairwise manner via a stochastic method, conditioned that d1,m+d2,m>dD for each cluster m, where dD is the 3D Euclidean distance between the gNB and the UE.

- FFS: the underlying distribution(s) used to generate d1,m and d2,m values.

Furthermore, non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side has been discussed, with the following options due for down selection by RAN1#120:

- Option-1: generated by leveraging the existing blockage Model-B with potential updates.

  - At most 2 hand type blockers and one head type blocker for a specific UE is assumed.

- Option-2: generated by leveraging the existing blockage Model-A with potential updates.

- Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity.

  - FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity.

For Option-1, a geometric method for blockage modeling based on blockage Model B in Clause 7.6.4.2 of TR 38.901 is proposed with tabulated dimensions, i.e., the size of the blocker(s) are selected from a set of candidate tabular values, however modeling of the attenuated antenna element(s) at the UE side and their corresponding attenuation level may need to be modeled randomly, e.g., via a stochastic method, and hence, even under geometric blockage modeling approaches, the model may be partly stochastic. On the other hand, purely stochastic modeling, e.g., similar to Option-2 stochastic method for blockage Model A in Clause 7.6.4.1 of TR 38.901, better depicts the different types of blockers corresponding to different handgrips and different methods of holding a UE device and is more compatible with the shadow fading and penetration loss defined as part of the basic modeling components of different scenarios in Clause 7.4 of TR 38.901. Option-3 is not clear and can be regarded as a more generic approach that includes both random and fixed blockage values. Using fixed blockage values is easier to implement, however, randomness in hand grip and characterizing the antenna element(s) subject to attenuation needs to be modeled. Given that, we are open to further discuss Option-2 and Option-3 for non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side, with Option-1 being down prioritized.

For non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side, further discuss Option-2 and Option-3, whereas Option-1 is down prioritized.

# Conclusion

This contribution studied aspects on channel model extension for 7-24 GHz. We have the following proposals:

- For antenna element-wise modeling of delay for the near-field channel for the direct and non-direct path(s), further clarification is needed on the working assumption regarding the applicability of the additional modeling component in Clause 7.6.2 of TR 38.901 to the near-field channel with antenna element-wise delay modeling.

- For antenna element-wise modeling of Doppler shift for the near-field channel for direct and non-direct path(s), reuse the legacy modeling in TR 38.901 with antenna-common Doppler shift, i.e., without introducing any additional modeling steps.

- For non-angular parameters in near-field channels, the parameters are modeled to either have antenna-element common parameter values or antenna group-wise parameters, i.e., antenna elements of a sub-array of the gNB antenna array comprising multiple antenna elements are modeled to have the same parameter values.

- For modeling the non-direct path(s) between a UE and a gNB for near-field channels, the two reference distance positions d1,m and d2,m associated with the BS side and the UE side, respectively are generated in a pairwise manner via a stochastic method, conditioned that d1,m+d2,m>dD for each cluster m, where dD is the 3D Euclidean distance between the gNB and the UE.

- FFS: the underlying distribution(s) used to generate d1,m and d2,m values.

- For non-stationarity modeling of the attenuation per antenna element due to blockage at the UE side, further discuss Option-2 and Option-3, whereas Option-1 is down prioritized.

# References

| [1] | 3GPP TSG RAN WG1#118bis, "Draft Meeting Report," Hefei, Anhui, China, Oct. 14-18, 2024. |
| --- | --- |
| [2] | 3GPP TR 38.901, "Study on channel model for frequencies from 0.5 to 100 GHz," Mar. 2022. |