3GPP TSG-RAN WG1 Meeting #119			R1-2410093

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	9.4.2.2

Title:	Discussion on frame structure and timing aspects of A-IoT communication

Source:	OPPO

Document for:	Discussion and Decision

1. Introduction

From the past RAN1 meetings, the followed agreements/progress are reached (except for the TP templates) for the AI of frame structure and timing aspects under the NR Ambient IoT study item [1].

Agreement (RAN1#116) From RAN1 perspective, at least when a response is expected from multiple devices that are intended to be identified, an A-IoT contention-based access procedure initiated by the reader is used. Agreement (RAN1#116) For A-IoT contention-based access procedure, at least slotted-ALOHA based access is studied. Agreement (RAN1#116) At least the following time domain frame structure is studied for A-IoT R2D and D2R transmission. - For R2D transmission, - A R2D timing acquisition signal (e.g. R2D preamble) is included at least for timing acquisition and for indicating the start of the R2D transmission in time domain. - For D2R transmission, - A D2R timing acquisition signal (e.g. D2R preamble) is included at least for timing acquisition and for indicating the start of the D2R transmission in time domain. - FFS other necessary component(s), e.g. midamble, postamble, periodic sync signal, control fields, guard period Agreement (RAN1#116) For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device. - The study should consider at least following aspects - Implementation restrictions for the existing BS/UE - [Processing time is common or different for different A-IoT devices] - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)] - FFS other timing aspects Agreement (RAN1#116-bis) For R2D transmission, if OFDM-based waveform is used, the start of R2D transmission from reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation. Agreement (RAN1#116-bis) To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information. Agreement (RAN1#116-bis) For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information Agreement (RAN1#116-bis) For D2R transmission, study the necessity of midamble at least for the purpose of performing timing/frequency tracking or channel estimation or interference estimation, considering at least the following: - Modulation and Coding schemes, e.g., data modulation, line/channel coding - Receiving methods, e.g., coherent or non-coherent - D2R transmission length/packet size - Midamble overhead - Timing/frequency accuracy - Phase accuracy Agreement (RAN1#116-bis) RAN1 study the R2D transmission without midamble as the baseline if Manchester encoding is used. - FFS the necessity for the R2D transmission with midamble if PIE is used. Agreement (RAN1#117) Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission. Agreement (RAN1#117) Scheduling information of PDRCH transmission is provided by a corresponding PRDCH. Conclusion (RAN1#117) RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2. Agreement (RAN1#117) Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D Agreement (RAN1#118) When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices Agreement (RAN1#118) Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following - How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 - How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1 Agreement (RAN1#118) For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection. Agreement (RAN1#118-bis) TP in R1-2409187 is endorsed in principle for section 6.2 and Annex X of TR38.769, subject to possible revisions. - The contents in the Tables are further checked by companies and targeted for completion in RAN1#119. Agreement (RAN1#118-bis) The start indicator part of the R2D time acquisition signal is not included in TD2R_min. Agreement (RAN1#118-bis) For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code. Agreement (RAN1#118-bis) For R2D waveform generation using DFT-s-OFDM-based transmitter, from transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used. Agreement (RAN1#118-bis) - The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options: - Option 1: D2R preamble only - Option 2: D2R preamble + X midamble(s), where X 1 - Option 3: D2R preamble + postamble - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 - For the above options, companies are encouraged to report at least the following: - Purpose(s) of the preamble, midamble and postamble - Whether companies assume multiple options can be supported Agreement (RAN1#118-bis) - For analysing the trade-offs among the D2R amble(s) options, companies can refer to the Table 3.2.4 in section 3.2.4 of R1-2408993 for information. Agreement (RAN1#118-bis) RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). - The study includes - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain - Determination of the X time domain resource(s) by the device - Addressing timing errors for adjacent time domain resources due to residual SFO of the device Agreement (RAN1#118-bis) Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following - How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3 Agreement (RAN1#118-bis) RAN1 studies the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices Agreement (RAN1#118-bis) RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception. Agreement (RAN1#118-bis) From RAN1 perspective, capture in the TR that at least the following aspect of the air interface for DO-DTT and DT traffic types is not sufficient for the DO-A traffic type. - For DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic. Conclusion (RAN1#118-bis) For the purpose of the Rel-19 study, the study on DO-A in RAN1 is considered completed based on the above agreement.

During RAN#103 and RAN#105 meetings, further clarifications were made on the study scope of NR A-IoT. In particular, the following endorsed proposal in [2] and conclusion in [3] would have impact to the discussion under this AI.

Proposal 2 (RAN#103) - Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19 - The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.” - No SID revision is necessary Conclusion 9 (RAN#105) RAN reiterates that “study a harmonized framework with minimized differences (where necessary)” includes studying minimized differences (where necessary) between devices, e.g. for frequency synchronization between device 1/2a and device 2b, etc.

Based on the above agreements/endorsement so far in this SI, in this contribution we provide discussions and our views on the following aspects related to this agenda item.

- Device operating states (ON / OFF / SLEEP)

- Synchronization and frame structure aspects

- Random access aspects

- Scheduling and timing aspects

- Energy harvesting aspects

During the RAN#103 meeting, the following proposal was agreed for the study of potential impacts of energy harvesting in a device.

Proposal 2 - Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19 - The potential impact of energy harvesting on device availability for transmission and reception procedures can be considered for the study [RAN2, RAN1] - Duration of one device’s unavailability due to charging by energy harvesting can be assumed up to several tens of seconds - Note: this value can be revisited in future RAN plenary meetings, if necessary - TR 38.848 clause 5.6 statement on latency remains the case with respect to a single device, i.e.: “NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only.” - No SID revision is necessary

From this clarification agreement on device energy harvesting, it is our understanding that the assumption of “up to several tens of seconds” of one device’s unavailability time due to charging is derived base on RF energy harvesting. Unless this value is changed and/or further clarified the type of energy source was assumed in RAN plenary, RAN1 should consider that device energy harvesting is based only on RF energy in the device architecture discussion. Furthermore, although it is possible that all types of energy source could be deployed in practice (e.g., thermal, kinetic, solar, wind, etc.) and no restriction should be assumed/placed on device implementation to use any of the possible energy sources, but it is only the RF energy source that would have impact to device unavailability due to charging by energy harvesting and impact to our A-IoT communication system design (impact to the Tx and Rx operation). Therefore, only the RF energy harvesting needs to be considered during the study.

Observation 1: The energy source that would cause a device unavailability for Rx and Tx in A-IoT communication due to charging by energy harvesting is only the RF energy source. When thermal, kinetic, solar, and wind energy sources are used in practice, it is reasonable to assume that they will not cause device unavailability due to charging by energy harvesting, as the device can perform energy harvesting and communication at the same time without any significant limitation/degradation to either function.

Proposal 1: It is proposed that only the RF energy source needs to be assumed during the Rel-19 study for A-IoT.

During the RAN1#118 and RAN1#118-bis meetings, the following agreement on Direction 1 and Direction 2 for device energy harvesting was made, and a TP that collects potential solutions, results/analysis and spec impacts was agreed in principle for TR 38.769. To facilitate future discussions and solution design (which may be carried out during the WI phase), RAN1 should also discuss different operating states and the associated functions in each state that can be assumed in a device in order to design the operation for energy harvesting.

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection. Agreement TP in R1-2409187 is endorsed in principle for section 6.2 and Annex X of TR38.769, subject to possible revisions. - The contents in the Tables are further checked by companies and targeted for completion in RAN1#119.

For both Direction 1 and Direction 2 in the above agreement, only two “modes” are mentioned (available and unavailable). But it is yet unclear what function(s) can assume to be operating in each of these “modes”, especially for the unavailable mode. If the unavailable mode includes both the ON and SLEEP states as described below marked in yellow highlights (which was discussed in RAN#104), it is unclear a timer can be maintained in the unavailable mode, where the timer in a device can be used for state transition (e.g., from unavailable to available). Furthermore, in a typical implementation of devices/tags, the switching from the OFF to ON states is based on whether the energy storage level has been charged to a certain power-ON threshold, and the switching from the ON to OFF state is based on whether the energy storage level is completely depleted. The only time that a device would perform energy harvesting is only during the OFF state. This means, if a device operates either in available mode or unavailable mode only, RAN1 cannot study and design solutions for energy harvesting since it is unknown whether a device is able to retain a memory, run a clock and wake-up at a certain timing to communicate. Although from the reader’s point of view, devices appear only to be either available for unavailable for communication, but from device point of view, it may be in a SLEEP state and capable of keeping a timer to wake-up at a certain timing (not based on energy storage status). In this sense, it is beneficial to define three states for the devices (i.e., ON / OFF / SLEEP) to serve as a basis for energy harvesting solution study and discussion in RAN1. Since this topic has been extensively discussed in RAN1#118, the latest proposal (FL3 High Priority Proposal 4.1-1d) in [4] is a good starting point for this meeting.

- RAN WGs are directed to study energy harvesting impacts in the following way: - Device 1 is assumed to have two states: ON, OFF - Device 2a/2b is assumed to have three states: ON, OFF, SLEEP - Identify function(s) of the device that can be assumed supported and assumed not supported in each of the above device states, subject to: - ON state supports at least: transmission, reception for communication - OFF state does not support at least: transmission, reception for communication - OFF state supports at least: energy harvesting - SLEEP state supports at least: - maintaining a memory content from ON state - Maintaining a timer (RAN1 to discuss purpose(s) of timer) - SLEEP state does not support at least: transmission - No additional physical layer signals/channels specific to support of SLEEP are introduced - Identify reader knowledge / control of the above states - Approximately identify durations of the above device states, by company reports. RAN1 is not required to establish a consensus model. - Companies can structure their report(s) of durations in their preferred way - No modeling of device/component (dis)charging characteristics - This part is aimed to be completed in RAN1#118, with RAN#105 to review progress and finalize if necessary

Proposal 2: The following device state definitions should be used for designing device energy harvesting solutions

- Case 1: Device is assumed to have two states: ON, OFF

- Case 2: Device is assumed to have three states: ON, OFF, SLEEP

- Identify function(s) of the device that can be assumed supported and assumed not supported in each of the above device states, subject to:

  - ON state supports at least: transmission, reception for communication

  - OFF state does not support at least: transmission, reception for communication, running a clock

  - OFF state supports at least: energy harvesting

  - SLEEP state supports at least:

    - Maintaining a memory content from ON state

    - Running a clock or maintaining a timer (RAN1 to discuss purpose(s) of clock)

    - Energy harvesting

    - FFS: monitoring some reception

  - SLEEP state does not support at least: transmission, reception for communication

For Direction 2, where the reader can provide information to a device based on which the device may become available or unavailable, it is detailed in our paper [5] that a device should perform energy harvesting in two separate stages in order to maximize the energy stored and prolong the device sustainable operating time. The two states are: before and after an inventory / command process starts.

During the first stage of device energy harvesting before an inventory / command process starts, besides the intention of maximizing the energy storage charge in each device for the following inventory/command process, the goal should also to avoid every device trying to access/participate in a same inventory process once it is triggered. If too many devices try to access the same inventory process together that exceeds the capacity (i.e., number of expected or allocated Msg. 1 resources), transmission collision(s) will occur. If the reader allocates a large number of resources trying to cater for all potential devices in a same process,

  - firstly, it is questionable whether there can be many frequency domain resources for small / large frequency shifts,

  - secondly, it is also questionable whether there can be many time domain resources considering device SFO problem and available device energy, and

  - thirdly, it will be wasteful if the reader over-allocates time and frequency domain resources.

Considering the above, it is more beneficial if devices are allowed to access at different timing, rather than trying to get every device to access / participate in a same inventory process together. When devices are accessing at different times / inventory processes, it also allows devices more time to harvest energy / maximize energy storage before a chosen or assigned inventory process starts (since devices are expected to charge at different rates). Therefore, it is necessary and desirable to manage/coordinate the access timing among the devices. If all devices try to access / respond to the reader at once when a select/paging message is received from the reader, although the Q protocol from the slotted-ALOHA may randomize the access timing among the devices (but not a very efficient mechanism), there still a high probability that a TX collision/conflict will occur.

Observation 2: It is undesirable for all devices, after charging, try to respond to the reader at once when a select/query is received due to TX collision, although the Q protocol from the slotted-ALOHA may randomize the access timing among the devices within an inventory process (but not a very efficient mechanism). Even if the reader allocates a large number of resources trying to cater for all potential devices in a same process, the following issues still remain.

  - firstly, it is questionable whether there can be many frequency domain resources for small / large frequency shifts,

  - secondly, it is also questionable whether there can be many time domain resources considering device SFO problem and available device energy, and

  - thirdly, it will be wasteful if the reader over-allocates time and frequency domain resources.

To resolve this over-crowding device issue, the use of a periodic signal (e.g., a dedicated synchronization or the existing R2D preamble) could further randomize / coordinate the inventory/command process timing among the devices to avoid a large number of devices trying to access at the same time causing conflicts / collisions. When the devices are distributed across different inventory/command processes, it reduces the probability of conflict/collision within a process (thus improves the success rate and reduces the required time completing a process using the slotted-ALOHA contention algorithm). Furthermore, it allows a device to continue and to have more opportunities for energy harvesting / charging over inventory/command processes that are not allocated/selected to the device.

Figure 1: Periodic timing acquisition signals (preambles) for distribute devices across multiple inventory/command processes to minimize collision probability and to reduce device power consumption.

In the above Figure 1, when periodic preamble signals are sent by a reader, the time gaps between the preambles can be used for individual inventory/command process (n, n+1, n+2, …) and devices are coordinated/distributed in different processes to reduce the probability of conflict/collision. As such, a device only needs to (wake up slightly earlier to account for SFO) to monitor the R2D TAS (DL preamble) at the beginning of the allocated or scheduled inventory/command process. During the time of other processes, the device would switch to a SLEEP state for continuous energy harvesting (very little power consumption for the clock) and not monitoring R2D preamble transmissions from the reader. In Figure 2, this is illustrated for a device power-ON before the inventory/command process (n) and starts to deplete its energy by monitoring a R2D TAS (preamble). Once the device has detected the R2D TAS (preamble) for the process (n) and acquired the coordination or scheduling information from the reader, the device switches to the SLEEP state for energy harvesting. In this case, it is assumed the received coordination / scheduling information in the R2D TAS (preamble) from the reader indicates process (n+2) for the device. Hence, the device goes to sleep and wakes up a short time just before the process (n+2) for access. As seen, for the Stage 1 of energy harvesting of the A-IoT device, the device is able to continue its energy harvesting and keep its energy storage level full/high until the intended / scheduled inventory/command process timing. Alternatively, the selection of an inventory/command process for A-IoT communication access could be performed / determined by the A-IoT device itself, by randomly selecting a process number among a total number of processes indicated by the reader in the R2D TAS (preamble). In this way, it could greatly reduce the scheduling effort and signaling in the R2D TAS (preamble) from the reader.

Proposal 3: For Stage 1 of energy harvesting (before inventory / command process starts), periodic transmissions of R2D preamble should be studied to coordinate or schedule inventory / command processes among the devices, to minimize collision probability and to reduce device power consumption from blind monitoring (and hence more time for energy harvesting).

- Synchronization and frame structure aspects

From the past RAN1 meetings, the following agreements relating to synchronization and frame structures are reached so far for A-IoT R2D and D2R transmissions.

Agreement At least the following time domain frame structure is studied for A-IoT R2D and D2R transmission. - For R2D transmission, - A R2D timing acquisition signal (e.g. R2D preamble) is included at least for timing acquisition and for indicating the start of the R2D transmission in time domain. - For D2R transmission, - A D2R timing acquisition signal (e.g. D2R preamble) is included at least for timing acquisition and for indicating the start of the D2R transmission in time domain. - FFS other necessary component(s), e.g. midamble, postamble, periodic sync signal, control fields, guard period Agreement For the R2D timing acquisition signal immediately preceding the transmission of a physical channel, study a preamble with at least two parts which includes a start-indicator part and a clock-acquisition part, where the start-indicator part immediately precedes the clock-acquisition part: - Start-indicator part provides the start of the R2D transmission - FFS: Details of start-indicator part - Clock-acquisition part provides at least the chip synchronization of the subsequent physical channel transmission - FFS: Details of clock-acquisition part, e.g. structure, encoding, length, etc. - FFS: Methods to determine chip duration of the subsequent physical channel transmission - FFS: Other functionalities - Note: the preamble is considered not to be part of a physical channel - FFS: other part(s) of the preamble, if any - FFS: whether the above clock acquisition is sufficient for all devices - FFS: how to make the preamble compact Agreement For R2D transmission, if OFDM-based waveform is used, the start of R2D transmission from reader perspective is assumed to be aligned with the boundary of an NR OFDM symbol (including the CP) for in-band/guard-band operation. Agreement To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information. Agreement For the reader to acquire the end of PDRCH transmission, study at least following options: - Option 1: D2R postamble immediately follows the PDRCH - Option 2: Based on control information Agreement For D2R transmission, study the necessity of midamble at least for the purpose of performing timing/frequency tracking or channel estimation or interference estimation, considering at least the following: - Modulation and Coding schemes, e.g., data modulation, line/channel coding - Receiving methods, e.g., coherent or non-coherent - D2R transmission length/packet size - Midamble overhead - Timing/frequency accuracy - Phase accuracy Agreement RAN1 study the R2D transmission without midamble as the baseline if Manchester encoding is used. - FFS the necessity for the R2D transmission with midamble if PIE is used. Agreement Study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed), and if so for how long after receiving an R2D transmission. Agreement The start indicator part of the R2D time acquisition signal is not included in TD2R_min. Agreement For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code. Agreement For R2D waveform generation using DFT-s-OFDM-based transmitter, from transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used. Agreement - The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options: - Option 1: D2R preamble only - Option 2: D2R preamble + X midamble(s), where X 1 - Option 3: D2R preamble + postamble - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 - For the above options, companies are encouraged to report at least the following: - Purpose(s) of the preamble, midamble and postamble - Whether companies assume multiple options can be supported Agreement - For analysing the trade-offs among the D2R amble(s) options, companies can refer to the Table 3.2.4 in section 3.2.4 of R1-2408993 for information.

- R2D time domain frame structure

On determining or deriving the end of a PRDCH transmission at the device, two options are listed in the last meeting. In Option 1, the end of a PRDCH transmission is indicated by transmitting by the reader a R2D postamble immediately follows the PRDCH. This means an additional signal needs to be defined and to be detected by the device, more resources are needed for the postamble transmission, and adding latency delay to the decoding. Since PHY layer control information is anyway need to be defined (under this AI and from other AIs), adding another parameter field seems to be a simpler approach. Of course, sending control information in a R2D preamble is also an option without increasing decoding complexity and delays.

For the same reasons, the reader should acquire the end of a PDRCH transmission based on control information.

Proposal 4: On determining or deriving the end of a PRDCH transmission at the device, it is proposed to be based on R2D control information (possibly using the R2D preamble).

Proposal 5: On acquiring the end of a PDRCH transmission at the reader, it is proposed to be based on D2R control information (possibly using the D2R preamble).

- D2R preamble, midamble, postamble

As captured above, it was agreed to analysis and evaluate the following 4 options for supporting preamble, midamble and/or postamble in D2R transmission.

  - Option 1: D2R preamble only

  - Option 2: D2R preamble + X midamble(s), where X 1

  - Option 3: D2R preamble + postamble

  - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1

It is generally understood that D2R ambles are intended to help the reader to decode transmissions from the devices. And hence, the D2R ambles should be used for the purposes of time/frequency tracking due to device residual SFO (after calibration based on a preceding R2D transmission), channel estimation for D2R data demodulation/decoding, and interference estimation if necessary to suppress any interference coming from inter-device transmissions or Uu operation.

However, as SFO of a device considered in this release is up to 105 ppm, line coding should be used for timing tracking, mid-amble for timing tracking is not necessary. Furthermore, D2R transmission length is expected to be <142ms (assuming 1000bit, 7 kbps), based on the study so far, the SFO of the clock for small frequency shift is >104, the SFO drift during 142ms is trivial even we are assuming 10ppm/s clock drift speed. Therefore, it is sufficient to use preamble for frequency tracking. In addition, preamble and vacant duration can be used for interference measurement, there is no need to introduce mid-amble for this purpose.

We also evaluated the BLER performance of D2R under 1kbps, OOK, Manchester, and 1/3 TBCC, coherent detector is used at the reader, the results are shown in Figure 2 and Figure 3 below. In our evaluation mid-amble and post-amble if added are used for channel estimation. It can be observed that no obvious gain can be achieved by the addition of preamble (and also post-amble).

Figure 2

Figure 3

Observation 3: When OOK is used in PDRCH transmissions, there is no noticeable performance gain from using midamble(s) and/or postamble, regardless of the payload size (96 and 400 bits).

Proposal 6: Based on the above observation from the evaluation results shown, it is proposed to support Option 1 (D2R preamble only) since the performance gain from using D2R midamble and D2R postamble is not noticeable.

Proposal 7: It is proposed to capture the following trade-off analysis table for D2R amble options in TR 38.769.

| Option(s) | Purpose(s) for amble(s) | Assumptions | Aspects to be considered for trade-offs analysis (observations) |
| --- | --- | --- | --- |
| - Opt.1: D2R preamble only - Opt.2: D2R preamble + X midamble(s) - Opt.3: D2R preamble + postamble - Opt.4: D2R preamble + Y midamble(s) + postamble | Channel estimation | - Device residual SFO: 104 ppm - SFO estimation: perfect - PDRCH data rate:1 kbps - Payload: 96 / 400 bits (672 / 2496 chips) - Coding scheme: 1/2 Manchester coding, 1/3 TBCC - Modulation: OOK - Coherent receiver - X = 1 (96 bits) - X=3, Y = 2 (400 bits) - Preamble: 63 chips - Midamble: 31 chips - Postamble size: 31 chips | - When OOK is used in PDRCH transmissions, there is no noticeable performance gain from using midamble(s) and/or postamble, regardless of the payload size (96 and 400 bits). - Amble(s) overhead for 96 bits D2R - Option 1: 9.4% - Option 2: 13.4% - Option 3: 13.4% - Amble(s) overhead for 400 bits D2R - Option 1: 2.52% - Option 2: 6.25% - Option 3: 3.77% - Option 4: 6.25% |

- Frequency synchronization for device 2b

So far for the study of synchronization aspect of Rel-19 A-IoT, the main focus has been on the studying necessary signals/channels and techniques that are needed and harmonized/common for all device types. That is,

1)	a R2D and D2R timing acquisition signal (e.g. preamble) is included at least for timing acquisition,

2)	the clock-acquisition part of a R2D preamble provides at least the chip synchronization of the subsequent physical channel transmission,

3)	study the necessity of midamble and postamble for D2R transmission for the purposes of performing timing/frequency tracking, channel and interference estimation, and

4)	study whether/how an A-IoT device can count the time with sufficient accuracy (with a certain timing error due to SFO) at least for the purposes related to TDM(A) (if needed).

While it is understandable the study has been predominantly focused on harmonized design aspects for all device types, but other necessary design aspects that may be specific to a certain device type (i.e., Device 2a and 2b) should be considered during the study as well. For instance, RAN1 has done extensive studies on large frequency shift that is specific only to device 2a. In RAN#105, the following conclusion in [3] is endorsed.

Conclusion 9: RAN reiterates that “study a harmonized framework with minimized differences (where necessary)” includes studying minimized differences (where necessary) between devices, e.g. for frequency synchronization between device 1/2a and device 2b, etc.

Based on the above listed agreements, frequency synchronization aspect has not been considered so far at least during online discussion. In our understanding, the tracking of frequency carrier in R2D for device 1 and 2a is not necessary as the RF receiver at the device will operate in a wideband manner at a frequency range that is preset by the manufacturer (i.e., needed for the operator or vendor). Furthermore, D2R transmissions for device 1 and 2a are backscattering based, and thus, fully reliant on a carrier waveform provided by the reader or an external node. Therefore, there is no frequency synchronization issue that needs to be resolved (including small frequency shift). For large frequency shift, it is already concluded that device 2a requires a clock for IF generation which needs to be accurate enough to avoid large guard band and interference to adjacent channels/bands, and the large frequency shift may result in [e.g., 5kHz~50kHz] of frequency uncertainty in target frequency for clock accuracy of [e.g., 0.01%~0.1%] assuming the large frequency shift range is 50MHz. Based on this conclusion and observation, it is understood that no further study is needed in RAN1 on this aspect.

For Device 2b, on the other hand, the same cannot be assumed since the active transmission is based on a local oscillator (LO), a mixer and a power amplifier to generate a D2R carrier at a desired frequency that may be configured/indicated by the reader. Under the device architecture agenda (AI 9.4.1.2), it is already agreed that a feasible LO implementation for Device 2b should be based on a FLL/PLL to synthesize the desired frequency. Depending on the receiver type for Device 2b (RF-ED, ZIF or IF receiver), the same or separate LOs for Tx and Rx may be used. It is understood that when the same LO is used for both Tx and Rx, when the clock for the LO can be calibrated according to a R2D timing acquisition signal (TAS) / preamble by observing the rising and falling edges of a chip. But when separate LOs are used for Tx and Rx, a reference frequency is needed for the FLL/PLL to perform the frequency locking operation and then the LO will adjust the output frequency according to the specified/desired frequency separation for the FDD band. This operation can also work for the case when the same LO is used for both Tx and Rx. As such, an initial reference frequency would be needed for device 2b and the best reference frequency would be a dedicated sync signal transmitted in the R2D direction.

Proposal 8: Since a reference frequency is needed when separate LOs are used for Tx and Rx in device 2b, the existing R2D timing acquisition signal / preamble may not be sufficient or workable. It is proposed that a new / dedicated synchronization signal is needed for synthesizing the frequency needed for D2R transmissions in device 2b.

4. Random access aspects

- Msg. 1 transmission

The following agreement of Msg. 1 transmission were achieved in RAN1#118-bis.

Agreement RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). - The study includes - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain - Determination of the X time domain resource(s) by the device - Addressing timing errors for adjacent time domain resources due to residual SFO of the device

During RACH procedure, a R2D transmission should be associated with X ≥ 1 time domain resources to increase system capacity and at the same time minimizing Tx collision probability, considering a reader may need to make an inventory for hundreds or thousands of devices. The maximum number of X time domain resources may depend on several factors and some of which may not be controllable or pre-definable in 3GPP. For instance, the exact power consumption of a device cannot be known by 3GPP (as it depends on the actual implementation) and it varies dramatically between device 1 and device 2. In a system where only device 1 is deployed, the maximum value for X could be much larger than a system where only device 2 is deployed.  Furthermore, the residual SFO after calibration based on a prior R2D transmission could also dramatically varying between device 1 and device 2. If the residual SFO for device 2 is much lower than device 1, the X value could vary as well. In general, when the residual SFO is low, the X can be larger. In the end, the X time domain resources for Msg. 1 transmission in a system would need to take at least these two factors into account. In addition, the maximum value for X also depends on how many devices can a reader process concurrently and what is the capacity / number of devices can be Msg. 2 transmission respond to. But these design details depend on the design of Msg. 2 and possibly also Msg. 3 transmissions. This could be also tie to the RA procedure design in RAN2. It is, therefore, proposed to postpone on deciding a maximum value for X. In the end, a different approach may need to be taken to address this issue, and/or 3GPP could simply provide a range of values for X and leave the decision of how many time domain resources to allocate by the reader based on the actual deployment.

As for the size(s) of a time domain resource, it also depends on the details of Msg. 1 transmission, e.g., a RN16 or a RACH sequence based on Gold or m-sequence.

For contention based random access, a device autonomously determines one time domain resource within the X ≥ 1 time domain resources for Msg. 1 transmission. Similar mechanism as RFID can be used. For example, the device can determine the time domain resource based on a random number.  If FDMA is supported for Msg. 1 transmission, device can further determine frequence resource for a specific time domain resource. To minimize the collision probability among devices, the frequency resource can be determined by device identity, or device type, etc.

Proposal 9: A R2D transmission triggering random access determines X time domain resource(s) for Msg. 1 transmission(s), where X should be ≥ 1 for higher system capacity and lower Tx collision probability.

Proposal 10: For Msg. 1 transmission, the following factors can be considered by device for determining time/frequency resources: random number, device ID, device type, etc.

- Msg. 2 transmission

Agreements of Msg. 2 transmission were as follows:

Agreement RAN1 studies the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices Agreement RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception.

If TDMA is applied for Msg. 1 transmission, when reader receives Msg. 1 from device(s), the reader needs to send Msg. 2 to the devices in response to Msg 1 transmission. Figure 4(a) and Figure 4(b) below correspond to Option 1 and Option 2 respectively.  For Option 1, the transmission time of each Msg. 2 can be determined by TD2R_min of associated Msg. 1 transmission. If TD2R_max is introduced to limit the maximum delay of Msg. 2 transmission, the Msg. 2 transmission should be within [TD2R_min , TD2R_max]. For Option 2, the transmission time of Msg. 2 is determined by the last Msg. 1 transmission opportunity, as shown in Figure 4(b).

Figure 4

If FDMA is applied for Msg. 1 transmission, reader can receive multiple Msg. 1 transmissions from multiple devices simultaneously but using different frequency resources.  Figure 5(a) and Figure 5(b) below correspond to Option 1 and Option 2 respectively. For Option 1, reader needs to transmit multiple Msg. 2 sequentially corresponds to multiple received Msg. 1. The transmission time of first Msg. 2 can be determined by TD2R_min of associated Msg. 1 transmission. The transmission time of other Msg. 2 can be determined by previous Msg. 2 transmission. TD2R_max can be introduced to limit the maximum delay of Msg 2 transmission. Device can monitor Msg. 2 within [TD2R_min , TD2R_max]. For Option 2, the transmission time of Msg. 2 is determined by the Msg. 1 transmission opportunity, such as Msg. 2 is transmitted after TD2R_min.

Figure 5

Proposal 11: For Msg. 2 transmission, TD2R_max can be introduced to limit the maximum delay of Msg. 2 transmission. The time range for Msg. 2 monitoring for Msg. 2 reception is [TD2R_min , TD2R_max].

- Msg. 3 transmission

Agreements of Msg. 3 transmission were as follows:

Agreement Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following - How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3

Regarding multiple access mechanism of Msg. 3, both TDMA and FDMA can be supported. If TDMA is applied to Msg. 1 transmission, TDMA is also preferred for Msg. 3 transmission. If FDMA is applied to Msg. 1 transmission, either TDMA or FDMA can be considered for Msg. 3 transmission. In case FDMA is applied, and if Msg. 2 is received by device and the associated ID in Msg. 2 is same as the ID transmitted in Msg 1, which means the frequency resource of Msg. 1 is workable, so that the same frequency resource as Msg. 1 transmission can be used for Msg. 3 transmission.

Proposal 12: For Msg. 3 transmission, following options can be considered:

- Option 1: Msg. 1 is TDMA, Msg. 3 is TDMA;

- Option 2: Msg. 1 is FDMA, Msg. 3 is TDMA;

- Option 3: Msg. 1 is FDMA, Msg. 3 is FDMA;

  - The same frequency resource as Msg. 1 transmission can be used for Msg. 3 transmission.

5. Scheduling and timing aspects

5.1 Multiple time gaps for TR2D, TD2R, TR2D_R2D and TD2R_D2R

Multiple time gaps are introduced in RAN1#116 meeting to assist further discussion.

Agreement For further discussion, the following terminologies are used for A-IoT for studying processing time aspects: - TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it. - TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it. - TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device. - TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device. - The study should consider at least following aspects - Implementation restrictions for the existing BS/UE - [Processing time is common or different for different A-IoT devices] - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)] - FFS other timing aspects

Regarding the maximum time between R2D transmission and associated D2R transmission following it, the following are agreed in RAN1#117 meeting.

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

Comparing between the above two options, we prefer option 2, where the D2R transmission time is determined based on control information of reader. Considering FDMA needs to be studied in A-IoT system for D2R transmission, option 2 is beneficial for FDMA. A set of A-IoT devices can be scheduled to perform D2R transmission simultaneously by control information. For option 1 where only maximum time gap is defined, it is hardly to support FDMA.

Proposal 13: The following option can be supported in A-IoT system

- Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min

  - FFS the maximum value(s) for TR2D

For different type of A-IoT devices, the architectures of transmitter and receiver are different. For example, for device 1, only RF ED receiver is assumed, transmission is based on backscatter. For device 2a, RF ED, IF ED or ZIF receiver can be assumed, transmission is based on backscatter. For device 2b, RF ED, IF ED or ZIF receiver can be assumed, transmission is based on active transmission. Different architecture of transmitter or receiver may correspond to different processing time. While from reader perspective, it is hardly for the reader to know which type of device is performing reception or transmission, especially during access procedure. So that even if different process time is defined for different type of devices, it is hardly for the reader to exactly know the time gaps between 2 adjacent transmissions. Therefore, it is preferred to define common processing time for all device types.

Proposal 14: Processing time is common for different A-IoT device types.

Regarding the relationship between processing time and traffic/command type, we think there is no necessary to combine them together.  Otherwise, that will complex the scheduling procedure and need additional effort of specification work. Furthermore, the benefit for defining different processing time for different traffic type is not clear.

Proposal 15: Processing time is common for different traffic types or command types.

The following was agreed in RAN1#118 meeting.

Agreement When a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device, study to define a maximum time TD2R_max between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. - FFS: whether there is a necessity to define different maximum times for different A-IoT devices

It was agreed that for Msg. 2 transmission in response to Msg. 1 transmission, the R2D transmission timing is expected to be within [TD2R_min, TD2R_max]. One open issue is whether needs to define different maximum times for different A-IoT devices. In our view, a same maximum time can be defined and applied to all kinds of A-IoT devices. When device has transmitted Msg. 1, it can turn to reception state for following R2D reception. Same transition time for T2R (transmission to reception) can be expected from all types of devices.

Proposal 16: A same maximum time can be defined and applied to all kinds of A-IoT devices.

The above time gaps are introduced between reader and device. In topology 1, the reader is gNB. While in topology 2, the reader is intermediate UE. There is another link between gNB and intermediate UE, and the scheduling information or command transmitted by intermediate UE is obtained from gNB. The following time gap maybe related to topology, such as TD2R_min, TR2D_R2D_min. For TD2R_min, when the intermediate UE receives a D2R message from device, it can forward the message to gNB and obtain response from gNB and forward it to the device. The time gap of TD2R_min of topology 2 also includes the time for message transmission between intermediate UE and gNB, which is not included in topology 1. Similarly, for TR2D_R2D_min, the R2D message transmitted by intermediate UE is obtained from gNB in topology 2, so that the time gap maybe different for topology 1 and topology 2.

Proposal 17: The following processing time may be different for different topologies.

- TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it.

- TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device.

5.2 Scheduling aspects

In legacy NR system, both dynamic scheduling and configured scheduling are supported. Dynamic scheduling is used for one shot scheduling, which is based on DCI. This mechanism can also be supported for A-IoT system. Configured grant or semi-persistent scheduling which is used to configure periodic transmission resources are supported and more suitable for periodic transmission. This kind of scheduling is benefit for scheduling signaling overhead reduction. If periodic transmission is supported for A-IoT system, configured scheduling can be supported. according to the SID, two kinds of use cases, rUC1 (indoor inventory) and rUC4 (indoor command), are supported. For these two kinds of use cases, it is less motivated to introduce periodic transmission.

Observation 4: It is less motivated to introduce periodic transmission for A-IoT system.

Proposal 18: Dynamic scheduling is supported for A-IoT system.

Proposal 19: Configured grant or semi-persistent scheduling are down-prioritized in R19.

For R2D or D2R transmission, some scheduling information can be (pre-)defined to reduce the required scheduling signaling, such as the modulation mechanism. Other information can be indicated by the scheduling information, such as time resource (starting position or ending position), frequency resource (if device supports FDM), device identification, coding rate, TBS, etc. If multiple modulation mechanisms are introduced for R2D or D2R, the modulation mechanism can also be indicated.  Some of these information can be implicitly determined. For example, the starting position of R2D or D2R transmission can be determined by preamble sequence in front of it. The ending position of R2D or D2R transmission can be determined by coding rate and TBS.

Proposal 20: Control information for scheduling R2D or D2R transmission is needed.

- The following control information can be considered: time and/or frequency resource, device identification, coding rate, TBS, modulation scheme.

The following was agreed in RAN1#117 meeting:

Agreement Scheduling information of PDRCH transmission is provided by a corresponding PRDCH.

The scheduling information of D2R transmission is provided by PRDCH. Although the control information of R2D/D2R transmission is carried in PRDCH, we think they should be separately encoded with R2D payload since they have different reliability requirements. And the control information and data payload should be appended with CRC separately. Detailed analysis can be found in companion paper [6].

Proposal 21: Separately encoding for control information and data payload in PRDCH can be studied for A-IoT system.

6. Conclusion

Energy harvesting

Observation 1: The energy source that would cause a device unavailability for Rx and Tx in A-IoT communication due to charging by energy harvesting is only the RF energy source. When thermal, kinetic, solar, and wind energy sources are used in practice, it is reasonable to assume that they will not cause device unavailability due to charging by energy harvesting, as the device can perform energy harvesting and communication at the same time without any significant limitation/degradation to either function.

Proposal 1: It is proposed that only the RF energy source needs to be assumed during the Rel-19 study for A-IoT.

Proposal 2: The following device state definitions should be used for designing device energy harvesting solutions

- Case 1: Device is assumed to have two states: ON, OFF

- Case 2: Device is assumed to have three states: ON, OFF, SLEEP

- Identify function(s) of the device that can be assumed supported and assumed not supported in each of the above device states, subject to:

  - ON state supports at least: transmission, reception for communication

  - OFF state does not support at least: transmission, reception for communication, running a clock

  - OFF state supports at least: energy harvesting

  - SLEEP state supports at least:

    - Maintaining a memory content from ON state

    - Running a clock or maintaining a timer (RAN1 to discuss purpose(s) of clock)

    - Energy harvesting

    - FFS: monitoring some reception

  - SLEEP state does not support at least: transmission, reception for communication

Observation 2: It is undesirable for all devices, after charging, try to respond to the reader at once when a select/query is received due to TX collision, although the Q protocol from the slotted-ALOHA may randomize the access timing among the devices within an inventory process (but not a very efficient mechanism). Even if the reader allocates a large number of resources trying to cater for all potential devices in a same process, the following issues still remain.

  - firstly, it is questionable whether there can be many frequency domain resources for small / large frequency shifts,

  - secondly, it is also questionable whether there can be many time domain resources considering device SFO problem and available device energy, and

  - thirdly, it will be wasteful if the reader over-allocates time and frequency domain resources.

Proposal 3: For Stage 1 of energy harvesting (before inventory / command process starts), periodic transmissions of R2D preamble should be studied to coordinate or schedule inventory / command processes among the devices, to minimize collision probability and to reduce device power consumption from blind monitoring (and hence more time for energy harvesting).

Synchronization and frame structure

Proposal 4: On determining or deriving the end of a PRDCH transmission at the device, it is proposed to be based on R2D control information (possibly using the R2D preamble).

Proposal 5: On acquiring the end of a PDRCH transmission at the reader, it is proposed to be based on D2R control information (possibly using the D2R preamble).

Observation 3: When OOK is used in PDRCH transmissions, there is no noticeable performance gain from using midamble(s) and/or postamble, regardless of the payload size (96 and 400 bits).

Proposal 6: Based on the above observation from the evaluation results shown, it is proposed to support Option 1 (D2R preamble only) since the performance gain from using D2R midamble and D2R postamble is not noticeable.

Proposal 7: It is proposed to capture the following trade-off analysis table for D2R amble options in TR 38.769.

| Option(s) | Purpose(s) for amble(s) | Assumptions | Aspects to be considered for trade-offs analysis (observations) |
| --- | --- | --- | --- |
| - Opt.1: D2R preamble only - Opt.2: D2R preamble + X midamble(s) - Opt.3: D2R preamble + postamble - Opt.4: D2R preamble + Y midamble(s) + postamble | Channel estimation | - Device residual SFO: 104 ppm - SFO estimation: perfect - PDRCH data rate:1 kbps - Payload: 96 / 400 bits (672 / 2496 chips) - Coding scheme: 1/2 Manchester coding, 1/3 TBCC - Modulation: OOK - Coherent receiver - X = 1 (96 bits) - X=3, Y = 2 (400 bits) - Preamble: 63 chips - Midamble: 31 chips - Postamble size: 31 chips | - When OOK is used in PDRCH transmissions, there is no noticeable performance gain from using midamble(s) and/or postamble, regardless of the payload size (96 and 400 bits). - Amble(s) overhead for 96 bits D2R - Option 1: 9.4% - Option 2: 13.4% - Option 3: 13.4% - Amble(s) overhead for 400 bits D2R - Option 1: 2.52% - Option 2: 6.25% - Option 3: 3.77% - Option 4: 6.25% |

Proposal 8: Since a reference frequency is needed when separate LOs are used for Tx and Rx in device 2b, the existing R2D timing acquisition signal / preamble may not be sufficient or workable. It is proposed that a new / dedicated synchronization signal is needed for synthesizing the frequency needed for D2R transmissions in device 2b.

Random access

Proposal 9: A R2D transmission triggering random access determines X time domain resource(s) for Msg. 1 transmission(s), where X should be ≥ 1 for higher system capacity and lower Tx collision probability.

Proposal 10: For Msg. 1 transmission, the following factors can be considered by device for determining time/frequency resources: random number, device ID, device type, etc.

Proposal 11: For Msg. 2 transmission, TD2R_max can be introduced to limit the maximum delay of Msg. 2 transmission. The time range for Msg. 2 monitoring for Msg. 2 reception is [TD2R_min , TD2R_max].

Proposal 12: For Msg. 3 transmission, following options can be considered:

- Option 1: Msg. 1 is TDMA, Msg. 3 is TDMA;

- Option 2: Msg. 1 is FDMA, Msg. 3 is TDMA;

- Option 3: Msg. 1 is FDMA, Msg. 3 is FDMA;

  - The same frequency resource as Msg. 1 transmission can be used for Msg. 3 transmission.

Scheduling and timing

Proposal 13: The following option can be supported in A-IoT system

- Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min

  - FFS the maximum value(s) for TR2D

Proposal 14: Processing time is common for different A-IoT device types.

Proposal 15: Processing time is common for different traffic types or command types.

Proposal 16: A same maximum time can be defined and applied to all kinds of A-IoT devices.

Proposal 17: The following processing time may be different for different topologies.

- TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it.

- TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device.

Observation 4: It is less motivated to introduce periodic transmission for A-IoT system.

Proposal 18: Dynamic scheduling is supported for A-IoT system.

Proposal 19: Configured grant or semi-persistent scheduling are down-prioritized in R19.

Proposal 20: Control information for scheduling R2D or D2R transmission is needed.

- The following control information can be considered: time and/or frequency resource, device identification, coding rate, TBS, modulation scheme.

Proposal 21: Separately encoding for control information and data payload in PRDCH can be studied for A-IoT system.

7. References

[1] RP-240826 “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103

[2] RP-240854 “Moderator's summary on R19 Ambient IoT”, RAN#103

[3] RP-242360 “Moderator's summary for offline discussion on Ambient IoT”, RAN#105

[4] R1-2407532 “Final FL summary on frame structure and timing aspects for Rel-19 Ambient IoT”, RAN1#118, Moderator (vivo)

[5] R1-2408148 “Discussion on frame structure and timing aspects of A-IoT communication”, RAN1#118, OPPO

[6] R1-2408149 “Discussion on downlink and uplink channel/signal aspects for A-IoT”, RAN1#118-bis, OPPO