3GPP TSG RAN WG1 #119                                                                                         R1-2410095

Orlando, US, November 18th – 22nd, 2024

Source:	OPPO

Title:	Discussion on waveform characteristics of carrier-wave provided externally to the A-IoT device

Agenda Item:	9.4.2.4

Document for:	Discussion and Decision

Introduction

During the last few meetings, finalization for the study of carrier wave characteristics was extensively discussed and further progressed. In this contribution, we will continue discussing the following remaining aspects for the necessary characteristics of carrier-wave waveform provided externally to the Ambient IoT devices:

- Gap between two unmodulated single-tones or two hops of single-tone with frequency hopping

- Carrier wave transmission on FDD band

- AIOT device with large frequency shift function

- Transmission power of two unmodulated single-tones

- Two unmodulated single-tones from two different carrier wave nodes

Discussion

  - Gap between two unmodulated single-tones or two hops of single-tone with frequency hopping

In RAN1#118, the following observation for the frequency gap between two unmodulated single-tones was achieved.

RAN1#118 Observation For the gap between two tones to be able to leverage frequency diversity gain, bandwidth and spectrum characteristics of the D2R transmission, and coherence bandwidth, should be taken into account.

As can be seen in the above, it was only agreed that the gap between two tones is related to the bandwidth and spectrum characteristics of the D2R transmission. It is still unclear what the “bandwidth and spectrum characteristics” refers to. From the perspective of one D2R transmission, the backscattered D2R waveform in frequency domain is bi-sided of a single-tone carrier wave frequency point. For the case of FDMA, different backscattered D2R waveforms will be located with different small frequency shifts with respect to the single-tone carrier wave. Simultaneously, guard band may be needed between D2R transmissions as shown in Figure 1. Hence, we propose to further clarify that the bandwidth and spectrum characteristics at least include D2R transmission bandwidth from the backscattering of one or more devices, small frequency shift(s) for one or more devices and guard band associated with each D2R transmission.

In addition, the frequency gap between the two hops of single-tone with frequency hopping has not been discussed in RAN1. In our view, the above observation for two unmodulated single-tones is also applicable for single-tone with frequency hopping. Thus, we suggest to update the above observation such that it can cover the case of single-tone with frequency hopping.

Observation 1: For the gap between two tones or two frequency hops to be able to leverage frequency diversity gain, bandwidth and spectrum characteristics of the D2R transmission, and coherence bandwidth, should be taken into account.

- The bandwidth and spectrum characteristics of the D2R transmission at least include the following:

  - D2R transmission bandwidth from the backscattering of one or more devices.

  - Small frequency shift(s) for one or more devices.

  - Guard band associated with each D2R transmission.

Figure 1. Carrier wave and backscattered D2R signal

  - Carrier wave transmission on FDD band

RAN1#116 Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

In RAN1#116 meeting, the above cases were agreed to be studied on CW transmission for Topology 1 and Topology 2, and the cases are shown in Figure 2 and Figure 3. According to the SID, FR1 licensed spectrum in FDD band is considered. The A-IoT devices are not supposed to be equipped with large frequency shift, which means that the backscattered signal is on the same spectrum with that of CW transmitted on. As per spectrum regulation in China, BS/UE is prohibited from transmission on UL/DL spectrum, as this would result in TDD operation on FDD spectrum. FDD spectrum contains UL sub-band and DL sub-band which allows only UL or DL respectively on a sub-band. Operating both UL and DL on one sub-band (e.g., case 1-2 on UL sub-band) violates the regulation for FDD spectrum.

Topology 1

BS is the receiver of the backscattered signal from A-IoT devices, and the CW is provided by BS or a CW node (CWN) other than the BS. When there is large number of A-IoT devices and co-site UEs within the same cell, it is infeasible for a BS to maintain such number of A-IoT devices by transmitting control/data, CW and meanwhile receiving backscattered signal. A Carrier-wave node (CWN) other than the BS can be the CW provider, and only provide CW to A-IoT devices.

In case 1-1, the CW is transmitted on DL spectrum, and the backscattered signal is also on the same DL spectrum. BS uses DL spectrum to transmit normal NR DL, to transmit A-IoT R2D (e.g. control/data), to transmit CW, and to receive backscattered signal. In case 1-2, BS uses UL spectrum to receive normal NR UL, to transmit CW, and to receive backscattered signal. Based on the contributions and discussion in the last few meetings, regulation framework is violated by case 1-1 and case 1-2. According to the regulation, DL spectrum of FDD band only allows BS to transmit signal, and UL spectrum only allows UE to transmit signal. It is not authorized that BS can use UL spectrum to transmit CW, or that UE can use DL spectrum for backscattered signal. In case 1-4, a standalone CWN is used to provide CW, and UL spectrum is used to transmit CW and backscattered signal, which is a proper design without the above concerns.

Topology 2

An intermediate node (UE type device) can provide the CW to A-IoT devices and receive the backscattered signal. It requires the intermediate node to support DL/UL communication with BS, transfer A-IoT data/signaling between BS and A-IoT device, and provide CW to A-IoT device and receive backscattered signal. The UE should have high capability to be able to support these functions. Besides, a CWN other than the intermediate node can also provide the CW to A-IoT devices.

In case 2-2, the intermediate node should have high capability to transmit A-IoT R2D (e.g. control/data), to transmit CW and to receive backscattered signal simultaneously. Not like case 1-2 since BS is always supposed to be with high capability for transmission and reception, it requires the UE with higher complicated transmission and reception capability to be as an intermediate node in case 2-2. In case 2-3, the similar regulation violation issue exists as that in Topology 1, because it is authorized that DL can be used by a standalone CWN to transmit CW and by a A-IoT device to backscatter. Case 2-4 is a proper design that a standalone CWN and UL spectrum are used for CW transmission and backscattering.

Proposal 1: The regulation violation issue should be considered for case 1-1, case 1-2, and case 2-3.

Figure 2. Topology 1: Carrier-wave Transmission on FDD band

Figure 3. Topology 2: Carrier-wave Transmission on FDD band

  - AIOT device with large frequency shift function

If an A-IoT device does not support large frequency shift, which means that the backscattered wave is on the same band of CW. In such case, A-IoT device only backscatters the input CW without shifting the frequency band, which helps the device to have very low cost and complexity. However, when CW transmitter is also the receiver of the backscattered wave, it may lead to the issue of full duplex and the interference between CW leakage and backscattered wave on the same band.

If an A-IoT device is capable with large frequency shift function, the center frequency of backscattered wave can be different from the center frequency of CW. For example, when CW is transmitted on DL band, A-IoT device shifts the backscattered wave to UL band, which can help to eliminate the interference between CW leakage and backscattered wave on the same band and solve the issue of full duplex. However, to realize such frequency shifting, A-IoT device should support large frequency shift function. According to the study so far, large frequency shift may be technically feasible in terms of power consumption, but remaining technical feasibility of large frequency shift in terms of image suppression, harmonics suppression may be relevant to device emission requirement, spectrum usage. Thus, large frequency shift may highly increase the device cost and complexity.

Proposal 2: It can be assumed that A-IoT devices do not support large frequency shift as the baseline for study.

  - Transmission power of two unmodulated single-tones

Moreover, it was already agreed by RAN1 that CW with two unmodulated single-tones is regarded as a starting point for the study of multi-tone CW in R19 SI. Obviously, the total transmission power of CWN needs to be split between each tone when two tones are transmitted. Thus, this will cause an issue of how to allocate the total transmission power for each of the two tones to maximize the gain of two unmodulated single tones. The most straightforward manner is to divide the total transmission power of CWN equally into each carrier wave of two unmodulated single-tones. However, the coverage of D2R transmission may degrade as the frequency diversity gain may not compensate the loss caused by power reduction. On the contrary, to split the power unequally between the two tones can also be considered.

Proposal 3:  For two unmodulated single-tones, how to allocate the total transmission power for each of the two tones to maximize the gain should be considered.

  - Two unmodulated single-tones from two different carrier wave nodes

Two unmodulated single-tones from two different carrier wave nodes can achieve both frequency and spatial diversity gain of D2R transmission. Additionally, it reduces the complexity of CW generation and ensures better D2R coverage than single-tone CW. However, it requires the timing alignment between the two tones from two CWNs and makes the existing topologies (i.e., A1, A2 and B) more complicated. From our point of view, it can be studied with a very limited scope during the SI phase. As exemplified in Figure 4, the additional carrier wave node could be the D2R receiver, such that no new topology in addition to topology A1/B would be introduced. Whether to enable additional carrier wave transmission could be determined/requested by the D2R receiver. By following this way, we can simplify the issue of two tones from two CWNs as BS or UE behaviors in D1T1 or D2T2, which can reduce the normative work in RAN1.

Proposal 4: In topology A1/B, reader can transmit an additional carrier wave to exploit frequency and spatial diversity gain.

Figure 4. Additional carrier wave transmitted by reader

Conclusion

In this contribution, the necessary characteristics of carrier-wave waveform provided externally to the Ambient IoT devices are discussed. The observations and proposals can be concluded as follows:

Observation 1: For the gap between two tones or two frequency hops to be able to leverage frequency diversity gain, bandwidth and spectrum characteristics of the D2R transmission, and coherence bandwidth, should be taken into account.

- The bandwidth and spectrum characteristics of the D2R transmission at least include the following:

  - D2R transmission bandwidth from the backscattering of one or more devices.

  - Small frequency shift(s) for one or more devices.

  - Guard band associated with each D2R transmission.

Proposal 1: The regulation violation issue should be considered for case 1-1, case 1-2, and case 2-3.

Proposal 2: It can be assumed that A-IoT devices do not support large frequency shift as the baseline for study.

Proposal 3:  For two unmodulated single-tones, how to allocate the total transmission power for each of the two tones to maximize the gain should be considered.

Proposal 4: In topology A1/B, reader can transmit an additional carrier wave to exploit frequency and spatial diversity gain.

Reference

[1] RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, 3GPP RAN meeting #103, Maastricht, Netherlands, March 18-21, 2024.

[2] Chairman Notes, 3GPP RAN1#118bis, Hefei, China, October 14th – 18th, 2024.

[3] R1-2409297, Final FL summary on CW waveform characteristics for A-IoT, Moderator (Spreadtrum Communications), 3GPP RAN1#118bis, Hefei, China, October 14th – 18th, 2024.

Appendix

According to the guidance from RAN plenary, how to control the characteristics of the CW waveform in D1T1 and D2T2 is not included in the SID objective. However, it is still valuable to point out some potential directions for the upcoming WI phase. Thus, we provide analysis for control details in the following.

Power control of CW

In legacy NR UL transmission, UE adjusts its transmission power through both open-loop power control and close-loop power control mechanism. More specifically, P0 and α are configured for UE and the pathloss between UE and gNB is utilized to calculate the transmission power in open-loop power control. In order to achieve the pathloss, some measurements according to SSB or CSI-RS are performed by UE. Meanwhile, gNB is able to control the transmission power of UE in a more dynamic manner by sending TPC in DCI to the UE (i.e., close-loop power control).

However, it seems that the open-loop power control mechanism is not applicable to determine the transmission power of carrier wave because the pathloss between CWN and A-IoT device as well as the pathloss between A-IoT device and reader are difficult to be estimated as device very likely cannot perform RSRP measurement. It is also not accurate to use the pathloss corresponding to the direct link between CWN and reader as it is usually smaller than the total pathloss above. By contrast, close-loop power control is a better choice. For example, reader indicates an offset of transmission power to CWN by either UU interface or backhaul if it observes the received power from CWN and/or A-IoT device is out of range.

Time resource of CW

Considering the characteristics of backscattering, the backscattered wave always occurs within the same duration of incident CW to A-IoT device. When a CWN is providing the CW, BS or intermediate node should be ready to receive the backscattered wave on the same duration(s). In Topology 1, A-IoT R2D (e.g. control/data) is transmitted to A-IoT device using DL, while A-IoT R2D (e.g. control/data) is transmitted to A-IoT device on UL in Topology 2. Hence, it is infeasible for intermediate node to transmit R2D and receive backscattering simultaneously. In order to reduce the complexity of intermediate node, it is recommended that CW and R2D transmission are TDMed when they are on the same band in D2T2.

As mentioned in the above, the timing and frequency resource of carrier wave should ensure that AIOT device is able to finish its D2R transmission. Thus, it requires that carrier wave is provided in an on-demand manner (i.e., CW is provided only when D2R transmission is scheduled). To achieve that, the transmission of carrier wave needs to start no later than the D2R transmission and stop no earlier than the end of D2R transmission. On this basis, a guard period can be introduced to accommodate the large SFO of AIOT devices. In an example, CW transmission may need to start in advance if the actual D2R transmission from AIOT device occurs a bit earlier. In the meanwhile, the preparation time for energy harvesting can also be considered if carrier wave is utilized as a kind of RF energy source. Furthermore, the transmission of carrier wave should be aligned with OFDM symbol/slot boundary, which is similar with the design of R2D transmission for coexistence with NR system.

Frequency hopping pattern of CW

The time and frequency resource of CW frequency hopping ought to be controlled by reader as well. More specifically, reader may allocate one-shot frequency hopping pattern or periodic frequency hopping pattern. As an example of one-shot frequency hopping, reader allocates one pair of CW frequency hopping resource for D2R transmission and D2R transmission repetition. On the basis of one-shot frequency hopping allocation, reader can indicate a periodicity to CWN such that CWN performs periodic frequency hopping accordingly.