3GPP TSG RAN WG1#119	        	R1-2410230

Orlando, US, November 18th – 22nd, 2024

Agenda Item:					    9.5.3

Source:	Sony

Title:	Adaptation of common signal/channel transmissions

Document for:	Discussion

# Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

According to the WID for Rel-19 enhancements of NES [1], adaptation of common signal/channel transmissions will be specified in Rel-19.

In this contribution, our considerations on adaptation of SSB and PRACH in time domain are provided.

# Discussion

## Adaptation of SSB in time domain

### Scenario

Agreement (RAN1#116bis)

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement (RAN1#118)

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

In RAN1#116bis meeting [2] and RAN1#118 meeting [3], the following agreement regarding scenarios for adaptation of SSB in time-domain was made.

- Rel-19 NES-capable UE’s PCell (Connected mode)

If any SSB on PCell would not support for adaptation of SSB, NES gain would become marginal as applied scenario for SSB adaptation is very limited. Therefore, in our view, SSB adaptation should be applicable to some SSB on PCell.

This scenario is Rel-19 NES-capable UE’s PCell where both legacy UEs and Rel-19 NES capable UE can access. In this scenario, not only Rel-19 NES-capable UE but also legacy UEs are used for the cell. Therefore, we need to carefully consider legacy UE impact. On the other hand, NCD-SSB and SSB not on sync raster on the cell could be used for adaptation of SSB. Since legacy idle/inactive UEs could not recognise these SSBs unless receiving configuration/indication by gNB, there would not be impact to legacy UEs. We do not need to rule out adaptation for NCD-SSB and SSB not on sync raster unless any issues to support these SSBs arise.

Proposal 1: 	For 	Rel-19 NES-capable UE’s PCell (Connected mode),

- Support adaptation for SSB that is not CD-SSB is supported

- Support adaptation for SSB not on sync raster is supported

- Rel-19 NES-capable UE’s SCell

In the agreement made in last meeting, adaptation of SSB not on sync raster and NCD-SSB are supported. There is still FFS whether to support CD-SSB. This scenario is Rel-19 NES-capable UE’s SCell where both legacy UEs and Rel-19 NES capable UE can access. Although it is SCell from connected mode UE perspective, idle/inactive UEs are not aware of the cell is SCell. Therefore, even in the case, we need to consider legacy UE impact for CD-SSB on the Rel-19 NES-capable UE’s SCell.

To avoid legacy UE impact, some mechanism to prevent accessing to the CD-SSB by legacy UEs would be necessary. One option is to indicate “cell barred” in the MIB of the SSB for the legacy UE. The MIB contains a field of “cellbarred” which indicates access barring to the legacy UE. If it would be applied, CD-SSB could be adapted in time domain with minimizing idle/inactive UE impact on this scenario.

Proposal 2: For 	Rel-19 NES-capable UE’s SCell,

- Support adaptation for CD-SSB

### Signalling

In RAN1#118bis meeting [4], it was made that “There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19”. Therefore, only connected mode UE is applicable to support adaptation of SSB.

For signalling for SSB adaptation for connected mode UE, DCI, MAC CE, and RRC signalling could be considered as candidate signalling.

DCI has the most benefit in terms of signalling latency and signalling overhead in the case of group common DCI, while reliability would be concerned since no feedback for DCI reception is transmitted. On the other hand, for RRC signalling, it has the most reliable transmission, while it is concerned for the signalling latency and signalling overhead due to UE-specific signalling. MAC CE is faster than RRC but slower than DCI from signalling latency perspective and MAC CE also has high reliability since HARQ-ACK corresponding to PDSCH carrying MAC CE is sent.

We think that adaptation of SSB (AI 9.5.3) may have some commonality with on-demand SSB (AI 9.5.1). In RAN1#118 [3], the following agreement in on-demand SSB has been made.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Therefore, since periodicity of the on-demand SSB can be configured by RRC and indicated by MAC CE, the mechanism for adaptation of (normal) SSB could be reused from that of on-demand SSB to reduce specification works. Therefore, in our view, we support at least RRC and MAC CE for signalling of SSB adaptation. Since DCI has not been agreed yet also in on-demand SSB, whether to support DCI for adaptation of SSB could be discussed later.

Proposal 3: Support signalling for adaptation of SSB via RRC and MAC CE.

## Adaptation of PRACH in time domain

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

In RAN1#118 meeting [3], the following agreement about adaptation of PRACH in time domain was made.

For signalling design, we prefer alt 2 (subset of PRACH resource level) because it enable fine and dynamic control of PRACH resource activation/deactivation depending on PRACH loads so that more network energy saving gain could be expected compared to alt 1(PRACH resource configuration level).

Proposal 4: For DCI-based adaptation for additional PRACH resources, 	Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not should be supported.

# Conclusions

The following observations and proposals were made in this contribution.

Proposal 1: 	For 	Rel-19 NES-capable UE’s PCell (Connected mode),

- Support adaptation for SSB that is not CD-SSB is supported

- Support adaptation for SSB not on sync raster is supported

Proposal 2: For 	Rel-19 NES-capable UE’s SCell,

- Support adaptation for CD-SSB

Proposal 3: Support signalling for adaptation of SSB via MAC CE.

Proposal 4: For DCI-based adaptation for additional PRACH resources, 	Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not should be supported.

# References

[1]			 RP-241650	, “Revised WID: Enhancements of network energy savings for NR,” 3GPP TSG RAN Meeting #104, June 2024.

[2] RAN1#116bis meeting chairman’s note

[3] RAN1#118 meeting chairman’s note

[4] RAN1#118bis meeting chairman’s note