3GPP TSG RAN WG1 #119			                  R1-2410294

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.6.1

Source: 	LG Electronics

Title: 	Discussion on LP-WUS and LP-SS design

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss and provide our views on LP-WUS and LP-SS design for the WI on low-power wake-up signal and receiver for NR was approved in RAN#102 and lately revised in RAN#105 [1].

- Discussions on LP-WUS design

2.1. LP-WUS waveform

Regarding SCS for LP-WUS generation, FL suggested the following proposal for further discussion.

FL suggested proposal Potentially down-select the above two options in Proposal 3.1-2 in RAN1#118bis - Proposal 3.1-2: Single SCS for LP-WUS is used by LP-WUR, further discuss following options - The single SCS is configured by gNB - The single SCS is determined by pre-defined rule

It is desirable that the SCS for LP-WUS is determined by DL BWP SCS of MR. When LP-WUS is generated based on the same SCS as MR's active DL BWP, gNB (or UE) can maintain the common SCS for transmitting (or receiving) LP-WUS and the NR signal/channel within the DL BWP of MR, and it means those two signals can be multiplexed easily. In other words, for Idle/Inactive mode, LP-WUS and MR signal/channel in the initial DL BWP can be easily multiplexed based on the common SCS for the initial DL BWP. For Connected mode, with a common SCS for active DL BWP, gNB do not need to change SCS frequently when LP-WUS and MR signal/channel are transmitted alternately.

Proposal #1: Support that the SCS for LP-WUS is determined based on the MR DL BWP

- For Idle/Inactive state, SCS for LP-WUS is same as initial DL BWP SCS

- For Connected state, SCS for LP-WUS is same as active DL BWP SCS

For the other option that the single SCS is configured by gNB only, it might be happened that the configured SCS for LP-WUS is different from that for the other NR signal/channel. In this case, how to handle such case may need to be discussed especially when LP-WUS and NR signal/channel with different SCS are overlapped in time. For example, if LP-WUS and NR signal which have different SCS is transmitted simultaneously, it can be considered to skip the transmission of LP-WUS with different SCS for complexity reduction at both gNB and UE sides. The default SCS for LP-WUS may also be discussed to ensure LP-WUS transmission even when there is no relevant configuration by gNB. In addition, for the case that SCS for SSB is different from SCS for MR DL BWP, what SCS of LP-WUS should be configured can be discussed.

Proposal #2: Regarding SCS for LP-WUS, the followings need to be discussed for coexistence of LP-WUS and NR signal/channel

- Whether to define the default SCS for LP-WUS/LP-SS before gNB configuration

- Whether and how to handle the case that SSB SCS is different from DL BWP SCS

- Whether to allow the simultaneous transmission of LP-WUS and NR signal/channel with different SCS

2.2. LP-WUS structure

Both OOK-1 and OOK-4 waveform were agreed to be supported for LP-WUS generation. OOK-1 waveform can be more robust to timing error because it contains single OOK symbol within an OFDM symbol. On the other hand, OOK-4 with multiple OOK symbols per OFDM symbol can provide a range of spectral efficiencies at the cost of reduced timing error tolerance. Moreover, OOK symbols can be transmitted with overlaid OFDM sequence where the sequence might be transmitted in different way in different use cases. Considering these multiple types of LP-WUS transmission, it is desirable to include a preamble part that precedes the message part, i.e., information bits for wake-up indication. With the preamble, LP-WUR can be provided the useful information (e.g., OOK modulation method, data rate, payload size) about message part to be transmitted subsequently through the preamble part. Moreover, depending on UE implementation, this information may help reduce the detection/decoding complexity of LP-WUR. Additionally, a certain type of CRC can be attached to the message part of LP-WUS to reduce the FAR. It should be noted that the length of CRC needs to be carefully determined depending on the size of the message part to be transmitted through LP-WUS and the use of CRC needs to be optional to reduce transmission overhead. It was agreed that LP-WUS includes a codepoint value to indicate one or more subgroup(s) for Idle/Inactive state. For Connected state, it is still need to be discussed. Regardless of how to transmit the information bits of LP-WUS in Connected state, we think the structure of ‘preamble-message-CRC’ can be a unified structure of LP-WUS for both Idle/Inactive and Connected states.

Proposal #3: Support the LP-WUS structure including preamble part, message part and CRC

- Preamble part: Configurations on message part and CRC can be included

- Message part: UE ID or subgroup ID can be included

- CRC part: It can be optionally attached according to the length of message part

In the previous meetings, there were some brief discussions about the necessity of preamble. Many companies, including us, agreed on the need for a preamble, while most discussions were about the need for a preamble being determined based on tolerable timing error. However, it should be noted that the preamble has many advantages in itself. From the LP-WUR perspective, the preamble can serve as a delimiter for LP-WUS reception, and through the preamble part before the reception of message part, LP-WUR can perform AGC stabilization. Additionally, the preamble can serve as an indication for the information bit transmitted subsequently. Moreover, with well-designed channel structure, it can also help mitigate inter-cell interference. Therefore, when discussing the necessity of LP-WUS preamble, we suggest considering the potential benefits of a preamble as well as timing errors.

Proposal #4: Discuss the necessity of preamble part with consideration of its potential benefit for LP-WUS transmission on top of the need for timing error compensation

For LP-WUS channel structure, the following agreement was made in RAN1#118bis for further discussion about some types of redundancies need to be considered for LP-WUS.

Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/MOs

As discussed earlier, a certain type of CRC can be added to LP-WUS for ensuring the target FAR and MDR. However, for the short size of LP-WUS information bits, simple repeated structure could be better than adding CRC in terms of performance and resource efficiency. The detailed condition of CRC addition and length of CRC can be further discussed with considerations of the maximum size of the information bits for Idle/Inactive state and Connected state, respectively.

Proposal #5: Support CRC and repetition alternatively according to the size of LP-WUS information bits

- For larger size of the information bits, adding CRC is needed to meet MDR target requirement

- For smaller size of the information bits, simple repetition is more efficient in terms of performance and resource efficiency.

Regarding LP-WUS information bits for Connected state, it is still need to be discussed based on the relevant agreement as below.

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs FFS details for extension of option 1 and/or 3 when UE is configured with CA

On the NW overhead aspects and power saving gains for each option, it is closely related to the number of information bits used for each codepoint or bitmap as well as the traffic arrival probability of UEs. For UE detection complexity, as discussed in the last meeting and pointed out by companies, there is no significant differences between two options. Note that it was agreed to support codepoint-based LP-WUS design for Idle/Inactive state and there are several ways to indicate multiple UEs by the codepoint-based LP-WUS. To have a unified LP-WUS design for both Idle/Inactive and Connected states, we prefer Option 3 as the LP-WUR information to trigger PDCCH monitoring for Connected state UEs. In our view, the least preferred design would be a hybrid structure of Option 1 and Option 3, since it may cause the complicated LP-WUR processing and procedures without benefits than either option. For Option 3, whether to support a codepoint value corresponding to multiple UEs can be further discussed with considerations of the maximum size of information bits of LP-WUS and related procedures for Connected state UEs.

Proposal #6: For Connected state, support a codepoint value corresponding to [one or more] UEs as the LP-WUS information bits.

2.3. Overlaid OFDM sequence

It was agreed that, at least OOK-4 with M>1, the overlaid OFDM sequence(s) for LP-WUS are the sequence(s) of an OOK ON symbol before DFT/LS processing as shown below.

Agreement (in RAN1#118) For overlaid OFDM sequences for LP-WUS, support option 1-1 for OOK-4 M>1. - Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing Working Assumption (in RAN1#118bis) For overlaid OFDM sequences for LP-WUS in time or frequency domain, down-select between following two options for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n - Note1: OOK-1 is considered as specific case of OOK-4 with M=1.

We support the working assumption for OOK-4 with M=1 and hope to confirm it as agreement. The common design of OOK-4 for all M values can reduce the gNB and UE complexity as well as specification efforts. Especially, gNB doesn’t need to generate both OOK-1 and OOK-4 (with DFT block) which could keep the generation complexity of OOK waveform reasonable. As described in the working assumption, the DFT size should be the power of 2 for low complexity of DFT block. However, the number of PRBs for LP-WUS can be 11 PRBs and it is closely related to the DFT size as well as the number of input/output samples. So, zero-padding and/or truncation processing around DFT block should be included in the LP-WUS generation process.

Proposal #7: Confirm the working assumption about overlaid OFDM sequence for OOK-4 with M=1, and further discuss on whether to specify zero-padding and/or truncation around DFT block

- Support ZC sequence for overlaid OFDM sequence

- Support DFT size is 2^n

- Further discuss on whether to support zero-padding and/or truncation around DFT block

As an overlaid OFDM sequence for LP-WUS, it was agreed to support ZC sequence for at least OOK-4 with M>1 as shown below.

Agreement (in RAN1#117) Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS - Down select among gold sequence, m sequence and ZC sequence - FFS the overlaid OFDM sequence is time or frequency domain sequence. - FFS how to reuse the existing sequences - Note: Strive to minimize the impact on OOK detection performance If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS Agreement (in RAN1#118) For overlaid OFDM sequence at least for LP-WUS, - For OOK-4 with M>1, support ZC sequence for overlaid sequence. - FFS details

In current NR spec, ZC sequence is specified for several UL signal/channels e.g., PUCCH. To reduce the standardization efforts, we prefer to reuse the ZC sequence specified in NR spec as the overlaid OFDM sequence for LP-WUS. For example, we can consider ZC sequence in 38.211 (as below) as a starting point.

### 5.2.2 Low-PAPR sequence generation type 1 The low-PAPR sequence is defined by a cyclic shift of a base sequence according to where MZC=mNscRB2δ is the length of the sequence. Multiple sequences are defined from a single base sequence through different values of α and δ. Base sequences are divided into groups, where is the group number and is the base sequence number within the group, such that each group contains one base sequence (v=0) of each length MZC=mNscRB2δ, and two base sequences (v=0,1) of each length MZC=mNscRB2δ, . The definition of the base sequence depends on the sequence length .

ZC sequence can be specified with the base sequence and cyclic shift. For the legacy UL signals/channels implemented by ZC sequence in the current NR spec, base sequence and cyclic shift value can be determined at each OFDM symbol. Meanwhile, LP-WUS can be transmitted over multiple OFDM symbols. If the base sequence hopping or cyclic shift hopping happens within one LP-WUS transmission, it would be expected the complicated correlation processing and higher power consumptions at LP-WUR. Instead, the base sequence hopping or cyclic shift hopping can be applied at every LP-WUS MO or multiple MOs. To reuse the existing ZC sequence for LP-WUS, we would like to propose the followings.

Proposal #8: For overlaid OFDM sequence for LP-WUS, reuse the existing ZC sequence in the current specifications

- Sequence hopping and/or cyclic-shift hopping should not be applied within an LP-WUS transmission

- Support sequence hopping and cyclic-shift hopping for different LP-WUS MOs

- FFS: cyclic-shift hopping for repeated transmission of LP-WUS

One more discussion point for Option 1-1 can be whether/how to apply the same overlaid sequence over consecutive two OOK ON symbols. For OOK-4 with M=4, there are two OOK ON symbols in the OFDM symbol. When the same overlaid sequence is repeated in those two OOK ON symbols, the OFDM symbol may have undesirable frequency spectrum due to the repeated sequences. So, it would be better to have different phases for the overlaid sequences on different OOK ON symbols, at least, within OFDM symbol. Simple method can be the use of two sequences with different phase for each OOK symbol. Since Manchester encoding is already agreed to support, two different sequences are alternatively used either on the first OOK ON symbol or the second OOK ON symbol according to the different ON-OFF patterns of Manchester code.

Proposal #9: For the overlaid OFDM sequence on each OOK ON symbol within an OFDM symbol, two sequences with different phases are alternatively used on each OOK ON symbol

For the overlaid OFDM sequence not carrying information bits, the following was agreed in the last meeting.

Agreement In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

According to the agreement, one sequence is selected from multiple candidates to indicate the information bits. The number of multiple candidate sequences can be configurable. There are two different sub options. With Option 2-1, the part of information bits is delivered by overlaid sequence. However, it may increase the LP-WUR complexity since LP-WUR should perform two different types of decoding to get the whole information, i.e., correlation for the overlaid sequence and envelop detection for OOK symbols, which is undesirable. In addition, it would be better to remind that the specific LP-WUR architecture cannot be specified or restricted in RAN1. When the overlaid OFDM sequence carry only part of information bits such as Option 2-1, the OFDM-based LP-WUR should be capable of both sequence decoding and envelop decoding at the same time. In this perspective, we support Option 2-2. For Option 2-2, depending on the number of candidates, the amount of information bits per OOK symbol can be different. Therefore, when the number of candidates is large enough, repeated transmission of overlaid sequence can be supported so that LP-WUS coverage is increased.

Proposal #10: In case of overlaid OFDM sequence carrying information, support Option 2-2 such as

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

For the overlaid OFDM sequence not carrying information bits, the following was agreed.

Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

The single overlaid sequence for this case can be simply determined as one of multiple sequences which can be configured for the overlaid sequences carrying information bits.

Proposal #11: In case of overlaid OFDM sequence not carrying information, one sequence is selected from multiple candidates overlaid OFDM sequences which can be configured for the overlaid sequences carrying information bits

Agreement Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below: - 4 - 8 - 16 - 32 - 64 for M not larger than 2 - FFS whether the same or different set of candidate overlaid sequences are used for different cells

The length of overlaid OFDM sequence can be determined by the number of PRBs on which LP-WUS is transmitted and the value of M for OOK-4 generation. For the larger value of M, the length of overlaid sequence would be shorter than that of small M. With the short length of sequences, it is not be easy to specify the multiple sequences which can be distinguished. We think the different number of candidate sequences for overlaid OFDM sequence should be considered depending on the value of M. Moreover, in our view, the one reason of having multiple candidate sequences for the overlaid OFDM sequence is early termination of LP-WUS monitoring. But, for the larger value of M, the gain from early termination seems marginable compared to that for smaller value of M. In this perspective, we prefer the smaller number of candidate sequences for larger value of M while the larger number of candidate sequences for smaller value of M.

Proposal #12: Support different maximum number of candidate overlaid sequence for each value of M for one cell

In addition, it should be discussed about how to map the overlaid OFDM sequence(s) to the effective samples of OFDM symbol (or OOK symbol). The sampling rate of LP-WUR may be different for each UE. LP-WUR with poor sampling rate may not decode the overlaid sequence when the overlaid sequence is mapped to OFDM samples in high rate. For instance, with the length-N overlaid sequence {s(0), …, s(N-1)}, each element s(n) of the sequence can be mapped to multiple samples of one OOK ON symbol, i.e., multiple samples of a OOK symbol is mapped to s(n). It can reduce the burden of LP-WUR in terms of sampling rate. If each element of the overlaid sequence is mapped to one sample of OOK ON symbol, only LP-WUR having such sampling rate can decode the overlaid sequence, which is undesirable. In other words, by configuring number of OOK symbol to be mapped to each overlaid sequence, LP-WUS can be supported for various types of LP-WUR with different capabilities. To support various sampling rates of LP-WUR, it is desirable that each bit of the overlaid sequence is mapped to multiple samples of OFDM symbol. The length of the overlaid sequence can be determined based on the number of samples in a OFDM symbol to which each bit of the overlaid sequence is mapped. Moreover, it is expected that the large amount of BW can be used for LP-WUS in FR2. It should be check whether the LP-WUR could operate with high sampling rate due to larger BW in FR2.

Proposal #13: Discuss on the sequence length (and detailed mapping to OFDM symbol) of overlaid OFDM sequence considering LP-WUR sampling rate

2.5. Bandwidth and frequency location configurations

In RAN1#118bis meeting, two agreements on the LP-WUS/LP-SS bandwidth were made.

Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 60kHz and 120kHz for FR2.

As a result, RAN1 support 11 PRBs for LP-WUS and LP-SS with 15kHz, 30kHz, 60kHz, 120kHz SCS for a channel bandwidth equal to or larger than 5 MHz. In our view, one clarification is still needed. In FR2, there are two additional SCS, i.e., 480 kHz and 960 kHz. For those SCS, the bandwidth for LP-WUS and LP-SS should be carefully determined. For instance, 11 PRB with 960 kHz SCS, the total bandwidth is greater than 100 MHz, which seems too much for low power receiver.

Proposal #14: Clarification is needed about whether to support LP-WUS and LP-SS generated by FR2-2 SCS (i.e., 480kHz and 960kHz)

- If supported, discuss the bandwidth of LP-WUS and LP-SS for FR2-2 with consideration of low power receiver implementation

To achieve the moderate NW flexibility and LP-WUR complexity at the same time, the frequency location of LP-WUS and LP-SS can be determined based on the association with active BWP for MR or NR signal/channel for the cell (e.g., SSB). For example, LP-WUS can be transmitted in the frequency region as initial BWP or SSB for Idle/Inactive mode UE. As another example for Connected mode UE, the frequency location of LP-WUS can be configured by frequency offset from the lowest RB/RE of the active BWP and the bandwidth of LP-WUS can be configured so that LP-WUS is fully overlapped with the active BWP of UE. Moreover, for some UEs having the shared RF for receiving LP-WUS and NR signal/channel, the association between LP-WUS and MR BWP can remove the burden for frequent RF change.

Proposal #15: Discuss on the frequency location for LP-WUS and LP-SS with consideration of at least the following aspects

- NW flexibility and LP-WUR complexity

- Association with active/initial BWP of MR

In addition, RAN1 should discuss where LP-WUS and LP-SS are located in frequency domain. As a simple solution, RAN1 can introduce the dedicated resources in frequency for LP-WUS and LP-SS transmission. The frequency location of LP-WUS and LP-SS can be configured within the dedicated resources. It is noted that the dedicated resources are in common for multiple UEs (e.g., all or a group of Idle/Inactive UEs should monitor LP-WUS only within the frequency resources). This can also keep both NW flexibility and LP-WUS complexity at a reasonable level.

Proposal #16: Discuss the dedicated frequency resources for LP-WUS and LP-SS

- Discussions on LP-SS design

3.1. LP-SS signal / structure design

3.1.1. LP-SS waveform

In RAN1#116bis, the following was agreed as working assumption for LP-SS waveform generation.

Working Assumption Support the following options for LP-SS - Option 1: OOK-1 - Option 2: OOK-4 with M=2,4 FFS:1,8,16 - FFS whether value of M depends on SCS - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation FFS how OOK-1 and OOK-4 are specified

Considering a low complexity at LP-WUR, it is beneficial to generate LP-SS waveform in the same way as LP-WUS waveform, so that LP-WUR has no additional complexity for LP-SS.

Proposal #17: Confirm the working assumption for LP-SS waveform generation

Candidate values of M for LP-SS can be determined based on the value of M for LP-WUS. Generally speaking, timing synchronization at LP-WUR should be guaranteed without any ambiguity on receiving/decoding LP-WUS information bits. Thus, OOK-4 with small M might not ensure the timing estimation accuracy for receiving LP-WUS. It is desirable to ensure that the value of M for OOK-4 for LP-SS is equal to or greater than the value of M for LP-WUS to provide fine time synchronization for LP-WUS reception, as described in Table 1 and related discussions in Section 3.2.1. For example, when OOK-4 with M=2 is configured for LP-WUS, either M=2 or M=4 should be configured for LP-SS. In other aspects, the configuration of M value should be based on the SCS as captured in working assumption that is agreed in RAN1#116bis. According to the SCS used for waveform generation, duration of OFDM symbol varies leading to different tolerance to timing error. Therefore, value of M may be configured depending on the SCS. For example, M=2,4 can be used for 15 kHz SCS and M=2 can be used for 30 kHz SCS.

Proposal #18: The value of M for OOK-4 for LP-SS should be equal to or greater than the value of M for LP-WUS, to provide fine time synchronization at LP-WUR

Observation #1: Applicable value of M for LP-SS can be configured depending on SCS for LP-WUS and SCS for LP-SS

Additionally, LP-SS periodicity can also be considered to configure the value of M for LP-SS since the amount of timing drift varies according to the LP-SS periodicity. Since according to the value of M for LP-SS, tolerance to timing error varies (e.g. higher value of M requires robust timing synchronization requirement). Accordingly, since short LP-SS periodicity causes small amount of timing drift compared to longer LP-SS periodicity, higher value of M for LP-SS/WUS can be used for short LP-SS periodicity.

Moreover, as agreed in RAN1#118bis, different LP-SS duration (number of OFDM symbols occupied by LP-SS) may vary according to the sequence length which is also associated with the value of M for LP-SS. Since as shown in our simulation result (see section 3.1.2), longer sequence length shows finer time synchronization performance. In this manner, SCS of LP-SS/WUS which is a factor related to tolerance to timing error can be used as a predefined rule to configure L value among multiple L values. Additionally, since different number of OFDM symbols may be occupied by LP-SS according to the value of M and also one or more than L value(s) can be applied to each of M value, sequence length (or duration of LP-SS) can also be considered when configuring the value of M for LP-SS.

Observation #2: Applicable L for LP-SS can be configured depending on SCS for LP-WUS and SCS for LP-SS when multiple L values are configured to each M

Proposal #19: Discuss whether and how to configure the value of M for LP-SS considering the followings:

- Value of M for LP-WUS waveform

- SCS of LP-WUS/LP-SS

- LP-SS periodicity

- Sequence length of LP-SS (i.e., LP-SS duration)

3.1.2. LP-SS sequence

To further discuss and potentially determine the binary sequences for LP-SS, the following evaluation assumptions on timing estimation accuracy and RRM measurement accuracy were agreed in RAN1#118 and RAN1#118bis.

Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption

Agreement To determine the binary sequences for LP-SS, consider the following for performance comparison: - Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority)) - Assume the sliding window size is at least [-25us, 25us]. - RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison - Cross correlation: maximum cross-cor value/ maximum auto-cor value - FFS how to use cross-correlation for comparison - Note: refer to RAN4 RRM assumption for the inter-cell interference

Agreement To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values: - M=1, L= {4,6,8} - M=2, L= {8,12,16,24} - M=4, L= {16,24,32,56} - FFS whether one or more than one L value (s) applied to each of M values M=1,2,4 - FFS the L values applied to other applicable SCS(s) - FFS the L values if other M values in addition to M=1,2,4 are supported. - FFS whether the time estimation averaged across multiple LP-SS occasions is applied - Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS) - Note 2: This doesn’t preclude any of three agreed sequence types Above applies at least for both 15kHz and 30kHz

Based on our simulation assumptions described in Appendix A, Fig. 1 and subsequent figures show the timing estimation performances corresponding to two sequence types (M-sequence and Gold-sequence) with different lengths. In Fig. 1, we compared the sync accuracies of two sequence types where OOK-4 with M=4 waveform is assumed. It is shown that M-sequence shows slightly better synchronization performance compared to Gold-sequence, which simply reflects good correlation property of M-sequence. As it was agreed that 4 different binary LP-SS sequences are sufficient to differentiate cells, we think M-sequence with proper length should be considered as an LP-SS sequence type and there is no critical reason to adopt Gold-sequence instead of M-sequence. After the sequence type is decided, as a next step, further discussion on how to differentiate up to 4 LP-SS binary sequences could be accelerated. We think, as a starting point, applying cell-specific cyclic shift for can be considered to differentiate cells as similarly as the legacy NR PSS/SSS.

Figure 1. Timing error CDF according to sequence type

Proposal #20: Support M-sequence as a LP-SS binary sequence

Proposal #21: Consider cell-specific cyclic shift, as a starting point, to further discuss about how to differentiate cells with binary LP-SS sequences

Figure 2. Timing error CDF according to sequence length (OOK-4 with M=2)

Figure 3. Timing error CDF according to sequence length (OOK-4 with M=4)

Fig. 2 and Fig. 3 show timing estimation error CDF according to different sequence length of M-sequence where OOK-4 with M=2 and M=4 is assumed in Fig. 2 and Fig. 3, respectively. As shown in figures, the larger length of sequence provides the better sync accuracy. For M=2, we can see the 8-bit length sequence can meet the 2 us timing synchronization requirement. For M=4, at least 32-bit length is required to meet the 1 us sync requirement.

Observation #3: For LP-SS binary sequence, the minimum sequence length required to meet synchronization requirement can be different according to the value of M

In our view, 32-bit length sequence for M=4 and 8-bit length sequence for M=2 should be considered. As aforementioned in section 3.1.1, one or more than L value(s) can be applied to each of M value. When multiple L values can be applied to each of M value, predefined rule can be used to determine the L value. For example, a higher SCS used for LP-SS/WUS may necessitate a larger L value, as increased SCS reduces tolerance to timing error.

Proposal #22: Support following sequence length for LP-SS using M-sequence corresponding to the value of M for LP-SS

- L=8 (4 OFDM symbols) for OOK-4 with M=2

- L=32 (8 OFDM symbols) for OOK-4 with M=4

3.1.3. Manchester coding

Using Manchester coding in OOK waveform has many benefits in clock synchronization, error detection, and balancing DC component. Additionally, without Manchester coding, each OOK symbol within one OFDM symbol may have unequal power. This is because not all binary sequence with good correlation property (e.g., m-sequence or gold sequence) always guarantee the equal number of ‘1’ and ‘0’ in an OFDM symbol. When designing binary sequences (M>1) for LP-SS, it may be complicated to balance the number of '0's and '1's within one OFDM symbol. Moreover, without Manchester coding, determining threshold voltage value is crucial for OOK-based LP-WUR, but in general the complicated process is also needed. In addition, when each OOK symbol has unequal power across OFDM symbols, the determined threshold voltage value for energy detection may vary in symbol-by-symbol depending on how many ON chips are in the OFDM symbol. As aforementioned, issue of run property of LP-SS binary sequence and sliding window size may not occur when Manchester coding is applied to LP-SS since transition is occurred corresponding to each LP-SS binary bit within 1 OFDM symbol (when M>1 waveform is used). However, when M=1 with OOK-4 waveform or OOK-1 waveform may need another solution to solve the problem with sliding window size since two ON chips can be continued among two OFDM symbols even though Manchester coding is applied.

On the other hand, using Manchester coding in binary sequence may degrade correlation property which is expected by using specific sequence, even though Manchester coding may help to obtain determining threshold voltage, balancing DC component, clock synchronization, interferences, which are considered as noise caused by other cells or legacy NR signal/channel. Since the sequence of LP-SS is not yet decided, sequence of LP-SS could be designed considering Manchester coding.

Observation #4: If Manchester coding is not used for LP-SS, there might be power imbalance issue across OOK symbols within an OFDM symbol

Observation #5: Applying Manchester coding may solve the problem occurred by sliding window size and run property of LP-SS binary sequence

Proposal #23: Discuss whether to apply Manchester coding to LP-SS considering potential power imbalance issue

3.2. Time domain transmission of LP-SS

3.2.1. Additional synchronization signal

In RAN1#118bis, FL suggested the following proposal for future discussions on the additional synchronization signal.

[H][FL1] Proposal 5-1r1: Support additional synchronization signal: - The additional synchronization signal is configurable or some predefined rule is followed. - Strive to minimize the difference between sequence design for periodic LP-SS and the additional synchronization signal. - Strive to minimum overhead increase by the additional synchronization signal, e.g., the additional synchronization signal is transmitted only when at least one LP-WUS is transmitted - FFS the location of additional synchronization signal.

In our view, additional synchronization signal is needed for several reasons. The crucial reason for the need of additional synchronization signal is that synchronization requirement may not be always met with the current agreed periodicity (e.g. 320ms). Below table summarizes the maximum timing error that can be occurred according to the periodicity of LP-SS to receive LP-WUS.

| LP-SS periodicity P [ms] | Residual frequency error Fe | Residual timing error T by LP-SS [us] | Maximum timing error for LP-WUS detection (Te=T+Fe*P) [us] | Available waveform for LP-WUS |
| --- | --- | --- | --- | --- |
| 80 | 10 ppm | 5 (M=1 for LP-SS) | 5.8 | None |
| 80 | 10 ppm | 2 (M=2 for LP-SS) | 2.8 | M=1 |
| 80 | 10 ppm | 1 (M=1 for LP-SS) | 1.8 | M=1, 2 |
| 160 | 10 ppm | 5 (M=1 for LP-SS) | 6.6 | None |
| 160 | 10 ppm | 2 (M=2 for LP-SS) | 3.6 | M=1 |
| 160 | 10 ppm | 1 (M=1 for LP-SS) | 2.6 | M=1 |
| 320 | 10 ppm | 5 (M=1 for LP-SS) | 8.2 | None |
| 320 | 10 ppm | 2 (M=2 for LP-SS) | 5.2 | None |
| 320 | 10 ppm | 1 (M=1 for LP-SS) | 4.2 | M=1 |

Table 1. Summary of available waveform for LP-WUS according to LP-SS periodicity

As shown in the Table 1, it can be seen that the synchronization requirements cannot be achieved for LP-WUS with M=2 or M=4 when the 320 ms periodicity is assumed. Even with the short periodicity (i.e., 80ms, 160ms), LP-WUS with M=4 cannot be used for LP-WUS due to large timing error. Therefore, for some LP-SS periodicity including 320 ms periodicity, additional synchronization signal is required to achieve the target synchronization target. One may concern about the increased overhead of additional synchronization signal especially when preamble of LP-WUS is transmitted as the additional synchronization signal in every LP-WUS transmission. However, with the predefined rule, additional synchronization signal can be introduced without significant overhead increase.

Proposal #24: Both periodic LP-SS and additional synchronization signal should be supported

3.2.2. LP-SS periodicity

In RAN1#118bis, the following was agreed for LP-SS periodicity.

Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

As aforementioned in section 3.2.1, additional synchronization signal can be supported to flexibly use different value of M for LP-WUS which enables various deployment scenarios with LP-WUS. To support various deployment scenarios or to flexibly manage overhead caused by additional synchronization signal, multiple LP-SS periodicities can be supported. For example, short periodicity may not support additional synchronization signal to reduce overhead and longer periodicity may support additional synchronization signal to meet the synchronization requirement to receive LP-WUS. It is up to gNB implementation to select the preferred LP-SS periodicity considering several factors like channel conditions, overhead of number of LP-WUS transmission when preamble is considered as additional synchronization signal.

Proposal #25: Multiple LP-SS periodicities need to be supported for various scenarios

- Conclusions

In this contribution, we have discussed on the various aspects for LP-WUS and LP-SS design, and the followings are proposed.

Proposal #1: Support that the SCS for LP-WUS is determined based on the MR DL BWP

- For Idle/Inactive state, SCS for LP-WUS is same as initial DL BWP SCS

- For Connected state, SCS for LP-WUS is same as active DL BWP SCS

Proposal #2: Regarding SCS for LP-WUS, the followings need to be discussed for coexistence of LP-WUS and NR signal/channel

- Whether to define the default SCS for LP-WUS/LP-SS before gNB configuration

- Whether and how to handle the case that SSB SCS is different from DL BWP SCS

- Whether to allow the simultaneous transmission of LP-WUS and NR signal/channel with different SCS

Proposal #3: Support the LP-WUS structure including preamble part, message part and CRC

- Preamble part: Configurations on message part and CRC can be included

- Message part: UE ID or subgroup ID can be included

- CRC part: It can be optionally attached according to the length of message part

Proposal #4: Discuss the necessity of preamble part with consideration of its potential benefit for LP-WUS transmission on top of the need for timing error compensation

Proposal #5: Support CRC and repetition alternatively according to the size of LP-WUS information bits

- For larger size of the information bits, adding CRC is needed to meet MDR target requirement

- For smaller size of the information bits, simple repetition is more efficient in terms of performance and resource efficiency.

Proposal #6: For Connected state, support a codepoint value corresponding to [one or more] UEs as the LP-WUS information bits.

Proposal #7: Confirm the working assumption about overlaid OFDM sequence for OOK-4 with M=1, and further discuss on whether to specify zero-padding and/or truncation around DFT block

- Support ZC sequence for overlaid OFDM sequence

- Support DFT size is 2^n

- Further discuss on whether to support zero-padding and/or truncation around DFT block

Proposal #8: For overlaid OFDM sequence for LP-WUS, reuse the existing ZC sequence in the current specifications

- Sequence hopping and/or cyclic-shift hopping should not be applied within an LP-WUS transmission

- Support sequence hopping and cyclic-shift hopping for different LP-WUS MOs

- FFS: cyclic-shift hopping for repeated transmission of LP-WUS

Proposal #9: For the overlaid OFDM sequence on each OOK ON symbol within an OFDM symbol, two sequences with different phases are alternatively used on each OOK ON symbol

Proposal #10: In case of overlaid OFDM sequence carrying information, support Option 2-2 such as

- Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

Proposal #11: In case of overlaid OFDM sequence not carrying information, one sequence is selected from multiple candidates overlaid OFDM sequences which can be configured for the overlaid sequences carrying information bits

Proposal #12: Support different maximum number of candidate overlaid sequence for each value of M for one cell

Proposal #13: Discuss on the sequence length (and detailed mapping to OFDM symbol) of overlaid OFDM sequence considering LP-WUR sampling rate

Proposal #14: Clarification is needed about whether to support LP-WUS and LP-SS generated by FR2-2 SCS (i.e., 480kHz and 960kHz)

- If supported, discuss the bandwidth of LP-WUS and LP-SS for FR2-2 with consideration of low power receiver implementation

Proposal #15: Discuss on the frequency location for LP-WUS and LP-SS with consideration of at least the following aspects

- NW flexibility and LP-WUR complexity

- Association with active/initial BWP of MR

Proposal #16: Discuss the dedicated frequency resources for LP-WUS and LP-SS

Proposal #17: Confirm the working assumption for LP-SS waveform generation

Proposal #18: The value of M for OOK-4 for LP-SS should be equal to or greater than the value of M for LP-WUS, to provide fine time synchronization at LP-WUR

Proposal #19: Discuss whether and how to configure the value of M for LP-SS considering the followings:

- Value of M for LP-WUS waveform

- SCS of LP-WUS/LP-SS

- LP-SS periodicity

- Sequence length of LP-SS (i.e., LP-SS duration)

Proposal #20: Support M-sequence as a LP-SS binary sequence

Proposal #21: Consider cell-specific cyclic shift, as a starting point, to further discuss about how to differentiate cells with binary LP-SS sequences

Proposal #22: Support following sequence length for LP-SS using M-sequence corresponding to the value of M for LP-SS

- L=8 (4 OFDM symbols) for OOK-4 with M=2

- L=32 (8 OFDM symbols) for OOK-4 with M=4

Proposal #23: Discuss whether to apply Manchester coding to LP-SS considering potential power imbalance issue

Proposal #24: Both periodic LP-SS and additional synchronization signal should be supported

Proposal #25: Multiple LP-SS periodicities need to be supported for various scenarios

Observation #1: Applicable value of M for LP-SS can be configured depending on SCS for LP-WUS and SCS for LP-SS

Observation #2: Applicable L for LP-SS can be configured depending on SCS for LP-WUS and SCS for LP-SS when multiple L values are configured to each M

Observation #3: For LP-SS binary sequence, the minimum sequence length required to meet synchronization requirement can be different according to the value of M

Observation #4: If Manchester coding is not used for LP-SS, there might be power imbalance issue across OOK symbols within an OFDM symbol

Observation #5: Applying Manchester coding may solve the problem occurred by sliding window size and run property of LP-SS binary sequence

- References

RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, RAN#105

RP-233068, “Study on low-power wake up signal and receiver for NR”, 3GPP TR 38.869 V2.0.0

Table A. Simulation assumptions for LP-SS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| Waveform | OOK-4 |
| Coding scheme | No line coding |
| Subcarrier spacing | 30kHz |
| Bandwidth | 11 PRB |
| Filter | 5th order Butterworth LPF |
| Sampling Rate | 7.68 MHz |
| ADC bit width | Ideal ADC |
| Channel Model | TDL-C 300ns |
| Number of Rx for LP-WUS | 1 Rx |
| Timing error | Based on 20 ppm frequency error |
| Frequency error | 5 ppm |
| Inter-cell interference | No |