3GPP TSG RAN WG1 #119 		                                            R1-2410312

Orlando, USA, Nov 18th – 22nd, 2024

Agenda Item:	9.4.1.1

Source:	InterDigital, Inc.

Title:	Evaluation results for Ambient IoT

Document for:	Discussion and Decision

# Introduction

Evaluation assumptions for Ambient IoT including methodology for coverage evaluation were discussed in the previous RAN1 meetings [1]. In this contribution we present coverage analysis results for reader-to-device (R2D) and device-to-reader (D2R) using the agreed assumptions.

# Coverage Evaluations

Table 1 to Table 4 present the coverage for various deployments and device types. Link level simulation results for R2D and D2R are provided in Section 3.

From Table 1 and Table 2 we can see that, for 1kbps data rate, coverage is limited by the R2D transmission in most of the scenarios. However, for D1T1-A2 scenario, coverage of D2R is worse due to the CW interference caused on the received D2R transmission at the reader. For D2T2-A2 scenario, although coverage of D2R is slightly better than R2D, it is still low compared to other cases. In the coverage analysis, CW cancellation is assumed to be 140 dB and the D2R coverage will increase for higher interference cancellation values. For example, with 160 dB CW interference cancellation, the coverage of D2R would become higher than R2D for the D1T1-A2 scenario.

Proposal 1: Capture the following observations for 1 kbps data rate in the TR:

- Observation 1:  For 1kbps, R2D coverage is lower than D2R coverage except for the D1T1-A2 scenario.

- Observation 2:  For 1 kbps, D2R coverage is limited by the CW interference in D1T1-A2 scenario.

Table 1 Coverage for Device 1 (1kbps)

| Scenario | CW case | Device type | R2D MPL (dB) | D2R MPL (dB) | R2D Distance (m) | D2R Distance (m) |
| --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | 1-1 | Device 1 | 67.1 | 74.36 | 36.59 | 79.44 |
| D1T1-A1 | 1-2 | Device 1 | 67.1 | 69.36 | 36.59 | 46.66 |
| D1T1-A2 | 1-1 | Device 1 | 67.1 | 58.24 | 36.59 | 13.11 |
| D1T1-A2 | 1-2 | Device 1 | 67.1 | 58.22 | 36.59 | 13.09 |
| D1T1-B | 1-4 | Device 1 | 67.1 | 82.42 | 36.59 | 186.01 |
| D2T2-A1 | 2-2 | Device 1 | 51.1 | 64.70 | 13.61 | 83.22 |
| D2T2-A2 | 2-2 | Device 1 | 51.1 | 51.58 | 13.61 | 14.51 |
| D2T2-B | 2-3 | Device 1 | 51.1 | 95.30 | 13.61 | 4883.78 |
| D2T2-B | 2-4 | Device 1 | 51.1 | 85.30 | 13.61 | 1290.39 |
| D2T2-B | 2-4 | Device 1 | 51.1 | 80.62 | 13.61 | 692.51 |

Table 2 Coverage for Device 2a (1kbps)

| Scenario | CW case | Device type | R2D MPL (dB) | D2R MPL (dB) | R2D Distance (m) | D2R Distance (m) |
| --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | 1-1 | Device 2a | 71.1 | 79.36 | 56.22 | 134.67 |
| D1T1-A1 | 1-2 | Device 2a | 71.1 | 74.36 | 56.22 | 79.44 |
| D1T1-A2 | 1-1 | Device 2a | 71.1 | 63.24 | 56.22 | 23.89 |
| D1T1-A2 | 1-2 | Device 2a | 71.1 | 63.22 | 56.22 | 23.86 |
| D1T1-B | 1-4 | Device 2a | 71.1 | 92.42 | 56.22 | 532.58 |
| D2T2-A1 | 2-2 | Device 2a | 55.1 | 69.70 | 23.18 | 161.89 |
| D2T2-A2 | 2-2 | Device 2a | 55.1 | 56.58 | 23.18 | 28.22 |
| D2T2-B | 2-3 | Device 2a | 55.1 | 105.30 | 23.18 | 18483.79 |
| D2T2-B | 2-4 | Device 2a | 55.1 | 95.30 | 23.18 | 4883.78 |
| D2T2-B | 2-4 | Device 2a | 55.1 | 90.62 | 23.18 | 2620.97 |

Regarding 7 kbps data rate, one can observe from Table 3 and Table 4 that the D2R coverage is worse for A2 scenarios due to CW self-interference and for CW Case 1-2 due to lower transmit power in the UL band. In the remaining scenarios, R2D coverage is lower.

Proposal 2: Capture the following observations for 7 kbps data rate in the TR:

- Observation 3:  For 7 kbps, R2D coverage is lower than D2R coverage except for the A2 scenarios and CW case 1-2.

- Observation 4:  For 7 kbps, D2R coverage is limited by the CW interference in A2 scenarios.

- Observation 5: For 7 kbps, D2R coverage is limited by the CW transmit power in CW case 1-2.

Table 3 Coverage for Device 1 (7kbps)

| Scenario | CW case | Device type | R2D MPL (dB) | D2R MPL (dB) | R2D Distance (m) | D2R Distance (m) |
| --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | 1-1 | Device 1 | 67.1 | 69.31 | 36.59 | 46.41 |
| D1T1-A1 | 1-2 | Device 1 | 67.1 | 64.31 | 36.59 | 26.93 |
| D1T1-A2 | 1-1 | Device 1 | 67.1 | 57.07 | 36.59 | 11.20 |
| D1T1-A2 | 1-2 | Device 1 | 67.1 | 57.00 | 36.59 | 11.09 |
| D1T1-B | 1-4 | Device 1 | 67.1 | 72.32 | 36.59 | 64.03 |
| D2T2-A1 | 2-2 | Device 1 | 51.1 | 59.19 | 13.61 | 39.92 |
| D2T2-A2 | 2-2 | Device 1 | 51.1 | 49.93 | 13.61 | 11.64 |
| D2T2-B | 2-3 | Device 1 | 51.1 | 84.26 | 13.61 | 1123.96 |
| D2T2-B | 2-4 | Device 1 | 51.1 | 74.26 | 13.61 | 296.97 |
| D2T2-B | 2-4 | Device 1 | 51.1 | 69.59 | 13.61 | 159.38 |

Table 4 Coverage for Device 2a (7kbps)

| Scenario | CW case | Device type | R2D MPL (dB) | D2R MPL (dB) | R2D Distance (m) | D2R Distance (m) |
| --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1 | 1-1 | Device 2a | 71.1 | 74.31 | 56.22 | 79.02 |
| D1T1-A1 | 1-2 | Device 2a | 71.1 | 69.31 | 56.22 | 46.41 |
| D1T1-A2 | 1-1 | Device 2a | 71.1 | 62.07 | 56.22 | 20.91 |
| D1T1-A2 | 1-2 | Device 2a | 71.1 | 62.00 | 56.22 | 20.75 |
| D1T1-B | 1-4 | Device 2a | 71.1 | 82.32 | 56.22 | 184.05 |
| D2T2-A1 | 2-2 | Device 2a | 55.1 | 64.19 | 23.18 | 77.67 |
| D2T2-A2 | 2-2 | Device 2a | 55.1 | 54.93 | 23.18 | 22.65 |
| D2T2-B | 2-3 | Device 2a | 55.1 | 94.26 | 23.18 | 4253.87 |
| D2T2-B | 2-4 | Device 2a | 55.1 | 84.26 | 23.18 | 1123.96 |
| D2T2-B | 2-4 | Device 2a | 55.1 | 79.59 | 23.18 | 603.19 |

Based on the above, it can be concluded that coverage enhancement is needed for both R2D and D2R. Repetition, which has already been agreed to study, can be used for both R2D and D2R for coverage enhancement.

Observation 6: Coverage enhancement is needed for D2R and R2D.

Proposal 3: Repetition is supported for both D2R and R2D.

# Link level simulation results

The R2D and D2R link level are provided below. The detailed simulation assumptions can be found in the Appendix. In the simulations, it was observed that there is not any meaningful difference between the BLER with and without SFO when the SFO is estimated at the reader/device receiver using the D2R/R2D preamble; therefore, BLER curves with SFO are not provided here.

Figure 1 BLER for D2R in TDL-A channel

Figure 2 BLER for D2R in TDL-D channel

Figure 3 BLER for R2D in TDL-A channel

Figure 4 BLER for R2D in TDL-D channel

# Conclusion

This contribution provided coverage analysis for R2D and D2R. The following were observed and proposed:

Proposal 1: Capture the following observations for 1 kbps data rate in the TR:

- Observation 1:  For 1kbps, R2D coverage is lower than D2R coverage except for the D1T1-A2 scenario.

- Observation 2:  For 1 kbps, D2R coverage is limited by the CW interference in D1T1-A2 scenario.

Proposal 2: Capture the following observations for 7 kbps data rate in the TR:

- Observation 3:  For 7 kbps, R2D coverage is lower than D2R coverage except for the A2 scenarios and CW case 1-2.

- Observation 4:  For 7 kbps, D2R coverage is limited by the CW interference in A2 scenarios.

- Observation 5: For 7 kbps, D2R coverage is limited by the CW transmit power in CW case 1-2.

Observation 6: Coverage enhancement is needed for D2R and R2D.

Proposal 3: Repetition is supported for both D2R and R2D.

# References

[1] Draft Report of 3GPP TSG RAN WG1 #118bis

# Appendix

Table 5 Link level simulation assumptions

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz |
| SCS | 15 kHz |
| Channel model | TDL-A, TDL-D, 30 ns delay spread, 3 km/h |
| Number of Tx/Rx chains | 1 for device, 2 for reader |
| Reference data rate | 1 kbps, 7 kbps |
| Message size | 20 bits, 96 bits |
| BLER target | 10% |
| Sampling frequency offset | Randomly select from the range [0.1 ~ 1] *10^4 ppm for device 2a Randomly select from the range [0.1 ~ 1] * 10^5 ppm for device 1 |
| Waveform (CW) | Unmodulated single tone |
| Modulation | OOK |
| Line code | Manchester encoding |
| FEC (D2R) | Convolutional coding, 1/3 |
| Receiver (Reader and device) | Non-coherent |