3GPP TSG RAN WG1 #119		 	     R1-2410448

Orlando, US, November 18th – 22nd, 2024

Source:           ZTE Corporation, Sanechips

Title:             Discussion on channel modelling for ISAC

Agenda item:      9.7.2

Document for:     Discussion

Introduction

Current 5G-Advance network is basically designed for data transmission, which may not support sensing on many dimensions, e.g., signal, network structure, deployment mode and so on. To support evolution to integrated sensing and communication (ISAC) system, a study item on channel modelling for ISAC for NR is approved in RAN#102 meeting [1]. The objective of the SID is copied below.

Objective of SI The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency. It will be discussed at RAN#105 whether to include additional study beyond channel modelling for ISAC.

Similar as channel modelling for communication, typical channel modelling generation methods can include statistical channel modelling and deterministic channel modelling for sensing, i.e. stochastic geometry and hybrid with ray tracing channel modelling respectively in TR 38.901 [2]. In this contribution, we provide our views for both stochastic geometry and hybrid with ray tracing channel modelling.

Especially on stochastic geometry modeling, our detailed analysis and preference are provided in the following sections on RCS / CPM for UAV, Human, Vehicle, multi-scattering points, concatenation details, interaction between sensing targets, Doppler frequency, spatial consistency, background modeling, power normalization between target and background, EO type 2, etc.

It is noted that the real experiment results are provided for UAV, Human and Vehicle in Appendix of our previous contribution [17]. In the following sections, Ray-tracing simulation results are shown after the EM calibration based on the real measurement.

RCS calculation

Agreement RAN1 strives to define a single option per target per monostatic/bistatic sensing mode from the following two options to generate RCS values/patterns for a scattering point of a target. - Option 2: The RCS=A*B of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - A is [mean] RCS value. FFS value(s) A - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B - B is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS - Option 3: The RCS=A*B=A*B1*B2 of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - FFS: A = 1 m2 or [mean] RCS value - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B is further split into B1, B2, i.e., B=B1*B2 - B1 is deterministic based on incident/scattered angles - FFS: B1 is defined by a function or by a table - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS Agreement RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic - B2 of UAV is modelled using log-normal distribution for monostatic - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc. - For UAV of small size (option 2 for UAV size in UAV parameters table) - B1=1 - A is mean RCS value - For UAV of large size (option 1 for UAV size in UAV parameters table) - B1 have dependency on incident/scattered angles - A is mean RCS value

In RAN#118bis meeting, the above agreements about RCS model has been approved, which split RCS value in linear scale into three parts A, B1 and B2 as shown in the above Option 3. According to the agreements, A is the mean value of large scale RCS, and B (including B1 and B2) is the normalized values in small scale. However, there are two following implicit issues to be clarified.

- The average operation to obtain A should been done for linear-scaled RCS value rather than dB-scale RCS value.

Since the mean value of linear-scaled RCS and the mean value of dB-scale RCS are not the same, which is

Hence, it should be clarified that A should be obtained as  from linear-scaled RCS values.

- The B2 obeys log-normal distribution, which has mean value 1 (1 is a linear-scaled value), and variance V. However, the linear-scaled mean value of 1 of B2 is not equal to 0 mean value in dB-scale.

For example, if B2 obeys a log-normal distribution with parameters  and  , which is noted as . Then  obeys a normal distribution . The mean value of  is expected as  and variance as . Then the linear-scaled mean value of B2 should be derived as ( it should be noticed that is not equal to ).  If we assume that  has a mean value , then  is larger than 1.

To make E[B2] be 1,  should be satisfied.

In short, for log-normal distribution, μ = 1 in linear-scaled value is not equal to μ = 0 in dB-scaled value. To ensure companies provide correct distribution of B2 with linear-scaled mean value 1, we propose to use the following procedure for fitting RCS parameters including A, B1 and B2.

- Step 1: Obtain RCS measurement and simulation results.

Based on measurement or simulation, the RCS of interested sensing target could be sampled, and a set of samples  could be obtained.

- Step 2: Split  into  dependent with angular and independent with angular.

Using smooth operation, or other fitting operation, the total  could be split into two parts. The first part is , which is dependent with angular. The second part is , which is independent with angular.

The  focuses on relationship between shape of sensing target and RCS value, thus there are spatial co-relationship between  and  with different angles of incident or departure . Thus the part of should be modeled as pattern.

The  focuses on the angular glint phenomena of RCS value.

There are following equations:

- Step 3: Normalize the angular dependent .

Estimate the linear-scaled mean value of , and denoted it as .

is the contribution of pattern to , then is obtained by normalizing  with , i.e.

where the  describes the normalized pattern at the corresponding ZOA/AOA/ZOD/AOD representing by sample index .

- Step 4: Normalize the angular independent .

Estimate the linear-scaled mean value of , and denoted it as , which is the contribution of  to .

Then the samples could be normalized as , which could be used to fit the parameters of . The  obeys normal distribution, of which the mean and variance value could be fitted as  and  respectively.

We know the log-normal distribution has mean value equal to , thus we could derive the accurate mean value of  as

to ensure the mean value of  be equal to 1.

- Step 5: Calculate A

According to the discussion before, the A could be derives by

Proposal 1: Clarify RCS components A, B1, B2 as following

- For B2,  to ensure the mean value  be equal to 1

- For B1,  to ensure the mean value  be equal to 1

- For A,  and

CPM normalization

Agreement To model the effect of polarization for each direct/indirect path: - Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For a LOS ray from Tx to target or from target to Rx, for CPMtx,sp or CPMsp,rx - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio  and initial random phases referring to TR 38.901 as start point - FFS how to normalize on CPMtx,sp,rx - FFS CPMsp of a scattering point of the target - FFS: how to model the effect of polarization when EO type-2 is present

In RAN#118bis meeting, the above agreements about CPM has been approved. According to the agreement, total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp. CPMtx,sp. However, there is a FFS about how to normalize on CPMtx,sp,rx in order to ensure the amplitude of the final CPM be 1. Based on our analysis, the following normalization method can be utilized.

Draw CPMtx,sp,rx as

CPMtx,sp,rx = CPMsp,rx . CPMsp . CPMtx,sp

= a11a12a21a22∙b11b12b21b22∙c11c12c21c22 = d11d12d21d22.

The normalized CPMtx,sp,rx  can be represented as

CPMtx,sp,rxnorm=d11d12d21d2212(d11+d22)=2(d11+d22)d11d12d21d22

Proposal 2: Denote

CPMtx,sp,rx = CPMsp,rx . CPMsp . CPMtx,sp

= a11a12a21a22∙b11b12b21b22∙c11c12c21c22 = d11d12d21d22.

The normalized CPMtx,sp,rx  can be represented as

CPMtx,sp,rxnorm =d11d12d21d2212(d11+d22)=2(d11+d22)d11d12d21d22

RCS/CPM for UAV

Agreement RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic - B2 of UAV is modelled using log-normal distribution for monostatic - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc. - For UAV of small size (option 2 for UAV size in UAV parameters table) - B1=1 - A is mean RCS value - For UAV of large size (option 1 for UAV size in UAV parameters table) - B1 have dependency on incident/scattered angles - A is mean RCS value

For UAV of small size (option 2 for UAV size in UAV parameters table), the UAV size (Length x Width x Height) is 0.3m x 0.4m x 0.2m. The RCS simulation results using RT simulation at 4.9 GHz are as following.

| H-H | V-V |
| --- | --- |
| H-V | V-H |

Figure 4-1 RCS pattern of polarization and mean value for UAV

Based on the agreement of RCS Option 3 of small size UAV, the RCS can be modeled as RCS=A*B=A*B1*B2 , where, B1=1 and A is mean RCS value. From the above simulations results, the linear mean RCS value, i.e. A equals 0.053m2, corresponding -12.74dBsm. B2 corresponding to glint is generated by log-Normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

Furthermore, according to the above RCS values for four polarization, the cross polarization ratios can be achieved. It can be calculated that the mean XPR in dB is about 24.4 and standard deviation is about 3.05. Therefore, the cross polarization power ratios (XPR) for the UAV can be modelled as log-Normal distribution. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 24.4and = 3.05.

The CPM for UAV obtained using the above XPR can be represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

Proposal 3: For UAV of small size (Length x Width x Height: 0.3m x 0.4m x 0.2m) at 4.9GHz

  - B1=1

  - A is mean RCS value of 0.053m2 corresponding -12.74 dbsm

  - B2 of UAV is modelled using log-normal distribution. Draw B2 linear-scaled value as

B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for the UAV is modelled as log-Normal distribution. Draw linear-scaled XPR value as

 = 10Y/10,

Where,  is Gaussian distributed with 24.4and = 3.05.

  - The CPM for UAV is represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

RCS/CPM for Pedestrian

To verify pedestrian RCS values in different scattering angles, we further provide some Ray-tracing simulation results based on the calibrated EM parameters [10] [11]. An adult model selected in these simulations are approximately 0.5 meters in length, 0.5 meters in width, and 1.75 meters in height. The distance between the Tx/Rx and the pedestrian model is 15m. The monostatic simulation results at 4.9 GHz are as following.

| H-H | V-V |
| --- | --- |
| H-V | V-H |

Figure 5-1 RCS pattern of Pedestrian

Based on the agreement of RCS Option 3 in last RAN1 meeting, the RCS of pedestrian can be modeled as RCS=A*B=A*B1*B2, where B1=1 and A is mean RCS value. From the simulations results, mean RCS value equals to 0.4m2, corresponding -4.0 dBsm.

B2 corresponding to glint is generated by log-Normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

Furthermore, according to the above RCS values for four polarization, the cross polarization ratios can be achieved. It can be calculated that the mean XPR in dB is about 25.8 and standard deviation is about 2.99. Therefore, the cross polarization power ratios (XPR) for the pedestrian can be modelled as log-Normal distribution. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 25.8 and = 2.99.

The CPM for pedestrian obtained using the above XPR can be represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

Proposal 4: RCS Option 3 is selected to model RCS of pedestrian with single scattering point

  - For Adult pedestrian (approximately 0.5 meters in length, 0.5 meters in width, and 1.75 meters in height) at 4.9GHz

    - B1=1

    - A is mean RCS value of 0.4m2

  - B2 of pedestrian is modelled using log-normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for pedestrian is modelled using log-Normal distributed. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 25.8 and = 2.99.

  - The CPM for UAV is represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

RCS for Vehicle

The 3D car model with calibrated EM parameters is believed to reflect real EM properties. In this section, the simulation with the calibrated EM parameters is carried on at 4.9GHz as center frequency. The car's dimensions are approximately 5 meters in length, 2.0 meters in width, and 1.6 meters in height.

In the following results, from the car perspective, AOA/ZOA are the input angles, and AOD/ZOD are the output angles. ZOA/ZOD = 0°corresponds z axis, and AOA = 0, 90, 180, 270°correspond to the car left side, back, right side and front respectively.

Figure 6-1 Vehicle model in coordinate

Mono-static

For mono-static sensing mode in the carrier frequency of 4.9GHz, the RCS values for four polarization combinations are simulated and the H-H polarization results are shown as an example in the following figures. For the 3D RCS simulation, Tx (Rx) and ST may be in the different heights.

Figure 6-2 shows the 3D RCS distribution for mono-static. The 3D RCS values are simulated with different ZOA/ZOD and AOA/ZOA values from the vehicle perspective.

Figure 6-2 3D RCS for mono-static for H-H polarization

Based on the 3D RCS distribution, Figure 6-3 provides the top view with region division of 3D RCS values for mono-static. The overall region can be quantified into 5 areas for simplicity.

Figure 6-3 Region division of 3D RCS for mono-static for H-H polarization from top view

Specifically, the area 1 corresponds to the car left side, area 2 corresponds to the back of the car, area 3 corresponds to the car right side, area 4 corresponds to the front of the car, area 5 corresponds to the car roof.

For each region after fitting by gradient descent algorithm, the RCS modeling can refer to the modeling of the 3D antenna beam in TR 38.901. In details, the input parameters are listed in the following table into the formula to obtain the corresponding mono-static RCS pattern rcsdB(θ,φ) in dBsm.

rcsdB(θ,φ)=Gmax−min−AVdBθ+AHdB φ,Amax

Wherein,

AVdBθ=−min12θ−θcenterθ3dB2, Amax,

AHdB φ=−min12φ−φcenterφ3dB2, Amax,

Table 1 Parameters for mono-static RCS modeling for vehicle

| Area/n | Side of vehicle | θcenter | φcenter | φ3dB | θ3dB | Amax | Gmax [dBsm] | Applicable range of ZoA/θ | Applicable range of AoA/φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Left | 78.8 | 0° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [0°, 45°]∪[315°, 360°] |
| 2 | Back | 71.6° | 90° | 39.6° | 43.9° | 2.76 | 4.62 | [20°, 90°] | [45°,135°] |
| 3 | Right | 78.8° | 180° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [135°,225°] |
| 4 | Front | 20.0° | 270° | 95.7° | 48.4° | 10.05 | 9.49 | [20°, 90°] | [225°,315°] |
| 5 | Roof | 0° | - | - | 28.42 | 3.63 | 10.99 | [0°, 20°] | [0°,360°] |

The 3D RCS based on the above fitting pattern is shown in below.

Figure 6-4 3D fitting RCS pattern without glint

Figure 6-5 Top view of 3D fitting RCS without glint

Based on the above observation, the slow fast fading of RCS can be modeled as rcsdB(θ,φ). However, the glint, i.e. fast change part is still not reflected. As described in our previous contribution [10], Gaussian distribution without any spatial correlation can be used to model the fast RCS variation due to glint. Hence, the final RCS pattern is shown as below:

RCS_car_dB = rcsdB(θ,φ) + glint, where glint is Normal distribution, N(0,σ)

Also, the glint modeling is shown in the following table after fitting, wherein the standard deviation is around 3.28.

The fitting modeling of the final RCS_car with both slow fast and glint with standard deviation is shown as below. It can be seen the modeling is well matched with the original patterns output by RT simulation.

Figure 6-6 3D fitting RCS with glint without smoothing

Based on the agreement of RCS Option 3 in last RAN1 meeting, the RCS of a car can be modeled as RCS=A*B=A*B1*B2 of a scattering point. The component A as large scale loss is commonly applied to any incident/scattered angles at the scattering point.

From the simulations, for the pattern, the mean value in linear value is 4.58 m2, corresponding 6.61dBsm. For the glint, the mean value in linear value is 2.56 m2, corresponding 4.08dBsm. Therefore, the value of A is 11.72 m2, i.e. 10.69 dBsm, which is obtained by summing 6.61 and 4.08.

B1 in dB is deterministic based on incident/scattered angles as following:

B1dB  = rcsdB(θ,φ)−6.61

B2 corresponding to glint is generated by log-Normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

Furthermore, the mean RCS values for four polarization combinations are achieved in the following table.

Table 2 Mean value of statistics for difference areas

| Polarization pair (Tx-Rx) | mean value | Area 1 | Area 2 | Area 3 | Area 4 | Area 5 | All area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H-H |  | 4.9556217 | 1.4828804 | 5.0124016 | 3.2137318 | 9.813468 | 4.8956207 |
| H-V |  | -17.57543 | -17.800802 | -17.683006 | -19.035542 | -14.805525 | -17.380061 |
| V-H |  | -17.534603 | -17.70423 | -17.809626 | -19.084879 | -14.734744 | -17.3736164 |
| V-V |  | 4.907881 | 1.4824759 | 5.029778 | 3.1972172 | 9.783014 | 4.88007322 |

Based on the above mean RCS values for four polarization, the cross polarization ratios can be achieved. It can be observed that the mean XPR is about 22 dB. Therefore, the cross polarization power ratios (XPR) for the vehicle can be modelled as log-Normal distribution. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 22.3 and = 4.71.

Proposal 5: For mono-static, the RCS=A*B=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by

  - A has mean RCS value 11.72 m2 corresponding 10.69 dBsm

  - B1 is deterministic based on incident/scattered angles

B1dB = rcsdB(θ,φ)−6.61.

Where,

rcsdB(θ,φ)=Gmax−min−AVdBθ+AHdB φ,Amax

Where,

AVdBθ=−min12θ−θcenterθ3dB2, Amax,

AHdB φ=−min12φ−φcenterφ3dB2, Amax,

| Area/n | Side of vehicle | θcenter | φcenter | φ3dB | θ3dB | Amax | Gmax | Applicable range of ZoA/θ | Applicable range of AoA/φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Left | 78.8 | 0° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [0°, 45°]∪[315°, 360°] |
| 2 | Back | 71.6° | 90° | 39.6° | 43.9° | 2.76 | 4.62 | [20°, 90°] | [45°,135°] |
| 3 | Right | 78.8° | 180° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [135°,225°] |
| 4 | Front | 20.0° | 270° | 95.7° | 48.4° | 10.05 | 9.49 | [20°, 90°] | [225°,315°] |
| 5 | Roof | 0° | - | - | 28.42 | 3.63 | 10.99 | [0°, 20°] | [0°,360°] |

    - B2 is generated by log-normal distribution

      - Draw B2 values as B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for the Vehicle is modelled using log-Normal distributed. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 22.3 and = 4.71.

Bi-static

For 3D bi-static sensing mode in the carrier frequency of 4.9GHz, the RCS values for four polarization combinations are simulated and the H-H polarization results are shown in the following figures.

In all cases, the incident angle AOA/ZOA is fixed, and from the side of the car, the 3D figure is shown in different AOD/ZOD values.

- Case 1: (AOA=180°, ZOA=90°) is fixed, i.e. incident angle is perpendicular to the right side door of the car. Larger RCS values can be observed around the specular angles, i.e. AOD = 180°. In left side of the car, the received signal power is very weak because of blockage, the RCS in area around AOD = 0°is quite low with only diffraction or front scatter power.

Diffraction area

Diffraction area

Figure 6-7 3D view figure of bi-static with AOA=180°

- Case 2: (AOA=135°, ZOA=90°) is fixed, i.e. the incident angle is 45°from the right side door of the car. The first strongest RCS area can be observed around the specular angles reflected from the right door of the car, i.e. reflection angle AOD = 225°. The second strongest RCS area can be observed around the specular angles reflected from the back of the car, i.e. refection angle AOD = 45°. Because the car back and right side are usually inclined upward, the reflection angles are directed to the air, i.e. the reflection angle ZOD is closed to 0°. The diffraction or front scatter area is round AOD = 135+180= 315°where the RCS values are lowest.

Figure 6-8 Bi-static RCS value distribution area

Diffraction area

Strongest 1

Strongest 2

Figure 6-9 3D view figure of bi-static with AOA=135°

- Case 3: (AOA=100°, ZOA=90°) is fixed. i.e. the incident angle is 10°with the right side door of the car. The second strongest RCS area can be observed around the specular angles reflected from the right door of the car, i.e. reflection angle AOD = 260°. The first strongest RCS area can be observed around the specular angles reflected from the back of the car, i.e. reflection angle AOD = 80°. The diffraction or front scatter area is round AOD = 100+180= 280°.

Diffraction area

Strongest 1

Strongest 2

Figure 6-10 3D view figure of bi-static with AOA=100°

Based on the RCS RT simulation results of vehicles, there is only one strongest RCS area which can be further modelled similar as 3D beam in TR 38.901 for case 1 (incident angle perpendicular to the car’s right door). In contrast, both case 2 and case 3 show two distinct strongest RCS areas, wherein the peak magnitudes and center location of RCS area for case 2 and case3 vary significantly. These simulation results show that modeling the RCS of vehicles for bi-static mode is more complex than that for monostatic mode.

Furthermore, these two strongest RCS areas observed from the above simulation results may be caused by reflections/scattering from vehicle's two sides. For bi-static RCS modelling, we provide additional simulation validations covering AOA values that range from 100 to 180 degrees, with granularity 10 degrees.

Case 3: (AOA=100°, ZOA=90°)

Case 4: (AOA=110°, ZOA=90°)

Case 5: (AOA=120°, ZOA=90°)

Case 6: (AOA=130°, ZOA=90°)

Case 7: (AOA=140°, ZOA=90°)

Case 8: (AOA=150°, ZOA=90°)

Case 9: (AOA=160°, ZOA=90°)

Case 10: (AOA=170°, ZOA=90°)

Case 1: (AOA=180°, ZOA=90°)

Figure 6-11 Bistatic simulations from different AOA

To analyze the right side of the vehicle, the RCS pattern can be modeled as a beam pattern related with a moving center position which related to the AOA. From the above simulation results, the center positions are approximately close to the location of the mirror reflection points of the vehicle's right side as shown in the following figure. The RCS pattern is modeled as a beam pattern wherein the peak point in the beam is around the reflection direction. The actual RCS value for the scattering angle (θs,φs) can be computed based on this beam pattern. It is noted that the refection angle (θreflect,φreflect), i.e. the peak point in the beam pattern relies on the incident angle (θi,φi).

In the figure, the angle (θn,φn) is the normal direction, i.e. it is in the middle between incident and reflection angles.

Figure 6-12 Reflection beam pattern

In summary, the RCS pattern of the car can be modeled as a 3D beam pattern:

rcsdB(θi,φi,θs,φs)=Greflection−min−AVdBθs+AHdB φs,Amax

Wherein,

AVdBθs=−min12θs−θreflectθ3dB2, Amax,

AHdB φs=−min12φs−φreflectφ3dB2, Amax

To calculate the direction of the reflection angle (θreflect,φreflect), we can use vector reflection formulas as following steps.

Step1. Convert angles to vectors.

The normal vector to the plane is denoted as

N=(sin(θn)cos(φn), sin(θn)sin(φn), cos(θn) ).

The incident vector is:

I=(sin(θi)cos(φi), sin(θi)sin(φi), cos(θi) )

Step2. Calculate the reflection vector.

Based on the parallelogram principle for vector addition, there is R+I = 2(I∙N)N.

Thus, the reflection vector R is given by: R= 2(I∙N)N− I, where the I∙N is the dot product of I and N.

Step3. Convert the reflection vector back to angles.

The reflection vector R=(sin(θreflect)cos(φreflect), sin(θreflect)sin(φreflect), cos(θreflect) )

To extract the angles (θreflect,φreflect) from the reflection vector:

θreflect=arccos(R∙(0, 0, 1))

φreflect=mod(2π+arctan2(R∙(0, 1, 0),  R∙(0, 1, 0)), 2π)

Note: For function arctan2(y, x), the range of values falls within [-π, π).

In addition, the 3dB beam width can be consistent with that used in the mono-static scenario for the vehicle. Regarding the beam's peak value Greflection, for AOA values within the range of [90°,180°], the RCS values with AOD range [90°, 270°] are shown in Table 3, which can be modeled as

Greflection = Gmax_RCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2)

.

Table 3 The simulation results of beam’s peak value for different AOAs

| AOA/degree | 100 | 110 | 120 | 130 | 140 | 150 | 160 | 170 | 180 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gmax from RT | 0.78 | 3.93 | 6.02 | 7.31 | 7.69 | 8.1 | 8.29 | 7.70 | 8.33 |
| Gmax_modelling | 1.52 | 3.0 | 4.39 | 5.64 | 6.73 | 7.61 | 8.25 | 8.64 | 8.78 |

Figure 6-13 reflection beam’s peak values

Based on the above analysis, we have the following proposal:

Proposal 6: For Bi-static, the RCS modeling is RCS_car = rcsdB(θi,φi,θs,φs) + glint, where glint is Normal distribution, and

rcsdB(θi,φi,θs,φs)=Greflection−min−AVdBθs+AHdB φs,Amax

Wherein,

AVdBθs=−min12θs−θreflect(θi)θ3dB2, Amax,

AHdB φs=−min12φs−φreflect(φi)φ3dB2, Amax,

Greflection= Gmax_RCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2),

| Side of vehicle | ZoA/θi | AoA/φi | (θn,φn) | φ3dBn | θ3dB | Amax | Gmax_RCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [20°, 90°] | [0°, 90°]∪ [270°, 360°] | (78.8, 0) | 47.5° | 99.5° | 5.8 | 9.5 |
| Back | [20°, 90°] | (0°,180°) | (71.6, 90) | 39.6° | 43.9° | 2.76 | 4.62 |
| Right | [20°, 90°] | (90°,270°) | (78.8, 180) | 47.5° | 99.5° | 5.8 | 9.5 |
| Front | [20°, 90°] | (180°,360°) | (20, 270) | 95.7° | 48.4° | 10.05 | 9.49 |
| Roof | [0°, 20°] | (0°,360°) | (0, -) | - | 28.42 | 3.63 | 10.99 |

Note： if θn=0,  then  Greflection = Gmax_RCS*cos(θi).

Multi-scattering points

As shown in the following figure, if a vehicle is modeled with multiple scattering points, different numbers of scattering points my be visible for the two TRPs in reality. However, it is hard to determine how many / which scattering points are visible for each TRP by explicit model, especially when the vehicle shape is irregular.

Figure 7-1 RCS issues for Multi-points modelling for a car

To simplify the modeling, we suggest M=5 scattering points are modeled for a vehicle including left side, right side, front, back and the roof, and the number of visible scattering points for each TRP is always fixed to M=5. As shown in the above figure, for TRPB, the RCS values for the non-invisible scattering points (yellow points) will be extremely low based on the RCS patterns shown in section 5, it is equivalent that only three red scattering points are modelled for the TRPB.

For other types of sensing targets including UAV, pedestrian and AGV, even though single scattering point is sufficient for the use case of intrusion, location and tracking as described in Rel-19 SID, we are open to support multiple scattering points for forward compatibility.

Proposal 7: For a target modeled with M>=1 scattering points, the target channel for those M scattering points are generated independently

- The number of visible scattering points is fixed to M, i.e. not depending on distance between Tx/Rx and the target.

- The number of scattering points of a target is generated at the beginning of the simulation and keeps unchanged

- For vehicle, M = 5

- The relative locations of the M scattering points in LCS of the target are based on the shape of the target

Concatenation

Agreement The following options are to be studied for the concatenation of Tx-target and target-Rx link in the target channel - Direct path (if present) is always kept - Indirect paths of LOS+NLOS, NLOS+LOS (if present) are generated - On other indirect paths of NLOS + NLOS - Option 0: ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 0A: ray level full convolution between Tx-target link and target-Rx link only for radio propagation Case 4 - Option 1: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4 - Option 1A: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4 - Option 2: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4 - Option 2A: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4 - Option 3: ray level 1-by-1 coupling between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3A: ray level 1-by-1 coupling between Tx-target link and target-Rx link only for radio propagation Case 4 - Note: reducing the number of rays per cluster and/or reducing the number of clusters can be considered for the options above - Any indirect path with power metric less than [threshold] is dropped - the power metric of a path is the product of power of a ray in Tx-target link, power of a ray in target-Rx link and RCS of the pair of rays - FFS power normalization of target channel after path dropping - FFS the set of remaining indirect paths can be updated during movement of Tx, target or Rx

With regard to the number of concatenated paths / clusters, we provided simulation results on LOS/NLOS case 1, 2, 3 and 4 for UMi and Indoor office scenarios. In both UMi and Indoor office scenarios, the power CDF curves of direct path and indirect paths are provided for case 1, 2, 3, and 4. For indirect path of NLOS + NLOS, the above Option 0 is adopted in the following simulation results. In order to intuitively compare the powers of NLOS + NLOS paths with the powers of NLOS + LOS / LOS + NLOS paths and direct path, powers of direct path and indirect paths are normalised by subtracting the maximum power of indirect paths. The normalised power CDF curves of direct path and indirect paths are provided respectively for case 1, 2, 3, and 4.

Case 1 in UMi scenario:

(a) Power CDF                                   (b) normalised power CDF

Figure 8-1 power CDF and normalised power CDF for case 1 in UMi

Case 2 in UMi scenario:

(a) Power CDF                                   (b) normalised power CDF

Figure 8-2 power CDF and normalised power CDF for case 2 in UMi

Case 3 in UMi scenario:

(a) Power CDF                                   (b) normalised power CDF

Figure 8-3 power CDF and normalised power CDF for case 3 in UMi

Case 4 in UMi scenario:

(a) Power CDF                                   (b) normalised power CDF

Figure 8-4 power CDF and normalised power CDF for case 4 in UMi

Observation: In Umi scenario, it can be observed that:

- the normalised power of almost all NLOS + NLOS paths in case 1, 2 and 3 are less than -25dB, that means almost all of NLOS + NLOS paths have a power less than 25dB compared with the maximum power of LOS + NLOS / NLOS + LOS paths.

Case 1 in Indoor office scenario:

(a) Power CDF                                   (b) normalised power CDF

Figure 8-5 power CDF and normalised power CDF for case 1 in Indoor Office

Case 2 in Indoor office scenario:

(a) Power CDF                                   (b) normalised power CDF

Figure 8-6 power CDF and normalised power CDF for case 2 in Indoor Office

Case 3 in Indoor office scenario:

(a) Power CDF                                   (b) normalised power CDF

Figure 8-7 power CDF and normalised power CDF for case 3 in Indoor Office

Case 4 in Indoor office scenario:

(a) Power CDF                                   (b) normalised power CDF

Figure 8-8 power CDF and normalised power CDF for case 4 in Indoor Office

Observation: In Indoor office scenario, it can be observed that:

- the normalised power of almost all NLOS + NLOS paths in case 1, 2 and 3 are all less than -25dB, that means almost all of NLOS + NLOS paths have a power less than 25dB compared with the maximum power of LOS + NLOS / NLOS + LOS paths.

As we can see in above simulation results, almost all of NLOS + NLOS paths have a power less than 25dB compared with the maximum power of LOS + NLOS / NLOS + LOS paths for case 1, 2 and 3 in both UMi and Indoor Office scenarios. Especially, the powers of all NLOS + NLOS paths are less than about 50dB compared with the power of LOS + LOS path in case 1 of both UMi and Indoor Office. That means, the power of NLOS + NLOS in case 1, 2 and 3 is too weak to cause the sensing performance impact. Taking into account of high complexity and computation cost of NLOS + NLOS paths, NLOS + NLOS paths can be ignored in case 1, 2 and 3.

In terms of of NLOS + NLOS concatenation in case 4, Option 0A and Option 1A will cause huge computation cost. For example, if there are 20 clusters in both Tx-target and target-Rx link, and if each cluster has 20 rays, Option 0A will generate 160000 rays after ray level full convolution, Option 1A will generate 8000 rays. As shown in normalised power CDF in Figure 8-4 and 8-8, the power gap between the minimum power of NLOS + NLOS path and the maximum power of NLOS + NLOS path is larger than 60dB in UMi scenario, while the power gap between the minimum power of NLOS + NLOS path and the maximum power of NLOS + NLOS path is 80dB in Indoor Office scenario. That means, there are too many NLOS + NLOS paths with very low power cause weak impact on sensing performance after ray level full convolution.

Furthermore, to demonstrate the concatenation impact on sensing channel by adopting Option 2A and Option 3A, the circulate angle spreads are simulated respectively based on Option 0A, Option 2A and Option 3A. Figure 8-9 provides the circulate angle spread CDF curves in UMi scenario, and Figure 8-10 provides the circulate angle spread CDF curves in Indoor office scenario. It can be seen from Figure 8-9 and Figure 8-10 that circulate angle spread (including AOD spread, AOA spread, ZOD spread and ZOA spread) CDF curves of Option 3A is very close to that of Option 0A in both UMi and Indoor office scenarios. It validates that concatenation impact on sensing channel of Option3A is almost same as that of Option 0A. Thus, considering the trade-off between computation complexity and performance impact on sensing channel, we prefer Option 3A for NLOS + NLOS path concatenation in case 4.

(a) circulate AOD spread CDF                           (b) circulate AOA spread CDF

(c) circulate ZOD spread CDF                           (d) circulate ZOA spread CDF

Figure 8-9 Circulate angle spread CDF in UMi scenario

(a) circulate AOD spread CDF                           (b) circulate AOA spread CDF

(c) circulate ZOD spread CDF                           (d) circulate ZOA spread CDF

Figure 8-10 Circulate angle spread CDF in Indoor office scenario

Observation: The circulate angle spreads (including AOD spread, AOA spread, ZOD spread and ZOA spread) of Option 3A is almost same as that of Option 0A for sensing channel in both Umi and Indoor office scenarios.

After paths dropping of NLOS + NLOS concatenation, it can be observed there is no impact on circulate angle spread. We can infer there is no impact on sensing channel as a result of dropping some NLOS + NLOS paths. In contrary, if power is normolised after paths dropping, the power of each of the remaining NLOS + NLOS path will be increased after power normalization, which will severely degrade sensing performance. Therefore, power normalization should be not employed after paths dropping.

Proposal 8: For the concatenation of Tx-target and target-Rx link in the target channel:

  - On indirect paths of NLOS + NLOS, support Option 3A

- Option 3A: ray level 1-by-1 coupling between Tx-target link and target-Rx link only for radio propagation Case 4

  - Power normalization is not needed after paths dropping

To check the number of clusters for target channel, some RT simulations are provided here. We put a target car in the well-known Manhhatan city model to compare the number of paths of the two cases in UMi scenario with bi-static sensing mode.

- Case 1: the number of rays not passing through the target

- Case 2: the number of rays indirectly passing through the target

Figure 8-11: Bi-static sensing in UMi scenario of Mahhantan city model

In the simulation, the rays of which power 25dB lower than the maximum power of the ray passing through the target are neglected for simplicity. It can be seen that even in such a complex UMi environment, compared with the number of total rays, the number of rays passing through the target is much less.

Figure 8-12: Clusters related to sensing target marked in yellow

Based on the above RT simulation shown in the above figure, the total number of NLOS clusters is about three times of the number of NLOS clusters passing through the target. Therefore, we have the following proposal:

Proposal 9: For single scattering point modeling, the number of NLOS clusters is 13 of that defined in 38.901

  - Each NLOS cluster includes 20 rays as the same as defined TR 38.901

Interaction among sensing targets

To check the necessity of interaction among sensing targets, two cars are dropped in the high way scenario for RT simulation as shown in the following figure. Concrete floor is set under cars as road of highway scenario.

Figure 9-1 Side view and top view of highway scenario with two cars

Based on the results listed in the following table, it observes that the interaction power between the two cars is much smaller than the direct path, i.e. direct link of Tx-ST-Rx. For smaller sensing targets, such as pedestrian, UAV or AGV, less interaction power is expected.

Table 4 Interaction power between two targets

| Related types of objects | Receiving signal power | Receiving signal power [dB] |
| --- | --- | --- |
| Direct link for Tx-ST1-Rxor link for Tx-ST2-Rx | 1.887503e-10 | -97.24 |
| Indirect link for Tx-ST1-ST2-Rx or Tx-ST2-ST1-Rx | 1.1370469e-13 | -129.44 |

Proposal 10: It is unnecessary to model a propagation path from Tx to Rx interacting with more than one sensing targets.

Doppler frequency

In the current TR 38.901, Clause 7.6.10, dual mobility has been considered, i.e. the mobility of both Tx and Rx are considered in the Doppler frequency, e.g. both Tx and Rx are Vehicles.

- 7.6.10 Dual mobility in TR 38.901 To support simulations that involve dual Tx and Rx mobility or scatterer mobility, the Doppler frequency component in the channel coefficient generation in step 11 in clause 7.5 should be updated as follows. For the LOS path, the Doppler frequency is given by vn,m=rrx,n,mT∙vrx+rtx,n,mT∙vtxλ0 (7.6-45) where rrx, n, m and rtx, n, mare defined in (7.5-23) and (7.5-24) respectively, and For all other paths, the Doppler frequency component is given by: vn,m=rrx,n,mT∙vrx+rtx,n,mT∙vtx+2αn,mDn,mλ0 (7.6-46) where Dn,m is a random variable from −vscatt to vscatt, αn,m is a random variable of Bernoulli distribution with mean p, and vscatt is the maximum speed of the clutter. Parameter p determines the proportion of mobile scatterers and can thus be selected to appropriately model statistically larger number of mobile scatterers (higher p) or statistically smaller number of mobile scatterers (e.g. in case of a completely static environment: p=0 results in all scatteres having zero speed). A typical value of p is 0.2.

For sensing, the sensing target may also move with a certain speed, e.g. UAV with 160km/h speed, human body with 3km/h speed, car with 60km/h speed, etc. In short, all of Tx, ST (sensing target) and Rx may have mobility. Hence, the Doppler frequency should be updated accordingly.

- For direct path between sensing transceiver and the sensing target, the Doppler phase formula for m-th ray in n-th cluster in a link of Tx-ST-Rx at time t can be given by

vn,mISAC = 0trrx,n,mT(τ)(vrx(τ)−vST,n,m(τ))dτλ0 + 0trtx,n,mT(τ)(vtx(τ)−vST,n,m(τ))dτλ0   (1)

where vST,n,m is the velocity of the sensing target for the ray m of cluster n. It can be further split into macro motion and micro motion parts where micro motion modeling  can be used for the use cases such as human gesture identification, i.e. vST,n,m= , and the detailed model can be discussed in the phase of performance evaluation.

- For indirect path, one more component is suggested for Doppler modeling similar as agenda 7.6.10 of TR 38.901. The motivation is to model the impact from the mobile clutters between Tx/Rx and ST. Specifically, Dn,m is a random variable from −vscatt to vscatt, αn,m is a random variable of Bernoulli distribution with mean p, and vscatt is the maximum speed of the clutter. Parameter p determines the proportion of mobile scatterers and can thus be selected to appropriately model statistically larger number of mobile scatterers (higher p) or statistically smaller number of mobile scatterers (e.g. in case of a completely static environment: p=0 results in all scatteres having zero speed). A typical value of p is 0.2.

vn,mISAC = 0trrx,n,mT(τ)(vrx(τ)−vST,n,m(τ))dτλ0 + 0trtx,n,mT(τ)(vtx(τ)−vST,n,m(τ))dτλ0 + 2αn,mDn,mλ0t  (2)

Like as the existing TR 38.901, if the simulation duration time is short compared to the moving speed of target, Tx and Rx, i.e. the geometry relationship of target, Tx and Rx do not change significantly, the and   are approximated as and . The Doppler frequency formula (2) can be simplified as

vn,m=rrx,n,mT∙(vrx−vST)+rtx,n,mT∙(vtx−vST)λ0t + 2αn,mDn,mλ0t   (3)

Proposal 11: For Htarget, the Doppler frequency frequency is adopted as

vn,m=rrx,n,mT∙(vrx−vST)+rtx,n,mT∙(vtx−vST)λ0t + 2αn,mDn,mλ0t

Where the component 2αn,mDn,mλ0 specified in TR 38.901 section 7.6.10 is disabled for the direct path

Note: the detailed modeling of vST can be discussed in performance evaluation stage.

Spatial consistency

Agreement 3D spatial consistency needs to be studied for at least UAV scenario

In TR 38.901 clause 7.6.3, the spatial consistency procedure and parameter generation method are described. According to the characteristics of different cluster specific parameters and ray specific parameters, the following types on spatial consistency are defined:

- Site-specific: parameters for different BS-UT links are uncorrelated, but the parameters for links from co-sited sectors to a UT are correlated.

- All-correlated: BS-UT links are correlated.

Table 5 Correlation type among TRPs

| Parameters | Correlation type |
| --- | --- |
| Delays | Site-specific |
| Cluster powers | Site-specific |
| AOA/ZOA/AOD/ZOD offset | Site-specific |
| AOA/ZOA/AOD/ZOD sign | Site-specific |
| Random coupling | Site-specific |
| XPR | Site-specific |
| Initial random phase | Site-specific |
| LOS/NLOS states | Site-specific |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

For ISAC with bi-static sensing mode, the correlation of those random parameters should be considered for the link from Tx to ST, as well as the link from ST to Rx.

As shown in the following figure, a network grid is generated in site-specific or all-correlated manner, and the random parameters, i.e. P1, P2 ... P9 are generated in vertices of each square as the same as legacy where the length of each square is the correlation distance.

- For the link 11 or link 12 from TRP to ST1 or to ST2, the random parameters are based on the interpolation according to the positions of ST1 or ST2 and random values P2 P7, P4 and P8.

- For the link 21 or link 22 from ST1 to UE1 or from ST2 to UE2, the random parameters are based on the interpolation according to the positions of UE1 or UE2 and random values P4 P6, P8 and P9.

Based on this procedure, the spatial consistency between sensing targets is realized for the link from TRP to ST. Similarly, the spatial consistency between UEs is realized for the link from ST to UE.

Figure 11-1 ISAC spatial consistency

In addition, the current spatial consistency modeling in TR38.901 is primarily 2D-based. However, for certain scenarios, such as UAVs, a 3D-based spatial consistency model may be required. Straightforwardly, the existing 2D-based spatial consistency method can be easily extended to meet 3D spatial consistency needs. This can be achieved by first extending the 2D grid topology to a 3D grid topology and then applying a 3D interpolation method to obtain random values. This can be applicable for both Htarget and Hbackground.

Proposal 12: For spatial consistency of ISAC channel modeling, reuse the existing site-specific or all-correlated type parameters, and random variables are generated separately for the link from Tx to ST and the link from ST to Rx

- Extend the existing 2D-based spatial consistency to 3D-based spatial consistency for at least UAV case

Background for mono-static

Agreement On the background channel for TRP-TRP and UE-UE bistatic sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as starting point - Update on values of the LSP/SSP parameters can be discussed based on validation data acquired by measurement or ray-tracing model - FFS The power threshold for removing clusters in step 6, i.e., -25 dB is revised to X<-25 dB. FFS X - FFS whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height

As shown in the above agreement, for background channel modeling in the case of bi-static sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 have been agreed as the starting point.

In the current TR 38.901 for stochastic channel modeling between gNB and UE, the basic procedures are shown in the following figure, where Pathloss computation, AOA/AOD and d3D for LOS are based on the UE location in the scenario. Pathloss is usually increased according to d3D.

Figure 12-1 The procedures of TR 38.901

Assuming TR 38.901 framework can reused also for mono-static sensing modes, the key issue is how to compute pathloss, angle for LOS reference, and d3D for the LOS reference. One approach is to drop a virtual Rx location, and further reuse the methodology and procedures specified in 3GPP TR 38.901. As shown in the following the figure, a virtual Rx location is introduced dedicated for background environment channel modeling in the case of BS mono-static sensing mode [9]. When dropping the virtual Rx locations, it is better to ensure the generated channel aligning with the background channel from the real measurement or Ray-tracing simulation.

Figure 12-2 Dropping a virtual Rx to generate background channel based on TR 38.901 framework for mono-static sensing mode

In other words, the environment channel modeling can be generated between a BS Tx and the virtual Rx. The procedure of 3GPP TR 38.901 [2] can be reused then, for example, the pathloss computation is based on the setting scenario, the height of the virtual Rx, and distance between BS Tx and the virtual Rx. For different scenarios, for example, UMa, or UMi, or Indoor, the location distribution of virtual Rx per BS or per sector could be different.

Figure 12-3 Virtual Rx to generate Hbackground

As shown in the above figure, the only issue is to study how to drop the virtual Rx for each sector or gNB, where the channel characteristic between the Tx and the virtual Rx shown in the right figure should well reflect the channel of gNB mono-static sensing shown in the left figure. From our view, the channel characteristic should at least include pathloss.

To verify the feasibility, a 500m x 500m urban area (UMa scenario) in Washington D.C. is selected as the Ray-tracing (RT) simulation environment to evaluate the positioning of virtual Receivers (RX) in the scenario. Specifically, mono-static sensing transceiver with 25m height is randomly dropped outdoor for 1000 times, and each drop time is assigned with eight random yaw angles to expand drops. The transceiver is set to 12 degree down-tilt. In each time, all rays with strength larger than strongest ray power minus 25dB are selected as valid rays. All AoD, ZoD, delay and pathloss information of valid rays can be seen as Monte-Carlo from RT-simulation world and describe RT-simulated distribution for pathloss, delay, AoD and ZoD specifically, which are used to fit the distribution of location of virtual Rx.

Meanwhile, virtual Rx is randomly dropped for 1000 times, and in each time, pathloss, a list of delay (for N clusters), a list of AoD, and a list of ZoD are generated based on formula defined in TR 38.901 and dropped locations of virtual Rx. Then all Monte-Carlo samples of pathloss, delay, AoD and ZoD could describe sampled distribution for pathloss, delay, AoD and ZoD specifically. We modify the distribution of virtual Rx, trying to make the sampled distribution fitting the RT-simulated distribution.

Figure 12-4  Map of selected UMa scenario in Washington D.C

Figure 12-5  3D model of buildings in selected area

The distribution of location of virtual Rx is defined by parameterized Gamma distribution with the method of gradient descent. Fitted parameters are shown as follows.

During the fitting process, we find it is better to set the LOS AOD of virtual Rx as 0. That means we assume that the LOS horizontal angle between Tx and the virtual Rx is always zero.

Figure 12-6  Gamma distribution to fit Pathloss

Based on the fitted distribution, virtual Rx could be dropped following the distribution, and the related sampled distribution for pathloss, delay, AoD and ZoD could be generated based on dropped virtual Rx and step 5 to step 7 defined in clause 7.5, TR 38.901. All parameters are generated in NLOS condition specified in TR 38.901 and the height of virtual Rx is forced to .

CDF of sampled distribution and RT-simulated distribution is visualized as following figure. The blue curve is derived from all valid rays of multiple iterations in RT simulation, and the yellow curve is derived from generated parameters based on fitted virtual Rx. To derive the statistical PDF of sampled distribution and RT-simulated distribution, a histogram is calculated for each parameter, and an envelope curve are generated as statistical PDF curve based on all centers of histogram bins.

| - CDF of pathloss | - PDF of pathloss |
| --- | --- |
| - CDF of Delay | - PDF of Delay |
| - CDF of AoD | - PDF of AoD |
| - CDF of ZoD | - PDF of AoD |

Figure 12-7 CDF and PDF of sampled distribution and RT-simulated distribution

Proposal 13: For Hbackground modelling in the case of mono-static sensing mode, a virtual Rx location can be randomly generated, and then the stochastic mechanism of TR 38.901 between the real Tx and the virtual Rx is reused.

- For UMa scenario, the virtual RX follows that

  - Gamma distributions can be used to generate the distance between Tx/Rx and the virtual Rx, and to generate the virtual Rx height

  - The virtual Rx can be fixed in the bore-sight direction of each TRP in horizontal domain

  - Pathloss can be derived by TR 38.901 based on Tx/Rx and the virtual Rx

  - Delay, AoD and ZoD of clusters can be derived by TR 38.901 based on Tx/Rx and the virtual Rx

Power normalization between target and background

A working assumption with a FFS “whether/how to normalize power accounting for target channel and background channel” was approved in RAN#118 meeting [16].

Working assumption The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B - A first RCS component A (m2) is included in large scale - FFS the first RCS component is deterministic or stochastic - FFS The first component is dependent on incident and scattered directions at target - A second RCS component B (unit ratio) is included in small scale - FFS The second component is dependent on incident and scattered directions at target - FFS the second RCS component is deterministic or stochastic or combination - Note: RCS component A or B can be disabled by setting its linear value to 1 - Whether to disable a component can be discussed per target type - FFS how to determine A and B for each target - FFS whether/how to model polarization impact at target - FFS whether/how to normalize power accounting for target channel and background channel

To verify the necessity of power normalization between target channel and background channel, some simulations regarding pathloss are conducted in this section.

- For the background channel, the NLOS pathloss is computed based on 3GPP TRs, e.g. the formulas in Table 7.4.1-1 of TR 38.901.

PLbackground=PLnlos(d)

- For the target channel of direct path, the pathloss of the target channel is based on the following formula, where PLlos is LOS pathloss based on 3GPP TRs. δ is RCS value in large scale.

PLtarget=PLlos(d1)+PLlos(d2)−10log10(δ)+10log10(λ24π)

In the simulation, bi-static sensing mode is assumed for all cases. For simplicity, the equal distance from Tx to target and from target to Rx is assumed.

UMa-UAV with δ= -20dBsm                          UMa-Vehicle with δ= 10dBsm

InF-Human with δ= 0dBsm                             InF-Vehicle with δ= 10dBsm

Indoor Office-Human with δ= 0dBsm

Case 1: For the UMa scenario

- For UAV case, it can be observed that the pathloss of the target channel is at least approximately 40 dB larger than that of the background channel.

- For Vehicle case, it can be observed that the pathloss of the target channel is at least approximately 11 dB larger than that of the background channel.

Case 2: For the Indoor Factory scenario

- For Human case, it can be observed that the pathloss of the target channel is at least approximately 30 dB and 35dB larger than that of the background channel of InF-DL and InF-SL respectively. Moreover, they are greater differences approximately 40 dB for InF-DH and InF-SH scenario.

- For Vehicle case, it can be observed that the pathloss of the target channel is at least approximately 19dB and 25dB larger than that of the background channel of InF-DL and InF-SL respectively. Moreover, they are greater differences approximately 30 dB for InF-DH and InF-SH scenario.

Case 3: For the Indoor Office scenario

- For Human case, it can be observed that the pathloss of the target channel is at least approximately 8 dB larger than that of the background channel.

It is noted that the pathloss formula was based on the real measurements carried on for NR channel modeling in Rel-14 around 2015. In that time, the sensing targets such as UAV, vehicles, AGV, etc. were not considered. On top of the background channel, if a sensing target newly comes in, the totally receiving power should be slightly increased correspondingly especially when there is direct path in the target channel link Tx-ST-Rx, and the RCS gain is large. This is similar as passive IRS (Intelligent Reflecting Surface) which is used to boost the communication power. Thus, normalization between target channel and background channel should not be used.

Furthermore, target channel power especially for the direct path should be independent from the background channel. If normalization is done between target channel and background channel, this would lead to different direct path power in different scenarios. For example, for a target channel with direct path of a UAV, the receiving power at Rx in UMi scenarios and RMa scenario will be different after normalization because the background channel is different. This would not be aligned with the reality.

In conclusion, there is no need to normalize power accounting for target channel and background channel.

Proposal 14: There is no need to normalize power accounting for target channel and background channel.

EO type 2

Agreement When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference - As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00−R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901) - FFS any update to b,R∥specular,R⊥specular - FFS any update taking EO orientation into account - FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size - FFS whether diffraction and scattering can be considered in addition to specular reflection - EO type-2 is an optional modelling component if supported in a sensing scenario - FFS which deployment scenario(s) EO type-2 will apply

We research the reflection model of EO type-2, and the academic works based on Maxwell equation shown that the reflection model could be described clearly based on plane of Incidence, containing the incident ray and the normal vector to the surface. The incidence electric wave should be split to the component perpendicular to the plane of incidence, and component parallel to the plane of incidence. For the ground reflection model, it is fortunately to see the vertically polarized wave as the component parallel to the plane of incidence and horizontal polarized wave as the component perpendicular to the plane of Incidence directly. This is the reason that we can give one matric between the antenna VH gain of Tx and Rx directly. However, for EO type-2 such as wall of buildings typically, which is perpendicular to the ground, we can’t easily describe the relationship between perpendicular / parallel component to the plane of incidence and V/H polarized wave using a simple diagonal matrix. Thus, it is hard to model the effect of EO type-2 with one matrix. We suggest that the effect of EO type-2 could be modeled as following equation.

In which the  and describe ZoD and AoD from Tx to the reflected point on the EO-type2, and  and  describe ZoA and AoA from Rx to the reflected point. The matrix  is used to transform the VH polarized components to the incident parallel / perpendicular components related to the plane of incidence. The matrix  is used to transform the reflected parallel / perpendicular components to VH polarized components of Rx. The matrix  describe the reflected coefficient of parallel / perpendicular components. All parameters in the above equation could be derived as following steps.

Step 1: Calculated the reflected point

The EO type-2 could be described as any plane equation perpendicular to the ground. Considering the EO-type2 is defined as a reflection plane perpendicular to ground, the EO-type2 could be visualized as the Figure 14-1.

Figure 14-1 illustration of EO-type2

Thus we assume the plane equation of EO type-2 is known as

.

in which the actual value of A and B could be given by

The location of Tx and Rx is also known as  and . The distance between Tx and EO is .

The reflected image of Tx is  and

Based on the reflected image of Tx and the plane equation of EO, the location of reflected point could be obtained as  and

If the reflected point is on the EO, which is , the EO is determined as activated to create a reflected path. Otherwise, the EO is not modeled for this Tx-Rx pair.

Step 2: Calculated the ZoD and AoD from Tx to the reflected point on the EO-type2

The ZoD and AoD from Tx to the reflected point could be derived based on the location of Tx and reflected point.

Step 3: Calculated the rotation angle  and the rotation matrix of .

To reuse the definition of reflected coefficients  and , we should rotate the LCS to a new coordinate system whose polar basis vector of the spherical basis vectors is parallel to the plane of incidence, and azimuth basis vector of the spherical basis vectors is perpendicular to the plane of incidence. Thus we should rotate the current polar basis vector of incident ray with  and  to make the new rotated polar basis vector on the plane of incidence.

Firstly, the polar basis vector  and azimuth basis vector before rotating could be described in cartesian coordinate system.

The V polarized wave is transmitted along the direction of  and the H polarized wave is transmitted along the direction of .

The normal vector of plane of incidence could be described a cross product , in which

and

Thus we assume an anticlockwise rotation angle  is used to polar basis vector  and azimuth basis vector with the rotation axis as incidence wave from Tx to reflected point. The anticlockwise rotated polar basis vector and rotated azimuth basis vector could be described as

We want to get the anticlockwise rotation angle  making the new rotated polar basis vector is parallel to the plane of incidence, which means the new rotated polar basis vector is perpendicular to normal vector of plane of incidence, which is

Thus the anticlockwise rotation angle  could be derived as

Thus the rotation matrix could be derived by .

Figure 14-2 Anticlockwise rotate with axis of incident direction⊙

Step 4: Calculated the incidence angle  and related reflection coefficients,

The formula of reflection coefficients  and  could be reused, and only the incidence angle should be modified. Here the incidence angle  is the angle between the incident vector from Tx to reflected point and the normal vector, which is

Thus keep the reflection coefficients defined in TR38.901 unchanged, and use the new incidence angle .

Step 5: Calculated the ZoA and AoA from Rx to the reflected point on the EO-type2]

The ZoA and AoA from Rx to the reflected point could be derived based on the location of Rx and reflected point.

Step 6: Calculated the rotation angle  and the rotation matrix of .

Similar as Step 4, the reflection component parallel and perpendicular to the plane of incidence could be generated by rotating VH wave from Rx to reflection point on EO-type2.

Firstly, the polar basis vector  and azimuth basis vector, which is the united direction vector of V&H wave from Rx to reflection point on EO-type2, could be described in cartesian coordinate system.

Thus we assume a anticlockwise rotation angle  is used to polar basis vector  and azimuth basis vector with the rotation axis as incidence wave from Rx to reflected point. The anticlockwise rotated polar basis vector and rotated azimuth basis vector could be described as

We want to get the anticlockwise rotation angle  making the new rotated polar basis vector is parallel to the plane of incidence, which means the new rotated polar basis vector is perpendicular to normal vector of plane of incidence, which is

Thus the anticlockwise rotation angle  could be derived as

Thus the rotation matrix could be derived by .

Figure 14-3 Anticlockwise rotate with axis direction ⊙ from Rx to reflected point on EO-type2

Step 7: Combine rotation equation and get the final fast fading model of EO-type2

Given the above discussion, we have the following geometry relationships:

- The anticlockwise rotation equation of incident wave from Tx to EO-type2

- The anticlockwise rotation equation of incident wave from Rx to EO-type2

As a reverse operation, the clockwise rotation equation of departure wave from EO-type2 to Rx

- The relationship of departure wave

The reflection coefficients  and  describe the relationship between the strength of electrical wave along with  and , and the strength of electrical wave along with  and . However, the above equations use the base vector of and . Thus we should notice the following relationship.

The whole transformation relationship could be described as Figure 14-4.

Figure 14-4 The transformation relationship from Tx to EO and from EO to Rx

Therefore, the fast fading channel model of EO-type2 could be described as

Here a clockwise rotation is used, which has not been defined in TR 38.901. Thus we define , and the final fast fading channel model of EO-type2 could be described as

Proposal 15: Endorse the above step 1-7 for EO type 2 into TR for ISAC channel modeling, where the channel coefficient for the EO reflected path is given by

LLS

ISAC link-level simulation (LLS) is crucial for comparative assessment of technical solutions such as waveform/signal design and reception processing algorithms, with a focus on the performance of single-link target detection and parameter estimation. This type of simulation particularly focuses on the performance of single-link target detection and parameter estimation. Link-level simulation can model various effects that actual signals undergo during propagation and reception, such as attenuation, multipath, and noise interference, thus providing essential insights for system design and optimization.

In terms of performance assessment, LLS can evaluate the performance of different technical solutions in target detection and parameter estimation, providing decision support for system design. In terms of parameter and system optimization, LLS can optimize system parameters, such as transmission power, reception sensitivity, and signal processing algorithm parameters, to achieve the best system performance. By simulating different signal designs and processing algorithms, LLS can investigate how to improve the signal's interference resistance or optimize the bandwidth utilization. Therefore, it plays a significant role in modeling ISAC link-level systems.

Regarding ISAC LLS, we think leveraging the CDL channel model from TR 38.901 is a strategic decision that should be upheld, ensuring continuity and efficiency in the simulations whenever feasible. This approach streamlines our efforts, allowing us to concentrate on the critical aspects of the system's performance. That is, the extension of CDL channel modeling is suggested for ISAC LLS where the evaluation metrics are the accuracy estimation of angle, delay, and Doppler. Like as traditional CDL model for communication, there is no need to explicitly model gNB, UE, and target locations in the LLS. Thus, it is also unnecessary to explicitly consider different sensing modes.

Basically, there are two methods to model the channel of ISAC LLS:

- Method 1: Select one or more clusters of the existing CDL channel models to serve as the sensing clusters corresponding to the sensing target. Then give Doppler or velocity parameters for these sensing clusters.

- Method 2: Newly generate time delay, power, angle, and velocity parameters for the sensing clusters based on the number of sensing targets. These parameters can be set or randomly generated.

Based on the CDL-B in TR 38.901, there are two examples as following:

Table 7.7.1-2. CDL-B based on Method 1

| Cluster # | Normalized delay |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] | Velocity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0000 |  | 0 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 2 | 0.1072 |  | -2.2 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 3 | 0.2155 |  | -4 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 4 | 0.2095 |  | -3.2 |  | -34.1 |  | 125.5 |  | 115.3 |  | 63.3 | 0 |
| 5 | 0.2870 |  | -9.8 |  | -65.4 |  | -88.0 |  | 119.3 |  | 59.9 | 0 |
| 6 | 0.2986 |  | -1.2 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 7 | 0.3752 |  | -3.4 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 8 | 0.5055 |  | -5.2 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 9 | 0.3681 |  | -7.6 |  | -67.2 |  | -89.8 |  | 118.2 |  | 82.6 | 0 |
| 10 | 0.3697 |  | -3 |  | 52.5 |  | 132.1 |  | 102.0 |  | 66.3 | 0 |
| 11 | 0.5700 |  | -8.9 |  | -72 |  | -83.6 |  | 100.4 |  | 61.6 | 0 |
| 12 | 0.5283 |  | -9 |  | 74.3 |  | 95.3 |  | 98.3 |  | 58.0 | 0 |
| 13 | 1.1021 |  | -4.8 |  | -52.2 |  | 103.7 |  | 103.4 |  | 78.2 | 0 |
| 14 | 1.2756 |  | -5.7 |  | -50.5 |  | -87.8 |  | 102.5 |  | 82.0 | 0 |
| 15 | 1.5474 |  | -7.5 |  | 61.4 |  | -92.5 |  | 101.4 |  | 62.4 | 0 |
| 16(sensing cluster) | 1.7842 |  | -1.9 |  | 30.6 |  | -139.1 |  | 103.0 |  | 78.0 | 20 |
| 17 | 2.0169 |  | -7.6 |  | -72.5 |  | -90.6 |  | 100.0 |  | 60.9 | 0 |
| 18 | 2.8294 |  | -12.2 |  | -90.6 |  | 58.6 |  | 115.2 |  | 82.9 | 0 |
| 19 | 3.0219 |  | -9.8 |  | -77.6 |  | -79.0 |  | 100.5 |  | 60.8 | 0 |
| 20 | 3.6187 |  | -11.4 |  | -82.6 |  | 65.8 |  | 119.6 |  | 57.3 | 0 |
| 21 | 4.1067 |  | -14.9 |  | -103.6 |  | 52.7 |  | 118.7 |  | 59.9 | 0 |
| 22 | 4.2790 |  | -9.2 |  | 75.6 |  | 88.7 |  | 117.8 |  | 60.1 | 0 |
| 23 | 4.7834 |  | -11.3 |  | -77.6 |  | -60.4 |  | 115.7 |  | 62.3 | 0 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |  |
| Value |  | 10 |  | 22 |  | 3 |  | 7 |  | 8 |  |  |
| Value(sensing clusters) |  | 2 |  | 2 |  | 2 |  | 2 |  | 8 |  |  |

Table 7.7.1-2. CDL-B based on Method 2

| Cluster # | Normalized delay |  | Power in [dB] |  | AOD in [°] |  | AOA in [°] |  | ZOD in [°] |  | ZOA in [°] | Velocity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.0000 |  | 0 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 2 | 0.1072 |  | -2.2 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 3 | 0.2155 |  | -4 |  | 9.3 |  | -173.3 |  | 105.8 |  | 78.9 | 0 |
| 4 | 0.2095 |  | -3.2 |  | -34.1 |  | 125.5 |  | 115.3 |  | 63.3 | 0 |
| 5 | 0.2870 |  | -9.8 |  | -65.4 |  | -88.0 |  | 119.3 |  | 59.9 | 0 |
| 6 | 0.2986 |  | -1.2 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 7 | 0.3752 |  | -3.4 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 8 | 0.5055 |  | -5.2 |  | -11.4 |  | 155.1 |  | 103.2 |  | 67.5 | 0 |
| 9 | 0.3681 |  | -7.6 |  | -67.2 |  | -89.8 |  | 118.2 |  | 82.6 | 0 |
| 10 | 0.3697 |  | -3 |  | 52.5 |  | 132.1 |  | 102.0 |  | 66.3 | 0 |
| 11 | 0.5700 |  | -8.9 |  | -72 |  | -83.6 |  | 100.4 |  | 61.6 | 0 |
| 12 | 0.5283 |  | -9 |  | 74.3 |  | 95.3 |  | 98.3 |  | 58.0 | 0 |
| 13 | 1.1021 |  | -4.8 |  | -52.2 |  | 103.7 |  | 103.4 |  | 78.2 | 0 |
| 14 | 1.2756 |  | -5.7 |  | -50.5 |  | -87.8 |  | 102.5 |  | 82.0 | 0 |
| 15 | 1.5474 |  | -7.5 |  | 61.4 |  | -92.5 |  | 101.4 |  | 62.4 | 0 |
| 16 | 1.7842 |  | -1.9 |  | 30.6 |  | -139.1 |  | 103.0 |  | 78.0 | 0 |
| 17 | 2.0169 |  | -7.6 |  | -72.5 |  | -90.6 |  | 100.0 |  | 60.9 | 0 |
| 18 | 2.8294 |  | -12.2 |  | -90.6 |  | 58.6 |  | 115.2 |  | 82.9 | 0 |
| 19 | 3.0219 |  | -9.8 |  | -77.6 |  | -79.0 |  | 100.5 |  | 60.8 | 0 |
| 20 | 3.6187 |  | -11.4 |  | -82.6 |  | 65.8 |  | 119.6 |  | 57.3 | 0 |
| 21 | 4.1067 |  | -14.9 |  | -103.6 |  | 52.7 |  | 118.7 |  | 59.9 | 0 |
| 22 | 4.2790 |  | -9.2 |  | 75.6 |  | 88.7 |  | 117.8 |  | 60.1 | 0 |
| 23 | 4.7834 |  | -11.3 |  | -77.6 |  | -60.4 |  | 115.7 |  | 62.3 | 0 |
| sensing cluster | 0.3681 |  | -3 |  | 52.5 |  | 102.1 |  | 102.0 |  | 66.3 | 20 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Parameter |  | cASD in [°] |  | cASA in [°] |  | cZSD in [°] |  | cZSA in [°] |  | XPR in [dB] |  |  |
| Value |  | 10 |  | 22 |  | 3 |  | 7 |  | 8 |  |  |
| Value(sensing clusters) |  | 2 |  | 2 |  | 2 |  | 2 |  | 8 |  |  |

For the above two methods, the generation of channel coefficients and other processes will continue to follow the methodology of SLS.

For Method 1, the CDL channel from TR 38.901 is essentially used in its entirety, with relatively minor modifications. For Method 2, additional random clusters corresponding to the target are generated, and the existing CDL channel can be considered as the background channel, aligning with the system-level simulation channel currently under standardization.

Proposal 16: Using the CDL channel as the starting point for research on ISAC LLS.

Hybrid channel modelling with RT simulation

Hybrid model based on ray tracing simulation is composed of a deterministic component and a stochastic component. In TR 38.901, the hybrid system is designed to evaluate or predict the impacts for communication from environmental structures and materials with the use of digital map. The sensing task makes a requirement for a more accurate and deterministic sensing channel.

For sensing with hybrid channel modelling, the sensing channel related to targets and some environmental objects affecting sensing targets severely should be modelled by ray-tracing method with a higher determinacy and predictability. The environmental channels related background wireless environment, such as ground, woods, buildings, and other things, can be assumed as random clutters and be modelled by stochastic geometry modelling method in TR 38.901 without any changes.

Figure 16-1 Hybrid Channel coefficient generation procedure

Basically, the procedures in TR 38.901 section 8 can be reused with slight modification. This type of modeling is very useful for sensing from our view especially on the impact from the important environmental objects. To align companies’ simulation results, it is better to specify some typical scenarios, characteristics of sensing targets. In other words, digital maps in the simulation should be aligned for scenarios to be evaluated by method of ray-tracing, and some typical maps can be recommended to be researched first.

Based on the further details of analysis in section 3.1, 3.2 and 3.3, we have the following proposal

Proposal 17: The procedure of hybrid channel modelling with RT simulation in TR 38.901 can be reused and enhanced for sensing. Specify the following typical maps and characteristics of sensing targets to align the simulation assumptions:

- Urban grid map defined in 3GPP TR 37.885

- The well-known Manhattan map from open source

- Indoor map defined in IEEE 802.11 WLAN

- EM parameters defined for radar material by ITU

Outdoor map

For outdoor scenarios, we recommend two well-known maps as follows.

- Urban grid in TR 37.885 [4].

Parameters regarding the road configuration for urban grid has been included. Figure 16-2 shows illustrative diagrams of urban grid which is simple for RT simulation. Based on this map, the NLOS links from buildings to sensing targets, and then further to Rx/Tx can be deterministically modeled. Since the location and EM characteristics of buildings are known in the real deployments, such NLOS links may be helpful for sensing detection.

Figure 16-2 Road configuration for urban grid in TR 37.885

- Manhattan map

Manhattan map is a common used urban city model for evaluation of wireless network configuration, and also be recommended to be starting point of hybrid channel modeling method. The digital map of Manhattan can be download from open street map [5] (https://www.openstreetmap.org), which is an open source containing comprehensive maps of most countries, and models of buildings, water objects, forests, roads, railways are all provided by open street map. For simplicity, only building models are proposed to be used for hybrid channel modeling.

For specified coordinate of latitude and longitude, a 500-meter square partial map of Manhattan with latitudes 40.7689 to 40.7736 and longitude -73.961 to -73.955 can be used. The detailed map is shown as follows.

Figure 16-3 Detailed map of part of Manhattan city

Figure 16-4 Digital map of Manhattan city

Indoor map

Indoor map defined in IEEE 802.11 WLAN can be reused in our view. Set up of the living room scenario is shown in Figure 16-5. The dimensions of a living room are 7 m x 7 m x 3 m (L x W x H). The living room has a table in the middle, a chair and a small chair with feet placed around the table. One outer wall has two windows and the opposite outer wall has a door placed in its middle. The target is a cubic object which is made of wood, as shown in Figure 16-6.

Figure 16-5 Set up of the living room scenario.

Figure 16-6 Target illustration and parameters.

The dimensions and materials of all furniture are given in Table 6.

Table 6 Parameters of the furniture dimensions and materials

| Object | Size | Material |
| --- | --- | --- |
| Chair | 2.8m x 1.05m x 1m (L x W x H) | wood |
| Chair with feet | 1.05m x 1.05m x 1m (L x W x H) | wood |
| Table1 | 1.5m x 0.7m x 0.8m (L x W x H) | wood |
| Table2 | 1m x 0.5m x 0.8m (L x W x H) | wood |
| Target | 0.3m x 1.5m (R x H) | wood |
| Door | 1m x 2.5m (W x H) | wood |
| Windows | 1.5m x 1.5m (W x H) | Glass |
| Wall |  | concrete |
| Floor |  | concrete |
| Ceiling |  | concrete |

EM parameters

For ray-tracing simulation, the radar material of object in digital map affect the simulation result a lot. The defined radar material by ITU is recommended to be reused [7] [8]. For each material, simple expressions for the frequency-dependent values of the real part of the relative permittivity, ', and the conductivity, , were derived. These are:

and:

where f is frequency in GHz and  is in S/m. (' is dimensionless.) The values of a, b, c and d are given in Table 7. Where the value of b or d is zero the corresponding value of  or  is a or c respectively, and independent of frequency.

If required, the imaginary part of the relative permittivity " can be obtained from the conductivity and frequency:

Parameters for air, metal and three conditions of ground are included in Table 7 for completeness.

Table 7 Material properties

| Material class | Real part of relative permittivity |  | Conductivity S/m |  | Frequency range |
| --- | --- | --- | --- | --- | --- |
|  | a | b | c | d | GHz |
| Vacuum (≈ air) | 1 | 0 | 0 | 0 | 0.001-100 |
| Concrete | 5.24 | 0 | 0.0462 | 0.7822 | 1-100 |
| Brick | 3.91 | 0 | 0.0238 | 0.16 | 1-40 |
| Plasterboard | 2.73 | 0 | 0.0085 | 0.9395 | 1-100 |
| Wood | 1.99 | 0 | 0.0047 | 1.0718 | 0.001-100 |
| Glass | 6.31 | 0 | 0.0036 | 1.3394 | 0.1-100 |
| Glass | 5.79 | 0 | 0.0004 | 1.658 | 220-450 |
| Ceiling board | 1.48 | 0 | 0.0011 | 1.0750 | 1-100 |
| Ceiling board | 1.52 | 0 | 0.0029 | 1.029 | 220-450 |
| Chipboard | 2.58 | 0 | 0.0217 | 0.7800 | 1-100 |
| Plywood | 2.71 | 0 | 0.33 | 0 | 1-40 |
| Marble | 7.074 | 0 | 0.0055 | 0.9262 | 1-60 |
| Floorboard | 3.66 | 0 | 0.0044 | 1.3515 | 50-100 |
| Metal | 1 | 0 | 107 | 0 | 1-100 |
| Very dry ground | 3 | 0 | 0.00015 | 2.52 | 1-10 only |
| Medium dry ground | 15 | −0.1 | 0.035 | 1.63 | 1-10 only |
| Wet ground | 30 | −0.4 | 0.15 | 1.30 | 1-10 only |

Conclusion

In this contribution, we provide our measurements, RT simulations, observations, and analysis on ISAC channel modelling, and we have the following observations and proposals:

Proposal 1: Clarify RCS components A, B1, B2 as following

- For B2,  to ensure the mean value  be equal to 1

- For B1,  to ensure the mean value  be equal to 1

- For A,  and

Proposal 2: Denote

CPMtx,sp,rx = CPMsp,rx . CPMsp . CPMtx,sp

= a11a12a21a22∙b11b12b21b22∙c11c12c21c22 = d11d12d21d22.

The normalized CPMtx,sp,rx  can be represented as

CPMtx,sp,rxnorm =d11d12d21d2212(d11+d22)=2(d11+d22)d11d12d21d22

Proposal 3: For UAV of small size (Length x Width x Height: 0.3m x 0.4m x 0.2m) at 4.9GHz

  - B1=1

  - A is mean RCS value of 0.053m2 corresponding -12.74 dbsm

  - B2 of UAV is modelled using log-normal distribution. Draw B2 linear-scaled value as

B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for the UAV is modelled as log-Normal distribution. Draw linear-scaled XPR value as

 = 10Y/10,

Where,  is Gaussian distributed with 24.4and = 3.05.

  - The CPM for UAV is represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

Proposal 4: RCS Option 3 is selected to model RCS of pedestrian with single scattering point

  - For Adult pedestrian (approximately 0.5 meters in length, 0.5 meters in width, and 1.75 meters in height) at 4.9GHz

    - B1=1

    - A is mean RCS value of 0.4m2

  - B2 of pedestrian is modelled using log-normal distribution. Draw B2 values as

B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for pedestrian is modelled using log-Normal distributed. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 25.8 and = 2.99.

  - The CPM for UAV is represented as follows:

CPMsp=expjΦn,mθθ n,m−1expjΦn,mθφ n,m−1expjΦn,mφθexpjΦn,mφφ

Proposal 5: For mono-static, the RCS=A*B=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by

  - A has mean RCS value 11.72 m2 corresponding 10.69 dBsm

  - B1 is deterministic based on incident/scattered angles

B1dB = rcsdB(θ,φ)−6.61.

Where,

rcsdB(θ,φ)=Gmax−min−AVdBθ+AHdB φ,Amax

Where,

AVdBθ=−min12θ−θcenterθ3dB2, Amax,

AHdB φ=−min12φ−φcenterφ3dB2, Amax,

| Area/n | Side of vehicle | θcenter | φcenter | φ3dB | θ3dB | Amax | Gmax | Applicable range of ZoA/θ | Applicable range of AoA/φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Left | 78.8 | 0° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [0°, 45°]∪[315°, 360°] |
| 2 | Back | 71.6° | 90° | 39.6° | 43.9° | 2.76 | 4.62 | [20°, 90°] | [45°,135°] |
| 3 | Right | 78.8° | 180° | 47.5° | 99.5° | 5.8 | 9.5 | [20°, 90°] | [135°,225°] |
| 4 | Front | 20.0° | 270° | 95.7° | 48.4° | 10.05 | 9.49 | [20°, 90°] | [225°,315°] |
| 5 | Roof | 0° | - | - | 28.42 | 3.63 | 10.99 | [0°, 20°] | [0°,360°] |

    - B2 is generated by log-normal distribution

      - Draw B2 values as B2 =10X/10,

Where,  is Gaussian distributed with and .

  - XPR for the Vehicle is modelled using log-Normal distributed. Draw XPR values as

 = 10Y/10,

Where,  is Gaussian distributed with 22.3 and = 4.71.

Proposal 6: For Bi-static, the RCS modeling is RCS_car = rcsdB(θi,φi,θs,φs) + glint, where glint is Normal distribution, and

rcsdB(θi,φi,θs,φs)=Greflection−min−AVdBθs+AHdB φs,Amax

Wherein,

AVdBθs=−min12θs−θreflect(θi)θ3dB2, Amax,

AHdB φs=−min12φs−φreflect(φi)φ3dB2, Amax,

Greflection= Gmax_RCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2),

| Side of vehicle | ZoA/θi | AoA/φi | (θn,φn) | φ3dBn | θ3dB | Amax | Gmax_RCS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [20°, 90°] | [0°, 90°]∪ [270°, 360°] | (78.8, 0) | 47.5° | 99.5° | 5.8 | 9.5 |
| Back | [20°, 90°] | (0°,180°) | (71.6, 90) | 39.6° | 43.9° | 2.76 | 4.62 |
| Right | [20°, 90°] | (90°,270°) | (78.8, 180) | 47.5° | 99.5° | 5.8 | 9.5 |
| Front | [20°, 90°] | (180°,360°) | (20, 270) | 95.7° | 48.4° | 10.05 | 9.49 |
| Roof | [0°, 20°] | (0°,360°) | (0, -) | - | 28.42 | 3.63 | 10.99 |

Note： if θn=0,  then  Greflection = Gmax_RCS*cos(θi).

Proposal 7: For a target modeled with M>=1 scattering points, the target channel for those M scattering points are generated independently

- The number of visible scattering points is fixed to M, i.e. not depending on distance between Tx/Rx and the target.

- The number of scattering points of a target is generated at the beginning of the simulation and keeps unchanged

- For vehicle, M = 5

- The relative locations of the M scattering points in LCS of the target are based on the shape of the target

Proposal 8: For the concatenation of Tx-target and target-Rx link in the target channel:

  - On indirect paths of NLOS + NLOS, support Option 3A

- Option 3A: ray level 1-by-1 coupling between Tx-target link and target-Rx link only for radio propagation Case 4

  - Power normalization is not needed after paths dropping

Proposal 9: For single scattering point modeling, the number of NLOS clusters is 13 of that defined in 38.901

  - Each NLOS cluster includes 20 rays as the same as defined TR 38.901

Proposal 10: It is unnecessary to model a propagation path from Tx to Rx interacting with more than one sensing targets.

Proposal 11: For Htarget, the Doppler frequency frequency is adopted as

vn,m=rrx,n,mT∙(vrx−vST)+rtx,n,mT∙(vtx−vST)λ0t + 2αn,mDn,mλ0t

Where the component 2αn,mDn,mλ0 specified in TR 38.901 section 7.6.10 is disabled for the direct path

Note: the detailed modeling of vST can be discussed in performance evaluation stage.

Proposal 12: For spatial consistency of ISAC channel modeling, reuse the existing site-specific or all-correlated type parameters, and random variables are generated separately for the link from Tx to ST and the link from ST to Rx

- Extend the existing 2D-based spatial consistency to 3D-based spatial consistency for at least UAV case

Proposal 13: For Hbackground modelling in the case of mono-static sensing mode, a virtual Rx location can be randomly generated, and then the stochastic mechanism of TR 38.901 between the real Tx and the virtual Rx is reused.

- For UMa scenario, the virtual RX follows that

  - Gamma distributions can be used to generate the distance between Tx/Rx and the virtual Rx, and to generate the virtual Rx height

  - The virtual Rx can be fixed in the bore-sight direction of each TRP in horizontal domain

  - Pathloss can be derived by TR 38.901 based on Tx/Rx and the virtual Rx

  - Delay, AoD and ZoD of clusters can be derived by TR 38.901 based on Tx/Rx and the virtual Rx

Proposal 14: There is no need to normalize power accounting for target channel and background channel.

Proposal 15: Endorse the above step 1-7 for EO type 2 into TR for ISAC channel modeling, where the channel coefficient for the EO reflected path is given by

Proposal 16: Using the CDL channel as the starting point for research on ISAC LLS.

Proposal 17: The procedure of hybrid channel modelling with RT simulation in TR 38.901 can be reused and enhanced for sensing. Specify the following typical maps and characteristics of sensing targets to align the simulation assumptions:

- Urban grid map defined in 3GPP TR 37.885

- The well-known Manhattan map from open source

- Indoor map defined in IEEE 802.11 WLAN

- EM parameters defined for radar material by ITU

Reference

- RP-234069 RAN New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, 3GPP TSG RAN Meeting #102.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz, V17.1.0, 2024-01.

- 3GPP TR 36.777, Enhanced LTE support for aerial vehicles, V15.0.0, 2018-01.

- 3GPP TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR, V15.3.0, 2019-06.

- Map, Open Street. "Open street map." Acessado em 12 (2017).

- 11-21-0782-02-00bf-channel-models-for-wlan-sensing-systems.doc

- https://www.itu.int/rec/R-REC-P.2040/en

- https://www.itu.int/rec/R-REC-P.527/en

- Jiang, Chuangxin, et al. "A Novel Approach to Model the Scattering Environment in Channel Modeling for Integrated Sensing and Communications." 2024 international wireless communications and mobile computing conference (IWCMC). IEEE, 2024 (accepted).

- R1-2402704 in RAN1#116bis meeting, Discussion on channel modelling for ISAC,  ZTE

- Feldman, Yuri, et al. "The electromagnetic response of human skin in the millimetre and submillimetre wave range." Physics in Medicine & Biology 54.11 (2009): 3341.

- Degli-Esposti, Vittorio, et al. "Measurement and modelling of scattering from buildings." IEEE transactions on antennas and propagation 55.1 (2007): 143-153.

- Schuh M J , Woo A C , Simon M P .The monostatic/bistatic approximation[J].IEEE Antennas & Propagation Magazine, 1994, 36(4):p.76-78.DOI:1045-9243(1994)36:4<76:TMBA>2.0.TX;2-D.

- Jr R L E , Collins P J , Jr A J T ,et al.Bistatic scattering characterization of complex objects[J].Geoscience & Remote Sensing IEEE Transactions on, 2000, 38(5):2078-2092.DOI:10.1109/36.868867.

- R1-2404190 in RAN1#117 meeting, Views on Rel-19 ISAC channel modelling, vivo

- Draft_Minutes_report_RAN1#118_v010

- R1-2408515 in RAN1#118bis meeting, Discussion on channel modelling for ISAC,  ZTE