3GPP TSG-RAN WG1 Meeting #119                                                                                                            	                                          R1-2410463

Orlando, Florida, USA, November 18th-22nd, 2024

Agenda item:	5

Source: 	Qualcomm Incorporated

Title: 	Discussion for LS reply on applicable functionality reporting for beam prediction

Document for:	Discussion and Decision

# Introduction

RAN2 has sent an LS to RAN1 on applicable functionality reporting for beam management UE-sided models. RAN2 has provided a call flow diagram and has posed questions for RAN1 with regards to the whole flow, for the purpose of enabling beam management operation during inference for UE-sided models. In the following, we cite some aspects from the LS including some agreed RAN2 terminologies on functionality and the call flow diagram.

RAN2 agreed the following understandings on terminologies: Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference. RAN2 further has made following agreements and signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. RAN2 also agreed the applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality. The above agreements were made based on the following assumptions: NW-side additional condition is assumed as associated ID in RAN2 (which is assumed by majority of companies).

# Discussion

Prior to answering RAN2 questions in Section 2.2, we first discuss some aspects related to RAN2 terminologies and the corresponding RAN1 terminologies, as well as our view in how the call flow diagram should be designed to enable inference operation of beam prediction for UE-sided models. In section 5 of our beam management contribution, we describe our view with regards to the agreed options for applicability for inference for UE-sided models as well as the feasibility of UE determining applicable functionalities without associated ID, and we do not include those here to avoid duplication. Major intent of this contribution is to answer to the remaining RAN2 questions.

## Enabling inference operation of beam prediction for UE-sided models.

We present some aspects that are related to the overall flow in this section.

### Converting agreed RAN2 terminologies to RAN1 Specification language

Let us consider the following RAN2 agreed terminologies on functionality

Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) Applicable functionalities refer to functionalities that the UE is ready to apply for inference Activated functionalities refer to functionalities already enabled for performing inference

In any of the steps 2 through 5, RAN1 should try to map the agreed RAN2 terminologies on functionality to the RAN1 specification language and discuss how the agreed RAN2 terminologies can be materialized considering RAN1 agreed terminologies. As an example, “supported functionalities” would refer to AI/ML-specific UE capability parameters that are going to be dependent on FG definitions for the beam prediction use case. As another example, the proper mechanism for activating a functionality for beam prediction in RAN1 would be to utilize CSI-ReportConfig (further details mentioned in the remainder of the document). So RAN1 should strive to map these RAN2 terminologies to RAN1 language in a concrete way, rather that repeating those terminologies with no clear RAN1 mapping.

### Call flow diagram for enabling inference operation for beam prediction

Prior to answering RAN2 questions, we think it is important to have a bird’s eye view to see how the entire process can work, and then focus on each individual step in the call flow diagram. The steps in the call flow diagram are inter-dependent, and unless we do not have an overall vision of how the entire flow works, it would be very difficult to design each individual step in a standalone manner and then move to the next step. With that said, we discuss our vision of how the overall flow should be structured in this section, and then we move on to Section 2.2 to answer the RAN2 questions.

We describe our proposed solution in the following:

In Step 3, NW sends the associated ID(s) (for current NW-side additional conditions), as well as at least one CSI-ReportConfig. In the following we discuss why these two pieces of information are needed in order for the UE to determine the applicable functionality between Steps 3 and 4:

  - Necessity of signalling associated ID(s) in Step 3: It is important to highlight that RAN1 has only a single supported solution for ensuring consistency of NW-side additional conditions across training and inference, which is based on associated ID. The other solution which is ensuring consistency based on performance monitoring (still FFS in RAN1) is not feasible in our view, at least due to the following reasons:

    - If only inference configuration is indicated in Step 3, the UE will not know which AI/ML model to download (will have no idea about applicability).

    - Even if UE downloads multiple AI/ML models blindly to try and see which one (if any) performs better for this cell, this “trying” would not be possible between Steps 3 and 4, as there is even no inference, hence no monitoring.

- Therefore, signalling of associated ID(s) in Step 3 is essential for the UE to determine the applicable functionality, and without such information determining applicable functionality is not feasible.

  - Necessity of signalling inference configuration in Step 3: Per RAN1 agreements so far, there is not a one-to-one mapping between associated ID and inference configuration (i.e., CSI-ReportConfig). In other words, a single associated ID may be attributed to multiple inference configurations in which case it will not be possible for the UE to determine the applicable functionality. Therefore, the inference configuration should also be included in Step 3. Even though inference configuration is included in Step 3, it is not activated until after Step 4 (which is the time that UE has the model available for inference and notifies NW about this in Step 4).

Between Steps 3 and 4: Based on the associated ID(s) and at least one CSI-ReportConfig UE should be able to download the right model or models between Steps 3 and 4. Also, based on UE-side additional conditions, UE should be able to identify the applicable CSI-ReportConfig.

Step 4: UE signals the applicable CSI-ReportConfigId(s) to the gNB in Step 4. After Step 4, the applicable functionality is ready to be activated.

Step 5: Step 5 is the activation step in which the inference configuration in Step 3 is activated. For A/SP configurations, MAC-CE/DCI can be used for activation. For periodic report configurations the activation of the inference configuration in Step 3 should wait until Step 5.

In the next section, we address the questions posed by RAN2 on the above procedures.

## RAN2 Questions

In RAN1 #118-bis we made the following agreements with regards to RAN2 LS questions:

Agreement

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

Agreement

Answer to Q1 in R1-2407604 as below,

In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Agreement

Answer to Q2 in R1-2407604 as below:

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

In this section, we address the remaining questions posed by RAN2 on applicable functionality reporting for beam management of UE-sided models.

### Question 3

RAN2 question: Q3: Is NW-side additional condition functionality specific? - The definitive answer to this question at least depends on the following factors: - The answer to Q1. - Further progress of RAN1 on where/how to configure associated ID.

### Question 4

RAN2 question: Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition? - RAN1 has agreed to support signalling of associated ID from NW to UE for the purpose of ensuring consistency of NW-side additional conditions across training and inference, which is a critical issue pointed out in WID. In order for the UE to determine whether it can predict beams during inference, it must know whether it has been trained with the same NW-side additional conditions before (which means receiving the same associated ID across training and inference). In the absence of such information on NW-side additional conditions (abstracted as associated ID), the UE will NOT be able to identify the applicability of its AI/ML models for the NW-side additional conditions during inference. Associated ID is the only solution supported in RAN1 to address the issue of consistency, So RAN2 can rely on the supported method for future design. The other solution based on monitoring is still FFS in RAN1, and there are serious feasibility concerns related to this method, including at least: - If only inference configuration is indicated in Step 3, the UE will not know which AI/ML model to download (will have no idea about applicability). - Even if UE downloads multiple AI/ML models blindly to try and see which one (if any) performs better for this cell, this “trying” would not be possible between Steps 3 and 4, as there is even no inference, hence no monitoring. - In our view, here’s the sequence of events for Step 3, Step 4, and Step 5: - In Step 3, NW sends the current associated ID(s) as well as the corresponding CSI-ReportConfig(s) to the UE. This CSI-ReportConfig is the one that is eventually going to be used for the purpose of enabling inference operation for beam prediction (hence different from legacy CSI-ReportConfig). - In Step 4, UE reports whether the indicated associated ID(s) and accompanying configuration(s) are applicable or not. - In Step 5, given applicability indication in Step 4 from UE, the configuration indicated in Step 3 is activated. For AP/SP CSI reporting, MAC-CE/DCI can be used for such activation. For P CSI reporting, the AI/ML CSI-ReportConfig behaviour should be different from legacy CSI-ReportConfig in that the configuration is NOT activated in Step 3, and it should wait until Step 5. - In summary, the inclusion of associated ID is necessary to enable UE to determine the applicability of its AI/ML models for the NW-side additional conditions during inference. - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities? - First, it is important to align the understanding that when RAN2 is referring to “inference configuration”, the equivalent in RAN1 terminology would be CSI-ReportConfig. Yes, the AI/ML configuration (CSI-ReportConfig for AI/ML, in contrast to legacy CSI-ReportConfig) needs to be provided to the UE in order for the UE to identify the applicable functionalities. Without such information, and in the absence of configuration information, UE will not be able to identify applicability of functionality. Note that there may not be a one-to-one mapping from associated ID to the underlying configuration, and we may have different configurations for the same associated ID, hence the reply above. - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc? - If “inference configuration” is meant to refer to AI/ML CSI-ReportConfig, then it should be included in Step 3. RAN1 has a working assumption that the associated ID at least can be configured within CSI framework, but further details are FFS. Associated ID can be configured within CSI-ReportConfig, hence it can be part of inference configuration. Further details about whether associated ID can be configured per RS resource set (e.g., Set A or Set B), or per a pair of RS resource sets (e.g., Set A and Set B) are still to be discussed in RAN1. - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities? - The inference configuration here is CSI-ReportConfig, and this is the available information that RAN1 has so far on this topic: - RAN1 has agreed that two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report. Further details with regards to inference configuration are still FFS. The inference configuration can be enabled by a CSI-ReportConfig for beam prediction and most of legacy contents for beam management and reporting are also applicable for beam prediction. RAN1 has a working assumption that the associated ID at least can be configured within CSI framework, but further details are FFS. The two main differences with legacy CSI-ReportConfig are: - How to configure Set A within CSI-ReportConfig, which is FFS - How/where to include associated ID, which is also FFS - This at least depends on whether associated ID can be configured per RS resource set (e.g., Set A or Set B), or per a pair of RS resource sets (e.g., Set A and Set B), that are still to be discussed in RAN1.

### Question 5

RAN2 question: Q5: What is the content of applicable functionality reporting in Step 4? In Step 4, UE reports whether the indicated associated ID and accompanying configuration is applicable or not.

### Question 6

RAN2 question: Q6: What is the content of inference configuration in Step 5? - There is no need to have the inference configuration in Step 5. Given applicability indication in Step 4 from UE, the configuration indicated in Step 3 is activated in Step 5. For AP/SP CSI reporting, MAC-CE/DCI can be used for such activation. For P CSI reporting, the AI/ML CSI-ReportConfig behaviour should be different from legacy CSI-ReportConfig in that the configuration is NOT activated in Step 3, and it should wait until Step 5.

### Questions 7-10

RAN2 question: Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? We cannot activate the functionality immediately upon receiving Step 3, as UE has not determined the applicable functionality yet. Activation should wait until Step 5, as explained above. Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5? As explained above, inference configuration needs to be provided in Step 3. Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? It is possible that multiple functionalities can be activated, and the details are up to further RAN1 discussions. Q10: Is L1/L2 signaling for functionality activation/deactivation needed? The current signalling in Spec for configuring/activating/triggering P/SP/AP CSI reports can be reused for activating a corresponding functionality, in which case L1/L2 signalling are applicable to at least enabling SP and AP reports.

# Conclusions

In this contribution, we present the replies to RAN2 LS on applicability functionality reporting for beam prediction of UE-sided models.