# 3GPP TSG-RAN WG1 Meeting #119															           R1-2410470

Orlando, Florida, USA, November 18th-22nd, 2024

Agenda item:	9.1.4.2

Source: 	Qualcomm Incorporated

Title: 	Other aspects of AI/ML model and data

Document for:	Discussion/Decision

# Introduction

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured into the Technical Report TR 38.843 [2]. Based on the discussions at RAN #105, the Work Item on artificial intelligence/machine learning for NR air interface was revised [3], with the following objectives:

Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work. Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

In this contribution, for the sub-agenda “other aspects of AI/ML model and data”, we discuss the following aspect among the Rel-19 objectives.

- Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1] - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

The following item was discussed in RAN2 and later the results were discussed in RAN #105, after which the outcome was sent to SA working groups for further discussion. Therefore, we do not discuss the topic in this document.

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

# Model identification concept and procedure in the context of LCM for two-sided models

Let us consider the updated objective in the WID, with regards to model identification:

- Necessity and details of model Identification concept and procedure in the context of LCM for two-sided models [RAN2/RAN1]

The change made in RAN #105 compared to original objectives in the WID was that the focus of model identification is on two-sided models. We first review the background on model identification.

## Background

Two LCM flavors, namely functionality-based LCM and model-ID-based LCM have been discussed in the Rel-18 study. Regardless of the naming, for the UE-side models (including the UE-part of two-sided models), the primary objective is to establish common understanding between NW and UE regarding the AI/ML capability/applicability at the UE-side and to facilitate UE-side model operations (either via model ID or transparently inside a functionality).

Now, let us review the agreements made so far in this agenda item related to model identification:

Agreement (RAN1 #116) - To facilitate the discussion, RAN1 studies the model identification type A with more details related to use cases. - To facilitate the discussion, RAN1 studies the following options as starting point for model identification type B with more details related to all use cases - MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) - MI-Option 2: Model identification with dataset transfer - MI-Option 3: Model identification in model transfer from NW to UE - FFS: The boundary of the options - Note: the names (MI-Opton1, MI-Option 2, MI-Option 3) are used only for discussion purpose - Note: other options are not precluded Observation (RAN1 #116) The other options are proposed for model identification type B by companies during the discussion: - MI-Option 4. Model identification via standardization of reference models. (for CSI compression) - MI-Option 5. Model identification via model monitoring Agreement (RAN1 #116-bis) From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification. Agreement (RAN1 #117) From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset(s) Conclusion (RAN1 #118) From RAN1 perspective, model identification is at least applicable to some of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported). Conclusion (RAN1 #118) The model identification procedure dedicated to MI-Option5 is not pursued for Rel-19 normative work. Conclusion (RAN1 #118) The model identification procedure dedicated to MI-Option2 for one-sided model is not pursued for Rel-19 normative work. Agreement (RAN1 #118-bis) Regarding the study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, ID (denoted as ID-X) can be transmitted from network/network-side to UE/UE-side for the dataset. - Note: The notation “ID-X” is used for discussion purpose

We believe the progress that has been made so far including identifying different model identification options, and further details and procedures associated with those options (cited above) are sufficient as far as the scope of this agenda item is concerned. The concepts and procedures related to two-sided models will be discussed further in the 9.1.4.1 agenda item, in which some tailored solutions will be discussed based on the specific requirements of two-sided models. With that said, we propose the following:

Proposal 1: Conclude that the progress made so far including identifying different model identification options, and further details and procedures associated with those options are sufficient as far as the scope of this agenda item is concerned.

# Model transfer/delivery

The following regarding model delivery/transfer has been agreed and captured in the TR through Rel-18 study:

From TR 38.843 ... Scenario/configuration specific (including site-specific configuration/channel conditions) models may provide performance benefits in some studied use cases (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites). - At least, when UE has limitation to store all related models, model delivery/transfer, if feasible, to UE may be beneficial, at the cost of overhead/latency associated with model delivery/transfer. - Note: On-device Finetuning/retraining, if feasible, of a single model may be an alternative to model delivery/transfer. - Note: a single model may generalize well in some studied use cases. - Note: Model transfer/delivery to UE may also face challenges, e.g., proprietary issues /burdens in some scenarios ... Table 4.3-1 introduces different options for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models: Table 4.3-1: Model delivery/transfer cases Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top.; Outside 3GPP Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format.; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format.; 3GPP Network; NW-side. z3; model transfer in open format.; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support.; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known.; 3GPP Network; NW-side. Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization.. When a model of a known structure at UE (e.g., Case z4) is transferred from the Network, the new model being identified (e.g., via Type B2) has the same structure as a previously identified model at the Network and UE. For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models): - Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement. - Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale. - Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE. - For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model. - For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden. - Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage. - Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue.

In RAN1 #118-bis, the following agreement was made:

Conclusion

From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized for two-sided model in Rel-19.

With the above background, let us review a summary of the status of model transfer/delivery cases so far:

Table 2 Summary of status for model transfer/delivery cases

| Model delivery/transfer | UE-sided model | Two-sided model |
| --- | --- | --- |
| Case y |  |  |
| Case z1 |  |  |
| Case z2 | Deprioritized | Deprioritized |
| Case z3 | Deprioritized | Deprioritized |
| Case z4 |  |  |
| Case z5 | Deprioritized | Deprioritized |

Down-prioritization of model transfer/delivery Case z2 for two-sided models was based on the fact that it was concluded in the 9.1.4.1 agenda item that such a model transfer/delivery case will not be utilized within that use case. Similar argument holds true for model transfer/delivery Case z4, as it relates to UE-sided models. The prospect of training UE-side AI/ML models at the NW side comes with numerous challenges. Considering devices’ capability and engineering practices, it is mandatory that, for any model delivered/transferred to the target UEs, the model (including its structure and parameters) should have been fully tested for the target UEs, and its support should have been indicated by the UE capability. Case z4 for UE-sided models does not satisfy this requirement, and therefore should be deprioritized. Regarding Case z4 for two sided models, since it is being discussed in 9.1.4.1 agenda item (inter-vendor collaboration options 3 and 5), we should wait for the discussions taking place in 9.1.4.1.

Proposal 2: Conclude that model transfer/delivery case z4 is deprioritized for UE-sided models.

# Conclusions

Proposal 1: Conclude that the progress made so far including identifying different model identification options, and further details and procedures associated with those options are sufficient as far as the scope of this agenda item is concerned.

Proposal 2: Conclude that model transfer/delivery case z4 is deprioritized for UE-sided models.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94e, Dec. 2021

- TR 38.843 v18.0.0, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), Dec. 2023

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #105, Sep. 2024