3GPP TSG-RAN WG1 Meeting #119			R1-2410578

Orlando, US, November 18th – 22nd, 2024

Source:	CEWiT

Title:	Discussion on Frame structure and timing aspects

Agenda Item:	9.4.2.2

Document for:	Discussion and Decision

- Introduction

In RAN1 #118-bis meeting [1], different aspects of frame structures and timings related relationships for the device were discussed, and the following agreements were made.

Agreement TP in R1-2409187 is endorsed in principle for section 6.2 and Annex X of TR38.769, subject to possible revisions. - The contents in the Tables are further checked by companies and targeted for completion in RAN1#119. Agreement The start indicator part of the R2D time acquisition signal is not included in TD2R_min. Agreement For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code. Agreement The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options: - Option 1: D2R preamble only - Option 2: D2R preamble + X midamble(s), where X 1 - Option 3: D2R preamble + postamble - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 For the above options, companies are encouraged to report at least the following: - Purpose(s) of the preamble, midamble and postamble - Whether companies assume multiple options can be supported Agreement RAN1 studies following: - A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s). - The study includes - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency. - Size(s) for resource allocation in the time domain - Determination of the X time domain resource(s) by the device - Addressing timing errors for adjacent time domain resources due to residual SFO of the device Agreement Study FDMA and/or TDMA of D2R transmissions for Msg3 from multiple devices in response to a given set of one or multiple Msg2 transmission(s) during access procedure, including following - How the frequency and time domain resources are allocated for the FDMA and/or TDMA of D2R transmissions for Msg3 Agreement From RAN1 perspective, capture in the TR that at least the following aspect of the air interface for DO-DTT and DT traffic types is not sufficient for the DO-A traffic type. - For DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic. Conclusion For the purpose of the Rel-19 study, the study on DO-A in RAN1 is considered completed based on the above agreement. Agreement RAN1 studies the following options for Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access. - Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device - Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices Agreement RAN1 studies the starting time and time duration for Msg2 monitoring for Msg2 reception. Agreement For analysing the trade-offs among the D2R amble(s) options, companies can refer to the Table 3.2.4 in section 3.2.4 of R1-2408993 for information. Agreement For R2D waveform generation using DFT-s-OFDM-based transmitter, from transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used.

In this document, we focus on the synchronization signal, scheduling, timing aspects, aspects related to random access and the impact of device unavailability.

- Discussion

# 2.1 Synchronization signal, scheduling, and timing aspects

In the RAN1 #116 meeting [4], it was agreed to investigate the inclusion of preamble signals for both R2D and D2R transmissions. Preambles serve the crucial role of indicating the commencement of transmission and aiding in timing acquisition. Based on the device type, the periodic synchronization signal can also be studied. The active device is capable of detecting the periodic synchronization signal and can maintain the synchronization with the reader. The predefined pattern can be considered for the periodic synchronization signal by the reader. The details of the synchronization signal can be further investigated.

Proposal 1: Support to study the periodic synchronization signal based on the device type.

- Predefined pattern can be provided by the reader.

For R2D midamble, the following agreement was made in the RAN1# 116-bis meeting [3].

Agreement RAN1 study the R2D transmission without midamble as the baseline if Manchester encoding is used. - FFS the necessity for the R2D transmission with midamble if PIE is used.

The midamble is unnecessary with Manchester line coding, as the inherent transitions within the Manchester code provide synchronization. Similarly, PIE line coding also eliminates the need for a midamble because its power level transitions can be used for synchronization purposes.

Proposal 2: Update the agreement that RAN1 study the R2D transmission without midamble in case of line coding.

To determine or derive the end of PRDCH transmission, two options were agreed for further study in [3]. For option 2, the length of the R2D transmission is indicated in the control information in the R2D signal. Currently in NR, the data length is given in control channel, the reader can inform the A-IoT device the length of the packet. For the cases where data size of the PRDCH is small like query or configured based transmission like initial message for random access procedure, the postamble may not be needed and the length of the data can be indicated by the control information. Thus, Option 2 is preferred.

Proposal 3: Option 2 is preferred for determination of the end of PRDCH.

For D2R amble the following agreement was made in RAN1# 118-bis meeting [1].

Agreement - The TR will capture the following options, and companies are encouraged to analyze the tradeoffs among the following D2R amble(s) options: - Option 1: D2R preamble only - Option 2: D2R preamble + X midamble(s), where X 1 - Option 3: D2R preamble + postamble - Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 - For the above options, companies are encouraged to report at least the following: - Purpose(s) of the preamble, midamble and postamble - Whether companies assume multiple options can be supported

The purposes and functionalities for D2R amble are given below:

Table 1: Analysis on D2R amble options

| Option(s) | Purpose(s) for amble(s) | Assumptions | Aspects to be considered for trade-offs analysis |
| --- | --- | --- | --- |
| Option 1: D2R preamble only | Start indicator, SFO/CFO tracking, Interference estimation and channel estimation | For smaller payload size | Based on the power consumption constraint of the device and its capabilities, the preamble needs to be studied. |
| Option 2: D2R preamble + X midamble(s), where X 1 | Time/frequency/phase tracking, Interference estimation and channel estimation | For larger payload size | Based on the residual SFO value and the midamble purpose, the design of midamble can be studied. |
| Option 3: D2R preamble + postamble | Time/frequency/phase tracking, Interference estimation and channel estimation | When the size of the payload is not known to the reader or for larger payload size | Postamble design needs to be studied based on its purpose for e.g. whether the postamble is used for channel estimation or time/frequency/phase tracking. |
| Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 | Time/frequency/phase tracking, Interference estimation and channel estimation | When the size of the payload is not known to the reader or for larger payload size or larger SFO residual | Designing of the D2R amble can be studied based on device capability, their functionalities and the purposes. |

Proposal 4: Capture the above Table 1 in the TR.

Proposal 5: The further detailed study on the D2R amble options can be considered in the later stage.

In the RAN1 #116 meeting [4], the agreement below was made for the time related aspects for

Agreement

For further discussion, the following terminologies are used for A-IoT for studying processing time aspects:

- TR2D_min: Minimum Time between a R2D transmission and the corresponding D2R transmission following it.

- TD2R_min: Minimum Time between a D2R transmission and the corresponding R2D transmission following it.

- TR2D_R2D_min: Minimum Time between two different consecutive R2D transmissions to the same A-IoT device.

- TD2R_D2R_min: Minimum Time between two different consecutive D2R transmissions from the same A-IoT device.

- The study should consider at least following aspects

  - Implementation restrictions for the existing BS/UE

  - [Processing time is common or different for different A-IoT devices]

  - [Processing time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command)]

FFS other timing aspects

the device.

The processing time for the devices needs to be studied based on the device type. The active devices or passive devices with enhanced capabilities may exhibit shorter processing times for preparing D2R transmissions compared to passive devices with lower capabilities. Moreover, the processing time may vary depending on the device's application, such as inventory management or command-based operations, given the distinct traffic characteristics associated with each use case. The D2R transmissions in inventory management scenarios might involve larger payloads compared to those in command-based applications. Consequently, the processing time for different devices can diverge based on their capabilities, allowing devices proper preparation time for initiating D2R transmissions effectively.

Proposal 6: Different processing times can be considered based on the device type.

Proposal 7: Different processing times can be considered based on the device application.

When reader transmits the R2D to the device, it expects a D2R in response and if it doesn’t get any uplink transmission from the device, reader has to wait for the unknown time period. Therefore, the time interval between the R2D transmission and D2R transmission is needed. If there is no maximum time interval is defined, the waiting period of the reader can become very large which can delay the process and will waste the energy. Therefore, it is necessary to define a maximum time TD2R_max between the end of the D2R transmission and the start of the expected R2D transmission following it.

Proposal 8: Support to define a maximum time TD2R_max between the end of the D2R transmission and the start of the expected R2D transmission following D2R.

# 2.2 Device availability for Tx/Rx procedures

Considering the fact that A-IoT devices are very low power consumption devices with limited energy storage and due to which device can run out of the energy during transmissions. For e.g. the device needs to be charged for a longer period to complete the inventory round but due to the device’s low capability, consumes all energy before completing the inventory round and becomes unavailable due to lack of energy. In result to this, device must wait for the next inventory round.

In RAN1# 118 Meeting [2], following agreement was made related to device unavailability.

Agreement For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

In RAN1#118-bis meeting [1], there is an agreement on TP capturing the study of device availability/unavailability.

Agreement TP in R1-2409187 is endorsed in principle for section 6.2 and Annex X of TR38.769, subject to possible revisions. The contents in the Tables are further checked by companies and targeted for completion in RAN1#119.

In direction 1, the reader neither provides any input to the device nor is notified about when the device becomes available or unavailable. Essentially, the device becomes operational once it is fully charged and will continue to carry out transmission and reception (Tx/Rx) functions until it runs out of the energy. In this case device can have two states i.e. ON and OFF state. The device can perform the Tx/Rx in ON state, but it cannot perform any operation during OFF state. As there is no configuration for monitoring the triggering signal provided from the reader to the device, the device may continuously monitor R2D trigger message.

The following solution for direction 1 can be considered.

- Reduce the time interval between the paging messages based on the device capacitor size and the threshold value of the device.

- The device can report its energy status during the random access.

- Scheduling for R2D/D2R can be based on the reported energy status.

In direction 2, the reader can control the availability/unavailability of the device. In this case the device can have the three states i.e. ON, OFF, and sleep state. The device can perform the Tx/Rx operation during ON state and device can do the energy harvesting during its sleep state. In this direction the reader is aware of the device availability status and based on this reader can transmits the paging messages to the device.

For this direction 2 the following solution can be considered.

- Design the duty cycle for Tx and Rx for the device

- Increase the operational time for the device

- The time interval between the paging message can be defined based on the device energy storage.

- Reader can provide the information related to switching between the states by the device.

Proposal 9: Capture the following Table 2 in TR.

Table 2: Details from Source [x, R1-240xxxx] for Direction 1

| Source | Details |
| --- | --- |
| Source [x] | Solution description # The reader does not provide information to a device nor be aware of a device regarding when the device may become available/unavailable. The device can report the energy status to the reader. In this case, the device is assumed to have two states i.e. ON and OFF state. Observations or Analysis or Evaluations - Reduce the time interval between the paging messages based on the device capacitor size and the threshold value of the device - The device can report its energy status during the random access - Scheduling for R2D/D2R can be based on the reported energy status # Specification impacts, if any - Define the paging message transmission mechanism and relevant signaling indication. - Device behavior for e.g. reporting of energy status - Content of R2D/D2R control information |

Proposal 10: Capture the following Table 3 in TR.

Table 3: Details from Source [x, R1-240xxxx] for Direction 2

| Source | Details |
| --- | --- |
| Source [x] | Solution description # The reader provides control information to assist a device to determine whether/when to become available or unavailable. The device is assumed to have three states i.e. ON, OFF, and sleep state. Observations or Analysis or Evaluations - Design the duty cycle for Tx and Rx for the device - Increase the operational time for the device - The time interval between the paging message can be defined based on the device energy storage - Reader can provide the information related to switching between the states by the device Specification impacts, if any - Scheduling can be based on the duty cycle - Designing of the timer of sleep state - Device behavior for monitoring the paging message - Signaling indication for switching the states - Impacts on random access procedure - Potential information feedback on the energy state |

Due to device unavailability, the energy harvesting time should be considered for the defined time intervals for R2D and D2R transmission in [4] to avoid the retransmissions of the packets. Unnecessary retransmission can waste the energy of the device.

Proposal 11: Support to consider the energy harvesting time due to unavailability of device for the following time interval within inventory round

- The time interval between a R2D transmission and the corresponding D2R transmission.

- The time interval between a D2R transmission and the corresponding R2D transmission following it.

- The time interval between two different consecutive R2D transmissions to the same A-IoT device.

- The time interval between two different consecutive D2R transmissions from the same A-IoT device.

# 2.3 Random access

Resource allocation for 2-step and 4-step RACH procedures can be defined using multiple access schemes to support multiple devices during random access. To prevent collisions between devices, their responses can be multiplexed across time, frequency, and code.

RAN1 should explore methods to signal time and frequency resources to the devices. One potential solution is to include this information in the paging message for MSG1 and MSG3 transmissions. The paging message could also specify parameters such as small frequency shifts, time offsets, starting times, and the allocated frequency resources.

Proposal 12: The following points can be considered about the study scope of random access from RAN1 perspective:

- Support TDM/FDM/CDM in random access

- How to indicate the time and frequency resources to single and multiple devices.

Proposal 13: The resources for MSG1 and MSG3 transmission can be indicated in the paging message.

Proposal 14: The resources for MSG1 and MSG3 transmission can be identical or different.

Proposal 15: The resources for MSG3 transmission can be indicated in the MSG2.

# Conclusion

In this document, we focus on the synchronization signal, scheduling, random access related aspects, timing aspects and the impact of device unavailability and the following proposals are made:

Proposal 1: Support to study the periodic synchronization signal based on the device type.

- Predefined pattern can be provided by the reader.

Proposal 2: Update the agreement that RAN1 study the R2D transmission without midamble in case of line coding.

Proposal 3: Option 2 is preferred for determination of the end of PRDCH.

Proposal 4: Capture the following Table 1 in the TR.

Table 1: Analysis on D2R amble options

| Option(s) | Purpose(s) for amble(s) | Assumptions | Aspects to be considered for trade-offs analysis |
| --- | --- | --- | --- |
| Option 1: D2R preamble only | Start indicator, SFO/CFO tracking, Interference estimation and channel estimation | For smaller payload size | Based on the power consumption constraint of the device and its capabilities, the preamble needs to be studied. |
| Option 2: D2R preamble + X midamble(s), where X 1 | Time/frequency/phase tracking, Interference estimation and channel estimation | For larger payload size | Based on the residual SFO value and the midamble purpose, the design of midamble can be studied. |
| Option 3: D2R preamble + postamble | Time/frequency/phase tracking, Interference estimation and channel estimation | When the size of the payload is not known to the reader or for larger payload size | Postamble design needs to be studied based on its purpose for e.g. whether the postamble is used for channel estimation or time/frequency/phase tracking. |
| Option 4: D2R preamble + Y midamble(s) + postamble, where Y1 | Time/frequency/phase tracking, Interference estimation and channel estimation | When the size of the payload is not known to the reader or for larger payload size or larger SFO residual | Designing of the D2R amble can be studied based on device capability, their functionalities and the purposes. |

Proposal 5: The further detailed study on the D2R amble options can be considered in the later stage.

Proposal 6: Different processing times can be considered based on the device type.

Proposal 7: Different processing times can be considered based on the device application.

Proposal 8: Support to define a maximum time TD2R_max between the end of the D2R transmission and the start of the expected R2D transmission following D2R.

Proposal 9: Capture the following Table 2 in TR.

Table 2: Details from Source [x, R1-240xxxx] for Direction 1

| Source | Details |
| --- | --- |
| Source [x] | Solution description # The reader does not provide information to a device nor be aware of a device regarding when the device may become available/unavailable. The device can report the energy status to the reader. In this case, the device is assumed to have two states i.e. ON and OFF state. Observations or Analysis or Evaluations - Reduce the time interval between the paging messages based on the device capacitor size and the threshold value of the device - The device can report its energy status during the random access - Scheduling for R2D/D2R can be based on the reported energy status # Specification impacts, if any - Define the paging message transmission mechanism and relevant signaling indication. - Device behavior for e.g. reporting of energy status - Content of R2D/D2R control information |

Proposal 10: Capture the following Table 3 in TR.

Table 3: Details from Source [x, R1-240xxxx] for Direction 2

| Source | Details |
| --- | --- |
| Source [x] | Solution description # The reader provides control information to assist a device to determine whether/when to become available or unavailable. The device is assumed to have three states i.e. ON, OFF, and sleep state. Observations or Analysis or Evaluations - Design the duty cycle for Tx and Rx for the device - Increase the operational time for the device - The time interval between the paging message can be defined based on the device energy storage - Reader can provide the information related to switching between the states by the device Specification impacts, if any - Scheduling can be based on the duty cycle - Designing of the timer of sleep state - Device behavior for monitoring the paging message - Signaling indication for switching the states - Impacts on random access procedure - Potential information feedback on the energy state |

Proposal 11: Support to consider the energy harvesting time due to unavailability of device for the following time interval within inventory round

- The time interval between a R2D transmission and the corresponding D2R transmission.

- The time interval between a D2R transmission and the corresponding R2D transmission following it.

- The time interval between two different consecutive R2D transmissions to the same A-IoT device.

- The time interval between two different consecutive D2R transmissions from the same A-IoT device.

Proposal 12: The following points can be considered about the study scope of random access from RAN1 perspective:

- Support TDM/FDM/CDM in random access

- How to indicate the time and frequency resources to single and multiple devices.

Proposal 13: The resources for MSG1 and MSG3 transmission can be indicated in the paging message.

Proposal 14: The resources for MSG1 and MSG3 transmission can be identical or different.

Proposal 15: The resources for MSG3 transmission can be indicated in the MSG2.

# Reference

[1]. Chairman Notes, RAN1#118-bis, 14th -18th October 2024, Hefei, China.

[2]. Chairman Notes, RAN1#118, 19th -23rd August 2024, Maastricht, Netherlands.

[3]. Chairman Notes, RAN1#116-bis, 15th April-19th April 2024, Changsha, China.

[4]. Chairman Notes, RAN1#116, 26th Feb-1st Mar 2024, Athens.