3GPP TSG RAN WG1 #119                                                               R1-2410673

Orlando, US, November 18th – 22nd, 2024

Source:	vivo

Title:	Study on consistency issue for CSI prediction

Agenda Item:	9.1.3

Document for:	Discussion and Decision

Introduction

In RAN #105, it was agreed to study the following objective with start from Q1 of 2025. However, the discussion of the study on consistency of training/inference can be carried out first in Q4 of 2024. In the following, we give our view on the issue of consistency of training/inference.

CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

Discussion and evaluation results

The following agreement was made in RAN1#118bis [1] on identifying NW-side additional conditions to ensure training-inference consistency.

Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (1 2 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843 Note: report how to map antenna port to antenna elements Note: report the backbone/structure of AI/ML model used

Among the optional evaluation schemes given above, we consider various TXRU mapping cases which may impact UE assumption for CSI prediction using UE-sided model and evaluate their generalization performance based on the three different antenna layout configurations. Spatially consistent modeling is used. The dataset used for modeling consists of three different antenna layout under the same cell, they are：

- Antenna layout 1: [M N P Mg Ng] = [2 8 2 1 1], each antenna element in the vertical direction and each antenna element in the horizontal direction is mapped to a port;

- Antenna layout 2: [M N P Mg Ng] = [8 8 2 1 1], every 4 antenna elements in the vertical direction and every antenna element in the horizontal direction are mapped to a port.

- Antenna layout 3: [M N P Mg Ng]=[12 8 2 1 1], every 6 antenna elements in the vertical direction and every antenna element in the horizontal direction are mapped to a port.

The evaluation is carried out with user distribution of 20% and 100% outdoor ratios respectively, the speed of outdoor users is 60km/h, the AL/ML model backbone is 2D-FC. Other simulation assumptions are given in Appendix. The simulation results are shown below:

Generalization performance case2 loss for various TXRU virtualization (20% outdoor users)

.

Generalization performance case2 loss for various TXRU virtualization (100% outdoor users)

From the generalization performance case2 simulation results, we can observe that in the TR38.901 UMa channel model with spatial consistency modeling turned on, there is a significant generalization impact of different TXRU virtualization mappings under the same cell. When the user distribution ratio is 8:2, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation ranges from -5.6% to -9.4%; when the user distribution ratio is 100% outdoor, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation ranges from -13.0% to -44.4%. The simulation results show that at least for 60km/h, or when the proportion of high-speed data collected is high, significant performance loss can be caused by the mismatch in terms of TXRU virtualization mapping model between training and the inference data.

at least for 60km/h, or when the proportion of high-speed data collected is high, significant performance loss can be caused by the mismatch in terms of TXRU virtualization mapping model between training and the inference data.

- When the user distribution ratio is 8:2, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation case2 ranges from -5.6% to -9.4% in generalization performance case2;

- When the user distribution ratio is 100% outdoor, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation case2 ranges from -13.0% to -44.4%.

The simulation results for generalization case 3 is shown in Fig.3.

Generalization performance case3 loss for various TXRU virtualization (100% outdoor users)

From the generalization performance case3 simulation results, we can observe that when the user distribution ratio is 100% outdoor, the SGCS loss of different TXRU virtualization mappings in the mixed dataset generalization evaluation ranges from -3.7% to -14.6%. The simulation results of the generalization evaluation Case3 show that even the model trained after mixing the data is difficult to erase the impacts caused by different TXRU virtualization mappings as NW side additional conditions.

The SGCS loss of different TXRU virtualization mappings in the generalization evaluation case3 ranges from -3.7% to -14.6%.

At least TXRU mapping is identified as one NW-side additional condition which will have significant impact on generalization performance.

For a BS in one cell, gNB can choose to turn off some antenna elements for NW energy saving, which means different TXRU mapping approaches will be used from time to time for a BS. This is studied as one important use case in Rel-18 NES, which is called Type 2 shutdown. Therefore for a given cell, multiple NW side additional conditions like TXRU mapping approaches can be used. That is, the management of NW-side additional conditions needs to be considered for functionality based LCM. Given NW-side additional conditions such as TXRU virtualization impacts the performance of CSI prediction, it is not clear in which case the models trained on data collected in different NW-side additional conditions are applicable to the UE, so it is desirable that the data collected in the NW-side additional conditions and the corresponding models have unique identifiers, so that the UE can find the corresponding models for inference through the data identifiers when the AI function is turned on. In order to ensure the consistency between model training and model inference, the introduction of association IDs can be a solution and related content has been fully discussed in BM, so we only need to follow the solution based on associated ID as BM use case to address the consistency issue of training and inference for UE- sided model.

Follow the solution based on associated ID as BM use case to address the consistency issue of training and inference for UE-side model.

Conclusions

We have the following observations for this meeting:

- at least for 60km/h, or when the proportion of high-speed data collected is high, significant performance loss can be caused by the mismatch in terms of TXRU virtualization mapping model between training and the inference data.

- When the user distribution ratio is 8:2, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation case2 ranges from -5.6% to -9.4% in generalization performance case2;

- When the user distribution ratio is 100% outdoor, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation case2 ranges from -13.0% to -44.4%.

The SGCS loss of different TXRU virtualization mappings in the generalization evaluation case3 ranges from -3.7% to -14.6%.

We have the following proposals for this meeting:

- At least TXRU mapping is identified as one NW-side additional condition which will have significant impact on generalization performance.

- Follow the solution based on associated ID as BM use case to address the consistency issue of training and inference for UE-side model.

References

[1] Chairman’s notes, RAN1#118bis

Appendix A: Dataset parameter configuration

| Parameter | Value |
| --- | --- |
| Scenario | UMa |
| Channel model | Uma 38.901 |
| Carrier frequency | 2GHz |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15KHz |
| Data type | raw channel |
| Time domain part | 200 consecutive slots |
| Antenna configuration for Cell | [Mg Ng M N P; Mp Np] = [1 1 2 8 2; 2 8]; [Mg Ng M N P; Mp Np] = [1 1 8 8 2; 2 8]; [Mg Ng M N P; Mp Np] = [1 1 12 8 2; 2 8] |
| Antenna configuration for UE | [Mg Ng M N P; Mp Np] = [1 1 2 1 1; 1 1] |
| User distribution | 20% outdoor; 100% outdoor |
| Site number | 1 |
| Sector number | 1 |
| User number per cell | 1000 |
| Drop number | 50 |
| Observation windows | 5/5ms |
| Prediction windows | 1/1/1ms |