3GPP TSG RAN WG1 Meeting #119	R1-2410636

Orlando, Florida, USA, November 18th – 22nd, 2024

Source: 	Moderator (Intel Corporation)

Title:	Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation

Agenda item:	9.8.1

Document for:	Discussion

# Introduction

In this contribution, moderator summarizes issues identified by the submitted maintanence contributions for RAN1 #118 agenda 9.8.1 regarding validation of channel models for 7 – 24 GHz. Based on the workplan presented in RAN1 #116-bis, R1-2402128, RAN1 should target the following for this meeting.

RAN1 #119 - Objective #1 & #2:

- Continuation of collection of measurement/simulated data and finalize identification of channel modeling changes required.

- For the identified aspects and issues, continue study of details of required channel modeling updates.

- Initial discussion for evaluation assumption for channel model calibration (if performed)

- Initial drafting of CR to TR38.901

RAN1 #120 - Objective #1 & #2:

- Converge on required changes to channel modeling and modeling updates.

- Identification of evaluation assumption for channel model calibration (if performed)

- Update draft CR to TR38.901.

# Suggested proposals for agreement/conclusion

No further suggestion for proposal. Final moderator summary for RAN1 #119.

# Status summary of Proposal/TPs

| Proposal/TP | Status | Moderator Notes |
| --- | --- | --- |
| #1-1 | Agreed on Thu session |  |
| #1-2A | Modified proposal agreed on Tue session |  |
| #1-3C | Agreed on Thu session |  |
| #1-4 | Not pursue |  |
| #2.1-1C | Modified proposal agreed on Thu session |  |
| #2.2-1A | Modified version agreed on Wed session |  |
| #2.2-2A | Not pursue in this meeting | Not pursued in this meeting with the assumption company who wanted to no conclude in this meeting can provide reasonable explanation. |
| #2.2-3A | Modified proposal agreed on Thu session |  |
| #2.3-1 | Not pursue in this meeting | Not pursued in this meeting with the assumption company who wanted to no conclude in this meeting can provide reasonable explanation. |
| #2.3-2A | Modified version agreed on Wed session |  |
| #2.3-3A | Modified proposal agreed on Thu session |  |
| #2.4-1A | Agreed on Thu session |  |
| #2.4-2B | Agreed on Thu session |  |
| #2.4-3A | Not pursue in this meeting |  |
| #2.4-4B | Agreed on Thu session |  |
| #2.5-1A | Not pursue in this meeting |  |
| #2.5-2C | Agreed on Thu session |  |
| #2.5-3B | Agreed on Wed session |  |
| #2.5-4 | Not pursue in this meeting |  |
| #2.6-1A | Not pursue in this meeting | #3.1-6B is agreed, skipped. |
| #2.7-1 | Not pursue in this meeting |  |
| #2.8-1A | Agreed on Thu session |  |
| #2.8-2B | Agreed on Thu session |  |
| #2.9-1A | Agreed on Thu session |  |
| #2.10-1A | Agreed as working assumption on Thu session | Propose as working assumption |
| #2.10-2A | Agreed on Thu session |  |
| #2.11-1C | Agreed on Thu session |  |
| #3.1-1 | Agreed on Tue session |  |
| #3.1-2 | Agreed on Tue session |  |
| #3.1-3A | Agreed on Tue session |  |
| #3.1-4B | Agreed on Thu session |  |
| #3.1-5 | Not pursue in this meeting | Suggest postponing until BS height and ISD is settled. |
| #3.1-6B | Agreed on Thu session |  |
| #3.1-7B | Agreed on Thu session |  |
| #3.1-8B | Agreed on Thu session |  |
| #3.2-1B | Agreed on Thu session |  |
| #3.3-1 | Not pursue in this meeting |  |
| #4-1 | Modified proposal agreed on Tue session |  |
| #4-2A | Agreed on Thu session |  |
| #4-3A | Agreed on Thu session |  |
| #4-4 | Not pursue in this meeting |  |
| #4-5C | Modified proposal agreed on Thu session |  |
| #4-6 | Agreed on Thu session |  |

# Summary of issues

## 4.1 General Proposals

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Proposal 3: Capture the updated 6-24 GHz channel parameters in a separate table, together with the parameters that remain unchanged. - When the carrier frequencies of an evaluation all belong to 6-24 GHz range, the new table applies. - When any carrier frequency of an evaluation goes beyond 6-24 GHz range, the original table in current TR 38.901 is adopted. |
| [2] Ericsson | Observation 1 If changes are made to the baseline model there is a risk of confusion about which version of the model was/is used in different studies. Proposal 1 Discuss and decide on how baseline model changes should be most transparently captured in TR 38.901. |
| [5] vivo, BUPT | Proposal 7: RAN1 should replace the existing parameters values for any new channel modelling parameter updating and the frequency continuity can be ensured by the new fitting mathematical model based on the new measurement results and the previous measurement results. |
| [7] Intel | Proposal 1: RAN1 to choose among the following options: - Option 1) For any new channel modeling parameter updates, keep the existing parameters values in the TR and add the updated parameters as alternative modeling parameters for frequencies of interest. - Option 2) For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable) in the for frequencies of interest. For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19. - FFS details on how to capture the parameters. Proposal 2: Define a new scenario SMa, and capture SMa modeling parameters similar to other deployment scenarios, InH, InF, UMi, UMa, RMa. |
| [10] CATT | Proposal 3: Option-1 is supported to implement suburban use case: - Option-1: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments. Proposal 8: For any new channel modeling parameter updates, the following method is used for capturing updates: - Option 1: Keep the existing parameter values in the TR and add the updated parameters as alternative modeling parameters for frequencies of interest. |
| [11] Lenovo | Proposal 3 For the 7-24 GHz carrier frequency case, all scenarios should remain unrestricted to particular ISD values by default. Proposal 4 Unless companies are able to show solid field test results that justify the need of variations of UMa channel model behavior for different ISD values, the feasibility of supporting UMa channel model for < 500 m. is not to be referred to in TR 38.901. |
| [17] AT&T | Observation 14: Coupling loss and geometry SINR distributions are not enough to evaluate the necessity of channel model changes. Observation 15: Variation in the statistics in random variables may not be enough to evaluate the necessity of channel model changes Proposal 3: Companies to agree on essential KPIs that may affect the system design and performance to evaluate the necessity of channel model changes. Proposal 6: For validation of the channel model in TR38.901, companies to provide documentation on assumptions and methodology for deriving the parameters of the channel model. |
| [20] Nokia | Proposal 1: RAN1 to strive keeping the existing parameter values of models in TR 38.901 for the 7-24GHz band unless it is justified that they are not correct, and required modifications cause significant impacts on system-level performance. Observation 3: It is pre-mature to discuss how to capture the model parameters updates in TR 38.901 before it is agreed that these updates are needed and how to address continuity of the parameters in frequency domain. Proposal 2: If necessary, any potential model parameter updates for 7-24GHz can be listed as a single set of parameters in the appendix to the feature lead summary for information. |

#### Summary of Issues

There are two main questions regarding how to capture any updates on channel modeling into the TR. First issue is general updates to the channel modeling for existing scenarios. The second issue is the capturing suburban macrocell scenarios.

For capturing general channel modeling parameter updates to existing scenarios

- Option 1) For any new channel modeling parameter updates, keep the existing parameters values in the TR and add the updated parameters as alternative modeling parameters for frequencies of interest.

  - Supported: Huawei, CATT, Nokia

- Option 2) For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable) in the for frequencies of interest. For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

  - Supported: vivo, BUPT

For capturing general channel modeling parameter updates regarding suburban macrocell scenario

- Option-A: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments.

- Option-B: Define a new scenario, e.g., SMa

Lenovo proposed to enable any ISD value for each of the supported scenarios in the TR.

##### Proposal #1-1:

To check and review the following results and measurement data provided in RAN1 #119 for further discussion in next RAN1 meeting. R1-2410631 contains the list of data sources for the results and measurements provided in RAN1 #119, #118-bis, #118, #117, and #116-bis.

##### Proposal #1-2:

Down-select among the following options on capturing general channel modeling updates to existing parameters.

- Option 1) For any new channel modeling parameter updates, keep the existing parameters values in the TR and add the updated parameters as alternative modeling parameters for frequencies of interest.

  - Example 1)

Table 7.5-6 Part-1: Channel model parameters for UMi-Street Canyon and UMa

| Scenarios |  | UMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -6.955 - 0.0963 log10(fc) | -6.28 - 0.204 log10(fc) | -6.62 |
|  | lgDS | 0.66 | 0.39 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 1.06 + 0.1114 log10(fc) | 1.5 - 0.1144 log10(fc) | 1.25 |
|  | lgASD | 0.28 | 0.28 | 0.42 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 1.81 | 2.08 - 0.27 log10(fc) | 1.76 |
|  | lgASA | 0.20 | 0.11 | 0.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 0.95 | -0.3236 log10(fc) + 1.512 | 1.01 |
|  | lgZSA | 0.16 | 0.16 | 0.43 |
| Number of clusters N |  | 12 | 20 | 12 |
| Number of rays per cluster M |  | 20 | 20 | 20 |

Table 7.5-6A: Alternate Channel model parameters for UMa

| Scenarios |  | UMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -7.1 | -6.9 |  |
|  | lgDS |  |  |  |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD |  | 0.7 |  |
|  | lgASD |  | 0.3 |  |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 2.00 | 2.08 - 0.5 log10(fc) |  |
|  | lgASA | 0.3 | 0.05 |  |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA |  |  |  |
|  | lgZSA |  |  |  |
| Number of clusters N |  | 9 | 11 |  |
| NOTE 1: For values not provided, values from Table 7.5-6 is used. |  |  |  |  |

- Option 2) For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable) in the for frequencies of interest. For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

  - Example 2)

Table 7.5-6 Part-1: Channel model parameters for UMi-Street Canyon and UMa

| Scenarios |  | UMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -6.955 - 0.0963 log10(fc) -7.1* | -6.28 - 0.204 log10(fc) -6.9* | -6.62 |
|  | lgDS | 0.66 | 0.39 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 1.06 + 0.1114 log10(fc) | 1.5 - 0.1144 log10(fc) 0.7* | 1.25 |
|  | lgASD | 0.28 | 0.28 0.3* | 0.42 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 1.81 2.00* | 2.08 - 0.27 log10(fc) 2.08 - 0.5 log10(fc)* | 1.76 |
|  | lgASA | 0.20 0.3* | 0.11 0.5* | 0.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 0.95 | -0.3236 log10(fc) + 1.512 | 1.01 |
|  | lgZSA | 0.16 | 0.16 | 0.43 |
| Number of clusters N |  | 12 9* | 20 11* | 12 |
| Number of rays per cluster M |  | 20 | 20 | 20 |
| NOTE *: These parameters were updated in Release 19 as part of enhancements to 7-24 GHz channel modeling SI. |  |  |  |  |

- FFS: applicability of either option 1 or option 2 for penetration loss

##### Proposal #1-3:

Down-select among the following options on capturing channel modeling updates to suburban macrocell scenario

- Option-A: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments.

  - Example A)

Table 7.2-1: Evaluation parameters for UMi-street canyon and UMa scenarios

| Parameters |  | UMa |
| --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site (Opt 1: ISD = 500m, Opt 2: ISD = 1299m) |
| BS antenna height |  | Opt 1) 25m Opt 2) 35m |
| UT location | Outdoor/indoor | Outdoor and indoor |
|  | LOS/NLOS | LOS and NLOS |
|  | Height | Opt 1) Same as 3D-UMa in TR36.873 Opt 2) 1.5 or 4.5 m for residential buildings,1.5/4.5/7.5/10.5/13.5 m for commercial buildings |
| Indoor UT ratio |  | Opt 1) 80% Opt 2) 80%, of which 50% indoor residential building ground floor, 21% indoor residential building upper floors, and 9% uniformly distributed across commercial building floors |
| UT mobility (horizontal plane only) |  | 3km/h |
| Min. BS - UT distance (2D) |  | 35m |
| UT distribution (horizontal) |  | Uniform |

Table 7.4.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMa | Opt 1) PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out631-18d2D-out1+C'hUT54d2D-out1003exp-d2D-out150,18m<d2D-out where C'(hUT)=0,hUT≤13mhUT-13101.5,13m<hUT≤23m Opt 2) PrLOS=exp-d2D-outexpα∙hBSβ+γhUEδ |

Table 7.5-6 Part-1: Channel model parameters for UMi-Street Canyon and UMa

| Scenarios |  | UMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | Opt 1) -6.955 - 0.0963 log10(fc) Opt 2) -7.1 | Opt 1) -6.28 - 0.204 log10(fc) Opt 2) -6.9 | -6.62 |
|  | lgDS | 0.66 | 0.39 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 1.06 + 0.1114 log10(fc) | Opt 1) 1.5 - 0.1144 log10(fc) Opt 2) 0.7* | 1.25 |
|  | lgASD | 0.28 | Opt 1) 0.28 Opt 2) 0.3 | 0.42 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | Opt 1) 1.81 Opt 2) 2.00 | Opt 1) 2.08 - 0.27 log10(fc) Opt 2) 2.08 - 0.5 log10(fc)* | 1.76 |
|  | lgASA | 0.20 | 0.11 | 0.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 0.95 | -0.3236 log10(fc) + 1.512 | 1.01 |
|  | lgZSA | 0.16 | 0.16 | 0.43 |

- Option-B: Define a new scenario, e.g., SMa

  - Example B)

Table 7.2-1: Evaluation parameters for UMi-street canyon and UMa, and SMa scenarios

| Parameters |  | UMa | SMa |
| --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m) | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 1299m) |
| BS antenna height |  | 25m | 35m |
| UT location | Outdoor/indoor | Outdoor and indoor | Outdoor and indoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Height | Same as 3D-UMa in TR36.873 | 1.5 or 4.5 m for residential buildings,1.5/4.5/7.5/10.5/13.5 m for commercial buildings |
| Indoor UT ratio |  | 80% | 80%, of which 50% indoor residential building ground floor, 21% indoor residential building upper floors, and 9% uniformly distributed across commercial building floors |
| UT mobility (horizontal plane only) |  | 3km/h | 3km/h |
| Min. BS - UT distance (2D) |  | 35m | 35m |
| UT distribution (horizontal) |  | Uniform | Uniform |

Table 7.4.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMa | PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out631-18d2D-out1+C'hUT54d2D-out1003exp-d2D-out150,18m<d2D-out where C'(hUT)=0,hUT≤13mhUT-13101.5,13m<hUT≤23m |
| SMa | PrLOS=exp-d2D-outexpα∙hBSβ+γhUEδ |

Table 7.5-6 Part-1: Channel model parameters for UMi-Street Canyon and UMa, and SMa

| Scenarios |  | UMa |  |  | SMa |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -6.955 - 0.0963 log10(fc) | -6.28 - 0.204 log10(fc) | -6.62 | -7.1 | -6.9 | -6.62 |
|  | lgDS | 0.66 | 0.39 | 0.32 | 0.66 | 0.39 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 1.06 + 0.1114 log10(fc) | 1.5 - 0.1144 log10(fc) | 1.25 | 1.06 + 0.1114 log10(fc) | 0.7 | 1.25 |
|  | lgASD | 0.28 | 0.28 | 0.42 | 0.28 | 0.3 | 0.42 |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 1.8 | 2.08 - 0.27 log10(fc) | 1.76 | 2.00 | 2.08 - 0.5 log10(fc) | 1.76 |
|  | lgASA | 0.20 | 0.11 | 0.16 | 0.20 | 0.11 | 0.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 0.95 | -0.3236 log10(fc) + 1.512 | 1.01 | 0.95 | -0.3236 log10(fc) + 1.512 | 1.01 |
|  | lgZSA | 0.16 | 0.16 | 0.43 | 0.16 | 0.16 | 0.43 |

##### Proposal #1-4:

- Update text in TR 38.901 Section 7.2 Scenarios:

  - The detailed scenario description in this clause can be used for channel model calibration. All scenarios are unrestricted to specific ISD values and represented ISD values are for calibrations purposes.

#### Round #1 Discussion

Please provide comments on any proposal on general aspects of the SI that requires discussion and approval. Moderator will draft the proposal numbers and list them for discussions. Please provide comments for Proposal #1-1, #1-2, #1-3, #1-4.

| Company | Comments |
| --- | --- |
| Apple | Proposal #1-1: Okay Proposal #1-2: We prefer Option 2 to keep a unified channel model for simple simulations Proposal #1-3: We prefer Option B since quite a lot of different parameters/settings between UMa and SMa. Proposal #1-4: Not sure about the intention of this proposal. |
| vivo | To Proposal #1-2, RAN1 should make a down-selection between Option 1 and 2. We prefer to have Option 2, because the parameter value and the frequency point can be guaranteed one by one aligned. Only one issue should be resolved is that how RAN1 captures the new value of updated parameter into the existing modelling and ensure the frequency continuity. As a potential solution, the new measurement results of updated parameter can be captured and combined with the existing measurement results in the measurement pool, and then the overall combined measurement results can be used for the mathematical fitting. |
| Moderator | Just to clarify, it would be ideal to try to down-select options in this meeting. |
| Huawei, HiSilicon | Proposal #1-1: Support. Proposal #1-2: Suggest to add the following option: - Option 3) For any new channel modeling parameter updates, keep the existing parameter tables in the TR, and capture the updated as well as remained 6-24 GHz parameters in separate table(s) - When the carrier frequencies of an evaluation all belong to 6-24 GHz range, the new table(s) apply - When any carrier frequency of an evaluation goes beyond 6-24 GHz range, the existing tables are adopted. Proposal #1-3: Generally fine and can accept Option-A with “Opt 3: ISD < 500m” added. Proposal #1-4: Open to further discuss. |
| Samsung | Proposal #1-2: we prefer to set the separate table (Option 2) Proposal #1-3: we prefer to se the separate table (Option B) with clear description for what is the sub-urban macro scenario. |
| QC | Proposal #1-1: Okay Proposal #1-2: To preserve frequency continuity, we should make it clear that for simulations or comparisons that involve multiple carrier frequencies all parameters are to be chosen from a single table. It seems okay to preserve the existing tables as they have been extensively used for 5G simulations. Proposal #1-3: Strong preference for Option B. This will then merely follow the general classification used in ITU/Winner-II channel models. Proposal #1-4: Okay |
| CATT | Proposal #1-1: Okay Proposal #1-2: We prefer Option 1 Proposal #1-3: We prefer Option A Proposal #1-4: Not sure about the intention of this proposal. |
| Lenovo/ MotM | Proposal #1-1: Fine Proposal #1-2: Prefer to keep the existing table and add a separate table for FR3, similar to what QC has mentioned. Proposal #1-3: In legacy study, the evaluation table were simulation assumptions that were used for calibration between of the channel model between different companies. Whether we support Option A or Option B, are we doing a calibration campaign? If not then the motivation for adding/updating values is not clear. Proposal #1-4: Following on the comment in the previous proposal, TR 38.901 does not limit any scenario to a specific ISD; the ISD values in evaluation tables were used for simulation assumptions for calibration. Prefer to keep it the same way |
| ZTE | Proposal #1-1: Fine. Proposal #1-2: Option 1 is slightly preferred Proposal #1-3: It may depend on how different it is from UMa scenario which is not clear now. |
| AT&T | Proposal #1-1: ok Proposal #1-2: Seems this discussion can be postponed till we have a clearer understanding of what parameters will change in the existing models – Frequency continuity needs to be ensured for both options. And if we go with a separate table, a clear justification of how to use one table versus the other needs to be provided for readability and usability of our work in Rel. 19 Proposal #1-3: prefer option B, add a separate new model for SMa. We do not agree with ISD = 1299m even though it is used as a first example here. Proposal #1-4: do not support. Channel models do need ISD restrictions. Case in point, for UMa for example, if ISD of 1000m or higher is used, the parameters for the channel model start to make little sense. |
| MediaTek | Proposal #1-1: ok Proposal #1-2: Prefer to keep the existing table for frequency continuity. Information about the measurement bands could be needed for a separate table. |
| Ericsson | Proposal #1-1: Ok Proposal #1-2 : Prefer Alt 2, for clarity Proposal #1-3 : Prefer Alt B, large enough difference to existing scenarios Proposal #1-4 : Support |
| Sony | Proposal #1-1: Okay. Proposal #1-2: We prefer Option 1. Proposal #1-3: We prefer Option B. Proposal #1-4: Okay. |

#### Summary of Round #1 Discussion

Summary of inputs on Proposal #1-2.

- Option 1) For any new channel modeling parameter updates, keep the existing parameters values in the TR and add the updated parameters as alternative modeling parameters for frequencies of interest.

  - Main motivation of Option 1

    - Since the measurements that were taken during this SI may not be completely identical to the environments in which measurements were taken as part of the original work for TR38.901, any changes introduced for 7-24GHz may not be directly comparable for below 6GHz or above 24GHz scenarios. If companies wish to perform comparison across below 6GHz, 6-24GHz, and above 24GHz all at the same time, there is chance such comparison may not be possible due to the changes introduced for 6-24GHz. Having a separate set of values allow companies to conduct evaluations using older parameters if needed.

  - Main drawback of Option 1

    - It may not be enough to add an alternative table in the TR. If new table is provided then some explanation of why the new table was added and conditions in which the new table should be applicable is needed. If the new table was added due to more accurate measurements, then the need for the explanation and keeping the old table is unnecessary.

  - Supporting company

    - CATT, ZTE, MediaTek Samsung, Qualcomm,

- Option 2) For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable) in the for frequencies of interest. For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

  - Main motivation of Option 2

    - RAN1 will not introduce change to the channel modeling for the sake of new measurement sets. If changes are introduced, then the changes are introduced for the right reasons and significance of it. In such case, it would be not preferred for companies to perform evaluations for 6-24GHz with older set parameters. They can be referenced using older release of the TR in the worst case.

  - Main drawback of Option 2

    - Lack of ability to refer to older set of values for certain type of simulations (e.g. those that compares across different bands) may be questionable with new parameter replacement.

  - Supporting company

    - Apple, vivo, Lenovo, Ericsson

- Option 3) For any new channel modeling parameter updates, keep the existing parameter tables in the TR, and capture the updated as well as remained 6-24 GHz parameters in separate table(s)

  - When the carrier frequencies of an evaluation all belong to 6-24 GHz range, the new table(s) apply

  - When any carrier frequency of an evaluation goes beyond 6-24 GHz range, the existing tables are adopted.

  - Alternative to Option 1 and 2

  - Supporting company

    - Huawei

- Additional notes on Option 1, 2, and 3:

  - AT&T noted that they prefer not to decide at this meeting and prefer to see how the required changes are before deciding this. Rapporteur notes that strictly this decision may not be needed in this meeting, as there are additional detail that would needed in order for rapporteur to capture necessary changes to the TR. However, having a general guidance to the Rapporteur on the direction of the TR drafting helps the Rapporteur to figure out the missing pieces (from the TR perspective) that requires further work and allows incremental updates to be made as agreements are made in the upcoming meeting, instead of having one gigantic CR at the very end.

Summary of inputs on Proposal #1-2.

Down-select among the following options on capturing channel modeling updates to suburban macrocell scenario

- Option-A: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments.

  - Main motivation of Option A:

  - Supporting company:

    - Huawei, CATT

- Option-B: Define a new scenario, e.g., SMa

  - Main motivation of Option B:

  - Supporting company:

    - Apple, Intel, Samsung, Qualcomm, AT&T, Ericsson

From the feedback and offline checking with Lenovo, moderator think there is no need to discuss proposal #1-4 at this time. Need for any clarification can be done along with the proposal related to introduction of UMa below 500m ISD. Therefore, moderator suggests to not pursue the proposal in this meeting.

Proposal #1-2A and #1-3A are the same content as Proposal #1-2 and #1-3 without the examples.

##### Proposal #1-2A:

RAN1 provides general guidance to the rapporteurs on capturing channel modeling updates to existing parameters as follows:

- Down-select among the following options on capturing general channel modeling updates to existing parameters.

  - Option 1) For any new channel modeling parameter updates, keep the existing parameters values in the TR and add the updated parameters as alternative modeling parameters for frequencies of interest.

  - Option 2) For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable) in the for frequencies of interest. For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

- Note: If deemed important, it should be still possible to discuss how specific channel model parameters need to be updated. The guidance should be understood as general framework of how parameters updates should be made but may not be applicable for all parameters.

- FFS: applicability of the guidance for penetration loss

#### Outcome of Tue Session

Conclusion

For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable). For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

- Note: If deemed important, it should be still possible to discuss how specific channel model parameters need to be updated.

#### Round #2 Discussion

Please provide comments on any proposal on general aspects of the SI that requires discussion and approval. Moderator will draft the proposal numbers and list them for discussions (if any).

Proposal #1-1 will be suggested on Thursday for approval. No need to further comment on this proposal.

Please provide comments for Proposal #1-3.

##### Proposal #1-3A:

RAN1 instructs rapporteur to capture channel modeling updates regarding suburban macrocell scenario as follows:

- Down-select among the following options on capturing channel modeling updates to suburban macrocell scenario

  - Option-A: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments.

  - Option-B: Define a new scenario, e.g., SMa

| Company | Comments |
| --- | --- |
| Nokia | Proposal #1-1: OK. Proposal #1-2: Option 1 is not preferred by us because keeping multiple sets of parameters is confusing when results are compared at different carrier frequences. However, for now, both options are suffering the possibility to provide continuity in frequency domain, i.e., it is not clear how the parameters updates will be applied: to the whole 0.5-100 range or specifically to 7-14. In the letter case, a fixed new value only to 7-24 is not physical and cannot be applied directly in the TR 38.901. If necessary to keep track of parameters specifically, then it can be done not in the CR to TR, but in the appendix to the FL summary. Otherwise, the decision about how to capture new aspects in the TR can be postponed until it is more clear what parameters may need to be updated, what is their impact on system level and how frequency continuity will be ensured. Proposal #1-3: We prefer to define a separate SMA model and not to modify existing scenarios, i.e., Option B. Proposal #1-4: Some clarifications are needed about this proposal: This change does not look to be 7-24GHz channel specific, is it expected to be introduced in the previous releases of the TR as well? In general, isn’t it clear already that the detailed scenario description (including ISD) can be used for channel model calibration, and other values are also allowed? Is there an intention to change the ISD value for calibration? |
| AT&T | For proposal #1-3A: we prefer option B. define a separate SMa model and not modify existing UMa scenario |
| Apple | We support Option B. |

#### Summary of Round #2 Discussion

Based on feedback so far, there seems more companies that prefer option B for capturing suburban macrocell scenarios. I would like to suggest RAN1 to take option B.

##### Proposal #1-3B:

RAN1 instructs rapporteur to capture channel modeling updates regarding suburban macrocell scenario as follows:

- Down-select among the following options on capturing channel modeling updates to suburban macrocell scenario

  - Option-A: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments.

  - Option-B: Define a new scenario, e.g., SMa

#### Round #3 Discussion

| Company | Comments |
| --- | --- |
| Sharp | Support. SMa to be captured as a new scenario. |
| AT&T | Support. |

#### Outcome of Wed Session

##### Proposal #1-3C:

Capture channel modeling updates regarding suburban macrocell scenario as follows:

- Define a new scenario, e.g., SMa

- The key differentiating aspects of SMa (compared to UMa and RMa) in high level description will be highlighted in Section “6.2 Scenario of Interest” of the TR.

#### Round #4 Discussion

| Company | Comments |
| --- | --- |
| AT&T | Support. |

#### Outcome of Thursday Session

Agreement

Capture channel modeling updates regarding suburban macrocell scenario as follows:

- Define a new scenario, e.g., SMa

- The key differentiating aspects of SMa (compared to UMa and RMa) in high level description will be highlighted in Section “6.2 Scenario of Interest” of the TR.

## 4.2 Discussion on Modeling Parameters (Scenarios in TR)

### 4.2.1 Penetration Loss

| Company | Proposals & Observations |
| --- | --- |
| [5] vivo, BUPT | Table 2: The final measurement results of three different materials. Material; Penetration loss [dB]; Empirical result; Experiment result; Gap. Standard multi-pane glass; 3.6dB; 1.5dB; 2.1dB. Concrete; 37dB; 8.6dB; 28.4dB. Wood; 5.81dB; 5dB; 0.81dB. Observation 3: The gap of penetration loss between the measurement and the empirical value for wood is within an acceptable range. Observation 4: The penetration loss from measurement for concrete wall and glass are smaller than the empirical value with a large gap. Proposal 2: RAN1 further validates the O-to-I penetration loss, with different materials in consideration of the thickness and the density. |
| [6] ZTE, Sanechips | Table 2: Material penetration losses Material; Penetration loss [dB]. Concrete; Lconcrete=α(5+4f). Wood; Lwood=α(4.85+0.12f). Standard multi-pane glass; Lglass=α(2+0.2f). IRR glass; LIRRglass=α(23+0.3f). Note: f is in GHz. α=thick/thickref+a/b, thick is in cm. - Concrete: thickref = 23cm, a = 0, b = 1 - Wood: thickref = 6cm, a = 0, b = 1 - Std glass: thickref = 3cm, a = 0.7, b = 1.7 - IRR glass: thickref = TBD, a = TBD, b = TBD. - Penetration loss for concrete (b) Penetration loss for wood (c) Penetration loss for std glass (d) Penetration loss for IRR glass Figure 7: Penetration loss for four materials in ITU-R and TR38.901 with thickness factor Observation 6: The penetration losses obtained by electromagnetic calculation in ITU-R P.2040-3 are closest to the results of current TR 38.901 when the thicknesses of the materials are as follows, respectively. Further, these values of the thicknesses can be considered as the reference values for penetration loss validation. - Concrete: 23cm - Wood: 6cm - Std glass: 3cm - IRR glass: 5/8mm (glass)+15nm (metal-coating)+12A(air)+5/8mm(glass). Observation 7: The penetration loss of IRR glass is not only related to the composed glass on its both sides, but also largely depends on the thickness of its internal metal-coating. Proposal 8: A thickness factor (α) can be introduced into the formula of penetration loss in TR 38.901 to reflect the effect of the material’s thickness on penetration loss, especially if the thickness differs significantly from the reference values. |
| [9] Apple | Observation 1: The measured penetration loss of wood with thickness of 1.75 cm is less than 3.5 dB below TR 38.901 at frequencies of 7.5 GHz to 30 GHz. Figure 2: Penetration loss of wood at frequencies of 7.5 GHz to 30 GHz Observation 2: The measured penetration loss of concrete with thickness of 4.7 cm is between 30 dB and 100 dB below TR 38.901 at frequencies of 7.5 GH to 30 GHz. Figure 3: Penetration loss of concrete at frequencies of 7.5 GHz to 30 GHz Figure 4. Penetration loss of one and two layers concrete of 7.5 GHz to 18 GHz Penetration loss per unit length model as Lconcrete(dB/cm) = 0.043+0.187 f. Observation 3: The measured penetration loss of IRR glass of one layer with thickness of 0.55 cm is between 4 dB and 9 dB above TR 38.901 at frequencies of 7.5 GH to 30 GHz. LIRR glass=30+0.3f. Observation 4: The measured penetration loss of glass with thickness of 0.55 cm is less than 5 dB below TR 38.901 at frequencies of 7.5 GH to 24 GHz. The measured glass penetration loss does not linearly increase with frequency. Figure 6: Penetration loss of glass at frequencies of 7.5 GHz to 30 GHz Proposal 4: RAN1 to consider - express concrete penetration loss in terms of unit thickness (e.g., L(dB/cm) = 0.043+0.187 f) - increasing IRR glass penetration loss from TR 38.901 (e.g., L=30+0.3f) - whether or not to reduce wood penetration loss from TR 38.901 - whether the glass penetration loss is linearly increasing with frequency. Proposal 5: RAN1 to introduce an additional O2I building penetration loss model. |
| [12] Sony | Figure 14. Transmission losses of “standard” glass, from [8][10]. Observation 13. The TR 38.901 model on material penetration loss is valid for the 7-24 GHz range. The TR 38.901 model on window glass penetration loss depends on the window glass layer design. If the thickness of the glass is in the order of multiple half wavelengths, it has resonant behaviors in the 7–24 GHz frequency range. This effect must be considered. |
| [16] Sharp, Nokia | Table 1. Measured Penetration Loss of different Materials using Co-polarized antennas at the TX and RX at 6.75 GHz and 16.95 GHz [2, 3]. Frequency; TR 38.901 Material; Measured Material; Thickness (cm); Penetration Loss (dB); Penetration Loss (dB/cm). 6.75 GHz; Standard multi-Pane Glass; Clear Glass; 1; 3.6; 3.6. IRR Glass; Low-e Tinted Glass Wall; 3; 33.7; 11.23. Low-e Tinted Glass Window; 2; 29.7; 14.85. Concrete; Cinderblock Wall; 22; 13.4; 0.61. Wood; Birch Wood Panel; 2; 2.4; 1.2. Wooden Door; 4.5; 5.8; 1.29. 16.95 GHz; Standard multi-Pane Glass; Clear Glass; 1; 3.7; 3.7. IRR Glass; Low-e Tinted Glass Wall; 3; 42.3; 14.1. Low-e Tinted Glass Window; 2; 32.7; 16.35. Concrete; Cinderblock Wall; 22; 15; 0.68. Wood; Birch Wood Panel; 2; 6.1; 3.05. Wooden Door; 4.5; 6.1; 1.36. Observation 1: RAN1 should consider the penetration loss per unit centimeter (cm) of thickness for the various materials listed in Table 1 when employing co-polarized antennas at both the TX and RX at 6.75 GHz and 16.95 GHz. Note that the measured materials are of fixed thickness and the penetration loss per unit centimeter (cm) in Table 1 is for a fixed thickness. Proposal 1: RAN 1 should gather data from various sources on common materials of varying thicknesses to determine the average penetration loss in dB per cm and the penetration loss for specific thicknesses. Observation 2: The difference in measured penetration loss for concrete and those estimated from TR 38.901 is primarily due to the difference in the composition of concrete. Measurements in [1] mostly measure concrete materials which have similar composition as that of a cinderblock wall, while TR 38.901 models the penetration loss for concrete considering reinforced concrete (heavy/slight) walls. Thus, all sources [1] reporting extremely high deviation in penetration loss from the penetration loss specified in TR 38.901 for concrete, actually measured concrete walls which more closely matches cinderblock walls rather than reinforced concrete walls. Proposal 2: RAN1 should consider modeling penetration loss of cinderblock walls, commonly found as indoor partitions, external walls in houses, low-rise residential and commercial buildings based on measurement data provided by different sources [1]. Proposal 3: No updates are required for the existing material penetration loss model for concrete and O2I penetration loss model in TR 38.901. |
| [19] Qualcomm | Figure 23 Glass penetration loss measurements at 13 GHz with comparative measurements at 3.4 GHz. Observation 6: Standard Glass penetration losses at 13 GHz are in line with the expected losses from the penetration loss model in TR 38.901. For IRR glass, the measurements at multiple locations with IRR glass showed smaller losses at 13 GHz than that predicted by the model. At 3.4 GHz, IRR glass loss measurements align with that of the model. Proposal 16: Further study penetration losses incurred due to IRR glass in FR3. Figure 25 Drywall penetration loss measurements at 13 GHz with comparative measurements at 3.4 GHz. Observation 7: Average drywall/wood penetration losses at 13 GHz are in line with the expected losses from the penetration loss model in TR 38.901. |

#### Summary of Issues

The following is a summary of results provided by companies on material penetration loss.

|  | Standard multi-pane glass | IRR glass | Concrete | Wood |
| --- | --- | --- | --- | --- |
| TR38.901 Penetration loss [dB] | Lglass=2+0.2f | LIRRglass=23+0.3f | Lconcrete=5+4f | Lwood=4.85+0.12f |
| TR38.901 result | 4 dB @ 10 GHz 5 dB @ 15 GHz 7 dB @ 25 GHz | 26 dB @ 10 GHz 27.5 dB @ 15 GHz 30.5 dB @ 25 GHz | 45 dB @ 10 GHz 65 dB @ 15 GHz 105 dB @ 25 GHz | 6. dB @ 10 GHz 6.7 dB @ 15 GHz 7.85 dB @ 25 GHz |
| ZTE proposal (variable T is thickness of material in cm) | Lglass=α(2+0.2f) α=Tglass3cm+0.7/1.7 | LIRRglass=α(23+0.3f) α=TBD ZTE estimate: 5/8mm (glass)+15nm (metal-coating)+12A(air)+5/8mm(glass). | Lconcrete=α(5+4f) α=Tconcrete23cm | Lwood=α(4.85+0.12f) α=Twood6cm |
| ZTE estimates: | 0.55 cm thickness 2.1 dB @ 10 GHz 2.6 dB @ 15 GHz 3.6 dB @ 25 GHz 1 cm thickness 2.4 dB @ 10 GHz 3 dB @ 15 GHz 4.3 dB @ 25 GHz |  | 4.7 cm thickness 9.2 dB @ 10 GHz 13.3 dB @ 15 GHz 21.5 dB @ 25 GHz 9.4cm thickness 18.4 dB @ 10 GHz 26.6 dB @ 15 GHz 42.9 dB @ 25 GHz 22cm thickness 43 dB @ 10 GHz 62 dB @ 15 GHz 100 dB @ 25 GHz | 1.7 cm thickness 1.6 dB @ 8 GHz 1.7 dB @ 10 GHz 1.9 dB @ 15 GHz 2.2 dB @ 25 GHz 4.5cm thickness 2.4 dB @ 10 GHz 3 dB @ 15 GHz 4.3 dB @ 25 GHz |
| vivo, BUPT Thickness not provided | 1.5dB @ 8 GHz |  | 8.6dB @ 8 GHz | 5dB @ 8 GHz |
| Apple (approx.) | Resonating behavior observed |  | 4.7 cm thickness 10 dB @ 10GHz 12 dB @ 15 GHz 22 dB @ 25 GHz 9.4 cm thickness 15 dB @ 10GHz 25 dB @ 15 GHz | 1.7 cm thickness 2.5 dB @ 8 GHz 2.5 dB @ 10 GHz 3.5 dB @ 15 GHz 5.2 dB @ 25 GHz |
| Sony | Resonating behavior observed |  |  |  |
| Sharp, Nokia | 1 cm thickness 3.6 dB @ 6.75 GHz 3.7 dB @ 16.95 GHz | 2 cm thickness 29.7 dB @ 6.75 GHz 32.7 dB @ 16.75 GHz | 22 cm thickness 13.4 dB @ 6.75 GHz 15 dB @ 16.95 GHz | 2 cm thickness 2.4 dB @ 6.75 GHz 6.1 dB @ 16.75 GHz 4.5 cm thickness 5.8 dB @ 6.75 GHz 6.1 dB @ 16.75 GHz |
| Qualcomm Thickness not provided | 2.2 ~ 6.6 dB @ 13 GHz | 19.3 ~ 22 dB @ 13 GHz |  | (dry wall) 3.6 ~ 7.1 dB @ 13 GHz |
| Moderator Assessment | For lower frequencies, results from companies does not seem too far off from each other. For higher frequencies, some mis-match is observed. Potentially due to thickness frequency resonation effect observed by two sources. Further discussion needed. | Company results are widely ranging, likely due to multiple material composition of IRR glass. Further discuss needed. | Results from Apple and ZTE’s modification to incorporate thickness seems to match up well. Sharp, Nokia results seem to be quite different from other companies and the TR. Either consider no change, or consider update from ZTE to factor into account concrete thickness | Results from companies does not seem too far off from each other for wide range of values. Either consider no change, or consider update from ZTE to factor into account wood thickness. |

##### Proposal #2.1-1:

Following updates on penetration loss modeling is adopted:

|  | Standard multi-pane glass | IRR glass | Concrete | Wood |
| --- | --- | --- | --- | --- |
| Existing TR | Lglass=2+0.2f | LIRRglass=23+0.3f | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | FFS | FFS | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |

- Tconcrete and Twood represent reference (default) thickness of the material.

- For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively.

- For FFS of standards multi-pane glass and IRR glass, possibility of no change of existing TR is not precluded and considered part of the study.

#### Round #1/2 Discussion

Please provide comments on issues regarding penetration loss. Moderator will update the proposal based on additional inputs from companies.

| Company | Comments |
| --- | --- |
| Apple | We may specify the material penetration loss per cm, at least for concrete. |
| Moderator | We will come back to this proposal. Meanwhile, if companies have specific proposals they think we can discuss regarding how to resolve the penetration loss validation, please feel to provide them here. Meanwhile, moderator would like to get inputs on what companies think about ZTE proposal. I’ve formulated this as Proposal #2.1-1. Let’s continue to use this sub-section for Round 2 discussion. |
| Nokia | If penetration loss is defined per cm, we need to consider that penetration loss will not be just x2 if the thickness is x2. In addition to the impact of the thickness, there are also losses due to penetration from one medium to another (i.e., from air to concrete and back). This is already considered by the 3GPP model when the thickness is fixed. |

#### Summary of Round #1/2 Discussion

It seems companies need more discussion on the penetration loss issues. Moderator suggest further discussion based on minor update of Proposal #2.1-1A.

##### Proposal #2.1-1A:

Following updates on penetration loss modeling is adopted considered for further study:

|  | Standard multi-pane glass | IRR glass | Concrete | Wood |
| --- | --- | --- | --- | --- |
| Existing TR | Lglass=2+0.2f | LIRRglass=23+0.3f | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | FFS | FFS | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |

- Tconcrete and Twood represent reference (default) thickness of the material.

- For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively.

- For FFS of standards multi-pane glass and IRR glass, possibility of no change of existing TR is not precluded and considered part of the study.

- For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

#### Round #3 Discussion

Please provide further comments on Proposal #2.1-1A as well as moderator summary of the original summary of issues.

| Company | Comments |
| --- | --- |
| Sharp | For standard multi-pane glass, wood no need to introduce thickness as the results are pretty well aligned among all companies among a wide range of frequencies. For IRR glass we need further discussion, and for concrete we support introduction of material thickness as a starting point. |
| Ericsson | We think the discussion is focused on the wrong topic. The material properties are just one component in the O2I loss model, and can in some cases even be considered a tuning parameter with little correspondence to physical reality. The important part is the total building penetration loss in the low-loss and high-loss model. The building penetration loss should be validated using measurements of outdoor to indoor loss for real buildings, not measurements of single isolated materials that may or may not be representative of the real buildings. Any proposed changes in the modeling should be accompanied by a clear motivation that the building penetration loss in the existing model does not match measurements of outdoor to indoor loss in real buildings. Keeping in mind that a substantial amount of building penetration loss measurements including in the 7-24 GHz range were performed when this model was developed. |
| Nokia | We still cannot support the new updates for penetration loss for concrete and wood because as we explained in our previous comment it is not sufficient to define the penetration loss per cm and then scale for other wall thicknesses. It is necessary to consider that penetration loss will not be just x2 if the thickness is x2. This is because in addition to the impact of the wall thickness, there are also losses due to penetration from one medium to another (i.e., from air to concrete and back). This is already considered by the 3GPP model when the thickness is fixed. If other typical thicknesses need to be defined (i.e., typical for SMA) then a proper penetration loss should be introduced, i.e., not just by scaling the existing penetration loss. |

#### Summary of Round #3 Discussion

There were comments that updates to the material penetration loss might not fully showcase the impact of penetation loss to the total building loss.

##### Proposal #2.1-1B:

Further study penetration loss modeling, including the following aspects:

- Need for change and details of the updates to the material penetration loss, for example, if any

|  | Concrete | Wood |
| --- | --- | --- |
| Existing TR | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |
| Tconcrete and Twood represent reference (default) thickness of the material. For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively. |  |  |

  - For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

  - FFS the impact of updates to material penetration loss to total build penetration loss

- Need for change and details of total building penetration loss, e.g. low-loss and high-loss model, if any

##### Proposal #2.1-1C:

Further study penetration loss modeling, including the following aspects:

- Need for change and details of the updates to the material penetration loss, for example, if any

|  | Concrete | Wood |
| --- | --- | --- |
| Existing TR | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |
| Tconcrete and Twood represent reference (default) thickness of the material. For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively. |  |  |

  - For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

  - FFS the impact of updates to material penetration loss to total build penetration loss

- Need for change and details of total building penetration loss, e.g. low-loss and high-loss model, if any

- Impact of material composition on material penetration loss

#### Round #4 Discussion

| Company | Comments |
| --- | --- |
| Sharp | We do not think that we need to introduce reference thickness for concrete and wood, or any other material. We agree with Ericsson’s and Nokia comments made earlier and based on our measurements the current material penetration loss model for all 4 materials in 38.901 is valid for 7-24 GHz for all materials including concrete and wood. For instance, the concrete material we measured was ~25 cm in thickness which is very close to the reference value of 23 cm, yet we saw a huge difference in penetration loss. This difference in penetration loss was not due to the thickness, but the composition of the material measured being different from what is currently there in 38.901 for concrete. Similarly for wood we measured approximately ~2/3 cm and the penetration loss were very well aligned with TR 38.901. Thus, our concern is even when we introduce material thickness the results can be significantly different and the primary reason for that is the composition/type of material being measured as pointed out by Ericsson. Thus, we are not clear how introducing thickness for wood and concrete will help going forward in the discussion as our measurements show that for the reference thickness specified for concrete and wood the results can still be quite different. Suggest adding an FFS: Impact of material composition on material penetration loss. |

#### Outcome of Thursday Session

Agreement

Further study penetration loss modeling, including the following aspects:

- Need for change and details of the updates to the material penetration loss, for example, if any

|  | Concrete | Wood |
| --- | --- | --- |
| Existing TR | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |
| Tconcrete and Twood represent reference (default) thickness of the material. For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively. |  |  |

  - Whether penetration loss can be scaled linearly with thickness for each material

  - For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

  - FFS the impact of updates to material penetration loss to total build penetration loss

- Need for change and details of total building penetration loss, e.g. low-loss and high-loss model, if any

- Impact of material composition on material penetration loss

### 4.2.2 Pathloss

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Figure 11 NLOS pathloss for UMa scenario Proposal 2: Consider Equation (1) if pathloss and shadowing fading models are decided to be updated. PL=13.5+20log10fc+38.6log10(d3D, σSF=6.9 dB (1) |
| [2] Ericsson | Proposal 9 The NLOS path loss in the UMa model is considered to be validated by new measurements at 0.8 GHz, 2 GHz, 5 GHz, 10 GHz, 22 GHz, and 37 GHz. Proposal 10 Consider adding to the above observation that there are multi frequency measurements suggesting no change have to be made to the UMa NLOS pathloss. |
| [4] Samsung | (a); (b). Figure 2. Pathloss: (a) 6.5 GHz and (b) 13.5 GHz Observation 1 The marginal difference compared to pathloss for UMi scenario in existing channel model is confirmed Proposal 1 RAN1 discuss whether the updates of pathloss in UMi scenario is needed Figure 6. LOS UMa pathloss considering breakpoint Observation 8 Upon plotting the pathloss across distance per frequency bands utilizing these equations, it was observed that beyond a particular distance (i.e., breakpoint), the values tend to converge Proposal 6 RAN1 discuss whether/how to handle the phenomena that converges regardless of the frequency band after the breakpoint in LOS pathloss equation |
| [5] vivo, BUPT | Observation 2: The pathloss gap between the measurement and the empirical formula is within the max range of 15dB under the NLOS conditions in indoor scenario, that can be considered under the agreeable level in between. Proposal 1: The pathloss model at least in indoor scenario is not modified for 7-24GHz. |
| [7] Intel | Proposal 3: - Confirm that no pathloss updates are needed for InH-Office LOS/NLOS, UMi Street Canyon LOS/NLOS, InF LOS/NLOS, RMa LOS/NLOS. - If updates to UMa LOS and NLOS is needed, consider the update as an alternative parameter set for UMa LOS and NLOS (i.e. keep the existing parameters in TR). |
| [9] Apple | Proposal 9: The pathloss updates for UMa LOS/NLOS scenarios are not needed. |
| [13] Rohde & Schwarz | The measurements were performed in a large open space or “atrium” of an office building Figure 10: Estimated and modelled path loss for 14 GHz Parameter; 14 GHz. -; min; max. PL exp. (n) w/o unit; 1.72. RMS DS in ns; -; 2.61; 41.74. RMS AS in degree; -; 7.00; 100.76. K-factor in dB; -; 0.08; 14.41. number of paths; -; 7; 250. Table 3: Estimated Channel Parameters |
| [16] Sharp, Nokia | Table 3. Comparison of Large-Scale Shadow Fading Values: Measured [8, 9] vs. TR 38.901 for InH-Office Scenario in both LOS and NLOS Channel Conditions at 6.75, 16.95, 28, and 73 GHz using Single Frequency FI Path Loss Model and across 7-24 GHz and 0.5-100 GHz bands using ABG Path Loss Model [8, 9]. Frequency; Env.; Measured shadow fading; TR 38.901 shadow fading; Absolute difference in shadow fading (Measured shadow fading - TR 38.901) using FI/ABG path loss model. 6.75 GHz; LOS; 3.4; 3; 0.4. NLOS; 9; 8.03/8.29; 0.97/0.71. 16.95 GHz; LOS; 2.4; 3; 0.6. NLOS; 8.1; 8.03/8.29; 0.07/0.19. 28 GHz; LOS; 1.8; 3; 1.2. NLOS; 9.2; 8.03/8.29; 1.17/0.91. 73 GHz; LOS; 1.4; 3; 1.6. NLOS; 11.2; 8.03/8.29; 3.17/2.91. 7-24 GHz; LOS; 2.9; 3; 0.1. NLOS; 8.6; 8.03/8.29; 0.57/0.31. 0.5-100 GHz; LOS; 2.7; 3; 0.3. NLOS; 10.1; 8.03/8.29; 2.07/1.81. Observation 4: Based on measurements conducted at 6.75, 16.95, 28, and 73 GHz, the shadow fading values, are frequency-independent, and mostly align with those specified in TR 38.901 for InH-Office scenario. The discrepancies in shadow fading values are less than 0.3 dB for LOS and less than 2 dB for NLOS channel conditions across the entire frequency range of 0.5-100 GHz. The small discrepancies in shadow fading values can be considered negligible. Proposal 4: Shadow fading should remain frequency-independent, as defined in TR 38.901. Hence, there is no need to update shadow fading values in TR 38.901 for 7-24 GHz specifically. Table 4. Comparison of CI [13] path loss model parameters, measurement-based [13], and 3GPP TR 38.901 [6] UMi – Street Canyon path loss model in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. Frequency; Env.; Measured; TR 38.901. PLE; SF (dB); PLE; SF (dB). 6.75 GHz; LOS; 1.79; 2.56; 2.2; 4.6. NLOS; 2.56; 6.52; 3.2; 8.2. 16.95 GHz; LOS; 1.84; 4.03; 2.1; 4. NLOS; 2.59; 8.76; 3.2; 8.2. Fig. 2. CI path loss model fit in both LOS and NLOS channel conditions for measured data and TR 38.901 path loss model for the UMi scenario over a distance range of 1 m to 1000 m at 6.75 GHz. The height of the TX used for measured data and TR 38.901 are 4 m and 10 m, respectively. The height of the RX is 1.5 m for both measured data and TR 38.901. Fig. 3. CI path loss model fit in both LOS and NLOS channel conditions for measured data and TR 38.901 path loss model for the UMi scenario over a distance range of 1 m to 1000 m at 16.95 GHz. The height of the TX used for measured data and TR 38.901 are 4 m and 10 m, respectively. The height of the RX is 1.5 m for both measured data and TR 38.901. Observation 5: The path loss exponent (PLE) from the CI path loss model in LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz, as shown in Fig. 2 and Fig. 3, is similar to that obtained from TR 38.901 Table 7.4.1-1 for UMi - Street Canyon scenario [6]. However, the measurements show a slightly lower PLE compared to TR 38.901 because most of the measurements were conducted in an UMi open square like scenario rather than a typical UMi street canyon scenario, resulting in a slightly higher received power. Proposal 5: No updates are required for the existing path loss model for UMi - Street Canyon scenario in TR 38.901 based on measurements conducted at 6.75 GHz and 16.95 GHz in both LOS and NLOS channel conditions in a UMi scenario. Table 8. Comparison of CI [16] path loss model parameters, measurement-based [16], and 3GPP TR 38.901 [6] InF-SH path loss model in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. Frequency; Env.; Measured; TR 38.901. PLE; SF (dB); PLE; SF (dB). 6.75 GHz; LOS; 1.39; 1.88; 2; 3.9. NLOS; 1.78; 2.46; 2.4; 4.4. 16.95 GHz; LOS; 1.75; 3.13; 2; 4. NLOS; 2.11; 3.29; 2.4; 4.5. Fig. 5. CI path loss model fit in both LOS and NLOS channel conditions for measured data and TR 38.901 path loss model for the InF scenario over a distance range of 1 m to 40 m at 6.75 GHz. The height of the TX and RX used for measured data and TR 38.901 are 2.4 m and 1.5 m. Fig. 6. CI path loss model fit in both LOS and NLOS channel conditions for measured data and TR 38.901 path loss model for the InF scenario over a distance range of 1 m to 40 m at 16.95 GHz. The height of the TX and RX used for measured data and TR 38.901 are 2.4 m and 1.5 m. Observation 9: The path loss exponent (PLE) from the CI path loss model in LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz, as shown in Fig. 5 and Fig. 6, is similar to that obtained from TR 38.901 Table 7.4.1-1 for InF-SH scenario [6]. However, the measurements show a slightly lower PLE compared to TR 38.901 because most of the measurements were conducted in an InF-SH like have large open spaces with few big machines and extremely sparse clutter scenario rather than a typical InF-SH scenario defined in TR 38.901 Table 7.2-4 [6], resulting in a slightly higher received power. Proposal 7: No updates are required for the existing path loss model for the InF-SH scenario in TR 38.901 based on measurements conducted at 6.75 GHz and 16.95 GHz in both LOS and NLOS channel conditions in a InF-SH like scenario. |
| [17] AT&T | Observations (for InH): The 1-m intercept point, PL0: The intercept point depends on the materials blocking the signal within 1m of Tx-Rx separation and the building structure. The measured values of PL0 for NLOS were very close to that of LOS path loss plus a few dB more or less depending on the first blocking structure. The LOS and NLOS were very close to that of 3GPP model. See Table 4 for actual numbers comparison. Figure 14: The scatter plot of the measured PL in LOS environments at 7, 8 and 15GHz. Figure 15. The scatter plot of the measured PL in NLOS environments over 554 locations at 15 GHz. Observation 9: Measurements conducted at 7, 8, and 15 GHz over 650 RX locations on multiple floors of an office building show that the PLE for LoS and NLoS environments agree with the previously proposed 3GPP SCM InH channel model. Proposal 3: There is no need to change the PL expression and the PLE for the 3GPP InH channel model in TR 38.901 for extension to 7-24GHz. |
| [19] Qualcomm | Figure 18 Pathloss measurements from a transmitter mounted at a height of 26 meters. Measurements were made at 13 GHz. Observation 4: Pathloss measurements at 13GHz in a Rural Macro setting are in line with existing pathloss models in TR 38.901. There does not appear to be a need to update the Rural Macro pathloss models currently available in TR 38.901. Figure 19 Pathloss comparison between FR1 and FR3. A 12 dB difference in pathloss is observed between FR1 and FR3 --- in line with theoretical expectations. Observation 5: Pathloss comparison between measurements at 13GHz and 3.4 GHz are in line with expectations. A 12 dB difference in pathloss is observed between these frequency bands. Proposal 14: RAN1 to consider extending the RMa pathloss models to 7-24 GHz frequency range. Proposal 15: Generalize the pathloss models for UMa in TR 38.901 to accommodate different base station heights. Pathloss model in TR 36.873 can be used as a starting point. |

#### Summary of Issues

Based on the results provided by companies, moderator suggest discussions on the following proposals.

##### Proposal #2.2-1:

- Based on measurement data provided, RAN1 concludes that following pathloss models for 6 – 24 GHz frequency range are  validated and no updates to TR are made.

  - InH-Office LOS

  - InH-Office NLOS

  - UMi LOS

  - UMi NLOS

  - RMa LOS

  - RMa NLOS

  - InF LOS

  - InF NLOS

##### Proposal #2.2-2:

- For the following scenarios, while sources have observed some differences in pathloss model in the TR and measurement taken for 6 – 24 GHz frequency range, RAN1 concludes that there is no census to update pathloss models due to lack of consistent and significant observed difference between model and measurements.

  - UMa LOS

  - UMa NLOS

#### Round #1 Discussion

Please provide comments on issues regarding path loss. Please provide comments on Proposal #2.2-1 and #2.2-2.

| Company | Comments |
| --- | --- |
| Apple | Fine with Proposal #2.2-1 and #2.2-2. |
| vivo | ##### Support |
| Huawei, HiSilicon | ##### Proposal #2.2-1: Fine. ##### Proposal #2.2-2: Can live with. |
| Samsung | ##### Support |
| LGE | ##### Support |
| CATT | ##### OK |
| ZTE | ##### Support both proposals. |
| AT&T | ##### Proposal # 2.2-1: support for InH . For UMa, and UMi, we will be able to provide measurement results in February meeting for further confirmation. |
| MediaTek | ##### Support |
| Ericsson | ##### #2.2-1: Ok #2.2-2: Suggest reformulating the main bullet: ##### For the following scenarios, while some sources have observed some differences in pathloss model in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed pathloss consistent with the model in the TR. RAN1 therefore concludes that there is no consensus census to update pathloss models due to lack of consistent and significant observed difference between model and measurements. |
| Sony | ##### Okay with both proposals. |
| AT&T - 2 | ##### Proposal #2.2-2: we will be able to provide measurement results in February on UMa scenarios to make this conclusion |

#### Summary of Round #1 Discussion

While most companies seem to be ok with the proposals, AT&T seems to wish to check Proposal #2.2-2  and part of the Proposal #2.2-1 in the next meeting. For now, I have removed UMi from Proposal #2.2-1A. We can discuss further on whether to take Proposal #2.2-2 in this meeting or not.

##### Proposal #2.2-1A:

- Based on measurement data provided, RAN1 concludes that following pathloss models for 6 – 24 GHz frequency range are  validated and no updates to TR are made.

  - InH-Office LOS

  - InH-Office NLOS

  - UMi LOS

  - UMi NLOS

  - RMa LOS

  - RMa NLOS

  - InF LOS

  - InF NLOS

##### Proposal #2.2-2A:

- For the following scenarios, while some sources have observed some differences in pathloss model in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed pathloss consistent with the model in the TR. RAN1 therefore concludes that there is no consensus census to update pathloss models due to lack of consistent and significant observed difference between model and measurements.

  - UMa LOS

  - UMa NLOS

##### Proposal #2.2-3:

- For the following scenarios, further study path loss model parameters for 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

Proposal #2.2-3A:

- For the following scenarios, further study path loss model parameters for 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

#### Round #2/3 Discussion

Please further comment on Proposal #2.2-1A. Please provide comments on whether Proposal #2.2-2 should be concluded in this meeting or whether it can wake until future meetings. In case, it can be pushed to future meetings, moderator will add UMa to Proposal #2.2-3.

With the update of Proposal #2.2-3A. Proposal #2.2-2A is assumed to be not pursued in this meeting. However, companies who did not wish to conclude on “No consensus” should provide some reasonable explanation on why the conclusion should not be made and any other relevant information.

| Company | Comments |
| --- | --- |
| Nokia | OK for both proposals. |
| AT&T | ##### Prefer to make the conclusion in the next meeting for UMa and UMi. |
| Apple | ##### OK |
| ZTE | ##### Fine with both proposals. |
| Moderator | ##### @AT&T: It would be good if AT&T can also provide some explanation of why the conclusions should be pushed to next meeting. This would allows companies to review and/or bring additional materials that would aid the discussion. |
| Ericsson | ##### The SID is clear that continuity to lower and higher bands shall be ensured. This requires that validation measurements are performed also outside the 6-24 GHz range. Therefore, we suggest to remove “for 6-24 GHz frequency range” from the proposals. ##### Otherwise we are fine with the proposals. |

#### Outcome of Wed Session

Proposal #2.2-1A was agreed with some modifications.

Conclusion

- Based on measurement data provided, RAN1 concludes pathloss models in TR38.901 for the following scenarios are validated and no updates to TR are made.

  - InH-Office LOS

  - InH-Office NLOS

  - RMa LOS

  - RMa NLOS

  - InF LOS

  - InF NLOS

#### Round #4 Discussion

Proposal #2.2-2A is no longer needed as Proposal #2.2-3A will cover them.

Please check proposal #2.2-3A.

##### Proposal #2.2-3A:

- For the following scenarios, further study path loss model parameters for 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

| Company | Comments |
| --- | --- |
| AT&T | Support |
| Sharp | Support. We are okay with further study - Just a note that our measurements conducted in New York for UMi LOS, NLOS show that the current TR 38.901 path loss model is valid for 7-24 GHz range as well. |

#### Outcome of Thursday Session

Agreement

- For the following scenarios, further study path loss model parameters for at least 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

### 4.2.3 Delay Spread

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Table 1 Fast fading parameters Scenario; InH @10 GHz; UMi @10 GHz; UMa @6.5 GHz; UMa @13 GHz. TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement. LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS. Delay spread (DS) lgDS=log10 (DS/1s); μlgDS; -7.7; -7.46; -7.79; -7.45; -7.39; -7.08; -7.47; -7.37; -7.03; -6.45; -7.32; -7.01; -7.06; -6.51; -7.59; -7.12. [ns]; 20; 34.7; 16.2; 35.5; 40.7; 83.2; 33.9; 42.7; 93.3; 354.8; 47.9; 97.7; 86.6; 311; 25.3; 76.2. σlgDS; 0.18; 0.16; 0.11; 0.07; 0.38; 0.45; 0.28; 0.1; 0.66; 0.39; 0.38; 0.28; 0.66; 0.39; 0.5; 0.42. AOD spread (ASD) lgASD=log10 (ASD/1°); μlgASD; 1.6; 1.62; 1.21; 1.27; 1.16; 1.29; 1.04; 1.14; 1.15; 1.41; 0.82; 1.26; 1.18; 1.37; 1.08; 1.25. [deg]; 39.8; 41.7; 16.2; 18.6; 14.5; 19.5; 11; 13.8; 14.1; 25.7; 6.6; 18.2; 15.3; 23.6; 12.1; 17.9. σlgASD; 0.18; 0.25; 0.18; 0.14; 0.41; 0.44; 0.2; 0.07; 0.28; 0.28; 0.28; 0.27; 0.28; 0.28; 0.21; 0.3. AOA spread (ASA) lgASA=log10 (ASA/1°); μlgASA; 1.58; 1.75; 1.29; 1.5; 1.65; 1.73; 1.19; 1.37; 1.81; 1.86; 1.67; 1.72; 1.81; 1.78; 1.40; 1.66. [deg]; 38; 56.2; 19.5; 31.6; 44.7; 53.7; 15.5; 23.4; 64.6; 72.4; 46.8; 52.5; 64.6; 60.2; 25.3; 45.8. σlgASA; 0.24; 0.18; 0.13; 0.23; 0.29; 0.35; 0.13; 0.08; 0.2; 0.11; 0.19; 0.15; 0.2; 0.11; 0.13; 0.19. ZOD spread (ZSD) lgZSD=log10 (ZSD/1°); μlgZSD; 0.74; 1.08; 0.99; 1.1; 0.11; -0.11; 0.54; 0.61; 0.54; 0.69; 0.7; 0.78; 0.54; 0.69; 0.68; 0.83. [deg]; 5.5; 12; 9.8; 12.6; 1.3; 0.8; 3.5; 4.1; 3.5; 4.9; 5; 6; 3.5; 4.9; 4.8; 6.8. σlgZSD; 0.44; 0.36; 0.13; 0.12; 0.35; 0.35; 0.12; 0.09; 0.4; 0.49; 0.33; 0.34; 0.40; 0.49; 0.31; 0.35. ZOA spread (ZSA) lgZSA=log10 (ZSA/1°); μlgZSA; 1.17; 1.23; N/A; N/A; 0.63; 0.88; N/A; N/A; 0.95; 1.25; 1.08; 1.15; 0.95; 1.15; 0.72; 1.15. [deg]; 14.8; 17; N/A; N/A; 4.3; 7.6; N/A; N/A; 8.9; 17.8; 12; 14.1; 8.9; 14.2; 5.3; 14.18. σlgZSA; 0.22; 0.65; N/A; N/A; 0.3; 0.34; N/A; N/A; 0.16; 0.16; 0.17; 0.12; 0.16; 0.16; 0.06; 0.15. Number of clusters; 15; 19; 10; 11; 12; 19; 5; 7; 12; 20; 13; 17; 12; 20; 11; 15. Observation 1: The sparsity characteristics can be observed at least for 6-13 GHz: - The measured DSs are smaller than that in TR 38.901. - The measured ASDs/ASAs are smaller than that in TR 38.901. - The measured numbers of clusters are smaller than that in TR 38.901. Proposal 1: Support to update the following fast fading parameters for 6-24 GHz considering measurement results in Table 1: - Delay spread (mean, variance) - AoD spread (mean, variance) - AoA spread (mean, variance) - Number of clusters |
| [2] Ericsson | Proposal 6 Consider the delay spread model in the TR 38.901 UMa scenario to be validated at 3.5 GHz. Figure 6 Comparison of RMS delay spread between the measurements and the UMa model. |
| [4] Samsung | (a); (b). Figure 3. Delay spread: (a) 6.5 GHz and (b) 13.5 GHz Observation 2 As shown in below table, in the UMi scenario, the mean delay spread in the LOS environment was found to be lower compared to the existing channel model parameter Delay spread; UMi_LOS (TR38.901) @ 6.5 GHz; Measured DS @ 6.5 GHz; TR38.901 @ 13.5 GHz; Measured DS @ 13.5 GHz. μDS (log_10⁡〖DS/1s〗 ); -7.35; -8.16; -7.41; -8.19. (ns); 44.67; 6.91; 38.13; 6.32. σDS; 0.38; 0.83; 0.38; 0.91. Observation 3 Even in a visually LOS environment between Tx and Rx, the delay spread can vary depending on the surrounding environment. In particular, if an update of delay spread in the LOS environment is required, it will be necessary to collect and analyze sufficient measurement data from various LOS environment to reflect actual conditions Proposal 2 RAN1 discuss whether the updates of delay spread in UMi LOS scenario is needed |
| [6] ZTE, Sanechips | (a) Source-1[11] (b) Source-2[12] (c)Source-3[13] Figure 8. Fitting distributions based on mixed data sets, comprising 50% measurement data and 50% TR 38.901 data. Observation 8: The fitted curve based on mixed data set by combining new measurement results and existing data set can match the large scale parameters in TR 38.901. Proposal 9: Further study whether parameter needs updates based on mixed data set by combining new measurement results and existing data set. |
| [7] Intel | Proposal 4: Update the delay spread of UMa NLOS scenarios for 7 to 24 GHz based on measurements provided in R1-2407251. - FFS how to handle the frequency continuity beyond the 7 to 24 GHz |
| [9] Apple | Observation 5: For Indoor office scenario at 13 GHz, the measured delay spread in logarithm has mean of -7.26 and standard deviation of 0.11 for LOS case and has mean of -7.19 and standard deviation of 0.12 for NLOS case. Table 2: Comparison of delay spread for Indoor office at 13 GHz Measurement (LOS/NLOS); Indoor office in TR 38.901 (LOS/NLOS). Delay spread mean; -7.26 / -7.19; -7.7 / -7.5. Delay spread standards deviation; 0.11 / 0.12; 0.18 / 0.17. Figure 8: Distributions of delay spread in indoor office scenario at 13 GHz Table 3: Comparison of delay spread for UMa NLOS at 8 GHz Ray tracing simulations; UMa NLOS in TR 38.901. Delay spread mean; -7; -6.46. Delay spread standards deviation; 0.5; 0.39. Proposal 6: RAN1 to consider reducing the delay spread mean value for UMa NLOS scenario in FR3. Table 4: Comparison of delay spread for UMi NLOS at 8 GHz Ray tracing simulations; UMi NLOS in TR 38.901. Delay spread mean; -7.05; -7.06. Delay spread standards deviation; 1; 0.4. Proposal 7: The delay spread updates for UMi NLOS scenario are not needed. |
| [12] Sony | Observation 9. For an indoor scenario, ray-tracing simulations could predict the stronger scattering, which reflects the multiplexing richness of the environment, reasonably well. However, ray-tracing did not predict lower-level scattering well. Proposal 9. Continue to study the delay spread for the other applicable scenarios in the 7–24 GHz frequency range. Table 3. Delay-spread scaling factors. Delay in [ns]; TR-38.901 model; Measurements; Ray-tracing detail model; Ray-tracing simple model. Short-delay profile; 16; 15; 15; 15. Normal-delay profile; 24; 40; 38; 38. Long-delay profile; 47; 55; 55; 58. Observation 10. The 3GPP TR 38.901 delay profile mode is valid in the 7–24 GHz frequency for the InH office scenario. Observation 11. Ray-tracing simulations could predict the major features of the power delay profile of InH office channels well. Proposal 10. Continue to study at least the delay profile for the other applicable scenarios in the 7–24 GHz frequency range. |
| [13] Rohde & Schwarz | Figure 13: Estimated RMS DS for 14 GHz Table 3: Estimated Channel Parameters Parameter; 14 GHz. -; min; max. PL exp. (n) w/o unit; 1.72. RMS DS in ns; -; 2.61; 41.74. RMS AS in degree; -; 7.00; 100.76. K-factor in dB; -; 0.08; 14.41. number of paths; -; 7; 250. |
| [16] Sharp | Table 2. Measured omnidirectional DS, ASA, ASD, ZSA, ZSD at 6.75 GHz and 16.95 GHz in the InH scenario for both LOS and NLOS channel conditions [2, 4, 11, 12] computed using Table 13 in Appendix and Comparison with TR 38.901 parameters listed in Table 7.5.6-Part 2 Indoor-Office scenario [6]. The height of the TX and RX used for measured data and TR 38.901 are 4 m and 1.5 m, respectively. Parameter; Env.; InH @ 6.75 GHz; InH @ 16.95 GHz. Measured; TR 38.901; Measured; TR 38.901. Delay Spread (DS) lgDS = log10(DS/1s); μlgDS; LOS; -7.44; -7.7; -7.75; -7.7. NLOS; -7.35; -7.42; -7.46; -7.52. σlgDS; LOS; 0.37; 0.18; 0.29; 0.18. NLOS; 0.21; 0.14; 0.27; 0.18. AOA Spread (ASA) lgASA = log10(ASA/1°); μlgASA; LOS; 1.54; 1.61; 1.14; 1.54. NLOS; 1.74; 1.77; 1.68; 1.73. σlgASA; LOS; 0.42; 0.23; 0.47; 0.27. NLOS; 0.23; 0.17; 0.34; 0.21. AOD Spread (ASD) lgASD = log10(ASD/1°); μlgASD; LOS; 1.84; 1.6; 1.79; 1.6. NLOS; 1.67; 1.62; 1.68; 1.62. σlgASD; LOS; 0.14; 0.18; 0.11; 0.18. NLOS; 0.2; 0.25; 0.33; 0.25. ZOA Spread (ZSA) lgZSA = log10(ZSA/1°); μlgZSA; LOS; 1.2; 1.21; 1.01; 1.11. NLOS; 1.02; 1.25; 0.96; 1.2. σlgZSA; LOS; 0.08; 0.23; 0.06; 0.21. NLOS; 0.29; 0.67; 0.1; 0.63. ZOD Spread (ZSD) lgZSD = log10(ZSD/1°); μlgZSD; LOS; 1.07; 0.96; 0.91; 0.43. NLOS; 1.07; 1.08; 0.89; 1.08. σlgZSD; LOS; 0.06; 0.42; 0.05; 0.46. NLOS; 0.12; 0.36; 0.15; 0.36. Observation 3: RAN1 should consider the parameter values listed for Omnidirectional DS, ASA, ASD, ZSA, ZSD in Table 2 for the InH scenario in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. Table 5. Measured omnidirectional DS, ASA, ASD, ZSA, ZSD at 6.75 GHz and 16.95 GHz in the UMi scenario for both LOS and NLOS channel conditions [13] computed using Table 14 in Appendix and Comparison with TR 38.901 parameters listed in Table 7.5.6-Part 1 UMi - Street Canyon scenario [6]. The height of the TX used for measured data and TR 38.901 are 4 m and 10 m, respectively. The height of the RX is 1.5 m for both measured data and TR 38.901. Parameter; Env.; UMi @ 6.75 GHz; UMi @ 16.95 GHz. Measured; TR 38.901; Measured; TR 38.901. Delay Spread (DS) lgDS = log10(DS/1s); μlgDS; LOS; -7.26; -7.35; -7.39; -7.44. NLOS; -7.14; -7.04; -7.24; -7.13. σlgDS; LOS; 0.35; 0.38; 0.33; 0.38. NLOS; 0.21; 0.42; 0.36; 0.48. AOA Spread (ASA) lgASA = log10(ASA/1°); μlgASA; LOS; 1.17; 1.66; 1.17; 1.63. NLOS; 1.36; 1.74; 1.25; 1.71. σlgASA; LOS; 0.32; 0.29; 0.43; 0.3. NLOS; 0.33; 0.34; 0.27; 0.36. AOD Spread (ASD) lgASD = log10(ASD/1°); μlgASD; LOS; 1.46; 1.17; 1.34; 1.15. NLOS; 1.52; 1.33; 1.45; 1.24. σlgASD; LOS; 0.29; 0.41; 0.28; 0.41. NLOS; 0.32; 0.43; 0.32; 0.47. ZOA Spread (ZOA) lgZSA = log10(ZSA/1°); μlgZSA; LOS; 1.22; 0.64; 1; 0.6. NLOS; 1.09; 0.88; 0.92; 0.87. σlgZSA; LOS; 0.08; 0.3; 0.08; 0.29. NLOS; 0.16; 0.35; 0.13; 0.32. ZOD Spread (ZOD) lgZSD = log10(ZSD/1°); μlgZSD; LOS; NA*; NA*; NA*; NA*. NLOS; NA*; NA*; NA*; NA*. σlgZSD; LOS; 0.08; 0.35; 0.17; 0.35. NLOS; 0.14; 0.35; 0.13; 0.35. Observation 6: RAN1 should consider the parameter values listed for Omnidirectional DS, ASA, ASD, ZSA, ZSD in Table 5 for the UMi scenario in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. Table 9. Measured omnidirectional DS, ASA, ASD, ZSA, ZSD at 6.75 GHz and 16.95 GHz in the InF scenario for both LOS and NLOS channel conditions [16] computed using Table 15 in Appendix and Comparison with TR 38.901 parameters listed in Table 7.5.6-Part 3 InF scenario [6]. The height of the TX and RX used for measured data and TR 38.901 are 2.4 m and 1.5 m. The Volume (V) and Surface Area (S) of the measured factory are 3470 m3 and 1958 m2. Parameter; Env.; InF @ 6.75 GHz; InF @ 16.95 GHz. Measured; TR 38.901; Measured; TR 38.901. Delay Spread (DS) lgDS = log10(DS/1s); μlgDS; LOS; -7.89; -7.57; -7.93; -7.57. NLOS; -7.52; -7.51; -7.65; -7.51. σlgDS; LOS; 0.26; 0.15; 0.27; 0.15. NLOS; 0.09; 0.19; 0.48; 0.19. AOA Spread (ASA) lgASA = log10(ASA/1°); μlgASA; LOS; 1.25; 1.62; 1.05; 1.55. NLOS; 1.8; 1.72; 1.61; 1.72. σlgASA; LOS; 0.5; 0.31; 0.46; 0.35. NLOS; 0.21; 0.3; 0.43; 0.3. AOD Spread (ASD) lgASD = log10(ASD/1°); μlgASD; LOS; 1.43; 1.56; 1.34; 1.56. NLOS; 1.64; 1.57; 1.49; 1.57. σlgASD; LOS; 0.5; 0.25; 0.2; 0.25. NLOS; 0.21; 0.2; 0.32; 0.2. ZOA Spread (ZOA) lgZSA = log10(ZSA/1°); μlgZSA; LOS; 1.25; 1.32; 0.97; 1.25. NLOS; 1.13; 1.33; 0.99; 1.29. σlgZSA; LOS; 0.07; 0.35; 0.09; 0.35. NLOS; 0.15; 0.45; 0.12; 0.45. ZOD Spread (ZOD) lgZSD = log10(ZSD/1°); μlgZSD; LOS; 1.06; 1.35; 0.81; 1.35. NLOS; 0.95; 1.2; 0.81; 1.2. σlgZSD; LOS; 0.07; 0.35; 0.09; 0.35. NLOS; 0.11; 0.55; 0.09; 0.55. Observation 10: RAN1 should consider the parameter values listed for Omnidirectional DS, ASA, ASD, ZSA, ZSD in Table 9 for the InF scenario in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. |
| [17] AT&T | Figure 16: CDF of delay spread in InH LOS and NLOS environments at 7GHz. Figure 17: CDF of delay spread for InH LoS and NLoS at 8GHz Figure 18: CDF of delay spread in InH LoS and NLoS for 15GHz Observation 11: Measurements conducted at 7, 8 and 15 GHz over 650 RX locations on floors of an office building show that the mean delay spread for InH LoS and NLoS environments agree with the 3GPP SCM InH channel model. Proposal 5: There is no need to change the distribution of the delay spread (mean and standard deviation of the normal distribution) for the 3GPP InH channel model in TR 38.901 for extension to 7-24 GHz |

#### Summary of Issues

Based on inputs from companies, moderator suggests further discussion based on the following proposals.

Proposal #2.3-1:

- Based on measurement data provided, RAN1 concludes that following delay spread parameters for 6 – 24 GHz frequency range are validated and no updates to TR are made.

  - InH-Office LOS

  - InH-Office NLOS

##### Proposal #2.3-2:

- For the following scenarios, while sources have observed some differences in delay spread parameters in the TR and measurement taken for 6 – 24 GHz frequency range, RAN1 concludes that there is no census to update delay spread parameters due to lack of consistent and significant observed difference between model and measurements.

  - InF LOS

  - InF NLOS

  - UMa LOS

  - UMa NLOS

##### Proposal #2.3-3:

- For the following scenarios, further study delay spread parameters for 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

#### Round #1 Discussion

Please provide comments on issues regarding delay spread. Please provide comments on Proposal #2.3-1, #2.3-2, #2.3-3.

| Company | Comments |
| --- | --- |
| Apple | Proposal #2.3-1: Based on our measurement results in R1-2409819, the delay spread for InH is not aligned with TR38.901. Hence, we prefer to hold this proposal and we will make more measurements to further check it in next RAN1 meeting. Proposal #2.3-2: We suggest further companies further checking the delay spread of UMa NLOS, or move UMa NLOS to Proposal #2.3-3. Based on our measurements, the delay spread of UMa is quite different from 38.901. Proposal #2.3-3: Although our measurement results are aligned with 38.901 for UMi NLOS, we are fine to further study it, together with UMa NLOS. |
| vivo | Support |
| Huawei, HiSilicon | ##### Proposal #2.3-1: Prefer to postpone. Proposal #2.3-2: Prefer to postpone for UMa LOS/NLOS. Proposal #2.3-3: Fine. |
| Samsung | ##### Support |
| CATT | ##### OK |
| ZTE | Support |
| AT&T | Proposal # 2.3-1: support |
| MediaTek | Support |
| Ericsson | #2.3-1 Support #2.3-2 Support but suggest reformulating the main bullet: - For the following scenarios, while some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR. RAN1 therefore concludes that there is no consensus scensus to update delay models due to lack of consistent and significant observed difference between model and measurements. #2.3-3 Support |

#### Summary of Round #1 Discussion

It seems at least two companies wish to perform further check and study. Moderator has updated proposal #2.3-3A to reflect the inputs.

##### Proposal #2.3-1:

- Based on measurement data provided, RAN1 concludes that following delay spread parameters for 6 – 24 GHz frequency range are validated and no updates to TR are made.

  - InH-Office LOS

  - InH-Office NLOS

##### Proposal #2.3-2A:

- For the following scenarios, while some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR. RAN1 therefore concludes that there is no consensus scensus to update delay models due to lack of consistent and significant observed difference between model and measurements.

  - InF LOS

  - InF NLOS

  - UMa LOS

  - UMa NLOS

Proposal #2.3-3A:

- For the following scenarios, further study delay spread parameters for 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - InH-Office LOS

  - InH-Office NLOS

  - UMa LOS

  - UMa NLOS

#### Round #2/3 Discussion

Please comment on Proposal #2.3-1, #2.3-2A, and #2.3-3A.

While it might be alright to postpone the conclusion, it might be equally important for companies who do no wish to take conclusions in this meeting to provide some information on what they would like to double check specifically so that other companies can also participate in the investigation process. Having a blanket study may not help to get conclusions in the next meeting. Moderator asks companies in favor of postponing the conclusion to provide more specific and concrete information on what they would like more time to verify.

| Company | Comments |
| --- | --- |
| BUPT | Proposal #2.3-2A: Prefer to further study the delay spread for UMa LOS/NLOS Proposal #2.3-3A: Fine. |
| AT&T | ##### Proposal 2.3-3A: support. For InH-office LoS and InH-office NLOS, we think we can conclude in this meeting, but we are ok if companies want to defer |
| Apple | ##### With the update of Proposal #2.3-3A (thanks to FL), we think Proposal #2.3-1 is not needed. |
| Moderator | ##### @Apple: While it is alright to defer the conclusion to next meeting, it would be good if Apple can provide some information on why conclusion should not be made in this meeting. Any related information would aid other companies to better understand the issue. |
| Sharp | ##### Proposal #2.3-2A: We recommend putting InF LOS, NLOS in 2.3-1. There is only 1 source so far that has provided InF measurements, and based on those measurements for DS we don’t think updates are needed. ##### Proposal #2.3-3A: InH LOS/NLOS is also mentioned in 2.3-1. Suggest removing either 2.3-1 or InH LOS/NLOS from 2.3-3A as these proposals are contradictory. |
| Ericsson | ##### The SID is clear that continuity to lower and higher bands shall be ensured. This requires that validation measurements are performed also outside the 6-24 GHz range. Therefore, we suggest to remove “for 6-24 GHz frequency range” from all the proposals. ##### Proposal #2.3-1A: Support (with removal of 6-24 GHz) ##### Proposal #2.3-2A and Proposal #2.3-3A: We prefer to keep UMa LOS/NLOS in this proposal rather than move it to proposal #2.3-3A since we believe the observation that there is no consensus due to diverging measurement results is a fair assessment of the situation. Though if companies have already planned additional measurement campaigns relevant for this topic that will deliver results before next RAN1 meeting we can keep further study open. (also remove of 6-24 GHz) |
| Apple | ##### To respond to FL, as we already provided the reason in Round #1. It is copied below. ##### Based on our measurement results in R1-2409819, the delay spread for InH is NOT aligned with TR38.901. Hence, we prefer to hold this proposal and we will make more measurements to further check it in next RAN1 meeting. Our measurement results show below: Table 2: Comparison of delay spread for Indoor office at 13 GHz Measurement (LOS/NLOS); Indoor office in TR 38.901 (LOS/NLOS). Delay spread mean; -7.26 / -7.19; -7.7 / -7.5. Delay spread standards deviation; 0.11 / 0.12; 0.18 / 0.17. |
| Nokia | ##### Based on measurement results from R1-2410319 we observe that UMi and InH delay spread parameters are very close to the values provided by the 3GPP model. ##### We support that both UMi and InH models can be moved to Proposal #2.3-1 or at least to Proposal #2.3-2A. |

#### Outcome of Wed Session

Modified version of Proposal #2.3-2A was agreed.

Observation

- For the following scenarios, some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR.

  - InF LOS

  - InF NLOS

Conclusion

- For the following scenarios, there is no consensus to update delay models due to lack of consistent and significant observed difference between model and measurements.

  - InF LOS

  - InF NLOS

#### Round #4 Discussion

Proposal #2.3-1 is not pursued as it is covered by Proposal #2.3-3A.

Please provide comments on Proposal #2.3-3A.

##### Proposal #2.3-3A:

- For the following scenarios, further study delay spread parameters for 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - InH-Office LOS

  - InH-Office NLOS

  - UMa LOS

  - UMa NLOS

| Company | Comments |
| --- | --- |
| AT&T | Support. We are also ok to make conclusion on InH-office LoS and NLoS delay spread parameters this meeting if companies are ready. |
| Sharp | Support. Same comment as AT&T. Also, we can maybe conclude on RMa LOS/NLOS DS for sake of completeness similar to InF LOS/NLOS as we have not seen any significant support from any company wanting to update RMa LOS/NLOS DS. Might be a simple proposal to get agreement on. |

#### Outcome of Thursday Session

Agreement

- For the following scenarios, further study delay spread parameters for at least 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - InH-Office LOS

  - InH-Office NLOS

  - UMa LOS

  - UMa NLOS

Agreement

- Based on measurement data provided, RAN1 concludes angular spread parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

### 4.2.4 Angular Spread

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Table 1 Fast fading parameters Scenario; InH @10 GHz; UMi @10 GHz; UMa @6.5 GHz; UMa @13 GHz. TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement. LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS. Delay spread (DS) lgDS=log10 (DS/1s); μlgDS; -7.7; -7.46; -7.79; -7.45; -7.39; -7.08; -7.47; -7.37; -7.03; -6.45; -7.32; -7.01; -7.06; -6.51; -7.59; -7.12. [ns]; 20; 34.7; 16.2; 35.5; 40.7; 83.2; 33.9; 42.7; 93.3; 354.8; 47.9; 97.7; 86.6; 311; 25.3; 76.2. σlgDS; 0.18; 0.16; 0.11; 0.07; 0.38; 0.45; 0.28; 0.1; 0.66; 0.39; 0.38; 0.28; 0.66; 0.39; 0.5; 0.42. AOD spread (ASD) lgASD=log10 (ASD/1°); μlgASD; 1.6; 1.62; 1.21; 1.27; 1.16; 1.29; 1.04; 1.14; 1.15; 1.41; 0.82; 1.26; 1.18; 1.37; 1.08; 1.25. [deg]; 39.8; 41.7; 16.2; 18.6; 14.5; 19.5; 11; 13.8; 14.1; 25.7; 6.6; 18.2; 15.3; 23.6; 12.1; 17.9. σlgASD; 0.18; 0.25; 0.18; 0.14; 0.41; 0.44; 0.2; 0.07; 0.28; 0.28; 0.28; 0.27; 0.28; 0.28; 0.21; 0.3. AOA spread (ASA) lgASA=log10 (ASA/1°); μlgASA; 1.58; 1.75; 1.29; 1.5; 1.65; 1.73; 1.19; 1.37; 1.81; 1.86; 1.67; 1.72; 1.81; 1.78; 1.40; 1.66. [deg]; 38; 56.2; 19.5; 31.6; 44.7; 53.7; 15.5; 23.4; 64.6; 72.4; 46.8; 52.5; 64.6; 60.2; 25.3; 45.8. σlgASA; 0.24; 0.18; 0.13; 0.23; 0.29; 0.35; 0.13; 0.08; 0.2; 0.11; 0.19; 0.15; 0.2; 0.11; 0.13; 0.19. ZOD spread (ZSD) lgZSD=log10 (ZSD/1°); μlgZSD; 0.74; 1.08; 0.99; 1.1; 0.11; -0.11; 0.54; 0.61; 0.54; 0.69; 0.7; 0.78; 0.54; 0.69; 0.68; 0.83. [deg]; 5.5; 12; 9.8; 12.6; 1.3; 0.8; 3.5; 4.1; 3.5; 4.9; 5; 6; 3.5; 4.9; 4.8; 6.8. σlgZSD; 0.44; 0.36; 0.13; 0.12; 0.35; 0.35; 0.12; 0.09; 0.4; 0.49; 0.33; 0.34; 0.40; 0.49; 0.31; 0.35. ZOA spread (ZSA) lgZSA=log10 (ZSA/1°); μlgZSA; 1.17; 1.23; N/A; N/A; 0.63; 0.88; N/A; N/A; 0.95; 1.25; 1.08; 1.15; 0.95; 1.15; 0.72; 1.15. [deg]; 14.8; 17; N/A; N/A; 4.3; 7.6; N/A; N/A; 8.9; 17.8; 12; 14.1; 8.9; 14.2; 5.3; 14.18. σlgZSA; 0.22; 0.65; N/A; N/A; 0.3; 0.34; N/A; N/A; 0.16; 0.16; 0.17; 0.12; 0.16; 0.16; 0.06; 0.15. Number of clusters; 15; 19; 10; 11; 12; 19; 5; 7; 12; 20; 13; 17; 12; 20; 11; 15. Observation 1: The sparsity characteristics can be observed at least for 6-13 GHz: - The measured DSs are smaller than that in TR 38.901. - The measured ASDs/ASAs are smaller than that in TR 38.901. - The measured numbers of clusters are smaller than that in TR 38.901. Proposal 1: Support to update the following fast fading parameters for 6-24 GHz considering measurement results in Table 1: - Delay spread (mean, variance) - AoD spread (mean, variance) - AoA spread (mean, variance) - Number of clusters |
| [2] Ericsson | Figure 7 Azimuth angular spread in three different measurement campaigns compared with the original UMa NLOS model and a proposed revision to the UMa model. Observation 9 The measured angular ASD at 3.4 GHz, 3.5 GHz, 13 GHz, and 28 GHz is lower than expected from TR 38.901. Observation 10 The ASD and ZSD for 13 and 28 GHz are very similar, which is in line with TR 38.901. Table 2 Suggested updates to the TR 38.901 UMa ASD model Scenarios; UMa. LOS; NLOS; O2I. AOD spread (ASD) lgASD=log10(ASD/1); lgASD; 0.39 + 0.1114 log10(fc); 0.83 - 0.1144 log10(fc); 0.58. lgASD; 0.4; 0.7; 0.7. Cluster ASD () in [deg]; 1.5; 1.5; 1.5. Proposal 7 The ASD parameters for the UMa model are adjusted according to Table 2 to better represent measurements at 3.4 and 3.5 GHz in two different cities, and at 13 GHz and 28 GHz in a third city. Proposal 8 The ZSD in the UMa model is considered to be validated by new measurements at 3.4 GHz, 3.5 GHz, 13 GHz, and 28 GHz. |
| [4] Samsung | (a); (b). Figure 4. Azimuth spread of departure angles: (a) 6.5 GHz and (b) 13.5 GHz Observation 4 As shown in below table, slight difference compared to mean and standard deviation of ASD for UMi LOS scenario in the existing channel model were confirmed Azimuth spread of departure angles; UMi_LOS (TR38.901) @ 6.5 GHz; Measured ASD @ 6.5 GHz; TR38.901 @ 13.5 GHz; Measured ASD @ 13.5 GHz. μASD (log_10⁡〖ASD/1°〗 ); 1.16; 1.04; 1.15; 0.98. (degree); 14.66; 11.18; 14.18; 9.6. σASD; 0.41; 0.21; 0.41; 0.20. Figure 5. Azimuth spread of arrival angles: (a) 6.5 GHz and (b) 13.5 GHz Observation 5 As shown in below table, significant difference compared to mean and standard deviation of ASA for UMi scenario in the existing channel model were confirmed. Azimuth spread of arrival angles; UMi_LOS (TR38.901) @ 6.5 GHz; Measured ASA @ 6.5 GHz; TR38.901 @ 13.5 GHz; Measured ASA @ 13.5 GHz. μASA (log_10⁡〖ASA/1°〗 ); 1.65; 1.15; 1.64; 1.11. (degree); 45.71; 14.26; 43.36; 12.7. σASA; 0.29; 0.28; 0.29; 0.33. Proposal 3 RAN1 discuss whether the updates of azimuth spread of departure/arrival angles in UMi LOS scenario is needed |
| [9] Apple | Table 6: Comparison of angular spread for UMa NLOS at 8 GHz Ray tracing simulations; UMa NLOS in TR 38.901. AoA spread mean; 1.72; 1.84. AoD spread standard deviation; 0.6; 2. AoD spread mean; 0.6; 1.4. AoD spread standard deviation; 0.765; 2. ZoA spread mean; 0.54; 1.22. ZoA spread standard deviation; 0.6; 1.4. Proposal 10: RAN1 to consider reducing the AoD/ZoA spread mean values for UMa NLOS scenario in FR3. Table 7: Comparison of angular spread for UMi NLOS at 8 GHz Ray tracing simulations; UMi NLOS in TR 38.901. AoA spread mean; 1.41; 1.73. AoD spread standard deviation; 0.8; 0.35. AoD spread mean; 0.88; 1.31. AoD spread standard deviation; 1; 0.44. ZoA spread mean; 0.18; 0.88. ZoA spread standard deviation; 1.1; 0.34. Proposal 11: RAN1 to consider reducing the AoA/AoD/ZoA spread mean values for UMi NLOS scenario in FR3. |
| [12] Sony | Figure 13. Comparison of the angle of departure (AoD) at 15 GHz between measurements (circles) and ray-tracing simulations (crosses), as seen from the transmitter. Observation: The measurements for indoor scenarios show a similar number of strong scattering multipath components, which reflect the multiplexing richness, compared to ray-tracing simulations, but ray-tracing simulations did not capture as many weak scatterings in the measurements. |
| [13] Rohde & Schwarz | Figure 14: Estimated RMS AS for 14 GHz Table 3: Estimated Channel Parameters Parameter; 14 GHz. -; min; max. PL exp. (n) w/o unit; 1.72. RMS DS in ns; -; 2.61; 41.74. RMS AS in degree; -; 7.00; 100.76. K-factor in dB; -; 0.08; 14.41. number of paths; -; 7; 250. |
| [15] Vodafone, Ericsson | Figure 2 Vertical angular spread (ZSD) as observed at the base station Observation 1 The measured elevation angular spreads (ZSD) at 3.4 GHz for a very large number of communication links in an operational urban macro 5G NR network match the 38.901 UMa model. Figure 3 Horizontal angular spread (ASD) as observed at the base station Observation 2 The measured azimuth angular spreads (ASD) at 3.4 GHz for a very large number of communication links in an operational urban macro 5G NR network are several times lower than predicted by the 38.901 UMa model. Table 1 Parameters of the TR 38.901 UMa ASD model Scenarios; UMa. LOS; NLOS; O2I. AOD spread (ASD) lgASD=log10(ASD/1); µlgASD; 1.06 + 0.1114 log10(fc); 1.5 - 0.1144 log10(fc); 1.25. σlgASD; 0.28; 0.28; 0.42. Cluster ASD () in [deg]; 5; 2; 5. Table 2 Suggested updates to the TR 38.901 UMa ASD model Scenarios; UMa. LOS; NLOS; O2I. AOD spread (ASD) lgASD=log10(ASD/1); µlgASD; 0.39 + 0.1114 log10(fc); 0.83 - 0.1144 log10(fc); 0.58. σlgASD; 0.4; 0.7; 0.7. Cluster ASD () in [deg]; 1.5; 1.5; 1.5. Proposal 1 The ASD parameters for the UMa model are adjusted according to Table 2 to better represent measurements. Figure 5 ZSD as observed at the suburban macro base station. A model curve (black dashed) with tuned parameters provides a good match to the suburban measurements. Observation 3 The measured ZSDs at 3.4 GHz in a suburban 5G NR macrocell are very similar to the ZSDs in urban macrocells, however at the upper percentiles there are less high outlier values. Observation 4 The measured suburban ZSD can be well represented by a lognormal distribution with µlgZSD = 0.14 and σlgZSD = 0.16. |
| [16] Sharp, Nokia | Table 2. Measured omnidirectional DS, ASA, ASD, ZSA, ZSD at 6.75 GHz and 16.95 GHz in the InH scenario for both LOS and NLOS channel conditions [2, 4, 11, 12] computed using Table 13 in Appendix and Comparison with TR 38.901 parameters listed in Table 7.5.6-Part 2 Indoor-Office scenario [6]. The height of the TX and RX used for measured data and TR 38.901 are 4 m and 1.5 m, respectively. Parameter; Env.; InH @ 6.75 GHz; InH @ 16.95 GHz. Measured; TR 38.901; Measured; TR 38.901. Delay Spread (DS) lgDS = log10(DS/1s); μlgDS; LOS; -7.44; -7.7; -7.75; -7.7. NLOS; -7.35; -7.42; -7.46; -7.52. σlgDS; LOS; 0.37; 0.18; 0.29; 0.18. NLOS; 0.21; 0.14; 0.27; 0.18. AOA Spread (ASA) lgASA = log10(ASA/1°); μlgASA; LOS; 1.54; 1.61; 1.14; 1.54. NLOS; 1.74; 1.77; 1.68; 1.73. σlgASA; LOS; 0.42; 0.23; 0.47; 0.27. NLOS; 0.23; 0.17; 0.34; 0.21. AOD Spread (ASD) lgASD = log10(ASD/1°); μlgASD; LOS; 1.84; 1.6; 1.79; 1.6. NLOS; 1.67; 1.62; 1.68; 1.62. σlgASD; LOS; 0.14; 0.18; 0.11; 0.18. NLOS; 0.2; 0.25; 0.33; 0.25. ZOA Spread (ZSA) lgZSA = log10(ZSA/1°); μlgZSA; LOS; 1.2; 1.21; 1.01; 1.11. NLOS; 1.02; 1.25; 0.96; 1.2. σlgZSA; LOS; 0.08; 0.23; 0.06; 0.21. NLOS; 0.29; 0.67; 0.1; 0.63. ZOD Spread (ZSD) lgZSD = log10(ZSD/1°); μlgZSD; LOS; 1.07; 0.96; 0.91; 0.43. NLOS; 1.07; 1.08; 0.89; 1.08. σlgZSD; LOS; 0.06; 0.42; 0.05; 0.46. NLOS; 0.12; 0.36; 0.15; 0.36. Observation 3: RAN1 should consider the parameter values listed for Omnidirectional DS, ASA, ASD, ZSA, ZSD in Table 2 for the InH scenario in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. Table 5. Measured omnidirectional DS, ASA, ASD, ZSA, ZSD at 6.75 GHz and 16.95 GHz in the UMi scenario for both LOS and NLOS channel conditions [13] computed using Table 14 in Appendix and Comparison with TR 38.901 parameters listed in Table 7.5.6-Part 1 UMi - Street Canyon scenario [6]. The height of the TX used for measured data and TR 38.901 are 4 m and 10 m, respectively. The height of the RX is 1.5 m for both measured data and TR 38.901. Parameter; Env.; UMi @ 6.75 GHz; UMi @ 16.95 GHz. Measured; TR 38.901; Measured; TR 38.901. Delay Spread (DS) lgDS = log10(DS/1s); μlgDS; LOS; -7.26; -7.35; -7.39; -7.44. NLOS; -7.14; -7.04; -7.24; -7.13. σlgDS; LOS; 0.35; 0.38; 0.33; 0.38. NLOS; 0.21; 0.42; 0.36; 0.48. AOA Spread (ASA) lgASA = log10(ASA/1°); μlgASA; LOS; 1.17; 1.66; 1.17; 1.63. NLOS; 1.36; 1.74; 1.25; 1.71. σlgASA; LOS; 0.32; 0.29; 0.43; 0.3. NLOS; 0.33; 0.34; 0.27; 0.36. AOD Spread (ASD) lgASD = log10(ASD/1°); μlgASD; LOS; 1.46; 1.17; 1.34; 1.15. NLOS; 1.52; 1.33; 1.45; 1.24. σlgASD; LOS; 0.29; 0.41; 0.28; 0.41. NLOS; 0.32; 0.43; 0.32; 0.47. ZOA Spread (ZOA) lgZSA = log10(ZSA/1°); μlgZSA; LOS; 1.22; 0.64; 1; 0.6. NLOS; 1.09; 0.88; 0.92; 0.87. σlgZSA; LOS; 0.08; 0.3; 0.08; 0.29. NLOS; 0.16; 0.35; 0.13; 0.32. ZOD Spread (ZOD) lgZSD = log10(ZSD/1°); μlgZSD; LOS; NA*; NA*; NA*; NA*. NLOS; NA*; NA*; NA*; NA*. σlgZSD; LOS; 0.08; 0.35; 0.17; 0.35. NLOS; 0.14; 0.35; 0.13; 0.35. Observation 6: RAN1 should consider the parameter values listed for Omnidirectional DS, ASA, ASD, ZSA, ZSD in Table 5 for the UMi scenario in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. Table 6. Measured omnidirectional ZSD at 6.75 GHz and 16.95 GHz in the UMi scenario for both LOS and NLOS channel conditions [13] and Comparison with TR 38.901 parameters listed in Table 7.5.6-Part 1 UMi - Street Canyon scenario [6]. The height of the BS used for measured data and TR 38.901 are 4 m and 10 m, respectively. The height of the UT is 1.5 m for both measured data and TR 38.901. Frequency; Env.; Distance (m); Measured ZSD = log10(ZSD/1°); TR 38.901 ZSD. (hBS = 4, hUT = 1.5); (hBS = 10, hUT = 1.5). 6.75 GHz; LOS; 40; 0.93; 0.32. 100; 0.93; -0.21. 193; 1.11; -0.21. 424; 1.12; -0.21. 58; 0.99; 0.06. 120; 1.06; -0.21. 52; 1.02; 0.15. NLOS; 560; 0.98; -0.5. 880; 0.76; -0.5. 51.5; 0.99; 0.04. 100.9; 1.14; -0.11. 71; 0.89; -0.02. 45; 0.93; 0.06. 125; 0.99; -0.19. 88.7; 0.91; -0.07. 185; 0.51; -0.37. 170; 1.08; -0.33. 141; 1.03; -0.24. 16.95 GHz; LOS; 40; 0.6; 0.32. 100; 0.75; -0.21. 193; 0.75; -0.21. 410; 0.95; -0.21. 58; 0.95; 0.06. 120; 0.88; -0.21. 52; 0.95; 0.15. NLOS; 560; 0.89; -0.5. 880; 0.73; -0.5. 51.5; 0.86; 0.04. 100.9; 0.99; -0.11. 71; 0.92; -0.02. 45; 0.99; 0.06. 125; 1.06; -0.19. 88.7; 0.91; -0.07. 185; 0.72; -0.37. 170; 0.86; -0.33. 141; 0.62; -0.24. Observation 7: RAN1 should consider the parameter values listed for Omnidirectional ZSD in Table 6 for the UMi scenario in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. Table 9. Measured omnidirectional DS, ASA, ASD, ZSA, ZSD at 6.75 GHz and 16.95 GHz in the InF scenario for both LOS and NLOS channel conditions [16] computed using Table 15 in Appendix and Comparison with TR 38.901 parameters listed in Table 7.5.6-Part 3 InF scenario [6]. The height of the TX and RX used for measured data and TR 38.901 are 2.4 m and 1.5 m. The Volume (V) and Surface Area (S) of the measured factory are 3470 m3 and 1958 m2. Parameter; Env.; InF @ 6.75 GHz; InF @ 16.95 GHz. Measured; TR 38.901; Measured; TR 38.901. Delay Spread (DS) lgDS = log10(DS/1s); μlgDS; LOS; -7.89; -7.57; -7.93; -7.57. NLOS; -7.52; -7.51; -7.65; -7.51. σlgDS; LOS; 0.26; 0.15; 0.27; 0.15. NLOS; 0.09; 0.19; 0.48; 0.19. AOA Spread (ASA) lgASA = log10(ASA/1°); μlgASA; LOS; 1.25; 1.62; 1.05; 1.55. NLOS; 1.8; 1.72; 1.61; 1.72. σlgASA; LOS; 0.5; 0.31; 0.46; 0.35. NLOS; 0.21; 0.3; 0.43; 0.3. AOD Spread (ASD) lgASD = log10(ASD/1°); μlgASD; LOS; 1.43; 1.56; 1.34; 1.56. NLOS; 1.64; 1.57; 1.49; 1.57. σlgASD; LOS; 0.5; 0.25; 0.2; 0.25. NLOS; 0.21; 0.2; 0.32; 0.2. ZOA Spread (ZOA) lgZSA = log10(ZSA/1°); μlgZSA; LOS; 1.25; 1.32; 0.97; 1.25. NLOS; 1.13; 1.33; 0.99; 1.29. σlgZSA; LOS; 0.07; 0.35; 0.09; 0.35. NLOS; 0.15; 0.45; 0.12; 0.45. ZOD Spread (ZOD) lgZSD = log10(ZSD/1°); μlgZSD; LOS; 1.06; 1.35; 0.81; 1.35. NLOS; 0.95; 1.2; 0.81; 1.2. σlgZSD; LOS; 0.07; 0.35; 0.09; 0.35. NLOS; 0.11; 0.55; 0.09; 0.55. Observation 10: RAN1 should consider the parameter values listed for Omnidirectional DS, ASA, ASD, ZSA, ZSD in Table 9 for the InF scenario in both LOS and NLOS channel conditions at 6.75 GHz and 16.95 GHz. |
| [17] AT&T | Figure 19: CDF of azimuth angular spread in InH LOS and NLOS environments. Figure 20: Norm plot of the InH ASA and ZSA shows that it follows a log normal distribution at 15GHz Figure 21: CDF of zenith angular spread in InH LOS and NLOS environments at 15GHz. Figure 22: Norm plot of the InH ASA and ZSA shows that it follows a log normal distribution at 8 GHz Observation 12: Measurements conducted at 8GHz and 15 GHz 650 RX locations on floors of an office building show that the mean angular spread (ASA) for LoS and NLoS environments agree with the 3GPP SCM InH channel model. Observation 13: Measurements conducted at 8GHz and 15 GHz 650 RX locations on floors of an office building show that the mean angular spread (ZSA) for LoS and NLoS environments do not agree with the 3GPP SCM InH channel model. Table 4: Channel parameters comparison between AT&T indoor measurements at 15GHz and 3GPP InH model in TR38.901 InH Parameters; 7 GHz; 8 GHz; 11 GHz; 15GHz. LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS. Pathloss, dB; PLo; 48.3/49.2; 42. 6/38.2; 51.1/50.8; 45.0/40.2; FFS; FFS; 56.4 /55.6; 51.3/46.2. PLE; 1.5/1.7; 3.2/3.8; 1.4/1.7; 3.2/3.8; FFS; FFS; 1.5/1.7; 3.4/3.8. SF, dB; 2.9/3.0; 6.6/8.0; 2.6/3.0; 6.6/8.0; FFS; FFS; 2.3/3.0; 7.3/8.0. Delay spread (DS) lgDS=log10(DS/1s); μlgDS; -7.9/-7.7; -7.6/-7.43; -7.9/-7.7; -7.6/-7.44; FFS; FFS; -7.9/-7.7; -7.6/-7.5. σlgDS; 0.34/0.18; 0.23/0.064; 0.34/0.18; 0.21/0.065; FFS; FFS; 0.34/0.18; 0.22/0.067. AOD spread (ASD) lgASD=log10(ASD/1°); μlgDS; FFS; FFS; FFS; FFS; FFS; FFS; FFS; FFS. σlgDS; FFS; FFS; FFS; FFS; FFS; FFS; FFS; FFS. AOA spread (ASA) lgASA=log10(ASA/1°); μlgDS; FFS; FFS; 1.7/1.60; 1.8/1.758; FFS; FFS; 1.567/1.552; 1.776/1.731. σlgDS; FFS; FFS; 0.12/0.234; 0.12/0.174; FFS; FFS; 0.146/0.264; 0.148/0.204. ZOA spread (ZSA) lgZSA=log10(ZSA/1°); μlgDS; FFS; FFS; 1.3/0.192; 1.2/1.244; FFS; FFS; 0.940/1.127; 0.935/1.206. σlgDS; FFS; FFS; 0.03/0.226; 0.025/0.660; FFS; FFS; 0.048/0.216; 0.064/0.638. K-Factor (K), dB; μlgDS; FFS; FFS; FFS; FFS; FFS; FFS; FFS; FFS. σlgDS; FFS; FFS; FFS; FFS; FFS; FFS; FFS; FFS. Cross-Correlations; ASA vs DS; FFS; FFS; 0.1/0.8; 0.12; FFS; FFS; /0.8; /0. ASA vs SF; FFS; FFS; -0.012/-0.5; -0.15; FFS; FFS; /-0.5; /-0.4. DS vs SF; FFS; FFS; -0.59 /-0.8; -0.45/-0.5; FFS; FFS; -0.59/-0.8; -0.45/-0.5. |
| [19] Qualcomm | Proposal 10: RAN1 to study how any potential change to the marginal distribution of angular spreads, K-factor and shadow fading impacts their joint distribution and consequently their pairwise correlation. Figure 13 Angular spread of the azimuth angle of departure obtained using a ray tracing simulation in a UMi-like environment. Figure 14 Angular spread of the zenith angle of departure obtained using a ray tracing simulation in a UMi-like environment. Proposal 11: RAN1 to study the impact on frequency continuity of any potential changes to the distribution of angular spreads, K-factor, and shadow fading for FR3 bands. Before proceeding to change the marginal distribution for any large-scale parameter, relative comparisons to FR1 and FR2 bands need to be taken into account. |

#### Summary of Issues

The following is observations on angular spread for each parameter. Company information was gathered from data that were submitted in this meeting as well as previous meetings.

| Scenario | LOS / NLOS | Angle | Measurement Summary |  |  | Moderator Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Lower than TR | Comparative to TR | Higher than TR |  |
| InH | LOS | ASD | Huawei/HiSilicon, Keysight | BUPT | Sharp/Nokia | Inconclusive |
|  |  | ASA | Huawei/HiSilicon, Sharp/Nokia, Keysight | AT&T, BUPT |  | Inconclusive |
|  |  | ZSA | AT&T, Keysight | Huawei/HiSilicon, Sharp/Nokia, BUPT |  | Inconclusive |
|  |  | ZSD |  | Huawei/HiSilicon | Sharp/Nokia | Inconclusive |
|  | NLOS | ASD | Huawei/HiSilicon, Keysight |  | Sharp/Nokia | Inconclusive |
|  |  | ASA | Huawei/HiSilicon, Sharp/Nokia | AT&T |  | Inconclusive |
|  |  | ZSA | Sharp/Nokia, AT&T, Keysight | Huawei/HiSilicon |  | Inconclusive |
|  |  | ZSD | Sharp/Nokia, Keysight | Huawei/HiSilicon |  | Inconclusive |
| UMi | LOS | ASD | Huawei, HiSilicon, Samsung, Qualcomm, Keysight |  | Sharp/Nokia, BUPT | Inconclusive |
|  |  | ASA | Huawei/HiSilicon, Samsung, Sharp/Nokia, Southeast University/ Purple Mountain Lab/ China Telecom, Keysight | BUPT |  | Majority of sources show lower ASA, with 1 exception. Discuss |
|  |  | ZSA |  | Huawei/HiSilicon | Sharp/Nokia, BUPT, Southeast University/ Purple Mountain Lab/ China Telecom, Keysight | Inconclusive |
|  |  | ZSD | Qualcomm | Huawei/HiSilicon |  | Inconclusive |
|  | NLOS | ASD | Huawei/HiSilicon, Apple, Qualcomm, Keysight |  | Sharp/Nokia, BUPT | Inconclusive |
|  |  | ASA | Huawei/HiSilicon. Apple, Sharp/Nokia, Southeast University/ Purple Mountain Lab/ China Telecom, Keysight | BUPT |  | Majority of sources show lower ASA, with 1 exception. Discuss |
|  |  | ZSA | Apple, | Huawei/HiSilicon | Sharp/Nokia, BUPT, Southeast University/ Purple Mountain Lab/ China Telecom, Keysight | Inconclusive |
|  |  | ZSD | Qualcomm | Huawei/HiSilicon |  | Inconclusive |
| UMa | LOS | ASD | Huawei/HiSilicon, Ericsson | BUPT |  | Inconclusive |
|  |  | ASA | Huawei/HiSilicon, BUPT |  |  | Aligned results to update |
|  |  | ZSA | BUPT | Huawei/HiSilicon |  | Inconclusive |
|  |  | ZSD |  | Huawei/HiSilicon |  | Conclude to not update |
|  | NLOS | ASD | Huawei/HiSilicon, Ericsson, Apple, Vodafone, BUPT |  |  | Aligned results to update |
|  |  | ASA | Huawei, HiSilicon, Apple, BUPT |  |  | Aligned results to update |
|  |  | ZSA | Huawei/HiSilicon, Apple, BUPT |  |  | Aligned results to update |
|  |  | ZSD |  | Huawei/HiSilicon, Ericsson, Vodafone |  | Conclude to not update |
| InF | LOS | ASD | Sharp/Nokia |  |  | Single source, discuss on way forward |
|  |  | ASA | Sharp/Nokia |  |  | Single source, discuss on w Single source, discuss on way forward ay forward |
|  |  | ZSA | Sharp/Nokia |  |  | Single source, discuss on way forward |
|  |  | ZSD | Sharp/Nokia |  |  | Single source, discuss on way forward |
|  | NLOS | ASD |  | Sharp/Nokia |  | Single source, discuss on way forward |
|  |  | ASA | Sharp/Nokia |  |  | Single source, discuss on way forward |
|  |  | ZSA | Sharp/Nokia |  |  | Single source, discuss on way forward |
|  |  | ZSD | Sharp/Nokia |  |  | Single source, discuss on way forward |

There are issues that require discussion among companies.

Proposal #2.4-1:

- Based on measurement data provided, RAN1 concludes that following angular spread parameters for 6 – 24 GHz frequency range are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

##### Proposal #2.4-2:

- For the following scenarios, while sources have observed some differences in angular spread parameters in the TR and measurement taken for 6 – 24 GHz frequency range, RAN1 concludes that there is no census to update angular spread parameters due to lack of consistent and significant observed difference between model and measurements.

  - InH LOS and NLOS: ASD, ASA, ZSA, ZSD

  - UMi LOS: ASD, ZSA, ZSD

  - UMi NLOS: ASD, ZSA, ZSD

  - UMa LOS: ASD, ZSA

##### Proposal #2.4-3:

- For the following scenarios, RAN1 concludes there are necessary changes required to align angular spread measurements to the channel model for 6 – 24 GHz frequency range. Details of the actual change needed is FFS.

  - UMa LOS: ASA

  - UMa NLOS: ASA, ASD, ZSA

##### Proposal #2.4-4:

- For the following scenarios, further study delay spread parameters for 6 – 24 GHz frequency range.

  - UMi LOS: ASA

  - UMi NLOS: ASA

  - InF LOS and NLOS: ASD, ASA, ZSA, ZSD

#### Round #1 Discussion

Please provide comments on issues regarding angle distribution. Please provide comments on the general observation table (to correct any mistakes) and comments on the proposal #2.4-1, #2.4-2, #2.4-3, #2.4-4.

| Company | Comments |
| --- | --- |
| Apple | Proposal #2.4-1: Fine Proposal #2.4-2: Fine except for UMi NLOS/ASD, ZSA. Our measurement shows they are not aligned with 38.901. Maybe we could move these parameters to Proposal #2.4-4. Proposal #2.4-3: Support except for UMa NLOS/ASA. Our measurement shows this is aligned with 38.901. Maybe we could move this parameter to Proposal #2.4-4. Proposal #2.4-4: Fine with more parameters for further check: UMi NLOS/ASD, ZSA; UMa NLOS/ASA |
| vivo | Support |
| Huawei, HiSilicon | ##### Proposal #2.4-1: Fine. Proposal #2.4-2: Prefer to postpone for ASD under listed scenarios and ASA under InH LOS/NLOS. Proposal #2.4-3: Fine. Proposal #2.4-4: Fine. |
| Samsung | ##### Support |
| ZTE | Fine with Proposal #2.4-1, Proposal #2.4-2 and Proposal #2.4-4 For Proposal #2.4-3, we think the decision should be made by combining the new data set and the existing data set in TR 38.901, the original source in TR 38.901 can also be considered in the parameter update in this SI, then according to the combined data set, we can determine whether to modify the value of the parameters. |
| MediaTek | Proposal #2.4-1: Fine. Proposal #2.4-2: Fine. Proposal #2.4-4: Fine. |
| Ericsson | #2.3-1 Support #2.3-2 Support but suggest reformulating the main bullet: - For the following scenarios, while some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR. RAN1 therefore concludes that there is no consensus scensus to update delay models due to lack of consistent and significant observed difference between model and measurements. #2.3-3 Support |

#### Summary of Round #1 Discussion

Proposal #2.4-1: no negative comment so far.

Proposal #2.4-2: Apple has concerns on UMi NLOS: ASD, ZSA. Huawei has concerns on InH LOS/NLOS ASA, and ASD values for liste scenarios.

Proposal #2.4-3: Apple has concerns on UMa NLOS: ASA, ZTE is asking to postpone decision.

Proposal #2.4-4:

##### Proposal #2.4-1:

- Based on measurement data provided, RAN1 concludes that following angular spread parameters for 6 – 24 GHz frequency range are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

##### Proposal #2.4-2A:

- For the following scenarios, while some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR. RAN1 therefore concludes that there is no consensus scensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

  - InH LOS and NLOS: ASD, ASA, ZSA, ZSD

  - UMi LOS: ASD, ZSA, ZSD

  - UMi NLOS: ASD, ZSA, ZSD

  - UMa LOS: ASD, ZSA

#### Proposal #2.4-3A:

- For the following scenarios, RAN1 concludes there are necessary changes required to align angular spread measurements to the channel model for 6 – 24 GHz frequency range. Details of the actual change needed is FFS.

  - UMa LOS: ASA

  - UMa NLOS: ASA, ASD, ZSA

#### Proposal #2.4-4A:

- For the following scenarios, further study angular spread parameters for 6 – 24 GHz frequency range.

  - InH LOS/NLOS: ASA, ASD

  - UMi LOS: ASA, ASD

  - UMi NLOS: ASA, ASD, ZSA

  - UMa LOS: ASD

  - UMa NLOS: ASA

  - InF LOS and NLOS: ASD, ASA, ZSA, ZSD

#### Round #2 Discussion

Moderator askes companies to provide further comments on Proposal #2.4-1, #2.4-2A, #2.4-3A, and #2.4-4A.

| Company | Comments |
| --- | --- |
| Nokia | For Proposal #2.4-3, we do not agree on the necessity of change. Even though in some scenarios, it could be the difference to the current model, it does not mean that the current model is wrong and require change. Multiple verifications should be done to verify changes, including measurement at different carries in the same conditions, evaluation of system-level impacts due to change. Additionally, it is not clear how to achieve continuity with existing model in frequency domain. |
| Moderator | ##### @Nokia: If you can accept that companies who provided measurement data have conducted their diligence and performed the measurement correctly and if you think the differences may not result in material changes to evaluation of the system, it would be good to also provide some evaluations yourself or at least some reasonable information why you think there may not be material differences. From moderator understanding, AS is known to have impact to frequency selectivity of the channel covariance, which may have material impact to system. If it is the case that more measurement data is needed before making a conclusion, that could be fair thing to ask, but moderator worries concerning companies can always simply ask for more measurement without any material work on their own. This simply disrupts SI progression. It would be helpful if Nokia can provide some further information what specific they are concerned about (e.g. measurement setup assumptions not belonging to UMa, some data that shows no material system performance difference, etc.). |
| AT&T | Proposal #2.4-4A: (editorial comment): update the main bullet to reflect angular spread and not delay spread For InH ASA and ZSA we think we are ready to make conclusions in this meeting. For UMa and UMi we prefer to defer the conclusion to next meeting to bring our measurement results |
| Apple | OK |
| ZTE | For Proposal #2.4-3A, we suggest to do the curve fitting using combined data as we commented in last round before concluding on the necessity. The reason behind is that, in this SI, we have 2 or 3 sources providing measurement results, however, we also need to respect the measurement results contributing to existing values in TR 38.901, so the new and previous data set should be combined to make the decision. Moreover, since we already agreed that the new value will replace the existing value in TR 38.901 if we decide to make the change, we need to be even more careful on making the decision. |

#### Round #3 Discussion

| Company | Comments |
| --- | --- |
| Ericsson | Same comment as previously, remove “6-24 GHz”. Considering the comments, maybe it is indeed premature to conclude the changes for UMa in this meeting if companies are planning on bringing results to the next meeting. Still, all the evidence so far points toward changing the model – this could be reflected better in the proposal. How about this: - For the following scenarios, RAN1 observes, based on the measurements supplied in and before RAN1#119, that concludes there are necessary changes required to align angular spread measurements to the channel model for 6 – 24 GHz frequency range. Details of whether/how to change the modelthe actual change needed is FFS is to be decided in RAN1#120 based on additional measurement inputs. - UMa LOS: ASD, ASA - UMa NLOS: ASA, ASD, ZSA |
| Nokia-2 | We still have a concern about Proposal #2.4-3A that the changes are necessary. In other scenarios, e.g., in Umi, we also observe that there are differences to the current 38.901 model but we still think that current parameterization captures the propagation characteristics sufficiently well. The conclusion about the necessity of change cannot be concluded just based on the measurements in 6-24. It is not clear whether the changes are applied to the whole 0.5-100 model or only to the given band, and how it is even possible to introduce the change. Moreover, it is necessary to check if there are any system-level impacts of such a change. It is too early to make such a strict conclusion at this stage for UMa scenario. We are OK to capture as an observation that current measurements results by the companies indicate the difference to the existing model parameters. However, we need to study further whether and how there changes need to be captured in the model. |

#### Summary of Round #3 Discussion

Moderator has updated the proposal #2.4-1 and #2.4-2 based on conclusions made on Wednesday session for another parameter (delay spread).

##### Proposal #2.4-1A:

- Based on measurement data provided, RAN1 concludes angular spread parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

##### Proposal #2.4-2B:

Suggested Observation

- For the following scenarios, some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed angular spread consistent with the model in the TR.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Suggested Conclusion

- For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

##### Proposal #2.4-3A:

- For the following scenarios, RAN1 concludes there are necessary changes required to align angular spread measurements to the channel model for 6 – 24 GHz frequency range. Details of the actual change needed is FFS.

  - UMa LOS: ASA

  - UMa NLOS: ASD, ZSA

##### Proposal #2.4-4A:

- For the following scenarios, further study angular spread parameters for 6 – 24 GHz frequency range.

  - InH LOS/NLOS: ASA, ASD

  - UMi LOS: ASA, ASD

  - UMi NLOS: ASA, ASD, ZSA

  - UMa LOS: ASD

  - UMa NLOS: ASA

  - InF LOS and NLOS: ASD, ASA, ZSA, ZSD

##### Proposal #2.4-4B:

- For the following scenarios, further study angular spread parameters for 6 – 24 GHz frequency range.

  - InH LOS/NLOS: ASA, ASD

  - UMi LOS: ASA, ASD

  - UMi NLOS: ASA, ASD, ZSA

  - UMa LOS: ASD, ASA

  - UMa NLOS: ASA, ASD, ZSA

  - InF LOS and NLOS: ASD, ASA, ZSA, ZSD

#### Round #4 Discussion

Please provide comments on Proposal #2.4-1A, #2.4-2B, and #2.4-4B.

| Company | Comments |
| --- | --- |
| Moderator | Proposal #2.4-3A will not be pursued in this meeting. |
| AT&T | - observation 2.4-2A: Ok to support. Editorial change: For the following scenarios, some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay angular spread consistent with the model in the TR. - InH LOS and NLOS: ZSA, ZSD - UMi LOS: ZSA, ZSD - UMi NLOS: ZSD - UMa LOS: ZSA - Propsal 2.4-4B: support |
| Moderator | Correct the typo identified by AT&T |

#### Outcome of Thursday Session

Observation

- For the following scenarios, some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed angular spread consistent with the model in the TR.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Conclusion

- For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Agreement

- For the following scenarios, further study angular spread parameters for at least 6 – 24 GHz frequency range.

  - InH LOS/NLOS: ASA, ASD

  - UMi LOS: ASA, ASD

  - UMi NLOS: ASA, ASD, ZSA

  - UMa LOS: ASD, ASA

  - UMa NLOS: ASA, ASD, ZSA

  - InF LOS and NLOS: ASD, ASA, ZSA, ZSD

### 4.2.5 Clusters

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Table 1 Fast fading parameters Scenario; InH @10 GHz; UMi @10 GHz; UMa @6.5 GHz; UMa @13 GHz. TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement; TR 38.901; Measurement. LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS. Delay spread (DS) lgDS=log10 (DS/1s); μlgDS; -7.7; -7.46; -7.79; -7.45; -7.39; -7.08; -7.47; -7.37; -7.03; -6.45; -7.32; -7.01; -7.06; -6.51; -7.59; -7.12. [ns]; 20; 34.7; 16.2; 35.5; 40.7; 83.2; 33.9; 42.7; 93.3; 354.8; 47.9; 97.7; 86.6; 311; 25.3; 76.2. σlgDS; 0.18; 0.16; 0.11; 0.07; 0.38; 0.45; 0.28; 0.1; 0.66; 0.39; 0.38; 0.28; 0.66; 0.39; 0.5; 0.42. AOD spread (ASD) lgASD=log10 (ASD/1°); μlgASD; 1.6; 1.62; 1.21; 1.27; 1.16; 1.29; 1.04; 1.14; 1.15; 1.41; 0.82; 1.26; 1.18; 1.37; 1.08; 1.25. [deg]; 39.8; 41.7; 16.2; 18.6; 14.5; 19.5; 11; 13.8; 14.1; 25.7; 6.6; 18.2; 15.3; 23.6; 12.1; 17.9. σlgASD; 0.18; 0.25; 0.18; 0.14; 0.41; 0.44; 0.2; 0.07; 0.28; 0.28; 0.28; 0.27; 0.28; 0.28; 0.21; 0.3. AOA spread (ASA) lgASA=log10 (ASA/1°); μlgASA; 1.58; 1.75; 1.29; 1.5; 1.65; 1.73; 1.19; 1.37; 1.81; 1.86; 1.67; 1.72; 1.81; 1.78; 1.40; 1.66. [deg]; 38; 56.2; 19.5; 31.6; 44.7; 53.7; 15.5; 23.4; 64.6; 72.4; 46.8; 52.5; 64.6; 60.2; 25.3; 45.8. σlgASA; 0.24; 0.18; 0.13; 0.23; 0.29; 0.35; 0.13; 0.08; 0.2; 0.11; 0.19; 0.15; 0.2; 0.11; 0.13; 0.19. ZOD spread (ZSD) lgZSD=log10 (ZSD/1°); μlgZSD; 0.74; 1.08; 0.99; 1.1; 0.11; -0.11; 0.54; 0.61; 0.54; 0.69; 0.7; 0.78; 0.54; 0.69; 0.68; 0.83. [deg]; 5.5; 12; 9.8; 12.6; 1.3; 0.8; 3.5; 4.1; 3.5; 4.9; 5; 6; 3.5; 4.9; 4.8; 6.8. σlgZSD; 0.44; 0.36; 0.13; 0.12; 0.35; 0.35; 0.12; 0.09; 0.4; 0.49; 0.33; 0.34; 0.40; 0.49; 0.31; 0.35. ZOA spread (ZSA) lgZSA=log10 (ZSA/1°); μlgZSA; 1.17; 1.23; N/A; N/A; 0.63; 0.88; N/A; N/A; 0.95; 1.25; 1.08; 1.15; 0.95; 1.15; 0.72; 1.15. [deg]; 14.8; 17; N/A; N/A; 4.3; 7.6; N/A; N/A; 8.9; 17.8; 12; 14.1; 8.9; 14.2; 5.3; 14.18. σlgZSA; 0.22; 0.65; N/A; N/A; 0.3; 0.34; N/A; N/A; 0.16; 0.16; 0.17; 0.12; 0.16; 0.16; 0.06; 0.15. Number of clusters; 15; 19; 10; 11; 12; 19; 5; 7; 12; 20; 13; 17; 12; 20; 11; 15. Figure 10 Comparison between the measured and realistic number of clusters Observation 1: The sparsity characteristics can be observed at least for 6-13 GHz: - The measured DSs are smaller than that in TR 38.901. - The measured ASDs/ASAs are smaller than that in TR 38.901. - The measured numbers of clusters are smaller than that in TR 38.901. Proposal 1: Support to update the following fast fading parameters for 6-24 GHz considering measurement results in Table 1: - Delay spread (mean, variance) - AoD spread (mean, variance) - AoA spread (mean, variance) - Number of clusters Figure 13 Intra-cluster power ratio for UMi scenario Observation 6: The median intra-cluster power ratios are -1.1 dB and -2.5 dB for UMi LOS and NLOS scenarios, respectively, which means the power of the strongest ray in a cluster is not as dominant as the LOS path. |
| [2] Ericsson | Proposal 14 Encourage companies to perform measurements to further study whether the existing mechanisms for generating clusters and rays are inaccurate when simulating large antenna arrays. Figure 16 Number of resolved paths as a function of array size in measurements (solid lines) and for UMa-generated channels (dashed lines). 100 MHz bandwidth was used for measurements and UMa simulations alike. Observation 14 More paths can be detected in measurements than in comparable channels generated by the UMa model. Proposal 15 While measurements support increasing the number of clusters in the UMa channel, for complexity reasons it may be better to keep this number unchanged. |
| [5] vivo, BUPT | Proposal 3: RAN1 studies the impact of channel sparsity on the existing channel model based on the experiment result. |
| [6] ZTE, Sanechips | Figure 9. Power Angular Spectrum (PAS) of cluster Figure 10. Modeling method of MED (left) and MEA (right) for rays in a cluster Observation 9: If unequal power is assigned to the rays in a cluster, the angular spacing also has to be adjusted to maintain proper PAS characteristics. Observation 10: To model the Laplace distribution of PAS characteristic of a cluster, unequal power rays with equal angle offset (MED) is equivalent to equal power rays with unequal angle offset (MEA). - Step 1: Change equal power to unequal power per ray Specifically, the ray power component in the NLOS channel coefficient can be adjusted, i.e., , wherein , and βm is the ray power ratio given by Table 5 Hu,s,n,mNLOSt=Pn,mFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vλ0t Table 5: Ray power ratio within a cluster Ray number m; Ray power ratio βm. 1,2; 0.1193. 3,4; 0.0933. 5,6; 0.0729. 7,8; 0.0570. 9,10; 0.0446. 11,12; 0.0348. 13,14; 0.0272. 15,16; 0.0213. 17,18; 0.0166. 19,20; 0.0130. - Step 2: Change the unequal angle offset to equal angle offset Add offset angles m from Table 6 to the cluster angles , (7.5-13) where cASA is the cluster-wise rms azimuth spread of arrival angles (cluster ASA) in Table 7.5-6. Table 6: Ray offset angles within a cluster, given for rms angle spread normalized to 1 Ray number m; Basis vector of offset angles m. 1,2; ± 0.1134. 3,4; ± 0.3403. 5,6; ± 0.5671. 7,8; ± 0.7940. 9,10; ± 1.0208. 11,12; ± 1.2477. 13,14; ± 1.4745. 15,16; ± 1.7014. 17,18; ± 1.9282. 19,20; ± 2.1551. Observation 11: Whether to modify the number of clusters should be based on the aligned measurement methodology and condition. Table 3. Number of clusters with different offset Scenario; Indoor-office; UMi; UMa; RMa. LoS; NLoS; LoS; NLoS; LoS; NLoS; LoS; NLoS. Number of clusters（38.901）; 15; 19; 12; 19; 12; 20; 11; 10. Number of clusters (Simulated, Power offset, 30dB）; 9.86; 18.16; 9.16; 18.90; 10.03; 19.80; 8.21; 10. Number of clusters (Simulated, Power offset, 25dB）; 7.57; 17.47; 7.19; 18.74; 7.90; 19.51; 6.81; 9.98. Number of clusters (Simulated, Power offset, 20dB）; 5.14; 16.30; 4.63; 18.25; 4.64; 18.78; 4.86; 9.94. Number of clusters (Proposed by companies); 10; 11; 5; 7; None. Observation 12: The different number of clusters for LoS UE can be achieved by assigning different power offsets in the removal of paths, but the number of clusters for NLoS UE may need further study. Figure 11. Number of clusters after -25dB removal for UMi LOS UE Observation 13: If reduced number of clusters is adopted for TR 38.901, after removal of -25dB offset, there will be very few or even no NLOS clusters remained in simulation. |
| [9] Apple | Observation 6: For UMi NLOS scenario at 8 GHz, the number of clusters is 18 at 95% percentile. Figure 9: CDF of the number of clusters for UMi NLOS |
| [12] BUPT | (a) Gini index; (b) Capacity. Figure 6: The Gini index and capacity of measurements and simulations. The simulations are conducted in the LOS condition of UMa scenario at 13 GHz utilizing a 128-array element TX antenna and a 64-array element RX antenna. Observation 1: The measured cluster structure does not match that of TR38901. The cluster structure has an impact on system performance, such as channel capacity and sparsity. The performance of the system with the improved cluster structure better matches the measured channel. Proposal 1: The modification of the cluster structure based on the measurements should be considered to make the channel model more accurate. (a) (b) Figure 8. The CDF of Gini index generated by measured channels, by the 3GPP channel model, and by the 3GPP model with proposed intra-cluster power allocation model (modified model). (a) LoS scenario. (b) NLoS scenario. Observation 2: The ICP model has been proved to be applicable to indoor [2] and UMa scenarios. When ICP is introduced into the 3GPP model, the level of sparsity is closer to the measured results. Proposal 2: The ICP model can be introduced as an optional method of changing cluster structure to reflect channel sparsity. ICP=b∙10-a∙τnDS Table 3: Coefficients of the ICP model Scenarios; a; b. LOS; 0.04; 2.8. NLOS; 0.04; 1.7. |
| [13] Rohde & Schwarz | Figure 11: Estimated number of paths for 14 GHz Table 3: Estimated Channel Parameters Parameter; 14 GHz. -; min; max. PL exp. (n) w/o unit; 1.72. RMS DS in ns; -; 2.61; 41.74. RMS AS in degree; -; 7.00; 100.76. K-factor in dB; -; 0.08; 14.41. number of paths; -; 7; 250. |
| [16] Sharp, Nokia | Observation 11: RAN1 should consider the mean number of clusters observed in UMi LOS and NLOS channel conditions at 28 GHz and 73 GHz. - UMi LOS/NLOS number of clusters - LOS @ 28 GHz + 73 GHz, 1 GHz measurement BW: 12 ⇒ 5 - NLOS @ 28 GHz, 1 GHz measurement BW: 19 ⇒ 5 - NLOS @ 73 GHz, 1 GHz measurement BW: 19 ⇒ 5 Table 10. Number of clusters based on measurements conducted at 28 GHz and 73 GHz in both LOS and NLOS channel conditions for the UMi scenario [14]. Scenario; Urban Microcell (UMi). LOS; NLOS. Combined 27-73 GHz; 28GHz; 73 GHz. Clustering; KPowerMeans. # of clusters (µ, σ); 5.0, 3.5; 5.3, 2.4; 4.6, 3.3. Proposal 8: RAN1 should consider the number of clusters for different scenarios, channel conditions, and frequencies other than the 7-24 GHz range before deciding whether to reduce the number of clusters in the existing models in TR 38.901. |
| [19] Qualcomm | Proposal 9: Due to the large impact of any changes to the ray-cluster framework and impact to frequency continuity of the channel model, RAN1 strives to retain the existing ray-cluster framework. |

#### Summary of Issues

Companies observed lower cluster numbers in the frequency ranges of interest for following scenarios:

- InH LOS/NLOS (Huawei, Keysight, BUPT)

  - LOS: 15 ⇒ 10, 6, 13

  - NLOS: 19 ⇒ 11

- UMi LOS/NLOS (Huawei, Keysight, BUPT)

  - LOS: 12 ⇒ 5, 8, 11

  - NLOS: 19 ⇒ 7, 10, 12

- UMa LOS/NLOS (BUPT)

  - LOS: 12 ⇒ 9

  - NLOS: 20 ⇒ 14

| Scenario | LOS/LOS/NLOS | Different than TR | Comparative to TR | Other Observations | Moderator Notes |
| --- | --- | --- | --- | --- | --- |
| InH | LOS | Huawei/HiSilicon |  |  | Single source results, need further discussion |
|  | NLOS | Huawei/HiSilicon |  |  | Single source results, need further discussion |
| UMi | LOS | Sharp/Nokia, Huawei/HiSilicon |  |  | Two sources showed aligned observations. |
|  | NLOS | Huawei/HiSilicon, Sharp/Nokia | Apple |  | Inconclusive |
| UMa | LOS | BUPT | Huawei/HiSilicon | Ericsson1) | Inconclusive |
|  | NLOS | Huawei/HiSilicon, BUPT |  | Ericsson1) | Inconclusive |
| InF | LOS |  |  |  | No data |
|  | NLOS |  |  |  | No data |
| Note 1) Ericsson notes larger number of clusters with large BW and antenna arrays but suggest to keep the same number |  |  |  |  |  |

On the issue of intra-cluster power profiles, ZTE noted that application of intra-cluster power difference can result in overall changes to the PAS characteristics, and therefore, introduction of intra-cluster power profile should be updates along with PAS distribution, i.e. angular offset values within a cluster. BUPT shows application of intra-cluster power profiles will allow the measurements to be more aligned with models.

One company commented that changes to -25dB threshold to remove clusters could be one method to achieve lower cluster density for frequency of interest. One company commented the -25dB threshold to remove the lower powered clusters in the channel model may not be sufficient to replicate the reduction in clusters for frequencies of interest.

##### Proposal #2.5-1:

- For the following scenarios, while sources have observed some differences in number of clusters in the TR and measurement taken for 6 – 24 GHz frequency range, RAN1 concludes that there is no census to update number of clusters parameters due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

Proposal #2.5-2:

- For the following scenarios, further study number of clusters for 6 – 24 GHz frequency range.

  - InH LOS and NLOS

  - InF LOS and NLOS

##### Proposal #2.5-3:

- Study further on the need for introduction of intra-cluster power profile.

#### Round #1 Discussion

Please provide comments on issues regarding cluster modeling. Please provide comments on Proposal #2.5-1, #2.5-2, #2.5-3.

| Company | Comments |
| --- | --- |
| Apple | Fine with Proposal #2.5-1. No need of Proposal #2.5-3. |
| vivo | Support |
| Huawei, HiSilicon | Proposal #2.5-1: Prefer to postpone. @FL please capture our position for UMi LOS as “different than TR”. Proposal #2.5-2: Fine. Proposal #2.5-3: OK. |
| QC | Proposal 2.5-1: Seems to have a typo (referring to angular spread parameters). If the intention is to retain the number of clusters, then we are okay. Proposal 2.5-2: Okay Proposal 2.5-3: Okay, but can we narrow down this choice between the two options outlined by ZTE? |
| ZTE | For Proposal #2.5-1, ‘angular spread parameters’ should be updated to ‘number of clusters’. |
| Ericsson | #2.5-1: Support, but it should say number of clusters in the main bullet #2.5-2: Ok, though with the comment that the SID requires consistency below and above 6-24 GHz, hence the study needs to include such frequencies as well #2.5-3: No preference |
| Moderator | Corrected the typo in #2.5-1. |

#### Summary of Round #1 Discussion

Proposal #2.5-1: Looks like moderator has made a mistake in classification of observations for UMi LOS case. Update was made.

Proposal #2.5-2: no negative comment so far.

Proposal #2.5-3: suggestion to add the examples of options.

Moderator added proposal #2.4-4 in light of updated observation from Huawei.

##### Proposal #2.5-1A:

- For the following scenarios, while some sources have observed some differences in number of clusters in the TR and measurement taken for 6 – 24 GHz frequency range, other sources have observed number of clusters consistent with the model in the TR. RAN1 concludes that there is no consensus to update number of clusters parameters due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

##### Proposal #2.5-3A:

- Study further on the need for introduction of intra-cluster power profile.

  - Potential options for introduction of intra-cluster power profile:

    - Per ray power scaling sqrt(Pn/M) to sqrt(Pn,m) and scaling of ray offset angles without a cluster to enable unequal power rays (example in R1-2409724)

    - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

##### Proposal #2.5-3B:

- Study further on the need for introduction of intra-cluster power profile.

  - Potential options for introduction of intra-cluster power profile:

    - Per ray power scaling sqrt(Pn/M) to sqrt(Pn,m) and scaling of ray offset angles without a cluster to enable unequal power rays (example in R1-2409724)

    - Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724)

    - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

Proposal #2.5-2:

- For the following scenarios, further study number of clusters for 6 – 24 GHz frequency range.

  - InH LOS and NLOS

  - InF LOS and NLOS

Proposal #2.5-4:

- For the following scenarios, RAN1 concludes there are necessary changes required to align number of cluster measurements to the channel model for 6 – 24 GHz frequency range. Details of the actual change needed is FFS.

  - UMi LOS

#### Round #2/3 Discussion

Moderator has updated the proposal based on inputs. Please provide further comments on #2.5-1A, #2.5-2, #2.5-3A, and #2.5-4.

| Company | Comments |
| --- | --- |
| BUPT | Proposal #2.5-1A: Prefer to further study the number of clusters for UMa/UMi scenarios. Proposal #2.5-2: Okay Proposal #2.5-3A: Generally fine. The options need to be further validated. Proposal #2.4-4: Okay |
| Moderator | ##### It is asked for companies who wish to not conclude no consensus for change |
| AT&T | ##### Proposal #2.5-2: support |
| Apple | OK |
| ZTE | For Proposal #2.5-3A: To better capture the required change in our contribution, the first option can be updated as: Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724) For Proposal #2.5-4: The decision for reduction of number of clusters is based on the measurement in 6~24GHz, if the number of clusters is change for this frequency range, it means that the reduced value is also applied for other frequency ranges which is not justified, since the modelling of cluster number in TR 38.901 is not frequency dependent. |
| Sharp | Agree with ZTEs comment. The change in number of clusters should not be studied and done in isolation based on just 6-24 GHz measurements. The change in number of clusters applies to 0.5-100 GHz and thus if a change is made it should be applicable for the entire frequency range. In 2.5-2 and 2.5-4 we should add a note specifying that companies interested in changing number of clusters should also provide results beyond 6-24 GHz. Note: RAN1 should consider the number of clusters for frequencies other than the 6-24 GHz range before deciding whether to reduce the number of clusters in the existing models in TR 38.901. |
| Ericsson | Agree with ZTE about the issue with only considering 6-24 GHz in these proposals. As we suggest for other proposals above, the text “for 6 – 24 GHz frequency range” should be removed. Proposal #2.5-1A: Support Proposal #2.5-2: Ok Proposal #2.5-3A: Ok Proposal #2.5-4: We are concerned that such a change will have consequences on implementation without bringing any benefits. The necessity aspect of this proposal has not been discussed yet. Therefore, we prefer to add UMi LOS in proposal #2.5-2 that says further study. Maybe with an explicit “FFS on necessity”. |
| Nokia | We do not support Proposal #2.5-4. In the existing models, the number of clusters is the same across all the frequencies from 0.5 to 100. Even though the number of clusters may deffer for lower frequencies and for higher frequencies, that how the model is introduced. It is not clear to us why we cannot continue following this approach. Otherwise, changes might be needed for each of sub-bands. |

#### Outcome of Wed Session

Proposal #2.5-2B was agreed.

Agreement

- Study further on the need for introduction of intra-cluster power profile.

  - Potential options for introduction of intra-cluster power profile:

    - Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724)

    - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

#### Summary of Round #3 Discussion

Moderator has updated the proposal #2.5-1 and #2.4-2 based on conclusions made on Wednesday session for another parameter (delay spread).

##### Proposal #2.5-1A – not pursued:

Suggested Observation

- For the following scenarios, some sources have observed some differences in number of clusters in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed number of clusters consistent with the model in the TR.

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

Suggested Conclusion

- For the following scenarios, there is no consensus to update number of clusters due to lack of consistent and significant observed difference between model and measurements.

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

##### Proposal #2.5-2:

- For the following scenarios, further study number of clusters for 6 – 24 GHz frequency range.

  - InH LOS and NLOS

  - InF LOS and NLOS

##### Proposal #2.5-2A:

- For the following scenarios, further study number of clusters for 6 – 24 GHz frequency range.

  - InH LOS and NLOS

  - InF LOS and NLOS

  - UMi LOS

##### Proposal #2.5-4 – not pursued:

- For the following scenarios, RAN1 concludes there are necessary changes required to align number of cluster measurements to the channel model for 6 – 24 GHz frequency range. Details of the actual change needed is FFS.

  - UMi LOS

##### Proposal #2.5-2B:

- Further study number of clusters for existing deployment scenarios (InH, InF, UMi, UMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6GHz and above 24 GHz, including whether to update values for other frequencies based on measurements

  - Whether changes to number of clusters result in meaningful impact to system performances and evaluation of features

##### Proposal #2.5-2C:

- Further study number of clusters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6GHz and above 24 GHz, including whether to update values for other frequencies based on measurements

  - Whether changes to number of clusters result in meaningful impact to system performances and evaluation of features

#### Round #4 Discussion

Please provide comments on Proposal #2.5-1A and #2.5-2A.

| Company | Comments |
| --- | --- |
| Moderator | Proposal #2.5-4 is not pursued in this meeting. This will be revisited in the next meeting. |
| AT&T | Proposal 2.5-1A: conclusion: prefer to postpone to next meeting where we might bring our own measurement results and companies will have time to check consistency in the number of clusters definition and whether changing the number of clusters makes a difference in SLS simulations Proposla 2.5-2A: support |
| Moderator | If Proposal #2.5-1A is not ready be acceptable as conclusion by companies and there are companies willing to bring in more data try to convince other companies in the meeting, suggest to not pursue for this meeting. Updated Proposal #2.5-2A to #2.5-2B. Moderator has added consideration aspect based on feedback received. |
| Sharp | Generally Fine. Just a minor clarification – are we implicitly concluding that RMa doesn’t need to be studied. If not maybe for the sake of completeness we just mention RMa in existing deployment scenarios. That being said we are also okay to conclude that InF and RMa do not need any changes for number of clusters. |

#### Outcome of Thursday Session

Agreement

- Further study number of clusters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6GHz and above 24 GHz, including whether to update values for other frequencies based on measurements

  - Whether changes to number of clusters result in meaningful impact to system performances and evaluation of features

Conclusion

- Based on measurement data provided, RAN1 concludes shadow fading parameters in TR38.901for the following scenarios are validated and no updates to TR are made.

  - InH LOS

  - InH NLOS

  - UMi NLOS

### 4.2.6 LOS Probability

| Company | Proposals & Observations |
| --- | --- |
| [4] Samsung | Observation 7 Even when a visual LOS condition exists between the Tx (BS) and the Rx (UE), most paths can be obstructed by vegetation, resulting in a significant decrease in path loss compared to an environment with pure LOS Proposal 5 RAN1 discuss whether/how to handle the vegetation impact |
| [5] vivo, BUPT | Observation 1: The pathloss gap between the measurement and the empirical formula is within the max range of 5dB under the LOS conditions in indoor scenario. |

#### Summary of Issues

Note conclusion from RAN1#118-bis:

Conclusion - RAN1 concludes LOS probability is validated for existing deployment scenarios defined in TR38.901. - Note: this does not preclude introduction of new LOS probability for new scenarios of interest and/or different BS height to be supported for new/existing scenarios.

Samsung has noted that vegetation plays a significant role in determination of LOS condition and suggested further study of vegetation to LOS. Given that RAN1 concluded in the last meeting to not update the LOS probability for existing scenarios, the study of vegetations should be done for suburban macrocell, and other new scenarios not captured in the TR.

##### Proposal #2.6-1:

- Further study impact of vegetation for LOS probability for new scenarios considered.

#### Round #1/2 Discussion

Please provide comments on issues regarding LOS probability. Please provide comments on Proposal #2.6-1.

| Company | Comments |
| --- | --- |
| vivo | Support |
| Huawei, HiSilicon | OK. |
| ZTE | OK |
| Ericsson | Support. In our contribution R1-2409441 we observed the strong dependence of the vegetation on the LOS probability in a SMa scenario. |
| Moderator | Suggest to continue discussion and feedback in this sub-section. |
| AT&T | While we sympathize with the effect of vegetation – we also need a proposal on how to include the effect of vegetation in the channel model. Will there be different LoS expressions based on whether vegetation is considered or not. |
| Ericsson | AT&T makes a good point. One attractive approach is to reuse the LOS probability functions from the InF scenario which were derived analytically assuming a certain clutter size/height and density. If this height and density is adjusted to the suburban case a suitable LOS probability function can be determined. - Further study impact of vegetation for LOS probability for new scenarios considered. FFS: Reuse the InF LOS probability functions but with clutter size/height and density values tuned to the suburban scenario |

#### Summary of Round #3 Discussion

Updated the proposal based on discussion from Wednesday session.

##### Proposal #2.6-1A:

- Further study impact of vegetation for LOS probability for new scenarios considered. The study included whether or not vegetation should be explicitly considered and modeled for determining LOS probability, if determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

##### Proposal #2.6-1B:

- Further study impact of vegetation for LOS probability for new scenarios considered. The study included whether or not vegetation should be explicitly considered and modeled for determining LOS probability

  - If determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

#### Round #4 Discussion

Please comment on Proposal #2.6-1A.

| Company | Comments |
| --- | --- |
| AT&T | support |
| Sharp | Support. Minor correction. For better readability and clarity, we suggest the following proposal: - Further study the impact of vegetation on LOS probability for the new scenario. The study includes whether or not vegetation should be explicitly modeled for determining LOS probability - If vegetation is determined to be explicitly modeled, how to model the impact of vegetation, e.g. reuse the concept of InF clutter to change the LOS probability. |

#### Outcome of Thursday Session

### 4.2.7 Polarization

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Observation 5: Considering the impact of inter-polarization power imbalance can be effectively mitigated by X-pol BS antenna configuration, the motivation of modelling polarization power variability is not clear. |
| [2] Ericsson | Observation 11 In the TR 38.901 model, the two co-polar components in the channel always have exactly equal power, and the two cross-polar components are equally attenuated according to a stochastic XPR. Figure 8 Power delay profiles from a 200 MHz bandwidth macrocell outdoor measurement at 5.25 GHz. From [4]. Figure 9 Co- and cross-polar powers as a function of total received power (here given by pVV). Error bars denote +/- one standard deviation per 5 dB pVV interval. Results are from all indoor measurements. From [4]. Figure 12 Angular resolved scans for position 2, 3, and 8. Figure 13: Power of the 8 strongest peaks for the NLOS locations and the 4 strongest peaks for the LOS position. Observation 12 Measurements and ray tracing experiments show a slow variability around the mean co-polar and cross-polar power that is independent between different components. Observation 13 The potential rank reduction of the channel due to polarization power variability is independent on whether the Tx or Rx uses V/H-polarized or ±45° polarized antennas. It manifests as a power imbalance in the former case and a correlation between fading in the latter case. Proposal 13 Introduce a random variability of the co- and cross polar powers in the TR 38.901 model, such as an i.i.d zero-mean Gaussian with 2-3 dB standard deviation, via the following changes to step 9 and eqs (7.5-22) and (7.5-28) in clause 7.5 in TR 38.901. -- Step 9: Generate the cross polarization power ratios Generate the cross polarization power ratios (XPR) for each ray m of each cluster n. XPR is log-Normal distributed. Draw XPR values as , (7.5-21) where is Gaussian distributed with and from Table 7.5-6. Note: is independently drawn for each ray and each cluster. Generate polarization variability powers ηθθ, ηθϕ, ηϕθ and ηϕϕ for each ray m of each cluster n. η is log-Normal distributed. Draw values as ηn,m,θθ,θϕ,ϕθ,ϕϕ=10Qn,m,θθ,θϕ,ϕθ,ϕϕ/10, (7.5-21b) where Qn,m,θθ,θϕ,ϕθ,ϕϕ~N(0,32) is Gaussian distributed. Note that Qn,m,θθ,θϕ,ϕθ,ϕϕ is independently drawn for each ray, cluster, and polarization component. -- Hu,s,nNLOSt=PnMm=1MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATηn.m.θθexpjΦn,mθθηn,m,θϕκn,m-1expjΦn,mθϕηn,m,ϕθκn,m-1expjΦn,mϕθηn,m,ϕϕexpjΦn,mϕϕ Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT⋅drx,uλ0expj2πrtx,n,mT⋅dtx,sλ0expj2πrrx,n,mT⋅vλ0t (7.5-22) -- Hu,s,n,mNLOS(t)=PnMFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATηn.m.θθexpjΦn,mθθηn,m,θϕκn,m-1expjΦn,mθφηn,m,ϕθκn,m-1expjΦn,mφθηn,m,ϕϕexpjΦn,mφφ Ftx,s,θθn,m,ZOD,φn,m,AODFtx,s,φθn,m,ZOD,φn,m,AODexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,sλ0expj2πrrx,n,mT.vλ0t (7.5-28) -- |
| [3] Interdigital | Observation 4: In the conducted measurement [6], the receive antennas were installed on a moving van having a fixed polarization. However, there is no fixed positioning of a UE in practice. Proposal 4: RAN1 deprioritizes the study of random power variability in each polarization. |
| [6] ZTE, Sanechips | Observation 4: The power ratio for co-polarization and cross-polarization is influenced by the depolarization effects of the environment and the antenna assumptions. Observation 5: Under ±45° assumption, the ratios of cross-polarization received power to co-polarization received power have a consistent mean value. For co-polarization, the power for -45° co-polarization and the power for +45° co-polarization can be assumed the same as existing TR 38.901. Proposal 7: Based on ±45° polarization slant assumption, no need to change the polarization matrix in the channel realization formula in TR 38.901. |
| [19] Qualcomm | Figure 10 CDF of the difference in received power across polarization Observation 2: In FR3, at a centre frequency of 13 GHz, a difference in received power across polarizations is observed in an outdoor UMi-like scenario, where it is seen that median difference is about 1 dB, with the 90th percentile difference around 3 dB. Figure 11 Polarization imbalance using the Ground Reflection model. Observation 3: Ground reflection model in 38.901 offers a mode to realize polarization power imbalance in the channel realizations. Whether additional other factors such as specular reflections cause polarization power imbalance needs more study. Proposal 8: Study whether the ground reflection model in 38.901 is adequate to model the observed polarization power imbalance before introducing any explicit variations directly in the polarization matrix of the channel. |

#### Summary of Issues

Companies provided inputs on variability of polarization power in the frequency ranges of interest:

- Introduce variability of polarization power matrix

- Do not introduce changes to polarization power matrix

- Study further if ground reflection model can provide the effective behavior for polarization power variability

##### Proposal #2.7-1 – not pursued:

- Conclude that there is no consensus to introduce polarization variability for each rays of cluster for NLOS component of the channel.

#### Round #1/2/3/4 Discussion

Please provide comments on issues regarding polarization modeling. Please provide further comments on Proposal #2.7-1.

| Company | Comments |
| --- | --- |
| InterDigital | Support the conclusion |
| vivo | Support |
| Huawei, HiSilicon | Support. |
| LGE | Support |
| CATT | OK |
| ZTE | Support |
| Ericsson | The existence of polarization variability is demonstrated by several companies, while there are no measurements showing the non-existence. We agree that there is no consensus at this point in time, but this is mainly due to different views of the necessity of changing the model. We suggest spending some more time to see if companies can reach a common understanding on the necessity. |
| Moderator | Continue discussion using this sub-section. |
| Nokia | OK |
| Sharp | Support |
| Moderator | Moderator suspect discussions on this and agreement on the conclusion to be difficult. Suggest to revisit the proposal in the next meeting. |

#### Summary of Round #4 Discussion

Moderato suggest discussing this issue in the next meeting. More offline discussion is needed, which the group did not have time for.

### 4.2.8 Shadow Fading

| Company | Proposals & Observations |
| --- | --- |
| [16] Sharp, Nokia | Table 7. Comparison of Large-Scale Shadow Fading Values: Measured [13] vs. TR 38.901 for UMi Scenario in both LOS and NLOS Channel Conditions at 6.75 GHz and 16.95 GHz using the CI Path Loss Model [13]. Frequency; Env.; Measured shadow fading; TR 38.901 shadow fading; Absolute difference in shadow fading (Measured shadow fading - TR 38.901) using the CI path loss model. 6.75 GHz; LOS; 2.56; 4.60; 2.04. NLOS; 6.52; 8.20; 1.68. 16.95 GHz; LOS; 4.03; 4; 0.03. NLOS; 8.76; 8.20; 0.56. Observation 8: Based on measurements conducted at 6.75 GHz and 16.95 GHz, the shadow fading values mostly align with those specified in TR 38.901 for the UMi – Street Canyon scenario. The discrepancies in shadow fading values are less than 2 dB for both LOS and NLOS channel conditions and can be considered negligible. Proposal 6: Shadow fading can remain frequency-independent, as defined in TR 38.901 for UMi – Street Canyon scenario in both LOS and NLOS channel conditions. Hence, there is no need to update shadow fading values in TR 38.901 for 7-24 GHz specifically. |
| [17] AT&T | Figure 16: Distribution of shadow fading in InH LOS and NLOS environments for 7, 8 and 15 GHz. Observation 10: Measurements conducted at 7, 8, 15 GHz over 650 RX locations on floors of an office building show that shadow fading distribution for InH LoS and NLoS environments agree with the previously proposed the 3GPP SCM InH channel model. Proposal 4: There is no need to change the distribution of the InH shadow fading for the 3GPP InH channel model in TR 38.901 for extension to 7-24GHz |
| [19] Qualcomm | Proposal 10: RAN1 to study how any potential change to the marginal distribution of angular spreads, K-factor and shadow fading impacts their joint distribution and consequently their pairwise correlation. |

#### Summary of Issues

Companies observed aligned shadow fading related parameters in the frequency ranges of interest for following scenarios:

- InH LOS (Sharp, AT&T, Keysight)

- InH NLOS (Sharp, AT&T)

- UMi NLOS/NLOS (Sharp)

- UMi NLOS (Sharp, Keysight)

Companies observed changes to shadow fading related parameters in the frequency ranges of interest for following scenarios:

- InH NLOS SF (Keysight-previous meeting)

  - 4 dB ⇒ 3 dB

- UMi LOS SF (Keysight-previous meeting)

  - 1.5 dB ⇒ 2.7 dB

##### Proposal #2.8-1:

- Based on measurement data provided, RAN1 concludes that following shadow fading parameters for 6 – 24 GHz frequency range are validated and no updates to TR are made.

  - InH LOS

  - InH NLOS

  - UMi NLOS

##### Proposal #2.8-2:

- For the following scenarios, while sources have observed some differences in shadow fading parameters in the TR and measurement taken for 6 – 24 GHz frequency range, RAN1 concludes that there is no census to update shadow fading parameters due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

##### Proposal #2.8-2A:

- For the following scenarios, while some sources have observed some differences in shadow fading parameters in the TR and measurement taken for 6 – 24 GHz frequency range, other sources have observed shadow fading consistent with the model in the TR. RAN1 therefore concludes that there is no consensus to update shadow fading parameters due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

#### Round #1/2/3 Discussion

Please provide comments on issues regarding shadow fading modeling. Please provide further comments on proposal #2.8-1, #2.8-2.

| Company | Comments |
| --- | --- |
| vivo | Support |
| Huawei, HiSilicon | Fine. |
| LGE | Fine |
| CATT | OK |
| ZTE | Fine |
| AT&T | Proposal #2.8-1: support |
| MediaTek | Fine |
| Ericsson | #2.8-1 Support #2.8-2 Support, though with same comment on the main bullet as above |
| Moderator | Suggest to continue discussion using this sub-section. |

#### Summary of Round #3 Discussion

Proposal #2.8-1 and #2.8-2A is updated based on the conclusion made on Wednesday for another parameter (delay spread).

##### Proposal #2.8-1A:

- Based on measurement data provided, RAN1 concludes shadow fading parameters in TR38.901for the following scenarios are validated and no updates to TR are made.

  - InH LOS

  - InH NLOS

  - UMi NLOS

##### Proposal #2.8-2B:

Suggested Observation

- For the following scenarios, some sources have observed some differences in shadow fading in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed shadow fading with the model in the TR.

  - UMi LOS

Suggesed Conclusion

- For the following scenarios, there is no consensus to update shadow fading due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

#### Round #4 Discussion

Please comment on Proposal #2.8-1A and #2.8-B

| Company | Comments |
| --- | --- |
| AT&T | Proposal 2.8-1A: support |
| Sharp | Support 2.8-1A and 2.8-2B. |

#### Outcome of Thursday Session

Observation

- For the following scenarios, some sources have observed some differences in shadow fading in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed shadow fading with the model in the TR.

  - UMi LOS

Conclusion

- For the following scenarios, there is no consensus to update shadow fading due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

### 4.2.9 K-Factor

| Company | Proposals & Observations |
| --- | --- |
| [13] Rohde & Swartz | Figure 12: Estimated K-factor for 14 GHz, values for first 3 TPs at 160 are above 30 dB Parameter; 14 GHz. -; min; max. PL exp. (n) w/o unit; 1.72. RMS DS in ns; -; 2.61; 41.74. RMS AS in degree; -; 7.00; 100.76. K-factor in dB; -; 0.08; 14.41. number of paths; -; 7; 250. Table 3: Estimated Channel Parameters |
| [19] Qualcomm | Proposal 10: RAN1 to study how any potential change to the marginal distribution of angular spreads, K-factor and shadow fading impacts their joint distribution and consequently their pairwise correlation. |

#### Summary of Issues

Companies observed changes to K-Factor parameters in the frequency ranges of interest for following scenarios:

- UMi LOS KF (Keysight-previous meeting, Southeast Univ.)

  - Mean 9, stddev 5 ⇒ Mean 5.1, stddev 3.2

  - Mean 9, stddev 5 ⇒ @7GHz: Mean 3.52, stddev 5.15, @10GHz: Mean 4.41, stddev 4.8, @13GHz: Mean 5.18, stddev 4.7

- InH LOS KF (Keysight-previous meeting)

  - Mean 7, stddev 4 ⇒ Mean 8.5, stddev 3.5

Rohde and Swartz provided data on K-factor, where the spread was between 0.08 dB to 14.41 dB. It was not clear if these results are aligned or not-aligned with the TR.

Given that results are from a single source, it is difficult to conclude on the results without further discussion.

##### Proposal #2.9-1:

- Study further on need for update of K-factor parameters.

##### Proposal #2.9-1A:

- Further study the need for updating the K-factor parameters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6 GHz and above 24 GHz, including whether to update values for other frequencies based on measurements.

  - Whether changes to K-Factor result in meaningful impact to system performances and evaluation of features.

#### Round #1/2/3/4 Discussion

Please provide comments on issues regarding K-factor. Please provide comments on Proposal #2.9-1.

| Company | Comments |
| --- | --- |
| vivo | Support |
| Huawei, HiSilicon | Fine. |
| ZTE | Fine |
| Moderator | Continue discussion in this sub-section. |
| Sharp | Support |
| Ericsson | Note: Due to the need for continuity in frequency, also bands outside 7-24 GHz should be included in the study |
| Sharp | Agree 100% with Ericsson’s comments. Suggest taking a similar approach as we did for number of clusters. - Further study the need for updating the K-factor parameters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following: - Consistency/continuity across frequencies below 6 GHz and above 24 GHz, including whether to update values for other frequencies based on measurements. - Whether changes to K-Factor result in meaningful impact to system performances and evaluation of features. |

#### Outcome of Thursday Session

Agreement

- Further study the need for updating the K-factor parameters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6 GHz and above 24 GHz, including whether to update values for other frequencies based on measurements.

  - Whether changes to K-Factor result in meaningful impact to system performances and evaluation of features.

### 4.2.10 TRP Delay

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Table 4 Parameters for the absolute time of arrival model Scenarios; InH; UMi; UMa. lgΔτ=log10Δτ1s; μlgΔτ; -7.9; -7.5; -7.4. σlgΔτ; 0.3; 0.5; 0.2. Correlation distance in the horizontal plane [m]; 10; 15; 50. Proposal 6: Introduce the absolute delay for InH, UMi and UMa scenarios according to Table 4. |
| [2] Ericsson | Figure 17 Difference between the propagation distance of the first path in the impulse response and the geometric distance as estimated using GPS, for the three different UMa sites. For comparison, a Gaussian distribution with m = 15 m and s = 15 m is also shown. From [16], Fig 7. Figure 18 Three models for the excess delay times the speed of light. The dark blue line corresponds to the example parameters in the agreement from RAN1#118bis. The red line corresponds to the Gaussian model from [16]. The light blue line corresponds to an approximate lognormal fit to the data in Figure 17. Observation 15 The example parameters for UMa in the agreement from RAN1#118bis on the absolute time of arrival model are not supported by measurements. Observation 16 The parameters μ_lgΔτ=-7.4, σ_lgΔτ=0.2 provides an excellent match to measurements of the absolute time of arrival in a UMa scenario. Proposal 16 In the UMa scenario, model absolute time of arrival with the parameters μ_lgΔτ=-7.4, σ_lgΔτ=0.2. |
| [6] ZTE, Sanechips | Proposal 5: Introduce absolute delay for InH-Office scenario by re-using the absolute time of arrival for InF scenarios. Proposal 6: Table 7.6.9-1 in TR 38.901 can be revised for UMi, UMa and InH scenarios as follows: Table 7.6.9-1: Parameters for the absolute time of arrival model Scenarios; InF-SL, InF-DL; InF-SH, InF-DH; UMi; UMa; InH-Office. lgΔτ=log10Δτ1s; μlgΔτ; -7.5; -7.5; -7; -6.8; -8.6. σlgΔτ; 0.4; 0.4; 0.4; 0.6; 0.1. Correlation distance in the horizontal plane [m]; 6; 11; 15; 50; 10. |

#### Summary of Issues

Companies showed good alignment on the time of arrival model parameters.

##### Proposal #2.10-1:

- Adopt the following absolute delay for UMi and UMa scenarios.

- Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

##### Proposal #2.10-2:

- Introduce absolute delay for InH scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - Down-selection among option 1 and 2.

- Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

#### Round #1 Discussion

Please provide comments on issues regarding other channel modelling aspects. Please provide comments on Proposal #2.10-1 and #2.10-2.

| Company | Comments |
| --- | --- |
| vivo | Support |
| Moderator | Just to clarify, it would be ideal to try to down-select among options. |
| Huawei, HiSilicon | Proposal #2.10-1: Support. Proposal #2.10-2: Generally fine. But rather than down-selection, other handle can be considered. |
| ZTE | We understand that current values are based on measurement, it would be better that companies can clarify the methodology and set-up that is used in the measurements. |
| Ericsson | Proposal #2.10-1: Support the values for UMa. For UMi, the median value seems reasonable but the standard deviation results in a very long tail in the distribution, see the figure below. It may be prudent to either truncate the lognormal distribution or reduce the sigma value for UMi. |

#### Summary of Round #1 Discussion

Limited number of companies have provided comments. ZTE wanted to obtain more information on the proposal.

##### Proposal #2.10-1A:

Suggested Working Assumption

- Adopt the following absolute delay for UMi and UMa scenarios.

- Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

##### Proposal #2.10-2A:

- Introduce absolute delay for InH scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - FFS details of the parameter for InH. Consider the following for further study.

  - Down-selection among option 1 and 2.

- Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

#### Round #2/3/4 Discussion

Continue provide comments on Proposal #2.10-1. Moderator asks Huawei and Ericsson to provide additional information that ZTE is requesting for.

| Company | Comments |
| --- | --- |
| Ericsson | ##### The UMa measurements are reported in [16] and summarized in our contribution R1-2409441. We are happy to answer additional questions beyond what is reported in the conference paper and the 4contribution. [16] J. Medbo, I. Siomina, A. Kangas and J. Furuskog, "Propagation channel impact on LTE positioning accuracy: A study based on real measurements of observed time difference of arrival," 2009 IEEE 20th International Symposium on Personal, Indoor and Mobile Radio Communications, Tokyo, Japan, 2009, pp. 2213-2217. On proposal #2.10-2, we agree with Huawei that other values could be considered. We have some preliminary ray-tracing results in an indoor mixed-office scenario where we observed the following for NLOS links: - In roughly 50% of the links the first detectable, i.e. power within 25 dB of the strongest path, NLOS path has an excess delay of essentially 0. This is common when the NLOS condition is due to cubicles, desks, or thin plaster walls. - In the other 50% of the links, the excess delay can be reasonably approximated by a lognormal distribution with mlgDt = -8.8, slgDt = 0.5 |
| Huawei, HiSilicon | Per Moderator’s request, following information is provided: Methodology: Calculating the delay difference between 3D-distance and first-arrived NLOS cluster. ##### Set-up: As described in our contribution R1-2409402 [1], where each measurement point corresponds to a raw data point. |

#### Outcome of Thursday Session

Working Assumption

- Adopt the following absolute delay for UMi and UMa scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

Agreement

- Introduce absolute delay for InH scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - FFS details of the parameter for InH. Consider the following for further study.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

### 4.2.11 Other Parameters

| Company | Proposals & Observations |
| --- | --- |
| [3] Interdigital | Observation 1: It is not clear from the agreement in Example 2 whether or not wrapTo180 is applied to azimuth angles. Proposal 1: The agreement for Example 2 should explicitly indicate that wraptTo180 must be applied when generating the intermediate azimuth angles. That is, ϕn,intermediate=s∙WrapTo180ϕn,model-μϕ,model for azimuth angles. Observation 2: Both Example 1 and Example 2 provide accurate performance in terms of how close ASmeasured is to ASdesired. However, Example 1 provides more accurate angle spread compared to Example 2. Proposal 2: Support Example 1 to generate the scaled set of angles for MIMO CDL channel. Figure 1 –Measured angle spread vs Desired angle spread for Azimuth AoD (L) and AoA (R) of CDL-A channel model Figure 2 –Measured angle spread vs Desired angle spread for Azimuth AoD (L) and AoA (R) of CDL-B channel model Figure 3 –Measured angle spread vs Desired angle spread for Azimuth AoD (L) and AoA (R) of CDL-C channel model Figure 4 –Measured angle spread vs Desired angle spread for Azimuth AoD (L) and AoA (R) of CDL-D channel model Figure 5 –Measured angle spread vs Desired angle spread for Azimuth AoD (L) and AoA (R) of CDL-E channel model Observation 3: For line-of-sight channels (CDL-D and CDL-E), both Example 1 and Example 2 generate Az angles for which the angle spreads, ASmeasured, diverge for desired angle spread, ASdesired, values greater than specific values. Proposal 3: RAN1 defines a maximum desired angle spread for use of Example 1. |
| [4] Samsung | Observation 6 When examining frequency dependent properties of channel parameter, dynamic range of the measured data affect to results. Proposal 4 RAN1 discuss whether/how to apply dynamic range when it comes to frequency dependent properties for channel parameter. |
| [6] ZTE, Sanechips | Observation 15: In the commonly used range of angular spread in 3GPP simulation work, the error of angular spread and mean angle caused by the linear scaling is small enough to ignore. Proposal 11: The modification to angle scaling formula in TR38.901 is unnecessary. Proposal 12: If accurate angular spread and mean angle are needed, adopt draft CR in Annex A-1 to TR38.901 Section 7.7.5.1. #### 7.7.5.1 CDL extension: Scaling of angles The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation: ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, (7.7-5) where ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model, and Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle. ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}. ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn (7.7-5) in which: ϕn,model is the tabulated CDL ray angle ASmodel is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A μϕ,model is the mean angle of the tabulated CDL, calculated using the definition in Annex A μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A μϕ,desired is the desired mean angle ASdesired is the desired rms angular spread ϕn,scaled is the resulting scaled ray angle WrapTo180(⋅) is a function which wraps an azimuth angle to the interval [-180, 180]. |
| [7] Intel | Proposal 5: - RAN1 to consider correcting the angle handling for MIMO simulation extension for CDL as part of the 7 – 24 GHz channel model validation SI. - To address the angle-scaling issue when generating CDL channels for link-level evaluations adopt the following two-step approach to compute the scaled angles: ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, ϕn,intermediate=s× ϕn,model-μϕ,model, where s is a scale factor chosen to change the angles. |
| [10] CATT | Figure 1 Difference between the generated desired AOD spread based on the model in TR 38.901 and the accurate desired angle spread in CDL-A, CDL-B, CDL-C, CDL-D and CDL-E models Observation 1: The difference between the accurate desired angle spread and the model in TR38.901 is not evident. Proposal 7: There is no need to update the scaling of the angles in TR38.901 to enable accurate desired angle spread. |
| [19] Qualcomm | Proposal 12: To address the angle-scaling issue when generating CDL channels for link-level evaluations, consider one of the following options: - Decouple the desired angular spread and the linear scale factor into two separate parameters. Provide a finite list of values for desired angular spread along with the corresponding linear scale factor to realize each of them. - Move away from scaling the angular spread and instead focus on scaling the angular range. Use linear scaling to achieve the desired range. Proposal 13: To address the angle-scaling issue when generating CDL channels for link-level evaluations adopt the following two-step approach to compute the scaled angles: ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, and ϕn,intermediate=s* ϕn,model-μϕ,model, where s is a scale factor. |

#### Summary of Issues

Companies suggested changes angle calculations for CDL channel model.

##### Proposal #2.11-1:

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

- Down-select among option 1, 2 and 3.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles, e.g. s=ASdesiredASmodel.

  - Option 2)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

#### Round #1 Discussion

Please provide comments on issues regarding other channel modelling aspects. Please provide comments on Proposal #2.11-1.

| Company | Comments |
| --- | --- |
| InterDigital | OK, but why we are keep adding options instead of down-selection of the identified approach from the last meeting. |
| Moderator | Just to clarify, it would be ideal to try to down-select among options in this meeting (if possible). |
| CATT | Not necessary. |
| ZTE | Fine with the proposal, and prefer Option 2 which is the only option that provides the accurate result. |

#### Summary of Round #1 Discussion

Limited number of companies provided inputs.

CATT provided comments that the proposal was not necessary. It was not clear to the moderator whether CATT is commenting they prefer option 3 or they prefer no change to TR. As far as moderator understands, the two step changes to the TR would be still needed to accurately account for mean value to applied to the scaled angles correctly. It would be good for CATT to clarify.

Proposal was slightly updated to provide more accurate information.

##### Proposal #2.11-1A:

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

  - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - WrapTo180(⋅) is a function which wraps an azimuth angle to the interval [-180, 180].

- Down-select among option 1, 2 and 3.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

##### Proposal #2.11-1B:

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

  - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180].

- Down-select among options.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2a)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 2b)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s is chosen such that AS(ϕn,intermediate) = AS_desired

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

##### Proposal #2.11-1C:

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

  - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180].

- Down-select among options.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2a)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 2b)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s is chosen such that AS(ϕn,intermediate) = AS_desired

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

    - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread.

- FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

#### Round #2/3/4 Discussion

Please provide further comments.

Also, it would be good for companies who provided AS results, whether the AS results were computed with all possible values of target mean value, or whether the plots are only for target mean value of 0 or equal to mean value of the original model. From moderator understanding, the AS of the existing equation in TR changes as a function of the target mean value of the angles, which is one of the problems identified.

| Company | Comments |
| --- | --- |
| Ericsson | The function WrapTo180 is not defined. Define it as the mapping of an angle to the half-open interval (-180, 180] degrees, such that the output angle modulo 360 is the same as the input angle modulo 360. We prefer option 3. In option 1, the definition of s is missing. Option 2 might be acceptable if the Angle function in the definition of AS(x) can be clarified and the definition of s can be simplified, e.g. “s is chosen such that AS(s) = AS_desired”. We suggest adding the following note to Section 7.7.5.1: Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread. We also suggest investigating if it is desirable to ensure that the specular ray of CDL-D and E ends up in the desired directions, to ensure that, if a specular ray exists, it points in the direction of the signal source. For example, by using the specular direction as mu, or by updating the specular ray separately after the scaling procedure is completed. |
| Moderator | ##### Updated based on Ericsson comments. @Ericsson, for option 1, it is stated it is a scaling factor chosen to change the distribution angles. |

#### Outcome of Thursday Session

Agreement

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

  - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180].

- Down-select among options.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2a)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 2b)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s is chosen such that AS(ϕn,intermediate) = AS_desired

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

    - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread.

- FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

## 4.3 Discussions on Other Scenarios (not currently captured in TR)

### 4.3.1 Suburban Macro

| Company | Proposals & Observations |
| --- | --- |
| [2] Ericsson | Table 1 Comparison of the WINNER-II and IMT-Advanced SMa models WINNER-II SMa; IMT-Advanced SMa; Comment. BS height; 25 m; 35 m; Both path loss models supports variable BS height. ISD; -; 1299 m. UT distribution; -; 50% in vehicles and 50% indoors. Path loss model; The path loss equations are different. IMT-Advanced was based on WINNER-II but additional measurements were considered; Both models are valid only for 2—6 GHz. Shadow fading; LOS: 4/6 dB before/after breakpoint NLOS: 8 dB; LOS: 4/6 dB before/after breakpoint NLOS: 8 dB; Identical. LOS probability; PLOS=exp-d200; PLOS=1, d≤10exp-d-10200, d>10; Essentially the same even though different BS antenna heights are assumed. Doesn’t account for the UE antenna height, i.e. when UEs are deployed on upper floors in buildings.. LSPs; Very similar, but some parameter values are changed; Neither model includes elevation angular spreads. Observation 2 The WINNER-II and IMT-Advanced SMa scenario and modeling parameters can be used as inspiration for a potential new SMa scenario in TR 38.901. Observation 3 The elevation angular spread, the frequency dependence of the path loss and the UE height dependence of the LOS probability need further study and validation. Figure 1 Measured excess path loss in the SMa scenario, with error bars denoting the measurement uncertainty. Note that some percentiles are missing for 5 GHz due to an equipment misconfiguration that affected the sensitivity. Obversation 4 In a suburban residential scenario, new measurements show that the path loss has a 10⋅log_10 (f) frequency dependence up to 10 GHz and a rather flat frequency dependence above 10 GHz. Observation 5 The SMa NLOS path loss model in the WINNER II and IMT-Advanced models are only valid for 2—6 GHz and don’t capture the frequency dependence across the 7—24 frequency range that is shown in the measurements. Figure 4 LOS probability as a function of 2D distance between BS and UE for different combinations of BS and UE antenna heights for the suburban residential area in Santa Clara, CA. Trees are present in the map. Figure 5 LOS probability as a function of 2D distance between BS and UE for different combinations of BS and UE antenna heights for the suburban residential area in Santa Clara, CA. Trees have been removed from the map. Observation 6 The SMa LOS probability model in the WINNER II and IMT-Advanced models do not capture the dependence on BS and UE height. Observation 7 The SMa LOS probability is strongly dependent on the tree density in the scenario. Observation 8 The LOS probability in an SMa scenario may create unique interference conditions, hence addition of an SMa scenario can support better resiliency to these in future 3GPP releases. Proposal 2 Narrow down the range of antenna heights and determine a typical tree density before attempting to curve fit and specify a LOS probability model for a new SMa scenario. Proposal 3 The SMa scenario in the IMT-Advanced model is used as a starting point for specifying other large-scale parameter values in a new SMa scenario in TR 38.901. Proposal 4 The starting point values can later be adjusted based on new measurements reported during the study item. Proposal 5 If neither IMT-Advanced nor new measurements address some parameter values, consider reusing values from the UMa scenario. |
| [6] ZTE, Sanechips | Proposal 1: RAN1 to converge on a single set of values for building density, BS height and ISD to determine the scenario. Proposal 2: BS height 25m and ISD 1000m can be adopted for studying SMa scenario. Table 1. Simulation parameters configuration Setup/Para; Map1. Area; 2500m*1150m. UE height; 1.5m. BS height; 25m. Residential buildings height; 5m~7m. Commercial building height; 14~16m. Ratio of commercial buildings to residential buildings; 1:100. Building density; 373 buildings/km2. Frequency; 7GHz. Figure 2. LoS probability results from RT simulation Observation 1: The LoS probability with non-zero value in case of large Tx-Rx distance (e.g., over 1000m) can be achieved for the scenario with lower building density at the edge of the map and open areas in the distant region (e.g., as Map-1). Observation 2: The simulation results of SMa scenario highly depend on the assumption of environment (e.g., building density, building height, the presence of open square). Proposal 3: For the LoS probability in the SMa scenario, the following formula can be considered: PrLOS=exp(-d2D-out-10exp(0.02044hBS+5.746)) Figure 3. Pathloss results from RT simulation Observation 3: In case of LoS UEs, the pathloss of SMa scenario can well match the pathloss model of UMa in TR 38.901. Proposal 4: Suburban scenario can be implemented when necessary by defining alternate parameters for UMa based on different assumption on building density/heights related to sub-urban macro deployments. |
| [7] Intel | Proposal 2: Define a new scenario SMa, and capture SMa modeling parameters similar to other deployment scenarios, InH, InF, UMi, UMa, RMa. |
| [8] Nvidia | Proposal 1: Define a new scenario, e.g., SMa, to enable necessary model to support “sub-urban macro deployments.” Proposal 2: Enable frequency continuity beyond 7-24 GHz for the SMa scenario. |
| [9] Apple | Proposal 1: Consider the following parameters for suburban scenario: - Layout with ISD: 1000 m (Option 3) - BS height: 15 m (Option 3) or 25 m (Option 2) - Min BS - UT distance (2D): 20 m (Option 1) - Building types: 90% buildings are residential buildings and 10% buildings are commercial buildings (Option B) - UT height: 1.5 m for outdoor; 1.5+3(Nfl-1) m for indoor, where Nfl is the floor and Nfl is uniform (1, 2) for residential buildings or is uniform (1, 5) for commercial buildings. (Option 2) Proposal 2: For SMa, the LOS probability is given PrLOS=exp-d2D-outexpα∙hBSβ+γhUEδ - FFS: α, β, γ, δ parameters Proposal 3: Confirm the pathloss formula for SMa (based on ITU M.2135-1), where it is applicable to UE height up to 13.5 meters. Table 5: Comparison of delay spread for SMa NLOS at 8 GHz Ray tracing simulations; UMa NLOS in TR 38.901. Delay spread mean; -6.98; -6.46. Delay spread standards deviation; 0.74; 0.39. Proposal 8: RAN1 to consider the delay spread mean value for SMa NLOS is smaller than that for UMa NLOS scenario in TR 38.901 in FR3. Table 8: Comparison of angular spread for SMa NLOS at 8 GHz Ray tracing simulations; UMa NLOS in TR 38.901. AoA spread mean; 1.27; 1.84. AoD spread standard deviation; 0.86; 2. AoD spread mean; 0.86; 1.4. AoD spread standard deviation; 1; 2. ZoA spread mean; -0.388; 1.22. ZoA spread standard deviation; 1.17; 1.4. Proposal 12: RAN1 to consider the AoA/AoD/ZoA spread mean values for SMa NLOS scenario are smaller than those mean values for UMa NLOS scenario in TR 38.901 in FR3. |
| [10] CATT | Proposal 1: For Suburban use case, the following modeling parameters are supported: - Cell layout, BS antenna height, and building type ratio: - Alt 1) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m - UT distribution (vertical): - Vertical Distribution Option 1: 70% indoor residential users are on ground floor, 30% are on upper floor, uniform distribution across all floors for commercial - Min BS - UT distance (2D): - BS Height Option 1: 35 m Proposal 2: The following option is supported for LOS probability for suburban use case: - Option 2: (ITU M.2135-1) PLOS=1, d2D-out≤10exp-d2D-out-10200, d2D-out>10 - BS height of 35m Proposal 3: Option-1 is supported to implement suburban use case: - Option-1: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments. |
| [11] Lenovo | Proposal 5 For SMa pathloss modeling, UE height values beyond 15 m. are not preferred, and the condition h_UT≤h_BS is maintained for all evaluations. |
| [15] Vodafone, Ericsson | Figure 6 ASD as observed at the suburban macro base station. For reference, the ASD according to the WINNER II Suburban Macro model is shown in the two dotted curves. A fit to the suburban measurements is shown as a black dashed curve. Observation 5 The measured ASDs at 3.4 GHz in a suburban 5G NR macrocell are very similar to the ASDs in urban macrocells, however at the upper percentiles there are less high outlier values. Observation 6 The WINNER II Suburban Macro channel model overestimates the ASD by 2-3 times. Observation 7 The measured suburban ASD can be well represented by a lognormal distribution with σlgZSD = 0.55 and µlgZSD = 0.25. Proposal 2 If RAN1 agrees to add a Suburban Macro scenario, then use the ASD and ZSD parameters according to Table 3 and Table 4 as a starting point. Proposal 3 Further measurements of ASD and ZSD in a Suburban Macro scenario, including measurements that distinguish LOS vs NLOS vs O2I, can later be used to refine the suggested parameter values. Table 3 Proposed ASD parameters for a SMa scenario Scenarios; SMa. LOS; NLOS; O2I. AOD spread (ASD) lgASD=log10(ASD/1°); µlgASD; 0.55; 0.55; 0.55. σlgASD; 0.25; 0.25; 0.25. Cluster ASD () in [deg]; 1.5; 1.5; 1.5. Table 4 Proposed ZSD parameters for a SMa scenario Scenarios; SMa. LOS; NLOS; O2I. ZOD spread (ZSD) lgZSD=log10(ZSD/1°); µlgZSD; 0.55; 0.55; 0.55. σlgZSD; 0.25; 0.25; 0.25. |
| [17] AT&T | Observation 1: Deployment scenarios identified to develop the channel models in 3GPP TR38901 do not include typical urban scenarios in North America where ISDs are on average larger than 1500m. Observation 2: UMa deployment scenario and RMa deployment scenarios in TR38.901 cannot be used to model typical urban and suburban deployment scenarios in North America. Proposal 1: Parameters for the new channel model on suburban macro deployment scenarios are validated through measurements - Reuse of parameters from UMa model is subject to validation, and applicability. - Correlations between parameters is validated for the new suburban macro deployment Proposal 2: The Following are sets of option combinations prioritized for study and experimental validation of the SMa use case. Other option combinations are not precluded. - Alt 5) ISD 1732 m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m - Alt 6) ISD 1732 m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m Table 1: Parameters for the SMa measurements conducted in Austin TX Deployment Scenarios; Specification. Suburban Macro (SMa) – Location 1/Location 2; Clutter Height, foliage; 20 m. Building height, h; 10 m. Street Width, w; 10 m. hBS; 25m and 35m. hUT; 2.2 m. Observations (BS height 35m): The 1-m intercept point, PL0: The intercept point depends on the materials blocking the signal within 1m of Tx-Rx separation and the building structure. Figure 4: The scatter plot of the measured PL in LOS environments at 7 GHz Figure 5: The scatter plot of the measured PL in NLOS environments at 7GHz Figure 6: Scatter plot of measured PL in LOS environments for SMa at 15GHz Figure 7: Scatter plot of measured PL in NLOS environments for SMa at 15GHz Observation 3: The pathloss observed in the SMa measurements for LOS matches that predicted by the formula expression in ITU M.2135-1 for hBS = 35m. Observation 4: The pathloss observed in the SMa measurements for NLOS was lower than the pathloss predicted by the formula expression in ITU M.2135-1 for hBS = 35m. Observation 5: Measurements conducted at 7, and 15 GHz for SMa with base station height of 35m show that shadow fading distribution for LoS and NLoS environments agree with the ITU M.2135-1 formula expression. Table 2: Pathloss parameters comparison for SMa at 7GHz and 15GHz (hBS = 35m, W = 10m, h = 10m) SMa Parameters; 7 GHz; 15GHz. LOS; NLOS; LOS; NLOS. Pathloss, dB; PLo; 48.3/46.9; 31.7/29.5; 56.4/53.3; 49.5/37.5. PLE; 2.1/2.2; 3.4/3.9; 2.1/2.2; 3.1/3.9. SF, dB; 4.7/4; 6.3/8; 5.6/4; 6.0/8.0. Observations (BS height 25m): The 1-m intercept point, PL0: The intercept point depends on the materials blocking the signal within 1m of Tx-Rx separation and the building structure. The Pathloss Exponent, PLE: The PLE from all LOS SMa measurements are 2., 2.1, and 2.2dB/decade in LOS for 7, 8, and 15 GHz respectively. These results agree with the PLE in the formula expression in ITU M.2135-1, as also shown in Figures 8, 10 and 12 for 7GHz, 8GHz, and 15GHz, respectively. Note that the SCM SMa in the Figures shows the ITU M.2135-1.PL formula expression. The PLE of {3.6 dB/decade, 3.6 dB/decade, 4.0 dB/decade } for 7, 8 and 15GHz respectively, in NLOS deviates from the NLOS PLE in the ITU expression of 3.9 dB/decade. Moreover, PL0 at d0 = 100m (86.8 dB, 101.9 dB, and 108.9 dB) deviates from the NLOS path loss predicted by the ITU expression at 100m. Figure 13: Scatter plot of measured PL in NLOS environments for SMA at GHz for base station height of 25m Observation 6: The pathloss observed in the SMa measurements for LOS largely matches that predicted by the formula expression in ITU M.2135-1 for hBS = 25m. Observation 7: The pathloss observed in the SMa measurements for NLOS was lower than the pathloss predicted by the formula expression in ITU M.2135-1 for hBS = 25m. The shadow fading standard deviation for LOS, NLOS, 7 , 8 and 15GHz is shown in Table 3 for the SMa outdoor measurements with BS height of 25m. Observation 8: Measurements conducted at 7GHz, 8GHz, and 15 GHz for SMa with base station height of 25m show that shadow fading distribution for LoS and NLoS environments agree largely with the ITU M.2135-1 formula expression. Table 3: Pathloss parameters comparison for SMa at 7GHz , 8GHz and 15GHz (hBS = 25m, W = 10m, h = 10m) SMa Parameters; 7 GHz; 8GHz; 15GHz. LOS; NLOS; LOS; NLOS; LOS; NLOS. Pathloss, dB; PLo; 86.8/90.3; 101.9/110.2; 85.9/91.9; 108.9/112.7; 95.3/96.7; 110.3/118.3. PLE; 2.0/2.2; 3.6/3.9; 2.1/2.2; 3.6/3.9; 2.1/2.2; 4.0/3.9. SF, dB; 3.9/4.0; 5.5/8.0; 2.8/4.0; 7.2/8.0; 3.6/4.0; 4.7/8.0. |
| [18] NTT Docomo | Proposal 1: For SMa scenario, the following modeling parameters are supported: Parameters; SMa. Cell layout; Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = Option 2: 1299 m or Option 3: 1000 m) Up to two floors for residential buildings, up to five floors for commercial buildings. Building Type Ratio Option B: 90% residential, 10% commercial. BS antenna height hBS; Option 1: 35 m or Option 2: 25m. UT location; Outdoor/indoor; Outdoor and indoor. LOS/NLOS; LOS and NLOS. Height hUT; 1.5 m for outdoor, 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings. Indoor UT ratio; 80% indoor and 20% outdoor. UT mobility (horizontal plane only); Indoor UTs: 3 km/h, outdoor UTs: 90 km/h. Min. BS - UT distance (2D); Option 2: 25m or Option 3: 35m. UT distribution (horizontal); Uniform. UT distribution (vertical); Vertical Distribution Option 2: uniform distribution across all floors for a building type. Penetration model; FFS: low-loss penetration model. Proposal 2: For SMa scenario, the following combinations of cell layout, BS antenna height, and building type ratio for modeling parameters are supported: - Alt 1) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m - Alt 2) ISD 1000m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m Proposal 3: The parameters of channel model for SMa scenario in Table A1-7 of ITU-R M.2135-1 can be used as start point. |
| [19] Qualcomm | Proposal 1: Define a new scenario in 38.901 to introduce the modelling parameters for Suburban Macro scenario. Define this scenario for at least the 7-24 GHz and < 7 GHz bands. Figure 1 Comparison of different pathloss models at 1 GHz centre frequency. Figure 2 Comparison of different pathloss models at 3.5GHz centre frequency. Figure 3 Comparison of different pathloss models at 13 GHz centre frequency. Observation 1: For SMa scenario, the proposed changes to the pathloss model in ITU M.2135-1 lead to more optimistic pathloss estimate (i.e., smaller pathloss values) for sub-2GHz frequency range compared to ITU M.2135-1, and a more conservative pathloss estimate (i.e., largerer pathloss values) for the > 2 GHz frequency range. To avoid any unintended changes to the pathloss model at frequencies beyond the core frequencies of interest in this study (i.e., 7-24 GHz), it is suggested that we exercise more caution when making changes to the model. Proposal 2: Strive to minimize changes to the suburban pathloss model in ITU M.2135-1. Ensure impact of changes to frequencies outside 7-24 GHz is well justified. Figure 4 Comparison of different LOS probability models. Proposal 3: Adopt the following two guidelines for downselection of the LOS probability model for SMa scenario: - LOS probability of a UE in SMa scenario should lie between the LOS probabilities for UMa and RMa scenario. - LOS probability model can be independent of UE height as most UEs in SMa scenario are at the ground level. Proposal 4: For SMa scenario, consider the following two alternatives for ISD, BS antenna height, and building type ratio: - Alt 1) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m - Alt 2) ISD 1000m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m Note: This is subject to alignment with eventual pathloss model adopted for SMa to ensure a high percentage of UEs fall within coverage. |
| [20] Nokia | Proposal 3: RAN1 to select either ISD Option 1 or 2 with building type ratio option B, which specifies an ISD of 1732 or 1299 meters with 90% residential and 10% commercial, correspondingly, for SMa scenarios. Proposal 4: RAN1 to select either BS height Option 1 or 2, which specifies a BS height of 35 or 25 meters, correspondingly, for SMa scenarios. Figure 4. Antenna heights summary in Chicago Suburbs. Proposal 5: RAN1 to consider the following model assumptions for SMa scenarios: - UT distribution: Option 2, 0.5 m for outdoor; 1.5+3(Nfl-1) m for indoor, where Nfl is the floor and Nfl is uniform (1, 2) for residential buildings or is uniform (1, 5) for commercial buildings. (framework in 36.873 for UMa with changes to distribution of Nfl (number of floors).) - Min BS-UT distance (2D): Option 2 and Option 3, 25 and 35 m Proposal 6: RAN1 to consider reusing the UT mobility parameters of Umi and Uma scenarios for SMa scenarios: - 3 km/h for pedestrians (indoors and outdoors) - 30 km/h for in-car user terminals Figure 5. LOS probability Option 2 and Option 3 of SMa scenario compared to RMa and UMa scenarios with hUT=1.5 m. Observation 4: Considering the SMa scenario is not significantly different from the UMa scenario, and the proposed path loss model also aligns with the ITU M.2135-1 LOS probability Option 2 could provide a more realistic representation than Option 3. Proposal 7: RAN1 should gather additional measurements or simulation data to further validate the LOS probability models for Option 2 and Option 3. Proposal 8: RAN1 should consider introducing a new parametrization for low loss outdoor-to-indoor (O2I) building penetration loss model for SMa scenarios which consists of 70% wood and 30% plain glass, and the penetration loss through wall is: PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10 |

#### Summary of Issues

Several companies have provided inputs on SMa deployment scenario.

##### Proposal #3.1-1

For suburban scenario, adopt the following assumption for calibration purposes:

- Building type ratio: 90% residential, 10% commercial

##### Proposal #3.1-2

For suburban scenario, adopt the following assumption for calibration purposes:

- UT Vertical Distribution Option 2: uniform distribution across all floors for a building type

##### Proposal #3.1-3

For suburban scenario, adopt the following assumption for calibration purposes:

- UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 90 km/h

##### Proposal #3.1-4

For suburban scenario, further consider following options for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

- Option 3) BS height = 25 or 35m

##### Proposal #3.1-5

For suburban scenario, adopt the following assumption for calibration purposes:

- Minimum BS to UT distance in 2D:

  - 25m for BS height of 25m

  - 35m for BS height of 35m

Proposal #3.1-6

For suburban scenario, the LOS probability is given as

- PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ

- FFS: dm, α, β, γ, δ parameters

##### Proposal #3.1-7

For suburban scenario, further consider following options for ISD assumption for calibration purposes:

- TBD (1000, 1299, 1732)

##### Proposal #3.1-8

Introduce low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

#### Round #1 Discussion

Please provide comments on issues regarding support of suburban use case. Please provide comments on Proposal #3.1-1 to #3.1-8.

Further discussion on Proposal #3.1-7 and #3.1-4 seem critical to resolve the scenario details.

| Company | Comments |
| --- | --- |
| Apple | Proposal #3.1-1: Support Proposal #3.1-2: Support Proposal #3.1-3: Not sure if outdoor UT speed is too fast. It seems to be highway. Proposal #3.1-4: We are fine with Option 1 Proposal #3.1-5: This depends on Proposal #3.1-4, and we could decide it later. Proposal #3.1-6: Support Proposal #3.1-7: Fine Proposal #3.1-8: It was agreed in RAN1# 118bis to consider additional O2I building penetration loss mode. We may consider either the existing O2I low-loss model or the potential additional O2I model for SMa. Maybe we could hold this proposal till the conclusion on whether or not additional O2I model is introduced, then we decide whether or not the existing O2I low-loss model is used for SMa. |
| Moderator | Just to clarify, I plan to make updates to Proposal #3.1-7 based on feed and inputs and do not intent to get agreement as it stands. |
| Samsung | Generally fine. For proposal #3.1-5, we are not sure that minimum distance is comparative with BS height only |
| LGE | Generally fine with proposals. Proposal #3.1-4: prefer option 2 (35m) Proposal #3.1-7: prefer 1299m |
| QC | Proposal #3.1-1: Okay Proposal #3.1-2: Okay Proposal #3.1-3: Outdoor UT speed seems too high. Proposal #3.1-4: Okay, but strictly downselect one value. Base it on operator preference. Proposal #3.1-5: Okay Proposal #3.1-7: Okay, but downselect one value. Proposal #3.1-8: Support |
| ZTE | Proposal #3.1-6: Not clear how this can be fitted with different UE height. |
| AT&T | Proposal #3.1-1: ok Proposal #3.1-2: ok Proposal #3.1-3: outdoor speed is too high Proposal #3.1-4: ok – we are also open to considering a range of BS heights Proposal #3.1-5: ok Proposal #3.1-7: Do not support. 1000m is too low Proposal #3.1-8: what is the rationale behind this proposal? |
| Ericsson | #3.1-1: Ok #3.1-2: Ok #3.1-3: 90 km/h seems very high, but maybe ok for calibration purposes. #3.1-4: No opinion #3.1-5: Don’t really see the need of this. All UEs will be lower than the BS anyway, so the 3D distance will be >10 m even if the 2D distance is 0 m. #3.1-6: As observed in our contribution R1-2409441 the tree density has a very strong impact on the LOS probability in SMa. We suggest converging on how to handle this before deciding on the detailed expression for the LOS probability function. #3.1-7: Ok, although 1000 m seems too low #3.1-8: We sympathize with the intent of the proposal but it is unclear if this should be the only O2I model for suburban or if the existing low-loss and high-loss models in 38.901 should also be possible to use. We prefer the latter, since the building construction and use of energy-efficient glass is very dependent on the geographical region (Note that Proposal #3.1-1 specifies that some buildings in the SMa scenario are commercial buildings.). In Rel 14 there was a long discussion on this that is captured in this paragraph in the TR: “The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world (see note). Note: One example survey for the US market can be found in [5]. The survey does not necessarily be representative for all the scenarios. Other ratios outside of the survey should not be precluded.” |

#### Summary of Round #1 Discussion

Proposal #3.1-1 (building type ratio): No negative comments so far.

Proposal #3.1-2 (vertical UE distribution): No negative comments so far.

Proposal #3.1-3 (mobility): Few comments on outdoor speed being too high.

Proposal #3.1-4 (BS height): Samsung is uncure how this matches or relates with BS height

Proposal #3.1-5 (min distance): Apple askes to postpone proposal until #3.1-4

Proposal #3.1-6 (LOS prob): ZTE is asking if the formula will work with different UE height. Orignal proponent, Apple, is asked to provide further information.

Proposal #3.1-7 (ISD): Further discussion is needed to down-select proposal as it stand is incomplete.

Proposal #3.1-8 (pen-loss): AT&T is asking for more information, e.g. motivation and further explanation

##### Proposal #3.1-3A

For suburban scenario, adopt the following assumption for calibration purposes:

- UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 40 90 km/h

#### Outcome of Tue Session

Proposal #3.1-1, #3.1-2, and #3.1-3 were agreed.

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- Building type ratio: 90% residential, 10% commercial

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT Vertical Distribution Option 2: uniform distribution across all floors for a building type

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 40 km/h

#### Round #2 Discussion

Further discussion to refine BS height and ISD is likely needed for further work. Moderator strongly thinks some form of down-selection would be extremely helpful in this meeting.

Please provide further comment on Proposal #3.1-4A, #3.1-6, #3.1-7A, and #3.1-8A.

Moderator suggests to postpone #Proposal #3.1-5 until we have agreement on BE height and ISD.

##### Proposal #3.1-4A

For suburban scenario, further consider following options for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

- Option 3) BS height = 25 or 35m

##### Proposal #3.1-7A

For suburban scenario, further consider following options for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

##### Proposal #3.1-6

For suburban scenario, the LOS probability is given as

- PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ

- FFS: dm, α, β, γ, δ parameters

Proposal #3.1-8A

Additionally introduce low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

| Company | Comments |
| --- | --- |
| Nokia | Proposal #3.1-1: OK Proposal #3.1-2: OK Proposal #3.1-3: we propose 30kmph speed for in-ca user terminals Proposal #3.1-4: Option 3 Proposal #3.1-5: OK Proposal #3.1-6: The model is currently too generic. It is better to keep the Option 2 model or Optoin 2/3 type of model from RAN1#118bis with the parameters FFS. Proposal #3.1-7: 1000m seems too short ISD. Proposal #3.1-8: Support. The composition of the buildings in suburban is different from the current assumptions for the low loss O2I. We propose to adapt the materials to more typical, i.e., 70% wood and 30% glass. Maybe the better world would be to adjust the existing low loss model, rather than to introduce. |
| AT&T | ##### Proposal #3.1-4A and proposal #3.1-7A: we agree that a downselection for calibration purposes is necessary, but perhaps as the FL explained, we can add a note that says that the channel model is actually applicable to a range of base station heights e.g. [25-35] and a range of ISDs, e.g. [1500-1732m]. For the downselection, we prefer base station height = 25m and ISD = 1732m Note that we also updated our reference [17] with the revised document number (R1-2410655) that contains 8GHz results at 25m base station height. (The observations are also updated in the SMa table of results/proposals above) |
| Apple | ##### Proposal #3.1-4A: we prefer Option 1 Proposal #3.1-6: OK Proposal #3.1-7A: we slightly prefer Option 1. Proposal #3.1-8A: OK |
| ZTE | To clarify our comment in first round on Proposal #3.1-6, according to our observation from ray tracing simulation, UEs in residential buildings have height 1.5m or 4.5m, the LoS probability would be much lower than UE height e.g. 10.5 in commercial building, so it’s impossible to fit a curve as a function of UE height in SMa scenario. |

#### Round #2 Discussion

Moderator has updated the proposal based on feedback here as well as feedback received offline.

Proposal #3.1-4B

For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

- Option 3) BS height = 25 or 35m

- Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35]m.

Proposal #3.1-7B

For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

- Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

##### Proposal #3.1-6A

For suburban scenario, further study the LOS probability is given as and consider the following equation for study to model fit the LOS probability:

- PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ

- FFS: dm, α, β, γ, δ parameters

- FFS whether the equation can support LOS probability difference of UEs in residential and commercial building, which have different building heights. If determined to be difficult to curve fit measurements with the equation, FFS on how to better formulate the LOS probability.

##### Proposal #3.1-8A

Additionally introduce low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

#### Round #3/4 Discussion

| Company | Comments |
| --- | --- |
| Sharp | Support. |
| Ericsson | Proposal #3.1-6: As observed in our contribution R1-2409441 the tree density has a very strong impact on the LOS probability in SMa. We suggest converging on how to handle this before deciding on the detailed expression for the LOS probability function. Please see our comment to proposal #2.6-1 where we suggest to instead reuse the InF LOS probability function but with clutter density and height parameters tuned to the suburban scenario. Proposal #3.1-8: While we agree that concrete is rarely used in suburban residential buildings, there can be other materials such as various insulation materials that may have a different impact than wood on the building penetration loss. We are not against the proposal as such, but would like to see at least one validation measurement of the building penetration loss for a residential building before agreeing to it. |
| Moderator | Updated Proposal #3.1-6 and #3.1-6 based on Ericsson comments. |

#### Summary of Round #3 Discussion

Proposal #3.1-6 and #3.1-8 was updated based on comments received.

##### Proposal #3.1-4B

For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

- Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35]m.

##### Proposal #3.1-7B

For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

- Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

##### Proposal #3.1-6B

For suburban scenario, further study the LOS probability equation.

- Example of LOS probability equation for study:

  - PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, where dm, α, β, γ, δ are modeling parameters

- Study includes whether distinction of LOS probability should be made for UEs in residential and commercial buildings, which have different building heights.

- Study includes whether or not vegetation should be explicitly considered and modeled for determining LOS probability, if determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

##### Proposal #3.1-8B

Further study the need of additional low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario.

- Potential example of the new low loss O2I building penetration loss:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

#### Round #4 Discussion

Please comment on Proposal #3.1-4B, #3.1-7B, #3.1-6B, #3-17B.

| Company | Comments |
| --- | --- |
| AT&T | Proposal 3.1-4B: support Proposal 3.1-7B: support Proposal 3.1-6B: support Proposal 3.1-8B: support. It would be good to further study this and bring measurements/study next meeting. |
| Sharp | Proposal 3.1-4B: support Proposal 3.1-7B: support Proposal 3.1-6B: support in general. But we feel 2.6-1A and last bullet in this proposal are similar. Consider merging 2.6-1A into this proposal as 2.6-1A is anyway targeted for SMa scenario. This is a much better and appropriate place for 2.6-1A rather than having a standalone proposal. Suggest removing 2.6-1A as its not required and the same thing is more accurately and appropriately captured in the right context in this proposal. Proposal 3.1-8B: support in general. Same comment as Ericsson and AT&T. |

#### Outcome of Thursday Session

Agreement

For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35]m.

Agreement

For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

Agreement

For suburban scenario, further study the LOS probability equation.

- Example of LOS probability equation for study:

  - PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, where dm, α, β, γ, δ are modeling parameters

- Study includes whether distinction of LOS probability should be made for UEs in residential and commercial buildings, which have different building heights.

- Study includes whether or not vegetation should be explicitly considered and modeled for determining LOS probability, if determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

Agreement

Further study the need of additional low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario.

- Potential example of the new low loss O2I building penetration loss:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

### 4.3.2 Urban Macro with < 500m ISD

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Table 2 Fast fading parameters Scenario; UMa @6.5 GHz; UMa @13 GHz. TR 38.901; Measurement; TR 38.901; Measurement. LOS; NLOS; LOS; NLOS; LOS; NLOS; LOS; NLOS. Delay spread (DS) lgDS=log10(DS/1s); μlgDS; -7.03; -6.45; -7.32; -7.01; -7.06; -6.51; -7.59; -7.12. [ns]; 93.3; 354.8; 47.9; 97.7; 86.6; 311; 25.3; 76.2. σlgDS; 0.66; 0.39; 0.38; 0.28; 0.66; 0.39; 0.5; 0.42. AOD spread (ASD) lgASD=log10(ASD/1°); μlgASD; 1.15; 1.41; 0.82; 1.26; 1.18; 1.37; 1.08; 1.25. [deg]; 14.1; 25.7; 6.6; 18.2; 15.3; 23.6; 12.1; 17.9. σlgASD; 0.28; 0.28; 0.28; 0.27; 0.28; 0.28; 0.21; 0.3. AOA spread (ASA) lgASA=log10(ASA/1°); μlgASA; 1.81; 1.86; 1.67; 1.72; 1.81; 1.78; 1.40; 1.66. [deg]; 64.6; 72.4; 46.8; 52.5; 64.6; 60.2; 25.3; 45.8. σlgASA; 0.2; 0.11; 0.19; 0.15; 0.2; 0.11; 0.13; 0.19. ZOD spread (ZSD) lgZSD=log10(ZSD/1°); μlgZSD; 0.54; 0.69; 0.7; 0.78; 0.54; 0.69; 0.68; 0.83. [deg]; 3.5; 4.9; 5; 6; 3.5; 4.9; 4.8; 6.8. σlgZSD; 0.4; 0.49; 0.33; 0.34; 0.40; 0.49; 0.31; 0.35. ZOA spread (ZSA) lgZSA=log10(ZSA/1°); μlgZSA; 0.95; 1.25; 1.08; 1.15; 0.95; 1.15; 0.72; 1.15. [deg]; 8.9; 17.8; 12; 14.1; 8.9; 14.2; 5.3; 14.18. σlgZSA; 0.16; 0.16; 0.17; 0.12; 0.16; 0.16; 0.06; 0.15. Number of clusters; 12; 20; 13; 17; 12; 20; 11; 15. Observation 2: The fast fading parameters of UMa with <500 m ISD differ from that of UMa in TR 38.901. Proposal 4: Support to define <500 m ISD for UMa. |

#### Summary of Issues

One company have provided inputs on changes to channel modeling parameters for specifically UMa deployment scenario with smaller than 500 m ISD.

##### Proposal #3.2-1

- In case ISD less than 500m is used for UMa, adopt alternative fast fading parameters, e.g. DS, ASA, ASD, ZSD, ZSA, number of clusters. FFS on the details of the fast fading parameters.

##### Proposal #3.2-1A

- Include ISD = 200m for UMa at least for calibration purpose.

#### Round #1/2 Discussion

Please provide comments on issues regarding support of UMa with ISD less than 500m use case. Please provide comments on Proposal #3.2-1.

| Company | Comments |
| --- | --- |
| Apple | It is better to first determine whether or not ISD less than 500 m is used for UMa, before going to the details. |
| Huawei, HiSilicon | Support. |
| Samsung | Similar view with Apple. We prefer to discuss whether this case can be considered as UMa or another new scenario, first. |
| LGE | Same view with Apple and Samsung |
| QC | Why are these parameters sensitive to ISD? Does dependence on ISD also apply to other scenarios? Note that if ISD is only used for calibration, any ISD is technically a valid ISD. |
| Ericsson | Don’t agree, this proposal is in conflict with proposal #1-4. If alternate parameters are required for “dense” UMa, we’d prefer to identify scenario that is different from UMa. |
| Moderator | More discussion is clearly needed. Moderator suggests continuing discussion in this sub-section. To the proponent, Huawei, if you can provide further information to the companies that have asked questions, moderator thinks it would be helpful. |
| Nokia | In our view, it is not prohibited to used ISD of 500m and below for the current UMa model. The update of the parameters only for the specific ISDs is not physical. |
| Huawei, HiSilicon | While our measurement campaigns show that the current UMa may not perfectly match the real deployment scenario with <500 m ISD, given the non-negligible measurement effort, we kindly suggest companies to consider the following updated proposal, where 200 m ISD for UMa comes from IMT-2020 evaluation report. ##### Updated Proposal #3.2-1 Include ISD = 200m for UMa at least for calibration purpose. |

#### Summary of Round #3 Discussion

Proposal #3.2-1 was discussed during Wednesday session. However, there were several questions and comments on the proposal.

Given the comments it not clear if there could be a better formulation that would satisfy the concerning companies. Moderator has updated Proposal to better reflect what is expected to be updated in the TR by Proposal #3.2-1. Let’s continue discussion.

##### Proposal #3.2-1B

- Include add ISD = 200m for UMa at least for calibration purpose to Table 7.2-1 of TR38.901 Section.

#### Round #4 Discussion

| Company | Comments |
| --- | --- |
| - | - |

#### Outcome of Thursday Session

Agreement

- UMa can also be used for other ISDs between 200m and 500m, including the ISD proposed in ITU-R M.2412 -0

- One ISD value will be selected for calibration with ISD = 500m

- No change to any other parameters due to other ISD values

- FFS: How to reflect it in TR38.901

### 4.3.2 Low-altitude airspace scenario

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | (a) CDF of DS (b) CDF of ASA (c) CDF of K Factor Figure 12 Comparison of fast fading parameters Observation 3: At least some fast fading parameters of low-altitude airspace scenario are altitude-dependent. Table 3 Fast fading parameters Scenario; Delay spread (DS) lgDS=log10(DS/1s); K-factor (K) [dB]; AOA spread (ASA) lgASA=log10(ASA/1°). μlgDS; [ns]; σlgDS; μK; σK; μlgASA; [deg]; σlgASA. TR 38.901 UMa LOS; -7.03; 92.35; 0.66; 9.00; 3.50; 1.81; 64.57; 0.20. 150 m; TR 36.777 UMa-AV LOS; -7.52; 30.23; 0.9; 14.96; 16.26; -7.04; 0; 3.72. Measurement; -8.15; 7.1; -7.86; -1.8; 1.82; 1.53; 33.76; 0.84. 250 m; TR 36.777 UMa-AV LOS; -7.59; 25.8; 1.04; 15.9; 25.76; -7.59; 0; 8.7. Measurement; -7.86; 13.83; -7.51; -1.1; 1.66; 1.53; 33.78; 0.68. 350 m; TR 36.777 UMa-AV LOS; -7.63; 23.24; 1.19; 16.52; 40.81; -7.96; 0; 20.35. Measurement; -7.75; 17.86; -7.83; 3.68; 7.99; 1.42; 26.21; 1.13. 450 m; TR 36.777 UMa-AV LOS; -7.67; 21.5; 1.37; 16.98; 64.65; -8.23; 0; 47.62. Measurement; -8.05; 8.95; -8.01; 8.63; 7.02; 1.26; 18.12; 1.14. 550 m; TR 36.777 UMa-AV LOS; -7.69; 20.2; 1.58; 17.34; 102.41; -8.45; 0; 111.4. Measurement; -7.97; 10.77; -7.88; 9.43; 6.82; 1.25; 17.69; 1.13. Observation 4: The fast fading parameters of low-altitude airspace scenario differ from that of UMa in TR 38.901/UMa-AV in TR 36.777. Proposal 5: Support to introduce low-altitude airspace scenario with: - Aerial UTs vertically ranging from 0 to 600 m. - Altitude-dependent fast fading parameters. |

#### Summary of Issues

One company have provided inputs on SMa deployment scenario.

##### Proposal #3.3-1

- Introduce low-altitude airspace scenario in TR38.901.

#### Round #1/2 Discussion

Please provide comments on issues regarding support of low-altitude airspace scenario use case. Please provide comments on Proposal #3.3-1.

| Company | Comments |
| --- | --- |
| Apple | We think this is out of scope. |
| Huawei, HiSilicon | Support. |
| LGE | Low priority |
| QC | Should these changes be reflected in 38.901? Or do we need to update 36.777? This could have a large impact on the overall scope of this SI. We urge caution. |
| CATT | Support. |
| Ericsson | Don’t support. TR 36.777 already specifies (three different) channel models for the low-altitude airspace. Is the proposal to specify a fourth model? The scope of the SI is only validation and adaption/extension of the channel model in TR 38.901, not TR 36.777. |
| Moderator | More discussion is clearly needed. Moderator suggests continuing discussion in this sub-section. |
| Nokia | Historically, such scenarios are covered in 36.777. From the current SID it is not obvious that the extension of 38.901 can assume the transfer of models from 36.777 to 38.901. If there is such an intent, then it is better to clarify the SID. |
| Huawei, HiSilicon | Support. Considering the proposed scenario may bring a series of opportunities and challenges for wireless communication system, embodying it in TR 38.901, which makes sure most technical evolution may take it into consideration, is obviously more appropriate. |

#### Summary of Round #3 Discussion

There is large group number companies who do not think low-altitude airspace scenario is critical for the SI. Moderator suggests proponent company to discuss further with concerning companies and come back later. This proposal is suggested to be treated with lower priority in RAN1 #119 and will comeback if sufficient time is available.

Meanwhile, please continue to provide comments on Proposal #3.3-1.

#### Round #4 Discussion

| Company | Comments |
| --- | --- |
| - | - |

#### Summary of Round #4 Discussion

Moderator believes the proposal in this issue required further discussions. Suggest to not pursue in this meeting and further discuss in future meetings.

## 4.4 UE Antenna Modeling

| Company | Proposals & Observations |
| --- | --- |
| [1] Huawei, HiSilicon | Proposal 7: Down-select the UE-antenna-model-related aspects for calibration as following: - Antenna element/module placement Centres of each edge. - Antenna radiation pattern Both Option 1) (Adapt the Table 7.3-1: Radiation power pattern of a single antenna element) and 3) (omni-directional antenna pattern). - Antenna radiation direction Along antenna’s orientation (along the direction determined by the vector connecting the centre of the rectangle to the antenna location). - Antenna polarization Both single- and dual-polarized. - UE orientation Randomized in horizontal plane, 0 downtilt in vertical plane. |
| [2] Ericsson | Proposal 17 Introduce, at least, for modelling of UE radiation pattern in the TR 38.901 channel model, an omni-directional radiation pattern as well as parabolic radiation patterns with different beamwidths that is larger than or equal to 65°. Proposal 18 Add to TR 38.901 a stochastic model of antenna imbalance, with random antenna imbalance according to some distribution per antenna or per set of antennas. Proposal 19 Send LS to RAN4 asking for input on stochastic model of antenna imbalance. Proposal 20 The stochastic model of antenna imbalance may be applied to one link direction (e.g., UL) but not the other (e.g., DL), or both. |
| [4] Samsung | Observation 9 If the device considered here is a handheld device (i.e., smartphone), the antenna arrangement according to thickness would not matter much. Proposal 7 RAN1 consider the size of the device to 15 cm x 6 cm x 0 cm as baseline Observation 10 Some readers of channel model might want to know the difference in performance between having the FR1 antenna at the edge versus the corner, which could also apply to the FR2 antenna array. Additionally, whether antenna for new frequency band become antenna elements or arrays remains unknown. Proposal 8 RAN1 do not consider the detailed guidelines for how antenna elements and antenna modules are mapped to antenna location candidates Observation 11 UE antennas have many challenges in implementation compared to BS antennas, so even antennas in the same band can be degraded in terms of performance such as gain and radiation pattern Observation 12 With varying the value of HPBW, it tends that the larger the HPBW becomes, the weaker the directional properties get in both horizontal and vertical planes. And, with varying SLAv and Am_while maintaining HPBW. The lower bound boundary increases in both horizontal and vertical planes. Proposal 9 RAN1 consider the parameters for UE antenna radiation pattern modeling as below: θ3dB ,ϕ3dB∶ 90 – 120 degrees SLAV: 25 – 15 dB Am: 25 – 15 dB G(E,max): 0 – 5 dB Combining method for 3D antenna element pattern: same with TR38.901 Observation 13 The radiation pattern of the dipole antenna pattern, which is option 2 (E(θ) = cos(cos(θ)*π /2) / sin(θ)), may not be ideal. This is because there is a singularity where the value becomes very large if θ=0 or θ=π in sin(θ). Proposal 10 RAN1 discuss the optimization method of option 2 in antenna modeling. Proposal 11 RAN1 discuss the different dipole antenna radiation pattern with E(θ)=sin(π/2*cos(θ)) Proposal 12 When the antenna is mounted on the device (i.e., handheld device), factors such as structure, frame material and placement can reduce the gain in the back lobe direction. Then, the back side radiation component will not radiate as much as the front side radiation component. Proposal 13 RAN1 consider the limited range of θ from -90° to 90° when it comes to E(θ)=sin(π/2*cos(θ) ) |
| [5] vivo, BUPT | Observation 5: The measurement results on the power radiation of each antenna element on the smart user equipment show a power imbalance difference with the range of 3-5dB. Proposal 4: Power imbalance of each antenna element should be introduced to the new UE antenna model to reduce the implementation gap for the real product. Observation 6: The average angle range corresponding to parameters θ3dB and φ3dB of each antenna on the smart user equipment are 115 and 85 degrees, respectively, based on the measurement results. Proposal 5: 115 and 85 degrees can be used to define θ3dB and φ3dB in the new UE antenna model. Observation 7: The average values of each antenna element corresponding to SLAv and Am are 22dB and 26dB, respectively, based on the measurement result. Proposal 6: The value of 25dB can be selected for both SLAv and Am. |
| [6] ZTE, Sanechips | Proposal 10: For UE antenna radiation pattern modeling, the following options can be considered - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element - Option 3) Omni-directional antenna pattern |
| [7] Nvidia | Proposal 3: The size of the device X cm x Y cm x Z cm should include multiple options, including at least (1) 15 cm x 6 cm x 0 cm, and (2) 15 cm x 6 cm x 0.5 cm. Proposal 4: UE antenna radiation pattern modeling should include multiple options, (1) adapting the Table 7.3-1: Radiation power pattern of a single antenna element, (2) half-wavelength dipole, and (3) omni-directional antenna pattern. |
| [10] CATT | Proposal 4: The following options can be considered for UE antenna radiation pattern modeling: - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element Parameter; Values. Antenna element vertical radiation pattern (dB); AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB. Antenna element horizontal radiation pattern (dB); AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB. Combining method for 3D antenna element pattern (dB); FFS. Maximum directional gain of an antenna element GE,max; FFS dBi. - Option 3) omni-directional antenna pattern Proposal 5: In UE antenna modelling, an offset angle shall be introduced in UE antenna radiation pattern for each antenna. Proposal 6: For calibration purpose, the following antenna location candidates can be considered: - For UEs with 8 antennas, 4 antennas are placed in the four corners and the other 4 antennas are at the centres of each edge. - For UEs with 4 antennas, 4 antennas are placed in the four corners or at the centres of each edge. |
| [11] Lenovo | Proposal 1 For handheld UE devices antenna placement modeling, the dimension Z corresponding to the UE depth is omitted, i.e., the value of Z is set to zero. Proposal 2 For handheld UE devices antenna placement modeling, define a default antenna placement model for each of 2,3, 4, 6 and 8 antennas/antenna modules placed on the device. • Other antenna placement locations can be considered. Figure 2: candidate placement for 2, 3, 4, 6 and 8 antennas/antenna modules in a handheld UE device |
| [12] Sony | Observations 1 The antenna polarizations are dominated by linear or elliptical polarization. Figure 6. Measured two major cuts antenna patterns of antenna 1, at the left side of the mobile phone mockup, compared with the antenna model in Table 1 and θ3dB=131∘, SLAV=15 dB, and GE,max=5 dBi. Figure 7. Measured two major cuts antenna patterns of antenna 2, at the upper side of the mobile phone mockup, compared with the antenna model in Table 1 and θ3dB=131∘, SLAV=15 dB, and GE,max=5 dBi. Observation 1 The measured maximum gains and 3-dB beamwidths are close to the values in the model, i.e., 5 dBi and 131 degrees, respectively, with some variation depending on the antenna type and implementation. Observation 2 The antenna model proposed in Table 1 with the parametrization θ3dB=131∘, SLAV=15 dB, and G(E,max)=5 dBi shows a reasonably good agreement with measurements of the antennas of a smartphone mockup at 15 GHz. Observation 3 Since frequency 7-24GHz is a wide range, the antenna in a smartphone chassis may have different behaviors, and more validation measurements or simulations are desirable. Proposal 1 For the new UE antenna model, for handheld devices, the size of the device can be set to (X,Y,Z)=(15,6,0) cm. Proposal 2 Considering that antenna elements and antenna panels of UE devices in the FR1 and FR2 bands are typically co-located in system level simulations, it would be beneficial to clarify why non co-located antenna elements and panels must be considered in the 7–24 GHz frequency range, and what the expected difference is, for system level simulations, with respect to the baseline case (X,Y,Z)=(0,0,0). Observation 5. Measured antenna patterns of a smartphone mockup at 15 GHz show that depending on the antenna element type and location, the direction of maximum radiation goes i) away from the smartphone mockup but contained in the mockup plane, i.e., aligned with the red arrows in the agreement’s box—edge and notch antennas; or ii) away and perpendicular to the plane containing the mockup, i.e., facing backward—antennas. Proposal 3. For the new UE antenna model, for handheld devices, support, at least, aligning the direction of maximum radiation of the antenna filed pattern with either the red arrows in the agreement’s box, or facing backward. Proposal 4. For the new UE antenna model, for handheld devices, for FR1, all the blue boxes in the agreement’s figure can be considered candidates for the location antenna elements. Proposal 5. For the new UE antenna model, for handheld devices, for FR1, consider single polarized antenna elements, with the polarization direction aligned with the long edge of the antenna. For antenna elements located at the top or bottom of the handheld device, the polarization direction can be horizontal (left to right). For antennas on either side, the polarization direction can be vertical (top to bottom). For antenna elements located at the edges, randomly assign vertical or horizontal polarization directions independently to each antenna element. Proposal 6. For UE antenna radiation pattern modeling, support Option 1 and Option 3, to model directional and omni-directional antenna patterns, respectively. Proposal 7. For UE antenna radiation pattern modeling, for Option 1, support at least the parametrization θ3dB=131∘, SLAV=15 dB, and G(E,max)=5 dBi. Figure 8. Measured total efficiencies of the mockup antennas in free space; see also [4]. Observation 6. Measurements show power imbalances between antenna ports of a smartphone mockup at 15 GHz in free space of up to 1.5 dB. Observation 7. To accurately model the total power imbalance between antenna ports, one must consider imbalances between the components of the respective transmit and receive chains, such as different amplification gains or insertion losses. Observation 8. The study on a smartphone multi-antenna mockup for 15 GHz shows losses due to hand grip, and presence of head and body between 1 and 4 dB, approximately. The loss value varies across antenna ports. Proposal 8. Capture power imbalances between antenna ports in free space. Additionally and independently, capture as well losses due to hand grip and presence of head and body. |
| [19] Qualcomm | Figure 5 Exemplary radiation patterns for different choices of θ3dB, ϕ3dB, SLAv, and Amax. - Am = 25 dB, SLAv = 25 dB - ϕ3dB = 180°, θ3dB = 90°. Figure 7 Illustration showing how the parameters for the antenna pattern model in Table 7.3-1 can be used to approximate realistic UE antenna patterns. Proposal 5: At least adopt the half-wavelength dipole antenna model for UE antenna pattern. In addition to the half-wavelength dipole antenna model, if the antenna radiation pattern provided in Table 7.3-1 is reused to model UE antennas, the following values for the parameters can be considered: - Am = 25 dB - SLAv = 25 dB - ϕ3dB = 180° to 270° - θ3dB = 90° - Maximum directional gain is chosen to normalize the antenna pattern to unit power. Additional losses to reflect antenna efficiency may also be included with this parameter. Figure 8 Antenna placement in a CPE modelled as a cube. Proposal 6: Model CPEs as having a cubical form factor with 15-20 cm sides. The antennas are placed along the central horizontal plane and are oriented outwards from the centre of the cube. The relative antenna locations are similar to that of a handheld device. Proposal 7: To model potential differences in antenna gain/efficiency and variations in insertion loss across tx/rx ports, introduce an antenna imbalance factor (in dB) when enhancing the UE antenna modelling framework. |
| [20] Nokia | Proposal 9: Specify that all used reference radiation power patterns for UE antenna modelling must have an efficiency of 0 dB. Table 1: Proposed UE antenna element pattern. Parameter; Values. Antenna element vertical radiation pattern (dB); AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=131°,SLAV=15dB. Antenna element horizontal radiation pattern (dB); AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=131°,Am=15dB. Combining method for 3D antenna element pattern (dB); A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am. Maximum directional gain of an antenna element GE,max; 5 dBi. Proposal 10: Consider evaluation parameters in Table 1 as baseline assumption for UE modelling in 7-24 GHz band. Proposal 11: Specify that at least one reference power patterns for UE modelling has a directional gain of 5 dB. Proposal 12: Split the 9 proposed antenna orientations into three subgroups: A first group for antennas operating at frequencies below 1 GHz (shown with orange areas), a second group for antennas operating at frequencies from 1 GHz to 20 GHz (shown with blue areas) and a third group for antennas operating at frequencies above 20 GHz (shown with green areas). Proposal 13: The reference physical size of the handheld UE should be (X, Y, Z) = (150mm, 70mm, 0mm). Figure 7: Location and orientation for the reference UE radiation gain pattern Proposal 14: RAN1 to assume positive Z-Axis of the phone’s local coordinate system (oriented perpendicular to the face plane of the phone) as a max gain direction/orientation of the UE reference antenna model radiation pattern (as illustrated in Figure 7), with all the gain in the theta field component. Figure 8: Polarized field rotation of the reference UE radiation gain pattern to a first location with a fist orientation. Figure 9: Polarized field rotation of the reference UE radiation gain pattern to a first, second, third and fourth location with a first, second, third and fourth orientation. Observation 5: The impact of an UE antenna being located away from (0,0,0) of the phone’s local coordinate system is already covered by factors of the form expj2πrrx,n,mT⋅drx,uλ0 in equations in TR 38.901, S. 7.5, Step 11 and hence does not require additional explicit handling. Proposal 15: Each polarized field component of the reference radiation pattern should be rotated according to the orientation of the UE antenna candidate locations and based on the orientation of the UE in the global coordinate system using the methods already specified in TR 38.901. |

#### Summary of Issues

Companies provided inputs on UE antenna modeling.

|  | Supported Patterns | For directional raditional pattern |  |  |
| --- | --- | --- | --- | --- |
|  |  | Vertical Radiation Pattern | Horizontal | Max Gain |
| Huawei/HiSilicon | Directional |  |  |  |
| Ericsson | Directional |  |  |  |
| Samsung | Directional, dipole | θ3dB=90° ~ 120° SLAV=15 dB ~ 25 dB | ϕ3dB=90°~120° Am=15 dB ~ 25 dB |  |
| vivo, BUPT | Directional | θ3dB=115° SLAV=22 dB | ϕ3dB=85° Am=26 dB |  |
| ZTE, Sanechip | Directional, omni |  |  |  |
| CATT | Directional |  |  |  |
| Sony | Directional | θ3dB=131° SLAV=15 dB | ϕ3dB=? Am=? | 5 dBi |
| Qualcomm | Directional, dipole | θ3dB=90° SLAV=25 dB | ϕ3dB=180°~270° Am=25 dB |  |
| Nokia | Directional | θ3dB=131° SLAV=15 dB | ϕ3dB=131° Am=15 dB | 5dBi |

##### Proposal #4-1

For handheld devices adopt the following device dimensions for UE antenna modeling:

- 15 cm x 6 cm x 0 cm

##### Proposal #4-2

For CPE devices adopt the following device dimensions for UE antenna modeling:

- 18 cm x 18 cm x 18 cm

- The antennas are placed along the central horizontal plane and are oriented outwards from the centre of the cube. The relative antenna locations are similar to that of a handheld device.

##### Proposal #4-3

Further refine previous agreement on UE antenna radiation pattern modeling as follows:

- For UE antenna radiation pattern modeling, consider the following options for further study:

  - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS 5 dBi |

  - Option 2) half-wavelength dipole.

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

      - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

  - Option 3) omni-directional antenna pattern

  - Other options

Proposal #4-4

For UE antenna modeling of handheld devices,

- Introduce power imbalance modeling to the UE antenna model.

- FFS details of antenna power imbalance

##### Proposal #4-5

For UE antenna modeling of handheld devices,

- Default antenna placements for 2, 3, 4, 6, and 8 antenna elements are defined as follows:

#### Round #1 Discussion

Please provide comments on issues regarding UE antenna model. Please provide comments on Proposal #4-1.

| Company | Comments |
| --- | --- |
| InterDigital | Proposal 4.3: Support Proposal 4.1, 4.2, 4.4, 4.5: We prefer to deprioritize the discussion, as we don’t see any meaningful outcome from the discussion. |
| Apple | Proposal #4-1: Okay Proposal #4-2: Unclear the need of considering CPE device separately. Proposal #4-4: This is related to individual smart phone design, and we prefer not to consider it. ##### Proposal #4-5: This is related to individual smart phone design, and we prefer not to consider it. |
| vivo | We are fine with the proposal, but Proposal #4-2 for CPE. I don’t fully understand the necessity of modeling CPE in this SI. |
| Moderator | Just to clarify, it would be ideal to down-select among options in this meeting (if possible). |
| Huawei, HiSilicon | Proposal #4-1: Fine. Proposal #4-2: Seems the centre of CPE is not appropriate for placing antenna element/module. Proposal #4-3: Fine. Proposal #4-4: Fine. Proposal #4-5: Centres of each edge should also be considered for 4 antenna case. |
| Samsung | Proposal #4-1: Support Proposal #4-2: Need to check it can be a representative shape for CPE. Proposal #4-3: We are open to discuss. Proposal #4-4: We consider that it is out of scope in channel model Proposal #4-5: It is better to leave it to the channel model user's degree of freedom. |
| LGE | Proposal #4-1: Support. Proposal #4-2: Low priority Proposal #4-3: Fine. Proposal #4-4: Fine Proposal #4-5: Need more discussion. |
| QC | Proposal 4-1: Okay Proposal 4-2: Okay Proposal 4-3: Okay Proposal 4-4: Support Proposal 4-5: This may be okay for FR1 UEs. We may need a separate agreement/discussion for FR2. Also, in the above, antenna locations in the top half tend to be more likely due to battery placement in a smartphone. May be good to skew towards using antennas in the top half for 2/4/6 antenna cases. Might be good to take more input from OEMs regarding this. |
| CATT | Proposal #4-1: ok. Proposal #4-2: need FFS Proposal #4-3: Fine. Proposal #4-4: No. Proposal #4-5: Ok |
| ZTE | For Proposal #4-4, it’s more of RAN4 discussion. For Proposal #4-5, we understand the spirit of the proposal to define reference antenna locations for future discussion, but the provided examples may not be appropriate, e.g. for 2 antennas case, it’s better to put the antennas at the above 2 corners. |
| Sony | Proposal #4-1: Support. Proposal #4-2: Not sure why we need to consider a cubic shape when all the antenna ports are coplanar. Also, as other companies mentioned, the antenna in the center of the cube seems to be completely blocked. Proposal #4-3: Support. Proposal #4-4: Support. Proposal #4-5: Support in principle. We tend to agree with QC’s comment that antenna placement can be skewed toward the upper half. For example, for 4 antenna ports, one could use the two upper edges and the two middle sides. |

#### Summary of Round #1 Discussion

Proposal #4-1 (handheld device dimension): Interdigital questioned the need for the proposal. All other companies seems to be generally ok with proposal.

Proposal #4-2 (CPE device dimension): few companies questioned the need for CPE device type dimension definition. One company questioned the location of the antenna, one companies wanted to further check on the dimension size.

Proposal #4-3 (refinement of UE ant radiation pattern):

Proposal #4-4 (power imbalance modeling): Few companies comments not to consider or out of scope, few companies supported this, one company thought this is an RAN4 issue.

Proposal #4-5 (default antenna element): Few companies had alternative suggestions for locations, and few companies wanted to leave the default locations open so that it can be looked into in the future.

#### Outcome of Tue Session

Agreement

For handheld devices adopt the following device dimensions for UE antenna modeling:

- 15 cm x 7 cm x 0 cm

#### Round #2/3 Discussion

Moderator suggest further discussion on Proposal #4-2A, #4-3 (including additional down-selection of the options), #4-4, #4-5A

Proposal #4-2A

Further study device dimension and antenna placement applicability for customer premise equipment (CPE) devices, include the need for defining CPUE device type dimensions.

For CPE devices adopt the following device dimensions for UE antenna modeling:

- 18 cm x 18 cm x 18 cm

- The antennas are placed along the central horizontal plane and are oriented outwards from the centre of the cube. The relative antenna locations are similar to that of a handheld device.

Proposal #4-3

Further refine previous agreement on UE antenna radiation pattern modeling as follows:

- For UE antenna radiation pattern modeling, consider the following options for further study:

  - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS 5 dBi |

  - Option 2) half-wavelength dipole.

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

      - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

  - Option 3) omni-directional antenna pattern

  - Other options

Proposal #4-4

For UE antenna modeling of handheld devices,

- Introduce power imbalance modeling to the UE antenna model.

- FFS details of antenna power imbalance

##### Proposal #4-5A

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for 2, 3, 4, 6, and 8 antenna elements, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

- Default antenna placements for 2, 3, 4, 6, and 8 antenna elements are defined as follows:

##### Proposal #4-5B

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

  - FFS on handling of antenna array panels

- Default antenna placements for 2, 3, 4, 6, and 8 antenna elements are defined as follows:

Proposal #4-6

For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations.

- For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

| Company | Comments |
| --- | --- |
| Nokia | Proposal #4-1: the proposed dimensions are too narrow. The typical aspect ratio of displays currently implemented on high-end smartphone is approximately 2.2. Hence, it should be (15cm x 7cm, 0cm). Proposal #4-3: Support Option 3 Proposal #4-5: It is necessary to clarify that these antenna placements are considred for below 20GHz (i.e., not for antenna arrays). |
| Apple | ##### Proposal #4-2A: We still do not think this proposal is needed. ##### Proposal #4-5A: We still do not think this proposal is needed. |
| Moderator | ##### Updated the proposal based on feedback. For Proposal 4-2A and 4-5B, the proposal is to study. Moderator was not sure what the concern from Apple was. It might be good if Apple can provide further information on why they think even the study is useful. Please note proposal #4-4 may require bit more discussion. Added Proposal #4-6 based on offline feedback from Nokia. |
| Nokia | ##### At the previous meeting we already agreed the following: ##### The four corners and the centres of each edge are identified as potential locations of the UE antenna @Apple, if we do not agree on any assumption on antenna placement it will not be possible to align the results in between the companies. These typical antenna locations are proposed for calibration purposes only. Othe antenna location should not be precluded, of caurse. We have a few additional comments on Proposal #4-5B. - Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2. - FFS on handling of antenna array panels in FR2 - Option 1: UE candidate antenna locations for calibration: - Orange for below 1 Ghz (2 antenna) - Blue for 1 GHz – 20GHz (4) antenna - Green for above 20GHz, i.e., for antenna panel arrays - Other options are not precluded |

#### Summary of #3 Discussion

Moderato has made updated to proposal #4-5 based on comments from Nokia.

##### Proposal #4-2A

Further study device dimension and antenna placement applicability for customer premise equipment (CPE) devices, include the need for defining CPUE device type dimensions.

##### Proposal #4-3A

Further refine previous agreement on UE antenna radiation pattern modeling as follows:

- For UE antenna radiation pattern modeling, consider the following options for further study:

  - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS dBi |
| Note: max direction gain and half power beam widths should be selected such that total antenna efficiency should be equal or less than 0dBi. |  |

  - Option 2) half-wavelength dipole.

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

      - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

  - Option 3) omni-directional antenna pattern

  - Other options

##### Proposal #4-4

For UE antenna modeling of handheld devices,

- Introduce power imbalance modeling to the UE antenna model.

- FFS details of antenna power imbalance

##### Proposal #4-5C

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

  - FFS on handling of antenna array panels

  - Examples of candidate antenna locations for calibration

    - Same default location for different frequency bands

    - Different default location for different frequency bands:

      - For below 1GHz

        - Top/bottom (Orange) locations for 2 antenna

      - For 1 ~ 20 GHz

        - 4 corners (Blue) locations for 4 antenna

      - Above 20 GHz

##### Proposal #4-6

For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations.

- For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

#### Round #4 Discussion

Please comment on Proposal #4-2A, #4-3A, #4-4, #4-5C, and #4-6.

| Company | Comments |
| --- | --- |
| - | - |

#### Outcome of Thursday Session

Agreement

Further study device dimension and antenna placement applicability for customer premise equipment (CPE) devices, include the need for defining CPE device type dimensions.

Agreement

Further refine previous agreement  (made in RAN1#118bis)  on UE antenna radiation patternmodeling as follows:

- For UE antenna radiation pattern modeling, consider the following options for further study:

  - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS dBi |
| Note: max direction gain and half power beam widths should be selected such that total antenna efficiency should be equal or less than 0dBi. |  |

  - Option 2) half-wavelength dipole.

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

      - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

  - Option 3) omni-directional antenna pattern

  - Other options

Agreement

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

  - FFS on handling of antenna array panels

Agreement

For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations.

- For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

## 4.5 Other Modeling Aspects

| Company | Proposals & Observations |
| --- | --- |
| [8] Nvidia | Observation 1: Wireless channel modelling needs to provide consistency and, above all, a correct representation of the frequency, spatial, and temporal correlation across base stations, devices, and objects in the environment. Observation 2: Deterministic, physics-based modelling for wireless propagation, especially ray tracing, are essential for studying, evaluating, and developing key technologies in 5G-Advanced toward 6G, including ISAC, RIS, larger antenna arrays in new spectrum such as 7-24 GHz and sub-THz bands, AI/ML, etc. Observation 3: Task Group IEEE 802.11bf has embraced ray tracing based channel model for WiFi sensing. Observation 4: Ray tracing simulations offer a valuable complement by providing cost-effective, controlled, and flexible tools for studying signal propagation characteristics in diverse scenarios. Proposal 5: Complement field measurements with ray tracing simulations to validate the channel model of TR38.901 at least for 7-24 GHz. |
| [20] Nokia | The only justified way to validate the changes in the existing models is to compare the measurements at different frequencies (in the whole 0.5-100GHz range) in the same location and conditions. Otherwise, the differences in between the existing model parameters and results of new measurements are unavoidable and it is impossible to infer whether the difference is due to the different frequency or scenario. Moreover, the significance of system-level impact of these potential changes is not obvious and requires thorough validation. The maintenance of frequency continuity outside of 7-24GHz requires either re-evaluation of all measurements in the whole 0.5-100GHz range or interpolation of the parameters around 7 and 24GHz what is not physical. |

#### Summary of Issues

Moderator will capture any other modeling aspects that could not be categorized above and missing from the initial version of the discussion document.

#### Round #1 Discussion

Please provide comments on other modeling issues.

| Company | Comments |
| --- | --- |
| - | - |

#### Summary of Discussion

No further inputs were provided for other modeling issues. Discussion is closed for this meeting.

## 4.6 Capturing measurement data

Companies are asked to provide inputs to the data source collection based on the template provided in R1-2403969.

Each company may update the excel sheet in ftp://tsg_ran/WG1_RL1/TSGR1_119/Inbox/drafts/9.8(FS_NR_7_24GHz_CHmod)/source data collection

Moderator will time to time clean up the excel sheet updates and ask companies to clarify information. The following table will be used to request updates or clarifications to the companies. Company do not need to fill in the table unless there is a request from the moderator.

| Data Entry Row | Company | Moderator Comment/Request for Update/Clarification | Company Response |
| --- | --- | --- | --- |
| - | - | - | - |

# Summary of Agreements/Conclusions from RAN1 #118bis

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- Building type ratio: 90% residential, 10% commercial

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT Vertical Distribution Option 2: uniform distribution across all floors for a building type

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 40 km/h

Agreement

For handheld devices adopt the following device dimensions for UE antenna modeling:

- 15 cm x 7 cm x 0 cm

Conclusion

For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable). For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

- Note: If deemed important, it should be still possible to discuss how specific channel model parameters need to be updated.

Conclusion

- Based on measurement data provided, RAN1 concludes pathloss models in TR38.901 for the following scenarios are validated and no updates to TR are made.

  - InH-Office LOS

  - InH-Office NLOS

  - RMa LOS

  - RMa NLOS

  - InF LOS

  - InF NLOS

Observation

- For the following scenarios, some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR.

  - InF LOS

  - InF NLOS

Conclusion

- For the following scenarios, there is no consensus to update delay models due to lack of consistent and significant observed difference between model and measurements.

  - InF LOS

  - InF NLOS

Agreement

- Study further on the need for introduction of intra-cluster power profile.

  - Potential options for introduction of intra-cluster power profile:

    - Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724)

    - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

Agreement

Further study penetration loss modeling, including the following aspects:

- Need for change and details of the updates to the material penetration loss, for example, if any

|  | Concrete | Wood |
| --- | --- | --- |
| Existing TR | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |
| Tconcrete and Twood represent reference (default) thickness of the material. For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively. |  |  |

  - Whether penetration loss can be scaled linearly with thickness for each material

  - For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

  - FFS the impact of updates to material penetration loss to total build penetration loss

- Need for change and details of total building penetration loss, e.g. low-loss and high-loss model, if any

- Impact of material composition on material penetration loss

Agreement

- For the following scenarios, further study path loss model parameters for at least 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

Agreement

- For the following scenarios, further study delay spread parameters for at least 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - InH-Office LOS

  - InH-Office NLOS

  - UMa LOS

  - UMa NLOS

Agreement

- Based on measurement data provided, RAN1 concludes angular spread parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

Observation

- For the following scenarios, some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed angular spread consistent with the model in the TR.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Conclusion

- For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Agreement

- For the following scenarios, further study angular spread parameters for at least 6 – 24 GHz frequency range.

  - InH LOS/NLOS: ASA, ASD

  - UMi LOS: ASA, ASD

  - UMi NLOS: ASA, ASD, ZSA

  - UMa LOS: ASD, ASA

  - UMa NLOS: ASA, ASD, ZSA

  - InF LOS and NLOS: ASD, ASA, ZSA, ZSD

Agreement

- Further study number of clusters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6GHz and above 24 GHz, including whether to update values for other frequencies based on measurements

  - Whether changes to number of clusters result in meaningful impact to system performances and evaluation of features

Conclusion

- Based on measurement data provided, RAN1 concludes shadow fading parameters in TR38.901for the following scenarios are validated and no updates to TR are made.

  - InH LOS

  - InH NLOS

  - UMi NLOS

Observation

- For the following scenarios, some sources have observed some differences in shadow fading in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed shadow fading with the model in the TR.

  - UMi LOS

Conclusion

- For the following scenarios, there is no consensus to update shadow fading due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

Agreement

- Further study the need for updating the K-factor parameters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6 GHz and above 24 GHz, including whether to update values for other frequencies based on measurements.

  - Whether changes to K-Factor result in meaningful impact to system performances and evaluation of features.

Working Assumption

- Adopt the following absolute delay for UMi and UMa scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

Agreement

- Introduce absolute delay for InH scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - FFS details of the parameter for InH. Consider the following for further study.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Agreement

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

  - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180].

- Down-select among options.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2a)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 2b)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s is chosen such that AS(ϕn,intermediate) = AS_desired

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

    - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread.

- FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

Agreement

For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35]m.

Agreement

For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

Agreement

For suburban scenario, further study the LOS probability equation.

- Example of LOS probability equation for study:

  - PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, where dm, α, β, γ, δ are modeling parameters

- Study includes whether distinction of LOS probability should be made for UEs in residential and commercial buildings, which have different building heights.

- Study includes whether or not vegetation should be explicitly considered and modeled for determining LOS probability, if determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

Agreement

Further study the need of additional low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario.

- Potential example of the new low loss O2I building penetration loss:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

Agreement

- UMa can also be used for other ISDs between 200m and 500m, including the ISD proposed in ITU-R M.2412 -0

- One ISD value will be selected for calibration with ISD = 500m

- No change to any other parameters due to other ISD values

- FFS: How to reflect it in TR38.901

Agreement

Capture channel modeling updates regarding suburban macrocell scenario as follows:

- Define a new scenario, e.g., SMa

- The key differentiating aspects of SMa (compared to UMa and RMa) in high level description will be highlighted in Section “6.2 Scenario of Interest” of the TR.

Agreement

Further study device dimension and antenna placement applicability for customer premise equipment (CPE) devices, include the need for defining CPE device type dimensions.

Agreement

Further refine previous agreement  (made in RAN1#118bis)  on UE antenna radiation patternmodeling as follows:

- For UE antenna radiation pattern modeling, consider the following options for further study:

  - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS dBi |
| Note: max direction gain and half power beam widths should be selected such that total antenna efficiency should be equal or less than 0dBi. |  |

  - Option 2) half-wavelength dipole.

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

      - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

  - Option 3) omni-directional antenna pattern

  - Other options

Agreement

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

  - FFS on handling of antenna array panels

Agreement

For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations.

- For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

# Reference

- R1-2409402, “Considerations on the 7-24GHz channel model validation,” Huawei, HiSilicon

- R1-2409441, “Discussion on validation of channel model,” Ericsson

- R1-2409467, “Further Discussion on Angle Scaling for MIMO CDL Channel,” InterDigital, Inc.

- R1-2409610, “Discussion on channel model validation of TR38.901 for 7 - 24 GHz,” Samsung

- R1-2409694, “Views on channel model validation of TR38.901 for 7-24GHz,” vivo, BUPT

- R1-2409724, “Discussion on the channel model validation,” ZTE Corporation, Sanechips

- R1-2409738, “Discussion on channel modeling verification for 7-24 GHz,” Intel Corporation

- R1-2409778, “Channel model validation of TR 38901 for 7-24 GHz,” NVIDIA

- R1-2409819, “Validation of Channel Model,” Apple

- R1-2409954, “Discussion on channel model validation for 7-24GHz,” CATT

- R1-2410015, “Channel model validation for 7-24 GHz,” Lenovo

- R1-2410213, “Discussion on channel model validation of TR38.901 for 7-24GHz,” BUPT

- R1-2410236, “Further Discussion of channel model validation of TR38.901 for 7–24GHz,” Sony

- R1-2410254, “Instantaneous directional channel measurements and parameter estimation at 14 GHz,” ROHDE & SCHWARZ

- R1-2410299, “Measurements of the angular spreads in a urban and suburban macrocells,” Vodafone, Ericsson

- R1-2410319, “Channel model validation of TR38.901 for 7-24 GHz,” Sharp, Nokia

- R1-2410655, “Discussion on Validation of the Channel Model in 38901,” AT&T

- R1-2410402, “Discussion on channel model validation for 7-24 GHz,” NTT DOCOMO, INC.

- R1-2410491, “Channel Model Validation of TR38.901 for 7-24 GHz,” Qualcomm Incorporated

- R1-2410594, “Discussion on Channel model validation of TR38.901 for 7-24GHz,” Nokia

# Appendix A: RAN1 Agreements

## RAN1 #116-bis (April-2023)

Conclusion

- To provide measurement data, and/or simulation results, and/or available publications with measurement information for frequencies 7 to 24 GHz to validate/update the channel model.

- For frequency continuity of the channel models, Measurement information outside 7 to 24 GHz is also encouraged

Agreement

The following provides list of modelling parameters for 7 – 24 GHz frequencies that could be further studied for validation. The parameters listed are starting point for further discussions and does not imply the parameters require validation nor imply parameters require updates for 7 – 24 GHz frequencies.

- Antenna modelling parameters (e.g. radiation power patterns, directional gain values, etc.)

- Pathloss

- LOS probability

- O-to-I penetration loss

- Delay spread (mean, variance)

- AoD spread (mean, variance)

- AoA spread (mean, variance)

- ZoA spread (mean, variance)

- ZoD spread (mean, variance)

- ZoD offset

- Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions)

- Shadow fading

- K factor (mean, variance)

- LSP cross correlations

- Delay scaling parameter

- XPR

- Number of clusters

- Number of rays per cluster

- Cluster delay spread

- Cluster ASD

- Cluster ASA

- Cluster ZSD

- Cluster ZSA

- Per Cluster shadowing

- Correlation distances

- LSP correlation type (e.g. site-specific or all correlated)

- Oxygen absorption

- Correlation distance for spatial consistency

- Blockage region parameters/blocker parameters

- Spatial correlation for blockages

- Material properties for ground reflector model

- Spatial consistency model A/B

Conclusion

RAN1 to continue discussion on the need for new modelling parameters/scenarios and modelling procedure. The following modelling parameters/aspects for 7 – 24 GHz frequencies that are currently not available in TR38.901 have been identified by companies in RAN1#116bis. At least the following is for further study, but does not imply parameters/scenarios and modelling procedure are required for 7 – 24 GHz frequencies.

- Intra-cluster K factor

- Random power variability in each polarization

- Addition of SMa deployment scenario

Conclusion

- RAN1 to compile measurement/simulation descriptions from companies into a Tdoc to be added as reference to TR38.901.

  - Rapporteur to update the Tdoc in each meeting based on inputs from companies.

- Rapporteurs to provide a template for the measurement/simulation descriptions capture to RAN1 #117 for initial review and endorsement.

## RAN1 #117 (May-2024)

Agreement

To check and review the following results and measurement data provided in RAN1 #117 and RAN1#116bis for further discussion in next RAN1 meeting. R1-2405646 contains the list of data sources for the results and measurements provided in RAN1 #117.

- measurements for penetration loss for various materials, including drywall/wood, clear glass, IRR glass, and concrete

- measurements for pathloss for following scenarios: InH_office LOS, InH-Office NLOS, InF LOS, InF NLOS, UMi LOS, UMi NLOS, UMa LOS, UMa NLOS, [Outdoor courtyard], RMa LOS, RMA NLOS, SMa NLOS

- measurements for polarization for UMa deployment scenario

- measurements for DS for following scenarios: InH-Office LOS, InH-Office NLOS, UMi LOS, UMI NLOS, UMa LOS, UMa NLOS, InF LOS, Inf NLOS.

- Measurements for angular distributions, such as ZOD, ZOA, AOD, AOA for following scenarios: InH, UMi, UMa

- Measurements for number of clusters for following scenarios: InH, UMi

- Simulations for LOS probability for SMa deployment scenario

- Measurement results regarding near-field model for following deployment scenarios: InH-Office LoS, UMa

- Measurement results regarding spatial non-stationarity for following deployment scenarios: UMa, [UE side]

- Simulation results regarding spatial non-stationarity for UMa deployment scenario

Observation

- Some companies provided information that sub-urban deployments cannot be represented by existing deployments in TR38.901 (such as UMi, UMa, RMa).

Conclusion

The following parameters are used as a starting point for aligning companies understanding of channel model parameters related to suburban use cases.

- BS height: [22.5] m

- Layout:	Hexagonal grid, 19 Macro sites, 3 sectors per site, ISD = [1732] m

- Typical building heights: [Up to two floors for residential buildings, up to five floors for commercial buildings]

- UT height: [1.5 or 4.5 m for residential buildings], [1.5/4.5/7.5/10.5/13.5 m for commercial buildings]

- UT distribution: [Uniform horizontally, 70% indoor residential users are on ground floor, 30% are on upper floor]

- FFS: ratio between residential and commercial buildings

- Indoor/Outdoor: [80% indoor and 20% outdoor, FFS on in-car users]

- LOS/NLOS: LOS and NLOS

- Min BS - UT distance(2D): [25] m

Conclusion

- To provide information about motivation and reasons why changes to the channel model are essential.

Agreement

- Further study whether the following parameters for existing deployment scenarios is necessary to be updated:

  - Delay spread, pathloss, penetration loss, AoD/AoA/ZoA/Zod spreads, ZoD offset, Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions), XPR, number of clusters, number of rays per cluster, LSP correlations type (e.g. site-specific or all correlated), UE antenna modeling parameters, K factor (mean, variance)

- Study of updates to other parameters are not precluded and subject to further study.

Agreement

- Further study whether/how to reflect absolute delay between links, or whether/how correlation type of the delay needs to be changed from site-specific to all-correlated type in the model

  - Note: site-specific and all-correlated definitions are provided in TR38.901 Section 7.6.3.4.

  - FFS: impact of ISD on correlation type for the deployment scenario

Agreement

- Further study on correcting the scaling of the angles and other alternative to address angle scaling in TR38.901 Section 7.7.5 to enable accurate desired angle spread.

Agreement

- Further study of whether/how to model the variability of the co- and cross polar powers, in both diagonal and anti-diagonals of the polarization matrix, in the TR 38.901 model.

  - FFS: variability is applied for per ray or per cluster or per link

  - FFS: impact of antenna configurations

  - For example, variability may be random with an i.i.d. zero-mean Gaussian with some standard deviation, via changes to step 9 and eq (7.5-22) and (7.5-28) in clause 7.5 in TR 38.901.

Agreement

- Further study whether/how to model intra-cluster K factor to the TR38.901 models, such as power re-normalization among intra-cluster rays of a cluster so that first intra-cluster ray has K times more average power compared to rest of the intra-cluster rays.

  - FFS: whether same or different intra-cluster K factor is applied for each clusters

  - FFS: which applicable deployment scenarios

Agreement

- Further study whether/how following UE antenna modelling aspects should be considered in the modelling:

  - UE antenna placement, e.g. placement along edges of a rectangle reflecting UE form factor,

  - UE antenna orientation of individual antenna elements, e.g. randomize UE antenna element orientation,

  - Antenna radiation pattern, e.g. consider more realistic antenna patterns, including a phase component, potential reuse the parabolic pattern,

  - Antenna imbalance

- Note: this is only used for calibration.

## RAN1 #118 (August-2024)

Agreement

- Enable necessary model to support “sub-urban macro deployments”. Scenario of interest can be implemented when necessary by one of the following options:

  - Option-1: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments.

  - Option-2: Define a new scenario, e.g., SMa.

  - FFS parameter commonality and differences between UMa and scenario of interest

  - FFS whether and how to enable frequency continuity beyond 7-24 GHz for scenario of interest

Agreement

- At least for calibration purposes, introduce new UE antenna model that potentially provides updates to following parameters:

  - UE antenna placement

    - E.g. placement along edges of a rectangle reflecting UE form factor.

  - UE antenna orientation

    - E.g. randomize UE antenna orientation

  - Antenna radiation pattern

    - E.g. consider more realistic antenna patterns, including a phase component

    - Consider reusing the parabolic pattern

- Note: not all parameters are necessarily updated from current calibration antenna model.

Agreement

- At least for calibration purposes, for a new UE antenna model, additionally

  - FFS: Antenna imbalance

Observation

- ‘standard’ Glass penetration loss model in TR38.901 seems to be aligned well with measurements in 7-24 GHz conducted by companies.

- Some exceptional cases are wood, concrete and IRR glass penetration losses. It should be noted that assumption of at least material thickness has impact to penetration loss and further alignment of at least material thickness is useful for further validation.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on penetration loss at least for the wood, concrete and IRR glass penetration loss and provide details of experimental setup used for penetration loss measurements.

Observation

- Preliminary study shows pathloss updates may not be needed at least for the following scenarios:

  - InH-Office LOS/NLOS, UMi Street Canyon LOS/NLOS, InF LOS/NLOS, RMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on at least pathloss for the following applicable scenarios, UMa LOS/NLOS.

Observation

- Preliminary study shows delay spread updates may not be needed at least for the following scenarios:

  - InH-Office NLOS

- Preliminary study shows delay spread updates may be needed at least for the following scenarios:

  - UMi LOS/NLOS

- Preliminary study shows different conclusions from different sources on whether delay spread updates are needed at least for following scenarios and further study and validation is needed.

  - InH-Office LOS

  - UMa LOS/NLOS/O2I

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on at least delay spread for applicable scenarios, including any frequency dependency analysis of delay spread.

- Companies are encouraged to provide methodology of frequency dependence analysis of delay spread, if performed

Observation

- Preliminary study shows some updates may be needed for azimuth and zenith angular spread for at least following scenarios:

  - InH LOS/NLOS, UMi LOS/NLOS, UMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on angular spread for applicable scenarios. The following are preliminary examples for identified scenarios:

| Scenario |  | InH @10 GHz |  | UMi @10 GHz |  | UMa @13 GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| ASD lgASD=log10(ASD/1°) | μlgASD | 1.21 | 1.27 | 1.04 | 1.14 | 1.08 | 1.25 |
|  | σlgASD | 0.18 | 0.14 | 0.2 | 0.07 | 0.21 | 0.3 |
| ASA lgASA=log10(ASA/1°) | μlgASA | 1.29 | 1.5 | 1.19 | 1.37 | - | - |
|  | σlgASA | 0.13 | 0.23 | 0.13 | 0.08 | - | - |

| Scenario |  | InH @10.1 GHz |  | UMi @10.1 GHz |  | UMi @ 10GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| ASD | μlgASD | 8.3 | 24.0 | 16.6 | 22.8 | 36.7 | 17.4 |
|  | σlgASD | 4.8 | 13.7 | 15.8 | 19.3 | 20.4 | 24.0 |
| ASA | μlgASA | 28.4 | 47.6 | 32.8 | 60.2 | 19.2 | 31.7 |
|  | σlgASA | 7.3 | 20.6 | 16.0 | 12.6 | 11.0 | 9.3 |
| ZSD | μlgASD | 10.5 | 6.6 | 6.8 | 7.9 | - | - |
|  | σlgASD | 8.6 | 10.5 | 2.8 | 1.3 | - | - |
| ZSA | μlgASA | 4.4 | 8.3 | 13.5 | 12.6 | - | - |
|  | σlgASA | 1.8 | 6.2 | 3.2 | 3.8 | - | - |

| Scenarios |  | UMa @ 3.5 GHz, 13 GHz, 28 GHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| ASD lgASD=log10(ASD/1) | µlgASD | 0.39 + 0.1114 log10(fc) | 0.83 - 0.1144 log10(fc) | 0.58 |
|  | σlgASD | 0.4 | 0.7 | 0.7 |
| Cluster ASD () in [deg] |  | 1.5 | 1.5 | 1.5 |

| Scenario |  | InH @15 GHz |  |
| --- | --- | --- | --- |
|  |  | LOS | NLOS |
| ASA lgASA=log10(ASA/1°) | μlgASA | 1.57 | 1.78 |
|  | σlgASA | 0.15 | 0.15 |
| ZSA lgASA=log10(ZSA/1°) | μlgASA | 0.94 | 0.94 |
|  | σlgASA | 0.05 | 0.06 |

| Scenario |  | UMa @6.5 GHz |  |
| --- | --- | --- | --- |
|  |  | LOS | NLOS |
| ASD lgASD=log10(ASD/1°) | μlgASD | 0.82 | 1.26 |
|  | σlgASD | 0.28 | 0.27 |
| AOA spread (ASA) lgASA=log10(ASA/1°) | μlgASA | 1.67 | 1.72 |
|  | σlgASA | 0.19 | 0.15 |

Observation

- Preliminary study shows some updates may be needed for number of cluster and/or the threshold for removing lower powered clusters in the channel model for at least following scenarios:

  - InH LOS/NLOS, UMi LOS/NLOS, UMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of number of clusters for applicable scenarios.

- The following are preliminary examples for identified scenarios:

  - InH LOS/NLOS number of clusters

    - LOS @ 10 GHz, 500 MHz measurement BW: 15 ⇒ 10

    - LOS @ 10.1 GHz, 500 MHz measurement BW: 15 ⇒ 6

    - NLOS @ 10 GHz, 500 MHz measurement BW: 19 ⇒ 11

  - UMi LOS/NLOS number of clusters

    - LOS @ 10 GHz, 500 MHz measurement BW: 12 ⇒ 5

    - LOS @ 10.1 GHz, 500 MHz measurement BW: 12 ⇒ 8

    - NLOS @ 10 GHz, 500 MHz measurement BW: 19 ⇒ 7

    - NLOS @ 10.1 GHz, 500 MHz measurement BW: 19 ⇒ 10

  - UMa LOS/NLOS number of clusters

    - LOS @ 13 GHz, 400 MHz measurement BW: 12 ⇒ 9

    - NLOS @ 13 GHz, 400 MHz measurement BW: 20 ⇒ 14

- Companies are encouraged to disclose measurement methodology and conditions for determining number of clusters, such as measurement noise floor, capture dynamic range, measurement bandwidth, angular/spatial resolution, etc.

Observation

- 1 source observed existence of dominant angle for each cluster angular power distribution in 13 GHz UMa NLOS scenario. The source noted that dominant angle representation of the channel may be done by introducing an intra-cluster power factor.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

- The following is an example of potential changes:

Hu,s,nNLOSt=ICP∙PnICP+1Frx,u,θθn,1,ZOA,ϕn,1,AOAFrx,u,ϕθn,1,ZOA,ϕn,1,AOATexpjΦn,1θθκn,1-1expjΦn,1θϕκn,1-1expjΦn,1ϕθexpjΦn,1ϕϕFtx,s,θθn,1,ZOD,ϕn,1,AODFtx,s,ϕθn,1,ZOD,ϕn,1,AOD

expj2πrrx,n,1T∙drx,uλ0expj2πrtx,n,1T∙dtx,sλ0expj2πrrx,n,1T∙vtλ0

+PnICP+1M-1m=2MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD

expj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vtλ0

where ICP is the intra cluster power ratio.

Conclusion

- To potentially reflect the channel angular domain sparsity, continue study of unequal intra-cluster power distribution for applicable scenarios.

Agreement

To check and review the following results and measurement data provided in RAN1 #118 for further discussion in next RAN1 meeting. R1-2407251 contains the list of data sources for the results and measurements provided in RAN1 #118, RAN1 #117, and RAN1 #116-bis.

Observation

- Preliminary study shows LOS probability do not need to be updated for the frequency range of interest for all existing scenarios. Further study is needed on LOS probability if existing scenario building/node deployment statistics are updated or if new deployment scenario(s) are adopted.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Observation

- 3 source observed variability in power between co-polarized and cross-polarized antennas.

  - Among the 3 sources, 1 source observed ground reflection model in 38.901 may be used to introduce the polarization variability.

- 1 source observed variability in power between co-polarized and cross-polarized antennas for fix antenna setup. The source noted that UE relative polarization is not fixed in practice and questioned the applicability of the measurements for deployments of interest.

- 1 source observed that power ratio for co-polarization and cross-polarization is influenced by the depolarization effects of the environment and the antenna assumptions. Furthermore, under ±45° antenna slant assumption, the ratios of cross-polarization received power to co-polarization received power have a consistent mean value and therefore existing polarization modeling in TR 38.901 is sufficient.

Conclusion

- Continue study on handling of polarization for applicable scenarios.

Observation

- Preliminary study shows inconclusive results on whether updates may be needed for shadow fading for pathloss parameters of the channel model.

  - 1 source observed shadow fading values, are frequency-independent based on measurements at 6.75, 16.95, 28, and 73 GHz for InH-Office scenario. The discrepancies in shadow fading values are less than 0.3 dB for LOS and less than 2 dB for NLOS channel conditions across the entire frequency range of 0.5-100 GHz.

  - 1 sourced observed frequency dependency of the shadow fading in UMa LOS/NLOS scenario in 7-24 GHz, which the current channel modeling does not have.

- Preliminary study shows inconclusive results on whether updates may be needed for shadow fading per cluster parameters of the channel model.

  - 1 source observed 3dB reduced per cluster shadowing standard deviation for UMi LOS scenario, 1 dB reduced per cluster shadowing standard deviation for InH LOS scenario.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on handling of shadow fading parameters for applicable scenarios.

Observation:

- Preliminary study shows some updates may be needed for K-Factor parameters of the channel model for at least following scenarios:

  - InH LOS, UMi LOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of K-factor parameters for applicable scenarios. The following are preliminary examples for identified scenarios:

  - InH LOS K-factor

    - Mean 9, std-dev 5 ⇒ Mean 5.1, std-dev 3.2

  - UMi LOS K-factor

    - Mean 7, std-dev 4 ⇒ Mean 8.5, std-dev 3.5

Observation:

- Inconclusive on whether CDL model angle calculation update is needed.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of other channel modeling aspects. The following are examples of suggested changes for angle calculation for CDL model by sources:

  - Example 1)

      - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,                  (7.7-5)

      - where

      - ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model,

      - and

      - Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle.

      - ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}.

      - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Example 2)

      - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, and

      - ϕn,intermediate=s* ϕn,model-μϕ,model, where s is a scale factor to achieve desired angular spread.

Conclusion

- Continue study on handling of channel delays between different UE-TRP links. The following are examples of how absolute delays between different UE-TRP link may be applied in 38.901 provided by companies.

  - Example 1) introduce a new correlation type called “physically consistent” that takes the individual UE-TRP distances into account when generating the link-specific delays.

  - Example 2) Reuse absolute delay modelling in section 7.6.9 in TR 38.901 with extension for other scenarios as follows:

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH | UMi | UMa |
| --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 | -7 | -6.8 |
|  | σlgΔτ | 0.4 | 0.4 | 0.4 | 0.6 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 | 15 | 50 |

Agreement

The following assumptions are modeling parameters related to suburban use case. For aspects with multiple options, FFS which option(s) to support.

- BS height:

  - Option 1: 22.5 m

  - Option 2: 20 m – 25 m

  - Option 3: 15 m

  - Option 4: 35 m

  - Option 5: 25 m

- Layout:	Hexagonal grid, 19 Macro sites, 3 sectors per site,

  - Option 1: ISD = 1732 m

  - Option 2: ISD = 500 m

  - Option 3: ISD <= 1000 m

  - Option 4: 500 ~ 1000 m

  - Option 5: 1299 m

  - Option 6: ISD < 500 m

- Building density

  - Option: [373, 440, others] buildings/km2

- UT height: 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings

  - Typical building heights: Up to two floors for residential buildings, up to five floors for commercial buildings

- UT distribution:

  - Option 1: Uniform horizontally, 70% indoor residential users are on ground floor, 30% are on upper floor

  - Option 2: 0.5 m for outdoor; 1.5+3(Nfl-1) m for indoor, where Nfl is the floor and Nfl is uniform (1, 2) for residential buildings or is uniform (1, 5) for commercial buildings. (framework in 36.873 for UMa with changes to distribution of Nfl (number of floors).)

- Indoor/Outdoor:

  - Option 1: 80% indoor and 20% outdoor,

    - FFS: ratio between residential and commercial buildings

    - FFS on in-car users

  - Option 2: 40% indoor in residential buildings, 40% indoor in commercial buildings, and 20% outdoor

- LOS/NLOS: LOS and NLOS

- Min BS - UT distance (2D):

  - Option 1: 25 m

  - Option 2: 10 m

  - Option 3: 35 m

- FFS: Penetration model: low-loss penetration model

- FFS: clutter height parameter, e.g. foliage

Conclusion

Study at least the following channel modelling aspects of suburban use case. The sub-bullets describing the detailed equations of the modelling aspect are examples for consideration:

- Pathloss

  - NLOS: A+B⋅log10d+20⋅log10fc+10⋅log10minfc,10

  - LOS: Reuse pathloss model of UMa scenario in TR 38.901

  - LOS: PLLoS=22.9log10(d3D)+19.6log10f+28.6 dB, σ=3.48 [dB]

  - For LOS and NLOS, reuse Winner II

- LOS probability

  - PrLOS=1d2D-out≤10 mexp-d2D-out-1040010 m<d2D-out

- ASD

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1°) | µlgASD | 0.55 | 0.55 | 0.55 |
|  | σlgASD | 0.25 | 0.25 | 0.25 |
| Cluster ASD () in [deg] |  | 1.5 | 1.5 | 1.5 |

- ZSD

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1°) | µlgZSD | 0.55 | 0.55 | 0.55 |
|  | σlgZSD | 0.25 | 0.25 | 0.25 |

## RAN1 #118-bis (October-2024)

Conclusion

Investigation and potential update of the propagation characteristics of the channel model for 6-7 GHz are part of the study for this item.

Agreement

For new UE antenna model, antenna placement candidate locations relative to centre of the handheld device is as follows: - UE antenna modeling (at least for calibration), the UE antennas are placed along the edges of a rectangle reflecting a UE form factor. - The size of the device can be set to be X cm x Y cm x Z cm - FFS (X,Y, Z), e.g., (X,Y,Z) can be15cm x 6 x 0 cm, - The four corners and the centres of each edge are identified as potential locations of the UE antenna. Centre of the device is also identified as potential location of the UE antenna. - Antennas (except the centre of device antenna) are assumed to be oriented along the direction determined by the vector connecting the centre of the rectangle to the antenna location. centre of device antenna is assumed to be oriented in either forward facing or backward facing the plane of the device. - FFS: antenna field pattern with respect to the direction of antenna - FFS: how antenna elements and antenna modules are mapped to antenna location candidates, e.g., for FR1, antenna location candidates are for antenna elements, and for FR2, antenna location candidates are for antenna array modules. - FFS: how to handle polarization aspects of the antenna element/module.

Agreement

For UE antenna radiation pattern modeling, consider the following options for further study:

- Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS |
| Maximum directional gain of an antenna element GE,max | FFS dBi |

- Option 2) half-wavelength dipole.

  - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

    - FFS: range of θ

- Option 3) omni-directional antenna pattern

- Other options

Agreement

The following assumptions are updated modeling parameters related to SMa use case.

| Parameters |  | SMa | Options for FFS |
| --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = FFS) Up to two floors for residential buildings, up to five floors for commercial buildings. FFS: ratio between residential and commercial buildings | ISD Option 1: 1732 m ISD Option 2: 1299 m ISD Option 3: 1000 m ISD Option 4: 250 m Building Type Ratio Option A: 50% residential, 50% commercial Building Type Ratio Option B: 90% residential, 10% commercial |
| BS antenna height hBS |  | FFS | BS Height Option 1: 35 m BS Height Option 2: 25m BS Height Option 3: 15m |
| UT location | Outdoor/indoor | Outdoor and indoor | - |
|  | LOS/NLOS | LOS and NLOS | - |
|  | Height hUT | 1.5 m for outdoor, 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings | - |
| Indoor UT ratio |  | 80% indoor and 20% outdoor |  |
| UT mobility (horizontal plane only) |  | FFS |  |
| Min. BS - UT distance (2D) |  | FFS | Min Distance Option 1: 20m Min Distance Option 2: 25m Min Distance Option 3: 35m |
| UT distribution (horizontal) |  | Uniform | - |
| UT distribution (vertical) |  | FFS | Vertical Distribution Option 1: 70% indoor residential users are on ground floor, 30% are on upper floor, uniform distribution across all floors for commercial Vertical Distribution Option 2: uniform distribution across all floors for a building type |
| Penetration model |  | FFS: low-loss penetration model | - |

Agreement

Further study and down-select cell layout, BS antenna height, and building type ratio for modeling parameters related to SMa use case. The Following are sets of option combinations prioritized for study and down-selection. Other option combinations are not precluded.

- Alt 1) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m

- Alt 2) ISD 1000m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 3) ISD 250m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 4) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 5) ISD 1732 m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m

- Alt 6) ISD 1732 m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

Agreement

Further study methods of achieving frequency continuity and handling frequency dependency parameters of the channel model.

Observation:

- 4 sources showed wood penetration loss model in TR38.901 seems to with acceptable range of deviation with measurements in 7-24 GHz. 1 source from RAN1 #117 showed wood penetration loss model in TR38.901 deviates from measurement by more than 5 dB for certain frequency of interest within 7 – 24 GHz.

- 3 sources and 1 source from RAN1 #117 showed Concrete penetration loss seems to deviate from measurements by more than 25 dB for certain frequency of interest within 7 – 24 GHz.

- 3 sources show IRR glass penetration loss seems to deviate from measurements for certain frequency of interest within 7 – 24 GHz.

  - It is noted that the spread of measurements from companies could be due to selection of the glass window type (e.g. fixed, sliding, swinging, etc), thickness of the window, and number of panes.

Continue study on penetration loss for Wood, Concrete, and IRR glass taking into account measurement study conducted for the original TR38.901 (available in http://www.5gworkshops.com/5GCMSIG_White%20Paper_r2dot3.pdf)

Agreement

Study possibility of introduction of additional O2I building penetration loss model type that consider the following:

- concrete penetration loss model that is potentially different from existing model

- Note: other aspects are not precluded

Companies are asked to provide information on the need for the different penetration loss model. Companies are encouraged to provide penetration loss per unit thickness (1 cm) of the material.

Observation

- 1 Source suggested updates to UMa NLOS pathloss based on measurement of UMa

- 1 Source suggested updates to UMi LOS and NLOS pathloss based on measurement of UMi

  - Note methodology to obtain the curve fit for measurement data has been questioned.

- For other scenarios InH and RMa, multiple sources reported reasonable alignment of measurements to current model in TR38.901.

Continue study on pathloss for applicable scenarios, UMa NLOS. The following are example of suggests changes:

- UMa NLOS: 13.54 + 38.6 39.09 log10( d3D ) + 20 log10( fc ) – 0.6 (hUT – 1.5)

Observation

- The following delay spread results were observed by 5 sources for UMi and UMa scenarios.

  - The center of the range of the values for a specific source in the figure represent the mean of the log delay spread.

  - The line range of the values for a specific source in the figure represent the standard deviation of the log delay spread.

Conclusion

- RAN1 concludes LOS probability is validated for existing deployment scenarios defined in TR38.901.

  - Note: this does not preclude introduction of new LOS probability for new scenarios of interest and/or different BS height to be supported for new/existing scenarios.

Observation

- 2 sources observed aligned shadow fading parameters in the frequency ranges of interest for InH LOS and NLOS scenarios.

- 1 source observed (from RAN1 #118) shadow fading parameters to deviate from TR38.901 by more than 1 dB for InH LOS, InH NLOS, and UMi LOS scenarios.

- 2 sources observed no need for updates to shadow fading parameters in the frequency ranges of interest.

Continue discuss on shadow fading parameters for InH and UMi scenarios.

Observation

- 1 source observed frequency dependency of K-Factor for UMi LOS scenario.

- 1 source observed (in RAN1 #118) deviation of mean K-factor by 4 dB for UMi LOS scenario and 1.5 dB for InH LOS scenario.

Continue study of K-factor parameters for InH LOS and UMi LOS scenarios.

Agreement

- Introduce absolute delay for UMi and UMa scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - Details of the absolute delay modeling for UMi and UMa FFS.

  - Note: this does not preclude potential introduction of absolute delay for other scenarios.

  - The following parameters are examples:

- Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH | UMi | UMa |
| --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 | -7 | -6.8 |
|  | σlgΔτ | 0.4 | 0.4 | 0.4 | 0.6 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 | 15 | 50 |

Observation

- 1 Source observed decorrelation distance for DS and KF for UMi LOS are different from measured results, and possibly have frequency dependency.

Continue study on correlation distances for applicable scenarios.

Agreement

Study the possibility of supporting a < 500 m ISD for UMa

- FFS whether any other modeling parameter is required to be updated

Agreement

Further study LOS probability assumption for SMa

- The following options are example of LOS probability assumption for SMa

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| SMa | Option1: PrLOS=exp-d2D-outexpα∙hBSβ+γhUEδ - FFS: α, β, γ, δ parameters Option 2: (ITU M.2135-1) PLOS=1, d2D-out≤10exp-d2D-out-10200, d2D-out>10 - BS height of 35m Option 3: PLOS=1, d2D-out≤10exp-d2D-out-10562, d2D-out>10 - For BS height of 25m, and UE height of 1.5m |

Working assumption

Use the following pathloss formula assumption for SMa (based on ITU M.2135-1).

- FFS: refinements based on additional new simulations/measurements

- FFS whether pathloss formula can be applicable for hUT higher than 10m

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters | Shadow fading std [dB] | default values | Applicability range |
| --- | --- | --- | --- | --- | --- |
| SMa | LOS | PL1=20log1040πdfc3+min0.03h1.72,10log10d-min0.044h1.72, 14.77+0.002log10hd PL2=PL1dBP+40log10ddBP | σ=4 σ=6 | 10 m < d< dBP dBP < d < 5000 m W = 20m, h = 10m | 0.8 ≤fc≤ 37 GHz 5 m < h < 50 m 5m < W < 50 m 10 m < hBS < [150] m 1 m < hUT < [10] m |
|  | NLOS | PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc-3.2log1011.75hUT2-4.97 FFS potential changes: PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc+10⋅log10minfc,10-10⋅log102-3.2log1011.75hUT2-4.97 | σ=8 | 10 m < dBP < 5000 m W = 20m, h = 10m |  |

Agreement

Further study the following modeling parameters and associated parameter assumption values for SMa:

- Correlation distance for spatial consistency

- Spatial correlation distance

- Blocking region parameters (For Blockage Model A)

- Fast fading parameters