3GPP TSG RAN WG1 #119			 R1-2410652

Orlando, US, November 18th – 22nd, 2024

Agenda item:	7

Source: 	Moderator (MediaTek Inc.)

Title: 	Summary on PDSCH reception scheduled by CORESET not following the unified TCI state

Document for:	Discussion and Decision

- Introduction

In RAN1#118bis, there were offline/online discussions on QCL assumption for PDSCH reception scheduled by PDCCH in CORESET not following unified TCI state [1]. However, due to the divergence of companies’ views, no final decision was reached in RAN1#118bis, and, per Mr. Chair’s guidance, this issue will be further discussed in RAN1#119.

According to the contributions to this issue, three alternatives are proposed as follows:

For a PDSCH reception scheduled by a CORESET not following the indicated unified TCI state with the scheduling offset equal to or greater than timeDurationForQCL: Atl1 [2][3]: Regardless of the TCI field present/absent in the DL DCI, the UE applies the same QCL assumption as the one applied to the scheduling PDCCH to the corresponding PDSCH reception. Alt2 [4][5][7]: Regardless of the TCI field present/absent in the DL DCI, the UE applies the unified (indicated) TCI state to the corresponding PDSCH reception. Alt3 [6] (note that this alternative is different from Alt3 captured in [1]): - If the TCI field is present in the DL DCI, the UE applies the TCI state indicated by the TCI field of the DL DCI to the corresponding PDSCH reception, and other Rel-15 QCL rules are applied. - If the TCI field is not present in the DL DCI, the UE applies the same QCL assumption as the one applied to the scheduling PDCCH to the corresponding PDSCH reception.

For the discussion on this issue in this meeting, we could try to down-select one alternative from above.

- Discussion

Table 1 Questions for Discussion

| # | Question |
| --- | --- |
| 1 | Please share your view on which one of the following alternatives should be adopted. Atl1: Regardless of the TCI field present/absent in the DL DCI, the UE applies the same QCL assumption as the one applied to the scheduling PDCCH to the corresponding PDSCH reception. - Support/fine: MTK, DOCOMO, Ericsson, Huawei, Hisilicon Alt2: Regardless of the TCI field present/absent in the DL DCI, the UE applies the unified (indicated) TCI state to the corresponding PDSCH reception. - Support/fine: Samsung, ZTE Alt3: Note that this alternative is different from Alt3 captured in [1] - If the TCI field is present in the DL DCI, the UE applies the TCI state indicated by the TCI field of the DL DCI to the corresponding PDSCH reception, and other Rel-15 QCL rules are applied. - If the TCI field is not present in the DL DCI, the UE applies the same QCL assumption as the one applied to the scheduling PDCCH to the corresponding PDSCH reception. - Support/fine: vivo Please check the corresponding draft CRs in Section 3 |

Table 2 Company input for Questions in Table 1

| Company | Support/fine to which Alt | Comment |
| --- | --- | --- |
| Mod | Alt1/Alt2/Alt3 | Please share your view on Question 1 |
| Google | Alt2 | Though sharing motivation of Alt1, we think Alt2 is a better way to go. We note that for Rel-18, UE also uses the indicated TCI state (first or second) to receive broadcast signal, when the scheduling offset is above threshold. It would not be a perfect solution. But unless companies have the consensus to modify Rel-18 behavior as well, otherwise, we do not see why UE behavior would be different under the same scenario. |
| ZTE | Alt2 | Besides for the reason raised in our contribution R1-2410120: a basic principle for unified TCI operation, avoiding complicated rule of beam collision, etc., one more thing is highlighted: - for current FR2 UE, the typical UE capability signaling timeDurationForQCL is indicated by s28, i.e., 28 OFDM symbol, but, K0 for non-UE-dedicated PDSCH scheduled by DCI format 1_0 is 1 or 0 slot. That means that the scheduling offset should be less than a threshold as a typical case. Per current spec, we can observe that, regardless of being following unified TCI or not, the PDSCH with scheduling offset less than a threshold should be aligned with indicated TCI state. Then, based on this analysis, the potential benefit of improving flexibility of TCI indication for non-UE dedicated PDSCH (going with either Alt-1 or Alt-3) is very weak. |

- Potential CR

  - Draft CR based on Alt1:

5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State, - if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception. - if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the UE may assume that the TCI state or the QCL assumption for the PDSCH of the serving cell is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell. <Unchanged parts are omitted>

  - Draft CR based on Alt2:

5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State, - if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception. - if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, for a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, the indicated TCI state is applied to the PDSCH of the serving cell. <Unchanged parts are omitted>

  - Draft CR based on Alt3:

5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the UE is provided dl-OrJointTCI-StateList-r17, and if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD', regardless of configuration of followUnifiedTCI-State, - if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to PDSCH reception. - if the indicated TCI state is associated with a PCI different from the serving cell, the UE may assume that the DM-RS ports of PDSCH(s) of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In the CA case, if the 'QCL-TypeD' of the PDSCH DM-RSs from respective CCs in a band are different in a slot, the QCL-TypeD assumption of the PDSCH DM-RS in the CC with lowest CC ID in the band is applied to all the PDSCH DM-RSs in the CCs in the band. In this case, if the qcl-Type is set to 'typeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers). For a UE that is provided dl-OrJointTCI-StateList-r17, if a PDSCH of a serving cell is scheduled by a CORESET that does not follow the indicated TCI state, and if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, - if the TCI field is present in the DL DCI scheduling the PDSCH of the serving cell, the UE may assume that the DM-RS ports of the PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state. For a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. For a multi-slot PDSCH or the UE is configured with higher layer parameter pdsch-TimeDomainAllocationListForMultiPDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. - if the TCI field is not present in the DL DCI scheduling the PDSCH of the serving cell, the UE assumes that the TCI state or the QCL assumption for the PDSCH of the serving cell is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell. <Unchanged parts are omitted>

- Conclusion

Agreement

For the case where scheduling offset >= timeDurationforQCL, clarify the behavior for PDSCH reception scheduled by CORESET NOT following the unified TCI state in Rel-17 spec (TS 38.214 Clause 5.1.5) by adopting Alt2

- Alt2: Regardless of the TCI field present/absent in the DL DCI, the UE applies the unified (indicated) TCI state to the corresponding PDSCH reception.

- References

| # | Tdoc | Title | Source |
| --- | --- | --- | --- |
| 1 | R1-2409264 | Summary on PDSCH reception scheduled by CORESET not following the unified TCI state (Final) | Moderator (MediaTek Inc.) |
| 2 | R1-2409368 | Discussion on PDSCH reception scheduled by CORESET not following the unified TCI state | MediaTek Inc. |
| 3 | R1-2409369 | Draft CR on PDSCH reception scheduled by CORESET not following the unified TCI state | MediaTek. Inc, NTT DOCOMO. INC, Ericsson, Huawei, Hisilicon |
| 4 | R1-2409573 | Discussion on TCI state determination for PDSCH reception scheduled by CORESET not following the indicated unified TCI state | Samsung |
| 5 | R1-2409574 | Draft CR on TCI state determination for PDSCH reception scheduled by CORESET not following the indicated unified TCI state | Samsung |
| 6 | R1-2409968 | Discussion on PDSCH reception scheduled by CORESET not following the unified TCI state | vivo |
| 7 | R1-2410120 | Discussion on QCL assumption for PDSCH scheduled by PDCCH in CORESET not following unified TCI | ZTE Corporation, Sanechips |