3GPP TSG-RAN WG1 Meeting #119	Tdoc R1-2410921

Orlando, US, November 18th – 22nd, 2024

Agenda Item:	9.1.2

Source:	Moderator (Ericsson)

Title:	Final summary of specification support for positioning accuracy enhancement

Document for:	Discussion, Decision

# Introduction

This document summarizes the discussions during RAN1#119 for the agenda item 9.1.2, Specification support for positioning accuracy enhancement.

This discussion corresponds to the objectives related to the positioning use case described in RP-242399 (revised WID) [1] below.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In the following, the delegates from participating companies are listed. Please check and add the contact information. We can use this list for future offline discussion, if such a need arises.

| Company/organization name | Contact person | Email of contact person |
| --- | --- | --- |
| Ericsson | Yufei Blankenship | yufei.blankenship@ericsson.com |
| Ericsson | Florent Munier | florent.munier@ericsson.com |
| Fujitsu | Xin WANG Yujia SHAN | wangxin@fujitsu.com shanyujia@fujitsu.com |
| MTK | harrison | harrison.chuang@mediatek.com |
| ZTE | Cong WANG | wang.cong8@zte.com.cn |
| CMCC | Yi Zheng Yongchang Liu | zhengyi@chinamobile.com liuyongchang@chinamobile.com |
| InterDigital | Fumihiro Hasegawa | fumihiro.hasegawa@interdigital.com |
| vivo | Huaming Wu | huaming.wu@vivo.com |
| vivo | Yuanyuan Wang | yuanyuan.wang.txyj@vivo.com |
| CATT | Yongqiang Fei | feiyongqiang@catt.cn |
| NEC | Chen Wei Miao Zhaobang | chen_wei@nec.cn miao_zhaobang@nec.cn |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| OPPO | Zhihua Shi | szh@oppo.com |
| Pengcheng laboratory | Pingye XIANG | xiangpy@pcl.ac.cn |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| TCL | Tianqi Wu | tianqi1.wu@tcl.com |
| LG Electronics | Jaehoon Chung | jhoon.chung@lge.com |
| Qualcomm | Mohammed Hirzallah (Ali) | mhirzall@qti.qualcomm.com |
| ETRI | Seungjae Bahng | sjbahng@etri.re.kr |
| Baicells | Xiang YUN Xiaonan WANG | yunxiang@baicells.com wangxiaonan@baicells.com |
| Apple | Kome Oteri | ooteri@apple.com |
| Fraunhofer | Mohammad Alawieh Georgios Kontes | mohammad.alawieh@iis.fraunhofer.de georgios.kontes@iis.fraunhofer.de |
| Sony | Basuki Priyanto Yujie Zhang | Basuki.priyanto@sony.com Yujie.zhang@sony.com |
| Samsung | Qi XIONG | Q1005.xiong@samsung.com |
| NTT DOCOMO | Luhua You Kosuke Shima | youlh@docomolabs-beijing.com.cn kousuke.shima.nr@nttdocomo.com |
| Intel | Chatterjee, Debdeep | debdeep.chatterjee@INTEL.COM; |
| Nokia | Dick Carrillo Melgarejo | dick.carrillo_melgarejo@nokia.com; |
| CEWiT | Shiv Shankar Dhivagar Baskaran | shivshankar@cewit.org.in dhivagar.b@cewit.org.in |
| Lenovo | Robin Thomas | rthomas7@lenovo.com |
| HW/HiSi | Thorsten Schier | Thorsten.schier@huawei.com |
| Honor | Guozeng Zheng | zhengguozeng@honor.com |
| IIT Madras | K Sai Prasanth | saiprasanth@5gtbiitm.in |
| PHY Wireless | Steve Thompson | steve.thompson@phywireless.com |
| Sharp | Makoto Kitahara Liqing Liu | kitahara.makoto@sharp.co.jp liu.liqing@sharp.co.jp |
| Tejas Networks | Devarakonda Pavan Kalyan | pavankalyand@tejasnetworks.com |
|  | Vikram Singh | vikramsin@tejasnetworks.com |

# Model input

## Sample-based vs path-based measurements for model input

### Definition sample-based measurement (e.g., starting time of Nt window, parameter value range, selection rule)

#### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 1: For the study of sample-based measurements for Case 3b, we do not need to pursue tightly specified rule (e.g., determination of the selection window) or tightly aligned parameters (e.g., k,N_t, and N_t'). - As long as the rule and parameters could be specified to any extent, that would be helpful for alleviating the ambiguity across vendors (if exists). Proposal 2: For the study of sample-based measurements for Case 3b, the starting time of the selection window is based on the timing of the first detected path determined by the gNB implementation. Proposal 3: For the study of sample-based measurements for Case 3b, gNB can flexibly determine the value of 〖k,N〗_t, and N_t^' for the reported samples after receiving the signaled parameters from LMF. - The parameters signalled from LMF could serve as a reference for determining the parameters of reported samples by gNB implementation. Proposal 4: To protect proprietary channel estimation, the maximum number of N_t^' shall not be larger than 16, and the maximum number of N_t shall not be larger than 64.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 1: For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, option C/D can be supported for determining the starting time of the list of Nt consecutive samples: - Option A. the starting time of the list of Nt samples is the timing of the first detected path (with timing granularity T). - Option B. the starting time of the list of Nt consecutive samples is the timing of the earliest sample that the first path power is detectable. - Option C/D. the starting point of the Nt consecutive samples is determined by a pre-configured/configured timing difference (with timing granularity T) relative to the reference time. Proposal 2: LMF configures the starting point of Nt continuous samples for UE/TRP, where the configuration is represented using timing granularity T. - The candidate value for timing granularity T is the same as the measurement report configuration. Proposal 3: For the definition of sample-based measurement, - The set of candidate values for Nt include at least: 32, 64, 128, 256; - The set of candidate values for Nt' include at least: 16, 32, 64, 128; Note: Nt'<= Nt. - The set of candidate values for k include at least: 0…5. Proposal 4: At least for case 3b, the procedure for determining the number of samples and timing granularity k for sample-based measurement report follows the current framework, while the selection of timing granularity should follow LMF’s configuration.

• CMCC (R1-2409500) Proposal 2: For selecting Nt' samples by UE, assuming that these Nt' samples are the strongest power samples in the sampling window or introducing a threshold could be considered. Proposal 3: For selecting Nt' samples, the entities that derive AI model could provide the recommended number of samples.

- Samsung (R1-2409582) Proposal 9. RAN1 supports the the value range of Nt is determined from UE reported value (i.e., UE capability) Proposal 10. RAN1 supports using a reference time and the offset to this reference time regarding the starting time of the Nt samples.

- Intel Corporation (R1-2409742) Proposal 11: For the definition of sample-based measurement, for an estimated channel response, support the following option for determining the starting time of the list of Nt samples: - Option A. the starting time of the list of Nt samples is the timing of the first detected path (quantized with timing granularity T). - At least for reporting multiple samples for sample-based measurement reporting, support differential time instance representation relative to the timing of the earliest sample for reporting of timing information of the channel response for model input in AI/ML based positioning using CIR/PDP/DP.

- Intel Corporation (R1-2409742) Proposal 12: If path-based measurement is supported, the existing representation/quantization framework for timing-based metric reporting (e.g., RSTD, RTOA), with the smallest value of ReportingGranularityfactor (‘k’) of -6 as introduced in Rel-18, can be reused for representation of timing information of the channel response for model input in AI/ML based positioning. - If sample-based measurement is supported, the reporting granularity factor ‘k’ can be from 0, 1, …, 5.

• Indian Institute of Tech (M), IIT Kanpur (R1-2410424) Proposal 1: Based on our simulations so far, we would recommend Option B for sample-based measurements. In our understanding, Option A is close to path-based measurements. The positioning accuracy that we obtained thus far are better with sample-based compared to path-based measurements. Note: Option B: Starting Time Based on First Detectable Path Power:

- Baicells (R1-2410414) Proposal 2. RAN1 to consider supporting multiple options, such as Option B and Option D. Option B. the starting time of the list of Nt consecutive samples is the timing of the earliest sample that the first path power is detectable. Option D. the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time.

• Sharp (R1-2410360) Proposal 1: For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, RAN1 study the following options for determining the starting time of the list of Nt consecutive samples: - Option A: the starting time of the list of Nt consecutive samples is the timing of the first detected path (with timing granularity T). FFS: whether the timing of the first detected path is reported - Option C: the starting point of the Nt consecutive samples is determined by a fixed offset (with timing granularity T) relative to the reference time. Note: value of the fixed offset is - 985024 T_c and N_t=1970048 - Option D: the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time. Note: value range of the configured offset is from - 985024 T_c to + 985024 T_c

- Ruijie Networks Co. Ltd (R1-2409841) Proposal 1: For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, for determining the starting time of the list of Nt consecutive samples, further down-selection between Option A and Option B. • Option A. the starting time of the list of Nt samples is the timing of the first detected path (with timing granularity T). • Option B. the starting time of the list of Nt consecutive samples is the timing of the earliest sample that the first path power is detectable.

- TCL (R1-2410215) Proposal 5: Down-select the following options for determining the starting time of the list of Nt consecutive samples: Option C. the starting point of the Nt consecutive samples is determined by a fixed offset (with timing granularity T) relative to the reference time. Option D. the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time.

- CEWiT (R1-2410571) Proposal 2: Based on the UE capability report and scenario related information, in Case 2b, the LMF can recommend sample selection criteria, optimal ranges for Nt' and k to UE. Proposal 3: Support reusing the existing value range for ‘k’ from {0..5} specified in the specification by LMF to provide recommendations to UE/gNB. Observation 1: The final decision of selection of ‘Nt’ and ‘k’ should be upto UE’s decision and specific to each UE. Proposal 4: Support considering time at which the first path is detected with respect to a reference time as the starting of the sampling window for model input data.

- Fujitsu (R1-2410049) Proposal-1: For sample-based method, Option D is suggested, in which the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time. - The configured offset is decided by LMF based on the potential UE’s distance to TRPs Proposal-2: The FL’s Proposal 2.1.1.4-2 in RAN1#118bis can be revised as follows: For the definition of sample-based measurement, - The set of candidate values for Nt include at least: 16, 32, 64, 128; - The set of candidate values for Nt' include at least: 9, 16, 32; Note: Nt'<= Nt. - The set of candidate values for k include at least: 0…5.

- Fraunhofer IIS, Fraunhofer HHI (R1-2410205) Proposal 2: The starting time of the list of Nt samples is determined by the first detected path in time in the Nt window Proposal 3: To minimize discrepancies between training and inference, the desired selection criteria can be signaled: - First paths - Strongest paths - First paths and strongest paths

- Tejas Networks Limited (R1-2409750) Proposal-1: Based on different deployment scenarios, the N_t∈{32,64,128,256} can be supported. Proposal-2: The value of N_t^' should be a fraction of the window size (N_t) given by N_t^'=αN_t where α should be either selected by the TRP and signalled to the LMF or vice-versa. Proposal-3: The N_t^' ∈{N_t,N_t/2,N_t/4,N_t/8} should be supported. Proposal-4: The N_t^' ∈{32,64,128,256} should be supported for N_t=256. Proposal-5: The N_t^' ∈{16,32,64,128} should be supported for N_t=128. Proposal-6: The N_t^' ∈{8,16,32,64} should be supported for N_t=64. Proposal-7: The N_t^' ∈{4,8,16,32} should be supported for N_t=32. Proposal-8: In outdoor scenarios, the value of n_0 may need to accommodate a significantly wide range, as 5G networks can connect with UEs located up to 100 km away from the TRPs. Proposal-9: In indoor scenarios, the values of n_0 can lie in the interval 0<n_0<512 for d_ISD=50m. Proposal-10: The existing values of k supported for path-based measurements can be reused for sample-based measurements as well.

- NEC (R1-2409852) Proposal 1: For the definition of sample-based measurement, the value range of Nt' is advised to: − Less than or equal to 9, or − Large than or equal to 9 but less than or equal to 256. Proposal 2: For the definition of sample-based measurement, the value range of granularity factor k is advised to: − Large than or equal to 0 but less than or equal to 5. Proposal 3: For selecting the starting time of the list of Nt samples, Option C, the starting point of the Nt consecutive samples is determined by a configured offset (quantized with timing granularity T), relative to the reference time, is more preferable than other options.

- Xiaomi (R1-2409878) Proposal 4：The starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time

• CATT, CICTCI (R1-2409926) Proposal 13: The starting point of the Nt consecutive samples is determined by a configured or predefined offset. - If configured, the offset is determined by the configured value. - If not configured, the offset is determined by a predefined manner. Proposal 14: For the definition of sample-based measurement, - The set of candidate values for Nt include at least: 16, 32, 64, 128. - The set of candidate values for Nt' include at least: 9, 16, 25, 32; Note: Nt'<= Nt. - The set of candidate values for k include at least: 0…5.

- Nokia (R1-2409986) Proposal 35: When sample-based representation is used, LMF configures the UE/gNB to determine N’t and choose sample selection strategy/rule-based for a fair trade-off between positioning requirements and signaling overhead. Proposal 36: For sample-based, LMF configure the UE/gNB for starting time of list of Nt samples and value range of Nt. The optimal value for the time offset and range of Nt is up to UE-implementation. Proposal 37: As a starting point, Option-D can be considered for further investigation while other options are not precluded.

- OPPO (R1-2410102) Proposal 1: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for UE-assisted/LMF based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - The existing reporting (from UE to LMF or gNB to LMF), which are path-based measurement, can be used as the input of LMF-side model - It can be done by LMF’s implementation. Thus, there is no RAN1 specification impact - If sample-based measurement is supported as an optional UE feature in Rel-19 for Case 2b/3b, support Option A to determine the starting time of the list of Nt samples - Option A. the starting time of the list of Nt samples is the timing of the first detected path (quantized with timing granularity T).

• ETRI (R1-2410258 Proposal 2: For UE-sided models (Case 1/2a), if sample-based measurement is adopted as the model input, the UE-side determines the parameters (Nt, Nt’, k). Proposal 4: For gNB-sided model (Case 3a), if sample-based measurement is adopted as the model input, the gNB-side determines the parameters (Nt, Nt’, k). Proposal 5: Support Option B for the starting time of the measurement window. • Option B. the starting time of the list of Nt consecutive samples is the timing of the earliest sample that the first path power is detectable. • FFS: practical number of Nt Proposal 6: For the definition of sample-based measurement, if bandwidth aggregation is applied, • the set of candidate values for k include [-6, 5]

- Qualcomm Incorporated (R1-2410467) Observation 19: For AI/ML positioning, the definition of Alt-A sample-based measurements, the existing specifications define ranges of timing granularity factor k and number of additional path measurements, which can be reflected to k and Nt’ range. The ranges and values of k, Nt’, and Nt can also be scenario dependent. Observation 20: For AI/ML positioning, the definition of Alt-A sample-based measurements, the best criteria for selecting starting point of Nt samples can be scenario dependent and needs to be tuned to capture useful part of time-domain channel response where multipath features are located, e.g., based on the first detected path.

- NTT DOCOMO, INC. (R1-2410377) Proposal 6: For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective， - For the report granularity k of sample-based measurement, legacy parameters Timing Reporting Granularity Factor and Timing Reporting Granularity Factor Extended in TS 38.455 can be considered as baseline, the value range can be reused, i.e., (0…5) and (-6…-1). - For the number of samples Nt’ which compose the measurement for sample-based measurement, legacy parameters Additional Path List and Extended Additional Path List in TS 38.455 can be considered as baseline, while large values can be introduced for sample-based measurement, e.g., the value range of Nt’ can be at least {8, 16, 32, …}. Proposal 7: For the definition of sample-based measurement, the start of the list of Nt samples is determined by a fixed/configured offset relative to the reference time.

- Lenovo (R1-2410019) Proposal 1: The maximum sample measurement window length corresponds to the maximum number of Nt samples, which may be up to Nt =128. Proposal 2: Study the following options and issues regarding the start time of Nt samples: - Starting time of the Nt consecutive samples is the timing of the first detected path (Option A) considering the issue that the sample prior to the first detected path may not be taken into account. - Starting point of the Nt consecutive samples is determined by a configured offset relative to the reference time (Option C) considering the issue that the reference time has not been defined for a UE/PRU. Proposal 4: Legacy values for the reporting range factor (k) may be used as a starting point, while the value range (v) of [-v×Tc, v× Tc], e.g., v = [-985024: 985024] or other min/max values, may be further discussed.

- MediaTek Korea Inc. (R1-2410531) Therefore we further consider an alternative that, - The start time could be on the sample that the first path power is detectable In this way, there is design degree of freedom to choose sample x-2, x-1 or x as the start time for reporting the Nt’ samples. Proposal 2a-1: Define the new measurement type for sample-based measurement, with the description: the channel response obtained from the resource elements that carry the reference signals configured for the measurement Proposal 2a-2: Consider not to configure the value of Nt, and Nt could be implicitly determined based on the applied IDFT size Proposal 2a-3: Instead of defining the start time of Nt consecutive samples for reporting the Nt’ samples, we tend to simply consider to define the start time to search for reporting Nt’ samples. The start time could be defined by the value of expected propagation delay minus the value of delay uncertainty. This is for case 3b only

- vivo (R1-2409669) Proposal 11: As for the rules of selecting samples from channel measurement, at least selecting earliest Nt’ samples and selecting strongest Nt’ samples are feasible, and can be considered as a start point.

- Ericsson (R1-2409443) Proposal 6 For the definition of sample-based measurement, the timing grid for the Nt consecutive samples is determined by the reference time Tref (seconds), together with the timing granularity T. Proposal 7 The starting time of the Nt consecutive samples is the timing of the first detected path, quantized to the timing grid (option A in Proposal 2.1.1.4-1 from RAN1#118bis). Proposal 8 For Case 3b (and 3a if needed), support {Nt, Nt'} parameter combinations: {9, 9}; {16, 16}, {16, 9}; {32, 16}, {32, 9}; {64, 32}, {64, 16}, {64, 9}. Proposal 9 For Case 3b (and 3a if needed), a small number of k values, e.g., k={4}, k={3,4}, or k={2,3,4,5}, is supported for timing reporting granularity factor in Rel-19.

#### 1st round discussion

Regarding the definition of sample-based measurements for model input, the following agreements were made in RAN1#117 (May 2024) and RAN1#118 (August 2024).

Agreement (RAN1#118) For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement (RAN1#118) For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. ===================================================================== Agreement (RAN1#117) Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement (RAN1#117) Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

The remaining issues are mainly: (a) how to define the grid to generate the samples; (b) how to define the starting point of Nt consecutive samples; (c) the candidate values for parameters Nt, Nt’, k.

Regarding the issue of defining the starting point of Nt consecutive samples, companies had intensive discussion in RAN1#118 and RAN1#118bis. The final proposal at the end of RAN1#118 is shown below, although no agreement could be made.

Proposal 2.1.1.4-1 For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, RAN1 study the following options for down-selection for determining the starting time of the list of Nt consecutive samples: - Option A. the starting time of the list of Nt samples is the timing of the first detected path (with timing granularity T). - Option B. the starting time of the list of Nt consecutive samples is the timing of the earliest sample that the first path power is detectable. - Option C. the starting point of the Nt consecutive samples is determined by a fixed offset (with timing granularity T) relative to the reference time. - Proponent companies provide details on the value of the fixed offset. - Option D. the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time. - Proponent companies provide details on the value range and signaling details of the configured offset.

In contributions to RAN1#119, companies performed further evaluation and shared further analysis. Extensive evaluation results and careful analysis can be found in several contributions. Please check companies’ contributions for their detailed evaluations and observations. The investigation provided by companies are very valuable and highly appreciated!

Based on companies’ input, companies’ view are captured in the table below (Please update/correct if your view is not captured correctly).

Question 2.1.1.2-1. How to define the starting point of the Nt consecutive samples?

| Options for defining starting point of the Nt consecutive samples | Supported by |
| --- | --- |
| Option A. First detected path (with timing granularity T) | vivo, OPPO, Ericsson, Huawei/HiSilicon, Apple, Ruijie Networks (A/B), CEWiT, Qualcomm(?), Intel, Lenovo, Sharp (A/C/D) [12 sources] |
| Option B. Timing of the earliest sample that the first path power is detectable | vivo (need new RAN4 requirement), Ericsson, MediaTek(?) Fraunhofer IIS/Fraunhofer HHI, ETRI, IITM/IITK, Ruijie Networks (A/B), Baicells (B/D) [10 sources] |
| Option C. determined by a fixed offset (with timing granularity T) relative to the reference time | NTT DOCOMO, ZTE/Pengcheng Laboratory (C/D), CATT/CICTCI (C/D), TCL (C/D), Sharp (A/C/D) [5 sources] |
| Option D. determined by a configured offset (with timing granularity T) relative to the reference time | Fujitsu, NTT DOCOMO, Lenovo, ZTE/Pengcheng Laboratory (C/D), CATT/CICTCI (C/D), TCL (C/D), Samsung, Nokia, Xiaomi, Sharp (A/C/D), Baicells (B/D), NEC [14 sources] |
| Other: [MediaTek R1-2410531]: defined by the value of expected propagation delay minus the value of delay uncertainty. This is for case 3b only |  |

In general, Option A/B are similar, and are based on the timing of first detected path or sample. Option C/D are similar, and are based on fixed or configured offset relative to the reference time. Both approaches have comparable number of supporters.

One compromise could be to follow some companies’ suggestion that Option A/B and C/D can be combined. For configurable offset, MediaTek’s gave an interesting suggestion of leveraging the existing signaling for search window. For NRPPa, the Search Window Information IE includes “Expected Propagation Delay” and “Delay Uncertainty”. The “Expected Propagation Delay” IE indicates the expected time SRS arrives at the TRP, relative to the UL RTOA Reference Time. See specification text copied from 38.455.  For LPP, one can refer to the existing IEs nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, which together define the search window for the target device. See specification text copied from 37.355.

Then in FL view, Option A/B and C/D can be combined as follows. In fact this procedure is not new, and it closely parallels the existing procedure for detecting SRS and PRS.

- Step 1 (follow Option C/D). The search window gives the range to search for starting time of the Nt samples. The search window is determined by a configured offset from the reference time.

- Step 2 (follow Option A/B). Within the search window, the starting time of the Nt samples is the first detected sample/path.

38.455

9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| … |  |  |  |  |  |  |

9.2.26	Search Window Information

This information element contains search window information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Expected Propagation Delay | M |  | INTEGER (-3841..3841,…) | Indicates when the SRS is expected to arrive in time at the TRP relative to the UL RTOA Reference Time. The UL RTOA Reference Time for a target SRS is defined as T0+tSRS, where - T0 is the SFN Initialisation Time - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. Granularity 4Ts, where Ts=1/(15103 2048) seconds. Centre of the search window. |
| Delay Uncertainty | M |  | INTEGER (1..246,…) | Indicates the uncertainty of the expected SRS arrival time at the TRP Granularity 4Ts, where Ts=1/(151032048) seconds. Single-sided search window. |

37.355

nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of DL-PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all DL-PRS Resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the DL-PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the DL-PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

Thus the proposal is suggested below, i.e., determine the starting time of the list of Nt samples within the configured search window.

Proposal 2.1.1.2-1

For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is the timing of the first detected sample (i.e., first detected path with timing granularity T) within a search window. The starting point of the search window is determined by a configured offset relative to the reference time.

- For measurement at TRP/gNB, the search window information is sent from LMF to gNB via NRPPa.

  - From RAN1 perspective, the existing “Search Window Information” IE in 38.455 can be reused.

Proposal 2.1.1.2-1A

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, for the starting time of the list of Nt consecutive samples:

- If LMF indicates an offset relative to a reference time

  - Up to TRP to follow LMF’s indication (i.e., starting time = reference time + offset), or to use the first detected sample (i.e., starting time = first detected path with timing granularity T).

- Otherwise

  - TRP uses the first detected sample (i.e., starting time = first detected path with timing granularity T).

Note: UE-side measurement is a separate discussion

|  | Company |
| --- | --- |
| Support |  |
| Not support | Baicells, HWHiSi |

| Company | Comments |
| --- | --- |
| Baicells | We don’t support this proposal. By doing this, the definition “the starting point of the Nt consecutive samples...” in option C/D is gone. As the FL says “Option A/B are similar, and are based on the timing of first detected path or sample. Option C/D are similar, and are based on fixed or configured offset relative to the reference time”, we suggest to firstly combine A/B together, C/D together respectively, then support them both to achieve the down-selection of the 4 options. |
| OPPO | Two questions need to be clarified: 1) is this only applied to AI/ML case 3b; 2) As mentioned in 38.455, ‘search information window’ field is optional, if this proposal is supported, the field is mandatory for Rel-19 AI/ML? |
| CMCC | It seems that if the delay uncertainty is small(like 4Ts in current spec), then this proposal have similar result like option C/D(i.e. an offset). |
| HW/HiSi | Not support. There is no need to have a configured offset for the start of the search window. The same search window as in legacy for path based reporting can be used. The hole discussion about sample based is to avoid potential ambiguity (which anyway inour understanding does exist for Case 3b). Having a configurable start of the search window does not resolve the claimed ambiguity issue. First detected path and first detected sample are not the same in our view. The first detected sample can be of lower energy resulting in ambiguity even for the same gNB. Updated Proposal 2.1.1.2-1 For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is the timing of the first detected path sample (i.e., first detected path with timing granularity T) within a search window. The starting point of the search window is determined by a configured offset relative to the reference time. - For measurement at TRP/gNB, the search window information is sent from LMF to gNB via NRPPa. From RAN1 perspective, the existing “Search Window Information” IE in 38.455 can be reused. |
| ETRI | Before we discuss the serch window, we need to narrow it down to one option out of the four. |
| Lenovo | There is a dependency with the existing UL/DL search window that the approximate UE location is known apriori in some sense to obtain the “expected propagation delay. Will the same assumption apply here? If not, this is more of a “measurement window” that the receiver has to apply instead of a search window. |
| mtk | 1, we expect to consider case 3b only for Q4 this year. So, it is not for “ for an estimated channel response between a pair of UE and TRP“. We can just say: “for an estimated channel response for case 3b” 2, The “Nt consecutive samples” is just an intermediate step to report Nt’ samples. We don’t think the restriction on the start of Nt samples, and the value of Nt will bring any benefit for performance. This is because, a), for case 3b, we have agreed the selection rule b), how NW can determine Nt properly? If we look at the “famous” channel models defined in RAN4, EPA, EVA and ETU, it is seen that there is one significant path at 5us for ETU, and at 2.5us for EVA. By using 32.55ns as sample period, they correspond to have the path around 154th and 77th sample respectively. If we restrict the max Nt to be 128 or 64, then this important characteristic (significant path at longer delay) can’t be captured. We suggest that the gNB can still report the sample larger than the recommended Nt by LMF. Or Nt can be determined implicitly by gNB, since after IFFT, the observation range is always much larger than Nt c) for the RSRPP, the per-path power requirement in RAN4 is defined in frequency domain, NOT in time domain. It is known that the power of the first path would be spread around the near-by samples in time domain. The gNB receiver should have the degree of freedom to determine the start of Nt sample. Whether the sidelobe power in the samples being reported are determined by the agreed selection rule. So one way to merge option A and B is: “the starting time of the list of Nt consecutive samples is the sample that the first path power is detectable“. Then the receiver can determine how wide the sidelobe can be contained for the final Nt‘ selection. We still want to emphasize that, the start time of Nt is JUST an intermediate step in the implementation. There is no need for specification to give restriction on it d) To consider how to merge option C and D: The “search window” is optionally provided, and when it is provided, the “expected delay” and “delay uncertainly” would be mandatorily provided. This concept could be similar to option D (configured offset) when “search window” is provided. When search window is NOT provided, it is to use option C (fixed offset). The fixed offset could be the reference time minus 0.5ms. This is because the legacy UL-RTOA reporting has the range between “reference time minus 0.5ms” and “reference time plus 0.5ms” e) To merge option A, B, C and D, we can consider e-1) a hierarchy structure that, option C & D are used to determine the start of search window, and further that, option A & B are used to determine the start of Nt samples within the search window e-2) a flat structure that using option C & D are enough. This is because it is to report Nt’ sample. The Nt consecutive samples is just the intermediate step so that there is no need to address too much From MTK point of view, we are fine for both e-1) and e-2). But if the majority prefers e-1), we want to highlight that the flexibility should be given for determining the start of Nt, and the value of Nt, according to the example above |
| CATT, CICTCI | Combination may be a way to considered. But in our view, a more straightforward way is to consider that ‘if starting time of Nt is configured then follows configuration, if not configured then up to the first detected sample’. Additionally, current statement use ‘first detected sample’. Is it correct understanding that Option A is used instead of Option B when combining options? |
| Apple | The key issue ist hat the TRP may need to have an idea of the window within which the search could take place. If unknown, and the window is large enough, we fall back to Option A. If small enough, if falls back to option D. If adopted, the window range needs to be updated to accomodate both cases. |
| DOCOMO | In our understanding, somehow search/measurement window is necessary for option A to measure samples before detecting the first detected path. Information of option C/D is missing in this proposal. |
| NEC | Change our preference option from Option C to Option D(as we cited a original Proposal 2.1.1.4-1 of last meeting.) |
| ZTE | In current 38455, the granularity of expected search window is 4Ts, which is far greater than the timing granularity T. And also, the duration for the window may cover longer time span. We know that FL tries to address all companies’ concern, but combining option A/B and C/D means uses 2 windows to determine the reported samples, not sure whether TWO windows are necessary for sample-based measurement. CATT’s suggestion is a good compromise for us. |
| Fujitsu | It seems difficult to merge these options. With current format in the proposal, it is more like Option A/B rather than Option C/D. The key difference between them is how to get the first Nt' sample, based on path detection or based on configuration? |
| Qualcomm | The search window/expected RSTD window can be quite large for initiating the starting point of Nt samples and dictating the sample-based definition does not buy alignment. In addition, many aspects in the proposal wording seem not necessary and not relevant. E.g., the search window can be helpful but not necessary as part of the definition. |

For the value range of parameters {Nt, Nt', k}, the last version of the proposal at RAN1#118bis is copied below.

Proposal 2.1.1.4-2 For the definition of sample-based measurement, - The set of candidate values for Nt include at least: 16, 32, 64, 128; - The set of candidate values for Nt' include at least: 9, 16, [32]; Note: Nt'<= Nt. - The set of candidate values for k include at least: 0…5.

Most companies can accept the above, with small variations, for example, adding or removing some values of Nt and Nt', adding or removing more candidate values for k.

One exception is, MediaTek propose that: “Consider not to configure the value of Nt, and Nt could be implicitly determined based on the applied IDFT size”.

Considering majority companies’ view, the proposal is suggested below. It is essentially the same as the proposal at RAN1#118bis, since this is what majority companies can accept.

Proposal 2.1.1.2-2

For the definition of sample-based measurement,

- The set of candidate values for include at least: 16, 32, 64, 128;

- The set of candidate values for Nt' include at least: 9, 16, 32; Note: Nt'<= Nt.

- The set of candidate values for k include at least: 0…5.

|  | Company |
| --- | --- |
| Support | CMCC, NEC |
| Not support | Hw/HiSi, Nokia |

| Company | Comments |
| --- | --- |
| Baicells | Suggest to add 64 to increase the range of Nt’ - The set of candidate values for Nt' include at least: 9, 16, 32, 64; Note: Nt'<= Nt. |
| New H3C | OK in general |
| HW/HiSi | We have a strong concern to include Nt‘>16 and Nt>64 in the definition, since it risks to expose proprietary information of the gNBs channel estimation. Also, our view is that sample-based measurement is not needed for Case 3b, since legacy path based reporting does not have an ambiguity issue in this case. |
| ETRI | We are fine with the proposal. |
| mtk | 1, the first subbullet is for Nt. We consider that Nt is optionally provided as a recommended value 2, when Nt is not provided, gNB can still determine on the own, because after IFFT, the observation range is much larger than Nt. What is the reason to give up the samples with longer delay that have significant power? 2, the value of k, it is related to the report granularity. It is not clear whether the recommended k is related to RS bandwidth. In our view it should be related to the RS bandwidth. If so, what is the reason that the gNB can’t decide k by self? |
| Nokia | We should desprioritize the discussion of this topic during this meeting. It may be easily discussed in case Sample Based representation is agreed. |
| DOCOMO | Generally fine. For the value of k, legacy value in Rel-18 can also be considered. |
| ZTE | For clarification, if Nt’=9, is there any signalling difference for sample-based and path-based measurement report? |
| Fujitsu | OK |
| Qualcomm | These are second priority details and related to signaling not the definition. We can discuss them if sample is supported. No need to waste time on them. |

#### 2nd round discussion

Proposal 2.1.1.3-1

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

- If LMF configures an offset relative to a reference time, and the gNB follows the configuration,

  - starting time = reference time + offset

    - FFS: value range of the offset

- Otherwise

  - starting time = first detected sample (i.e., first detected path with timing granularity T).

Note: UE-side measurement is a separate discussion

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| OPPO | Fine with minor modification. For case 3b, there is an agreement that the reference time is the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. We suggest to make minor modification to the first bullet: - If LMF configures an offset relative to a reference time, and the gNB follows the configuration, - starting time = reference time + offset - FFS: value range of the offset - Note: the reference time is the ULRTOA reference time |
| CMCC | For the first bullet, when LMF configures an offset relative to a reference time, whether the gNB must follow this reference time to determine the Nt ? |
| CATT | OK for progress |
| HW/HiSi | The proposal is not clear and we need more dicussion: Does the wording in the first bullet intend to allow that the gNB does not need to follow the provided configuration? If so, then there is no need to have a configured offset for the start of the search window. Any configuration based on an offset from the reference time requires several several assumptions, including a priori knowledge of the UE’s location and about the LOS/NLOS condition between the UE and each TRP. In addition, the configured offset needs to be TRP-specific. In our view, we believe the timing of the first detected path can be determined by the gNB implementation, since the gNB is better aware about the channel conditions between a UE and a TRP. For the second part if the first detected sample is the first detected path, does it mean that path and sample detection is the same? That is confusing to us. |
| Sharp | OK in principle. For the first bullet, If whether configured offset is used depends on the gNB implementation, how does LMF aware the configured offset is used? We think it is better gNB reports the timing of first detected path in case of Otherwise. |
| Nokia | As a compromise, we are ok with the proposal, but we also expect a similar compromise when the discussion will focus on Case 2b. It is important to clarify that the gNB has always the final decision on what approach to follow. |
| NEC | OK. Also fine with OPPO’s modification. |
| mtk | 1, we are afraid to say that how LMF can know the “offset“ and then provide it to gNB? In current spec, LMF can configure “expected propagation delay“ and “delay uncertainty“ within “search window“to the gNB. What is the relation between this “offset“ and the existing “search window“. We worry this proposal would be challenged 2, We suggest to change the wording: “i.e., first detected path with timing granularity T“ as “i.e., first detected path rounding down with timing granularity T“ The below is an example. When the exact path delay is between sample x-1 and x, the path-based method with reporting granularity T would select to report sample x, with smaller error. This is round-off. For sample-based method, it can consider round-down, so that the sample x-1 could be selected as the start for further selecting Nt‘ samples. |
| Qualcomm | The configured/fixed offset options shall not be the default as they require LMF knowledge about channel delay spread and timing drifts/offsets, which cannot be always available on measurement occasion basis. In the option of first detected path, device (gnB/TRP) observes channel response and can have better decision on where to locate useful part of channel response. |
| ZTE | The direction is fine for us. We can combine these two methods for determining the starting time of Nt. |
| DOCOMO | OK |
| vivo | We believe there’s a strong assumtion where LMF has prior knowledge of UE’s location for the configured offset to work. We have doubt on such assumption. |
| Tejas | We agree with the views expressed by CMCC and Huawei regarding the configuration of the time offset relative to the reference time. The proposal for configuring the time offset only makes sense when it is mandatory for the TRP to use it, which, in any case, is not advisable. We support the wording change suggested by Mediatek concerning the first detected sample. The phrase “first detected path with timing granularity T” is unclear. It should correspond to one of the following options: - First detected path rounded down with timing granularity T. - First detected path rounded up with timing granularity T. - First detected path rounded to the nearest with timing granularity T. We are okay with any of these options. |

### Enhancements for path-based measurement

#### Companies’ view from contribution

- Huawei, HiSilicon (R1-2409396) Proposal 7: For Case 3b, support enhancing the legacy path-based reporting by increasing the number of reported delay and power values from the gNB to LMF.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 5: For path-based measurement, enhancement on the number of reported paths is not needed.

- Samsung (R1-2409582) Proposal 8. Path-based measurement could be considered with potential overhead reduction.

- Tejas Networks Limited (R1-2409750) Proposal-11: For reporting path-based measurements, the existing framework from release-17 standards should be utilized. If required, the number of additional measurements should be increased from 8 to a higher value.

- Apple (R1-2409788) Proposal 2: Support modification of path-based feedback to support sample type feedback for case 3b (for inference) and all types (for data collection in model training). The existing path-based feedback specification can be modified to support sample-based model input by the following proposals: Proposal 2-1: Timing is eqi-spaced on a grid with signalling that indicates the equal spacing size - The relative time difference of each reported path (nr-RelativeTimeDifference) is an integer multiple of a configured timing and represents a sample timing - Option A: the starting time of the list of Nt samples is the timing of the first detected path (with timing granularity T). - The value of k can range from [-6,5] as in the existing specification Proposal 2-2: Increase the number of additional paths supported (the value range of Nt) to 16, 32, 64, 128. - The samples reported are the Nt consecutive samples - Option 2: The samples reported are the the Nt strongest samples - Set Nt’ to 9, 16 or 32 - The samples reported are the Nt strongest samples Proposal 3: Rel-19 AI/ML based positioning, when comparing sample-based measurement input and path-based measurement input: - Overhead: Path-based measurements have a lower overhead (6.4124 samples on average for 100MHz) than sample-based measurements (16.5393 samples on average for 100 MHz). - Performance: - Path based measurements showed worse performance than sample-based measurements in low complexity and/or smaller bandwidth scenarios. - Path based measurements showed approximately the same performance as sample-based measurements in large bandwidth scenarios with high complexity models. - Recommendation: Given that number of paths in 100 MHz is an average, increasing > 9 is prudent.

- NEC (R1-2409852) Proposal 5: Enhancement on increasing number of paths based on legacy path-based measurement for AI/ML positioning is necessary, if path-based measurement will be supported.

Proposal 16: For the model inference for NG-RAN node assisted positioning with LMF-side model (Case 3b) - gNB/TRP can do the measurement and report the results via existing NRPPa signaling - The existing measurement types are reused - FFS: whether or not to introduce new values (N) for the maximal number of additional paths (e.g., N > 8).

• ETRI (R1-2410258 Proposal 7: For the LMF-side model (Case 2b/3b) in Rel-19 AI/ML positioning, the existing reporting procedures can be reused - FFS: enhancement of the number of reporting paths - FFS: power reporting table – linear scale reporting vs. Logarithmic scale reporting

- Lenovo (R1-2410019) Proposal 5: Further consideration may be given to extend the maximum number of reported additional paths for path-based measurements. FFS the value of the maximum number of reported paths.

- vivo (R1-2409669) - Another question is whether there is a need to enhance path-based measurement reporting by increasing the number of maximal reported paths, if legacy path-based reporting is identified as one of the choices to be specified for AI/ML based positioning. In our view, this enhancement is unnecessary and unjustified due to the following reasons: - From our simulation observation, less than 8 paths are detected for most of channel measurements (e.g., 3 detected paths for 20MHz bandwidth on average), especially when beamforming (16 horizontal beams and 2 vertical beams) is considered. In other words, there’s no enough path physically that could be detected to begin with. - The performance gain provided by reporting more paths in addition to the existing 8 additional paths is questionable, if the path selection rule is not aligned and the accuracy of estimated paths is not guaranteed.

#### FL view

On the path-based measurement, the following agreement was made in RAN1#117:

Agreement (RAN1#117) Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed).

Regarding the potential enhancement on the number of reported paths, vivo, Huawei, IITM/IITK conducted simulations to check the potential enhancement, and have somewhat different observations.

No enhancement is needed (i.e., reuse existing spec of 1+8 paths):

  - vivo (R1-2409669): “less than 8 paths are detected for most of channel measurements (e.g., 3 detected paths for 20MHz bandwidth on average)”. “In other words, there’s no enough path physically that could be detected to begin with.”

  - IITM/IITK (R1-2410424): “Our observations indicate that the cumulative power captured for each Tx-Rx pair approaches the majority of the total power (e.g., 90%) within a relatively small number of paths. Specifically, the average cumulative power curve shows that most power is captured within the first few paths. Based on our simulations, we believe that enhancement on the number of reported paths is not needed.” “Observation 1: Approximately 16 paths are sufficient to capture most of the received power.”

Enhance from 9 paths to 16 paths total

  - Huawei/HiSilicon (R1-2409396) observed that “When comparing the performance of 16 paths with 9 paths, the positioning accuracy is improved, both for using 4 TRPs and for using 18 TRPs.”; “The positioning accuracy decreases when the number of TRPs is reduced from 18 to 4, but remains in sub-meter region for 16 paths. In contrast, for 9 paths and 4 TRPs the accuracy exceeds 1m.”

Thus, more evaluations are needed to understand whether Rel-19 should increase the maximum number of reported paths above 9 (1 first_path + 8 additional_paths), and if so, what should be the new maximum number. Proponent companies are encouraged to provide further simulation results and share the analysis in the upcoming RAN1 meetings. Please be concrete with your analysis and recommendation.

### Whether to support sample and/or path based measurements

#### Companies’ view from contribution

- Huawei, HiSilicon (R1-2409396) Proposal 6: Regarding the type of channel measurements for Case 3b, support the use of legacy mechanism subject to path-based measurements.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 6: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, support sample-based measurements.

• CMCC (R1-2409500) Proposal 1: Sample-based measurements for AI/ML positioning is slightly preferred.

- Intel Corporation (R1-2409742) Proposal 9: Support path-based measurement for representation of time domain channel measurements in AI/ML based positioning with potential enhancement of the number of reported paths.

- Baicells (R1-2410414) Proposal 1. Support sample-based measurements as model input at least for Case 3b. Preclude path-based measurements in R19.

- Sharp (R1-2410360) Proposal 2: For AI/ML based positioning, Legacy-like format and Bitmap format should be supported for channel measurement for sample-based measurement.

- Google (R1-2410150) Proposal 3: Support the NW to configure whether the UE should report the sample-based or path-based measurement results.

- CEWiT (R1-2410571) Proposal 1: In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, time domain sample-based measurement reporting is preferred.

- NVIDIA (R1-2409781) Proposal 2: Support both of the following alternatives for time domain channel measurements for AI/ML based positioning: • Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. • Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods.

- Sony (R1-2410217) Proposal 6: Support sample-based time domain channel measurements as the AI/ML model input.

- Fraunhofer IIS, Fraunhofer HHI (R1-2410205) Proposal 1: Support both Alternative (a) sample-based measurement and Alternative (b) path-based measurement report.

- Apple (R1-2409788) Proposal 1: Support both path-based measurement input and sample-based measurement input. - Sample based model input can be viewed as a special case of path-based input with equi-spaced timing samples Proposal 4: On the applicability to the different sub-use cases in the LCM stages of AI/ML positioning: Path/Sample Input for different Sub-use cases Training Measurement Input; Inference; Monitoring. Case 1; Specify both; N/A; Depends on location and monitoring input required. Case 3a; Specify both; Specify both. Case 3b; Specify both; N/A.

- NEC (R1-2409852) Proposal 6: The compromised scheme that supports both sample-based measurement and path-based measurement is acceptable in order to not block the process − Prioritized sample-based measurements over path-based measurement for model input collection, if down-selection between sample-based input and path-based input is inevitable.

- Xiaomi (R1-2409878) Proposal 1: - For Case 2b and Case 3b, support sample-based input. - For Case 1, Case 2a and Case 3a, it is up to implementation to determine the model input

• CATT, CICTCI (R1-2409926) Proposal 12: At least for case 3b and 2b, support sample-based channel measurements as the AI/ML model input. Proposal 16: For case 3b, if both sample-based reporting and path-based reporting are supported, the choice of sample-based reporting and path-based reporting is up to gNB/TRP selection, or up to LMF indication.

- OPPPO Proposal 16: For the model inference for NG-RAN node assisted positioning with LMF-side model (Case 3b) - gNB/TRP can do the measurement and report the results via existing NRPPa signaling - The existing measurement types are reused - FFS: whether or not to introduce new values (N) for the maximal number of additional paths (e.g., N > 8).

- Nokia (R1-2409986) Proposal 34: RAN1 to consider both: path-based and sample-based representation for AI/ML positioning cases.

- Qualcomm Incorporated (R1-2410467) Proposal 21: In AI/ML positioning Case3b, for model input measurements, support Alt-B path-based measurements as the default and baseline measurement reporting for LMF-side model. - FFS: necessity of Alt-A sample-based measurements.

- NTT DOCOMO, INC. (R1-2410377) Proposal 5: Support sample-based measurement for AI/ML based positioning in Rel-19. - If both sample and path-based measurements are supported, LMF indicates whether sample-based or path-based measurement is applied subject to UE capability.

- Lenovo (R1-2410019) Proposal 6: On the time domain channel information representation, consider support for a hybrid approach in relation to the cases: - For Cases 1, 2b, and 3b: Support sample-based time domain representation - For Cases 2a and 3a: Support legacy-based path timing time domain representation - Note1: For each of the above use cases, use of one type of time domain representation (e.g. sample-based approach) in training and another type of time domain representation (e.g. path-based approach) during inference is not supported. - Note2: Discussion on Cases 2a and 2b may be revisited in RAN1#120.

- InterDigital, Inc. (R1-2409845) Proposal 23: Conclude whether to introduce sample-based measurements for Case 1 Observation 14: Leaving measurement details up to implementation for Case 1 implies that a new positioning method with no associated measurements is specified Proposal 24: For case 3b and case 1, support Alternative (b) (path based measurements) for representation of time domain channel measurements and reuse path-based reporting with the granularity up to k=-6 as specified in Release 18. Proposal 25: For case 3a, support Alternative (b) (path based measurements) since an AIML model may be able to estimate ToA that is not aligned with sampling periods. Proposal 26: For direct AI/ML based positioning, for case 3b, no enhancements are needed for UL measurements (UL-RTOA, SRS-RSRP and SRS-RSRPP) reported from the gNB to LMF Proposal 27: For direct AI/ML based positioning, for case 1, adopt RSRP, RSRPP and DL-RSTD measurements for the UE to determine input for an AIML model for direct AIML positioning

- vivo (R1-2409669) Proposal 12: Specify sample-based channel measurement reporting for Case 2b and 3b of AI/ML based positioning. Observation 10: Sample-based measurement can always provide better positioning performance than path-based measurement for AI/ML based positioning under various timing error assumptions. Proposal 13: The starting time of the list of Nt consecutive samples should be related to the timing of the first detected path for each TRP. Proposal 15: For determine the starting time of the list of Nt consecutive samples, Option A should be supported. • Option A. The starting time of the list of Nt samples is the timing of the first detected path (with timing granularity T). Proposal 16: For determine the starting time of the list of Nt consecutive samples, Option B can be supported if a new requirement for first detected sample can be defined in RAN4. • Option B. The starting time of the list of Nt consecutive samples is the timing of the earliest sample whose power is detectable. Proposal 17: Deprioritize Option C and D for AI/ML based positioning in Rel-19. • Option C. The starting point of the Nt consecutive samples is determined by a fixed offset (with timing granularity T) relative to the reference time. • Option D. The starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time. Proposal 18: Support to round down the off-grid first detected path (the start of Nt consecutive samples) to a grid point when taking the first path with better timing resolution than samples as the start of Nt consecutive samples. Proposal 19: The candidate Nt could be {32, 64, 128}. UE/gNB can report samples within the list of Nt consecutive samples or within the duration time of Nt consecutive samples. Proposal 20: The candidate Nt’ could be {8, 16, 32}. LMF can indicate one of the candidate Nt’ values to UE/gNB, and UE/gNB should report Nt’ samples within the list of Nt consecutive samples or within the duration time of Nt consecutive samples. Proposal 21: The candidate k could be {3, 4, 5}. The k’ value associated with sample-based reporting should be no larger than the k value indicated by LMF.

- Ericsson (R1-2409443) Proposal 4 For Rel-19 AI/ML based positioning Case 3b/2b, the measurement report of (PDP, DP) support sample-based measurement. Proposal 5 For Rel-19 AI/ML based positioning Case 3b, LMF includes an indicator in the measurement request to gNB, where the indicator is on the LMF preferred measurement report: either (a) sample measurement, or (b) legacy path-based measurement.

#### 1st round discussion

Regarding sample-based and/or path-based measurements for model input, the investigation follows from the agreement made in RAN1#116 (Feb 2024).

Agreement (RAN1#116) In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation.

On the question of whether to support sample-based and/or path-based measurements for model input, companies’ views are captured below, based on the submitted contributions to RAN1#119. (Please update/correct if your view is not captured correctly)

| Method | Supporters |
| --- | --- |
| Alternative A (sample-based measurement) (23 sources) | vivo, CEWiT, CATT/CICTCI, NEC, CMCC, Baicells, Sony, ETRI, Mediatek, ZTE/Pengcheng laboratory, Ericsson, Sharp, Xiaomi (Case 2b/3b), TCL, Ruijie Networks, NTT Docomo, OPPO (optional), Lenovo (Case 1,2b,3b), Tejas, IITM/IITK |
| Alternative B (path-based measurement) (9 sources) | Qualcomm, Huawei/HiSilicon, Samsung, InterDigital, Intel, OPPO, Lenovo (Case 2a,3a), Tejas |
| Specify both (9 sources) | Apple, Nokia, Google, NVIDIA, Lenovo (vary case by case), NEC (compromise), Ericsson (compromise), Fraunhofer IIS/Fraunhofer HHI |

As a reference, the RAN1#118bis status is copied below (see R1-2409286):

| Method | Supporters |
| --- | --- |
| Alternative A (sample-based measurement), 21 sources | vivo, CEWiT, CATT/CICTCI, NEC, CMCC, Baicells, Sony, ETRI, Mediatek, ZTE/Pengcheng laboratory, Ericsson, Sharp, Xiaomi, TCL, Ruijie Networks, NTT Docomo, OPPO (optional), Lenovo (Case 1,2b,3b), Tejas |
| Alternative B (path-based measurement), 9 sources | Huawei/HiSilicon, Qualcomm, Samsung, InterDigital, Intel, OPPO, Lenovo (Case 2a,3a), Tejas |
| Specify both, 9 sources | Apple, Nokia, Google, NVIDIA, Lenovo, NEC (compromise), Ericsson, Fraunhofer IIS/Fraunhofer HHI, |

Indeed, companies’ view on sample-based vs path-based measurement stays almost the same as for all the meetings in 2024, except that more companies are open to support both so that RAN1 progress is not blocked. Since RAN1 is unlikely to converge to one Alternative only, it is suggested that both are supported. The LMF can signal which measurement is to be reported for Case 3b.

Proposal 2.1.3.2-1

At least for Rel-19 AI/ML based positioning Case 3b, for determining model input, support both Alternative (a) sample-based measurement report and Alternative (b) path-based measurement report.

- For Case 3b, LMF signals to gNB whether (a) or (b) is expected to be reported.

Monday offline:

Proposal 2.1.3.2-1A (compromise view)

At least for Rel-19 AI/ML based positioning Case 3b, for determining model input, support both Alternative (a) sample-based measurement report and Alternative (b) path-based measurement report.

- For Case 3b, LMF signals to gNB whether (a) or (b) is expected to be reported.

Proposal 2.1.3.2-1B (majority view)

At least for Rel-19 AI/ML based positioning, for determining model input, support Alternative (a) sample-based measurement report.

|  | Company |
| --- | --- |
| Support | CMCC, NEC |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| OPPO | Fine to support. |
| New H3C | OK in general |
| CEWiT | Fine with the proposal but the criteria for the LMF to select the type of input should also be discussed. |
| ETRI | Fine with us |
| Lenovo | Support, keeping in mind that the other cases also need to be considered in due time. |
| CATT, CICTCI | May be a way to process, but we think it’s better to make decision after: - The definition of sample-based measurement is clear - The FFS of ‘whether/how to increase the number of path for path-based measurement’ |
| Apple | Okay |
| DOCOMO | Support. If both (a) and (b) are supported, LMF indication is necessary, as captured in the current proposal. |
| ZTE | Fine to compromise to support both. |
| Fujitsu | For progress, support both is fine for us. |
| Qualcomm | We are spending 6 meetings on this topic and there is still a divergence and seem no way moving forward. One way to move forward is to scope enhancements to path-based measurements in which more aspects related to relation between first and additional path measurements can be discussed to help LMF obtain more alignment on the additional path measurements. |
| InterDigital | We need to clarify the definition of sample-based measurements first. |

#### Wednesday offline discussion

Proposal 2.1.3.3-1 (compromise view)

(For Working Assumption)

At least for Rel-19 AI/ML based positioning Case 3b, for determining model input, support both Alternative (a) sample-based measurement report and Alternative (b) path-based measurement report.

- For Case 3b, LMF signals to gNB whether (a) or (b) is requested to be reported.

Proposal 2.1.3.2-1B (majority view)

At least for Rel-19 AI/ML based positioning, for determining model input, support Alternative (a) sample-based measurement report.

| Method | Supporters |
| --- | --- |
| Alternative A (sample-based measurement) (23 sources) | vivo, CEWiT, CATT/CICTCI, NEC, CMCC, Baicells, Sony, ETRI, Mediatek, ZTE/Pengcheng laboratory, Ericsson, Sharp, Xiaomi (Case 2b/3b), TCL, Ruijie Networks, NTT Docomo, OPPO (optional), Lenovo (Case 1,2b,3b), Tejas, IITM/IITK |
| Alternative B (path-based measurement) (9 sources) | Qualcomm, Huawei/HiSilicon, Samsung, InterDigital, Intel, OPPO, Lenovo (Case 2a,3a), Tejas |
| Specify both (9 sources) | Apple, Nokia, Google, NVIDIA, Lenovo (vary case by case), NEC (compromise), Ericsson (compromise), Fraunhofer IIS/Fraunhofer HHI |

#### 4th round discussion

Regarding the measurement for obtaining model input, the existing agreements on sample-based measurement are aggregated below. This reflects the majority view.

Proposal 2.1.3.4-1

In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for obtaining model input,

- The measurement is composed of Nt' samples of the estimated channel response in time domain, The Nt’ samples are select from a list of Nt consecutive samples, where the Nt samples have timing granularity T.

- The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples.

- The timing information is defined relative to a reference time

For measurement by TRP/gNB,

- The Nt’ selected samples are expected to be those with the highest power.

- The starting time of the list of Nt consecutive samples is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

Proposal 2.1.3.4-1A

For Rel-19 AI/ML based positioning, support the following to obtain the time domain channel measurements,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time.

For measurement by gNB/TRP,

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candaiate set values:

  - Nt' ={9, 16, [24], [32]}

  - Nt = {32, 64, 128}

  - k = {3, 4, ,5}

  - The gNB/TRP may use different Nt' and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| OPPO | The contents in the proposal has already been agreed in several agreements, do we need to discuss this aggregated proposal? |
| TCL | Minor modifications: 1. In Rel-19 AI/ML based positioning, regarding the sample-based measurement for obtaining model input, 2. From RAN1 respective, LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. |
| mtk | 1, several meetings ago, the intention to down-selection failed. So now the proposal is still to try this way? 2, we support to contain both sample basedand the legacy path based |
| ZTE | Share similar view with OPPO, the above proposal collects the agreements we made before, not necessary. |
| Nokia | To agree between sample-based and/or path-based for Case 3b, we suggest the inclusion of off-set done by implementation in gNB side, but it should be indicated to LMF in the reporting. As commented in the online session, the first detected path in NLOS scenarios not necessarily will represent enough information of the channel response, at some point in certain SNR values, the main path may not be necessarily the first detected path. We understood explanation of vice-chair that it is up to implementation, but LMF should know that at least an offset was used, in the best scenario, the offset value mapped to the sample grid must be reported along with the measurement. Proposal 2.1.3.4-1(Updated) In Rel-19 AI/ML based positioning, regarding the time domain channel measurements for obtaining model input, - The measurement is composed of Nt' samples of the estimated channel response in time domain, The Nt’ samples are select from a list of Nt consecutive samples, where the Nt samples have timing granularity T. - The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - The timing information is defined relative to a reference time For measurement by TRP/gNB, - The Nt’ selected samples are expected to be those with the highest power. - The starting time of the list of Nt consecutive samples is determined as: starting time = first detected path rounded down with timing granularity T. - The starting time may consider optionally an offset, which is up to implementation, but it should be indicated to LMF. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. |
| Hw/HiSi | Our view is still that ambiguity for case 3B will not occur even with path based measurements. For the proposal on the on the table, it is not adding additoinal information to what we have agreed so far it seems, they need to be made concrete before moving on.: The critical issue is still missing, i.e. - the values for Nt, Kt‘ and k. - Whether the gNB must follow the signaled values. |
| InterDIgital | Similar view as other companies. We don’t understand the purpose of this proposal. The only new proposal compared to the agreement made in RAN1#117 is the last subbullet about NRPPa signaling but it is not useful information for downselecting between path and sample based measurements. |
| Tejas | We are okay with the proposal. |

#### Thursday offline discussion

The version of the proposal edited at Thursday online session is copied below:

For Rel-19 AI/ML based positioning, for case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. For measurement by gNB/TRP, - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - FFS: Nt' - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Proposal 2.1.3.5-1A

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from  gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

[Further compromise: Nt'={9,16}]

## Timing measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 8: For Case 3b, reuse the legacy reporting of timing information or timing and power information from gNB/UE to LMF. - The use of phase information for the measurement reporting would need further justification.

- ITL (R1-2410588) Proposal 1: For AI/ML based positioning Case 3b, the existing UL RTOA reference time T0+tSRS is applicable for only UL part, and TRP transmit timing of DL subframe should be used to support multi-RTT.

• Google (R1-2410150) Proposal 1: Support the following options for the channel measurement for training data collection - Option 1: Time information and power information - Option 2: Time information, power information and phase information

• CATT, CICTCI (R1-2409926) Proposal 15: For case 3b, the following one or two options of timing information reporting of sample-based measurements are considered: - Option 1: Sample-based reporting - Time offset: the time offset is the difference between the timing of first sample and UL RTOA reference time. - Bitmap: the bitmap is used to represent the timing information of Nt’ samples and the first bit is corresponding to the first sample of Nt’ samples. - Option 2: Path-based reporting - Resolution step of the timing information of path should be an integer multiple of sampling periods. - Path-based reporting may be enhanced to support reporting more samples.

## Power measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 8: For Case 3b, reuse the legacy reporting of timing information or timing and power information from gNB/UE to LMF. - The use of phase information for the measurement reporting would need further justification.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 11: The mapping table of RSRPP in TS 38.133 can be reused for AI/ML positioning.. Proposal 7: There’s no need to introduce power quality for channel measurement.

• CMCC (R1-2409500) Proposal 4: For path-based measurement reporting, current DL PRS-RSRPP and UL-SRS-RSRPP can be used as a starting point.

- Samsung (R1-2409582) Proposal 5. For paired timing information and power information, following two types are considered: (b-1): separate timing and power values with jointly reporting (b-2): joint time and power value and reporting

- Apple (R1-2409788) Proposal 2-3: Support both absolute and differential RSRPP mapping for RSRPP values - The magnitude depends on the input type - Delay profile: no magnitude - Power Delay Profile: RSPP - CIR: RSPP and phase

• CATT, CICTCI (R1-2409926) Proposal 19: For case 3b, if sample-based measurement is supported, reuse the existing definitions of UL SRS-RSRPP to represent the power information by replacing ‘path’ to ‘sample’, and reuse the existing report mapping table of UL SRS-RSRPP to report the power information.

## Phase measurement for model input

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 8: For Case 3b, reuse the legacy reporting of timing information or timing and power information from gNB/UE to LMF. - The use of phase information for the measurement reporting would need further justification.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 10: In AI/ML based positioning, phase information is used for determining model input.

• CMCC (R1-2409500) Proposal 5: For AI/ML based positioning, it needs more discussion on the feasibility of obtaining the ground-truth label via PRUs, in which case the training dataset size is large. Proposal 6: For AI/ML based positioning, more discussion is needed for the comparison between CIR and PDP as model inputs. Proposal 7: For AI/ML based positioning, additional configurations or limitations can be supported to improve the efficiency of the measurement data reporting for positioning.

- Intel Corporation (R1-2409742) Proposal 13: If phase information as model input is supported, the existing definitions of DL RSCP and UL RSCP should be extended to represent phase information for CIR as AI/ML model input so as to include phase information for the additional detected paths for path-based measurement or additional samples for sample-based measurement of the CIR as model input. - FFS: Further adaptations to the existing definitions.

- Baicells (R1-2410414) Proposal 3. Support phase information reporting for Case 3b.

• Sharp (R1-2410360) Proposal 3: For AI/ML based positioning, continuous phase information is supported for model input.

• Google (R1-2410150) Proposal 1: Support the following options for the channel measurement for training data collection - Option 1: Time information and power information - Option 2: Time information, power information and phase information

- New H3C Technologies Co., Ltd. (R1-2409543) Proposal 1: For AI/ML based positioning, CIR measurements is supported for model input for both direct positioning and assistant positioning. Proposal 2: The initial phase mismatch of CIR measurements must be eliminated before being input to AI/ML models during training and inference. Proposal 3: For the phase alignment algorithm of CIR measurement, support the relative phase method to reduce implementation costs and improve algorithm usability. Proposal 5: For the case where CIR is employed as the AI/ML positioning model input, in order to eliminate the initial phase mismatch in CIR, at least support selecting „reference sample“ through the „strongest path“ and „first satisfied path“. Proposal 7: For the case where CIR is employed as the AI/ML positioning model input, in order to eliminate the initial phase mismatch in CIR, at least support calculating „reference phase“ through „mean method“ and „minimum phase method“; Proposal 9: For the case where CIR is employed as the AI/ML positioning model input, in order to eliminate the initial phase mismatch in CIR，support the physical layer process and air interface signalling transmission scheme mentioned above;

- NVIDIA (R1-2409781) Proposal 1: Support reporting phase information in time domain channel measurements for AI/ML based positioning. Proposal 2: Support configurable CIR measurement report (e.g., report size, measurement window size) in an effort to reduce the signalling overhead.

- Sony (R1-2410217) Proposal 1: For data collection, support radio channel characteristics reporting in a form of channel impulse response (CIR) for AI/ML positioning. Proposal 2: Support configurable CIR measurement report (e.g., report size, measurement window size) in an effort to reduce the signalling overhead.

- Fujitsu (R1-2410049) Proposal-3: Study on phase measurement and reporting for the model input of Case2b/3b is deprioritized in Rel-19 normative work.

- Fraunhofer IIS, Fraunhofer HHI (R1-2410205) Proposal 4: Support a complex valued sample-based reporting offering: - Lossless reporting of the channel impulse response - Supporting future enhancements of the AI/ML model

- Apple (R1-2409788) Proposal 6: On the use of CIR model input for AI/ML positioning: - The relative performance of CIR and PDP depends on the complexity of the AI/ML model. - As complexity increases, CIR shows better performance than PDP. - With phase mismatch between the transmitter and receiver, the accuracy of AI/ML based positioning degrades. - Option 1: This can be mitigated by training the model with data that suffers a similar mismatch. - Option 2: This can be mitigated by training the model with data that is compensated to remove the effect of the mismatch. Proposal 7: On other aspects, limit the overhead, a model may be able to support mixed input with CIR input for the TRPs closest and PDP/DP for TRPs further away. Proposal 8: RAN1 to support using phase information (in addition to timing information and power information) for determining model input (i.e. support CIR based model input). Proposal 11: Enhance current measurement input reports to LMF for case 3b to support feedback of the CIR, PDP, and DP. - Examples include adding support to signal more elements and support for phase information.

- NEC (R1-2409852) Proposal 11: For case 1, whether support to use phase information as model input is up to UE implementation. Proposal 12: For case 3a, whether support to use phase information as model input is up to gNB implementation. Proposal 13: For case 3b, support phase information reported from gNB to LMF as model input. − The phase information introduced in Rel-18 can be a start point

- Xiaomi (R1-2409878) Proposal 5: The support of CIR should be deprioritized

• CATT, CICTCI (R1-2409926) Proposal 18: Phase measurement and reporting is supported for case 2b and case 3b, up to UE/TRP capability. Subject to UE/TRP capability indication, LMF makes the decision on whether to report the phase measurement and send the corresponding indication to UE/gNB/TRP.

- Nokia (R1-2409986) Proposal 39: CIR is not supported for inference input when the model running at the LMF-side, gNB-side or UE-side cases. In addition, if first legacy RSCP/RSCPD are considered as inference input, the LoS determination and integer ambiguity needs to be studied.

- OPPO (R1-2410102) Proposal 2: For R19 AI-based positioning, NOT support the reporting based on phase information (in additional to timing information and power information).

- Qualcomm Incorporated (R1-2410467) Proposal 22: In AI/ML positioning Case3b, for model input measurements, no support for reporting phase information (e.g., CIR) for model input running at LMF side.

- NTT DOCOMO, INC. (R1-2410377) Proposal 8: For AI/ML based positioning, in addition to timing information and power information, phase information report is considered for determining model input. - Rel-18 measurements (e.g., DL RSCPD, DL RSCP, UL RSCP) are considered as a baseline for phase information report.

- Lenovo (R1-2410019) Proposal 8: RAN1 to support the following additional model input types for DL-based Direct AI/ML positioning measurements based on DL-PRS: - Support channel observation measurements in the form of DL CIR measurements Proposal 9: RAN1 to support the following additional model input types for UL-based Direct AI/ML positioning measurements for case 3b based on SRS for positioning: - Support channel observation measurements in the form of UL CIR measurements - Support the inclusion of additional channel profiles such as UL-based angle-delay domain (paired angle/phase and timing information).

- InterDigital, Inc. (R1-2409845) Proposal 29: Do not introduce CIR where all paths or samples (if supported) have phase information Proposal 30: Support first-path phase measurement, namely RSCP and RSCPD, for AIML based positioning

- vivo (R1-2409669) Proposal 1: Channel measurements including CIR, PDP and DP, should be supported for data collection, which can provide more freedom of implementation for model developer. Proposal 24: In addition to delay and power, at least RSCP and RSCPD should be supported for AI/ML based positioning. Proposal 25: Raw CIR reporting can be supported, and LMF can eliminate the impact of random initial phases of transceiver by implementation.

- Ericsson (R1-2409443) Proposal 12 Do not support phase information for determining model input, including CIR and single phase value for first path/sample. Proposal 13 Before CIR can be adopted as model input, RAN1 need to investigate whether fingerprinting ML models can handle CIR phase measurements, which vary not only with the radio channel environment but also with the transmitter/receiver circuits.

### 1st round discussion

Regarding the issue of using phase information (in addition to time and power) for determining model input, the investigation follows from the agreement made in RAN1#116 (Feb 2024).

Agreement (RAN1#116) For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

For this topic, no decision could be made up to RAN1#118bis. In contributions to RAN1#119, companies provided further analysis and expressed their views as follows (Please update/correct if your view is not captured correctly).

| Support reporting CIR (i.e., phase info in addition to timing info and power info)? |  |
| --- | --- |
| Yes (17) | vivo, Sharp, NTT DOCOMO, Apple, Sony, Fraunhofer IIS/Fraunhofer HHI, ZTE/Pengcheng laboratory, NVIDIA, Lenovo, NEC, CATT/CICTCI, Google, Baicells, New H3C |
| No (11) | Qualcomm, Nokia, Fujitsu, MediaTek, OPPO, Huawei/HiSilicon, Xiaomi, InterDigital, Ericsson, Intel |

| Support reporting only first path phase or first sample phase? |  |
| --- | --- |
| Yes (5) | vivo, InterDigital, ZTE/Pengcheng laboratory, NTT DOCOMO (?) |
| No (9) | Qualcomm, Fujitsu, Huawei/HiSilicon, OPPO, Ericsson, Intel, Nokia, Sharp |

For reference, RAN1#118bis status (see R1-2409286) is shown below. Note that status has stayed similar from RAN1#117 to RAN1#118bis.

| Support reporting CIR (i.e., phase info in addition to timing info and power info)? |  |
| --- | --- |
| Yes (16) | vivo, Sharp, NTT DOCOMO, Apple, Sony, Fraunhofer IIS/Fraunhofer HHI, ZTE/Pengcheng Laboratory, NVIDIA, Lenovo, NEC, CATT/CICTCI, Google, Baicell, New H3C |
| No (11) | Qualcomm, Nokia, Fujitsu, MediaTek, OPPO, Huawei/HiSilicon, Xiaomi, InterDigital, Ericsson, Intel |

| Support reporting only first path phase or first sample phase? |  |
| --- | --- |
| Yes (4) | vivo, InterDigital, ZTE/Pengcheng Laboratory |
| No (7) | Qualcomm, Fujitsu, Huawei/HiSilicon, OPPO, Ericsson, Intel |

As shown above, companies’ view is almost the same in from RAN1#117 to RAN1#119. It is unlikely that RAN1 can converge even if more time is given. Based on the discussion status since the start of the work item, it is suggested to conclude this discussion as below.

Conclusion 2.4.2-1

For Rel-19 AI/ML based positioning for all use cases, there is no consensus in RAN1 to support CIR for determining model input.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| OPPO | Fine with the proposal |
| New H3C | Not support because it is too early to conclude “no concensus” |
| CMCC | Fine. |
| CATT, CICTCI | We think a more constructive way is to support it by specification, and then subject to UE/TRP is capability. |
| Nokia | ok |
| HwHiSi | Ok |
| DOCOMO | 17 companies proposed phase information. It is too early to conclude “no consensus.” |
| NEC | “all use cases“ can be changed to “case 3b“, becasue case 2a and case 2b should not be referred here, and it up to UE/gNB implementation to determine whether phase information can be used as model input for case 1 and case 3a. |
| ZTE | Share similar view with New H3C. |
| Fujitsu | OK |

Conclusion 2.4.2-2

For Rel-19 AI/ML based positioning for all use cases, in terms of using phase information (in addition to timing information and power information) for determining model input, there is no consensus in RAN1 to support using one phase value (e.g., Rel-18 measurements DL RSCPD, DL RSCP, UL RSCP) for the first path or first sample only.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| OPPO | Fine with the proposal. |
| New H3C | Not support because it is too early to conclude “no concensus” |
| CMCC | For path-based measurement, if not additional spec impacted is justified for supporting AI based positioning, we do not see a strong motivation to block a existed information in legacy positioning not used for AI. |
| InterDigital | As we discussed in our contribution, we see benefits for using already specified phase information. |
| Nokia | Ok |
| HW/HiSi | Ok |
| DOCOMO | Existing phase information in Rel-18 can be reused as much as possible. |
| ZTE | Agree with CMCC, IntegerDigital and DOCOMO. |

## Other

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) - Proposal 9: For Case 3b, there is no need to specify the purpose of the measurement report from the gNB to the LMF (e.g., whether the it is used by LMF as model input or non-AI/ML usage, whether it is used by LMF for training/inference/monitoring, etc.) as the usage of the measurements is LMF implementation and should be transparent to the gNB.

- Apple (R1-2409788) Proposal 21: To specify transfer of measurement data for both PRS and UL SRS for positioning, the following parameters may need to be configured: - The measurement type: delay Profile (timing only), Power Delay Profile (timing, power), Channel Impulse Response (timing, power and phase) - The number of measurements (taps): 8, 9, 16, 32, 64, [128], [256] (+1) - Feedback Format - First path parameters: first path magnitude, first path phase first path timing, timing granularity factor. - Additional path parameters - Relative time difference for additional paths - Time reporting granularity factor - Time stamp - Part A quality - Part B quality - Additional path magnitude and phase - Option 1: additional path magnitude, additional path phase - Option 2: additional path relative magnitude, additional path phase

- Nokia (R1-2409986) Proposal 33: For the data collection of Case 3b (LMF-side), for inference input, depending on the channel observation, UE/gNB can determine/select the required reporting configuration to enable the reporting windowing scheme without compromising the DL/UL measurements content. Observation 4: For assisted Case 1 monitoring and Inference Case 3b, to reduce the signaling overhead of reporting measurements from UE (case 1) and NG-RAN (Case 3b) to LMF, UE/NG-RAN may use a reporting scheme based on differential quantization indicating at least the number of RSRPP messages for carrying the measurement report.

• ETRI (R1-2410258 Proposal 1: For UE-sided models (Case 1/2a), when both training and inference are performed on the UE side, the model input format is determined by the UE vendors. Proposal 3: For gNB-sided model (Case 3a), when both training and inference are performed on the gNB-side, the model input format is determined by the gNB manufacturers.

# Model output for assisted AI/ML positioning

This section discusses model output for assisted AI/ML positioning and potential measurement enhancements.

## LOS/NLOS indicator for model output

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 10: For Case 3a, LOS/NLOS indicator can reuse the same format and definition as the existing IE “LoS/NloS Information” in 38.455. Proposal 11: When the LOS/NLOS indicator subject to Rel-19 Case 3a is reported, the LOS/NLOS indicator may be reported with other accompanied measurement information, e.g., legacy measurement information (e.g., timing information, AOA), or Case 3a based timing information. - Note 1: Case 3a based LOS/NLOS indicator reuses the legacy meaning, i.e., likelihood of a line-of-sight propagation path for the channel, regardless of the type of other accompanied measurement information. - Note 2: Any combination of legacy/Case 3a based LOS/NLOS indicator and other accompanied measurement information do not mutually conflict on their interpretations when jointly reported. Proposal 12: When timing information is reported for Rel-19 AI/ML positioning Case 3a, the report of the LOS/NLOS indicator should be “optional”.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 12: LOS/NLOS indicator cannot be reported without any other associated measurement. Proposal 13: From RAN1 perspective, when LOS/NLOS indicator is reported associated with other measurement for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) LOS/NLOS indicator; - (Optional) Quality of the LOS/NLOS indicator. Note: LOS/NLOS indicator share the same time stamp with the associated measurement. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Proposal 14: For AI/ML-assisted positioning Case 3a, when the LOS/NLOS indicator is reported together with the legacy timing measurement result, from RAN1 perspective, - The LOS/NLOS indicator provides the likelihood of a line-of-sight physical propagation path for the channel measured over the resources for which the measurement is reported; - One LOS/NLOS indicator value is optionally reported from the source to the receiver as in the existing specification. - The LOS/NLOS indicator can reuse the existing IE LoS/NloS Information in 38.455. Proposal 15: For AI/ML-assisted positioning Case 3a, when the LOS/NLOS indicator is reported with the AI/ML model output, from RAN1 perspective, - The LOS/NLOS indicator provides the likelihood of a line-of-sight virtual path for the model output over the resource for which the measurement is reported. - The LOS/NLOS indicator can reuse the same format as the existing IE LoS/NloS Information in 38.455.

• CMCC (R1-2409500) Proposal 11: For AI/ML assisted positioning, legacy LOS/NLOS information reporting could be used.

• Samsung (R1-2409582) Proposal 19. RAN1 supports LOS/NLOS indicator is reported together with other associated measurement (e.g., timing information).

- Intel Corporation (R1-2409742) Proposal 16: For AI/ML assisted positioning Case 3a, when the LOS/NLOS indicator is reported, - - Consider the below alternatives: - Alt A: the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NloS, where the propagation path corresponds to the associated timing metric that is determined as AI/ML model output and reported in the same report; - Alt B: the LOS/NLOS indicator provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver with a value of 1 corresponding to LoS and a value of 0 corresponding to NloS, where the propagation path corresponds to the actual radio propagation path between the transmitter and the receiver and may not correspond to the timing metrics included in the same report; - The LOS/NLOS indicator reuses the same format as the existing IE „LoS/NloS Information“ in TS 38.455; - One LOS/NLOS indicator is optionally reported per UE as in the existing specification.

- Baicells (R1-2410414) Proposal 4. Reuse existing “LoS/NloS information” in TS 38.455 for Case 3a.

• Sharp (R1-2410360) Proposal 4: For AI/ML assisted positioning in case 3a, study the following options for down-selection: - Option 1: the LOS/NLOS indicator is provided with definition change in current specification in case that the associated timing information is predicted by AI/ML. - Option 1-1: the LOS/NLOS indicator provides the confidence of a virtual LOS propagation path of the associated timing information of the AI/ML model output. - Option 1-2: the LOS/NLOS indicator provides the likelihood of a physical LOS propagation path of the channel measurement used for input of the AI/ML model. - Option 2: the LOS/NLOS indicator is not provided in case that the associated timing information is predicted by AI/ML. Proposal 5: For the case that the associated timing information is predicted by AI/ML, the LOS/NLOS indicator provides the confidence of a virtual LOS propagation path of the associated timing information of the AI/ML model output.

- Ruijie Networks Co. Ltd (R1-2409841) Proposal 2: From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp; - (Optional) LOS/NLOS indicator. - Existing IE „LoS/NloS Information“ in 38.455 can be reused. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

- Sony (R1-2410217) Proposal 7: For AI/ML assisted positioning case 2a and case 3a, the positioning measurement reporting includes LOS/NLOS indicator using the legacy reporting format (i.e., as in TS 37.355 for case 2a , and as in 38.455 for case 3a) and the indication whether the measurement is based on AI/ML computation or not.

- Fujitsu (R1-2410049) Proposal-4: For AI/ML assisted positioning Case3a, when the LOS/NLOS indicator is reported, from RAN1 perspective, - LOS/NLOS indicator is reported together with the timing information, which is also reported for Rel-19 Case 3a (FFS: other associated measurement) - The meaning of LOS/NLOS indicator is changed to a confidence level of the timing information corresponding to the (virtual) LOS path generated from the model output. - The LOS/NLOS indicator can reuse the same format as the existing IE „LoS/NloS Information“ in 38.455 for Case 3a.

- Tejas Networks Limited (R1-2409750) Proposal-13: We propose that, the receiver should report a new LoS/NloS indication that provides information on whether the reported measurement itself is LoS/NloS. (FL note: for training data collection)

- Apple (R1-2409788) Proposal 10: For Rel-19 Case 3a, if the LOS/NLOS indicator is reported with other associated measurement (e.g. timing information) obtained by legacy positioning methods, - the LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported - The LOS/NLOS indicator can reuse the existing IE „LoS/NloS Information“ in 38.455, which can be soft indicator or hard indicator.

- NEC (R1-2409852) Proposal 15: At least for case 3a, there is no need to report LOS/NLOS indicator to LMF if the model output is timing information.

• CATT, CICTCI (R1-2409926) Proposal 21: For AI/ML-assisted positioning, no need to report LOS/NLOS indicator when timing information is reported based on the output of AI/ML model.

- Nokia (R1-2409986) Proposal 40: In Case 3a, when timing information is estimated with AI/ML, the legacy reporting framework is reused with no LOS/NLOS indicator associated with this estimation.

• ETRI (R1-2410258 Proposal 8: In the AI/ML-assisted positioning method, the timing information output from the model should be used independently, without incorporating the LOS/NLOS indication. Proposal 9: In the AI/ML-assisted positioning method, the LOS/NLOS indicator output from the model should be incorporated with the legacy timing measurement.

- Qualcomm Incorporated (R1-2410467) Proposal 5: In AI/ML positioning Case3a, for model output, the LOS/NLOS indicator reuses the existing IE „LoS/NloS Information“ in 38.455, which can be soft indicator or hard indicator. Proposal 6: In AI/ML positioning Case3a, for model output, the LOS/NLOS indicator needs to be associated with at least one measurement results value (e.g., angle, timing, power, phase), which can be obtained using AI/ML or a non-AIML legacy approach. Proposal 7: In AI/ML positioning Case3a, for model output, support enhancing the meaning when reporting multiple LOS/NLOS indicators and their associated measurement results to indicate multiple hypothesis for the reported path/TRP.

- NTT DOCOMO, INC. (R1-2410377) Proposal 12: - For case 3a, LOS/NLOS indicator can reuse the existing IE “LOS/NLOS Information” in 38.455. LOS/NLOS indicator is reported associated with other information (e.g., timing measurement) by legacy positioning methods.

- Lenovo (R1-2410019) Proposal 11: The legacy LOS/NLOS indicator reporting framework can be re-used for the model output generated by AI/ML. Proposal 12: The LOS/NLOS indicator can be associated with same legacy timestamp when timing information is reported for Rel-19 AI/ML positioning Case 3a.

- MediaTek Korea Inc. (R1-2410531) Proposal 2c-1: For the case that both timing info and LOS/NLOS indicator are derived by AI/ML, change the LOS/NLOS indicator definition slightly. The suggested definition is: “the likelihood that the LOS delay could be derived” Proposal 2c-2: The reporting which is through the AI/ML model output may need to be indicated to NW

- InterDigital, Inc. (R1-2409845) Proposal 33: For Case 3a, keep the current definition of the LOS indicator shown in TS 37.355, “the likelihood of a Line-of-Sight propagation path from the source to the receiver”

- vivo (R1-2409669) Proposal 28: To avoid the ambiguity on the relationship between reported LOS/NLOS indicator and timing measurement generated by AI/ML model and non-AI/ML methods, there are two solutions: • Attach an indicator to the timing measurement, and the indicator is used to indicate the associated timing measurement is generated by AI/ML model. • Do not report LOS/NLOS indicator when the reported timing measurement is generated by AI/ML model.

- Ericsson (R1-2409443) Proposal 28 When timing information is reported for Rel-19 AI/ML positioning Case 3a, a LOS/NLOS indicator can be optionally reported, the LOS/NLOS indicator reuse the same format as the existing IE „LOS/NLOS Information“ in 38.455, and its definition. If a timing measurement is reported together with the LOS/NLOS indicator, the indicator status (for physical LOS/NLOS) is decoupled from the reported timing measurement (for virtual LOS). Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. Proposal 29 (for conclusion) For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, do not support an indication of whether the LOS/NLOS indicator is obtained based on a model output. Proposal 30 (for conclusion) For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, do not support reporting of the LOS/NLOS indicator as a separate measurement (i.e., without other associated measurement).

### 1st round discussion

On the LOS/NLOS indicator when it is generated as model output for AI/ML assisted positioning, companies have shared their views on the definition of the indicator, the possible combinations of LOS/NLOS indicator with timing measurements, and whether/how to reuse the existing IE. The final versions of the proposals at RAN1#118bis are copied below, although no agreements were made.

Proposal 3.1.5-1 For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the other associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective, - The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported; - The LOS/NLOS indicator can reuse the existing IE „LoS/NloS Information“ in 38.455, which can be soft indicator or hard indicator. - Note: it is up to RAN3 to decide how to differentiate a report for Rel-19 Case 3a from a report for pre- Rel-19 positioning. FFS: whether LOS/NLOS indicator is reported when the associated timing information is generated by AI/ML, and if so, the meaning of the LOS/NLOS indicator. Proposal 3.1.5-2 For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, RAN1 study whether the following reporting are supported: - LOS/NLOS indicator is reported together with other associated measurement (e.g., timing information), which is also reported for Rel-19 Case 3a; - If supported, study the meaning of the LOS/NLOS indicator; (2) LOS/NLOS indicator is reported without any other associated measurement (e.g., timing information); - If supported, the LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported.

Question 3.1.2-1: what’s the meaning of the LOS/NLOS indicator when generated by AI/ML for assisted positioning?

For the case where the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the associated measurement (e.g., timing information) is obtained by legacy positioning methods, companies’ views are shown below. (please update/correct if your view is not captured correctly)

- all companies support to reuse the same meaning as legacy LOS/NLOS indicator,

- except that Qualcomm suggest to repurpose the LOS/NLOS report to indicate multiple hypothesis for the reported path/TRP.

For the case (if supported, see Question 3.1.2-2 below) where the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the associated measurement (e.g., timing information) is also reported for Rel-19 Case 3a (i.e., both timing info and LOS/NLOS indicator are generated by AI), companies’ views on the meaning of LOS/NLOS indicator are shown below. (please update/correct if your view is not captured correctly)

- Always the same meaning as legacy LOS/NLOS indicator. LOS/NLOS indicator provides the likelihood of a line-of-sight physical propagation path for the channel measured over the resources for which the measurement is reported.

  - Supported by (13 sources): Huawei/HiSilicon, Baicells, Samsung, Ericsson, CMCC, Apple, InterDigital, Sony, NTT DOCOMO, Lenovo, Ruijie Networks, vivo

- Meaning of the LOS/NLOS indicator is changed when the associated timing information is also generated by AI/ML.

  - Supported by (5 sources). The suggested new meaning varies among the supporters.

    - Fujitsu (changed to: a confidence level of the timing information),

    - MediaTek (change to: “the likelihood that the LOS delay could be derived”),

    - ZTE/Pengcheng Laboratory (change to: likelihood of LoS virtual path),

    - Tejas (change to:  information on whether the reported measurement itself is LoS/NloS),

    - Sharp (change to: confidence of a virtual LOS propagation path of the associated timing information of the AI/ML model output)

- Repurpose for enhanced meaning. Enhance the meaning to indicate multiple hypothesis for the reported path/TRP

  - Supported by (1 sources): Qualcomm

If the combination above is supported (see Question 3.1.2-2 below), it is prudent to follow the majority view to always reuse the same meaning as legacy LOS/NLOS indicator. If the meaning is to be changed, it is unclear what the new meaning should be, given that proponent companies have divergent opinions. Note: if the combination is not supported in Rel-19 (see Proposal 3.1.2-2 below), then there is no need to further discuss the new meaning of LOS/NLOS indicator.

Proposal 3.1.2-1

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective,

- The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

- The LOS/NLOS indicator can reuse the existing IE “LoS/NloS Information” in 38.455, which can be soft indicator or hard indicator.

- Note: it is up to RAN3 to decide how to differentiate reporting for Rel-19 Case 3a from reports from pre- Rel-19 positioning.

Monday offline:

Proposal 3.1.2-1A

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective,

- The LOS/NLOS indicator reuse the meaning of the LOS/NLOS indicator from TS 38.455, i.e., provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

- The LOS/NLOS indicator can reuse the meaning and format of the existing IE “LoS/NloS Information” in 38.455, which can be soft indicator or hard indicator.

- Note: it is up to RAN3 to decide how to differentiate reporting for Rel-19 Case 3a from reports from pre- Rel-19 positioning.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital, Nokia |

| Company | Comments |
| --- | --- |
| InterDigital | For the first sub-bullet, our proposal is to state to reuse the definition of the LOS/NLOS indicator from TS 37.355. |
| Lenovo | Ok in principle, but the first bullet mentions over a “measured over a resource” but legacy also supports TRP-specific LOS/NLOS granularity. Suggest a clarification to the first bullet. |
| CATT, CICTCI | Support. |
| Nokia | We are strongly do not support this proposal. Why is the necessity to consider an hypothesis that a Legacy reporting and AIML reporting should be discussed in RAN1?. In or view this proposal is creating an hypothesis that may impact in RAN3. We propose a rewording starting for the following text: “ If RAN3 agrees to report only LOS/NLOS by using assisted-AIML, the following aspects may be considered …“ |
| Apple | Okay with proposal |
| HWHiSi | Ok |
| DOCOMO | OK |
| Qualcomm | Clarification Option C in the discussion above is not our intention. We are not proposing to change meaning of LOS/NLOS indicator. We find the legacy meaning is enough. What we are proposing is repurpose the whole reporting of LOS and other measurements to let device indicate multiple measurements (i.e., ‘hypothesis’) for a given resource when reported. In addition, it is not clear why the proposal only focuses on AIML LOS only when associated with legacy measurement. Let’s discuss association in general whether other measurement irrespective on whether other measurement is AIML or not. Updated Proposal 3.1.2-1 For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a with an associated measurement (e.g., timing information, power, angle, phase) is obtained by legacy positioning methods, from RAN1 perspective, - The LOS/NLOS indicator provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported; - The LOS/NLOS indicator can reuse the existing IE „LoS/NloS Information“ in 38.455, which can be soft indicator or hard indicator. - Note: it is up to RAN3 to decide how to differentiate reporting for Rel-19 Case 3a from reports from pre- Rel-19 positioning. |
| Mtk | 1, if it is only AI/ML for LOS/NLOS, we wonder whether it deserves 3GPP network to consume the resources to build it up? 2, We also want to check with companies whether this is a good use case? 3, for the 1st subbullet, we can simply say The LOS/NLOS indicator is to reuse the existing definition |

Question 3.1.2-2: whether LOS/NLOS indicator can be reported together with timing information which is also reported for Rel-19 Case 3a (i.e., both are generated by AI/ML model)?

Companies views are summarized below. (please update/correct if your view is not captured correctly)

| Support [7 sources] | Qualcomm, Ericsson, Samsung, Huawei/HiSilicon, Fujitsu, Lenovo (?) |
| --- | --- |
| Not support [7 sources] | vivo, NEC, CATT/CICTCI, Nokia, ETRI, Baicells |

Considering the above, it is difficult to converge. It’s suggested to conclude that this combination is not supported.

Question 3.1.2-3: whether LOS/NLOS indicator generated by AI/ML can be reported without any associated measurement (e.g., timing information)?

Companies’ views are summarized below. (please update/correct if your view is not captured correctly)

| Support [1 source] | Apple |
| --- | --- |
| Not support [10 sources] | Qualcomm, Ericsson, ZTE/Pengcheng Laboratory, Samsung, Nokia, Huawei/HiSilicon (?), Lenovo (?), Baicells |

Considering the above, it is suggested to follow majority view.

In summary, if following the suggestions above, the possible combinations of associated measurement and LOS/NLOS indicator are supported or not as shown in Table 1.

Table 1. The possible combinations of associated measurement and LOS/NLOS indicator

|  |  | Associated measurement (e.g., timing) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Absent | Legacy measurement* | Timing measurement generated by AI/ML |
| LOS/NLOS indicator | Absent | N/A | Support (Existing specification) | Support in Rel-19 |
|  | Generated by legacy methods | Not supported (Existing specification) | Support (Existing specification) | Not supported in Rel-19 |
|  | Generated by AI/ML | Not supported in Rel-19 | Support in Rel-19 | Not supported in Rel-19 |

* When the associated legacy measurement is reported, it is not limited to timing measurements. It can be timing measurement or angle measurement.

Note: after the issues in Proposal 3.1.2-2 below are settled, the FFS bullet in the previous agreement can be addressed quickly. For example, if Proposal 3.1.2-2 is agreeable, then this implies that LOS/NLOS indicator is an optional field to report for Case 3a.

Agreement (RAN1#118bis) From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Proposal 3.1.2-2

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

(1) When timing information is reported for Rel-19 Case 3a, LOS/NLOS indicator is not reported regardless of whether it is generated by legacy method or AI/ML.

(2) When no associated measurement (e.g., timing information) is reported, LOS/NLOS indicator is not reported for Rel-19 Case 3a.

Monday offline:

Proposal 3.1.2-2A

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

- When no associated measurement (e.g., timing information) is reported, LOS/NLOS indicator is not reported for Rel-19 Case 3a.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| OPPO | When the timing information is generated by AI/ML, LOS/NLOS indicator is not necessarily needed for positioning accuracy. We are fine with the proposal. |
| CMCC | For the first bullet, the intention is fine but the wording is not very clear for us. Maybe ‘When timing information generated by AI/ML is reported‘ is clearer. For second bullet, fine. |
| CEWiT | Similar view as CMCC |
| InterDigital | We don’t understand why AIML generated LOS indicator needs to be accompanied by timing information. |
| ETRI | Same view with CMCC for (1). Sugesst to change (2) to (2) When no associated legacy measurement (e.g., timing information) is reported provided, LOS/NLOS indicator is not reported for Rel-19 Case 3a. |
| Lenovo | In (1) it should be clarified that the timing information including UL RTOA or gNB Rx-Tx time difference are AI/ML generated timing information. This implies there is no need for another AI/ML generated LOS/NLOS indicator to accompany a AI/ML generated timing information measurement. |
| CATT, CICTCI | We can accept this conclusion. |
| Nokia | Looks ok, but it may be also a good chance to get a common understanding between companies about the necessity of LOS/NLOS reporting for AIML positioning Rel. 19. |
| HW/HiSi | This proposal unnecessarily restricts the usability of a Rel-19 Case 3a LOS/NLOS indicator. In addition, this proposal is related to the definition of LOS/NLOS indicator. As such, it may be better to first agree on the definition of LOS/NLOS indicator for the different combinations, i.e., that no change in the meaning of LOS/NLOS indicator is needed. Some further coments to each of the listed points is provided below: Regarding point (1), it is not clear why it should be mandated that the gNB does not report the LOS/NLOS indicator, when in legacy it can be reported optionally. In addition, this option may go against the RAN1#116 agreement, which does consider that the both LOS/NLOS indicator and timing information can be reported for Rel-19 Case 3a: Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Regarding point (2), it is also not clear why a Rel-19 Case 3a LOS/NLOS indicator can be reported for a timing measurement, but it can not be reported along with another legacy measurement, e.g., AOA. |
| DOCOMO | Fine with this proposal. Modification by CMCC is better for us. |
| NEC | Share the view with CMCC. |
| ZTE | The first bullet is not clear for us. Does this mean that the LOS/NLOS indicator should not be reported with timing information? It seems contradict to the second bullet. The general principle in (2) is fine, i.e., LOS/NLOS cannot be reported when other measurement is not reported. |
| Fujitsu | (2) is fine for us. Regarding (1), We think both the timing information and its LOS/NLOS being generated by AI/ML are needed. For a measurement report from gNB to LMF, the confidence level of the timing information indicated by the LOS/NLOS indicator is helpful for LMF to derive a high accuracy location information considerring the quality/reliablity of the timing information from different gNB could be quite diverse. Without this indicator, LMF has to treat all the reported timing information equally. |
| Qualcomm | This proposal and Proposal 3.1.2-1 read out as AIML LOS to be only reported when associated measurement is legacy one. Can you clarify why it cannot be associated with other measurement(s) when obtained using AIML? |
| Mtk | 1, LOS/NLOS reporting is optional when timing information is reported. In our view, it belongs to good-to-have. Why to restrict NOT to report? 2, Even though timing is derived by AI/ML, it doesnt guarantee that the model can always output the timing that is equal to “LOS path“ under NLOS channel. It may highly depend on training density. So, the model can also output the confidence that the timing to be LOS estimation. What is the concern? 3, Then we want to further ask the opponents that, if LOS/NLOS with the changed meaning is NOT supported, do you consider to create any new field to address the “virtual LOS path“? |

### 2nd round discussion

Regarding the meaning of the LOS/NLOS indicator, several companies commented that we can first agree that the meaning of LOS/NLOS indicator is not changed. Then we can discuss whether to support reporting the LOS/NLOS indicator together with associated measurement, when both are generated by AI/ML. Also: no companies proposed to support the combination when LOS/NLOS is generated by legacy method, associated measurement is generated by AI/ML.

Hence the proposals are updated accordingly.

Proposal 3.1.3-1

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective,

- The LOS/NLOS indicator can reuse the meaning and format of the existing IE “LoS/NloS Information” in 38.455, which can be soft indicator or hard indicator.

- Note: it is up to RAN3 to decide how to differentiate a report for Rel-19 Case 3a from a report for pre- Rel-19 positioning.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| CATT | We are OK if this proposal and the next one are agreed as package. |
| HW/HiSi | Ok with a small change. For the LOS indicator there is in our view no need to destinguish between legacy and Rel-19. Therefore, „whether“ could be added to the note. Note: it is up to RAN3 to decide whether/how to differentiate a report for Rel-19 Case 3a from a report for pre- Rel-19 positioning. |
| Nokia | In our view, the current proposal is not providing further information when compared to the following agreement. Here, the proposal is only emphasizing the definition for LOS/NLOS reporting done in NRPPa protocol. Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. |
| mtk | 1, when timing information is also derived by AI/ML, the LOS/NLOS indication by using the legacy definition becomes meaningless. So, the proponents are to consider to have any new signalling to indicate how confident the AI/ML can estimate the actual LOS delay? If the answer is yes, we will disagree with this proposal |
| ZTE | For the following 3 proposal, we are discussing the association of timing information and LOS/NLOS indicator, and there are 4 combinations: - Legacy timing + AI/ML LOS/NLOS indicator - AI/ML timing + AI/ML LOS/NLOS indicator - Legacy timing + legacy LOS/NLOS indicator (already supported in legacy LPP) - AI/ML timing + legacy LOS/NLOS indicator Among the above combinations, 1 and 2 could be discussed further, but the current proposal looks like a combining 1 and 2, but the meaning of LOS/NLOS indicator for 1 and 2 may be different. We prefer the previous version. |
| DOCOMO | OK in general. |
| Lenovo | In a similar observation to ZTE: We should strive to avoid redundant AI/ML generated model outputs. We have the following scenarios: Scenario 1: Report AI/ML generated timing information measurement( e.g., UL-RTOA/gNB Rx-Tx time difference) associated with AI/ML generated LOS/NLOS indicator Scenario 2: Report legacy UL-measurements, e.g., UL-RTOA/gNB Rx-Tx time difference/UL-AoA associated with AI/ML generated LOS/NLOS indicator Scenario 3: Report AI/ML generated timing information measurement( e.g., UL-RTOA/gNB Rx-Tx time difference) only Scenario 4: Legacy Procedure: Report legacy UL-measurements, e.g., UL-RTOA/gNB Rx-Tx time difference/UL-AoA associated with legacy LOS/NLOS indicator Scenario 1 is not to be supported according to our view, while Scenarios 2 and 3 are needed. We understand that the “the meaning and format of the existing IE “LoS/NloS Information” according to the proposal, is to re-use the legacy definition for AI/ML generated LOS/NLOS indicators in Case 3a. |
| Tejas | We are unclear about how this measurement will be utilized by the LMF for improving positioning accuracy and what specific information it conveys regarding the timing information inferred by the AI-ML model. |

Proposal 3.1.3-2

For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported for Rel-19 Case 3a,

- LOS/NLOS indicator is not reported if obtained by legacy method.

|  | Company |
| --- | --- |
| Support | CATT |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | It is not clear why it should be mandated that the gNB does not report the LOS/NLOS indicator, when in legacy it can be reported optionally. The LOS /NLOS indicator by legacy may be beneficial, e.g., depending on implementation the LMF may consider the Rel-19 Case 3a timing information from TRPs which have a higher likelihood of LOS. We propose thus the following updated proposal: Updated Proposal 3.1.3-2 For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported for Rel-19 Case 3a, LOS/NLOS indicator is optionally not reported if obtained by legacy method. |
| Nokia | We suggest the following rewording on HW comment/proposal: Updated Proposal 3.1.3-2 For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported for Rel-19 Case 3a, From RAN1 point of view, LOS/NLOS indicator is optionally not reported if obtained by legacy method using legacy premises. Note: RAN3 has the final decision if specification impact is identified. |
| NEC | If timing information is generated by AI/ML model, it make sense that there is no need to report LOS/NLOS indicator which is generated by legacy way, so we suggest add “generated by AI/ML“ after “timing informattion“. |
| mtk | 1, this is a use case? |
| Qualcomm | We also do not see the need to restrict the reporting of LOS indicator based on whether it is obtained using legacy or AI/ML. |
| ZTE | The principle is fine. |
| DOCOMO | Fine that if timing information is geberated by AI/ML, LOS/NLOS indicator is not necessary. In our understanding, if timing information is generated by legacy, LOS/NLOS indicator generated by legacy is out of Rel-19 case 3a. |
| Tejas | If timing information is obtained using a legacy method and the LoS/NLoS indication is not reported, then what is the AI-ML use case? The proposal is not nclear to us. |

Proposal 3.1.3-3

For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported for Rel-19 Case 3a,

- RAN1 study whether to support reporting LOS/NLOS indicator if obtained by AI/ML.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| CMCC | When timing information is reported for Rel-19 Case 3a, legacy LOS/NLOS information seems not useful. We can discuss whether and how to differentiate a report for Rel-19 Case 3a from a report for pre- Rel-19 positioning, and whether this indication could reuse the LOS/NLOS IE this up to RAN3. |
| HW/HiSi | Not agree with the proposal. This proposal unnecessarily restricts the usability of a Rel-19 Case 3a LOS/NLOS indicator. |
| Nokia | We suggest FL to focus in another important topics that are entirely in RAN1 scope. Currently RAN3 is discussing Case 3a and most of the decissions on Reporting Framework and include/update current IEs to enable Case 3a is up to them. We suggest to wait for RAN3 discussion and support them in case an LS is sent to RAN1. |
| NEC | Does the “timing information“ only refer to the one that generated by AI/ML? If the timing informaiton is generated by legacy one, LOS/NLOS indicator is necessary to be reported. |
| mtk | 1, this proposal has some conflict with 3.1.3-1? For 3.1.3-1, it is to reuse legacy definition for LOS/NLOS, when LOS/NLOS is derived by AI/ML, and timining information is derived by legacy or AI/ML. Or, the subbullet saying “The LOS/NLOS indicator can reuse“, and “can“ here is to create some more possibility? |
| Qualcomm | LOS indicator is optional in previous releases. It shall be kept same |
| ZTE | If LOS/NLOS indicator is supported to report together with AI/ML timing information, the meaning of LOS/NLOS indicator should be refined. |
| DOCOMO | If timing information is geberated by AI/ML, LOS/NLOS indicator is not necessary. If timing information is generated by legacy, LOS/NLOS indicator generated by AI/ML is optionally reported. |
| Tejas | We agree with ZTE that when timing information is inferred using AI-ML models, the definition of the LoS/NLoS indication can be refined such that it can be utilized by the LMF to improve positioning accuracy. Additionally, this indicator can remain optional, as was the case in previous releases. |

### 3rd round discussion

For the LOS/NLOS indicator related topics, it has been difficult to make agreements. During the offline discussion, it was pointed out that, if no further agreements are made, then it can be understood that:

- The LOS/NLOS indicator always has the meaning and format of the existing IE, regardless of whether it’s generated by legacy method or generated by AI/ML;

- The possible combinations of associated measurement and LOS/NLOS indicator are supported, or not supported, as shown in Table 2.

Thus, unless RAN1 companies decide to change (a) or (b) above, there is no need to make further agreements.

There is only the need to differentiate the measurement generated by AI/ML vs legacy, so that the LMF can use the received reports accordingly. The differentiation can be achieved by adding a “made by AI” tag in the measurement report.

Table 2. The possible combinations of associated measurement and LOS/NLOS indicator

|  |  | Associated measurement (e.g., timing) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Absent | Legacy measurement* | Timing measurement generated by AI/ML |
| LOS/NLOS indicator | Absent | N/A | Support (Existing specification) | Support in Rel-19 |
|  | Generated by legacy methods | Not supported (Existing specification) | Support (Existing specification) | Support in Rel-19 |
|  | Generated by AI/ML | Not supported in Rel-19 | Support in Rel-19 | Support in Rel-19 |

* When the associated legacy measurement is reported, it is not limited to timing measurements. It can be timing measurement or angle measurement.

Proposal 3.1.4-1

For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, it is indicated to LMF whether the timing information is obtained by legacy method or by AI/ML.

- FFS: how to indicate

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia, HW/HiSi |

| Company | Comments |
| --- | --- |
| Nokia | It is clear that there are a diversity of understanding on this topic with potential high impact on RAN3. As most of the wording is not in RAN1 scope, e.g. using the wording „…indicate…“. In our view, the Agreement done by RAN3 on monitoring, which is done only in the gNB (no other entity involved in monitoring) derives the possibility that the gNB will be aware of the quality of the intermediate feature (any, including timing information), so, there should not be necessity to indicate explcitly (using an INDICATOR) if the intermediate feature was obtained by Legacy / AIML. |
| HW/HiSi | The key message is that LMF can destinguish Rel-19 timing info from legacy. Indication is not the only possibility, also the a Rel-19 request from LMF could be used for that. We can have a more generic description. The key difference between legacy and Rel-19 timing info is that legacy is measuring the UL RTOA, thus it is for the propagation path. Whereas in Rel-19 the timing is not measured and reflecting the direct virtual path between TRP and UE. Updated Proposal 3.1.4-1 For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, it is indicated to LMF shall be able to destinguish whether the timing information is obtained by legacy method or by Rel-19 AI/ML. - FFS: how to differentiate indicate |
| Tejas | - We are okay with the proposal. However, as mentioned by Huawei, the definition of timing information differs between the legacy model and AI-ML-based positioning. For AI-ML-based positioning, the timing information represents the virtual LoS path between the TRP and UE. - We propose that this distinction should either be explicitly captured in the agreement or considered as a working assumption. |

### 4th round discussion

Proposal 3.1.5-1

For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, LMF shall be able to destinguish whether the timing information is obtained by legacy method or by Rel-19 AI/ML.

FFS: how to ensure that LMF can distinguish

|  | Company |
| --- | --- |
| Support | New H3C |
| Not support | Nokia, HW7HiSi |

| Company | Comments |
| --- | --- |
| mtk | 1, this proposal is that RAN1 to agree that LMF should know about it, and then leave the signalling design to RAN3? 2, we also suggest to think about whether to reuse legacy LOS/NLOS definition when both LOS/NLOS and timing are derived by AI/ML. This is because, when both LOS/NLOS and timing info are derived by legacy, the soft value of LOS/NLOS can be used as the weighting factor on the measurement (timing info) for position calculation, for example WLS algorithm (weighted least square) If both LOS/NLOS and timing info are derived by AI/ML, BUT LOS/NLOS still reuses the legacy definition, it will happen that the indicator will output the soft value to strongly lean to NLOS, but the timing info due to AI/ML has the chance to get LOS delay. This means, the original LMF algorithm to rely on LOS/NLOS indicator for weighting the measurement can’t be used. We expect companies to think more about this. |
| ZTE | It seems no spec impact, how to distinguish it comes from AI/ML or legacy is up to LMF’s implementation. Taking MTK’s comments into account, we can discuss this later when meaning of LOS/NLOS indicator for AI/ML is clear. |
| Nokia | In RAN3 there was an important agreement on monitoring. Here, only gNB is doing the monitoring. With such scenario, there are multiple options in which the gNB doing the monitoring of output model to decide independent of LMF request the best intermediate feature to be reported. For example, if legacy detects NLOS, gNB may assume that AIML is an alternative to define the ToA, after a monitoring, it may select to report AIML or legacy . In summary, as commented before, we suggest to wait to RAN3 progress and support them if a potential LS is coming from them on such type of decisions. |
| HW/HiSi | The LMF only needs to distinguish between a timing information obtained via legacy or Rel-19 method. The key point for Rel-19 method is that it gives the timing info for the direct virtual path, and not for the propagation path as in legacy. How the Rel-19 timing info is obtained is not relevant. Thus, we have the following small modification of the proposal: Updated Proposal 3.1.5-1 For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, LMF shall be able to destinguish whether the timing information is obtained by legacy method or by Rel-19 method AI/ML. FFS: how to ensure that LMF can distinguish |
| Tejas | We are okay with the proposal. However, we have a polite query for the fellow members: is there a common understanding regarding the definition of timing information obtained for R19 AI-ML? Specifically, does it represent the timing information for the direct virtual path, unlike the legacy method, which provides information about the first arrival path? If not, is there a reason this has not been discussed? |

## Timing information for model output

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 13: For Case 3a, to distinguish the reported timing information from the legacy measurement-based timing information, consider the following alternatives - Alt.1: Reuse the timing quality indicator, where reported timing information of Case 3a can be represented with high timing quality and NLOS. - Alt.2: Introduce a Rel-19 indicator to tell that the report is subject to measured timing information or non-measured timing information.

• Samsung (R1-2409582) Proposal 17. In attached to the model output, the time stamp and quality information is supported.

- Tejas Networks Limited (R1-2409750) Proposal-12: We propose that the timing of arrival of the LoS path—whether real or virtually inferred by the AI-ML model—should be reported by the receiver (UE/TRP).

- Nokia (R1-2409986) Proposal 41: The decision of enabling Case 3a by a new method and new reporting framework is up to RAN3. So, RAN1 should wait for potential queries from RAN3 before to discuss on this topic. Proposal 51: The AI/ML multi-RTT positioning method may be built on top of legacy multi-RTT positioning by including at least new assistance data sent by the LMF to the UE/gNB to coordinate the deployment and usage of AI/ML RTT functionalities.

- OPPO (R1-2410102) Proposal 15: For the model inference for NG-RAN node assisted positioning with gNB-side model (Case 3a), - No need to specify the format/contents of AI model inputs since they are up to gNB/TRP implementation and transparent from the perspective of air interface - gNB can report the measurement results that are based on AI model output via existing NRPPa signaling - Introduce new information to indicate the reported results are generated by legacy method or AI-based method

## Other

### Companies’ views

In the following, selected inputs from companies’ contributions are provided.

- ZTE Corporation, Pengcheng Laboratory (R1-2409480) - Proposal 16: For reporting model output of AI/ML assisted positioning, an indicator identifying whether the measurement is based on AI/ML model is not needed. Proposal 17: The model output of AI/ML assisted positioning can be angle information. • Other intermediate features in addition to the output listed in TR 38.843 are not precluded.

• CMCC (R1-2409500) Proposal 9: For AI/ML based positioning, whether the reported measurement is AI based could have an indication.

• Samsung (R1-2409582) Proposal 18. RAN1 supports TRP to signal the model output to LMF in case 3a.

- Intel Corporation (R1-2409742) Proposal 14: For AI/ML-assisted positioning (Cases 2a and 3a), in addition to timing information and time stamp and quality information, consider support of reporting of the following from UE and gNB to LMF respectively: - UE to LMF (Case 2a): RSRP/RSRPP for first and additional paths in addition to timing information (associated path timings, e.g., DL RSTD, UE Rx-Tx time difference), and optionally, LOS/NLOS indication, time - gNB to LMF (Case 3a): AoA and/or RSRP/RSRPP for first and additional paths in addition to timing information (associated path timings, e.g., UL RTOA, gNB Rx-Tx time difference), and optionally, LOS/NLOS indication.

• Sharp (R1-2410360) Proposal 6: For AI/ML assisted positioning Case 3a, indicator that provides whether the timing information is made by AI/ML is reported.

- Apple (R1-2409788) Proposal 9: For Rel-19 Case 3a, there is a scenario where only LOS/NLOS is reported based on AI/ML. In this case, LOS/NLOS may be reported by itself a “Made with AI” indicator may be also reported to indicate that this is a new report. Proposal 12: Case Specific Model Inference Summary Case Specific Model Inference Inference Location; New Measurement Input; Model Output Report; Assistance Signaling. Case 1; At UE; RS configuration to UE. Case 3a; At gNB; Report TOA, angle/phase to LMF, LOS/NLOS indicator, Time stamp, AI/ML indicator; RS configuration to TRP. Case 3b; At LMF; Report CIR/PDP/DP to LMF; Measurement required to TRP.

# Training data collection

This section discusses issues for the LCM stage of model training.

## Content of training data

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 19: For time stamp of channel measurement, there is no need to mandate UTC time to be included in the measurement report.

- Apple (R1-2409788) Proposal 15: The “Measurement time” (NRPPa) and UTC-time (LPP) are mandatorily included together with SFN time to ensure uniqueness of the time stamp.

- NEC (R1-2409852) Proposal 22: Use "UTC time + SFN + slot index" as a baseline, with details FFS Proposal 23: Consider following two options as baseline to design the time stamp for channel measurement in case 3b: - Option 1: use "UTC time + SFN + slot index" as baseline - Option 2: use "Measurement time" only as baseline

- Nokia (R1-2409986) Proposal 30: For reporting of UTC time with SFN time info as time stamp of channel measurements and label is considered in RAN1 as configurable.

• ETRI (R1-2410258 Proposal 13: For the quality indicator of timing information (label), the IE NR-TimingQuality-r16 from TS37.355 can be reused. Proposal 14: For the time stamp of measurement and label, the IE, the following IEs can be reused: • NR-TimeStamp-r16 from TS37.355, with the addition of UTCTime • TimeStamp in TS38.455

- Huawei, HiSilicon (R1-2409396) Proposal 13: For Case 3a, to distinguish the reported timing information from the legacy measurement-based timing information, consider the following alternatives - Alt.1: Reuse the timing quality indicator, where reported timing information of Case 3a can be represented with high timing quality and NLOS. - Alt.2: Introduce a Rel-19 indicator to tell that the report is subject to measured timing information or non-measured timing information. Proposal 15: For indicating the quality of measurement for training (if needed), consider indicating the quality criterion to the measurement entity for filtering the measurement results to be reported.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 7: There’s no need to introduce power quality for channel measurement. Proposal 8: For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A is associated with the whole channel measurement. Proposal 21: For AI/ML positioning, different data collection requirements can be configured by the data collection node, where the configuration includes at least: • Quality requirement of the channel measurement, report periodicity. Note: The data collection node can be different among different use cases.

• CMCC (R1-2409500) Proposal 10: For AI/ML assisted positioning, existing IE “Timing Measurement Quality” can be reinterpreted for reporting performance metric for gNB side model.

- Baicells (R1-2410414) Proposal 7. In NRPPa, the Timing Measurement Quality can be associated with the timing information of Part A for each TRP, which can later be transferred to LMF. In this way, the normative workload for the quality indicator can be reduced. Proposal 8. For Case 3a and 3b inference, support reusing Existing IE “Timing Measurement Quality” to align the measurement for both training and inference with traditional positioning methods. Proposal 9. There is no strong need for power quality indicator considering it’s only for PDP and has not been well evaluated in the past.

• Google (R1-2410150) Proposal 2: Support the L1-SINR as the quality indicator of channel measurement.

- Tejas Networks Limited (R1-2409750) Proposal-14: A single indicator representing the quality of the complete timing information, such as the time of arrival for Case-3b or sample time instances for Case-1. Proposal-15: For reporting the ToA quality and NLoS indication, using both hard values (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with 1 indicating the highest quality should be considered.

- Apple (R1-2409788) Proposal 13: For part A, the quality indicator of the channel may be mapped to a timing quality, a power quality and a phase quality depending on the type of input

- NEC (R1-2409852) Proposal 16: Introduce a quality indicator for power information in channel measurement for generating model input. Proposal 17: Support using dedicated signaling to indicate the quality of timing, power, and phase information respectively, when gNB reports its quality of channel measurement for case 3a.

• CATT, CICTCI (R1-2409926) Proposal 1: For training data collection of AI/ML based positioning, timing information of a channel measurement is only associated with one quality indicator. Proposal 20: For case 3b, a whole quality indicator of the channel measurement is supported and there is no separate quality indicator for timing information, power information or phase information.

- Nokia (R1-2409986) Proposal 38: RAN1 to study the definition of a specific quality indicator for Sample-based representation. FFS: the signalization involved on the reporting of the quality indicator.

• ETRI (R1-2410258 Proposal 10: For DP channel measurement, the IE NR-TimingQuality-r16 from TS37.355 can be reused as a quality indicator. Proposal 11: For PDP channel measurement, L1-SINR can serve as a quality indicator.

- Huawei, HiSilicon (R1-2409396) Proposal 16: For direct positioning, if needed, the UE location quality used in legacy reporting, i.e., LocationUncertainty, can be used for the quality indication of the label for training. There is no need to discuss other options. Proposal 17: For Case 3a, the label of the LOS/NLOS information generated from LMF can be in forms of the expected likelihood of a LOS propagation path from a TRP to the target device. Proposal 18: For Case 3a, the label of the timing information generated from LMF can be in forms of the ToF/propagation delay between the TRP and the PRU. - The provision of this label may not cause additional user privacy issue on top of legacy positioning.

- ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 18: For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented reusing an existing IE: - If the label refers to the location information, horizontalUncertainty is reused. - If the label refers to the timing information, NR-TimingQuality is reused. - FFS: other quality indicators if other intermediate features are supported.

• CMCC (R1-2409500) Proposal 8: For UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods, the reliability or the positioning accuracy should also be reported.

- ITL (R1-2410588) Proposal 3: For training data generation of AI/ML based positioning Case 1/2a/2b/3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF

- Baicells (R1-2410414) Proposal 10. For quality indicator of Case 1 label, support Option 1, reusing an existing IE.

- Ruijie Networks Co. Ltd (R1-2409841) Proposal 3: For training data collection of AI/ML based positioning Cases 1, 3a and 3b, from RAN1 perspective, when label reporting is applicable, it is suggested that RAN1 should support Option 1: - Option 1: reuse an existing IE. - For existing IE, it is suggested to reuse LocationUncertainty-r16 in TS 37.355.

- CEWiT (R1-2410571) Proposal 9: Support reporting UTC time to LMF everytime SFN completes its cycle. Proposal 12: For reporting measurement power quality, SNR relative to a predefined threshold to be specified.

- NVIDIA (R1-2409781) Proposal 4: For AI/ML based positioning, study the definition of the quality indicator of channel measurement and the definition of the quality indicator of label to introduce specification support.

- Fraunhofer IIS, Fraunhofer HHI (R1-2410205) Proposal 6: Support opportunistic/event-based training data collection for follow-up model improvement/adaptation. Data collection can be triggered for example when: - A specific input pattern (e.g., a specific range of SINR values), for which the model does not perform as expected, is detected. - The UE or a known PRU is in a sub-area that has become challenging for the model. - High quality GTLs can be generated. - A monitoring event indicating reduced model performance (and possibly switching to a different model or to fallback) has been reported.

- Tejas Networks Limited (R1-2409750) Proposal-16: For reporting the quality indication of location estimate, the legacy IE such as LocationUncertainty-r16 in 37.355 should be reused.

- Apple (R1-2409788) Proposal 14: For part B, the quality of the ground truth label (if measurement is the LOS/NLOS) may be set as a value from 0 to 1. - In the case that the ground truth is a timing or position value, RAN1 should use the legacy method e.g. an updated NR-TimingQuality IE. For LOS/NLOS, a new quality may be defined e.g. ranging from 0 to 1. Proposal 17: For model training, the following elements shall be specified as part of the data collection procedure: - Part A - Channel Measurement (corresponding to model input), - Quality indicator (for and/or associated with measurement at least for model training) - Time stamp (for and/or associated with measurement) - Part B - Ground truth label, - Quality indicator (for and/or associated with ground truth label) - Time stamp (for and/or associated with ground truth label) - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.)

- NEC (R1-2409852) Proposal 24: Further study how to match a measurement and a label as a valid instance of training data or monitoring data, if the measurements and the labels are generated by different entities. Proposal 10: Endorse the integration of mixed datasets from diverse drop/clutter parameters, network synchronization errors, or deployment scenarios to train a model with robust generalization capabilities, or for fine-tuning the model to achieve higher accuracy in the target scenario in Rel-19. Proposal 18: Support gNB reports the quality of LOS/NLOS indicator, which is derived from the deployed AI/ML model for case 3a. - FL note: this assume LOS is a model output, with associated ground truth Proposal 19: A quality indicator should be defined for a data sample, and it’s determined based on the quality indicator(s), if available, of associated measurement and ground truth label. Proposal 20: Data generation entity can initially report the data fulfilling the quality indicator threshold and then reports supplemental data in case that previously reported quality data is not adequate.

• CATT, CICTCI (R1-2409926) Proposal 2: For training data collection of case 1/3a/3b, at least an existing IE can be used to represent the quality indicator of label. - FFS: whether to support a new IE on UE location quality provided by LMF if the ground truth label and quality indicator is generated by LMF side. Proposal 5: For case 2b and case 3b, when LMF side collects training data, LMF side can use a quality indicator condition or criteria to indicate the required quality of the collected data. Proposal 6: A quality threshold can be provided from LMF to channel measurement/label generation entity: - If provided, channel measurements/labels reported by the generation entity are expected to have a quality better than the quality threshold. - If not provided, channel measurement/label generation entity provides all channel measurement/label generation entity to the LMF, regardless the quality. Proposal 7: For case 3b, the measuring time difference between Part A and Part B should be restricted to ensure the validity of a training data sample. Proposal 8: For training data collection of AI/ML based positioning, the time stamp can be defined as follows: - For part A, the time stamp of channel measurement could be the time instance at which the channel measurement is performed. - For part B, the time stamp of ground truth label can be: - The time for which the location estimate is valid. - The time when the associated SRS-pos is sent. - The time when the PRS is measured. Proposal 9: For training data collection of case 1 and case 2b, further study how to match the measurements from multiple TRPs with one label by related time stamps.

- Nokia (R1-2409986) Proposal 25: The quality of content of the dataset used for training, RAN1 considers the sample’s density (#samples/m2) and the distribution similarity between the target dataset and the uniform distribution. Proposal 26: For ground truth data collection (Part B), to assess the quality of ground truth, LMF may assist UE for obtaining label consistency score/quality of the positioning estimates from one or multiple positioning sources. Proposal 27: For data collection of high-quality ground truth, LMF assists UEs on linking the PRU location (absolute information) with the relative information generated by sensors (e.g. accelerometers, gyros, magnetometers) deployed in the UE. Proposal 28: For training data collection of AI/ML based positioning, quality indicator of label is represented by a value between a predefined range (e.g. between 0 and 1 with 1 indicating the highest quality). FFS the predefined range.

- OPPO (R1-2410102) Proposal 11: For AI-based positioning (including Case 1, Case 2a, Case 2b, Case 3a, Case 3b), Rel-19 is NOT to specify any mechanism to deliver the collected data from the entity that obtains the training data to the training entity when the training entity is not the same entity to obtain label and/or other training data.

• ETRI (R1-2410258 Proposal 12: For the quality indicator of UE location (label), the IE LocationUncertainiy-r16 from TS37.355 can be reused. Proposal 15: For training data collection, an additional tag is required for both Part A and Part B to ensure efficient dataset classification. • FFS: What specific information should be included in the tag.

- Qualcomm Incorporated (R1-2410467) Proposal 13: In AI/ML positioning data collection Case1, for Part B time stamp of label, when generated by LMF, consider IE UTC-time-r15 (i.e., UTCTime and utc-time-ms-r15) and IE NR-TimeStamp (TS 37.355). Proposal 14: In AI/ML positioning data collection Case3a, for Part B time stamp of label, when generated by LMF, consider Time Stamp (TS 38.455 Section 9.2.42). Proposal 15: In AI/ML positioning data collection Case3b, for Part A time stamp of channel measurement, consider Time Stamp (TS 38.455 Section 9.2.42). Proposal 16: In AI/ML positioning data collection Case3b, for Part B time stamp of label, if generated by device, consider IE UTC-time-r15 (i.e., UTCTime and utc-time-ms-r15) or IE NR-TimeStamp (TS 37.355), subject to device choice and capability. Proposal 17: In AI/ML positioning data collection Case1, for Part B label and quality indicator of direct AI/ML positioning, when generated by LMF, support location coordinate types and uncertainty of IE LocationCoordinateTypes (TS 37.355) that can be requested by device. Proposal 18: In AI/ML positioning data collection Case3b, for Part B label and quality indicator of direct AI/ML positioning, if generated by UE, consider location coordinate types and uncertainty of IE LocationCoordinateTypes (TS 37.355) that are subject to UE capability. Proposal 19: In AI/ML positioning data collection Case3a, for Part B label quality indicator of AI/ML assisted positioning with timing information, when generated by LMF, support IE timingQuality (TS 37.355). Proposal 20: In AI/ML positioning data collection Case3a, for Part B label quality indicator of AI/ML assisted positioning with LOS/NLOS indication, when generated by LMF, support IE LOS/NLOS Indicator Soft (TS 38.455).

- NTT DOCOMO, INC. (R1-2410377) Proposal 1: For data collection, for the time stamp of the channel measurement and label, existing IEs are reused. - SFN level report, i.e., NR-TimeStamp is applied as baseline. - Only when SFN level reporting is not sufficient, UTC time is applied as time stamp instead of SFN level report. - For case 1, UTC-time is optionally reported. - For case 3b, Measurement time is optionally reported.

- Lenovo (R1-2410019) Proposal 15: The content of ground truth label and related data may at least comprise of: - For Cases 1, 2b: - PRU UE location information, PRU UE location uncertainty, PRU UE location timestamp. - For Case 2a: - PRU UE LOS/NLOS indicator associated to the positioning measurement, timestamp associated to LOS/NLOS measurement. - For Case 3a: - PRU UE LOS/NLOS indicator associated to the positioning measurement, PRU UE location of SRS transmission, timestamp associated to LOS/NLOS measurement and PRU UE SRS Tx location, PRU UE location uncertainty - For Case 3b: - TRP Location and PRU UE location of SRS transmission, timestamp associated PRU UE SRS Tx location, PRU UE location uncertainty. - NOTE1: Determination or transfer of the ground truth label and related data may be up to other WGs. - NOTE2: Discussion on Cases 2a and 2b can be revisited in RAN1#120. Proposal 16: For Case 1, the label quality indicator reporting can re-use the existing Location Uncertainty IEs under the LocationCoordinates IE in TS 37.355 to represent location (label) quality, which are already well captured using the Universal GAD shape framework defined in TS 23.032.

- InterDigital, Inc. (R1-2409845) Proposal 11: In Case 1, the LMF provides a generated ground truth label quality indicator associated with location information to the UE or PRU Proposal 12: In Case 1, the LMF is the only entity that can generate the ground truth label quality indicator associated with location information Proposal 13: In Case 1, support the target UE to request to the LMF a specific level (e.g., above a threshold) of a ground truth label quality for LMF measurement forwarding Proposal 14: For Case 1, support both hard (1 or 0) and soft indicator (0, 0.1, 0.2, …, 1.0) for the ground truth label quality indicator which is generated by the LMF and sent to the training entity Proposal 15: For Case 1, a soft or hard ground truth label quality indicator x=1 is associated with PRU location Proposal 16: For Case 1, a soft ground truth label quality indicator 0<x<1 is associated with UE location estimate Proposal 27: For Case 1, include a timestamp in the measurement forwarded by the LMF in absolute time to indicate when the measurement was made Proposal 28: For Case 1, the timestamp with absolute time should be included per measurement in the measurement forwarded by the LMF

- vivo (R1-2409669) Proposal 4: For the time stamp, the existing mechanism can be reused for data collection. Proposal 5: Reuse the existing IE, i.e., the combination of confidence and uncertainty, as the quality indicator of label for AI/ML based positioning. Proposal 6: Reuse the existing IEs, i.e., NR-TimingQuality in 37.355 and “Timing Measurement Quality” in 38.455, as the quality indicator of channel measurement. Proposal 7: A channel measurement as a whole can be associated with one quality indicator.

- Ericsson (R1-2409443) Proposal 38 For training data collection of AI/ML based positioning, for both Part A and Part B, time stamp is mandatorily provided by the training data generation entity. Proposal 39 For training data collection of AI/ML based positioning, the time stamp should allow unambiguous mapping between Part A and Part B. • For channel measurement generated by TRP/gNB, at least when measurement periodicity exceeds the SFN/slot wrap around time (10.24 sec), field “Measurement time” is included in the time stamp. • For channel measurement and label generated by PRU or non-PRU UE, UTC Time (e.g., for utc-time-r16) is included in the time stamp. • Exactly how to specify time stamp for Part A and Part B is up to RAN2/RAN3. Proposal 42 For training data collection in all cases, the quality of power measurements is provided for PDP in Part A in units of dB. The candidate resolution includes 0.1 dB and 1 dB. Proposal 44 For training data collection Part B, support only ground truth labels in the format of UE location coordinates (x, y, z). Proposal 45 From RAN1 perspective, the same label (UE location) and label quality (UE location uncertainty) are collected, irrespective of the type of model architecture (e.g., assisted or direct positioning) that may be selected by the model training entity. Proposal 46 For training data collection, the label quality for Part B reuse the existing uncertainty IEs for indicating UE location error, e.g., horizontalUncertainty-r16 and verticalUncertainty-r16.

### 1st round discussion

Regarding time stamp for training data collection, the agreement below was made in RAN1#118 for Case 3b.

Agreement (RAN1#118) For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

The same can be applied to Case 3a, where the channel measurement is also generated by TRP/gNB. Also training data collection is not mentioned below, so that it is more generic.

Proposal 4.1.2-1

For AI/ML based positioning Case 3a, regarding the time stamp of channel measurement generated by TRP/gNB,

- Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Baicells | Support in principle. As RAN3 agrees that for Case 3a, “The AI/ML model (inference and training functions) is at the gNB”, there seems to be no spec impact. |
| CEWiT | Fine with the proposal |
| Lenovo | Support |
| Nokia | Ok in reusing the legacy reporting for Time-stamp. However, it is good to clarify that details of generating this time stamp are not the same because the Inference output generation may be different than the channel measurement used in legacy. |
| Apple | Okay |
| DOCOMO | Support |
| ZTE | OK |
| mtk | 1, this proposal should explicitly say it is for training data collection 2, for 3a, the model input format for channel measurement is not defined. Then is it needed to define the time stamp? The channel measurement needs to be reported under 3a? |

When the channel measurement is generated by UE (either PRU or non-PRU UE), the time stamp issue was discussed but not concluded. The main contention is whether UTC time should be reported together with SFN or not. As a reference, the measurementReferenceTime-r16 is defined as below for DL-TDOA and DL-AoD, i.e., a choice between SFN and UTC time.

| NR DL-TDOA Positioning | NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, … } OPTIONAL, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |
| --- | --- |
| NR DL-AoD Positioning | NR-DL-AoD-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { sfn-time-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, measurementReferenceTime This field specifies the time for which the location estimate is valid.. |

UTC-Time-r15 ::= SEQUENCE {

utcTime-r15					UTCTime,

utcTime-ms-r15				INTEGER (0..999),

…

}

utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.

Question 4.1.2-1: For the time stamp in training data collection of Case 1, whether to support reporting UTC time together with SFN?

Companies’ views are summarized below (Please update/correct if your view is not captured correctly).

| Support (8 sources) | Qualcomm, Ericsson, CEWiT, Apple, Xiaomi, ETRI, Nokia (configurable), NTT DOCOMO (optional), NEC |
| --- | --- |
| Not support (1 sources) | vivo |

Based on the above, it is suggested to follow the majority view. The UTC time can be an optional field, e.g., present at least when SFN wraps around.

In Proposal 4.1.2-2 below, the Note was copied from proposals in previous meetings to address companies’ concern that the two time fields are separate and have different units, but the intention is that they refer to the same clock time for the channel measurement and/or label.

Proposal 4.1.2-2

For AI/ML based positioning Case 1, if channel measurement and/or label is generated by UE (PRU or non-PRU UE) and reported, both IEs are included in the time stamp: NR-TimeStamp and UTCTime in TS 37.355;

- From RAN1 perspective, UTCTime can be an optional field (e.g., present at least when SFN wraps around).

- Note: NR-TimeStamp and UTCTime are intended to refer to the same clock time when channel measurement and/or label is generated.

|  | Company |
| --- | --- |
| Support | CMCC, NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| CEwiT | Fine with the proposal |
| HW/HiSi | Not support. The SFN is sufficient. No need to make UTC mandatory. |
| DOCOMO | In existing spec, CHOICE means either NR-TimeStamp or UTCTime is reported. To follow this, UTCTime can be optionally reported only when SFN level reporting is not sufficient, and no need to report both. |
| Qualcomm | We shall first discuss reporting direction of Case1 in data collection. We never agreed that UE shall report channel measurements for Case1. This is pending RAN2 and other groups discussion. We can at least discuss labeling assistance in Case1 (e.g., label generation by LMF). Updated Proposal 4.1.2-2 For AI/ML based positioning Case 1, if channel measurement and/or label is generated by UE (PRU or non-PRU UE) LMF and reported, both IEs are included in the time stamp: NR-TimeStamp and UTC-time-r15 in TS 37.355; - From RAN1 perspective, UTC-time-r15 can be an optional field (e.g., present at least when SFN wraps around). - Note: NR-TimeStamp and UTC-time-r15 are intended to refer to the same clock time when of corresponding channel measurement and/or label is generated. |

Regarding the quality indicator of label for Case 1/2b/3b, variants of the following proposal was discussed in RAN1#118 and RAN1#118bis, i.e., study and decide between Option 1 and 2.

Proposal 4.1.5-1A (RAN1#118bis, R1-2409286) For training data collection of AI/ML based positioning Cases 1, 3a, 3b, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented using one of the following options. RAN1 study and select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, proponent companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality. Use hard (0 or 1) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicating the highest quality. - Proponent companies provide information on how to determine the value, entity to generate the label quality, and the applicable signaling.

In contributions to RAN1#119, companies followed up on this topic and expressed their view. Based on the contributions, companies’ view are captured in the table below (Please update/correct if your view is not captured correctly).

Question 4.1.2-2: For Part B of training data collection, when the label is UE location, whether to support Option 1 or Option 2 for quality indicator of label?

Companies views are summarized below (Please update/correct if your view is not captured correctly).

| Option | Supported by: |
| --- | --- |
| Option 1 (existing IE) (16 Sources) | Qualcomm, Huawei/HiSilicon, Tejas, Ericsson, ZTE/Pengcheng Laboratory, vivo, Lenovo, Ruijie Networks, Apple, CATT/CICTCI, ETRI, Baicells , NTT DOCOMO |
| Option 2 (new IE, 0…1) (2 Sources) | InterDigital, Nokia |

For reference, companies’ view in RAN1#118 is shown below (see R1-2409286).

| Option | Supported by: |
| --- | --- |
| Option 1 (existing IE) 10 Sources | Qualcomm, Huawei/HiSilicon, Tejas, Ericsson, ZTE/Pengcheng Laboratory, vivo, Xiaomi, Lenovo |
| Option 2 (new IE, 0…1) 3 Sources | InterDigital, Apple, Nokia |

Also, regarding the existing IE for the label quality when the label is UE location (Option 1), majority companies propose to reuse IE LocationUncertainty-r16 described in TS37.355.

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r16				INTEGER (0..255),

horizontalConfidence-r16				INTEGER (0..100),

verticalUncertainty-r16					INTEGER (0..255),

verticalConfidence-r16					INTEGER (0..100)

}

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

Proposal 4.1.2-3

For AI/ML based positioning, from RAN1 perspective, when label is generated by UE (PRU or non-PRU UE) and reported, quality indicator of label is provided by reusing an existing IE.

- From RAN1 perspective, the existing IE can be LocationUncertainty-r16 as described in TS37.355.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital, Nokia |

| Company | Comments |
| --- | --- |
| CMCC | Whether this IE is optional or mandantary? Reusing an existing IE for quality indicator of label is fine. |
| CEWiT | Support |
| InterDigital | The options mentioned above have not been agreed yet. We propose to discuss each option technically. We suggest to start with the following proposal that was proposed in the last meeting. Modified Proposal 4.1.2-3 For training data collection of AI/ML based positioning Cases 1, 3a, from RAN1 perspective, quality indicator of label (when the label is reported) is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality provided by LMF to the UE (Case 1) or gNB (Case 3a). Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. FFS: how to determine the value. |
| DOCOMO | Reusing existing IE for quality of label is fine. Added DOCOMO in the above table. |
| ZTE | From our perspective, the IE LocationUncertation can only be used when the label refers to location information. Quality indicator for assisted positioning could be the quality of timing measurement, which should be discussed further. |
| Qualcomm | LocationUncertainty-r16 is only one option. Why excluding other options (e.g., uncertaintySemiMajor, uncertaintySemiMinor, confidence, uncertaintyAltitude, uncertaintyRadius, etc.) The quality indicator can depend on the location coordinate type (i.e., geometrical shape used to indicate location information). See below. We only need to refer to general IE describing the location types. RAN2 can handle the rest. Updated Proposal 4.1.2-3 For AI/ML based positioning, from RAN1 perspective, when label is generated by LMF or UE (PRU or non-PRU UE) and if reported, label and quality indicator of label is provided by reusing an existing IE. From RAN1 perspective, the existing IE can be one from LocationCoordinateTypes LocationUncertainty-r16 as described in TS37.355, subject to device capability. [TS 37.355] #### – LocationCoordinateTypes The IE LocationCoordinateTypes defines a list of possible geographic shapes as defined in TS 23.032 [15]. -- ASN1START LocationCoordinateTypes ::= SEQUENCE { ellipsoidPoint BOOLEAN, ellipsoidPointWithUncertaintyCircle BOOLEAN, ellipsoidPointWithUncertaintyEllipse BOOLEAN, polygon BOOLEAN, ellipsoidPointWithAltitude BOOLEAN, ellipsoidPointWithAltitudeAndUncertaintyEllipsoid BOOLEAN, ellipsoidArc BOOLEAN, ..., [[ highAccuracyEllipsoidPointWithUncertaintyEllipse-r15 BOOLEAN OPTIONAL, -- Need ON highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15 BOOLEAN OPTIONAL -- Need ON ]], [[ ha-EllipsoidPointWithScalableUncertaintyEllipse-r16 BOOLEAN OPTIONAL, -- Need ON ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16 BOOLEAN OPTIONAL -- Need ON ]], [[ local2dPointWithUncertaintyEllipse-r18 BOOLEAN OPTIONAL, -- Need ON local3dPointWithUncertaintyEllipsoid-r18 BOOLEAN OPTIONAL -- Need ON ]] } -- ASN1STOP |
| mtk | 1, The quality of label could have been within the reported location. We consider locationCoordinates-r17 2, the “location uncertainty“ seems to be a field for the IE that is the location relative to a known reference location. We may not say label is a known reference location, especially when the label is generated from a non-PRU UE |

### 2nd round discussion

Regarding the MTK comment to Proposal 4.1.2-1, it is better to be generic and not limiting to training data collection. For example, time stamp is necessary for sending channel measurement from DU to CU, if RAN3 decides to support split architecture. At the moment, RAN3 leaves this open, see this note from RAN3#125bis: “Consider aggregated gNB case first, then split architecture.” To be safe, the proposal is updated to say that this applies if the channel measurement is reported.

Proposal 4.1.3-1

For AI/ML based positioning Case 3a, regarding the time stamp of channel measurement generated by TRP/gNB, if the channel measurement needs to be reported,

- Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective.

|  | Company |
| --- | --- |
| Support | Sharp, DOCOMO |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| HW/HiSi | In Rel-19 Case 3a, it is our understanding that the gNB will report non-measured timing information or non-measured LOS, not channel measurements as written in the original proposal. Updated Proposal 4.1.3-1 For AI/ML based positioning Case 3a, regarding the time stamp of at least Rel-19 non-measured information channel measurement generated by TRP/gNB, if the channel measurement needs to be reported, Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. |
| Nokia | We have another more important topics to focus in RAN1, it is not effective to focus on a potential hypothesis that is up to RAN3. We strongly disagree and suggest to focus first on RAN1 dependent topics. In case RAN3 sends an LS, we may support them on the details. |
| mtk | Sending channel measurement from DU to CU has any RAN1 impact? It seems the DU, CU stuff is not the scope for RAN1 discussion |
| ZTE | Agree with HW/HiSi. For case 3a, it’s gNB side model, there’s no need for TRP/gNB to report channel measurement. |

For Proposal 4.1.2-2, regarding UTC-Time-r15, it is only used for displacement time stamp of sensor motion information. It seems better to follow the DL-TDOA and DL-AOD measurement reference time, which uses UTCTime (see citation of 37.355 spec text in section 4.1.2).

–	Sensor-MotionInformation

DisplacementTimeStamp-r15 ::= CHOICE {

utcTime-r15					UTC-Time-r15,

gnssTime-r15				MeasurementReferenceTime,

systemFrameNumber-r15		SFN-r15,

measurementSFN-r15			INTEGER(-8192..9214),

...

}

UTC-Time-r15 ::= SEQUENCE {

utcTime-r15					UTCTime,

utcTime-ms-r15				INTEGER (0..999),

...

}

utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.

If the proposal is limited to label, then there is no need to mention UE or LMF, since the label can only be generated by UE or LMF.

Proposal 4.1.3-2

For AI/ML based positioning Case 1, if channel measurement and/or label is reported, both IEs are included in the time stamp: NR-TimeStamp and UTCTime as described in TS 37.355;

- From RAN1 perspective, UTC-time-r15 can be an optional field (e.g., present at least when SFN wraps around).

- Note: NR-TimeStamp and UTCTime are intended to refer to the same clock time associated with the label.

|  | Company |
| --- | --- |
| Support | NEC |
| Not support |  |

| Company | Comments |
| --- | --- |
| HW/HiSi | In our view, the SFN is sufficient. No need to make UTC mandatory. The label measurement pairing in case 1 can be up to implementation, e.g. at the UE training side. Therfore this proposal is not needed or can be updated as: Updated Proposal 4.1.3-2 For AI/ML based positioning Case 1, if channel measurement and/or label is reported, both IEs are included in the time stamp: NR-TimeStamp and UTCTime as described in TS 37.355 can be optionally reported.; - From RAN1 perspective, UTC-time-r15 can be an optional field (e.g., present at least when SFN wraps around). Note: NR-TimeStamp and UTCTime are intended to refer to the same clock time associated with the label. |
| Qualcomm | It shall be clear what entity generates the label. Updated proposal based on HWHiSi version: For AI/ML based positioning Case 1, if channel measurement and/or label is reported from LMF to UE, NR-TimeStamp is mandatory and both IEs are included in the time stamp: NR-TimeStamp and UTC-Time-r15 as described in TS 37.355 can be optionally reported.; - From RAN1 perspective, UTC-time-r15 can be an optional field (e.g., present at least when SFN wraps around). Note: NR-TimeStamp and UTCTime are intended to refer to the same clock time associated with the label. |
| Nokia | Ok with Qualcomm proposal. To address Huawei concern, we suggest to include a note describing the necessity of using UTC time. Updated proposal based on QC/HWHiSi version: For AI/ML based positioning Case 1, if channel measurement and/or label is reported from LMF to UE, NR-TimeStamp is mandatory and both IEs are included in the time stamp: NR-TimeStamp and UTC-Time-r15 as described in TS 37.355 can be optionally reported.; - From RAN1 perspective, UTC-time-r15 can be an optional field (e.g., present at least when SFN wraps around). Note: NR-TimeStamp and UTCTime are intended to refer to the same clock time associated with the label. Note: the purpose of UTC Time r15 is when the SFN sequence numbering is reseted |

About label and label quality, Qualcomm and MTK comments both point to geographic shapes defined in TS 23.032. Thus we can use this description, instead of quoting exact IE. Also as Qualcomm pointed out, the description in TS 23.032 include both location estimate and its uncertainty. For example, “Ellipsoid point with uncertainty circle;” “Local 2D point with uncertainty ellipse”.

locationCoordinates-r17				LocationCoordinates				OPTIONAL,	-- Cond batch1

locationCoordinates This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [15]. NOTE 1.

Proposal 4.1.3-3

For AI/ML based positioning, from RAN1 perspective, when label is UE location and reported, label and quality indicator of label are provided by reusing existing IE.

- From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032.

|  | Company |
| --- | --- |
| Support | Hw/HiSi |
| Not support | InterDigital, Nokia |

| Company | Comments |
| --- | --- |
| Hw/HiSi | Support |
| InterDIgital | We do not support the proposal. If the quality indicator for the label is issued by the UE, it can be errorneous and we support the LMF to issue the quality indicator for the label. We propose to agree on the following first to clarify our proposal. Modified Proposal 4.1.3-3 For training data collection of AI/ML based positioning Cases 1, 3a and 3b from RAN1 perspective, quality indicator of label (when the label is reported) is represented using one of the following options. RAN1 study and decide on the option to select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality provided by LMF to the UE (Case 1) or gNB (Case 3a). Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. FFS: how to determine the value. |
| Qualcomm | Would you please add corresponding case and direction of reporting? |

### 3rd round discussion

Proposal 4.1.4-1

For AI/ML based positioning Case 1, if channel measurement and/or label is reported by LMF to UE, both IEs as described in TS 37.355 are included in the time stamp:

- NR-TimeStamp

- UTCTime

  - From RAN1 perspective, UTCTime can be an optional field

  - Note: UTCTime is provided to resolve ambiguity in time stamp. For example, UTCTime is present at least when SFN wraps around.

Note: NR-TimeStamp and UTCTime are intended to refer to the same clock time associated with the label.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia | Ok |
| HW/HiSi | Not support This proposal is not clear: - Is the label generated by LMF or PRU/UE? - If generated at PRU, shall the time-stamp correspond to the time when label is generated at PRU or when transfered from the LMF? - If PRU generate measurement and label together, there is no need for a time stamp, is this proposal for the cases when label and measurement are generated at different times? The UTC does not any value in our understanding. If label and measurement anyway are done at different times, their UTC time will be different and cannot be used for pairing. There seems no need to define some conditions to include the UTC time on top of SFN. |

Proposal 4.1.4-2

For AI/ML based positioning, when label is UE location and reported (e.g., LMF to UE over LPP), label and quality indicator of label are provided by reusing existing IE.

- From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label.

|  | Company |
| --- | --- |
| Support | HwHiSi, ZTE |
| Not support | Nokia, InterDigital |

| Company | Comments |
| --- | --- |
| Nokia | Probably one of the most sensible aspects of supervised learning ist he ground truth. For sure legacy spec. On quality indicator was not specified to assess the ground truth label quality. So, as we were explaining several times, it is required an common entity in doing such assessment to obtain the quality indicator, which for AIML positioning is the LMF. We are open to explain the necessity of this approach, which goes beyond the legacy. In summary, we suggest to FL to at least include again the another possibility and provide the chances to other companies to probe that including a new quality indicator is not beneficial. We suggest the following update in the proposal: Proposal 4.1.4-2 (Re-edited) For training data collection of AI/ML based positioning Cases 1, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented using one of the following options. RAN1 study and select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality provided by LMF. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. - Proponent companies provide information on how to determine the value, entity to generate the label quality, and the applicable signaling. |
| HW/HiSi | ok |
| ZTE | Reuse a current IE is feasible for location quality. |
| InterDigital | We have been explaining problems that can occur by reusing existing fields (e.g., error propagation by circulating the uncertainty metric reproted by PRU or UE). We support the proposal from Nokia. |
| Tejas | We support revised proposal from Nokia. |

### 4th round discussion

For the quality indicator of label when label is UE location, majority companies support to reuse an existing IE. For RAN1#119, the number of supporters for Option 2 has reduced from 3 to 2, while the number of supporters for Option 1 has increased to 16. It is not justified to continue with both Option 1 and Option 2.

Proposal 4.1.4-2

For AI/ML based positioning, when label is UE location and reported (e.g., LMF to UE over LPP), label and quality indicator of label are provided by reusing existing IE.

- From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label.

|  | Company |
| --- | --- |
| Support | New H3C, TCL, HW/HiSi |
| Not support | Nokia, InterDigital |

| Company | Comments |
| --- | --- |
| mtk | 1, the route for delivering the training data is NOT RAN1 scope. So whether it can be delivered from LMF to UE through LPP belongs to RAN2 territory 2, We may agree to reuse the existing location coordinate type in TS 23.032. and whether it is provided (it is not appropriate to use report from LMF to UE) through LPP, let RAN2 decide |
| Nokia | Probably one of the most sensible aspects of supervised learning ist he ground truth. For sure legacy spec. On quality indicator was not specified to assess the ground truth label quality. So, as we were explaining several times, it is required an common entity in doing such assessment to obtain the quality indicator, which for AIML positioning is the LMF. We are open to explain the necessity of this approach, which goes beyond the legacy. In summary, we suggest to FL to at least include again the another possibility and provide the chances to other companies to probe that including a new quality indicator is not beneficial. We suggest the following update in the proposal: Proposal 4.1.4-2 For AI/ML based positioning, when label is UE location and reported (e.g., LMF to UE over LPP), label and quality indicator of label are provided by the following options: reusing existing IE. - Reusing existing IE, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label. - Using a new IE on UE location quality supported by LMF. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. Note: Label quality indicator may be considered as an additional UE capability. |
| InterDIgital | We support the modified proposal from Nokia in the last round of discussion. Option 2 described in Nokia’s proposal in the last round is an accurate description of our proposal for the ground truth label quality indicator. We are also ok with the updated proposal from this round. |
| Tejas | We support Nokia’s proposal. We believe that indicating the quality of the label using an interval between (0, 1) is more suitable for AI-ML use cases. |

## Case 1: Assistance data and other signaling for training data collection

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

Baseline assistance data, AD enhancements, base method for case 1

- Huawei, HiSilicon (R1-2409396) Proposal 20: For training data collection of Case 1, no enhancement is needed for the provision of assistance information (regarding validity area, TRP location, PRS configuration, network synchronization error, antenna/beam, etc.) to the UE on top of legacy assistance information for DL-TDOA Proposal 21: For Case 1, there is no need to introduce a new positioning method. Legacy DL-TDOA UE-based positioning can be reused to support Case 1. - The legacy assistance data subject to DL-TDOA UE-based positioning can be indicated to UE if needed. - When reporting the UE location subject to Case 1, it can be indicated as a Rel-19 type of DL-TDOA. Proposal 22: Some unneeded Ies in the DL-TDOA assistance data can be excluded when the assistance data for Case 1, i.e., Rel-19 DL-TDOA UE-based positioning, is provided from the LMF to the UE.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 20: For option A (UE initialed DL PRS configuration), the following enhancement can be supported: - On-demand PRS request includes the suggested number of activated TRPs that transmit DL PRS.

- Samsung (R1-2409582) Proposal 6. Both option A and B can be supported in terms of DL PRS configuration for collecting training data

- Intel Corporation (R1-2409742) Proposal 3: For UE-based AI/ML positioning or UE-assisted AI/ML positioning, as part of assistance data regarding DL PRS configuration, the LMF could provide a UE with one or more configurations of measurement time windows during which the UE is expected to perform measurements towards data collection and/or report ground-truth labels. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across Ues is considered as a starting point. Proposal 4: For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide an NG-RAN node with one or more configurations of time windows to configure and transmit DL PRS with certain requested configuration(s) for data collection for model training. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across Ues is considered as a starting point. Proposal 5: For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide a UE with configuration of a measurement validity area within which the UE may perform the measurements and/or report ground-truth labels for data collection for model training. - The concept of AreaID-CellList, as described in TS 37.335, to list the NR Cell-IDs of the TRPs belonging to a particular network area within which the UE is to perform measurements and/or report ground-truth labels, can be reused. Proposal 8: For Cases 1, 2a, 2b, assistance information to generate ground-truth labels based on estimated location coordinates for a target UE are provisioned to a UE.

• Google (R1-2410150) Proposal 4: The model monitoring should not require the UE to report the model inference result to the NW. - Option A is preferred compared to Option B - The model performance monitoring should be transparent

- CEWiT (R1-2410571) Proposal 10: For all the use cases (Case 1, Case2a, Case 2b, Case 3a, Case 3b), RS configuration, related UE and gNB associations and Area validity from which training data is collected should be provided as assistance information to the data collecting entity. Proposal 11: Depending on the deployment scenario, network should provide a range/margin for quality-indicating metrics (e.g., SNR, channel estimation error, etc., for channel measurement), entity’s available storage/ entity’s capability (data collecting entity), and the type of input given to the model for any required preprocessing of the dataset as assistance. Observation 2: Use of IE AreaID-CellList for defining the validity of an area for data collection can be supported.

- NEC (R1-2409852) Proposal 21: Support to collect UE coordination information in a zone-based format where the zone size is predefined.

• CATT, CICTCI (R1-2409926) Proposal 29: For AI/ML-based positioning, both the following concepts of validity area for training data collection can be considered: - Concept 1: Assistance data or measurement validity area configured by LMF, i.e. AreaID-CellList introduced in Rel-17. - Concept 2: Validity area composed of TRP(s) corresponding to channel measurement, keeping the AI/ML model valid.

- OPPO (R1-2410102) Proposal 7: Regarding the training data collection at UE side for Case 1 and/or Case 2a - Reuse the legacy LPP signaling to configures UE with the corresponding positioning RS for UE-side data collection - The associated ID is signaled along in order to facilitate the insurance of the consistency between training and inference. - No additional signaling/triggering is needed from network to start the data collection procedure at UE side. - The format/content of collected data are up to implementation of UE and no specification is needed in 3GPP

- NTT DOCOMO, INC. (R1-2410377) Proposal 2: For data collection of 3b, the measurement report and related assistance information for AI/ML based positioning is determined by LMF, - For case 3b, for data collection of training, inference and performance monitoring, LMF initiates corresponding measurement reporting at gNB. Proposal 3: For data collection of case 1, at least following assistance information is indicated from LMF to UE for consistency between training and inference. Further study the necessity of other information. - PRS configuration - Training data validity area (e.g., AreaID-CellList), where PRS can be assumed to be consistent within a validity area Proposal 4: Regarding RS configurations of data collection for each case, - For case3a/3b, NW sends configurations to UE for SRS transmissions. - RS configurations may include/associate with an associated ID to implicitly indicate the NW side additional conditions.

- Lenovo (R1-2410019) Proposal 25: For Option A – target UE model monitoring for AI/ML positioning Case 1, support Options A-1, A-2 and A-3 in terms of receiving ground truth label information.

- vivo (R1-2409669) Proposal 8: The assistance information contained in the existing IE NR-PositionCalculationAssistance, can be reused for training data collection. Proposal 9: Suggest to firstly identify the assistance information necessary for model training and management, which should be mandatorily provided by LMF. Among the necessary assistance information, the privacy-sensitive information can be implicitly provided by an associated ID.

## Case 3a/3b: Assistance data and other signaling for training data collection

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 19: From RAN1 perspective, for Case 3b measurements, - The existing procedures can be reused in terms of SRS configuration. - These measurements can be used for multiple aspects related to case 3b, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications

- Intel Corporation (R1-2409742) Proposal 4: For UE-based AI/ML positioning or UE-assisted AI/ML positioning, the LMF could provide an NG-RAN node with one or more configurations of time windows to configure and transmit DL PRS with certain requested configuration(s) for data collection for model training. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous DL positioning measurements across Ues is considered as a starting point. Proposal 6: For NG-RAN node-assisted AI/ML positioning, the LMF could provide an NG-RAN node with one or more configurations of time windows during which the NR-RAN node is expected to schedule SRS transmissions from one or more of served Ues and perform associated measurements on the SRS resources for data collection for model training. - For the measurements on UL SRS from Ues in other cells, LMF could provide NG-RAN nodes with information for SRS scheduling in neighboring cells similar to the current UL positioning framework. - Reuse/adaptation of the Rel-18 framework for supporting simultaneous UL positioning measurements across Ues is considered as a starting point.

- Baicells (R1-2410414) Proposal 6. For Case 3a model training, support to explore the feasibility of training models at gNB side as the primary focus of the normative work. Consider discussing how labels are provided to gNB by NW entity (e.g., LMF) as starting point.

• CATT, CICTCI (R1-2409926) Proposal 3: For case 3a, the following methods for ground truth label generated by LMF are considered: - Method 1: UE/PRU provides the location related information to LMF for generating the ground truth label. - Method 2: Multiple gNBs/TRPs provide the SRS-pos measurements (e.g. UL RTOA) to LMF for estimating UE’s location coordinate and the UE’s location coordinate is used to generate the ground truth label. Proposal 4: For data collection in case 3a, consider the following options: - Option A. (gNB initiated) gNB/TRP makes a request to LMF on the preferred UL SRS-pos configuration for training data collection. LMF makes the decision on determining the UL SRS configuration for training data collection and provides the assistance data to the gNB/TRP. - Option B. (LMF initiated) LMF determines the UL SRS configuration for training data collection and provides the assistance data to the gNB/TRP.

- Nokia (R1-2409986) Proposal 42: During model inference for Case 3a, AIML model confidence is taken into account for the quality of timing information assessment. FFS how model confidence is captured in the definition of the timing quality indicator.

- OPPO (R1-2410102) Proposal 9: Regarding the training data collection for NG-RAN node assisted positioning with gNB-side model (Case 3a) - gNB do measurement based on SRS - The existing procedure/mechanism can be reused for this purpose - gNB can decide itself when/how to start the data collection procedure. - The contents of collected data are up to implementation and no specification is needed in 3GPP - LMF delivers ground-truth label(s) (or the information based on that the label can be derived) to gNB via NRPPa siganling - gNB needs to send request to LMF on what the groud-truth label (or the information based on that the label can be derived) is Proposal 10: For training data collection at NW side for NG-RAN node assisted positioning with LMF-side model (Case 3b), support the following mechanisms - gNB reports measurement results (corresponding to model input) and the associated timestamps via NRPPa protocol - The corresponding label(s) can be reported optionally or LMF generates the associated labels based on the know location of the corresponding PRU - NRPPa signaling from LMF to indicate gNB - The type of measurement results - How to report the collected data (e.g., periodic reporting, event-triggered reporting)

## Other (e.g., model training entity, deliver collected data to model training entity)

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 22: The association between part A and part B can be determined by the model training entity based on the collected measurement information and label information. - Part A can be provided by: - TRP measurement result in TS 38.455 (case 3a, case 3b). - SignalMeasurementInformation in TS 37.355 (case 1, case 2a and case 2b). - Part B can be provided by: - ProvideLocationInformation in TS 37.355 (case 2b, case 3b). - ProvideAssistanceData in TS 37.355 (case 1, case 2a). - New assistance information sent from LMF to gNB (case 3a). - Part A and Part B can be associated using: - The timestamp provided in measurement and/or location information. - UE ID associated with the measurement and/or location information.

- Samsung (R1-2409582) Proposal 7. Both truncated channel measurement and feature extracted channel measurement can be considered as candidate for the channel measurement in the collected data sample. Proposal 11. For data collection in training/finetuning, the data generation entity can be PRU/UE for case 1, TRP for case 3a, and TRP(input)/LMF(label) for case 3b. Proposal 12. For data collection in inference, the data generation entity is UE for case 1 and TRP for case 3a/3b. Proposal 13. RAN1 supports UE to notify network whether such UE is a data provider or not. Proposal 14. RAN1 supports UE to notify network that it is inapplicable to provide data sample when certain condition is not satisfied. FFS certain condition, e.g., data sample requirement related condition. Proposal 15. RAN1 supports the collected data signalled from PRU (other UE) to UE deployed with AI model. Proposal 16. RAN1 supports to specify data type and size, the data type includes the measurement results and processed measurement results.

• Intel Corporation (R1-2409742) Proposal 1: For AI/ML-based positioning, consider the entities for ground-truth label generation as presented in Table 2. Table 2. Summary of entities responsible for ground-truth labels for different positioning cases Entity; Case 1; Case 2a; Case 2b; Case 3a; Case 3b. UE with estimated/known location; ; ; ; . Positioning Reference Unit (PRU); ; ; ; . Network entity (other than LMF); *. LMF with known PRU location; ; ; ; ; . Note *: RAN1 #116bis agreement states “whether other network entities can generate label for Case 3a is out of RAN1 scope”.. Proposal 2: The necessity/applicability of information transfer (including one or more of: measurement data, ground-truth labels, inference results) and, if supported, the source-destination entities of information transfer for model input and model output depends on particular AI/ML based positioning sub-use-case as captured in Table 3. Table 3. Summary of information transfer support necessary for different positioning cases Case 1; Case 2a; Case 2b; Case 3a; Case 3b. Model input; Opt. A: No data transfer Opt. B: Data transfer including ground-truth labels: LMF  UE/UE-side server; No data transfer; Channel measurement transfer: UE  LMF; No data transfer; Channel measurement transfer: NG-RAN node LMF. Model output; NA; Model inference output transfer: UE  LMF; NA; Model inference output transfer: NG-RAN node  LMF; NA.

- Apple (R1-2409788) Proposal 16: the working assumption on entities to generate measurements and labels in RAN1 #116bis and RAN1 #117 should be approved Proposal 20: For data collection, the key specification impacts are in the transfer of the measurement data in case 2b (UE to LMF) and case 3b (gNB to LMF) for model inference and model monitoring. - This requires an update to support CIR, PDP and DP measurement data. - Additional specification is required to transfer ground truth labels, quality indicators and other necessary information from the measurement entity to the monitoring entity. Proposal 23: AI/ML model assistance data - Information that be provided to the AI/ML model at the UE/gNB/LMF before or during an ongoing AI/ML-based positioning session, to be then utilized for potential AI/ML-based positioning by the AI/ML model at a future time - The assistance information can be used to match the UE/network conditions between training data collection, inference and monitoring and may be unicast or broadcast to multiple entities. Proposal 24: For Rel-19 AI/ML positioning, assistance data be signaled as assistance information or configured as part of the procedure. Proposal 25: The information assistance data may be sent for any one of the LCM stages as - LCM procedure specific e.g. for data collection for training only, monitoring only, inference only - Signalling of assistance data may be Option A (UE initiated) and/or Option B (LMF initiated) - For use across LCM procedures e.g. to ensure consistency between model training and model inference. - The assistance data may be unicast to a single entity or broadcast to multiple entities.

- NEC (R1-2409852) Proposal 7: For case 1, confirm the following working assumptions for generating the measurement and its related data and for generating the label and its related date for training data: - For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. - For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF - Note: transfer of the label and its related data is out of RAN1 scope. Proposal 8: Confirm the following working assumptions reached in RAN1 #117 meeting for the generating the label for training data case 3b: - For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF - Note: transfer of label and its related data is out of RAN1 scope. - Note: It is assumed that user data privacy of non-PRU UE is preserved. - Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Proposal 9: Study necessary signaling exchange between UE/gNB and LMF to help UE/gNB to pair Part A and Part B as valid training data. Proposal 25: Support semi-supervised learning for AI/ML based positioning.

- Xiaomi (R1-2409878) Proposal 6: Confirm the following working assumption for Case 1 Proposal 5: The support of CIR should be deprioritized - For training data generation of AI/ML based positioning Case 1, the measurement and its related data (e.g., timestamp) are generated by PRU and/or Non-PRU UE. - For training data generation of AI/ML based positioning Case 1, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF - Note: transfer of the label and its related data is out of RAN1 scope. Proposal 11: For Case 3a, leave RAN3 to assess whether there is specification impact on the collected data format definition, information exchange between TRP and gNB and information exchange between different gNBs considering different training location

- Nokia (R1-2409986) Proposal 29: For training data collection of AI/ML based positioning, an additional data (e.g., TRP ID) may be included in Part A along with the data type to indicate if it is mandatory or optional. Proposal 31: Send an LS to RAN2 to query on the feasibility and validity of the working assumptions made for training data generation of AI/ML based positioning Case 1. Proposal 32: In data collection for ground truth generation by the target UE for Case 1, LMF may indicate UE positioning method(s) (e.g., non-NR and/or NR RAT-dependent and/or NR RAT-independent) with necessary criteria (e.g., required confidence value) for an estimation to be used as ground truth. Proposal 43: For Case 3b, No RAN1 specification impact is expected for inference.

- vivo (R1-2409669) Proposal 2: For pairing Part A and Part B as one training data sample, the following methods can be further considered: • Method 1: Paired Part A and Part B from different entities at least according to the attached time stamps of Part A and Part B. • Method 2: Reporting paired Part A and Part B if both information comes from the same entity Proposal 3: Specify the data format that includes paired Part A and Part B

- Ericsson (R1-2409443) Proposal 40 From RAN1 perspective, the existing Ies and their associated granularities can be reused for time stamp of Part A and Part B. It is acceptable to have different time stamp granularity for Part A and Part B. There is no need for RAN1 to discuss how to perform the pairing using the time stamps.

- TCL (R1-2410215) Proposal 6: For Case 3b, the assistance information for training data association should be discussed when models are trained at LMF and Part B is generated by UE. Proposal 7: Down-select the method for training data association: a. UE validates the data association by implementation and sends an availability indicator or causes of unavailability to LMF. b. LMF validates the data association with the information of immobility duration.

- CEWiT (R1-2410571) Proposal 8: To associate the measurement and its related data to collected labels and its related data, the data generating entity can record its ID (UE ID/ TRP ID/ LMF ID), RS configurations and its corresponding IDs, Area ID along with the collected measurement, labels and its corresponding time stamps.

- Sony (R1-2410217) Proposal 3: The association of part A and part B in the collected data sample (for training data collection of AI/ML based positioning) can be based on timing information (e.g., UTC time) or location information.

### 1st round discussion

Several companies (e.g., NEC, vivo, Sony, Samsung, CEWiT, TCL) proposed to discuss the pairing of Part A and Part B for training data reporting.

Related to this issue, RAN2 made the following agreements in RAN2#127.

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

Based on companies’ input, the pairing of Part A and Part B can be discussed separately, depending on whether they are generated by the same entity or different entities. Aggregating companies’ suggestions, the proposals are drafted below.

Proposal 4.4.2-1

For training data collection of AI/ML based positioning, if Part A and Part B are generated by the same entity, the paired Part A and Part B are provided in one report.

- If multiple entries of Part A and Part B are reported, the Part A entry and the corresponding Part B entry are grouped in a pair for reporting.

- The paired Part A and Part B entry share the same time stamp.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| CMCC | Fine. |
| CEWiT | Okay with the proposal. |
| InterDigital | Does this proposal apply to Case 1, Case 3a and Case 3b? Can the FL clarify in the proposal. We do not support the proposal in the current form. |
| Nokia | It requires further discussion, maybe a Case by Case analysis to get a mutual understanding on the spec. Impact on each of them. So, we suggest to delay this topic and focus on more urgent topics. |
| Apple | Okay |
| HW/HiSi | Same question as IDC. The case when Part A and Part B are generated by the same entity, seems to correspond only to the case when the entity is a PRU/UE. If necessary, the legacy procedures can be reused to enable the LMF to obtain Part A and Part B from a UE/PRU, i.e, not necessarily in a same report. From RAN1 perspective, we see no need to discuss the pairing of Part A and Part B by using the time stamps. Hence, we believe this proposal is not needed. |
| ZTE | Not sure whether this should be discussed by RAN1. In the current LPP, UE reports 1 location information in one measurement report. The pair of part A and part B can be done by LMF. The LMF can compare the timestamp of part A and part B, then pair them is they share the same timestamp. |
| Qualcomm | Is this within RAN1 scope? It looks like out of the scope. It needs also be clear whether this is intended for UE or another entity. |

Proposal 4.4.2-2

For training data collection of AI/ML based positioning, if Part A and Part B are generated by different entities, the pairing between a Part A entry and a Part B entry is supported via at least:

- A UE identifier, and

- The time stamp of Part A and the time stamp of Part B.

  - Note: Different time stamp granularity can be used for Part A and Part B. For example, “Measurement time” in 38.455 and UTCTime in 37.355 have different granularities.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | We are OK with the direction, but remind that, how to pair Part A and Part B using their time stamps is still unclear. |
| Qualcomm | Let’s wait on this until RAN2 and other groups makes progress on data collection framework. This proposal may or may not be needed depending on RAN2 progress. The proposal also shall be clear on applicable entities and cases. Many details in the proposal seem implementation. |

# Model inference

This section discusses issues for the LCM stage of model training.

## Ensure consistency between training and inference

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 23: For Case 1, the assistance data based on DL-TDOA UE-based positioning can be indicated in an explicit manner as in legacy. • No need to introduce implicit indicator to indicate change of parameters or to introduce new assistance data serving as associated ID.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 24: There is no need to introduce associated ID for AI/ML positioning.

• CMCC (R1-2409500) Proposal 15: For AI/ML based positioning, further discuss what kinds of RS configurations needs to keep consistency between training and inference.

- Samsung (R1-2409582) Proposal 27. Consistence check should be done when/before the entity deciding which model to use.

- Intel Corporation (R1-2409742) Proposal 21: To ensure consistency between model training and inference, the measurement validity area configuration that may be provided for model training is assumed to apply also for model inference.

- Intel Corporation (R1-2409742) Proposal 22: Consider information on RS configurations as a NW-side condition (explicitly provided) to ensure consistency between training and inference. Proposal 23: Consider channel/link conditions/antenna configuration and characteristics as NW-side additional conditions to ensure consistency between training and inference. - FFS details of conditions of consistency between channel/link conditions/ antenna configuration and characteristics beyond the assistance information discussed in context of AI/ML Positioning Case 1. Proposal 24: Consistency between model input formats should be taken into account to ensure consistency between training and inference. - FFS: Specification impact, depending on solutions for training data collection for UE-sided models. Proposal 25: - For AI/ML Positioning Case 1, the following information can be optionally provided by the LMF to the UE explicitly as assistance data: - Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) - LOS/NLOS indicators - Data facilitating the integrity results determination of the calculated location - TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) - Expected Angle Assistance information - PRS priority list - If not explicitly provided, then consistency regarding these aspects (between training and inference) are indicated via one or more Associated ID(s). - FFS: Details of mapping of information elements/characteristics and Associated ID(s).

- Baicells (R1-2410414) Proposal 5. For Case 1, support the transfer of UE-side model information to LMF via LPP.

- TCL (R1-2410215) Proposal 9: The assistance data Ies used for channel estimation and the assistance data Ies that will not have an impact on consistency between training and inference are explicitly indicated. The indication of the other assistance data Ies depends on the impact sensitivity for consistency between training and inference. Proposal 11: The associated ID can be adopted for consistency between training and inference for Case 1. FFS: the format of associated ID(s) for AI/ML based positioning.

- Sony (R1-2410217) Proposal 8: Define a set of parameters (e.g., part/all of DL-PRS configuration, received signal quality, etc) representing reference signal characteristics to be used for AI/ML positioning. Proposal 9: For ensuring the AI/ML model consistency between the training and inference operation, support the association of the reference signal (e.g., DL-PRS) characteristics to the trained AI/ML model and AI/ML model inference operation. Proposal 11: The assistance data information elements to be used for maintaining consistency between model training and model inference are at least the one shown in Table 1 (also provided in the attached excel file). Proposal 12: For case 1, in addition to providing AD-IE-Group1, support LMF to provide the UE with AD-IE-Group2 (NW-side additional condition).

- Fujitsu (R1-2410049) Proposal-7: The study on associated ID to indicate NW-side additional conditions involving multiple TRPs is suggested to be deprioritized in AI/ML-based positioning. Proposal-8: Existing assistance data Ies defined for UE-based DL-AoD but not for UE-based DL-TDOA can be provided to UE-based DL-TDOA with the explicit indication if it is justified necessary for ensuring the consistency related to positioning accuracy.

- Tejas Networks Limited (R1-2409750) Proposal-18: The associated ID should be computed as a function of TRP-ID or Area-Cell ID, Frequency layer ID, PRS/SRS Configurations COMB Factor, Number of RBs, PRS Resource ID, PRS Resource Set ID etc.

- Apple (R1-2409788) Proposal 28: The conditions include one or more of the following: - Validity Area Conditions: - Validity/ Geographical area information /Cell-list /scenario/ site information, TRP set (item 1, item 7) - Configuration Conditions: - Reference signal configuration(s) used to generate training data (item 3, item 12) - Data Quality Conditions - Measurement data quality range (e.g. SNR/SINR range), (new) - label data quality range (e.g. mean label positioning error) (new) - Time range when data generated (new) - Hardware Conditions - Network Synchronization Error (new) - Rx-Tx timing error (item 10) - Phase offset error (new) - o Antenna/beam pattern information (item 6, item 16)

- NEC (R1-2409852) Proposal 30: Support to introduce associated ID as an implicit way to ensure consistency between training and inference for AI/ML based positioning. Further study on: − Associated ID related to the additional conditions for one TRP/gNB separately or for multiple TRPs/gNBs jointly. − Whether disclose gNB’s additional conditions to LMF or not. Proposal 31: Support to introduce validity area to ensure the consistency between training and inference for UE side model. − The legacy AreaID-CellList specified in TS37.355 can be reused as the start point to specify the validity area. Proposal 32: For case 1, following three alternatives can be used as starting point to ensure the consistency between training and inference: − Alt.1: UE requests the expected metadata from LMF. UE can provide the expected metadata to LMF for UE side inference data collection based on the metadata for training data collection such that consistency between training and inference will be ensured. − Alt.2: LMF maintains the association between metadata and UE/model. LMF can ensure consistency between training and inference based on the maintained association. − Alt.3: UE maintains the association between metadata and model. UE firstly determine whether consistency is ensured and perform model inference accordingly.

- Xiaomi (R1-2409878) Proposal 19: To ensure consistency between training and inference, utilize explicit indication manner to indicate the existing assistance data E-based DL-TDOA and/or UE-based DL-AoD Proposal 20: Consider solutions to keep consistency regarding UE additional condition for network side model

• CATT, CICTCI (R1-2409926) Proposal 17: For channel measurement reporting in case 2b/3b, the handling of time offset need to be consistent between training and inference phase. Proposal 22: For case 1 and case 2a, when UE sends a data collection request to LMF, the data collection request contains some assistance information related to the PRS/TRP set expected by UE. Proposal 30: For case 1 and case 2a, if NW-side additional condition is identified as necessary for positioning, the following options can be considered to ensure the consistency between training and inference regarding NW-side additional conditions for inference at UE-side model. - Option 1: Based on associated ID. - FFS on what can be assumed by UE with the same associated ID across training and inference. - Option 2: Based on performance/model monitoring. Proposal 31: Different from TdoA and AoD positioning methods, AI/ML based positioning should be considered as an independent positioning method, and the assistance information for AI positioning needs to be re-considered (take the assistance information in TdoA and AoD methods as reference). Proposal 32: For AI/ML based positioning, the assistance information should at least include factors that can affect the consistency of characteristics of channel measurements. Proposal 33: For AI/ML based positioning, at least PRS configuration, validity area, beam information, and synchronize errors should be explicitly included in the assistance information. Proposal 34: For AI/ML based positioning, geographical coordinates of the TRPs should be implicitly included in the assistance information due to privacy issues, e.g., via associated ID. The granularity of an associated ID can be TRP-level or area-level.

- Nokia (R1-2409986) Proposal 18: RAN 1 to consider the following approaches to ensure consistency between training and inference regarding NW-side additional conditions: (i) consistency based on Assistance data, (ii) based on Associated ID, and (iii) consistency assisted by monitoring. Proposal 19: For UE-side models, to ensure consistency between training and inference related to NW-additional conditions, an assistance data (e.g. PRS configuration) used for data collection (training) or inference shall be associated with a global cell identity (GCI) and a network-identifier (associated ID). FFS: details of network identifier (associated ID). Proposal 20: For Case 1, RAN1 to study the possibility defining validity area and granularity requirements of this area for consistency purposes. Proposal 22: The association information of DL-PRS resources with TRP Tx TEG ID is explicitly indicated to UE as in the legacy assistance data. Proposal 23: LMF may request UE or gNB to transmit UL SRS / DL PRS or provide their measurements with specific Tx/Rx timing error. For ensuring consistency, UE may indicate to LMF timing errors it assumed for training of UE-side model. Proposal 24: To RAN1 further discuss the following options to ensure consistency between training and inference regarding NW-side additional conditions: Option 1 (UE-sided model assessment), Option 2 (UE-sided functionality assessment), and Option 3 (NW-side functionality assessment with UE-assistance).

- OPPO (R1-2410102) Proposal 3: Regarding the consistency of training and inference for AI/ML based positioning of Case 1 and Case 2a, the method(s) only based on “validity area of model training” and “validity area of model inference” is not workable for practical deployment since - It can only differentiate the AI models applicable for different areas - It cannot differentiate the AI models applicable for different times in the same areas (e.g., before and after some NW optimization operations) Proposal 4: For AI/ML based positioning of Case 1 and Case 2a, some indication (e.g., in form of associated ID) is signaled from network to ensure the consistency of AI model training and AI model inference (e.g., consistency on NW-sided conditions / additional conditions) - e.g., the ID can be a special ID for positioning configurations, and can be indicated to differentiate the associated training data - The proprietary information of network should not be disclosed. Proposal 5: For AI/ML based positioning of Case 1 and Case 2a, if the associated ID is NOT globally managed, some of the information in assistance data can be combined with the same associated ID, the same cell(s) and the same PRS configuration implicitly to ensure the consistency between training and inference (e.g., consistency on NW-sided conditions / additional conditions). - Regarding whether to signal the assistance data explicitly or implicitly, the following table captures the suggestions for each IE information. Proposal 12: For the model inference for UE-based positioning with UE-side model (Case 1), - The associated ID is signaled from network to ensure the consistency of AI model training and AI model inference - No need to specify the format/contents of AI model input since they are up to UE implementation and transparent from the perspective of air interface - UE can report the estimated location information via existing LPP signaling - Introduce new information to indicate the reported results are generated by legacy method or AI-based method - FFS: whether the current field about the uncertainty can be reused or some new field/IE should be introduced to report the associated quality/probability/confidence of AI model estimated location

- ETRI (R1-2410258 Proposal 16: For the AI/ML positioning model, ensure consistency between training and inference by adopting the associated-ID. • FFS: How to configure the associated-ID

- Qualcomm Incorporated (R1-2410467) Proposal 3: In AI/ML positioning Case1, for training and inference consistency, support both - Case1 as part of UE-based DL-TdoA, including all existing Ads of UE-based DL-TdoA - FFS: “other” indication for additional Ies not supported in existing Ads of UE-based DL-TdoA - Case1 as part of UE-based DL-AoD, including all existing Ads of UE-based DL-AoD - FFS: “other” indication for additional Ies not supported in existing Ads of UE-based DL-AoD. Proposal 4: In AI/ML positioning Case1, for training and inference consistency, deprioritize the implicit indication (e.g., associated ID) when providing existing assistance data of UE-based DL-TdoA and DL-AoD - FFS: study implicit indication for other/new assistance data.

- NTT DOCOMO, INC. (R1-2410377) Proposal 9 For AI/ML based positioning, to ensure consistency between training and inference, the existing assistance data Ies for DL-TdoA positioning method are reused as a starting point. Whether/how to indicate each IE is summarized in Table I. - At least TRP beam/antenna information and hardware conditions are additionally indicated to UE implicitly. Proposal 10 For AI/ML based positioning, to ensure consistency between training and inference, in addition to the existing Ies, associated ID or validity area can be considered to implicitly indicate other NW side conditions.

- MediaTek Korea Inc. (R1-2410531) Proposal 3a-1: The associated ID could be provided to UE at training data generation and at inference to discriminate whether there is significant change in NW side implementation Proposal 3a-2: The associated ID for positioning use case could be considered at area level. If it is at cell level, there would be a certain number of IDs sent to the UE which is not so efficient Proposal 5a-1: For use case 1 assistance data, the TRP location (nr-TRP-LocationInfo-r16) can be delivered implicitly through the associated ID Proposal 5a-2: For use case 1 assistance data, the beam response (NR-TRP-BeamAntennaInfo-r17) can be indicated explicitly or implicitly. The implicit indication is adopted when there is only a single beam in each TRP. And the explicit indication is used under the transmission of multiple beams

- InterDigital, Inc. (R1-2409845) Proposal 2: For Case 1, enhance assistance data used for the existing UE-based DL-TDOA positioning method to support the AIML-based positioning Proposal 3: For Case 1, support to combine assistance data used for UE-based DL-TDOA and UE-based DL-AoD to allow the UE to check NW side additional conditions Proposal 5: For Case 1, to ensure consistency between data collection phase and inference phase, the UE compares existing assistance information without an associated ID Proposal 6: For Case 1, in the provided PRS configurations, at least frequency layer IDs, cell IDs and TRP IDs should be the same between training and inference phase to maintain consistency Proposal 7: Tolerance for difference in assistance information (e.g., network synchronization error, difference in boresight angles) should be specified Proposal 8: Time validity for an AIML model(s) should be specified Proposal 9: For Case 3a and Case 3b, study how consistency in UE conditions can be checked between training and inference phase when the UE is configured to transmit SRS

- vivo (R1-2409669) Proposal 36: Consider adopting the following approaches to ensure the consistency between training and inference. - For beam pattern information, NW side provides indication to assist UE in maintaining the consistency between training and inference, such as “associated ID” information or explicit beam pattern information as specified in legacy UE-based AoD positioning. O UE can indicate applicable “associated ID” or beam pattern in other procedures. Proposal 37: Further study how to connect multiple TRPs or cells with associated ID for AI/ML based positioning, and connecting an associated ID with a cell/TRP list could be considered. Proposal 38: Consider adopting the following approaches to ensure the consistency between training and inference. - UE can report supported PRS bandwidth. Proposal 39: If NWs are concerned about the privacy of their implementations, the privacy-sensitive data, including the information with index 6, 7 and 16, can be implicitly indicated to support AI/ML positioning functionality Proposal 40: The existing assistance are sufficient for AI/ML based positioning Case 1, and there is no need to define new assistance data

- Ericsson (R1-2409443) Proposal 63 The model training entity records the model inference validity area as a part of metadata for the trained model. Proposal 64 When preparing for model inference, the model inference validity area is checked for compatibility with the deployment area. Proposal 65 When preparing for model inference, configurations of reference signal transmission and measurement are checked for compatibility between the trained model and the deployment environment. Proposal 66 For Case 2a/3a, support UE/gNB to send an indication to the LMF when the AI/ML model is ready for reporting measurements based on model inference. It is up to RAN2/3 to decide which procedure to use to convey the information. Proposal 67 For Case 1, to ensure the consistency of NW-side additional condition across training and inference, support associated ID for indicating NW conditions/configurations. Proposal 68 For AI/ML based positioning, the consistency between training and inference is checked for each TRP separately. Proposal 69 For a given TRP j, the UE assumes that the NW-side additional conditions with the same associated ID are consistent within the TRP j. Proposal 70 For measurements reported to the LMF in case 2b/3b, support the LMF to indicate the PRS/SRS measurement bandwidth. Proposal 71 For Case 1, the Ies categorized as “E” (i.e., explicit) are provided as assistance data from LMF to UE. Proposal 72 For Case 1, associated ID is introduced to indicate whether the Ies categorized as “I” (i.e., implicit) are consistent between training and inference. Proposal 73 For Case 1, the Ies categorized as “O” are not provided from LMF to UE, neither explicitly nor implicitly via associated ID.

### 1st round discussion

Related to network side additional condition and assistance data, the following agreement was made in RAN2#127bis:

Agreements (RAN2#127bis): 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

Thus not only for RAN1 issues, RAN2 is also waiting for RAN1 progress on assistance data Ies.

Companies’ views on the assistance data Ies are summarized in the summary excel sheet (see the ftp folder). For easy reference, the summary columns are copied below. The assistance data with much support to be sent explicitly (“E”), or not explicitly, are highlighted below.

|  | Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TdoA [1] or UE-based DL-AoD [2], as applicable | Total “E” | Total “I” | Total “O” |
| --- | --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement | 21 | 0 | 0 |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs | 14 | 1 | 4 |
| 3 | DL-PRS configuration of candidate NR TRPs | 21 | 0 | 0 |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation | 16 | 0 | 3 |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) | 19 | 0 | 0 |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB | 16 | 7 | 0 |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) | 13 | 7 | 3 |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs | 11 | 3 | 6 |
| 9 | PRS-only TP indication | 13 | 0 | 6 |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID | 11 | 2 | 7 |
| 11 | LOS/NLOS indicators | 10 | 2 | 7 |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation | 19 | 0 | 2 |
| 13 | Validity Area of the Assistance Data | 21 | 0 | 0 |
| 14 | PRU measurements together with the location information of the PRU | 13 | 1 | 7 |
| 15 | Data facilitating the integrity results determination of the calculated location | 11 | 1 | 7 |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) | 6 | 7 | 9 |
| 17 | Expected Angle Assistance information | 4 | 0 | 16 |
| 18 | PRS priority list | 5 | 0 | 15 |

Note that some companies’ views are not very clear, and can be clarified during RAN1#119. For example,

- Lenovo: the attached excel sheet (1 column provided) does not match Table 2 (two columns provided) in the word document of R1-2410019.  In Table 2 of the word document, the two columns are marked as “UE-based DL-TdoA” and “UE-based DL-AoD”. But there was no explanation which column is intended for Case 1, or both are possible. The input in attached excel sheet is used in summary sheet.

- Sony: There are several entries in Sony input left empty. It’s not clear what they are intended to be: transmitted implicitly, or not transmitted at all?

Based on the summary of companies’ view, the following proposals are provided.

Proposal 5.1.2-1

For AI/ML based positioning Case 1, at least the following assistance information are provided from LMF to UE explicitly:

|  | Information |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |

Monday offline:

Proposal 5.1.2-1A

For AI/ML based positioning Case 1, if necessary, at least the following assistance information are provided from LMF to UE explicitly:

- From RAN1 perspective, these assistance information can be provided as optional fields (as in existing specification)

Proposal 5.1.2-1B

For AI/ML based positioning Case 1, all assistance information for DL-TDOA are provided from LMF to UE, except #7.

Proposal 5.1.2-1C

For AI/ML based positioning Case 1, assistance information (#1-#15) can be provided from LMF to UE, except #7.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| OPPO | Support |
| InterDigital | Our preference is to start the discussion on whether the list of assistance data for one of the existing positioning method (e.g., DL-TDOA) can be supported or not. In addition, for Case 1, the discussion should include whether, from RAN1 perspective, the existing positioning method can be reused to support AIML based positioning. |
| Lenovo | Just to clarify that the intention of our Table in the word doc was for both Case 1 and it was merged into the one table in the excel sheet. The main reason was to further break down Case 1 into “UE-based DL-TdoA” and “UE-based DL-AoD” to highlight that some AD elements are needed from both methods and a cleaner design would be to create a new method at least capturing the AD elements in P5.1.2-1. Fine to support as a starting point. |
| HW/HiSi | Agree with IDC on „start the discussion on whether the list of assistance data for one of the existing positioning method (e.g., DL-TDOA) can be supported or not“ Adiitionally, if discussing the Ies, we suggest: We suggest the following updated main bullet for the proposal: Updated Proposal 5.1.2-1 For AI/ML based positioning Case 1, if necessary, at least the following assistance information are provided from LMF to UE explicitly: |
| DOCOMO | Support |
| ZTE | For item 4, i.e., bandwidth aggregation is not evaluated in SI. And also, if bandwidth aggregation is supported, does that mean finer timing granularity should be supported for sample-based report? |
| Fujitsu | Fine in general. |
| Qualcomm | Majority of companies strongly support assistance from 1 to 15. In addition, there is some support for 16 to 18. Let’s include all assistance items and have FFS for assistance 16 to 18. |

Proposal 5.1.2-2

For AI/ML based positioning Case 1, for ensuring consistency between training and inference, the target UE checks at least the assistance data which are provided from LMF to UE explicitly.

- The target UE signals to the LMF whether the consistency between training and inference is satisfied.

  - It is up to RAN2 to decide the details on how to convey this information.

- It is up to UE implementation to decide whether the consistency is still satisfied, when an assistance data value at inference does not equal that of training, i.e., tolerable range of the corresponding NW-side condition without violating consistency.

|  | Company |
| --- | --- |
| Support | NEC |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| InterDigital | The tolerable range should be specified. Otherwise, the LMF does not know how the UE is determining consistency by comparing assistance data. |
| Lenovo | This can implicitly achieved, if the UE requests for assistance data, then it can be assumed that the consistency is not satisfied |
| CATT, CICTCI | The last bullet is a bit confusing. For example, if the validity area is explicitly different between training and inference phase, how can a UE mark this as ‘consistent’? |
| HW/HiSi | The two bullet points seems to contradict one another. There is no need for the UE to indicate whether the consistency is satisified or not, as this is up to UE implementation. What is the intention if the UE indicates to the LMF that consistency is not satisfied? The UE can request the required assistance data via legacy procedure and can determine on its own whether consistency is valid with the received assistance data. |
| ZTE | If the table on the assistance information is agreed, we may spend more time on the consistency feedback. |
| Fujitsu | If the creteria to judge “whether the consistency between training and inference is satisfied“ is up to UE implementation, no sure why UE should make a report to LMF for it. |
| Qualcomm | Let’s first finalize assistance list. |

Proposal 5.1.2-3

For AI/ML based positioning Case 1, at least the following assistance information does not need to be transferred explicitly from LMF to UE.

- FFS: whether it needs to be sent implicitly or not.

| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |
| --- | --- |

|  | Company |
| --- | --- |
| Support | Nokia, DOCOMO |
| Not support | InterDigital, ZTE |

| Company | Comments |
| --- | --- |
| InterDigital | We explained our reasons in Proposal 5.1.2-1. |
| HW/HiSi | We believe that this assistance information is not needed (explicitly or implicitly). Consistency ensured by spatial direction information is enough. |
| ZTE | This assistance data already supported in legacy positioning, and also the TRP beam information affect the consistency, which could be explicitly provided to UE. |

Proposal 5.1.2-4

For AI/ML based positioning Case 1, at least the following assistance information are not needed from LMF to UE, neither explicitly nor implicitly:

| 17 | Expected Angle Assistance information |
| --- | --- |
| 18 | PRS priority list |

|  | Company |
| --- | --- |
| Support | Nokia, DOCOMO |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| InterDigital | We explained our reasons in Proposal 5.1.2-1. PRS priority list can be optionally provided to the UE if the LMF thinks it is needed. |
| HW/HiSi | Ok |
| Qualcomm | See our response to Proposal 5.1.2-1 |

### 2nd round discussion

For Proposal 5.1.2-1, there was a comment that this list is incomplete. Thus this proposal is updated by noting that other information is for further discussion.

Proposal 5.1.3-1

For AI/ML based positioning Case 1, at least the following assistance information are provided from LMF to UE explicitly.

- FFS: whether/how to provide the remaining information.

|  | Information |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |

|  | Company |
| --- | --- |
| Support | Nokia |
| Not support | InterDigital, Qualcomm |

| Company | Comments |
| --- | --- |
| OPPO | Fine with the proposal |
| CMCC | Fine |
| CATT | OK |
| HW/HiSi | We suggest to first discuss whether the assistance data for one of the existing positioning method (e.g., DL-TDOA) can be supported or not. But as the on-/ and offline discussion went yesterday the following compromise could be acceptable: Proposal 5.1.3-1 For AI/ML based positioning Case 1, at least the following assistance information from DL-TDOA are provided from LMF to UE explicitly. - FFS: whether/how to provide the remaining information. Information. 1; Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement. 3; DL-PRS configuration of candidate NR TRPs. 4; Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation. 5; SSB information of the TRPs (the time/frequency occupancy of SSBs). 6; Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB. 12; On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation. 13; Validity Area of the Assistance Data. |
| Sharp | OK |
| InterDigital | We have a concern on agreeing on a partial table. Starting from the proposal from HW/HiSi, we propose the following. Modified Proposal 5.1.3-1 For AI/ML based positioning Case 1, at least the following assistance information from DL-TDOA are provided from LMF to UE explicitly. FFS : which information is provided optionally FFS: whether/how to provide the remaining information |
| Qualcomm | All assistance data of UE-based DL-AoD and DL-TdoA are beneficial for Case1. UE shall be allowed to request such assistance for Case1. There is no need to start from scratch. Majority of companies support assistance #1 to 15. We don’t see this proposal helping in progressing the discussion. An updated version of InterDigital‘s proposal: For AI/ML based positioning Case 1, at least the following assistance information from UE-based DL-TDOA and DL-AoD are provided from LMF to UE explicitly. FFS : which information is provided optionally FFS: whether/how to provide the remaining information |
| ZTE | Generally fine, other IE can be further discussed. |
| DOCOMO | OK |
| Lenovo | Since we do not have any agreements related to consistency between training and inference. We can start with legacy as baseline. The revised proposal provided by InterDigital and Qualcomm is a reasonable starting point. Just added another FFS on which parameters needs to be explicitly or implicitly signalled An updated version of InterDigital‘s and QC’s proposal For AI/ML based positioning Case 1, at least the following assistance information from UE-based DL-TDOA and DL-AoD are provided from LMF to UE explicitly. FFS : which information is provided optionally FFS: which information is explicitly/implicitly signalled FFS: whether/how to provide the remaining information If we start cherry picking the Assistance Data Elements, as per the proposal, this would lead to a RAN2 understanding that a new method may be needed (which is also fine by us). |
| Tejas | We are okay with the proposal. |

Regarding Proposal 5.1.2-4, a note is added below to address InterDigital comment.

Proposal 5.1.3-2

For AI/ML based positioning Case 1, at least the following assistance information are not needed from LMF to UE, neither explicitly nor implicitly.

- Note: As in existing specification, the LMF may optionally provide them to the target UE for the purpose of supporting legacy UE-based DL-AoD method.

| 17 | Expected Angle Assistance information |
| --- | --- |
| 18 | PRS priority list |

|  | Company |
| --- | --- |
| Support | Hw/HiSi |
| Not support | Qualcomm |

| Company | Comments |
| --- | --- |
| Hw/HiSi | Support |
| Nokia | Including ecisiony DL-AoD we are indirectly indicating that Case 1 is a enhancement of legacy methods. However, RAN2 is the entity on doing such ecision. We suggest the Note promoted by HW/HiSi |
| Qualcomm | Expected angle can be helpful to select model and/or apply hybrid positioning by the device. Same applies to PRS priority list. |

### Wednesday offline discussion

Understanding of “network-side additional condition”:

Info about network side implementation details which the network side is not willing to send to UE explicitly, but such info is needed for UE-side model to ensure consistency between training and inference.

Proposal 5.1.4-1

For AI/ML based positioning Case 1, at least the following assistance information are not needed from LMF to UE, neither explicitly nor implicitly.

- Note: As in existing specification, the LMF may optionally provide them to the target UE for the purpose of supporting legacy UE-based DL-AoD method.

| 17 | Expected Angle Assistance information |
| --- | --- |
| 18 | PRS priority list |

Proposal 5.1.4-2

For AI/ML based positioning Case 1, at least the following assistance information are provided from LMF to UE implicitly.

- Associated ID is provided to indicate whether the corresponding network-side additional condition is consistent between training and inference.

- The associated ID is sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |

Huawei: no need of info under FFS

Proposal 5.1.4-3

For AI/ML based positioning Case 1, at least the following assistance information from DL-TDOA are provided from LMF to UE explicitly.

- The explicitly provided assistance information are sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

- FFS: whether/how to provide the remaining information; whether associated ID needs to be defined, and what network side additional condition contains

|  | Information |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |

- FFS: the following information

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |

MTK, Qualcomm, Apple, NEC

Proposal 5.1.4-3

For AI/ML based positioning Case 1, all assistance information from UE-based DL-TDOA and UE-based DL-AOD are provided from LMF to UE explicitly,

- The explicitly provided assistance information are sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

FFS: some info to be sent implicitly, e.g., via associated ID.

Proposal 5.1.4-3

For AI/ML based positioning Case 1, all assistance information from UE-based DL-TDOA and UE-based DL-AOD are provided from LMF to UE explicitly,

- The explicitly provided assistance information are sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

Except the following info is to be sent implicitly, e.g., via associated ID.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |

### 3rd round discussion

The following proposals seem to be the most promising ones considering all companies’ view. Two alternative proposals are provided below for further input.

Proposal 5.1.5-1 (first alternative proposal)

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA and UE-based DL-AOD may be provided from LMF to UE.

- At least the following info are provided implicitly via associated ID.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |

- Associated ID is provided to indicate whether the corresponding network-side additional condition is consistent between training and inference.

- The assistance information, including the associated ID, are sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

|  | Company |
| --- | --- |
| Support |  |
| Not support | ZTE, InterDigital |

| Company | Comments |
| --- | --- |
| Nokia | As a compromise, we are ok with the current version. However, our preference is mapped in Proposal 5.1.5-1 (Second alternative proposal) |
| Qualcomm | Positive with direction of the proposal The last bullet is crossing with RAN2 scope. Let’s delete it. A few changes: Updated Proposal 5.1.5-1 (first alternative proposal) For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA and UE-based DL-AOD may can be provided from LMF to UE. - FFS: At least the following info are can be provided implicitly via associated ID, e.g., 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). - Associated ID is provided to indicate whether the corresponding network-side additional condition is consistent between training and inference. - The assistance information, including the associated ID, are sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).” |
| HW/HiSi | #7 is not needed. The true locations are not important for fingerprinting, and they will not change after deployment. #16 is not needed. It contains relative power information between PRS resources per angle. This seems not relvant for consistency. For Case 1, consistency is achieved if a TRP transmits the PRS with the same beam angle across training and inference, which is already provided by nr-DL-PRS-BeamInfo under DL-TDOA. For the beam width/beam codebook aspects that are addressed by Entry 16, such configurations are considered to be static after the TRPs are deployed; even the reconfiguration occasionally occurs, the operator or factory owner is aware of that, and can trigger the retraining manually. |
| ZTE | Whether associated ID can be introduced requires more discussion. |
| InterDigital | We support the updated proposal from Qualcomm. Necessity of an associated ID needs t obe discussed. To address concern from HW/HiSi, we can modify the Qualcomm’s proposal as follows. Further Updated Proposal 5.1.5-1 (first alternative proposal) For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA and UE-based DL-AOD may can be provided from LMF to UE. FFS : whether some of assistance information can be indicated implicity by an associated ID |

Proposal 5.1.5-1 (second alternative proposal)

For AI/ML based positioning Case 1, at least the following assistance information from legacy UE-based DL-TDOA are provided from LMF to UE explicitly.

|  | Information |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |

At least the following info are provided implicitly via associated ID.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |

- Associated ID is provided to indicate whether the corresponding network-side additional condition is consistent between training and inference.

- The explicitly provided assistance information and the associated ID are sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

|  | Company |
| --- | --- |
| Support |  |
| Not support | Qualcomm, InterDigital |

| Company | Comments |
| --- | --- |
| Qualcomm | The explicit listing is missing many important assistance data that are beneficial for Case1. |
| HW/HiSi | The second part is not needed as we explain above. At least the following info are provided implicitly via associated ID. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). - Associated ID is provided to indicate whether the corresponding network-side additional condition is consistent between training and inference. - The explicitly provided assistance information and the associated ID are sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).” |
| ZTE | Agree with HW/HiSi, we can focus on the first part at this stage. |
| InterDigital | We don’t support this proposal. We should not discuss a subset of assistance data. We should start our discussion by assuming any of assistance information in the specifications can be provided to the UE by the LMF. Whether some can be indicated implicitly or not can be discussed further. |

### 4th round discussion

To respond to the question in RAN2 agreement below:

“2: 	Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).”

the proposal below is put together to address it, apart from the debate on the list of information. Regarding Step 5, it is explained that, since no measurement is reported from UE to LMF for Case 1, there is no need for LMF to indicate measurement format related information in Step 5.

Proposal 5.1.6-1

- For model inference of AI/ML based positioning Case 1,

- The inference configuration (e.g., DL-PRS configuration)  is contained in the assistance information, which is sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

- LMF is not expected to request measurement from the target UE, and no inference configuration is expected to be carried in Step 5 of RAN2 procedure: “Step 5: The LMF requests the inferred location information using the LPP request location information message.”

|  | Company |
| --- | --- |
| Support | TCL |
| Not support | InterDigital |

| Company | Comments |
| --- | --- |
| mtk | 1, we think network side additional condition can also be provided in step 3 |
| ZTE | It’s not clear what is “inference configuration”. From our perspective, the inference configuration may include the assistance data such as PRS configuration, and request location information, that’s the key point to determine the inference configuration is in step 3 or step 5. |
| Nokia | To provide a complete picture, we need to discuss/agree on the usability of network side additional condition enabled by Associated ID. So, our suggestion is to delay the discussion by now. |
| InterDigital | We support the first bullet. The inference PRS configuration shall be provided in Step 3. We are not sure about the intention of the second bullet. The second bullet can be removed from the proposal as the UE will report only location information as requested by the LMF. |

Proposal 5.1.6-2

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA may be provided explicitly from LMF to UE.

- At least the following info can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether the following info is provided implicitly or explicitly.

  - If provided implicitly, associated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

Note: LMF may provide #7 as in existing specification for the purpose of supporting legacy UE-based DL-TDOA.

|  | Company |
| --- | --- |
| Support |  |
| Not support | HW/HiSi, DOCOMO |

| Company | Comments |
| --- | --- |
| Nokia | It may be a compromise with UE vendors, in our view it is extremely optimistic to expect that operators will share sensitive information to enable Positioning. Several Ies may not be exposed implicitly in practice. We only suggest to add the following note to avoid a missunderstanding in RAN2 at the time to decide on the definition or not of a new method. Proposal 5.1.6-2 (Updated) For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA may be provided explicitly from LMF to UE. - At least the following info can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether the following info is provided implicitly or explicitly. - If provided implicitly, associated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). Note: LMF may provide #7 as in existing specification for the purpose of supporting legacy UE-based DL-TDOA. Note: As the final decision of a new method is up to RAN2, this agreement does not mean that Case 1 is based or not in DL-TDOA. |
| HW/HiSi | We should follow legacy, where the IEs may be provided optionally. Our view on #7 is that it is not needed. The true locations are not important for fingerprinting, but rather the identifiers of the TRPs which are provided with 1. The location of the TRPs are not expected to change after deployment, and even if they do, an implicit indication is useless, as the model at the UE would not have been trained with measurements from the TRPs at the new locations. In case the location of the TRPs change, retraining would be needed anyway. We sugget the following updates to the proposal: Updated Proposal 5.1.6-2 For AI/ML based positioning Case 1, the assistance information from legacy UE-based DL-TDOA may optionally be provided explicitly from LMF to UE. Note: LMF may provide #7 as in existing specification for the purpose of |
| InterDigital | We are not sure why UE capability is mentioned in the propsoal. For progress, we can agree on the first bullet. We can further study whether an assoicated ID is needed and how it can be used if supported. We propose the following. Modified Proposal 5.1.6-2 For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA may be provided explicitly from LMF to UE. |
| DOCOMO | We don’t support this proposal. From the operater’s perspective, operators cannot share such a sensitive information. In addition, for AI/ML positionig with fingerprinting method, the actual TRP location is not necessary. For consistency checking, we can consider reporting it implicitly. The original wording is fine for us. Proposal 5.1.6-2 (Updated) For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA may be provided explicitly from LMF to UE. - At least the following info are provided implicitly via associated ID. - At least the following info can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether the following info is provided implicitly or explicitly. - If provided implicitly, aAssociated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). Note: LMF may provide #7 as in existing specification for the purpose of supporting legacy UE-based DL-TDOA. |

Proposal 5.1.5-3

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-AOD may be provided explicitly from LMF to UE.

- At least the following info can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether the following info is provided implicitly or explicitly.

  - If provided implicitly, associated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference.

| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |
| --- | --- |

Note: LMF may provide #16 as in existing specification for the purpose of supporting legacy UE-based DL-AOD.

|  | Company |
| --- | --- |
| Support |  |
| Not support | HW/HiSi |

| Company | Comments |
| --- | --- |
| OPPO | If DL-TDOA and DL-AOD are discussed separately, assistance information 7 can also be provided implicitly by LMF using associated ID. |
| TCL | We don’t need to discuss DL-TDOA and DL-AOD separately. We should focus on whether the assistance information is explicitly or implicitly indicated. |
| Nokia | Similar comment than Proposal 5.1.6-2, we are including an important note in the proposal. Proposal 5.1.5-3 (Updated) For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-AOD may be provided explicitly from LMF to UE. - At least the following info can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether the following info is provided implicitly or explicitly. - If provided implicitly, associated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference. 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). Note: LMF may provide #16 as in existing specification for the purpose of supporting legacy UE-based DL-AOD. Note: As the final decision of a new method is up to RAN2, this agreement does not mean that Case 1 is based or not in DL-AOD. |
| HW/HiSi | We should start with assistance data from DL-TDOA. From the evaluations, the measurement type of timing and power information are widely adopted rather than the departing angle of beams, so DL-TDOA is more applicable instead than DL-AOD. And the ongoing discussion is already complicated enough. We do not need to diversify it further. In our view, the Ies that only are available for DL-AOD are not needed for consistency. Entry 16 includes relative power information between PRS resources per angle. It is not clear why this is necessary in Case 1 for consistency. For Case 1, consistency is achieved if a TRP transmits the PRS with the same beam angle across training and inference, which is already provided by nr-DL-PRS-BeamInfo under DL-TDOA. For the beam width/beam codebook aspects that are addressed by Entry 16, it is our understanding that such configurations are mostly static after the TRPs are deployed; even the reconfiguration occasionally occurs, the operator or factory owner is aware of that, and can trigger the retraining manually. |
| InterDigital | Suppor the main bullet. We do not support the sub-bullet. If it helps with convergence of the discussion, we can start from Proposal 5.1.6-2. |

### Thursday offline discussion

Proposal 5.1.7-1A

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA may be provided explicitly from LMF to UE.

- [Alternative suggestion 1] At least the following info #7 is provided implicitly via associated ID.

  - Associated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference.

- [Alternative suggestion 2] At least the following info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether the following info is provided implicitly or explicitly, and the UE can request the info being provided explicitly or implicitly.

  - If provided implicitly, associated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference.

- [Alternative suggestion 3] At least the following info #7 may not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume #7 is consistent between training and inference.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

Proposal 5.1.7-1B

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study and choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

FFS: assistance information from legacy UE-based DL-AOD

## Model inference operation (input, output, configuration)

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Tejas Networks Limited (R1-2409750) Proposal-19: We propose that, the receiver should report a new LoS/NloS indication that provides information on whether the reported measurement itself is LoS/NloS. (FL note: for inference)

- Apple (R1-2409788) Proposal 18: For model inference, the following elements shall be specified as part of the model inference procedure: - Part A - Measurement (corresponding to model input), - Time stamp, - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.)

- Xiaomi (R1-2409878) Proposal 16: In case 1 and case 2a, there is no specification impact on the delivery of input data for inference or the inference output data Proposal 17: For inference in Case 3a, leave RAN3 to assess whether is any specification impact on deliver the input from TRP to the associated gNB and deliver the input from one gNB to another gNB in different AI model deployment options. Proposal 18: For inference in Case 3b and Case 2b - Input data format for inference need to be specified and common input data format can be used for both data collection and inference - If multiple input data formats are defined, LMF configure the input data format for inference - The signaling exchange between gNB/UE and LMF is left to RAN3/RAN2

• CATT, CICTCI (R1-2409926) Proposal 10: For case 1 and case 2a, at least when UE-side model is trained by UE-side, UE generates the channel measurement for AI/ML model inference based on the PRS transmission. The input type or format is up to UE implementation. Proposal 11: For case 3a, at least when gNB-side model is trained by gNB/NW-side, gNB/TRP generates the channel measurement for AI/ML model inference based on the SRS-pos transmission. The input type or format is up to gNB/TRP implementation.

- Nokia (R1-2409986) Proposal 46: RAN 1 to discuss the potential approaches on reusing or enhancing the current LPP to enable the functionality framework for AI/ML positioning. Proposal 47: For cases 1, inference configuration for AI/ML positioning functionality is indicated to UE via the PRS configuration provided in LPP Assistance Data and the measurement configuration provided in LPP Request Location Information.

- Qualcomm Incorporated (R1-2410467) Proposal 23: In AI/ML positioning Case1, for inference related request, support at least the following information from LMF to UE: - Signaling on whether location information needs to be obtained using AI/ML, legacy, or both AI/ML and legacy method Proposal 24: In AI/ML positioning Case1, for inference related reporting configuration, start with reporting configuration of existing methods (i.e., NR DL-TdoA report configuration, NR DL-AoD report configuration)

- InterDigital, Inc. (R1-2409845) Proposal 31: For AI/ML based positioning case 1, at least the following types of measurements should be requested by the LMF to be made by the UE: - timing information; - paired timing information and power information. - phase measurement

# Model monitoring

This section discusses issues for the LCM stage of model performance monitoring.

## General principles and methods

This section discusses generic methods and principles for model monitoring, which are applicable to all cases.

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 24: Label-free monitoring should be up to implementation.

• ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 25: Label-free monitoring can be realized by implementation in a manner transparent to specification. No further discussion on label-free monitoring. Proposal 29: For UE side model and NG-RAN side model, model monitoring metric calculation function and model monitoring function can be located in the same or different entities.

• CMCC (R1-2409500) Proposal 12: For performance monitoring is based on the ground-truth labels, further study method to obtain ground-truth label. Proposal 13: For AI/ML based positioning, the relationship between model monitoring and positioning integrity can be considered. Proposal 14: For AI/ML based positioning, further discuss option 3 and 4. Deprioritize option 1 and 2.

• Samsung (R1-2409582) Proposal 20. RAN1 considers the model output based, model input based and other (measurement) based monitoring metrics. Proposal 21. RAN1 considers to define the measurement conditions (e.g., SNR level, LoS indication etc) to facilitate the performance (model) monitoring. Proposal 22. RAN1 considers the monitoring entity as UE/TRP/LMF for UE side model, TRP, LMF for TRP side model, LMF for LMF side model. Proposal 23. RAN1 considers the monitoring metric related threshold to be configured, dependent on the monitoring metric content. Proposal 24. RAN1 supports the signalling for the monitoring refence signal configuration, monitoring measurement and its report. Proposal 25. Monitoring outcome determination could be based on one time metric or multiple time metrics. Proposal 26. Post monitoring behaviour should be specified.

- Intel Corporation (R1-2409742) Proposal 17: Support both input-based and output-based functionality/model monitoring for AI/ML based positioning.

- TCL (R1-2410215) Proposal 2: It is recommended to perform the monitoring metric calculation at the model inference entity for label-free model monitoring. Proposal 3: LMF is responsible for functionality management decision making. Proposal 4: Further discuss whether the monitoring decision making is performed at LMF or the entity for monitoring metric calculation (i.e., UE side or NG-RAN node) for Case 1 and Case 3a.

- Fraunhofer IIS, Fraunhofer HHI (R1-2410205) Proposal 7: At least for Case 1 and Case 3a, functionality management is provided by the network (LMF). Proposal 8: At least for Case 1 and Case 3a, functionality monitoring is triggered by the NW (LMF). Proposal 9: For Case 1, UE-sided model monitoring can be triggered by the UE or the NW (LMF). Proposal 10: For Case 3a, gNB-sided model monitoring can be triggered by the gNB or the NW (LMF). Proposal 13: For both Case 1 and Case 3a consider a two-stage monitoring process: upon identifying of a potential model performance issue through early metrics, activate a broader set of monitoring metrics to confirm the actual presence and impact of the issue. Proposal 14: The possible actions/decisions of the functionality/model management entity can be: - More enhanced long-term monitoring for a specific model/functionality: - Configuration of event-based or scheduled monitoring in different time intervals, with the flexibility to adjust the monitoring frequency - Providing customization for reporting detail, notifying monitoring entity on the level of detail in monitoring reports. - Supporting different methods of generating ground truth labels for monitoring purposes, with varying quality of labels - Functionality or model (de-)activation/switching or fallback (temporary) - Functionality or model permanent marked as non-applicable in an area, with possible triggering for new data collection - AI/ML feature permanently deactivated in an area Proposal 15: Support validity indication for the AI/ML models. The indication shall include at least information about the existence of ML assisted areas. Proposal 17: Support monitoring the events resulting from functionality/model management decisions (fallback or (de)activation/switching) for a pre-defined time window. Proposal 18: Support a monitoring report including the outcome (performance, robustness, overhead/cost, etc.) of functionality or model management decisions when required. Proposal 19: In Cases 1 and 3a, the NW configures multiple functionalities at the UE/gNB and provides a configuration for automatic functionality switching and fallback, depending on specific parameters or threshold values.

- Tejas Networks Limited (R1-2409750) Proposal-20: For supporting UE side MPM metric calculation, the assistance data should be provided by the LMF to the UE or TRP for Case-1 and Case-3a respectively. Proposal-21: The range error or the positioning error should be considered as model performance metric for decision-making. Proposal-22: Both the MPM metric calculation and the MPM decision should be taken at one entity which can be either LMF or UE/TRP for Case-1 and Case-3a respectively. Proposal-23: The performance of the AI-ML models should be monitored periodically or aperiodically.

- Apple (R1-2409788) Proposal 29: the default option is for the monitoring to occur at the entity with the AI/ML model. Proposal 32: To ensure the accuracy of the GT - Step 1: use a LOS/NLOS identifier to identify >= 3 LOS links. Note that LOS probability may be used to derive the measurement quality. - Step 2: use legacy positioning method to estimate GT position Proposal 19: For model monitoring, the following elements shall be specified as part of the monitoring procedure: - Part A - Measurement (corresponding to model input), - Time stamp, - Quality indicator (for and/or associated with ground truth label and/or measurement at least for model monitoring), - Part B - Ground truth label, - Quality indicator (for and/or associated with ground truth label and/or measurement at least for model training), - Time Stamp - Assistance Information - RS configuration(s), - other necessary information (e.g., scenario identifier. LOS/NLOS condition, timing error, etc.) Proposal 39: For both LMF-side model and UE-side models, assistance information is needed. - Assistance information can be used to match the network/UE conditions between training data collection, inference and monitoring - New support needed to support assistance information transfer - Use LPP/NRPPa to transfer assistance data between PRU/UE, gNB and LMF Proposal 40: Case specific Assistance Signaling for UE/gNB-side model - LMF side Model: Assistance signalling from UE/PRU for network-side model monitoring (case 2b, 3b) - scenario change, Trigger LMF monitoring, Information on label and quality (case 2b, case 3b). - Update of both LPP and NRPPa. Proposal 41: Case specific Assistance Signaling for LMF-side model - UE/gNB-side model: Assistance signalling from LMF to UE/PRU/gNB for UE/gNB-side model monitoring (case 1, 2a, 3a) - Scenario change, LOS/NLOS condition, UE monitoring trigger. - signalling to indicate condition reached or action taken e.g. fallback (case 1, 2a) - o Update of both LPP and NRPPa.

• CATT, CICTCI (R1-2409926) Proposal 23: For label-free monitoring, for all cases, the performance metric calculation is up to implementation.

- Nokia (R1-2409986) Proposal 1: RAN1 to standardize both monitoring approaches, label-free based monitoring and label-based monitoring for first priority cases. Proposal 2: For Case 1 and Case 3a, label-free monitoring may consist of: i) confidence level associated with model inference output, e.g., UE location estimate; ii) statistics of measurement(s) compared to the statistics of measurements in the training data set; iii) standard deviation as monitoring metric based on the historical/buffered/successive inference output. Proposal 4: For AI/ML positioning Case 1 and Case 3a, label-free monitoring methods are supported, where the entity hosting the model for inference executes the performance monitoring without external information on the ground truth label. Proposal 6: Performance metric based on ground truth label (or its approximation) is defined as the statistical difference (e.g., MSE) between ground truth label and inference output, where measurements associated with ground truth information can be used as inference input. Proposal 7: Necessary data for label-based performance monitoring (or its approximation) consists of at least: i) ground truth (or its approximation), e.g., collected from PRU; ii) measurements corresponding to inference input, e.g., measurements collected from PRU; and iii) inference output, e.g., UE location estimation. Proposal 11: For label-free monitoring of UE-side models, for UE to derive monitoring metric, LMF may provide UE necessary data and configuration for monitoring.

- OPPPO Proposal 17: For functionality/model performance monitoring, Rel-19 is NOT to specify dedicated specification enhancement for the mechanism without ground-truth labels - Functionality/model performance monitoring without ground-truth labels can be done by implementation without any specification enhancement - Note: we also categorize some mechanisms based on “the approximate ground-truth label” in to that without ground-truth labels.

- NTT DOCOMO, INC. (R1-2410377) Proposal 14: For performance monitoring of AI/ML based positioning of case 1/3a, LMF is supported for functionality-level decision making. Observation 1: For AI/ML based positioning of case 1/3a, label-free monitoring methods are supported with self-monitoring by the model inference entity with no specification impact. • If the UE (Case 1) and gNB (Case 3a) sends the monitoring decision to LMF, signaling for label-based monitoring can be reused.

- vivo (R1-2409669) Proposal 34: Support both label-based and label-free monitoring for AI/ML based positioning.

- Ericsson (R1-2409443) Proposal 55 For AI/ML based positioning, support self-monitoring by model inference entity as a baseline. Furthermore, self-contained monitoring is preferred, where no estimated label needs to be signaled from external entities. Only when necessary, introduce signaling to provide information on ground truth label to the model inference entity. Proposal 56 The model inference entity is responsible for model-level performance monitoring and management, including calculation of model monitoring metrics. The detailed algorithm and procedure are up to implementation. Proposal 57 For model inference at UE (Case 1, 2a) or gNB (Case 3a), the LMF can provide assistance information to support the calculation of model monitoring metrics. Proposal 58 For AI/ML based positioning, LMF is responsible for functionality-level performance monitoring and management.

### 1st round discussion

Regarding functionality management in general, RAN2 made the following agreement in RAN2#125bis.

Agreements (RAN2#125bis): 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

Since model performance monitoring is a component supporting functionality management, the principles above apply. For positioning use case, network can be understood as LMF. Thus, LMF should be responsible for functionality level decision making related to monitoring.

For model performance monitoring, based on companies’ input (e.g., vivo, Nokia, Intel, Samsung, ETRI, OPPO, Ericsson), both label-free and label-based monitoring methods are to be supported. When UE-side performs label-free monitoring, it can be understood as “UE-autonomous”. Thus UE’s decision need to be reported to the network (i.e., LMF). In general, when label-free monitoring is used, the model inference entity (UE or gNB) should send the model-level monitoring decision to LMF, to support functionality-level management by LMF.

Proposal 6.1.2-1

For AI/ML positioning Cases, label-free monitoring methods are supported, where the model inference entity performs self-monitoring without external information on the ground truth label.

- For Case 1 and Case 3a, the UE (Case 1) and gNB (Case 3a) sends the monitoring decision to LMF, at least when the monitoring decision indicates that the model becomes inappropriate for inference.

- FFS: other specification impact (e.g., assistance data).

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| InterDigital | Due to lack of time, discussions related to label-free monitoring should be deprioritized. Is “if” missing from the proposal, i.e., “if label-free monitoring methods are supported,” |
| Lenovo | Could we have another proposal to further define what is meant by label-free monitoring or clarify in the above proposal? |
| Nokia | Using the wording “monitoring decision” is not accurate in our view. We suggested in our contribution to use the wording “monitoring outcome”. |
| Apple | Statement by the FL “LMF should be responsible for functionality level decision making related to monitoring“ is clear. The UE may perform label-free monitoring but should only be mandated to signal if there is a change in the UE side conditions that requires a functionality level decision by the LMF. |
| HW/HiSi | Label-free monitoring should be up to implementation. As stated in TR 38.843, the AI/ML functionality-based or model-based LCM is only applicable to UE-side models and/or UE-part of two-sided models. As such, functionality/model-based LCM is not applicable to gNB-side model of Case 3a. In addition, for Case 3a, the gNB can indicate a fallback as monitoring decision by reporting a measured value instead of a non-measured value, e.g., of the timing information based on the model output. The gNB indicates the non-measured value as a continuing activation of the model. |
| ZTE | Agree with HW/HiSi, Label-free monitoring is up to implementation. |

Proposal 6.1.2-2

For AI/ML based positioning, regarding model performance monitoring, the same entity performs the monitoring metric calculation and model-level monitoring decision making.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia, ZTE |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | Seems contradictory to previous agreement? This is not only precluding Option B-1 for case 1, but also precluding Option B for Case 3a. However we just agreed that in Case 3a, Option A and B are feasible from RAN1’s perspective. |
| Nokia | The sentence/message is not clear. |
| Apple | Okay |
| HW/HiSi | Ok |
| NEC | This proposal should be deprioritized. Besides, it seems this proposal is conflicted with proposal 6.2.2-2(if functionality-level management contain model-level monitoring decision). |
| ZTE | This can be discussed case by case. And also, the monitoring metric calculation entity for different cases haven’t determined yet. This should be discussed later. |

Proposal 6.1.2-3

For AI/ML based positioning all cases, LMF is responsible for functionality-level performance monitoring and management decision making (e.g., functionality selection, activation, deactivation, fallback to a legacy method).

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| CATT, CICTCI | Definition of functionality-level performance monitoring is unclear. But we think the later half is right, i.e. LMF is responsible for functionality-level management decision making. |
| Nokia | RAN1 need to discuss this topic case by case. |
| Apple | Okay.. see 1st comment in this section. |
| HW/HiSi |  |
| DOCOMO | OK |
| ZTE | The difference between model-level and functionality-level monitoring should be specified. Here, several behaviors are listed for functionality-level monitoring, while the behavior of model-level monitoring in the previous proposal is not clear. |
| Qualcomm | Let’s first scope and define functionalities in AI/ML positioning. |

## Case 1: Model monitoring

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Huawei, HiSilicon (R1-2409396) Proposal 25: For label-based model monitoring Case 1, there is no need to involve the LMF for metric calculation (i.e., Option B-1 and Option B-2), as the LMF may not have the knowledge to monitor/manage the UE-side models. Proposal 26: For label-based model monitoring Case 1, deprioritize options which need to restrict the model input which should be up to UE implementation (i.e., Options A-3 and Options B-2).

- ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 26: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, support option B-1 and option B-2: - Option B-1. At least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

- Intel Corporation (R1-2409742) Proposal 18: For AI/ML-assisted positioning Case 1, for label-based model monitoring at LMF, Options A-1 and A-2 should be considered further. - For Option A-1, ground-truth labels (or their approximations) are determined by the LMF based on legacy measurement metric(s) for RAT-dependent and/or RAT-independent positioning methods to LMF by target UE/NG-RAN node. - For Option A-2, ground-truth labels (or their approximations) are determined by the target UE based on legacy assistance information received from the LMF. - Option A-3 may be considered as a supplementary method in addition to one or both of Options A-1 and A-2. Proposal 19: For AI/ML-assisted positioning Case 1, for label-based model monitoring at LMF, Option B-1 should be considered further. - Estimated coordinates from AI/ML model at UE side are reported to the LMF by the target UE. - Ground-truth labels (or their approximations) are determined by the LMF based on known coordinates of PRUs or estimated coordinates for non-PRU Ues.

- Baicells (R1-2410414) Proposal 11. Support Option B-1 for Case 1, which can be achieved with minimum specification effort.

• Sharp (R1-2410360) Proposal 8: For performance monitoring in AI/ML based positioning Case 1, support Option A-3 in case that PRU is involving in the performance monitoring. Proposal 9: For performance monitoring in AI/ML based positioning Case 1, support Option A-2 in case that PRU is not involving in the performance monitoring.

- Ruijie Networks Co. Ltd (R1-2409841) Proposal 4: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, support Option A-3 as a baseline to generate information on ground truth label. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE.

- TCL (R1-2410215) Proposal 1: For model performance monitoring metric calculation in label-based model monitoring for AI/ML positioning Case 1, Option A-2 is preferred.

- Sony (R1-2410217) Proposal 13: For case 1, support Option A and the LMF sends criteria (e.g., parameters for performance evaluation, thresholds) to the UE performing performance metrics (PM) computation.

- Fujitsu (R1-2410049) Proposal-9: Regarding UE-side monitoring metrics calculation of Case 1: - Option A-1/A-2 is supportive, if the availability of the high-quality ground truth label for performance monitoring at UE-side can be confirmed. - Option A-3 is supportive, if the format of PRU measurement input can be the same as the format of UE’s model input. Proposal-10: Regarding LMF-side monitoring metrics calculation of Case 1: - Option B-1 is supportive, if the availability of the high-quality ground truth label for performance monitoring at LMF-side can be confirmed. - Option B-2 is supportive, if the format of PRU measurement input can be the same as the format of UE’s model input.

- Fraunhofer IIS, Fraunhofer HHI (R1-2410205) Proposal 11: For model performance monitoring of AI/ML positioning, ensure that Rel-19 supports the necessary signaling mechanisms for Options A-1, A-2, A-3, B-1, and B-2, including signaling for ground truth labels, monitoring events, time intervals, and assistance data.

- Apple (R1-2409788) Proposal 31: RAN1 should support at least Option A-2, A-3 and B-1.

- NEC (R1-2409852) Proposal 26: Support the main bullet of both Option A and Option B for case 1, i.e., both LMF and UE can perform monitoring metric calculation. Proposal 27: Prioritize Option A-3 if performs monitoring metric calculation at the target UE side. Proposal 28: Prioritize Option B-2 if performs monitoring metric calculation at the target LMF side.

- Xiaomi (R1-2409878) Proposal 21:For performance monitoring in Case 1 - The performance metric calculation can be performed on UE side - The LCM decision making can be performed by UE side

• CATT, CICTCI (R1-2409926) Proposal 24: For case 1 and case 2a, for model performance monitoring metric calculation in label-based model monitoring, Option A-1, A-2 and B-1 are supported. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option B-1. At least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. Proposal 26: For case 1 and case 2a, study the feasibility of using a higher-quality model deployed at LMF side to assist performance monitoring for the UE side model. Proposal 27: For model performance monitoring of AI/ML positioning Case 1, the following options with regard to how to generate information on ground truth label can be considered. - Option A. The target UE side performs monitoring metric calculation. - LMF sends the ground truth label (UE’s location) derived by AI/ML based method or non-AI/ML based method and UE performs monitoring metric calculation. - Option B. The LMF performs monitoring metric calculation. - UE sends the inference result (UE’s location) to LMF, and LMF performs monitoring metric calculation based on both UE’s inference results and its own information of UE’s location derived by LMF-side AI/ML model inference outcome.

- Nokia (R1-2409986) Proposal 3: For Case 1, necessary data for label-free monitoring consists of i) statistics of the measurements corresponding to inference input, i.e., RSRPP measurements; ii) historical/buffered inference output (UE location); and iii) characteristics (e.g., statistics) of training data. Proposal 5: For label-free monitoring in Case 1, UE sends at least the monitoring outcome (indication of events, e.g. failure) to LMF and/or monitoring metric output to LMF (LMF does always the final decision). FFS: other specification impact (e.g., assistance required by label-free monitoring, signalling of statistics of model input, statistics of model output). Proposal 8: For Case 1 monitoring, LMF may request UE to derive a performance monitoring metric, and UE reports the derived performance metric or reports monitoring outcome to LMF. Proposal 9: For label-based monitoring of Case 1, for UE to derive monitoring metric, support Option A-3: LMF provides the UE necessary data for monitoring over LPP which contains measurements (e.g., measurements (e.g., RSRPP) collected from PRU(s)) and associated ground truth (e.g., PRU location) including quality indicators. In addition, target UE computes the monitoring metric internally and provides it to the LMF if requested to do so. Proposal 10: For label-based monitoring of Case 1, for LMF to derive monitoring metric, support Option B-2: PRU(s) provides measurements (e.g., RSRPP) to LMF. LMF provides measurements collected from PRU(s) to the UE. UE reports inference output, based on measurements collected from PRU(s), to LMF. LMF compares the UE inference output with PRU ground truth labels associated with these measurements to calculate monitoring metric. Proposal 12: For label-free monitoring of UE-side models, for LMF to derive monitoring metric, UE may provide LMF necessary data for monitoring, which may contain: (i) statistics of UE measurements (e.g., RSRPP), (ii) standard deviation of UE inference output (e.g., UE location estimation in Case 1). Proposal 17: For label-based monitoring, LMF may provide to target UE, measurements collected from different PRUs along with a similarity context (e.g., threshold for a predetermined similarity score) to assist selecting a suitable PRU.

- OPPPO Proposal 18: For UE-based positioning with UE-side model (Case 1), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring - Functionality/model performance monitoring can be done by UE implementation without impact on air interface - The feasibility of Option A-1, A-3, A-4, B-2 has not been justified - For Option A-2, it is not clear what the solution it is. - For Option B-1, there is no additional spec impact and it is not clear how LMF use this information for monitoring metric calculation Proposal 19: Regarding the options for model performance monitoring of AI/ML positioning Case 1, further study the following options (including the feasibility) - Option A-1 - Option A-3 - If both Option A-1 and Option A-3 are supported, there should be separate UE capabilities for them

• ETRI (R1-2410258 Proposal 17: For label-based model performance monitoring of AI/ML positioning Case 1, Option A is preferable to Option B. Proposal 18: In Option A for label-based model performance monitoring in AI/ML positioning Case 1: - Option A-1 faces issues with the reliability of ground truth labels, especially in NloS environments where traditional methods fail. - Option A-2 raises concerns about the reliability of assistance data, which is not useful in environments where traditional positioning methods are ineffective. - Option A-3 is the most feasible and reliable solution for monitoring. - Option A-4 has no impact on the specifications. Proposal 19: For label-based model performance monitoring of AI/ML positioning Case 1, further study how to assess the reliability of approximate ground truth labels.

- Qualcomm Incorporated (R1-2410467) Proposal 29: In AI/ML positioning Case1, for monitoring, support the following assistance options (in RAN1#116bis agreement) for label-based monitoring metric calculation: - Option A-1 - Option A-2 - Option B-1 - FFS Option A-3 and A-4 (while considering specifications-based PRU data transfer, e.g., using SL)

- NTT DOCOMO, INC. (R1-2410377) Proposal 13: For performance monitoring metric calculation of AI/ML based positioning of case 1, both Option A and B are considered. Option B is a baseline if option 2 for decision making of case 1 is a baseline according to RAN2 agreement. Both legacy positioning-based and PRU-based ground truth label generations are considered.

- MediaTek Korea Inc. (R1-2410531) Proposal 4a-1: For case 1 performance monitoring, the metric calculation is conducted by UE Proposal 4a-2: For case 1 performance monitoring, Option B is not considered. Items within Option A can all be considered Proposal 4a-3: Once UE has identified the performance degradation after performance monitoring, the UE could send the error cause to LMF under the usage of AI/ML method. Then LMF may manage whether to terminate the AI/ML method and then to reconfigure another positioning method

- InterDigital, Inc. (R1-2409845) Proposal 18: For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, support Option A-1, A-2 and A-3.

- vivo (R1-2409669) Proposal 29: For model performance monitoring of AI/ML positioning Case 1, the target UE side performs monitoring metric calculation should be supported. Proposal 30: For model performance monitoring of AI/ML positioning Case 1, option A-1 and option A-3 can be supported for the target UE side performs monitoring metric calculation.

- Ericsson (R1-2409443) Proposal 59 For model performance monitoring of AI/ML positioning Case 1, support Option A-1 and A-2 for model performance monitoring metric calculation, if estimated label is considered necessary to be provided by an external entity (which is LMF for Case 1).

### 1st round discussion

At RAN1 #116bis and RAN1#118 meeting, the following was agreed or concluded for label-based monitoring for Case 1:

Conclusion (RAN1#118) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. Agreement (RAN1#116bis) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. At least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on the submitted contributions to RAN1#119, Companies’ preference for the Options is as follow (please update/correct if your view is not captured correctly):

|  | Supported by: | Not supported by: |
| --- | --- | --- |
| Option A in general | Sony, Xiaomi, Google, NTT DOCOMO, ETRI, MediaTek, NEC (prioritize A-3), Fujitsu(?) 8 sources | None |
| Option A-1 | Qualcomm, Ericsson, vivo, Fraunhofer IIS/Fraunhofer HHI, Lenovo, CATT/CICTCI, InterDigital, Intel, CMCC, OPPO (compromise) 12 sources | None |
| Option A-2 | Qualcomm, Ericsson, Apple, Fraunhofer IIS/Fraunhofer HHI, Lenovo, Sharp, CATT/CICTCI, TCL, InterDigital, Intel, CMCC 13 sources | OPPO (not clear what A-2 is) 1 source |
| Option A-3 | Apple, Fraunhofer IIS/Fraunhofer HHI, Lenovo, Sharp, vivo, Nokia, Ruijie Networks, InterDigital, OPPO (compromise) 10 sources | Ericsson, Huawei/HiSilicon 3 sources |
| Option B in general | NTT DOCOMO, NEC (prioritize B-2), Fujitsu(?) 3 sources | MediaTek, Ericsson, OPPO, Google, Huawei/HiSilicon 6 sources |
| Option B-1 | ZTE/Pengcheng Laboratory, Qualcomm, Apple, Fraunhofer IIS/Fraunhofer HHI, CATT/CICTCI, Intel, Baicells 10 sources | See above |
| Option B-2 | ZTE/Pengcheng Laboratory, Fraunhofer IIS/Fraunhofer HHI, Nokia 5 sources | See above |

For reference, RAN1#118bis status (see R1-2409286) is shown below.

|  | Supported by: | Not supported by: |
| --- | --- | --- |
| Option A in general | Sony, Xiaomi, Google, Docomo, ETRI, MediaTek 6 sources | OPPO 1 source |
| Option A-1 | Qualcomm, Ericsson, vivo, Fraunhofer IIS/Fraunhofer HHI; Lenovo; CATT/CICTCI; 8 sources | OPPO 1 source |
| Option A-2 | Qualcomm, Ericsson, Apple, Fraunhofer IIS/Fraunhofer HHI, Lenovo, Sharp, CATT/CICTCI, TCL 10 sources | OPPO (not clear what A-2 is) 1 source |
| Option A-3 | Apple, Fraunhofer IIS/Fraunhofer HHI, Lenovo, Sharp, vivo, Nokia, NEC 8 sources | OPPO, Ericsson 2 sources |
| Option B in general | ZTE/Pengcheng Laboratory 2 sources | MediaTek, Ericsson, OPPO 3 sources |
| Option B-1 | Qualcomm, Apple, Fraunhofer IIS/Fraunhofer HHI, CATT/CICTCI, Docomo 7 sources | MediaTek, Ericsson, OPPO 3 sources |
| Option B-2 | Fraunhofer IIS/Fraunhofer HHI, Nokia, NEC 4 sources | MediaTek, Ericsson, OPPO 3 sources |

Based on companies’ input, at least Option A in general, and Option A-1 in particular, should be supported in Rel-19.

Proposal 6.2.2-1

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1 is supported in Rel-19.

- Note: The definition of the monitoring metric is up to implementation.

Monday offline:

Proposal 6.2.2-1A

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1 and Option A-2, are supported in Rel-19.

- Option A-2 is clarified with a note below.

  - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. Note: the target UE can perform monitoring metric calculation by reusing the existing assistance data specified for legacy UE-based method  and no additional assistance data needs to be introduced for the purpose of monitoring metric calculation.

- Note: The definition of the monitoring metric is up to implementation.

Proposal 6.2.2-1B

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2, A-3 are supported in Rel-19.

- Option A-2 is clarified with a note below.

  - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. Note: the target UE can perform monitoring metric calculation by reusing the existing assistance data specified for legacy UE-based method.

- From RAN1 perspective, legacy framework and signaling can be reused for Option A-1, A-2, A-3.

- Note: The definition of the monitoring metric is up to UE implementation.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia, ZTE |

| Company | Comments |
| --- | --- |
| OPPO | We can first discuss proposal 6.2.2-2 with a focus on option A, and then address the supported methods within option A. |
| InterDigital | We are not sure why Option A-2 or A-3 is not considered. As we explained in our contribution, via Option A-3, ground truths of other locations (PRU locations) and associated measurements can be tranferred to the UE, improving efficiency in performance monitoring. Option A-1 or Option A-2 only allows the UE to obtain the ground truth corresponding to the UE. |
| Lenovo | Agree with InterDigital, it seems Option A-2 and A-3 can also be agreed upon. |
| Nokia | Using the wording “at least” in ambiguous, specially if Option A-3 and Option B-2 are not mentioned. All options must be treated as Option A-1. |
| Apple | We cannot agree to Option A-1 without considering at least Option A-2. In the case that the UE may not wish to expose its position to allow for GT estimation, then there is no label based monitoring for that UE. Both methods should be supported. |
| Hw/HiSi | Option A-1 can be supported by implementation in our view. Therfore a conclusion instead of agreement should be made. |
| DOCOMO | Agree with InterDigital. Especially, option A-3 should be also considered to generate the accurate ground truth label by PRU in NLOS condition. In our understanding, one of the motivations to introduce AI/ML based positioning is improving the positioning accuracy in heavy NLOS condition. |
| ZTE | LMF is the entity to perform monitoring metric calculation if UE doesn’t have the capability. |
| Fujitsu | Both Option A-1 and Option B-1 should be discussed for the conclusion. |
| Qualcomm | Let’s add Option A-2; it has also good supporters. |

Proposal 6.2.2-2

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE signals the monitoring decision to the LMF, at least when the monitoring decision indicates that the model becomes inappropriate for inference.

  - The LMF makes the functionality-level management decision for the target UE.

|  | Company |
| --- | --- |
| Support | InterDigital |
| Not support | Nokia |

| Company | Comments |
| --- | --- |
| OPPO | Fine with the direction. |
| New H3C | OK in general |
| CEWiT | Fine with the proposal |
| Nokia | we suggest to use the wording „Monitoring outcome“ instead that „monitoring decision“ for Case 1. |
| HW/HiSI | Should be discussed for the different sub-options of Option A. |
| DOCOMO | Fine for model-level performance monitoring metric calculation. The LMF makes the functionality-level management decision for the target UE. In addition, LMF side metric calculation of option B can be also considered. |
| ZTE | Same comment as 6.1.2-3, do we need to explain what is “functionality-level”, and specify the difference between “monitoring decision” and “functionality-level monitoring decision”? |
| Fujitsu | OK |
| Qualcomm | Important discussion but we need first to define functionality for Case1. |

### 2nd round discussion

Regarding Case 1 monitoring, the online discussion indicates that Option A-1 and A-2 are stable, while Option A-3 is more controversial. Thus the version discussed online is updated by removing Option A-3.

Proposal 6.2.3-1

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 are supported in Rel-19.

- Option A-2 is clarified with a note below.

  - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. Note: the target UE can perform monitoring metric calculation by reusing the existing assistance data specified for legacy UE-based method.

- From RAN1 perspective, legacy framework and signaling can be reused for Option A-1, A-2.

- Note: The definition of the monitoring metric is up to UE implementation.

- Note: No UE capability discussion about supporting A-1, A-2.

|  | Company |
| --- | --- |
| Support |  |
| Not support | InterDigital, Nokia, vivo |

| Company | Comments |
| --- | --- |
| CATT | We can accept; however if more options are added, we should also add B-1 since it has no new spec impact either. |
| HW/HiSi | Both Option A-1 and A-2 can be supported by implementation in our view, and hence, a conclusion instead of agreement is more appropriate. Also, both A-1 and A-2 resuse the existing measurements (from agreemement in RAN1#116b). It is not only the framework that is re-used, but also the measurements. Updated Proposal 6.2.3-1 Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 are supported in Rel-19. - Option A-2 is clarified with a note below. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. Note: the target UE can perform monitoring metric calculation by reusing the existing assistance data specified for legacy UE-based method. - From RAN1 perspective, legacy framework, measurements and signaling can be reused for Option A-1, A-2. - Note: The definition of the monitoring metric is up to UE implementation. - Note: No UE capability discussion about supporting A-1, A-2. |
| Sharp | OK |
| InterDigital | We do not support the proposal. We do not underestand why A-3 is not included as it is arealdy specified. We also do not understand the concern from some companies (e.g., potential support for sample-based measurements) as Option A-3 will use legacy measurements. Our proposal is to include A-3 in the modified proposal proposed by HW/HiSi. Modified Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 and A-3 are supported in Rel-19. - Option A-2 is clarified with a note below. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. Note: the target UE can perform monitoring metric calculation by reusing the existing assistance data specified for legacy UE-based method. - From RAN1 perspective, legacy framework, measurements and signaling can be reused for Option A-1, A-2 and A-3. - Note: The definition of the monitoring metric is up to UE implementation. Note: No UE capability discussion about supporting A-1, A-2 and A-3. |
| Nokia | Because Case A-3 is controversial, is not an argument to discard this proposal. As commented in the last online session. We are discussing label-based monitoring, which strongly depends on the GT quality and Option A-1 and Option A-2 are based on Labels that for sure cannot guarantee High quality of GT if compared to Option A-3. In addition, there were several companies that agreed on Option B-1 and Option B-2. However, we are focusing only in Option A. Discarting Option B automatically without any chance to explain the pros and cons is not a good way to proceed. |
| DOCOMO | We agree with InterDigital and CATT. Option A-3 is needed to improve positioning accuracy in NLOS condition and there is no specification impact to introduce option A-3. Moreover, LMF side metric calculation of option B can be also considered. |
| Lenovo | To perform label-based monitoring, the target UE would need to compare the ground truth measurement and ground label (UE location) of the the PRU location closest to its own inference generated location. This a reasonable use case for A-3. As Nokia mentioned, the UE location needs to have a low uncertainty to ensure good quality comparison, which is guaranteed by a static PRU UE. If companies have concerns, on sending proprietry measurements via LMF, then we can have an FFS on forwarding the PRU measurement to the target UE via LMF or restrict the forwarded PRU measurements to legacy PRU measurements, which are already supported. |
| Vivo | We don’t understand why A-3 is not included. FL listed Huawei/HiSi and Ericsson objected A-3. We would like to understand their concerns toward A-3. To Huawei/HiSi, Huawei support A-4 where PRU measurement and location is sent to UE for monitoring. If A-4 can work, why A-3 cannot be supported where A-3 is reusing existing signalling framework? To Ericsson, during offline discussion, Ericsson mentioned PRU measurement may not be the same as Case 1 UE model input and thus not feasible. We showed in our contribution (with evalaution results) where the PRU measurement does not have to exactly the same as model input. Even PRU’s lagecy measurement (e.g., carrier phase, timing and power) can be used for UE model monitoring. There’s also another comment that PRU measurement if not the same as model input may not be effective compared to A-1 and A-2. We’d like to point out that option A-3 should be more accrurate that option A-1 (where LMF can derive the information on ground truth label based on target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF) and option A-2 (where UE estimates the ground truth lable using legacy UE-based positioning via existing information for UE-based positioning method). |

### Wednesday offline discussion

Proposal 6.2.4-1

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE signals the monitoring outcome to the LMF, at least when the monitoring outcome indicates that the model becomes inappropriate for inference.

  - The LMF makes the management decision (e.g., activation, deactivation, fallback to a legacy method) for the target UE.

Proposal 6.2.4-1

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE signals the monitoring outcome to the LMF, at least when the monitoring outcome indicates that the model(s) become inappropriate for inference.

  - FFS: content of monitoring outcome

- FFS: Option B

Proposal 6.2.4-1

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF when the monitoring outcome indicates that the model(s) become inappropriate for inference.

  - FFS: content of monitoring outcome

- FFS: Option B

Proposal 6.2.4-1

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE signals the monitoring outcome to the LMF, at least when the monitoring outcome indicates that the model(s) become inappropriate for inference.

### 3rd round discussion

Proposal 6.2.5-1

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

FFS: Option B

|  | Company |
| --- | --- |
| Support | InterDigital |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia | As comented in the offline session, several companies are aware oft he benefits and feasibility of Option B. So, as a compromise, we are ok with the curren tversion. |
| ZTE | This version is better than the following proposals. But for option A, we agreed that the monitoring metric is up to UE’s implementation. How does LMF uses the monitoring outcome is a problem here. |
| InterDigital | What the LMF does with monitoring metrics can be left up to LMF implementation. |
| Tejas | We support the proposal. |

Proposal 6.2.5-2

- For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, Option A-2 is clarified with a note below.

- Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. Note: the target UE can perform monitoring metric calculation by reusing the existing assistance data specified for legacy UE-based method.

|  | Company |
| --- | --- |
| Support |  |
| Not support |  |

| Company | Comments |
| --- | --- |
| Nokia | We may include this clarification in case Option A-2 is selected, but we are not yet at that stage. Some companies believe that it is feasible, but some believes that Option A-3 is more effective for monitoring purposes (including Nokia). We suggest to delay this discussion. |

Proposal 6.2.5-3

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 are supported in Rel-19.

- From RAN1 perspective, legacy framework, measurements and signaling can be reused for Option A-1, A-2.

- Note: The definition of the monitoring metric is up to UE implementation.

- Note: No UE capability discussion about supporting A-1, A-2.

|  | Company |
| --- | --- |
| Support |  |
| Not support | Nokia, InterDigital |

| Company | Comments |
| --- | --- |
| Nokia | Excluding Option A-3 is the main reason of our objection. We have shared several arguments in favor of Option A-3 (check that Nokia is not the unique company). |
| InterDigital | As we have been commenting, the benefit of Option A-3 is that the UE can obtain the ground truth for locations other than the UE’s. We do not support the proposal. We propose the following. Modified Proposal 6.2.5-3 For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2, A-3 are supported in Rel-19. - From RAN1 perspective, legacy framework, measurements and signaling can be reused for Option A-1, A-2,A-3. - Note: The definition of the monitoring metric is up to UE implementation. - Note: No UE capability discussion about supporting A-1, A-2, A-3. |

### 4th round discussion

Proposal 6.2.5-3

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 are supported in Rel-19.

- From RAN1 perspective, legacy framework, measurements and signaling can be reused for Option A-1, A-2.

- Note: The definition of the monitoring metric is up to UE implementation.

- Note: No UE capability discussion about supporting A-1, A-2.

FFS: Option A-3

|  | Company |
| --- | --- |
| Support | TCL |
| Not support | Nokia, InterDigital |

| Company | Comments |
| --- | --- |
| Nokia | We have an strong rejection in Option A-1 and Option A-2. The compromise in our view is to include Option A-3. FFS: Option A-3 is not an option in this proposal. |
| HW/HiSi | There is no spec impact. We should make a conclusion. A-3 could be included in the conclusion as well if the group wants to do that. |
| InterDigital | As Option A-3 is already specified, we do not understand why it is not included in the main proposal. We repost the proposal we made in the previous round. Modified Proposal 6.2.5-3 For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2, A-3 are supported in Rel-19. - From RAN1 perspective, legacy framework, measurements and signaling can be reused for Option A-1, A-2,A-3. - Note: The definition of the monitoring metric is up to UE implementation. Note: No UE capability discussion about supporting A-1, A-2, A-3. |

## Case 3a: Model monitoring

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

• Huawei, HiSilicon (R1-2409396) Proposal 27: For performance monitoring of AI/ML positioning Case 3a, the request/report of a measured or non-measured result can be used to imply the activation/fallback between LMF and gNB - For Option A: NG-RAN node performs monitoring metric calculation, gNB can report LMF with measured result (implying fallback) or non-measured result (implying activation). - For Option B: LMF performs monitoring metric calculation, LMF can request gNB with measured result (implying fallback) or non-measured result (implying activation). Proposal 28: For performance monitoring of AI/ML positioning Case 3a, no need to specify the type of metric, i.e., the monitoring entity can calculate the metric by implementation.

- ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 27: For case 3a, LMF provides assistance data on the ground truth label to TRP. FFS: the detailed IE.

- CEWiT (R1-2410571) Proposal 14: For label free monitoring for Case 3a, gNB/TRP is preferred to perform the model monitoring metric calculation. Proposal 16: In case 3a, gNB is preferred to make model management decisions for its own model and notifies its decision to LMF. Proposal 17: LMF can provide its assistance such as threshold for performance degradation of the AI/ML model and recommendations such as which model to activate or which positioning method to fallback etc to gNB after receiving the management related decision from gNB .

- Sony (R1-2410217) Proposal 15: For case 1 and case 3a with option A, UE or gNB to provide an indication of AI/ML model validity to the AI/ML server/management (e.g., LMF).

- Fujitsu (R1-2410049) Proposal-11: Regarding performance monitoring metric calculation in label-based monitoring of Case 3a, model LCM control/decision is suggested to be done by gNB/TRP for both Option A and Option B.

- Apple (R1-2409788) Proposal 33: Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the NG-RAN. - Option A-2. Reuse Rel-18 assistance data transfer framework from LMF to the NG-RAN (based on NRPPa), where the PRU measurement (e.g., legacy measurement) and the corresponding PRU GT are sent via LMF to the NG-RAN. - Note as this is a PRU, the location is known by the LMF. Proposal 34: Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node - Option B-1: at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the NG-RAN node to LMF. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. {non-PRU with estimated position) - Option B-2: at least inference result (i.e., the model output corresponding to PRU channel measurement) “close” to the target UE is sent by the NG-RAN node to LMF. - In one example, PRU and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. Proposal 36: For Case 3a Option A - NG-RAN node performs monitoring metric calculation for its own model in the LOS/NLOS use case. - The NG-RAN node is not allowed to know the location of the non-PRU UEs connected to it. This is to maximize privacy for the non-PRU UEs connected to the NG-RAN node - The NG-RAN node is not allowed to perform the monitoring metric calculation for the timing estimation use case.

- Xiaomi (R1-2409878) Proposal 24: - For input-based performance monitoring - gNB calculates the performance metric and makes the LCM decision for the assisted positioning with multi-TRP construction - gNB calculates the performance metric and LMF makes the LCM decision for assisted positioning with single TRP construction -  Standardization effort is needed for the performance metric definition and report - For ground truth-based performance monitoring - LMF calculates the performance metric and makes the LCM decision

• CATT, CICTCI (R1-2409926) Proposal 25: For case 2b and case 3b, for model performance monitoring metric calculation in label-based model monitoring, LMF generates UE’s location as ground truth label and performs monitoring metric calculation.

- Nokia (R1-2409986) Proposal 13: RAN1 should not do any assumption or decision on Case 3a regarding performance monitoring.

- OPPPO Proposal 22: For NG-RAN node assisted positioning with gNB-side model (Case 3a), in order to facilitate ground-truth-label-based functionality/model performance monitoring at gNB, support the enhancement on NRPPa signaling to enable the delivery of the ground-truth label or the information that can derive the ground-truth label from LMF to gNB - Functionality/model performance monitoring can be done by gNB implementation

• ETRI (R1-2410258 Proposal 20: For Case 3a, in the absence of a PRU, explore methods to assess the reliability of ground truth labels used in label-based performance monitoring. Proposal 21: For label-free performance monitoring in direct AI/ML models, consider using legacy measurements and assisted positioning model outputs, in addition to leveraging model input/output statistics.

## Other proposals for model monitoring

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

• Huawei, HiSilicon (R1-2409396) Proposal 29: For model monitoring in Case 3b, no further assistance information or measurement report in addition to what is needed for inference is required to be sent to the LMF.

- ZTE Corporation, Pengcheng Laboratory (R1-2409480) Proposal 28: For LMF-side model, whether assistance information is required for model monitoring depends on whether LMF has prior information on UE/PRU’s location: - If LMF has prior information on UE/PRU’s location, there’s no need to let UE/PRU/gNB send any assistance information. - If LMF doesn’t have prior information on UE/PRU’s location, LMF can request UE to report legacy positioning result as ground truth label for model monitoring.

- Xiaomi (R1-2409878) Proposal 25: Before receiving the request from other WG, there is no need for RAN1 to study the assistance information for performance monitoring Proposal 26: Confirm the necessity of assessment/monitoring of inactive models / functionalities, with the following assumptions as the starting point: - One way to monitor inactive models/functionalities is by activating them and reusing mechanisms defined for monitoring of active models/functionalities. - The following aspects may be considered for further study or in WI to assess the applicability and expected performance of an inactive model/functionality: - Configuring an AI/ML model for monitoring without activation (e.g., monitoring-only mode without reporting predicted beams in BM Case 1 and 2) - Dataset delivery from the network to the UE for assessment/monitoring of the applicability and expected performance of the model/functionality. - NW may provide performance criteria/preference for UE’s model selection. - o Other aspects are not precluded for further study or specification.

- Nokia (R1-2409986) Proposal 14: RAN1 to delay the discussion on any aspect related to performance monitoring for Case 3b. Proposal 15: RAN1 to consider only the measurement reporting from gNB to LMF for assisting on the performance monitoring for LMF-side model cases. Proposal 16: RAN1 to consider that whether/what assistance information is sent from UE/PRU or gNB to LMF for performance monitoring of LMF-side model, the final decision on using such assistance is up to the responsible NF (to be determined by SA2).

- OPPPO Proposal 23: For NG-RAN node assisted positioning with LMF-side model (Case 3b), Rel-19 is NOT to specify dedicated specification enhancement for functionality/model performance monitoring at LMF - Functionality/model performance monitoring can be done by LMF implementation without impact on NRPPa signaling

# Model identification and model functionality

## Model identification, model ID

### Companies’ view from contribution

In the following, selected inputs from companies’ contributions are provided.

- Samsung (R1-2409582) Proposal 1. RAN1 considers the trigger conditions for enabling AI/ML based method for positioning, including potential model applicable conditions or the environment(channel) applicable conditions. Proposal 2. In addition to the entity where the model is deployed, the network can decide or /assist to decide the model used at UE side, the LMF can decide and LMF/UE can assist to decide the model used at TRP side, the TRP/UE can assist to decide the model used at LMF side. Proposal 3. RAN1 supports the indication of determined model from one entity to another entity Proposal 4. RAN1 supports the indication of measurement related threshold/report/feedback and model applicable condition to assist the model determination from one entity to another entity

- NVIDIA (R1-2409781) Proposal 9: For AI/ML based positioning, introduce specification support for additional conditions to be included in model description information during model identification.

- Apple (R1-2409788) Proposal 43: Regarding LCM of AI/ML based positioning accuracy enhancement, - Functionality based LCM: Applicable to a one-sided model without model transfer and should be used AI/ML based positioning. - For the UE sided model (Case 1) specification shall cover UE capability reporting using UE features and feature groups and help identify the capability of the AI/ML models including scenarios, positioning types (direct or AI-assisted AI-ML positioning), Measurement and Reporting capability and site/scenario/dataset specific capabilities. - For the network side models (Case 3a and Case 3b), the specification shall cover the capability of UEs/PRUs on data collection. - model-ID-based LCM: may be used for the one-sided model in AI/ML based positioning with a model-ID indicating the model(s) to be used. - In MI-option 1, the associated ID(s) is assumed as model ID(s) where the associated ID is the ID associated with a specific data collection configuration.

- Xiaomi (R1-2409878) Proposal 12: Support functionality identification for Case 1 and Case 2a Proposal 13: Whether there is functionality identification / model identification-like procedure between gNB and LMF is up to RAN3 Proposal 14: Categorize the AI/ML feature from the following 3 aspects for AI-based positioning - Output type of the AI models - The location where AI models are deployed - The positioning RS type i.e., SRS-based or PRS-based Proposal 15: At least measurement related aspects e.g., PRS bandwidth, TRP configuration can be considered as conditions to categorize the AI/ML functionality

## Model functionality, UE capability reporting

### Companies’ view from contribution

- Huawei, HiSilicon (R1-2409396) Proposal 30: Support functionality-based LCM for UE-side model of Case 1/2a.

- ZTE Corporation, Pengcheng Laboratory (R1-2409480) - Proposal 30: For AI/ML positioning, functionality is recognized as use case 1, use case 2a and use case 2b, where UE capability is reported per use case. FFS: detailed parameters for UE capability.

- Intel Corporation (R1-2409742) Proposal 20: At least for UE-side models, model/functionality selection or switching could be realized via explicit indication from a network entity (e.g., LMF) or implicitly based on configurations related to data collection for training and/or inference, e.g., configurations related to data collection in time dimension or over a certain geographical area.

- NVIDIA (R1-2409781) Proposal 7: For AI/ML based positioning, introduce specification support for UE capability signalling including model training, model inference and model monitoring. Proposal 8: For AI/ML based positioning, introduce specification support for conditions of a Feature/FG available for functionality.

- Apple (R1-2409788) Proposal 44: For AI/ML based positioning, discussion on whether to have AI/ML specific capabilities and positioning capabilities combined or separated is needed. Proposal 45: Information covered by the UE capability may include: - Positioning types (based on agreed on cases) - Capability may include parameters such as the specific AI/ML case type. - Measurement and Reporting capability: Reference signal configuration such as reference signal resources, measurement capability (for case 1, 2a and 2b) and measurement reporting capability (case 2a e.g. LOS/NLOS and 2b e.g. CIR, PDP, DP) - UE/Network conditions including Scenarios, site, or dataset specific information: As an example, site specific capabilities may be based on physical cell-IDs and even sub-site IDs to accommodate the scenario in which a cell may have to be supported by multiple different models due to differences within the cell. This may be sent as additional conditions for the AI/ML model. - Note that this may require an update to the capabilities procedure as discussed in RAN2 - Monitoring requirements such as ground truth labels and quality indicators

- Xiaomi (R1-2409878) Proposal 22: Further study the following two options for the case that UE make the LCM decision in Case 1 - Option 1: UE make the LCM decision based on UE’s implementation and then report the LCM decision to LMF - Option 2: LMF provide a performance monitoring metric threshold, UE could make the LCM decision by comparing the calculated performance metric and the configured threshold. o Standardization effort is needed for the performance metric definition Proposal 27: The fallback mode including the non-AI positioning approach and related configuration are preconfigured to UE Proposal 28: UE group-based LCM operation (e.g., model activation/deactivation/switch) can be considered

• CATT, CICTCI (R1-2409926) Proposal 35: On AI/ML functionality management for Case 1 and Case 2a, wait for more progress of applicable functionality reporting in AI/ML-based beam management in RAN1.

- Nokia (R1-2409986) Proposal 48: UE may report applicable conditions to LMF via UE capability report, where the applicable conditions are the subset of conditions that can be supported at the UE for the given network-side (additional-)condition. Proposal 49: For UE-sided models, no reconfiguration is expected on any functionality based on the preferred dynamic/online indications coming from the UE. Proposal 50: For all use cases (e.g., Cases 1, 3a, 3b) LMF is the only entity to determine functionality decision (e.g., functionality switching, selection, (de)activation, fallback) based on monitoring outcome.

- Lenovo (R1-2410019) Proposal 28: In order to determine applicable functionality, network-side/gNB considtions and UE-side conditions need to be considered and may consist of the following, respectively: - Network-side/gNB Conditions: DL-PRS Configurations, TRP/ARP Location Information, Synchronization information, Available on-demand PRS configurations, DL-PRS beam power/spatial direction information, Timing Error Group Information corresponding to DL-PRS resources, DL-PRS Area Validity Information, SSB Configuration Information. - UE Conditions: PRU measurement and location information, Expected LOS/NLOS at previous location/measurement, Requested Area for training/inference.

- InterDigital, Inc. (R1-2409845) Proposal 19: For Case 1, enhance the existing UE-based DL-TDOA positioning method to support AIML-based positioning without introducing a new positioning method for AIML-based positioning Proposal 20: For Case 1, inference configuration can be provided in Step 3 in the procedure for LCM agreed in RAN2#127b Proposal 21: For Case 1, unsolicited UE capability signalling can be sent in Step 4 by the UE to inform the LMF that the UE is ready to perform AIML based positioning Proposal 22: Specify a fallback positioning method which can be triggered if network conditions are inconsistent or AIML based performance is below the performance target

- vivo (R1-2409669) Proposal 43: Support specifying AI/ML based positioning Case 1 as a new positioning method.

# Proposals for online sessions

## Proposals for Monday online session

Proposal 6.2.2-1B

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2, A-3 are supported in Rel-19.

- Option A-2 is clarified with a note below.

  - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. Note: the target UE can perform monitoring metric calculation by reusing the existing assistance data specified for legacy UE-based method.

- From RAN1 perspective, legacy framework and signaling can be reused for Option A-1, A-2, A-3.

- Note: The definition of the monitoring metric is up to UE implementation.

Proposal 3.1.2-2A

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

- When no associated measurement (e.g., timing information) is reported, LOS/NLOS indicator is not reported for Rel-19 Case 3a.

Proposal 3.1.2-1A

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective,

- The LOS/NLOS indicator reuse the meaning of the LOS/NLOS indicator from TS 38.455, i.e., provides the likelihood of a line-of-sight propagation path for the channel measured over the resource for which the measurement is reported;

- The LOS/NLOS indicator can reuse the meaning and format of the existing IE "LoS/NLoS Information" in 38.455, which can be soft indicator or hard indicator.

- Note: it is up to RAN3 to decide how to differentiate reporting for Rel-19 Case 3a from reports from pre- Rel-19 positioning.

Proposal 5.1.2-1A

For AI/ML based positioning Case 1, if necessary, at least the following assistance information are provided from LMF to UE explicitly:

- From RAN1 perspective, these assistance information can be provided as optional fields (as in existing specification)

|  | Information |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |

Proposal 5.1.2-4

For AI/ML based positioning Case 1, at least the following assistance information are not needed from LMF to UE, neither explicitly nor implicitly:

| 17 | Expected Angle Assistance information |
| --- | --- |
| 18 | PRS priority list |

## Proposals for Tuesday online session

Proposal 2.1.1.3-1A

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

- If LMF configures an offset relative to a reference time, and the gNB follows the configuration,

  - starting time = reference time + offset

    - FFS: value range of the offset

- Otherwise

  - starting time = first detected sample (i.e., first detected path rounded down with timing granularity T).

Note: UE-side measurement is a separate discussion

Proposal 3.1.3-1A

For AI/ML assisted positioning, when the LOS/NLOS indicator is reported for Rel-19 Case 3a, and the associated measurement (e.g., timing information) is obtained by legacy positioning methods, from RAN1 perspective,

- The LOS/NLOS indicator can reuse the meaning and format of the existing IE “LoS/NloS Information” in 38.455, which can be soft indicator or hard indicator.

- Note: it is up to RAN3 to decide whether/how to differentiate a report for Rel-19 Case 3a from a report for pre- Rel-19 positioning.

Proposal 4.1.3-2A

For AI/ML based positioning Case 1, if channel measurement and/or label is reported, both IEs as described in TS 37.355 are included in the time stamp:

- NR-TimeStamp

- UTCTime

  - From RAN1 perspective, UTCTime can be an optional field (e.g., present at least when SFN wraps around).

Note: NR-TimeStamp and UTCTime are intended to refer to the same clock time associated with the label.

Proposal 4.1.3-3

For AI/ML based positioning, from RAN1 perspective, when label is UE location and reported, label and quality indicator of label are provided by reusing existing IE.

- From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032.

Proposal 5.1.3-2

For AI/ML based positioning Case 1, at least the following assistance information are not needed from LMF to UE, neither explicitly nor implicitly.

- Note: As in existing specification, the LMF may optionally provide them to the target UE for the purpose of supporting legacy UE-based DL-AoD method.

| 17 | Expected Angle Assistance information |
| --- | --- |
| 18 | PRS priority list |

Proposal 5.1.3-1

For AI/ML based positioning Case 1, at least the following assistance information from DL-TDOA are provided from LMF to UE explicitly.

- FFS: whether/how to provide the remaining information.

|  | Information |
| --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |
| 3 | DL-PRS configuration of candidate NR TRPs |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |
| 13 | Validity Area of the Assistance Data |

## Proposals for Wednesday online session

### Case 1 monitoring

Proposal 6.2.5-1

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

FFS: Option B

Proposal 6.2.5-2

The following agreement made in RAN1#116bis is updated as below.

Agreement (RAN1#116bis)

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label:

- Option A. The target UE side performs monitoring metric calculation.

  - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE.

    - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label.

  - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. Note: the target UE can perform monitoring metric calculation by reusing the existing assistance data specified for legacy UE-based method.

  - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE.

  - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server).

    - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method.

- Option B. The LMF performs monitoring metric calculation.

  - Option B-1. At least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF.

  - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF.

Note: exact method to perform the monitoring metric calculation is up to implementation.

Note: Other options are not precluded.

### Sample-based vs path-based measurement

Proposal 2.1.3.3-1 (compromise view)

(For Working Assumption)

At least for Rel-19 AI/ML based positioning Case 3b, for determining model input, support both Alternative (a) sample-based measurement report and Alternative (b) path-based measurement report.

- For Case 3b, LMF signals to gNB whether (a) or (b) is requested to be reported.

Proposal 2.1.3.2-1B (majority view)

At least for Rel-19 AI/ML based positioning, for determining model input, support Alternative (a) sample-based measurement report.

| Method | Supporters |
| --- | --- |
| Alternative A (sample-based measurement) (23 sources) | vivo, CEWiT, CATT/CICTCI, NEC, CMCC, Baicells, Sony, ETRI, Mediatek, ZTE/Pengcheng laboratory, Ericsson, Sharp, Xiaomi (Case 2b/3b), TCL, Ruijie Networks, NTT Docomo, OPPO (optional), Lenovo (Case 1,2b,3b), Tejas, IITM/IITK |
| Alternative B (path-based measurement) (9 sources) | Qualcomm, Huawei/HiSilicon, Samsung, InterDigital, Intel, OPPO, Lenovo (Case 2a,3a), Tejas |
| Specify both (9 sources) | Apple, Nokia, Google, NVIDIA, Lenovo (vary case by case), NEC (compromise), Ericsson (compromise), Fraunhofer IIS/Fraunhofer HHI |

### Assistance data

Proposal 5.1.5-1

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, except at least #7 below, (i.e., #1-#6, #8-#15), may be provided explicitly from LMF to UE.

- At least the following info is provided implicitly.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

- The assistance information is sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

Note: LMF may provide #7 as in existing specification for the purpose of supporting legacy UE-based DL-TDOA.

Proposal 5.1.5-2

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-AOD, except at least #16 below, may be provided explicitly from LMF to UE.

- At least the following info is provided implicitly.

| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |
| --- | --- |

- The assistance information is sent in Step 3 of RAN2 procedure: “Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition).”

Note: LMF may provide #16 as in existing specification for the purpose of supporting legacy UE-based DL-AOD.

### Training data collection

Proposal 4.1.4-2

For AI/ML based positioning, when label is UE location and reported (e.g., LMF to UE over LPP), label and quality indicator of label are provided by reusing existing IE.

- From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label.

## Proposals for Thursday online session

### Sample-based vs path-based measurement

Proposal 2.1.3.4-1A

For Rel-19 AI/ML based positioning, support the following to obtain the time domain channel measurements,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time.

For measurement by gNB/TRP,

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candaiate set values:

  - Nt' ={9, 16, [24], [32]}

  - Nt = {32, 64, 128}

  - k = {3, 4, ,5}

  - The gNB/TRP may use different Nt' and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

### Assistance data

Proposal 5.1.6-2A

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA may be provided explicitly from LMF to UE.

- [Alternative suggestion 1] At least the following info #7 is provided implicitly via associated ID.

  - Associated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference.

- [Alternative suggestion 2] At least the following info can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether the following info is provided implicitly or explicitly, and the UE can request the info being provided explicitly or implicitly.

  - If provided implicitly, associated ID is provided by LMF to indicate whether the corresponding network-side additional condition is consistent between training and inference.

- [Alternative suggestion 3] At least the following info #7 may not be provided from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

### Output of AI/ML assisted

Proposal 3.1.5-1

For measurement report of AI/ML assisted positioning Case 3a, when timing information is reported from gNB to LMF, LMF shall be able to destinguish whether the timing information is obtained by legacy method or by Rel-19 method.

FFS: how to ensure that LMF can distinguish

### Training data collection

Proposal 4.1.4-2

For AI/ML based positioning, when label is UE location and reported, label and quality indicator of label are provided by reusing existing IE.

- From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The uncertainty and confidence serve as quality indicator of the label.

### Case 1 monitoring

Proposal 6.2.5-3

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, at least Option A-1, A-2 are supported in Rel-19.

- From RAN1 perspective, legacy framework, measurements and signaling can be reused for Option A-1, A-2.

- Note: The definition of the monitoring metric is up to UE implementation.

- Note: No UE capability discussion about supporting A-1, A-2.

FFS: Option A-3

## Proposals for Friday online session

Proposal 2.1.3.5-1A

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from  gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

[Further compromise: Nt'={9,16}]

Proposal 5.1.7-1B

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study and choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

FFS: assistance information from legacy UE-based DL-AOD

# Agreements made at RAN1#119

Conclusion

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

- LOS/NLOS indicator can’t be reported independently from other measurements

Agreement

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

  - starting time = first detected path rounded down with timing granularity T.

Note: UE-side measurement is a separate discussion

Agreement

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

- FFS: Option B

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

# Future work

During RAN1#119, good progress was made on the issue of channel measurement for obtaining model input. On other topics, companies had intensive debate but progress was limited. Companies are encouraged to continue the investigation and share the views in contribution to RAN1#120.

In the following, several open issues are highlighted for discussion at RAN1#120.

- Model input

  - (Section 2) The remaining issues are the FFS points in the agreement. Mainly: (a) candidate values for Nt' and k. (b) “FFS: whether transmit offset from gNB to LMF”. (c) measurement by UE.

- Model output for AI/ML assisted positioning

  - (Section 3.1) Regarding LOS/NLOS indicator and the associated measurement (e.g., timing information), several proposals were drafted but not concluded. Please share your view on the points in section 3.1.4, including the default understanding described therein.

- Training data collection

  - (Section 4.1) Time stamp, label and quality indicator of label.

  - (Section 4.4) How to pair Part A and Part B, especially when they are generated by different entities.

- Ensure consistency between training and inference

  - (Section 5.1) Please share your view on Alternative 1-4 in the agreement for Case 1. For proponents of Alternative 1 and 2, please provide detailed design on associated ID.

  - (Section 5.1) Regarding assistance info related to legacy UE-based DL-AOD, please share your view whether they need to be provided to UE.

- Model monitoring

  - (Section 6.1) Regarding label-free methods and self-monitoring, there was an effort to identify the specification impact, if any. Here most monitoring procedures are up to implementation and out of 3GPP scope. It would be good if we can conclude on the specification impact, if any.

  - (Section 6.2) For Case 1, Option A in general is agreed to be supported. Further discussion is needed on individual Options A-1, A-2, A-3, B-1, B-2. Signaling subsequent to monitoring decision needs to be discussed, e.g., “FFS: content of monitoring outcome”, and when the UE should send the monitoring outcome to LMF.

    - Regarding Option A-2: there was an attempt to add a note to clarify it. Although not agreed, companies can assume that Option A-2 is clear and no need to further discuss the understanding of Option A-2. It was commented that the suggested Note simply rephrased the “e.g.” point.

# References

RP-242399, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024.

R1-2409396	Discussion on  AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2409443	AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2409480	Discussion on AI/ML-based positioning enhancement	ZTE Corporation, Pengcheng Laboratory

R1-2409500	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2409543	Discussion on AI/ML for positioning accuracy enhancement	New H3C Technologies Co., Ltd.

R1-2409582	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2409669	Specification support for positioning accuracy enhancement	vivo

R1-2409742	Specification support for positioning accuracy enhancement	Intel Corporation

R1-2409750	Specification support for positioning accuracy enhancement	Tejas Networks Limited

R1-2409781	Specification support for AI-enabled positioning	NVIDIA

R1-2409788	Discussion on Specification Support for AI/ML-based positioning	Apple

R1-2409841	Discussion on specification support for positioning accuracy enhancement	Ruijie Networks Co. Ltd

R1-2409845	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2409852	Discussion on specification support for AIML based positioning accuracy enhancement	NEC

R1-2409878	Discussion on AI/ML-based positioning accuracy enhancement	Xiaomi

R1-2409926	Specification support for AI/ML-based positioning	CATT, CICTCI

R1-2409986	AI/ML for Positioning Accuracy Enhancement	Nokia

R1-2410019	Specification impacts for AI/ML positioning	Lenovo

R1-2410049	Discussion on specification support for AIML-based positioning accuracy enhancement	Fujitsu

R1-2410102	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2410150	AI/ML based Positioning	Google

R1-2410205	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2410215	Discussion on specification support for positioning accuracy enhancement	TCL

R1-2410217	Support for AI/ML for positioning accuracy enhancement	Sony

R1-2410258	Discussion on specification support for positioning accuracy enhancement	ETRI

R1-2410360	Discussion on specification support for AI/ML based positioning accuracy enhancements	Sharp

R1-2410377	Discussion on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2410414	Discussion on specification support for AI-ML based positioning accuracy enhancement	Baicells

R1-2410424	Specification Support of AI/ML for Positioning Accuracy Enhancement	Indian Institute of Tech (M), IIT Kanpur

R1-2410467	Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

R1-2410531	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2410571	Discussion on specification support for AI/ML Positioning Accuracy enhancement	CEWiT

R1-2410588	Discussions on specification support for positioning accuracy enhancement for AI/ML	ITL

# Appendix A. Agreements from RAN1#118bis

Agreement

For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is:

- When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

  - FFS: details on how to associate quality indicator to timing information

Conclusion

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |

[1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

# Appendix B. Agreements from RAN1#118

Model input

Agreement

For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples

- The Nt samples have timing granularity T.

- FFS: the starting time of the list of Nt samples

- FFS: the value range of Nt

For the sample-based measurement (if accepted in Rel-19),

- For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power.

- Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation.

Agreement

For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective,

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa.

Agreement

For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input,

- For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point.

- For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

Model training

Agreement

For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement,

- For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective

- Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

Agreement

For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Model monitoring

Agreement

For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible,

- Option A.	NG-RAN node performs monitoring metric calculation for its own model.

- Option B.	LMF performs monitoring metric calculation for the model located at the NG-RAN node.

Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation.

Note: Exact method to perform monitoring metric calculation is up to implementation.

Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

Conclusion

For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring,

- Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

All LCM stages

Conclusion

From RAN1 perspective, for Case 3a measurements,

- The existing procedures can be reused in terms of SRS configuration.

  - Note: parameter values for SRS configuration can be further discussed

- These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures.

- Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications