3GPP TSG-RAN WG2 Meeting #127	R2-2406485

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.2.5

Source: 	CATT

Title: 	Discussion on Topology2 for Ambient IoT

Document for: 	Discussion and Decision

1	Introduction

In this contribution, the topology 2 for A-IoT will be discussed from RAN2’s perspective based on progress of RAN plenary and output of other groups including SA2 and RAN1.

2	Discussion

2.1	 Network architecture

In RAN#104, the network architecture of topology 2 was discussed based on the following figure [1]. But no consensus was reached on issue of whether the “Common reader function” of the UE reader belongs to a part of RAN or UE.

There is no doubt that there are two parts of the intermediate UE in the Topology 2 from functionality point of view, including reader part (Common Reader Function) and the UE part. From our perspective, there is no need to discuss and decide whether the common reader function is part of UE or RAN, and the terminology “RAS” is not needed. “AIoT-enabled gNB” and “AIoT-enabled UE” can be used directly. The proposed architecture is shown as below:

Figure 6.3.y Logical System Architecture for AIoT, Topology 2

Proposal 1: In Topology 2, no need to decide whether the common reader function of the UE reader is part of UE or part of RAN.

2.2	 Protocol stack and data transmission schemes

SA2 is discussing the architecture and protocol stack for topology 2. According to the TR 23.700-13 [2], it can be seen that the inventory/command is generated by CN (e.g., AMF or AIoTF) or external AF. And for almost all of the candidate solutions, the inventory/command is transmitted to UE reader and/or A-IoT device via upper layer. In detail:

- For inventory procedure: the request is raised by CN or external AF and transmitted to UE reader via upper layer. After receiving the inventory request, the UE reader will trigger inventory for A-IoT devices.

- For command procedure, the request is raised by CN or external AF and transmitted to the UE reader via upper layer, and the request could contain a command packet for A-IoT device which may invisible for the UE reader. After the UE reader receives the command, it will forward the command towards A-IoT device.

Focus on the transmission for the inventory/command service between UE reader and RAN node:

- For CP solution, reference to solution# 4, 21, 30 in TR 23.700-13 [2], the transmission of inventory/command is contained in the DL NAS message and sent towards UE. After receiving the response from A-IoT device(s), the UE reader packs the received A-IoT NAS packet into UL NAS message, and forwards to CN. The UL/DL transfer between UE reader and RAN can rely on legacy NAS transfer mechanisms. An example for the protocol stack of CP solution is given as below.

Figure 1: Protocol stack of CP solution for A-IoT topology 2

- For UP solution, reference to solution# 3, 4, 22, 32 in TR 23.700-13 [2], the UE reader could establish a PDU session for the A-IoT service. The inventory/command request raised by CN/AF is contained within PDU session established by UE reader. After receiving the response from A-IoT device(s), the UE reader forward the received package towards CN/AF via the PDU session. The UL/DL transfer between UE reader and RAN can rely on legacy PDU session transmission. An example for the protocol stack of UP solution is given as below.

Figure 2: Protocol stack of UP solution for A-IoT topology 2

In summary, the transmission between UE reader and RAN node for inventory/command service can be supported by legacy mechanism, i.e., via NAS message or PDU session. No RAN2 impact is identified so far. We can further check whether enhancement is needed when the network architecture and protocol stack are determined by SA2.

Proposal 2: RAN2 assumes the transmission between UE reader and RAN node for inventory/command service can be supported by legacy mechanism, i.e., via NAS message or PDU session, which does not bring stage-3 impact on Uu. RAN2 may come back when the network architecture and protocol stack are determined by SA2.

2.3	 Key issues on management of UE reader in Uu interface

Base on the assumption given in proposal 2, some key issues related to management of UE reader in Uu interface will be discussed in this section.

1) Resource allocation for the new A-IoT interface

For topology 2, RAN1 is studying the following options to support CW and D2R transmission [3]. The CW and D2R transmission may be transmitted in UL spectrum or DL spectrum. Considering there may be transmission requirement at Uu interface and the new A-IoT interface at the same time, that radio interfere and decoding failure may occur.

Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

To avoid this issue, the radio resource used in the new A-IoT interface should be allocated or controlled by RAN node.

Proposal 3: The radio resource(s) of the new A-IoT interface should be allocated/ controlled by RAN node.

Each inventory/command service procedure may have different requirement on the radio resource at the new A-IoT interface. For example, one inventory procedure is for a single device and another inventory procedure is for a group of devices, the latter needs more radio resource than the former. Hence, it is inefficient to allocate static resource for UE reader. To avoid congestion or resource waste in the new A-IoT interface caused by RAN node allocates insufficient or excessive resources, three way forwards for resource allocation are given as following:

- Option 1: After the UE reader receives inventory/command from CN/AF, based on the characteristic of the current service, it request resource towards the RAN node. Then the RAN node allocates resource(s) according to UE’s request

- Option 2: The RAN node acquires some assistance information from CN/AF, and assigns appropriate resource to UE reader. Take CP solution as an example, the assistance information from CN/AF is sent to RAN node within the N2 message which contains the inventory/command. The assistance information can be number of target devices and the service type of the inventory/command.

- Option 3: The RAN node assigns a common resource pool for the new A-IoT interface in advance and the UE reader could select resource from the resource pool without confirmation from the RAN node. The resource pool can be broadcast in SIB and common to all the UE within this cell.

Proposal 4: Study solutions on configuring radio resource for the new A-IoT interface, e.g.,

- UE reader requests resource to RAN node, and the RAN node assign suitable resource for the UE reader;

- RAN node acquires assistance information from CN/AF and configures suitable resource for the UE reader;

- RAN node assigns a common resource pool in advance and UE reader selects resource from the resource pool without confirmation from RAN node.

The detailed radio resource configured for the new A-IoT interface is still under discussion of RAN1, including dimensions of time domain, frequency domain and code domain. RAN2 can wait progress of RAN1.

Proposal 5: The detailed resource configuration of the new A-IoT interface depends on RAN1.

2) Cell selection for UE reader

Based on the analyzation in 1), it can be seen that the RAN node in topology 2 should be able to allocate radio resource for the new A-IoT interface, which is an optional capability in Rel-19. We could not assume all RAN nodes supports this capability. If UE camps on the cells which do not support A-IoT topology 2, the UE reader could not perform A-IoT related progresses. Hence, for a RRC_IDLE/ RRC_INACTIVE UE supports A-IoT topology 2, when it operating as a normal UE, there is no limitation on its mobility management progress, it could camp on any cells, but when the UE operating as a reader, it should select a cell which supports A-IoT topology 2.

Proposal 6: The UE in RRC_IDLE/RRC_INACTIVE operating as a reader should camp on cell supports A-IoT topology 2.

With respect to how to indicate the cell supports A-IoT topology 2 to UE reader. One solution is to introduce an indication in system information, e.g., SIB1. The occurrence of the indication shows the cell supports A-IoT topology 2 related capabilities. The UE could determine whether the cell supports A-IoT topology 2 by reading system information.

Proposal 7: The RAN node should indicate to UE whether it supports A-IoT topology 2, e.g. introduce an indication in SIB1.

3) Authorization for A-IoT service

Before the UE performing A-IoT service, it should be authorized for A-IoT service by CN firstly. This issue is under discussion of SA3 TR 33.713 [4], shown as below.

## 5.2 Key Issue #2: Authorization for 5G Ambient IoT services ### 5.2.1 Key issue details In TR 23.700-13 [4], Key Issues #1 and #3 describe the issues on the system architecture and procedure to support 5G Ambient IoT services. In the Topology 2 as defined in TR 38.769 [2], the UE acting as the intermediate node is responsible for transferring the information between AIoT device and 5GS. If the authorization of intermediate node is not supported, the attacker can play the role of intermediate node and arbitrarily deny 5G AIoT service. Therefore, it is necessary to study how to authorize the UE for acting as the intermediate node.

SA3 has proposed two candidate solutions on this issue, i.e., solution #2, #3. According to these two solutions, the authorization can be finished by CN procedures and possible NAS signalling, no RAN2 impact is identified.

Proposal 8: The UE should be authorized for A-IoT service by CN before it operates as a reader. No RAN2 impact is identified so far.

4)How the RAN node recognize an UE reader

Apart from CN, the RAN node also needs to know the UE supports the capability of operating as a reader. For example, if the RAN node knows the UE’s radio capability on the new A-IoT interface, it could configure suitable radio resource for the UE. Hence, it is proposed that UE supports A-IoT reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI phase.

Proposal 9: UE reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI phase.

5) RRC state of UE

Another issue is whether the UE could be transited to other RRC states after it is authorized for A-IoT service in RRC_CONNECTED. If UE keeps RRC_CONNECTED state, the UE has NAS connection and PDU session (for UP solution), the A-IoT service can be performed normally. The discussion will be carried for RRC_IDLE and RRC_INACTIVE under these two cases:

- Case A: when A-IoT service request arrives from CN/AF

- Case B: when UE needs to forward the received device data towards CN/AF

| RRC state of UE | Case A: when A-IoT service request arrives from CN/AF | Case B: when UE needs to forward the received device data towards CN/AF |
| --- | --- | --- |
| RRC_INACTIVE | The RAN paging the UE into RRC_CONNECTED state or send the A-IoT service request to UE via SDT procedure. | The UE resumes to RRC_CONNECTED firstly or send the received device data via SDT procedure. |
| RRC_IDLE | The CN paging the UE into RRC_CONNECTED state, and establish NAS connection and PDU session (for UP solution). | The UE setup RRC connection, NAS connection and establish PDU session for A-IoT service (PDU session only for UP solution), and forward the received device data towards CN/AF. |

It is worth noting that, if UE is in RRC_IDLE state under case B, the CN will release the context of the A-IoT service for the UE. When UE re-establish NAS connection and/or PDU session, it is not sure that whether the CN/AF could recognize the response is related to which A-IoT service. This issue should be determined by SA2.

Even if this issue is solvable by SA2, this procedure will bring significant signalling overhead for the UE, which is not benefit for power saving. Hence, we propose the UE is not released to RRC_IDLE when performing operations in the new A-IoT interface. Considering the gNB could estimate the end of A-IoT service by some assistance information, e.g., the end time of the A-IoT resource configured for the UE reader, additional stage-3 impact may not need.

Proposal 10: The UE is not released to RRC_IDLE when performing operations in the new A-IoT interface. Additional stage-3 impact on Uu is not needed.

6) UE mobility in topology 2

Mobility management is a key feature for NR. For the UE operating as a reader in topology 2, the issue is whether we need to consider the mobility of UE and the impact it brings. If UE is moveable, the possible impacts are analyzed as following:

- If the UE reader moves when performing A-IoT service, transmission failure between UE reader and A-IoT device may occur, because the channel condition of the new A-IoT interface may change with the movement of UE. This may cause inventory/command failure of some devices.

- If the UE reader moves and handover/cell reselection occurs when performing A-IoT service, the radio resource configured by source cell should be invalid, and the UE needs to obtain new resource of the current serving cell. In this situation, how to continue the ongoing inventory/command procedure in the new A-IoT interface needs to be discussed.

- The UE reader selection mechanism needs to consider the factor that UE reader’s location is changeable. For example, if the CN/AF wants to inventory all of the A-IoT devices in area X, the NW needs to know which UE readers are in coverage of area X currently, so that it could send inventory request towards appropriate UE(s).

Observation 1: If UE reader is moveable, the following impacts need to be considered.

- UE reader moves may bring inventory/command failure of some A-IoT devices.

- If handover/cell reselection occurs when performing A-IoT service, the radio resource configured by source cell will be invalid in the target cell. How to ensure the A-IoT service continuity should be considered.

- The UE reader selection mechanism needs to consider the factor that UE reader’s location is changeable.

RAN2 could firstly discuss the feasibility of movable UE reader with considering the issues identified by observation 1.

Proposal 11: Study the feasibility of movable UE reader in topology 2.

7) Selection of UE reader for A-IoT service

In practical needs, the inventory/command request raised by AF may be in format of geographical area. How to select suitable UE reader(s) and which network element make the decision need to be discussed. Reference to the solutions proposed in TR 23700-13 [2], the related mechanisms on reader selection are summarized as the following three kinds.

- AF raised inventory/command request towards AMF/AIoTF, the request include area information optionally. Then AMF/A-IoTF selects reader(s) for this service accordingly. Reference to solution #4, #6, #11, #21, #33.

- AF raised inventory/command request towards AMF/AIoTF, the request include area information optionally. AMF/A-IoTF selects some candidate reader(s) accordingly and sends the candidate reader(s) list to RAN node. The reader who is used to perform this A-IoT service is finally determined by RAN node. Reference to solution #30.

- AF determines reader directly based on the location of UE reader. Reference to solution #22.

After analysing the issue with reference to these candidate solutions, it can be seen that the decision can be totally depends on CN/AF and optional RAN node. The only requirement on UE reader is the NW needs to know UE reader’s location, which can be supported by legacy mechanisms, e.g., GNSS location report or 3GPP positioning feature. No additional RAN2 impact is identified so far.

Proposal 12: How to selected suitable UE reader(s) to perform A-IoT service depends on other groups. No RAN2 impact is identified so far.

2.4	 Stage 2 procedure for A-IoT topology 2

Based on the discussion in section 2.1- 2.3, the following figure is given to describe the stage 2 procedure for A-IoT service in topology2.

Figure 3: Overall procedure for A-IoT topology 2

- The cells support A-IoT topology 2 broadcast an indication in SIB1. The UE supports A-IoT topology 2 who stay in RRC_IDLE or RRC_INACTIVE state determines whether the cell supports A-IoT topology 2 by reading its SIB1. The UE operating as a reader selects and camps on a cell supports A-IoT topology 2.

- If the UE has not been authorized for A-IoT service, before it performing A-IoT service, it should be authorized firstly. The detailed procedure depends on other groups.

- The UE reports its capability on A-IoT topology 2 towards the RAN node.

- When AF/CN raises an inventory/command request, the CN/AF and optional with RAN node selects target UE reader(s) to perform this A-IoT service. Then, the inventory/command request will be sent to the determined UE reader(s).

- The RAN node configures radio resource to UE reader for the new A-IoT interface. The RAN node determines the radio resource according to request from UE reader or based on some assistance information from CN/AF. Or the RAN node assigns a common resource pool in advance and UE reader selects resource from the resource pool without confirmation from RAN node.

- The UE reader performs inventory/command procedure in the new A-IoT interface using the radio resource allocated by RAN node in step 5. The UE reader receives response of inventory/command from A-IoT devices.

- The UE reader forwards the received inventory/command response towards CN/AF.

Proposal 13: Take the above procedure for A-IoT topology 2 as baseline.

3	Conclusion

In this contribution, A-IoT topology 2 is discussed with giving corresponding observations and proposals:

Network architecture

Proposal 1: In Topology 2, no need to decide whether the common reader function of the UE reader is part of UE or part of RAN.

Protocol stack and data transmission schemes

Proposal 2: RAN2 assumes the transmission between UE reader and RAN node for inventory/command service can be supported by legacy mechanism, i.e., via NAS message or PDU session, which does not bring stage-3 impact on Uu. RAN2 may come back when the network architecture and protocol stack are determined by SA2.

Key issues on management of UE reader in Uu interface

Proposal 3: The radio resource(s) of the new A-IoT interface should be allocated/ controlled by RAN node.

Proposal 4: Study solutions on configuring radio resource for the new A-IoT interface, e.g.,

- UE reader requests resource to RAN node, and the RAN node assign suitable resource for the UE reader;

- RAN node acquires assistance information from CN/AF and configures suitable resource for the UE reader;

- RAN node assigns a common resource pool in advance and UE reader selects resource from the resource pool without confirmation from RAN node.

Proposal 5: The detailed resource configuration of the new A-IoT interface depends on RAN1.

Proposal 6: The UE in RRC_IDLE/RRC_INACTIVE operating as a reader should camp on cell supports A-IoT topology 2.

Proposal 7: The RAN node should indicate to UE whether it supports A-IoT topology 2, e.g. introduce an indication in SIB1.

Proposal 8: The UE should be authorized for A-IoT service by CN before it operates as a reader. No RAN2 impact is identified so far.

Proposal 9: UE reports its capability on A-IoT topology 2 towards RAN node. The detailed solution can be discussed in WI phase.

Proposal 10: The UE is not released to RRC_IDLE when performing operations in the new A-IoT interface. Additional stage-3 impact on Uu is not needed.

Observation 1: If UE reader is moveable, the following impacts need to be considered.

- UE reader moves may bring inventory/command failure of some A-IoT devices.

- If handover/cell reselection occurs when performing A-IoT service, the radio resource configured by source cell will be invalid in the target cell. How to ensure the A-IoT service continuity should be considered.

- The UE reader selection mechanism needs to consider the factor that UE reader’s location is changeable.

Proposal 11: Study the feasibility of movable UE reader in topology 2.

Proposal 12: How to selected suitable UE reader(s) to perform A-IoT service depends on other groups. No RAN2 impact is identified so far.

Stage 2 procedure for A-IoT topology 2

Proposal 13: Take the above procedure for A-IoT topology 2 as baseline.

4	Reference

- RP-241270 Views on Ambient IoT architecture for Topology 2.

- 3GPP TR 23.700-13: "Study on Architecture support of Ambient power-enabled Internet of Things" V0.4.0.

- Chair's notes RAN1#116 9.4 R19 Ambient IoT.

- 3GPP TR 33.713: "Study on Security Aspect of Ambient IoT Services in 5G" V0.2.0.

- 3GPP TR 38.331: "Radio Resource Control (RRC) protocol specification".