3GPP TSG-RAN WG2 Meeting #127  	                            R2-2406621

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.5.3

Source: 	Sony

Title:	On-demand SIB1 for IDLE/INACTIVE UEs

Document for:	Discussion

# Introduction

In RAN2#126 meeting, the agreements on on-demand SIB1 for IDLE/INACTIVE are as following:

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

In this contribution, we discussed some leftover issues in order to conclude the study on on-demand SIB1 for NES.

# Discussion

2.1 WUS configuration from NES cell

In RAN1#117 meeting, it was agreed that Case 2 (UE gets UL WUS configuration from Cell A, and send on-demand SIB1 request to and get reply from NES cell) is feasible and further study Case 3 (UE gets UL WUS configuration, send on-demand SIB1 request to and get reply from Cell A). In both cases, the Cell A will provide the UL WUS configuration for UE to send on-demand SIB1 request, so it raises the question of whether NES cell should provide UL WUS configuration or not.

If the NES cell doesn’t provide UL WUS configuration, then for the UE who performs the cell selection procedure and identify the NES cell as candidate cell from radio link quality point of view, it will then find it out that this cell is a NES cell and won’t provide UL WUS configuration. As a result, the UE won’t be able to camp on the NES cell because it is impossible for this UE to send on-demand SIB1 request without knowing the UL WUS configuration. So as a consequence, if the NES cell doesn’t provide UL WUS configuration, it may impose implicit restriction on the UE to select the NES cell to camp on during initial cell selection.

If the NES cell provides its UL WUS configuration, then it is possible for the UE to camp on the NES cell normally. Then it is feasible for UE to send on-demand SIB1 request to and get reply from NES cell. This is beneficial to support Case 1 (standalone NES cell) in the future and won’t impose any cell selection restrictions for UE to select the NES cell. For the UE connected to any cell i.e. NES cell or Cell A, it can also acquire the WUS configuration in RRC release message. On the other hand, as the Cell A may also provide the UL WUS configuration for the same NES cell, the mismatched configuration between Cell A and NES cell may need to be addressed.

Proposal 1: Network can include WUS config in RRC release from any type of cell.

2.2 UL WUS configuration for on-demand SIB1 and on-demand SIB1 acquisition procedure

In RAN1#116 meeting, it was agreed that for study of UL WUS design, consider at least PRACH as a starting point. In Rel-15, on-demand SIB transmission is supported via IDLE/INACITVE UE to trigger a random access procedure. In MSG1 solution, a subset of the PRACH resources is associated with the requested SI, and network will reply a MAC subPDU with RAPID only for acknowledgement. In MSG3 solution, the requested SI is indicated in MSG3 and network will acknowledge the request in MSG4.

It is important that network can identify the request to transmit on-demand SIB1 from very beginning, in order to reduce the access delay for UE. And it was agreed in RAN2#125bis that UL WUS configuration includes at least RACH configuration. Therefore, the network should reserve PRACH resources that is associated with on-demand SIB1 transmission request.

Proposal 2: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

SIB1 is an essential SIB for UE and contains important information e.g., initial access information, cell barring information for UE to further access the network, therefore it is vital to specify the UE’s behaviour when UE fails to acquire the SIB1, in order to allow the UE to take immediate action to re-acquire the SIB1.

In RAN2#126 meeting, RAN2 discussed the UE behaviour after RACH failure of on-demand SIB1 request. In our opinion, the overall failure cases include during and after RACH to acquire the SIB1 are listed as follows:

- The RACH is completed unsuccessfully due to the pre-configured maximum number of preamble transmissions is reached.

- The RACH procedure is successful but UE fails to receive the SIB1 from network.

In either of the above listed cases, UE’s behaviour needs to be specified and RAN2 can discuss the options listed as follows.

Option 1: Follow “unable to acquire MIB” behaviour i.e. override IFRI bit.

“3>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs or intraFreqReselection2RxXR for 2Rx XR UEs, is set to allowed;”

Option 2: Follow “unable to acquire SIB1” behaviour and respect IFRI bit.

For both options, UE will initiate RACH for on-demand SIB1 if the newly selected cell is a NES cell and its on-demand SIB1 is not being broadcasted.

Proposal 3: RAN2 to discuss the options to further specify when UE fails to acquire the on-demand SIB1.

Option 1: Follow “unable to acquire MIB” behaviour i.e. override IFRI bit.

Option 2: Follow “unable to acquire SIB1” behaviour and respect IFRI bit.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: Network can include WUS config in RRC release from any type of cell.

Proposal 2: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

Proposal 3: RAN2 to discuss the options to further specify when UE fails to acquire the on-demand SIB1.

Option 1: Follow “unable to acquire MIB” behaviour i.e. override IFRI bit.

Option 2: Follow “unable to acquire SIB1” behaviour and respect IFRI bit.