3GPP TSG-RAN WG2 Meeting #127											R2-2407154

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.1.2.2

Source: 	Sharp

Title:  	Discussion on LCM for UE-sided model for Beam Management

Document for: 	Discussion

# Introduction

RAN2 #125bis meeting [1]:

Agreements

1.	Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.   “supported” means that the UE is capable of supporting the functionality and doesn’t mean neccesarily that the UE has the model available.  FFS what functionality refers to.

2.	Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

3.	Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

In RAN2 #126 following agreements were made [2]:

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signalling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case,  As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

During the RAN2 #126, a post meeting email discussion [POST126][032] [AI/ML PHY] LCM [3] was also initiated with the following intended outcome:

Phase 1: Discussions were expected to reach consensus on the definitions of key initial terms.

Phase 2: Reach common understanding of reactive/proactive framework for applicable functionality.

In this contribution, we will address the open issues surrounding proactive and reactive reporting for AI/ML-enabled air interface functionalities or use-cases, such as Beam Management. Our focus is to clarify the configuration details for AI/ML functionalities, including the mechanisms and timing for their communication between the network and the UE.

# Discussion:

It was agreed during the RAN2 #125bis and #126 meetings that supported functionalities (whether or not they have available models) are to be reported in the UE capability. Meanwhile, the existing UE Assistance Information (UAI) procedure will be utilized for applicable functionality reporting in the case of proactive reporting scenario.

During the email discussion [3], the applicable functionality and related inference configuration were discussed in the context of proactive and reactive reporting respectively.

From the discussions, it appears that most companies are inclined towards adopting a unified signalling approach for both proactive and reactive reporting. This preference indicates a consensus on the benefits of having a consistent framework for reporting applicable functionalities, regardless of whether the reporting is initiated by the UE in anticipation of network requests (proactive) or in response to specific network configurations (reactive).

Observation 1: There is a consensus towards a unified signaling approach for both proactive and reactive reporting of applicable functionalities.

Both proactive and reactive reporting signaling mechanisms have been discussed with a reference or example signaling framework as discussed below:

Figure 1: Reference Proactive & Reactive reporting Signalling framework [3]

For proactive reporting the steps performed include as per discussions in [3]:

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

Step 3:  Network configures UE that it is allowed to provide its applicable functionalities (in case of proactive reporting) whereas in case of reactive reporting, Network provides configurations (e.g., an initial inference configuration) to the UE and initiates UE to report its applicable functionalities.

Step 4: UE sends applicable functionalities to network upon change of applicable functionality/condition.

Step 5: Network sends inference configuration for the applicable functionalities to the UE.

Step 6: Start inference/monitoring based on network/UE activation/deactivation.

It is assumed that UAI is used by the UE to report the applicable functionality.

Considering Fig. 1, proposal 5 of the [3], discuss that Network configures full inference configuration to UE after applicable functionality reporting, if configuration of supported functionality is not provided in Step 3 (i.e. full configuration is provided in Step 5). As per the definitions in [3], Supported functionalities refer to functionalities that UE can indicate by using UE capability signaling. In the outcome of the post RAN2 126 email discussion [3], it is proposed in proposal 5 that if configuration of supported functionality is agreed to be provided in Step 3 (for reactive reporting), it is up to network implementation whether to provide an updated configuration or not.

In principle, the proposal 5 in [3] outlines the network’s approach to provide configurations (e.g., inference configuration) to the functionalities for the UE. According to Step 5:

Observation 2: The network will provide the configuration (e.g., inference config.) to the UE after applicable functionality reporting if the configuration for the supported functionality was not provided in Step 3 (Fig.1), ensuring that the UE receives the necessary information in Step 5 (Fig.1).

Observation 3: If the configuration for the supported functionality is provided in Step 3 (Fig.1), it is at the network's discretion whether to update the configuration later.

In RAN2 125bis meeting [1], the following was agreed:

- Support reactive reporting of UE-sided applicable functionality.  The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

In this context, the configuration content may encompass a variety of functionality-related parameters, such as CSI-ReportConfig parameters, AI/ML model life cycle management (LCM) parameters for monitoring and inference, as well as measurement and reporting parameters, among others.

Proposal 1: RAN2 to discuss and clarify content of the configuration(s) (e.g., inference configuration) of supported functionality and applicable functionality.

During the RAN2 125bis [1] meeting, it was also agreed that for proactive reporting, the UE would report its applicable AI/ML functionalities via the UAI message or LPP message. Thus, it is essential that the UE should be able to report multiple applicable functionalities simultaneously.

Proposal 2: The UE can report multiple applicable functionalities through various signaling methods such as RRC, UAI, or LPP.

An initial configuration provided in step 3, may help the UE to accurately determine applicable functionality(s) together with UE side conditions and Network side conditions if available. This ensures that the UE’s functionality reporting is precise and reflective of both the network's requirements and its own capabilities/environment.

Proposal 3: A part of configuration or an initial configuration can be provided in step 3 for the supported functionality(s) and then remaining or further configuration maybe provided in step 5 after applicability functionality reporting.

For a given set of supported or applicable functionalities, irrespective of if proactive or reactive approach is used, the network may deliver multiple configurations (such as initial or full inference configurations) for each functionality. This approach helps to minimize signaling overhead, as it prevents the network from repeatedly sending functionality configurations. The UE should utilize these factors to determine the applicability of a given functionality. The applicability of functionalities on the UE side may be influenced by UE-side additional conditions, NW-side additional conditions, and any initial configuration (such as inference configuration) provided by the network. Therefore, the UE should utilize these factors to determine the applicability of a given functionality. By providing comprehensive or initial configurations upfront, the network can efficiently manage and optimize signaling resources.

Proposal 4: The network can provide one or more configurations for a given functionality(s), including inference configurations, either in Step 3 or Step 5.

If multiple configurations for a supported functionality is provided in Step 3, it remains at the network's discretion whether to update the configuration at a later stage. The network may also select one of the configurations that is best suitable for the applicable functionality at an appropriate time.

The proactive and reactive approaches are slightly different in the way that in case of proactive approach, the UE reports its applicable functionalities to the network (as per network’s request or pre-configuration) based on changes in conditions. Then the network provides configuration information after receiving these reports with applicable functionalities.

In case of reactive approach, the UE only reports applicable functionalities in response to a specific network request. The network provides full configuration details up front or decides whether to update the configuration based on the initial response from the UE.

Both approaches have their pros and cons

Proactive approach May be preferred when the network is unsure about the specific inference needs at a given time. It avoids sending unnecessary configurations from the gNB for functionalities that might not be relevant. On the other hand, if the conditions for proactive reporting is not appropriately set, The UE may update and report information on applicable functionalities that the gNB may not require, which could result in unnecessary signaling overhead.

Reactive approach offers faster reaction times for the UE as it can start inferencing immediately upon receiving the RRCReconfiguration message. This can be beneficial for functionalities requiring real-time decisions. Reduced signaling as the UE only provide applicable functionality updates to the network when an event has occurred. However, as a drawback, in this case, more time for deployment, update and activation of functionalities maybe needed causing delays.

Given the minor differences between proactive and reactive approaches, it is feasible to design a unified signaling framework that accommodates both. This common framework would streamline the process, reduce complexity, and ensure consistency across both approaches. In general, there is a shared perspective among several companies favouring a unified signaling framework for both proactive and reactive reporting of applicable functionalities.

Proposal 5: RAN2 should define the procedure for configuring applicable functionalities, focusing on necessary steps, activation/deactivation, and configuration parameters, potentially using a unified approach for both proactive and reactive reporting.

In addition to this, there is a lack of clarity on how to differentiate and identify between supported and applicable functionalities and maintain consistency in training and inferencing of an applicable functionality. The associated ID, as defined by RAN1, ensures consistent network conditions for both training and inference. Acting as a proxy for these conditions, it protects sensitive network details and reduces signaling overhead. While it can represent network conditions, it's unclear if it fully captures all necessary information. It is up to RAN1to determine whether additional explicit signaling is needed. Thus, it is essential to discuss potential methods for identifying applicable functionalities. This could involve using identifiers such as (re-using associated IDs), functionality IDs, or configuration IDs provided by the network.

Proposal 6: RAN2 to discuss how to identify applicable functionality e.g., using associated ID, functionality ID or the configuration ID provided by the network.

# Conclusions

Based on the discussion given above, we have the following proposals:

Observation 1: There is a consensus towards a unified signaling approach for both proactive and reactive reporting of applicable functionalities.

Observation 2: The network will provide the configuration (e.g., inference config.) to the UE after applicable functionality reporting if the configuration for the supported functionality was not provided in Step 3 (Fig.1), ensuring that the UE receives the necessary information in Step 5 (Fig.1).

Observation 3: If the configuration for the supported functionality is provided in Step 3 (Fig.1), it is at the network's discretion whether to update the configuration later.

Proposal 1: RAN2 to discuss and clarify content of the configuration(s) (e.g., inference configuration) of supported functionality and applicable functionality.

Proposal 2: The UE can report multiple applicable functionalities through various signaling methods such as RRC, UAI, or LPP.

Proposal 3: A part of configuration or an initial configuration can be provided in step 3 for the supported functionality(s) and then remaining or further configuration maybe provided in step 5 after applicability functionality reporting.

Proposal 4: The network can provide one or more configurations for a given functionality(s), including inference configurations, either in Step 3 or Step 5.

Proposal 5: RAN2 should define the procedure for configuring applicable functionalities, focusing on necessary steps, activation/deactivation, and configuration parameters, potentially using a unified approach for both proactive and reactive reporting.

Proposal 6: RAN2 to discuss how to identify applicable functionality e.g., using associated ID, functionality ID or the configuration ID provided by the network.

# Reference

[1] Chair notes, 3GPP TSG-RAN WG2 Meeting #125bis Changsha, China, April 15th – 19th, 2024

[2] Chair notes, RAN2 126 3GPP TSG-RAN WG2 Meeting #126	Fukuoka, Japan May 20th – 24th, 2024

[3] R2-2406381 Post meeting email discussion [POST126][032] [AI/ML PHY] LCM