3GPP TSG-RAN WG2 Meeting #128                                                                   R2-2409537

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.8.4

Source:	vivo

Title:	Further Discussion on MBS Broadcast Service Area Provision

Document for:	Discussion and Decision

- Introduction

In the RAN2#127bis meeting, there was a warm discussion on which message (i.e. SIB vs. MCCH) to carry the MBS broadcast service area information. The achieved working assumptions and agreement are listed below for reference [1]:

RAN2#127bis Working assumption: WA A: We introduce 1) a service area list for MBS broadcast intended service area configuration in SIBXX, with each intended service area associated with an ID and 2) a mapping between broadcast service session and intended service area into MCCH (also working on enhancements to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service) WA B: We rely on a full MCCH based solution (i.e. the configuration of intended service areas is directly provided in MCCH, with no changes to SIB) (also working on enhancements to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service) RAN2#127bis Agreements: For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this contribution, we would like to further discuss the signalling desgin for providing the MBS broadcast service area information and the UE behavior on MCCH (re)acquisition.

- Discussion

  - 2.1	Network signalling for intended service area of broadcast

In this clause, we will discuss the differences between service area information provision via SIB vs. MCCH. It takes as baseline the discussion and proposals in the last meeting, where the following summary points are captured.

Summary of SIB-based solution [1][2][3]:

- SIBx (e.g. a new SIB) contains all the broadcast service area information;

- Service area ID is used to label a specific service area provided in SIBx;

- One or more than one service area ID(s) is included in MCCH to associate with a broadcast service;

- The service area ID can also be associated with the ETWS primary notification (pending on SA discussion);

- The UE may skip MCCH acquisition when UE is not located within any indicated broadcast service area.

Summary of MCCH-based solution [1][4][5]:

- One or more than one service area(s) is directly included in MCCH to associate with a broadcast service;

- The UE may skip MCCH acquisition based on UE implementation or new notification in MCCH-PDCCH.

Regarding the SIB-based solution, the main argument is that it facilitates the UE to skip (re)acquiring up-to-date MCCH if the UE is not in any intended service area. However, we fail to see the feasibility of this. This is because the NW may also provide broadcast sessions that are not associated with the service area (i.e. the Rel-17 case) within a serving cell via the existing broadcast MCCH. Based on the discussion in RAN3 [6], RAN3 now only considers the enhancement for letting the gNB know that the intended service area is smaller than the cell area. Thus, the network will not associate a service area with a broadcast service if that service is intended for the whole cell area (i.e. same network behavior as Rel-17). On the UE side, the UE anyway has to acquire the MCCH and check whether there exists a broadcast service (not associated with the service area) that the user is interested in. In other words, even though the UE knows it is not located in any service area from the area configuration in SIB, it still has to check the MCCH for the potential broadcast PTM configuration addition/update of the service that is not associated with the service area.

Furthermore, there will be some drawbacks if using SIB to carry area information. Specifically, SIB is updated in the corresponding SI window of the next available modification period for BCCH while the broadcast MCCH is updated in the next available MCCH modification period, which is generally configured with a different value than the legacy modification period, as shown in Figure 1. As a result, once the network would like to change the service area configuration and association, it has to wait for MCCH/SI update until broadcast MTCH scheduling in the manner shown in Figure 2. This is to guarantee the UE always has obtained the latest SIB for service area information and MCCH for the association between the broadcast session and the service area, at the cost of network scheduling flexibility and service delivery latency. Otherwise, if the NW schedules the MTCH before the SIBx/MCCH update, the UE may use the out-of-date area information to receive the broadcast service, which is not for the UEs outside of the intended service area.

Last but not least, the SIB size cannot exceed 2976 bits (i.e. the maximum size a SIB can take by the physical layer). If the area configuration size is higher than that threshold, there is no way to carry it in SIB.

Figure 1: misalignment between MCCH update and SIB update

Figure 2: MTCH scheduling after updated SIB/MCCH

For the MCCH-based solution, the intended service area and the broadcast PTM configuration of a session are always carried within the same PDSCH. Thus, the UE can always acquire the latest area information and association/PTM configuration of a broadcast session at the same time, avoiding any configuration misalignment. And no other main drawbacks are found. Further, we can use the nonCriticalExtension field in MCCH to carry the service area information configuration. Also using the area ID(s) to associate the broadcast session and the actual service area, especially in the case where multiple broadcast services have the same intended area, saving the siganlling overhead. Specifically, we can introduce a session-area list separately from the service area information configuration, For that list, the first entry (including one or more than one area ID(s)) is associated with the first entry of the MBS session information list. If the association is not needed, then the network might not include an entry.

In conclusion, the main pros and cons of both solutions are categorized below,

Table 1: SIB vs. MCCH

|  | SIB-based solution | MCCH-based solution |
| --- | --- | --- |
| Pros | - The UE may be able to skip MCCH acquisition when it is outside of all areas and all the sessions are linked with the service area. | - Siganlling efficiency and simplicity; - Capacity size is not limited. |
| Cons | - More siganlling overhead; - Misalignment between area updated in SIB and area association configuration in MCCH; - Capacity size limited by 2976 bits. | - Power consumption on unnecessary MCCH acquisition in a specific case. |

As per the above analysis, we suggest the MCCH-based solution, considering the cons of the SIB-based solution while the benefit brought is quite limited and uncertain. Thus, we propose,

Proposal 1: MCCH (i.e. MBSBroadcastConfiguration) is used to include a list of intended service area and related ID by non-critical extension way.

Proposal 2: A service area ID list is used for the association between the broadcast session and the intended service area.

An example of MCCH ASN.1-level siganlling modification is given below for reference.

MBSBroadcastConfiguration message

-- ASN1START

-- TAG-MBSBROADCASTCONFIGURATION-START

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17                                              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  MBSBroadcastConfiguration-v19xy-IEs                                                           OPTIONAL

}

MBSBroadcastConfiguration-v19xy-IEs ::= SEQUENCE {

mbs-ServiceAreaInfoList-r19         MBS-ServiceAreaInfoList-r19         OPTIONAL,   -- Need R

mbs-SessionAreaList-r19      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionAreaIdList-r19                                         OPTIONAL,   -- Need R

nonCriticalExtension             SEQUENCE {}             OPTIONAL

}

MBS-SessionAreaIdList-r19                   SEQUENCE (SIZE (1..maxSession-AreaInfo-r19)) OF MBS-AreaId-r19

MBS-ServiceAreaInfoList-r19 ::=      SEQUENCE (SIZE (1..maxNrofMBSArea-r19) OF MBS-ServiceAreaInfo-r19

MBS-ServiceAreaInfo-r19 ::=       SEQUENCE {

mbs-AreaId-r19                 MBS-AreaId-r19,

referenceLocation-r19       	ReferenceLocation-r17,

distanceRadius-r19          	INTEGER(0..65535)

}

MBS-AreaId-r19 ::=  INTEGER (1.. maxNrofMBSArea-r19)

-- TAG-MBSBROADCASTCONFIGURATION-STOP

-- ASN1STOP

  - 2.2	UE behavior on MCCH (re)acquisition

With the network siganlling as mentioned above, UE can interpret the intended service area for the MBS broadcast session via MCCH (re)acquisition. That is when the UE becomes interested in MBS broadcast services, it will apply the MCCH information acquisition procedure. For the Rel-19 NTN case, we think the same behavior can be reused, i.e., UE acquires the MCCH when entering the cell providing SIB20 after becoming interested in MBS broadcast services and re-acquires the MCCH upon the MCCH information has changed.

In the last meeting, there raised a view that we should consider MCCH skipping even if the MCCH-based solution is used. For example, we may add a new notification bit in the MCCH-PDCCH to indicate whether the MCCH modification is changed due to the change of service area configuration/association. With that, once that indication is siganlled (i.e. the service area configuration/association in MCCH is not changed compared with the previous MCCH), the UE, outside of all the service areas obtained from the previous MCCH, can skip decoding the MCCH-PDSCH for the sake of power saving. This solution sounds reasonable. However, to fulfill this goal, MCCH-PDCCH DCI format needs to be modified, leading to RAN1 impacts. And the power-saving benefit is supposed to be very limited considering the both PTM configuration and service area configuration/association generally are quite static during the satellite service duration. Alternatively, we might leave it to pure UE implementation to skip the MCCH reacquisition. For another example, the UE could use stored service area configuration from the previously received MCCH. And then it may skip re-acquiring the up-to-data MCCH due to PTM configuration changes of the ongoing sessions (i.e. the LSB of MCCH change notification field is set to ‘1’) when it is outside the service area, based on the hypothesis that the service area configuration/association is static in fixed satellite cell. Therefore, we propose,

Proposal 3: If the MCCH-based solution is adopted, RAN2 considers the following options,

Opt 1: Reuse the Rel-17 MCCH information (re)acquisition procedure, i.e., the UE attempts to acquire MCCH upon receiving a change notification for MCCH;

Opt 2: It is up to UE implementation to use the service area info acquired in the received MCCH to avoid re-acquiring the MCCH when the UE is outside the service area.

  - 2.3	ETWS aspect

For the potential geo-fencing information provision for ETWS, we think the corresponding configuration should be included in SIB6. In our understanding, on one hand, the warning area generally is quite large compared to the intended service area of the broadcast session. Then, it may be very hard to reuse the area of broadcast for ETWS, and vice versa. On the other hand, if we use 2 SIBs to separately carry the ETWS primary notification and its corresponding intended area, it will cause more signalling overhead than just directly including the area within the SIB6, potentially also increasing the configuration delivery latency. Thus, for simplicity and efficiency, we propose,

Proposal 4: Waring area information for ETWS primary notification is provided in SIB6, if needed.

Besides, The example of SIB6 ASN.1 signalling modification is given below for reference.

-- ASN1START

-- TAG-SIB6-START

SIB6 ::=                            SEQUENCE {

messageIdentifier                   BIT STRING (SIZE (16)),

serialNumber                        BIT STRING (SIZE (16)),

warningType                         OCTET STRING (SIZE (2)),

lateNonCriticalExtension            OCTET STRING                                OPTIONAL,

...

[[

warningAreaInfoList-r19 ::=      SEQUENCE (SIZE (1..maxNrofWarningArea-r19) OF WarningAreaInfo-r19

]]

}

WarningAreaInfo-r19 ::=       SEQUENCE {

referenceLocation-r19       	ReferenceLocation-r17,

distanceRadius-r19          	INTEGER(0..65535)

}

-- TAG-SIB6-STOP

-- ASN1STOP

3 Conclusion

In this contribution, we have further discussed network signalling and UE behaviour on MCCH (re)acquisition when MBS broadcast service is provided within the intended service area. And we make the following proposals,

Network signalling for intended service area of broadcast:

Proposal 1: MCCH (i.e. MBSBroadcastConfiguration) is used to include a list of intended service area and related ID by non-critical extension way.

Proposal 2: A service area ID list is used for the association between the broadcast session and the intended service area.

UE behavior on MCCH (re)acquisition:

Proposal 3: If the MCCH-based solution is adopted, RAN2 considers the following options,

Opt 1: Reuse the Rel-17 MCCH information (re)acquisition procedure, i.e., the UE attempts to acquire MCCH upon receiving a change notification for MCCH;

Opt 2: It is up to UE implementation to use the service area info acquired in the received MCCH to avoid re-acquiring the MCCH when the UE is outside the service area.

ETWS aspect:

Proposal 4: Waring area information for ETWS primary notification is provided in SIB6, if needed.

4 Reference

[1] R2-2409502, RAN2#127bis Meeting Report.

[2] R2-2408592, Intended broadcast service area provision over NTN, Apple.

[3] R2-2408656, Consideration on broadcast service enhancements, ZTE Corporation, Sanechips.

[4] R2-2408016, Further Discussion on MBS Broadcast Provision in NTN	, vivo.

[5] R2-2407961, Discussion on support of broadcast service in NR NTN, CATT.

[6] R3-247002, RAN3#125-bis Meeting Report.