3GPP TSG-RAN WG2 Meeting #128	R2-2409555

Orlando, USA, Nov 18-22, 2024

Agenda item:	8.7.3

Source:	Qualcomm Incorporated

Title:	Discussion on measurement gap enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss two issues related to measurement gap (MG) cancelation:

- Whether MG cancelation affects UE’s DRX operation;

- Besides MG cancellation enabled by DCI, whether there are other options.

# Discussion

## Measurement gap and DRX

In the last RAN2 meeting, companies had a brief discussion on whether MG cancelation has any impact on DRX, and the following agreement was made:

RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear.

In the following, we discuss what UE behavior should be when a MG is canceled, considering whether the MG is within or outside the DRX active time.

When a MG within DRX active time is cancelled, most likely it is because network wants to prioritize data exchange (on either DL or UL) over RRM measurement in those slots. Therefore, UE should continue monitoring PDCCH in those slots and perform data exchange when scheduled. In addition, since in legacy DRX timers are not affected by the presence of a MG, cancelation of a MG does not change DRX timers or UE’s DRX state.

When a MG outside DRX active time is canceled, it is debatable whether UE should monitor PDCCH within the canceled MG. Before we analyze what the preferred UE behavior should be in that case, we should estimate how likely it may occur.

Video/audio traffic in XR applications typically have regular, well-defined arrival patterns and hence can be well aligned with DRX cycles. Even with jitters, start of data bursts center around the start of a DRX cycle. It is unlikely that many video/audio PDUs arrive within DRX inactive time between two DRX cycles. Other types of traffic such as haptic or control messages may be less bursty and arrive between DRX cycles. But they can be supported by configured grants and hence are not affected much by DRX states.

Moreover, with XR traffic, DRX cycle typically is short (e.g. 16 msec). An MG, which is 4~6 msec long, can occupy a considerable part of a DRX cycle. Consequently, DRX on duration very likely starts shortly after the end of the MG. Since that data is the beginning of a burst, its PDU set would experience only a short delay (equal to the length of the MG) even if the MG is not cancelled. Therefore, even if, for some reasons, network anticipate data arrivals with a MG during DRX inactive time, we do not believe the data is urgent enough to warrant a cancelation of the MG and justify the potential impact on RRM measurement.

Observation 1.	There is limited use case for canceling a measurement gap within DRX inactive time.

Now suppose that, for whatever reasons, network cancels a MG within DRX inactive time. On one hand, one may argue that the reason for the cancelation is to have data exchange. Hence UE should monitor PDCCH within the canceled MG. On the other hand, if network anticipates upcoming DL data, it has two options:

- Alt 1. The network first keeps UE in DRX active time (e.g. by restarting DRX inactivity timer). This step makes sense because the network is anticipating new data. It then cancels the MG if it desires so. As analysed earlier, since the MG is within DRX active time, cancelation of the MG does not affect UE’s DRX state.

- Alt 2. The network chooses not to keep UE in DRX active time (i.e. let DRX inactivity timer expires before the MG). In this case, we think there is limited impact if UE does not monitor PDCCH in the cancelled MG. As we analysed earlier, even if UE does not monitor PDCCH within the cancelled MG, the data would only experience only a short delay compared to its PDB. In exchange, what UE gets are 1. Avoid more power consumption; 2. a simpler implementation, as MG cancelation and DRX are two independent operations.

One may see that either option can work while not introduce much impact on UE’s power and delay performance. And they can keep MG cancelation independent from UE’s DRX operation. That simplifies UE implementation.

Observation 2.	UE can have simpler implementation and avoid extra power consumption by not monitoring PDCCH within a canceled measurement gap in DRX inactive time.

Therefore, we propose that

Proposal 1.	Cancelation of a measurement gap does not change UE’s DRX state, regardless of whether the gap occurs within or outside UE’s DRX active time.

The analysis above can be extended to other UE power saving states too, such as PDCCH skipping, dormant BWP, and dormant Scell. For example, suppose a PDCCH skipping duration overlaps with a MG. Then cancelation of the MG does not terminate the PDCCH skipping. Similarly, if a carrier is in dormant BWP or is a dormant SCell, then cancelation of an MG should switch UE out of those dormant states.

Proposal 2.	Power saving states such as PDCCH skipping, dormant BWP or dormant SCell are not affected by cancelation of a measurement gap that overlaps with them.

## Other options for measurement gap cancellation

Currently, the only solution for canceling a measurement gap is by DCI. That requires network to have full knowledge of UE’s mobility status, delay status of UE’s data, etc to cancel a MG. Network can obtain UE’s mobility status from measurement reports. However, network may not have knowledge of UE’s need for urgent uplink transmissions.

Observation 3.	Network may not be aware of UE’s need for urgent uplink transmission and cancel a measurement gap for it in time.

In some cases, those uplink transmissions warrant higher priority over measurement gaps, e.g. when UE has data who remaining time is very low, or when UE has a pending DSR triggered by a high-priority logical channel, etc. In those cases, if cancelation of a MG does not significantly impact UE’s mobility performance, it makes sense for UE to autonomously cancel the MG and perform the uplink transmission instead.

To enable autonomous cancellation of MGs in those cases, we think two aspects need to be studied:

- Which uplink transmissions can be considered urgent enough for UE to autonomously cancel an MG;

- Under what mobility conditions a UE is allowed to autonomously cancel an MG for urgent UL transmissions.

Proposal 3. 	Under proper conditions, UE is allowed to autonomously cancel a measurement gap for urgent UL transmissions. FFS which UL transmissions can be considered and what the conditions are.

Another option is to use RRC to pre-configure a set of MGs that can be canceled.

Since UE performs measurements on DL reference signals during MGs, time locations of MGs follow those of DL reference signals, which are configured per-cell and periodic (e.g. once every 20 msec). XR traffic is also periodic but has non-integer periodicity. As a result, there are always some occasions in a MG configuration overlapping with data bursts in XR traffic, regardless of how DL reference signals are configured (see Figure 1 for an example). On the other hand, since both MGs occasions and XR traffic are periodic, the time locations where they may overlap, at least those overlap with the start of data bursts, with jitter also taken into account, are also periodic and can be pre-determined once their periodicities and relative start offsets are known.

Observation 4. 	Measurement gaps that overlap with the start of data bursts are periodic and can be identified deterministically based on their periodicities and relative start offsets.

To mitigate the impact of MGs on XR traffic, among the occasions where MGs overlap with data, network can choose a subset of them and allow them to be overridden by data. This selection can be periodic, e.g. once every N overlapping MG occasions. And this selection can be relatively static, because they can be determined based on the periodicities of a MG configuration and XR traffic, and their relative start offset. It may need to be adjusted only when UE’s mobility status changes and so on.

Given the periodic and relatively static nature of these occasions, it is more efficient to indicate them to UE by RRC configuration instead of dynamic signaling methods such as DCI. That not only reduces signaling overhead, but also greatly simplifies UE implementation, because pre-determined occasions give UE more time to prepare for RF tuning and related procedures.

Based on the above analysis, we think RAN2 should discuss the option of using RRC configuration to indicate whether a measurement gap can be overridden by data.

Proposal 4. 	Network can RRC configure a set of measurement gaps that can be overridden by data.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1.	There is limited use case for canceling a measurement gap within DRX inactive time.

Observation 2.	UE can have simpler implementation and avoid extra power consumption by not monitoring PDCCH during a canceled measurement gap in DRX inactive time.

Proposal 1.	Cancelation of a measurement gap does not change UE’s DRX state, regardless of whether the gap occurs within or outside UE’s DRX active time.

Proposal 2.	Power saving states such as PDCCH skipping, dormant BWP or dormant SCell are not affected by cancelation of a measurement gap that overlaps with them.

Observation 3.	Network may not be aware of UE’s need for urgent uplink transmission and cancel a measurement gap for it in time.

Proposal 3. 	Under proper conditions, UE is allowed to autonomously cancel a measurement gap for urgent UL transmissions. FFS which UL transmissions can be considered and what the conditions are.

Observation 4. 	Measurement gaps that overlap with the start of data bursts are periodic and can be identified deterministically based on their periodicities and relative start offsets.

Proposal 4. 	Network can RRC configure a set of measurement gaps that can be overridden by data.