3GPP TSG-RAN WG2 Meeting #128	R2-2409558

Orlando, USA, Nov 18-22, 2024

Agenda item:	8.7.5

Source:	Qualcomm Incorporated

Title:	Discussion on RLC enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss enhancements for timely RLC retransmissions and avoiding unnecessary retransmissions. For both issues, we analyze pros and cons of different options and then propose way-forwards.

# Discussion

## Timely retransmission

At RAN2#127-bis meeting, the following agreements on timely retransmission were made:

- RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay.

For autonomous retransmission, there are several options proposed in [1]:

- after the remaining time of the PDU has dropped below a configured threshold; or

- after the PDU has failed a configured number of HARQ transmissions; or

- if the PDU is in the RLC retransmission buffer and there are spare PUSCH resources available after the LCP procedure.

Option #1 is useful because it gives the transmitter one more chance to send the PDU because its PDB depletes. If the threshold is configured to be compatible with the one-way delay from the transmitter to the receiver, then in the worst case, it creates only one extra transmission. Hence the impact on capacity is limited.

In typical network implementations, a HARQ process is terminated after a MAC PDU has gone through several failed transmissions. The remaining of the delivery is handled by RLC retransmission. Option #2 is designed based on this observation, i.e. if network lets UE know about its max number of HARQ retransmission via configuration of the trigger threshold, UE can timely trigger RLC retransmission without waiting for status reports. Even if in some network implementation maximum number of HARQ retransmissions varies across HARQ processes, network can choose a threshold such that it covers a large percentage of HARQ processes. Moreover, since HARQ failure typically has very low probability (e.g. 1% after 2 HARQ transmissions), this scheme introduces only a very small number of retransmissions. More importantly, the amount of this increase is full controllable by network through configuration the trigger threshold.

Option #3 uses only spare resources in a TB to retransmit RLC PDUs. Therefore, there is no impact on capacity at all.

Observation 1.	Autonomous retransmission, if configured properly, has no or very limited impact on capacity.

Based on the above analysis, we propose that RAN2 should focus its discussion on autonomous retransmission, e.g. the three options discussed above.

Proposal 1.	UE autonomously triggers retransmission of a RLC PDU when one of the following conditions are met:

- after the remaining time of a PDU has dropped below a configured threshold; or

- after a PDU has failed a configured number of HARQ transmissions; or

- if a PDU is in the RLC retransmission buffer and there are spare PUSCH resources available after the LCP procedure.

Next, we discuss polling enhancements.

First, we notice that currently very frequent polls can be configured. For example, the minimum pollPDU is 4, and pollByte is 1KB. With the typically high data rate of XR applications, these thresholds correspond to very short durations already. For example, if the data rate is 2 Mbps, and pollByte is 1 KB, then polls are triggered once every 4 msec.

We think these values are already very close to the shortest durations that may be configured on FR1 carriers, without increasing the risk of the resource waste and false RLF. For example, the round-trip time of a DL HARQ transmission on a FR1 carrier with 30KHz SCS takes 2 msec and the typical number of HARQ transmissions is 3. Hence one poll every 4 msec is already sufficiently short in pragmatic deployments.

In addition, the above analysis also gives us an upper bound on how much delay reduction enhanced polling can achieve, which is ~4 msec. Compared to the typical RLC round-trip time, this reduction is marginal at its best.

Observation 2.	Network currently can configure very frequent polls. Further enhancements only result in marginal reductions in RLC round-trip time.

If one breaks the total RLC round-trip time into individual steps in each protocol layer, it is not difficult to see that wait time before sending polls and status reports only contribute a small portion of the total delay, when they are properly configured. The longest component in the round-trip time is the wait time for the receiver to send back a status report (10~20ms), which it includes at least the delay for UE to get UL grant and UL HARQ transmission. More frequent polls do not eliminate or reduce that delay. Therefore, RAN2 should focus more on autonomous retransmission and deprioritize study on poll enhancements.

Proposal 2a. 	Deprioritize polling enhancements in the study for timely retransmission.

If RAN2 agree to introduce some polling enhancements, then we think it is important that additional polls are only triggered by selected PDUs, to avoid excessive status reports. A good selection criterion can be the remaining time of SDUs dropping below a configured threshold. In addition, the prohibit timer for status reports needs to be enhanced too, because otherwise the legacy status report prohibit timer will prevent receiver from sending back status reports in response to the new polls in a timely manner. Possible options may include that such polls are subject to a different (smaller) prohibit timer.

Proposal 2b. 	If RAN2 agree to introduce polling enhancements, new polling trigger should be based on SDU’s remaining time. In addition, such polls are subject to a different (smaller) status report prohibit timer.

## Avoid unnecessary retransmissions

At RAN2#127bis meeting, the following agreements were made on the options for avoiding unnecessary retransmissions:

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In the following, we discuss the remaining issues in the combined approach.

The first issue is related to transmitter behavior for “stopping transmission of an outdated SDU”. There can be different options for determining outdated SDUs and stopping their transmissions. For example, we may use expiry of PDCP discard timer to trigger the termination. Or network can configure the legacy maxReTxThreshold based on the PDB of SDUs. Either option requires some changes to the legacy procedure:

- The first option requires removal of SDUs from RLC retransmission buffer upon the indication from PDCP. It is a new procedure and has impact on the UE implementation of the RLC user-plane procedures. Since the user-plane procedures need to support high-throughput and operate on a very short timeline, new procedures such as PDU discard may require significant changes to the data path and hence are not desired.

- In the second option, we need to avoid transmitter triggering unnecessary RLF when number of retransmissions reaches maxReTxThreshold. Hence a new trigger for RLF needs to be defined. For example, the new trigger can be that RLF is triggered after N or more SDUs have reached maxReTxThreshold in the last T msec. The downside of this option is that the transmitter may not be able to immediately stop retransmission of an SDU once it becomes outdated. But its advantage is that it does not have any impact on the legacy user-plane procedures at RLC transmitter.

Observation 3.	Terminating outdated RLC SDUs based on indication from PDCP may have significant impact on UE implementation of RLC user-plane procedures and therefore is not desired.

Comparing the required changes, we can see that the second option has less impact on the legacy user-plane procedures at RLC transmitter. Although it may not allow a transmitter to immediately stop retransmission of an SDU once it becomes outdated, we do not believe that delay is a critical performance metric and hence can yield to the benefit of having minimal impact on the legacy procedures. Therefore, the second option is more preferred.

Proposal 3.	RLC transmitter stops retransmission of an outdated SDU after maxReTxThreshold is reached. Network can configure a new RLF trigger for such a DRB, e.g. RLF is triggered after N or more SDUs have reached maxReTxThreshold in the last T msec.

Regarding feedback from a RLC receiver to the transmitter about abandoned SDUs, we believe the existing status report can be reused. More specifically, the receiver can simply send an ACK for an abandoned SDU to help the transmitter advance the retransmission window. Since in the combined approach, transmitter has its own mean to determine whether an SDU is outdated or not, no extra indication from the receiver is needed.

Proposal 4.	It is confirmed that a RLC receiver can use the existing status report to inform the transmitter of abandoned SDUs, without the need of any additional indications.

Regarding the FFS on whether any special handling of control PDU is needed, we do not believe any special enhancements are needed. In our understanding, control PDUs do not have sequence numbers. Therefore, they will not trigger the “termination timer” at the RLC receiver and hence will not be discarded before they are successfully received.

Proposal 5.	No special handling of RLC control PDUs is needed.

Regarding whether some indication needs to be sent from Tx to Rx, we do not believe any such indication is needed. One of the advantages of the combined approach is that transmitter and receiver determine whether a SDU is outdated in a distributed way. The only coordination needed between them is the “fake ACK” sent in status report, which helps advance the transmission window at the transmitter. No input from the transmitter is needed by receiver’s operation.

Proposal 6.	No additional indication regarding outdated SDUs needs to be sent from transmitter to receiver.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Timely retransmission

Observation 1.	Autonomous retransmission, if configured properly, has no or very limited impact on capacity.

Proposal 1.	UE autonomously triggers retransmission of a RLC PDU when one of the following conditions are met:

- after the remaining time of a PDU has dropped below a configured threshold; or

- after a PDU has failed a configured number of HARQ transmissions; or

- if a PDU is in the RLC retransmission buffer and there are spare PUSCH resources available after the LCP procedure.

Observation 2.	Terminating outdated RLC SDUs based on indication from PDCP may have significant impact on UE implementation of RLC user-plane procedures and therefore is not desired.

Proposal 2a. 	Deprioritize polling enhancements in the study for timely retransmission.

Proposal 2b. 	If RAN2 agree to introduce polling enhancements, new polling trigger should be based on SDU’s remaining time. In addition, such polls are subject to a different (smaller) status report prohibit timer.

Avoid unnecessary retransmissions

Observation 3.	Termination of outdated SDUs based on expiry of PDCP discard timer requires new signaling and has impact on UE implementation of the RLC user-plane procedures.

Proposal 3.	RLC transmitter stops retransmission of an outdated SDU after maxReTxThreshold is reached. Network can configure a new RLF trigger for such a DRB, e.g. RLF is triggered after N or more SDUs have reached maxReTxThreshold in the last T msec.

Proposal 4.	It is confirmed that a RLC receiver can use the existing status report to inform the transmitter of abandoned SDUs, without the need of any additional indications.

Proposal 5.	No special handling of RLC control PDUs is needed.

Proposal 6.	No additional indication regarding outdated SDUs needs to be sent from transmitter to receiver.

# References

- R2-2402390, Discussion on RLC enhancements, Qualcomm Incorporated, April 2024.