3GPP TSG-RAN WG2 Meeting #128	R2-2409559

Orlando, USA, Nov 18-22, 2024

Agenda item:	8.7.6

Source:	Qualcomm Incorporated

Title:	Discussion on signaling enhancements for XR rate control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

At the last RAN2 meeting, the following agreement on the granularity of rate indication was made:

FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc.

In this paper, we first discuss whether the granularity of rate indication and then discuss other enhancements that can be useful for XR applications.

# Discussion

At the last RAN2 meeting, there were different views on whether the granularity of bit rate indication should be per QoS flow or per logical channel.

Justifications by the proponents for per QoS flow indication is that in some deployments, network may multiplex different QoS flows (e.g. video and audio) in a single DRB. As the goal of this study is to provide a better alternative to the L4S mechanism (which provides only binary indication of congestion), the bit rate indication for XR application should be signaled at the level of QoS flow.

We first observe that congestion (in a more generalized sense, available bandwidth) is global and not limited to a specific class of traffic, because congestion is caused by limited amount of radio resources or poor radio conditions. That is indifferent to any flows. However, different classes of traffic may not be impacted by congestion at the same time, if they have different priorities. For example, because high-priority traffic gets bandwidth first, it is the last one affected by congestion.

Based on this observation, then the key question to answer is whether network or application is in a better position to decide how to allocate available bandwidth among different flows. If network is in a better position to make this decision, then it should recommend bit rates per QoS flow to UE. Otherwise, network only needs to provide UE with the total available bandwidth for a DRB and leaves it to application to decide how to allocate available bandwidth among QoS flows in the DRB.

Observation 1.	Whether bit rate indication should be per QoS flow or per logical channel depends on whether network or application is in a better position to decide how to allocate available bandwidth among QoS flows in a DRB.

In general, we believe applications are in a better position to make that decision, because applications usually have more information about flows beyond their QoS attributes. For example, an application knows better how to prioritize bandwidth allocation among different QoS flows, especially when this priority may not be static. A case in point is that the relative priority between audio and video can change depending on scene/context. For example, audio typically has higher priority when human is in the loop. But video can be more important when the interaction is with a server. Only application itself has the necessary information or operation state to make this decision.

Observation 2.	Prioritizing bandwidth allocation among different QoS flows can depend on factors beyond just QoS attributes. Applications generally have better access to these factors and thus are in a better position than network to make such decisions.

Based on the above analysis, we think it makes more sense to have network estimate the available bandwidth and application decide how to allocate the available bandwidth among different QoS flows. Therefore, network only needs to provide recommended bit rate for a logical channel (DRB) in Recommended Bit Rate MAC CE.

Proposal 1.	Keep the granularity of bit rate indication per logical channel as in legacy.

One issue raised by a few companies (e.g. [1][2]) is how network identifies which flows or logical channels are subject to rate adaptation. We think the existing recommended bit rate procedure already has a built-in mechanism for that. More specifically, UE can initiate a bit rate query or a preferred bit rate indication for a logical channel. This message can serve as an indication that the indicated logical channel is subject to rate adaptation. Network does not need to send recommended bit rate for a logical channel before it receives a Recommended Bit Rate MAC CE from the UE for that logical channel.

Proposal 2.	Network determines the applicability of rate adaptation for a logical channel after it receives a Recommended Bit Rate MAC CE from the UE for that logical channel.

One of the special characteristics of XR applications is that it generates multiple flows with different QoS requirements, e.g. video, audio, haptic, control, etc. When an application performs rate adaptation, it typically adjusts the rates of individual flows simultaneously. The legacy Recommended Bit Rate MAC CE includes indication for only one logical channel. To better support simultaneous rate adaptation of multiple traffic flows in XR applications, the Recommended Bit Rate MAC CE can be extended to include bit rates of multiple logical channels or multiple QoS flows.

Proposal 3. 	Enhance the Recommended Bit Rate MAC CE to include bit rates of multiple logical channels.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Observation 1.	Whether bit rate indication should be per QoS flow or per logical channel depends on whether network or application is in a better position to decide how to allocate available bandwidth among QoS flows in a DRB.

Observation 2.	Prioritizing bandwidth allocation among different QoS flows can depend on factors beyond just QoS attributes. Applications generally have better access to these factors and thus are in a better position than network to make such decisions.

Proposal 1.	Keep the granularity of bit rate indication per logical channel as in legacy.

Proposal 2. 	Enhance the Recommended Bit Rate MAC CE to include bit rates of multiple logical channels.

Proposal 3.	Network determines the applicability of rate adaptation for a logical channel after it receives a Recommended Bit Rate MAC CE from the UE for that logical channel.

# Reference

- R2-2408884, On XR rate control, Ericsson, Oct 2024.

- R2-2409077, XR rate control, Nokia, Nokia Shanghai Bell, Oct 2024.