3GPP TSG-RAN WG2 Meeting #128	R2-2409561

Orlando, USA, Nov 18-22, 2024

Agenda item:	8.7.1

Source:	Qualcomm Incorporated, China Telecom, Huawei, HiSilicon, Lenovo, Nokia, Nokia Shanghai Bell, Xiaomi

Title:	Discussion on AL-FEC awareness at RAN

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In S2-2410999, SA2 provides RAN2 with an update on their normative work on AL-FEC awareness at NG-RAN. In addition, they request RAN2 to provide answers on the feasibility of using content ratio for discarding DL PDUs during congestion. In this paper, we discuss how to answer those questions from SA2.

# Discussion

In the first question, SA2 asks RAN2 to provide feedback on the feasibility of using content ratio information for discarding DL PDUs during congestion for RLC AM, based on the principles used in their normative work.

With RLC AM, NG-RAN as a transmitter is able to reliably determine which PDUs have been successfully received based on status reports from its ARQ operation. In a possible implementation, this information allows NG-RAN to identify PDU sets whose original content can be correctly recovered by the UE, i.e. whether a sufficient number of PDUs in a PDU set have been successfully received, based on the content ratio of the PDU set. Among these PDU sets, NG-RAN then can select PDUs to discard, ensuring that no more PDUs are discarded than when no content ratio is provided (one of the principles followed in SA2’s work).

Therefore, one can conclude that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC AM.

Proposal 1.	Reply to SA2 that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC AM.

In the first question, SA2 also asks RAN2 to provide feedback on applicability of RLC AM for XR services.

In Rel-19, per RP-240791, latency reduction enhancements for RLC AM is one of the WI objectives. The aim of the enhancements is to reduce the latency of RLC AM so that it is more feasible to support XR services. Therefore, RAN2 should reply to SA2 that it is feasible to use RLC AM for XR services, especially with the upcoming latency reduction enhancements for RLC AM.

Proposal 2.	Reply to SA2 that it is feasible to use RLC AM for XR services, especially with the upcoming latency reduction enhancements for RLC AM.

In the second question, SA2 asks RAN2 to provide feedback on the feasibility of using content ratio information for discarding DL PDUs during congestion for RLC UM.

The main difference between the two RLC modes is that RLC UM does not have ARQ operation. However, a transmitter in RLC UM can still determine whether a PDU has been successfully received based on HARQ feedback in its DL HARQ operation. This is because NG-RAN can determine with very high probability (≥99%, according to TS 38.104) whether a MAC PDU containing a specific RLC PDU is successfully received by UE. In a possible implementation, this capability enables NG-RAN to determine, with high probability, whether a sufficient number of PDUs in a PDU set have been successfully received by the UE so that the original content of that PDU set can be correctly recovered. Consequently, NG-RAN can use this information to identify which PDUs it may discard, in the same way as in the case of RLC AM.

Therefore, RAN2 should reply to SA2 that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC UM.

Proposal 3.	Reply to SA2 that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC UM.

Although gNB behaviors are generally not specified in RAN specifications, we think it would be beneficial to include an example of possible gNB implementation in the reply LS. This example can help illustrate the technical reasons behind the feasibility of using content ratio for discarding DL PDUs during congestion. And it would help the reply LS better facilitate the discussions in SA2.

Proposal 4.	Include an example of possible gNB implementation in the reply LS to illustrate the feasibility of using content ratio for discarding DL PDUs during congestion.

# Conclusion

Based on the above analysis, we respectively request RAN2 to agree to the following and then reply to SA2 accordingly:

Proposal 1.	Reply to SA2 that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC AM.

Proposal 2.	Reply to SA2 that it is feasible to use RLC AM for XR services, especially with the upcoming latency reduction enhancements for RLC AM (per RP-240791).

Proposal 3.	Reply to SA2 that it is feasible to use content ratio for discarding DL PDUs during congestion for DRBs configured with RLC UM.

Proposal 4.	Include an example of possible gNB implementation in the reply LS to illustrate the feasibility of using content ratio for discarding DL PDUs during congestion.