3GPP TSG-RAN WG2 Meeting #128                                         R2-2409571

Orlando, USA, Nov18th – 22th, 2024

Source:        ZTE Corporation

Title:           Discussion on random access procedure in SBFD

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

The WID [1] of Rel-19 SBFD related to random access is given as below:

- Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

This contribution provides our view on random access procedures in SBFD technique.

- Discussion

Supported RA events in SBFD RO

There is a list of RA triggering events (specified in TS38.300 [2]) summarized in the below table:

| RA scenario | RRC state | CBRA/CFRA | initiator |
| --- | --- | --- | --- |
| Initial access from RRC_IDLE | IDLE | CBRA | UE |
| RRC Connection Resume procedure from RRC_INACTIVE | INACTIVE | CBRA | UE |
| Request for Other SI | IDLE/INACTIVE/CONNECTED | CBRA（Msg3-based）/CFRA (Msg1-based) | UE |
| SDT in RRC_INACTIVE | INACTIVE | CBRA | UE |
| Positioning purpose during RRC_CONNECTED requiring random access procedure, e.g., when timing advance is needed for UE positioning | INACTIVE | CBRA | UE |
| RRC Connection Re-establishment procedure | CONNECTED | CBRA | UE |
| DL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" | CONNECTED | CBRA/CFRA(PDCCH order) | gNB |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when UL synchronisation status is "non-synchronised" | CONNECTED | CBRA | UE |
| UL data arrival, during RRC_CONNECTED or during RRC_INACTIVE while SDT procedure is ongoing, when there are no PUCCH resources for SR available; | CONNECTED | CBRA | UE |
| Handover, except for when RACH-less HO is configured; | CONNECTED | CFRA | gNB |
| SR failure | CONNECTED | CBRA | UE |
| Explicit request by RRC upon synchronous reconfiguration; | CONNECTED | CFRA | gNB |
| Beam failure recovery | CONNECTED | CBRA/CFRA | UE |
| To establish time alignment for a primary or a secondary TAG | CONNECTED | CFRA (PDCCH order) | gNB |
| Consistent UL LBT failure on SpCell | CONNECTED | CBRA | UE |
| Early UL synchronization with an LTM candidate cell | CONNECTED | CFRA(PDCCH order) | gNB |
| RACH-based LTM cell switch | CONNECTED | CBRA(RRC based)/CFRA(MAC CE based, RRC based) | UE |

In addition, the WID indicates SBFD-aware UE should be able to perform RA in SBFD resources under all of RRC states (RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode). Also in RAN1#116bis meeting [3], RAN1 has agreed to support both CBRA and CFRA for SBFD-aware UE to perform RA:

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

So from the RAN1’s agreement and RAN2’s trigger events, technically it is feasible to support all kinds of RA in SBFD resources for SBFD UE.

From motivation perspective, the benefit of supporting RA in SBFD resources can be:

- Reduce latency of RA procedure: supporting RA in SBFD resources provides more RA chances or earlier RA chances for SBFD-aware UE, since the number of D slot/symbol is usually larger than U slot/symbol in a TDD system, more ROs in legacy D slot/symbol can be well utilized by SBFD-aware UE;

- Increase UL coverage: Separate RACH configuration may provide a whole different RACH parameters for SBFD UE, e.g., longer preamble format. This helps to increase the UL coverage of SBFD-aware UE.

We do not observe any RA scenarios that does not need to reduce latency or increase UL coverage, for example, SBFD-aware UE wants to perform BFR, or SBFD-aware UE wants to perform re-synchronization. Therefore, there should be no restriction on the supported RA scenarios for RA in SBFD resource.

Some companies raised concern on the SI request in SBFD ROs due to the spec impact may be huge. However, SI request is a CFRA procedure, so gNB can directly indicate whether the RA for SI request should be performed on SBFD RO or legacy RO, e.g., 1 bit in SI-RequestConfig and SI-RequestConfigRepetition-r18 to indicate whether the SI request can be performed in SBFD RO or not, respectively. These two IEs cover all the SI request scenarios, i.e., normal UE SI/POS SI request, RedCap UE SI/POS SI request, normal UE/RedCap UE SI/POS SI request with Msg1 repetition.

Observation 1: The spec impact of supporting SI request in SBFD RO can be small, e.g., 1 bit in SI-RequestConfig and SI-RequestConfigRepetition-r18 to indicate whether the SI request can be performed in SBFD RO or not.

Therefore, we suggest to confirm the following WA made during RAN2#127[4] meeting:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Proposal 1: RAN2 to confirm the working assumption that, random access procedure in SBFD symbols is supported for all the existing RACH trigger events, including SI request.

SBFD RACH configuration in NUL carrier

Legacy RACH configuration can be provided within UL BWP configuration, where the UL BWP can be allocated on NUL carrier or SUL carrier. For legacy RACH configuration in NUL, ROs in DL slot/symbol is considered as invalid; however, SUL carrier can always work even in D slot/symbol, i.e., UE can perform RA in SUL regardless of cell-level TDD pattern.

For SBFD, UL subband should only be configured within DL carrier, and SBFD symbol should only be legacy D or F slot/symbol. If UE performs random access in SBFD symbols, the ROs should occupy or overlap with both SBFD symbol (time domain) and UL subband (frequency domain). As the fact that SUL usually has non-overlapping lower frequency than NUL DL carrier to provide larger UL coverage, in order to occupy or overlap with resources in the UL subband, ROs should be configured within NUL carrier, not SUL carrier.

Figure 1. Example relationship between UL subband, DL carrier, NUL carrier in a TDD system

Proposal 2: RAN2 confirms that SBFD RACH configuration option 1 and option 2 should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

CBRA in SBFD ROs

RAN2#127 has made the following agreements to discuss the RACH configuration in RRC signaling:

RAN2#127 Agreement RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

Wherein the SBFD RACH configuration in the SIB is for RRC_INACTIVE/IDLE UE to perform CBRA (or CFRA for SI request), and in dedicated RRC signaling is for RRC_CONNECTED UE to perform CFRA (or CBRA when fallback).

CBRA RACH resource is provided in SIB1 -> BWP-UplinkCommon for RRC_IDLE/INACTIVE UEs, or in RRCReconfiguration -> BWP-UplinkCommon for RRC_CONNECTED UEs. This section provides our thoughts on the RRC RACH configuration for SBFD-aware UEs in CBRA.

RACH configuration for option 1

In RAN1#118[5] meeting, the further agreement of RACH configuration option 1 is achieved as follows:

Agreement For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following: - For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

This means, both SBFD RO and non-SBFD ROs will be the valid RO type for option 1.

Observation 2: RACH configuration Option 1 can provide both valid SBFD ROs and valid non-SBFD ROs.

Single RACH configuration means SBFD-aware UE reuse the legacy RACH configuration to determine RO location/SSB mapping/power control/etc., but the valid RO of SBFD-aware UE may be different from the valid RO of legacy UE. So, at least gNB should indicate whether the provided RACH configuration in BWP-UplinkCommon in SIB can contain SBFD RO or not.

Note that this indication is the availability of SBFD ROs in option 1, not the priority indication for option 1.

Proposal 3: For CBRA, SBFD RACH configuration option 1, gNB should at least indicate whether the provided RACH configuration in SIB can contain SBFD RO or not, i.e., whether option 1 is available or not for the cell.

RACH configuration for option 2

In RAN1#118 meeting, the further agreement of RACH configuration option 2 is achieved as follows:

Agreement For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

This means, for option 2, only SBFD RO will be the valid RO type.

Observation 3: RACH configuration Option 2 can only provide valid SBFD ROs.

Separate RACH configuration provides independent basic RACH parameters compared with the legacy RACH configuration. So the valid RO of derived from separate RACH configuration and legacy RACH configuration can be totally in different pattern to increase the UL coverage, reduce the latency, etc.

According to the agreement in RAN2#127-bis [6], a new RACH configuration dedicated for SBFD-aware UE for option 2 will be needed:

Agreement in RAN2#127-bis For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. FFS dedicated RRC signalling detail.

We suggest to add the SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, like the following figure:

Figure 2. Recommended CBRA RACH configuration option 2

This way is to configure option 2 using similar principle of introducing 2-step RACH configuration, i.e., the new SBFD RACH configuration is also contained in each element in AdditionalRACH-ConfigList.

Proposal 4: For CBRA, SBFD RACH configuration option 2, add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config.

Configuration of both option 1 and option 2

RAN1#117 meeting [7] has made the following agreement:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

RAN2#127-bis meeting also suggest to discuss the co-existence between option 1 and option 2:

The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration - For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. - the RACH configuration for SBFD is transmitted via SIB1. - FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

For option 1 or option 2 of SBFD RACH configuration, RAN1’s agreement indicates ‘UE is not required to support both options’, which means UE will have separate UE capability indication for supporting option 1 or option 2. From UE capability perspective, UE can support only option 1, or only option 2, or support both.

Observation 4: UE will have separate UE capability indication for supporting option 1 or option 2. From UE capability perspective, UE can support only option 1, or only option 2, or support both.

So in CBRA, where gNB does not know UE’s identity and capability, gNB is better to provide both single SBFD RACH configuration and separate SBFD RACH configuration to suit different UE’s capabilities. Then, UE can be allowed to choose the configuration based on specified rules, UE capabilities, etc.

Therefore, RAN2 should specify both options for CBRA. CBRA RACH resource is configured in SIB1 -> BWP-UplinkCommon, or RRCReconfiguration -> BWP-UplinkCommon.

Proposal 5：For CBRA, gNB should be allowed to configure both option 1 and option 2 RACH configuration, with both option indications setting to OPTIONAL.

RACH configuration selection

Based on the analysis in section 2.3.3, for CBRA, gNB should be allowed to provide both option 1 and option 2 in BWP-UplinkCommon, wherein option 1 contains legacy ROs and SBFD ROs, option 2 contains SBFD ROs.

When processing RA in MAC layer, the order is: UE selects set -> UE determines the RACH configuration and corresponding RACH parameters -> UE selects RA type (2-step RACH or 4-step RACH). So after selecting set, SBFD-aware UE should determine which RACH configuration to use.

Proposal 6: In CBRA, SBFD-aware UE should firstly select SBFD RACH configuration (option 1 or option 2), then if SBFD RACH configuration selects to option 1, the UE should further selects RO type.

There are several kinds of RACH configuration selection rule proposed by companies, to be specific:

- SBFD-aware UE selects the RACH configuration based on UE’s implementation;

- SBFD-aware UE selects the RACH configuration that associates the highest configured priority;

- SBFD-aware UE firstly selects RACH configuration if SBFD RO is provided.

In summary, RAN2 should firstly acknowledge the issue that SBFD-aware UE should select one RACH configuration, and this UE behavior should be specified in MAC spec. The detailed rule of selection can be further discussed by RAN2/RAN1.

RO type selection

RAN2#127-bis has agreed to select RO type based on some pre-defined rules:

Agreement Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

Currently UE selects nearest RO after SSB/CSI-RS is selected, as specified in TS38.321 [8] below:

5.1.2 Random Access Resource selection If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall: <omitted> 1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB).

When a gNB configures SBFD resource and lets UE initiating RA on the SBFD resource, it aims to provide more ROs (more RA chances) to reduce the RA latency, so it is likely that the system has more SBFD ROs than non-SBFD ROs configured within one RACH configuration. If a SBFD-aware UE selects a nearest non-SBFD RO according to current spec for the first several Msg1/MsgA attempts, the whole RA time will very likely be longer than choosing SBFD ROs in the beginning, since the subsequent SBFD ROs will be pretty intense although SBFD RO is not nearest RO type. In addition, this is not good for load balancing, since the non-SBFD ROs may suffer from multiple UE’s access including SBFD-aware UEs. Therefore, always selecting the nearest RO may not be a good way when SBFD RO is introduced.

Observation 5: Always selecting the nearest RO may not be a good way when SBFD RO is introduced.

To solve the above problem and to balance the whole RA latency and total RA chances, UE can use a time window to determine whether to select SBFD RO or legacy RO. If during the time window UE finds the SBFD RO exists, no matter whether legacy RO is nearest or not, UE should choose the SBFD RO to insure more RA chances without losing too much latency; if during the time window UE finds there is no SBFD RO, UE should select the legacy RO based on legacy nearest rule to insure latency, since in this case, even if the SBFD RO may be intense after the time window, UE choosing SBFD RO will lose RA latency.

Proposal 7: For CBRA, introduce a time window to let UE determine whether to select SBFD RO or legacy RO:

- If UE finds the SBFD RO exists in the time window, no matter whether legacy RO is the nearest or not, UE should choose the nearest SBFD RO to insure more RA chances without losing too much RA latency;

- If UE finds there is no SBFD RO in the time window, UE should select the legacy RO based on legacy nearest rule to insure RA latency.

MAC order of RACH resource/RO type selection

Based on the analysis above, for CBRA, UE can select RACH configuration and RO type based on some criteria. RAN2 needs to consider that how does such selection affect the current RA procedure order in UE’s MAC layer.

The current RA procedure in MAC spec is as follows:

Figure 3. Current MAC order of initiating RA procedure

The RACH configuration selection/RO type selection can happen before resource set selection, or after resource set selection. The main difference is that, the definition of set will be different in these two ways. When before, the selected set of RACH resource the UE selected is per RO type; when after, the selected set of RACH resource is per UE.

Current fallback mechanism is restricted within one set, e.g., 2-step RACH falls back to 4-step RACH, even though 2-step RACH and 4-step RACH are two different RACH configurations, they belong to one set of RACH resources. Or fallback from lower repetition number to higher repetition number, the spec also restricts to select a fallback set that has the same associated feature combination, i.e., same set.

Since RAN2 already agreed on SBFD RO fallback to legacy RO, we think the similar principle should be applied here, i.e., fallback mechanism (from SBFD RO to non-SBFD RO) is restricted within one set. Further when option 2 is configured, SBFD RO and legacy RO will be in two separate RACH configurations. Therefore to achieve the fallback, RACH configuration selection should be after the set selection to ensure that legacy RACH configuration and SBFD RACH configuration option 2 can be selected within one set.

Proposal 8: RACH configuration selection should be after the set selection to ensure that legacy RACH configuration and SBFD RACH configuration option 2 can be selected within one set.

Furthermore, UE selects 2-step or 4-step RACH based on some specific criteria/configurations. Since currently RA in SBFD RO is only supported in 4-step RACH, when UE selects to 2-step RACH, this means UE selects to RA in legacy RACH configuration (or legacy RO); only when UE selects to 4-step RACH, UE should further select RACH configuration (or RO type). So, UE should select RACH configuration (or RO type) after RA type selection, and before RACH initialization.

Proposal 9: UE should select RACH configuration (or RO type) after RA type selection, and before RACH parameter initialization.

Fallback from SBFD RO to non-SBFD RO

RAN2#127-bis has already agreed to support fallback from SBFD RO to legacy RO:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

For RACH configuration option 1, the fallback is within one RACH configuration;

For RACH configuration option 2, the fallback is from SBFD RACH configuration to legacy RACH configuration. This is similar like 2-step RACH configuration fallback to 4-step RACH configuration.

We do not observe the benefit for a UE to select legacy RO first, after a number of preamble transmission failure, then selects to SBFD RO. SBFD RO may have larger interference.

Proposal 10: Do not support the fallback from legacy RO to SBFD RO.

CFRA in SBFD ROs

SBFD RACH configuration in CFRA

RAN1#117 has achieved the following agreements:

Agreement Confirm the working assumption: Working assumption: For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.

That is to say, RAN2 should specify SBFD RACH configuration in the CFRA resource.

Observation 6: Based on RAN1’s agreement, RAN2 should specify SBFD RACH configuration in the CFRA resource.

gNB configures different CFRA RACH resource for different CFRA scenarios, using RRC signaling. To be specific:

- For UE performing CFRA for SI request, the legacy RACH configuration is provided in SIB1 - SI-SchedulingInfo - SI-RequestConfig/SI-RequestConfigRepetition;

- For UE performing CFRA for BFR, the legacy RACH configuration is provided in BWP-UplinkDedicated - BeamFailureRecoveryConfig;

- For UE performing CFRA for ReconfigurationWithSync, the legacy RACH configuration is provided in ReconfigurationWithSync - rach-ConfigDedicated;

- For UE performing CFRA for LTM early sync, the legacy RACH configuration is provided in RRCReconfiguration - LTM-Config -  LTM-Candidate - ltm-EarlyUL-SyncConfig - EarlyUL-SyncConfig;

In Rel-18, a LTM cell switch commend MAC CE is introduced to indicate CFRA resource in LTM:

TS38.321-i30: - 6.1.3.75 LTM Cell Switch Command MAC CE Figure 6.1.3.75-1: LTM Cell Switch Command MAC CE

PDCCH order using DCI can also be used to indicate CFRA resource for LTM early sync.

The principle of CFRA is, gNB expects to receive certain preamble at certain places rather than blind decoding. Since the CFRA RACH resource is provided to UE via dedicated RRC signaling or MAC CE or DCI, gNB will know the UE capability before allocating the CFRA RACH resource for specific SBFD-aware UE. So in CFRA, gNB should explicitly indicate the RACH configuration type and/or RO type in the CFRA resource. UE does not need to make further selection procedure on the RACH configuration type and/or RO type (e.g., select based on certain criteria).

Legacy CFRA resource does not support fallback to CFRA resource, since CFRA resource can fallback to CBRA resource. For CFRA in SBFD ROs, we also think CFRA SBFD RO should not fallback to CFRA legacy RO. So, CFRA resource does not need to provide two RO types (i.e., option 1) nor two RACH configurations (option 2). CFRA resource should only provide one RO type, i.e., there is no need to differentiate option 1 and option 2 RRC configuration in CFRA. If the CFRA SBFD RO fails (e.g., no measured RS-RSRP is beyond the configured threshold), UE can fall back to CBRA resource to continue.

Proposal 11: CFRA resource should only provide one RO type, i.e., SBFD RO or legacy RO.

As analysed above, the CFRA resource can be given via RRC, MAC-CE or DCI:

- In RRC signaling, each CFRA RACH configuration is associated with a RACH-ConfigGeneric, the RACH-ConfigGeneric can naturally contain SBFD RO and non-SBFD RO, since the RO pattern is configured periodical and has nothing to do with TDD pattern. So one simple way is to add an indication in each RRC-configured CFRA RACH configuration to say that, whether UE should only consider SBFD ROs provided by the corresponding RACH-ConfigGeneric to be valid, or UE should only consider non-SBFD ROs provided by the corresponding RACH-ConfigGeneric to be valid.

  - The RRC signaling contains BFR CFRA resource, configured in BWP-UplinkDedicated – BeamFailureRecoveryConfig;

  - The RRC signaling contains ReconfigurationwithSync CFRA resource, configured in ReconfigurationWithSync - rach-ConfigDedicated;

  - The RRC signaling contains LTM early sync CFRA resource, configured in RRCReconfiguration - LTM-Config - LTM-Candidate - ltm-EarlyUL-SyncConfig - EarlyUL-SyncConfig.

- For CFRA resource provided via LTM cell switch MAC CE, an indication should be added in the MAC CE that, whether the RA triggered by the MAC CE can only be performed on SBFD RO;

- For PDCCH order, RAN1#118-bis meeting has the following agreement to explicit indicate the RO type in PDCCH order:

Agreement For SBFD-aware UEs and PRACH mask indication for CFRA triggered by PDCCH order, discuss whether/how to indicate/determine additional-RO only, legacy-RO only, or both is used.

RAN2 should focus on RRC and MAC spec impact, and leave DCI/PDCCH order to RAN1 decision. So we have the following proposal:

Proposal 12: For CFRA, the following RRC and MAC spec impact should be considered:

- Add an indication in SI-RequestConfig and SI-RequestConfigRepetition to say whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an indication in BeamFailureRecoveryConfig to say whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an indication in rach-ConfigDedicated to say whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an indication in EarlyUL-SyncConfig to say whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an indication in LTM cell switch command MAC CE to say whether the RA triggered by the MAC CE can only be performed on SBFD RO.

Fallback from CFRA to CBRA

In legacy RA mechanism, if CFRA resource is provided for the current RA event, UE should firstly try to use CFRA resource to initiate RA by comparing the measured RS-RSRP and configured RS-RSRP. If the measured RS-RSRP is larger than the configured RS-RSRP, UE should use the indicated CFRA resource; if not, UE should use the CBRA resource within the set selected at the beginning. Each time when the current preamble transmission fails, UE should compare the measured RS-RSRP and configured RS-RSRP to determine whether CFRA or CBRA should be used, again. The basic procedure flow is as follows:

Figure 4. CFRA falls back to CBRA

Since in legacy mechanism there is no SBFD RO, UE keeps the same RO type when CFRA fallbacks to CBRA, i.e., in fallback CBRA, UE should select the same RO type as the RO type indicated by gNB for CFRA resource. We think the similar procedure should be applied here, i.e., if gNB indicates to use SBFD RO for the CFRA procedure, when CFRA falls back to CBRA, UE should also select SBFD RO for the CBRA procedure. That is to say, UE should NOT use the criteria to select between SBFD RO and legacy RO in CBRA, when the CBRA is a fallback from CFRA. The selection criteria is only for pure CBRA case.

Proposal 13: When CFRA falls back to CBRA, if CFRA is indicated to perform on SBFD ROs, the fallback CBRA should also be performed on SBFD ROs.

Early identification of SBFD-aware UE

RAN2#127-bis meeting has agreed the following:

Agreement RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

That is to say Msg1 based early identification is not supported in legacy ROs. The question is whether to support Msg3 based early identification when legacy RO is choosen.

Figure 5. Example of initial access timeline

Firstly, as the figure indicates, if early identification is achieved in Msg3, the benefit is only on Msg5 UL, NAS security UL and AS security UL (see green arrows), these UL transmission can be scheduled on UL subband. When gNB acquires UE capability, the support of SBFD feature will be directly known by gNB. However the NAS security and AS security set up procedure is only applicable in RRC_IDLE, i.e., even if UE resumes from RRC_INACTIVE, these two steps are not needed. So the scenario of scheduling UL transmission in UL subband becomes more limited.

Secondly, Msg3 based early identification will achieve less benefit than Msg1 based early identification. If companies thinks early identification is not an urgent feature to support for Msg1 based early idnetification, then Msg3 based early identification shouldn’t be supported at all due to the lower benefit.

Based on the above, we have the following proposal:

Proposal 14: Do NOT support Msg3 based early identification for SBFD-aware UE.

Msg1 repetition in SBFD ROs

RAN1#118 has made the following agreement regarding Msg1 repetition in SBFD ROs:

Agreement For SBFD aware UEs, at least support the following: - PRACH transmission with preamble repetitions within additional-Ros (not across additional-Ros and legacy Ros) Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-Ros and legacy Ros) as in legacy releases.

RAN1#118-bis [9] has made the following WA regarding Msg1 repetition in SBFD ROs:

Working Assumption For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs. - the time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs

It can be seen that Msg1 repetition in SBFD ROs is supported in RAN1, so RAN2 should discuss solutions and specification impacts on this aspect.

The potential discussion point includes whether to support Msg1 repetition in SBFD ROs in CFRA and/or in CBRA , how to determine the Msg1 repetition number in SBFD ROs in CFRA and/or CBRA, whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA, etc.

Proposal 15: RAN2 to further discuss the solutions related to Msg1 repetition in SBFD ROs, including:

- Whether to support Msg1 repetition in SBFD ROs in CFRA and/or in CBRA;

- How to determine the Msg1 repetition number in SBFD ROs in CFRA or in CBRA;

- Whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA.

- Conclusion

In this contribution, we propose the following observation and proposals:

Supported RA events in SBFD RO:

Observation 1: The spec impact of supporting SI request in SBFD RO can be small, e.g., 1 bit in SI-RequestConfig and SI-RequestConfigRepetition-r18 to indicate whether the SI request can be performed in SBFD RO or not.

Proposal 1: RAN2 to confirm the working assumption that, random access procedure in SBFD symbols is supported for all the existing RACH trigger events, including SI request.

SBFD RACH configuration in NUL carrier:

Proposal 2: RAN2 confirms that SBFD RACH configuration option 1 and option 2 should only be configured in NUL carrier, no matter the SBFD RACH configuration is in SIB or in dedicated signaling.

CBRA in SBFD ROs:

Observation 2: RACH configuration Option 1 can provide both valid SBFD ROs and valid non-SBFD ROs.

Observation 3: RACH configuration Option 2 can only provide valid SBFD ROs.

Observation 4: UE will have separate UE capability indication for supporting option 1 or option 2. From UE capability perspective, UE can support only option 1, or only option 2, or support both.

Observation 5: Always selecting the nearest RO may not be a good way when SBFD RO is introduced.

Proposal 3: For CBRA, SBFD RACH configuration option 1, gNB should at least indicate whether the provided RACH configuration in SIB can contain SBFD RO or not, i.e., whether option 1 is available or not for the cell.

Proposal 4: For CBRA, SBFD RACH configuration option 2, add a SBFD RACH configuration parallel to RACH-ConfigCommon in BWP-UplinkCommon, and add the SBFD RACH configuration in each AdditionalRACH-Config.

Proposal 5：For CBRA, gNB should be allowed to configure both option 1 and option 2 RACH configuration, with both option indications setting to OPTIONAL.

Proposal 6: In CBRA, SBFD-aware UE should firstly select SBFD RACH configuration (option 1 or option 2), then if SBFD RACH configuration selects to option 1, the UE should further selects RO type.

Proposal 7: For CBRA, introduce a time window to let UE determine whether to select SBFD RO or legacy RO:

- If UE finds the SBFD RO exists in the time window, no matter whether legacy RO is the nearest or not, UE should choose the nearest SBFD RO to insure more RA chances without losing too much RA latency;

- If UE finds there is no SBFD RO in the time window, UE should select the legacy RO based on legacy nearest rule to insure RA latency.

Proposal 8: RACH configuration selection should be after the set selection to ensure that legacy RACH configuration and SBFD RACH configuration option 2 can be selected within one set.

Proposal 9: UE should select RACH configuration (or RO type) after RA type selection, and before RACH parameter initialization.

Proposal 10: Do not support the fallback from legacy RO to SBFD RO.

CFRA in SBFD ROs:

Observation 6: Based on RAN1’s agreement, RAN2 should specify SBFD RACH configuration in the CFRA resource.

Proposal 11: CFRA resource should only provide one RO type, i.e., SBFD RO or legacy RO.

Proposal 12: For CFRA, the following RRC and MAC spec impact should be considered:

- Add an indication in SI-RequestConfig and SI-RequestConfigRepetition to say whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an indication in BeamFailureRecoveryConfig to say whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an indication in rach-ConfigDedicated to say whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an indication in EarlyUL-SyncConfig to say whether the corresponding RACH-ConfigGeneric only contains valid SBFD RO;

- Add an indication in LTM cell switch command MAC CE to say whether the RA triggered by the MAC CE can only be performed on SBFD RO.

Proposal 13: When CFRA falls back to CBRA, if CFRA is indicated to perform on SBFD ROs, the fallback CBRA should also be performed on SBFD ROs.

Early identification of SBFD-aware UE:

Proposal 14: Do NOT support Msg3 based early identification for SBFD-aware UE.

Msg1 repetition in SBFD ROs:

Proposal 15: RAN2 to further discuss the solutions related to Msg1 repetition in SBFD ROs, including:

- Whether to support Msg1 repetition in SBFD ROs in CFRA and/or in CBRA;

- How to determine the Msg1 repetition number in SBFD ROs in CFRA or in CBRA;

- Whether/how to support Msg1 repetition number fallback in SBFD ROs in CBRA.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, 2024-06

- TS38.300-i20, ‘NR and NG-RAN Overall Description; Stage 2’, Release 18, 2024-06

- R1-2403789, Final Summary on SBFD random access operation, RAN1#116-bis, CMCC, 2024-04

- R2-2407572, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD, RAN2#127, CATT, 2024-08

- 1 118 Final Summary on SBFD random access operation, RAN1#118, CMCC, 2024-08

- R2-2409212, Report from session on R18 MIMOevo, R18 MUSIM, R19 LP-WUS and R19 SBFD, RAN2#127, CATT, 2024-10

- R1-2405656, Final Summary on SBFD random access operation, RAN1#117, CMCC, 2024-05

- TS38.321-i30, ‘Medium Access Control (MAC) protocol specification’, Release 18, 2024-09

- R1-2409047, Final Summary on SBFD random access operation, RAN1#118-bis, CMCC, 2024-10