3GPP TSG-RAN WG2 Meeting #128	R2-2409637

# Orlando, Florida, USA, 18 – 22 November 2024

Agenda item:		8.7.6

Source:	Xiaomi

Title:	XR rate control

Document for:		Discussion and Decision

Introduction

In RAN2#127bis meeting, following was agreed on XR rate control [1]:

- FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - - Extend the Bit Rate field - - Define a new bit rate table to provide sufficient granularity for XR traffic - - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required

In this contribution, we discus XR rate control.

Discussion

Rate control granularity

One FFS from RAN2#127bis meeting is whether rate control granularity is per DRB or per QoS flow. Arguments in favor of per QoS flow indication include that QoS enforcement is performed on QoS flow basis, while arguments in favor of per DRB include that UE implementation can handle the rate allocation for QoS flows mapped to one DRB.

One aspect for discussion is the interworking between Rel-19 rate control mechanism and L4S. In Rel-18, SA2 defines support of L4S based on ECN marking, as from TS 23.501:

ECN marking for L4S is enabled on a per QoS Flow basis in the uplink and/or downlink direction and may be used for GBR and non-GBR QoS Flows. ECN marking for the L4S in the IP header is supported in either the NG-RAN (see clause 5.37.3.2 and TS 38.300 [27]), or in the PSA UPF (see clause 5.37.3.3).

In TS 38.300 clause 16.16, ECN marking for L4S and congestion information exposure is described as below:

In order to support ECN marking for L4S at gNB as specified in TS 23.501[3], SMF provides ECN marking request per QoS flow level to the gNB as part of PDU Session Resource Management procedure. If the gNB supports ECN marking, it provides the status indication back to the SMF which is used by the SMF as specified in TS 23.501[3]. During Xn Handover Preparation procedure, source gNB provides the ECN marking request to target gNB.

When ECN marking for L4S at gNB is enabled for downlink or uplink, the gNB should set the Congestion Experienced (CE) codepoint in downlink or uplink SDAP SDU(s) as per the recommendations in IETF RFC 9330 [61], IETF RFC 9331 [62] and IETF RFC 9332 [63].

For ECN marking for L4S at UPF, SMF requests the gNB to report congestion information per QoS flow level as part of PDU Session Resource Management procedure. If the gNB supports ECN marking for L4S at UPF, it provides the status indication back to the SMF which is used by SMF as specified in TS 23.501[3]. During Xn Handover Preparation procedure, source gNB provides the ECN marking UPF request to target gNB.

For congestion reporting from gNB to UPF, SMF requests the gNB to report congestion information per QoS flow level as part of PDU Session Resource Management procedure. If the NG-RAN supports congestion information reporting, it provides the status indication back to the SMF which is used by the SMF as specified in TS 23.501[3]. During Xn Handover Preparation procedure, source gNB provides the congestion information request to target gNB.

From above, it can be seen that L4S can be enabled on a per QoS flow basis. Before discussing the relationship between L4S and Rel-19 rate control, it is helpful to first review the principles of L4S. L4S is based on ECN, with various enhancements like Dual-Queue Coupled Active Queue Management (AQM) and new congestion control algorithm TCP Prague. An example end-to-end L4S operation in NR is shown in Figure 1 below. For UL traffic, if gNB detects congestion for one IP flow / QoS flow, gNB sets the Congestion Experienced (CE) codepoint in IP header (as in step 1). When the UL packet arrives at the receiver (e.g. Internet Server as in this example), the receiver detects the ECN marking and echoed back the ECN feedback to the transmitter (i.e. UE in current example) through the transport protocol (as in step 2). For example, when using TCP, ECN TCP header bits and AccECN TCP Option can be used as ECN feedback. The transmitter (UE) receives the ECN feedback and adjusts the UL data rate (as in step 3) based on congestion control algorithm (e.g. TCP Prague as in L4S).

Figure 1: End-to-end operation of L4S

From above discussion, it can be seen that in L4S, UL data rate is adjusted based on ECN feedback via congestion control algorithm. In Rel-19 rate control, UL data rate is adjusted by MAC CE sent by gNB. Below is a comparison of L4S and Rel-19 rate control.

Table 1: Comparison of L4S and Rel-19 rate control

|  | L4S | Rel-19 rate control |
| --- | --- | --- |
| Data rate adjustment layer | Transport layer (e.g. TCP) | Application layer |
| Adjustment method | Congestion control algorithm based on ECN feedback | Based on bit rate indicated by gNB |
| Congestion place that can be handled by the solution | End-to-end link | Air interface congestion only |

Regarding the QoS enforcement, since the rate adjustment is eventually done in application layer, it is expected that the adjustment is in QoS flow granularity. One example is that per QoS flow Bit Rate is signalled as QoS parameter from 5GC to gNB (TS 38.413 clause 9.3.1.10 GBR QoS Flow Information). However, with proper network configuration of QoS flow to DRB mapping, per DRB adjustment can achieve similar result as per QoS flow adjustment, and is already used in legacy Recommended bit rate MAC CE. Per DRB adjustment also provides the UE the flexibility to partition the available resource to different QoS flows mapped to one DRB. In addition, since rate control will be specified in MAC layer, it is more natural to have per DRB / LCH granularity especially in CU-DU split scenario.

Proposal 1: Rate control is performed in DRB / LCH granularity.

MAC CE content

Existing Recommended bit rate MAC CE format is specified in TS 38.321, as below. It contains 4 fields: LCID, UL/DL direction, Bit Rate, X (Bit Rate Multiplier).

Figure 6.1.3.20-1: Recommended bit rate MAC CE

From RAN2#127bis discussion, if a new MAC CE is introduced for Rel-19 rate control, at least Bit Rate and identification for QoS flow or DRB will be included in the new MAC CE.

Proposal 2: If a new MAC CE is introduced for Rel-19 rate control, it includes at least the following fields: Bit Rate, identification for QoS flow or DRB (pending granularity discussion).

Bit rate indication enhancement is mainly related to the range / granularity of XR services. Given that RAN2 just sent SA4 LS asking about the required range/granularity, it is better for RAN2 to wait for SA4 reply before proceeding on RAN2 work.

Proposal 3: RAN2 waits for SA4’s reply on range/granularity before proceeding on detailed work on bit rate indication enhancements related to range / granularity.

No matter SA4’s reply on range / granularity, there are two issues of existing Recommended bit rate MAC CE due to the usage of bitRateMultiplier,.

- Duplicated code points: An example is that 100 times of 40 kbps is 4000 kbps, which is another code point when multiplier is not used. There are 56 codes points used in the MAC CE, for data rate in the range of 0~8000 kbps. Considering 4 multipliers, there are 56 * 5 = 280 code points. However, there are only 196 unique values among the code points, which means that (280 – 196) / 280 = 30% code points are wasted.

- The code points are not evenly distributed in linear or exponential scale. For example, following are a subset of codepoints in ascending order. It can be seen that there are code points close to each other, e.g. 450000 and 455000, and code points with larger distance like 800000, 900000, and 1000000. This is not an efficient way to utilize the available code points.

| Code point value | Ratio relative to previous value |
| --- | --- |
| 450000 | 1.071 |
| 455000 | 1.011 |
| 490000 | 1.077 |
| 500000 | 1.021 |
| 525000 | 1.050 |
| 550000 | 1.048 |
| 560000 | 1.018 |
| 600000 | 1.071 |
| 650000 | 1.083 |
| 700000 | 1.077 |
| 750000 | 1.071 |
| 800000 | 1.067 |
| 900000 | 1.125 |
| 1000000 | 1.111 |

Observation 1: Current Recommended bit rate MAC CE has the following issues regarding index to bit rate mapping:

1) There are 30% wasted code points.

2) The code points are not evenly distributed in linear or exponential scale.

If existing Recommended bit rate MAC CE is not sufficient for the range / granularity of XR service as from SA4’s reply, then a new table is at least needed to address the drawbacks of existing Recommended bit rate MAC CE.

Proposal 4: A new table with improved index to bit rate mapping is at least needed for rate control, pending SA4 reply.

In contributions submitted to RAN2#127bis meeting, there were proposals to introduce a congestion relief MAC CE to signal that there is no more congestion. It should be noted that with the bit rate indication, gNB can increase the bit rate if there is no more congestion. Hence there is no need to explicitly introduce a congestion relief MAC CE.

Proposal 5: A single MAC CE is used for both cases: 1) reduce the data rate when congestion happens; 2) increase the data rate when congestion is relieved.

UL/DL query

Recommended bit rate MAC CE is applicable for both DL and UL data, and can be sent from gNB to UE, or from UE to gNB (as “query”).

For Rel-19 rate control, WID [2] includes the following objective: “Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion”. The WID emphasizes “control signalling over downlink” and “uplink congestion”. However rate adaptation is a procedure in end-to-end manner. Considering the use case that gNB indicates to the UE to increase data rate when congestion is relieved. In RAN-assisted codec adaptation, the corresponding signalling flow in LTE is documented in TS 26.114 Figure 10.7-2, as copied below:

Figure 10.7-2 Uplink bitrate increase based on ANBR

In step 4, AN-1 (Access Network) sends to UE-1 ANBR (Access Network Bitrate Recommendation, i.e. Recommended bit rate MAC CE) to increase data rate. With the increase of DL data rate, the communication peer (UE-2) needs to increase the DL data rate accordingly, therefore UE-2 sends ANBRQ (Access Network Bitrate Recommendation Query, i.e. Recommended bit rate query MAC CE) to AN-2, and AN-2 response with ANBR to configure DL data rate.

Similarly, for Rel-19 rate control, from end-to-end perspective, it is also helpful to support bit rate query message from UE to gNB, as well as the rate control for DL.

Proposal 6: Rate control can be applicable to UL data as well as to DL data.

Proposal 7: Query indication (similar to Recommended bit rate query) from UE to gNB is supported.

Split bearer in DC

In DC scenario, split bearer can be used for XR services to boost the throughput. If both MN and SN send MAC CEs for rate control of the same split bearer or same QoS flow(s) mapped to split bearer, some discussion might be needed on UE behavior regarding which data rate to be applied for the split bearer or DRB as indicated in the MAC CEs, e.g. whether UE follows latest MAC CE from MN or SN, or UE should consider the MAC CEs jointly.

Proposal 8: RAN2 to discuss whether split bearer should be considered for rate control.

Conclusion

In this contribution, we discuss XR rate control, and have the following observation:

Observation 1: Current Recommended bit rate MAC CE has the following issues regarding index to bit rate mapping:

1) There are 30% wasted code points.

2) The code points are not evenly distributed in linear or exponential scale.

We propose the following:

Proposal 1: Rate control is performed in DRB / LCH granularity.

Proposal 2: If a new MAC CE is introduced for Rel-19 rate control, it includes at least the following fields: Bit Rate, identification for QoS flow or DRB (pending granularity discussion).

Proposal 3: RAN2 waits for SA4’s reply on range/granularity before proceeding on detailed work on bit rate indication enhancements related to range / granularity.

Proposal 4: A new table with improved index to bit rate mapping is at least needed for rate control, pending SA4 reply.

Proposal 5: A single MAC CE is used for both cases: 1) reduce the data rate when congestion happens; 2) increase the data rate when congestion is relieved.

Proposal 6: Rate control can be applicable to UL data as well as to DL data.

Proposal 7: Query indication (similar to Recommended bit rate query) from UE to gNB is supported.

Proposal 8: RAN2 to discuss whether split bearer should be considered for rate control.

- References

[1] R2-2409215, Session chair (Huawei), “Report from session on R18 MBS, R18 QoE and R19 XR”

[2] RP-241771, Nokia (Rapporteur), “Revised WID on XR (eXtended Reality) for NR Phase 3”