3GPP TSG-RAN WG2 Meeting #128                                               R2-2409650

Orlando, USA, Nov 18th – Nov 22nd, 2024

Source:	CATT

Title:	Discussion on MRO Enhancements for Rel-18 Mobility

Agenda Item:	8.10.2

Document for:	Discussion and Decision

1. Introduction

In RAN2#127bis meeting, MRO enhancements for LTM and CHO with candidate SCGs were discussed and the following agreements were achieved [1].

LTM

Agreements - The UE shall log cell IDs such as reestablishment cell ID, source, failed, reconnect cell ID, following the same principle as RLF, HOF and successful recovery, incl. the time between UE executing the LTM command and the failure. - If RA-based LTM failure happens the UE logs and reports RACH info in the RLF report. Additional information is TBD. - Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined). - Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

Agreements - Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell. - UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR.

CHO with candidate SCG(s)

Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

In this contribution, we continue the discussion on MRO enhancements for LTM and CHO with candidate SCGs.

2. Discussion

2.1 LTM

- Enhance RLF report for LTM MRO

In LTM, if the UE can acquire the valid TA value (included in LTM cell switch command or measured by UE itself) when receiving the LTM cell switch command, UE will perform RACH-less LTM execution. For RACH-less LTM, both dynamic grant and configured grant can be used, and the UE will apply the TCI state indicated in the LTM cell switch command for dynamic grant reception and configured grant resource selection. If RACH-less LTM fails, it may be because the TCI state indicated in LTM cell switch command is inappropriate, then the UE needs to log the TCI state ID indicated in the LTM cell switch command in RLF report to help the network to optimize and select a suitable TCI state to avoid the RACH-less LTM failure.

Proposal 1: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

- Enhance SHR for LTM MRO

For successful LTM execution but it’s still close to failure, legacy SHR trigger conditions (i.e., T310 threshold, T312 threshold, T304 threshold) can be applied to LTM.

Proposal 2: The legacy SHR trigger conditions can also be applied to LTM, i.e., T310 threshold, T312 threshold, and T304 threshold.

SHR should be enhanced to record the successful LTM information. Similar as L3 measurement of serving cell, target cell and other LTM candidate cell(s) are included in RLF report upon RLF or mobility failure, L3 measurement of serving cell, target cell and other LTM candidate cell(s) should also be recorded in SHR for successful LTM execution.

Proposal 3: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

- Enhance RACH report for LTM MRO

RAN2 has agreed to enhance RACH report for MCG LTM SON, raPurpose should be enhanced to cover successful RACH-based LTM execution, the detail can be discussed further, e.g., whether legacy raPurpose can be reused or a new raPurpose for LTM execution needs to be introduced.

Proposal 4: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. The detail can be discussed further, e.g., whether to reuse legacy raPurpose or introduce a new raPurpose.

For the TA value in LTM cell switch command (network-based TA acquisition), the early RACH procedure is needed. The source cell needs to send the PDCCH order command to UE, then the UE sends the preamble towards the indicated candidate cell. After the candidate cell acquires the TA value when receives the preamble from UE, it sends the TA value to the source cell and source cell includes the TA value in the LTM cell switch command to UE. The RACH procedure can be triggered multiple times towards same or different candidate cells before receiving LTM cell switch command. A new raPurpose can be introduced to indicate the network-based early TA acquisition.

Proposal 5: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Based on current spec, the UE may include up to 8 RACH report in RACH report list. If the RACH report is enhanced to support logging the RACH information of network-based early TA acquisition, the RACH report entry will reach max number (i.e., maxRAReport) soon as the frequent network-based early TA acquisition, this will affect the legacy RACH information logging, e.g., RACH information of handover as the UE will stop the RACH report logging if the entry number reaches max number, which will cause the legacy RACH information loss and this is not expected as RACH information is important for RACH optimization.

5.7.10.4 Actions for the Random Access report determination Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5, the UE shall: 1> if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report; or 1> if the registered SNPN identity or the SNPN identity selected by upper layers (see TS24.501 [23]) from the list of SNPN(s) included in the NPN-IdentityInfoList in SIB1is not included in snpn-IdentityList stored in a non-empty VarRA-Report: 2> clear the information included in VarRA-Report; 1> if the UE is not in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport: 2> if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is less than maxPLMN; or 2> if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report: 3> append the following contents associated to the random-access procedure as a new entry in the VarRA-Report: <omit part>

RAN2 needs to discuss how to log the RACH information of network-based early TA acquisition and avoid the impact on the legacy RACH information logging. Some solutions can be considered to address the issue, e.g., one entry in RACH report can be used to log the RACH information of all network-based early TA acquisition before one LTM execution or one entry in RACH report can be used to log the RACH information of all network-based early TA acquisition before LTM configuration release or log the RACH information of network-based early TA acquisition in a report other than RACH report, e.g., a new report or a current existing report.

Proposal 6: RAN2 to discuss how to differentiate RACH information logging of network-based early TA acquisition from the legacy RACH information logging (e.g., RACH information for handover):

- Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

- Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

- Option 3: log the RACH information of all network-based early TA acquisition in a report other than RACH report, e.g., a new report or a current existing report.

- Other

The LTM configuration will not be released after successful LTM execution, which means subsequent LTM can continue to be performed. However, LTM execution is based on the L1 measurement results which vary very rapidly and may cause the frequent LTM execution. Considering that the legacy RLF report/SHR only includes the latest failure/successful handover information, for LTM, there is high probability that the old RLF report/SHR will be overrode by new RLF report/SHR due to the frequent LTM execution, this will cause the information loss for network optimization. In order to address the issue, recording multiple RLF report/SHR can be considered as a candidate solution.

Proposal 7: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

2.2 CHO with candidate SCGs

- RLF report enhancement

For CHO with candidate SCG(s) mobility failure, it is with high possibility that the CHO and/or CPAC execution condition configurations are not properly configured, considering CHO candidate cells and associated execution conditions have been included in RLF report for CHO only mobility failure optimization, thus, for CHO with candidate SCG(s), some CPAC configuration information (e.g., CPAC candidate cells and associated execution conditions) should also be included in RLF report together with CHO configuration information for the CHO with candidate SCG(s) failure optimization.

Proposal 8: Enhance RLF report for CHO with candidate SCGs to include the CPAC configuration information, i.e., CPAC candidate cells and CPAC execution conditions.

Besides the CPAC configuration information, the execution condition fulfilled situation of each candidate PSCell (e.g., first fulfilled event and the time duration between two events fulfilled) should also be included in RLF report to help the network optimization for CHO with candidate SCG(s).

Proposal 9: Enhance RLF report for CHO with candidate SCGs to include the information for each candidate PSCell, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Furthermore, the association between CHO and CPAC is important for network to optimize the CHO with candidate SCG(s) configuration, thus, the association between CHO and CPAC should also be recorded and informed to network.

Proposal 10: Enhance RLF report for CHO with candidate SCGs to include the association between CHO and CPAC.

- SHR enhancement

In case of CHO with candidate SCGs, both PCell and PSCell are configured with execution conditions, and only the PCell and PSCell execution conditions are fulfilled at the same time, the UE triggers the CHO with candidate SCGs execution. If one of the execution condition (e.g., the execution condition of PSCell) is not fulfilled in a long time but another execution condition (e.g., the execution condition of PCell) is fulfilled, the UE will also not execute the CHO with candidate SCGs, which will cause the unexpected situation, i.e., the CPAC blocks the CHO execution, and vice versa. The ideal situation is the execution conditions of CHO and CPAC should be fulfilled as closely as possible in terms of time. A new trigger condition can be considered for successful CHO with candidate SCGs execution, e.g., the time difference between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold.

Proposal 11: A new trigger condition that “the time difference between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold” is introduced in SHR for successful CHO with candidate SCGs execution.

In case of successful CHO with candidate SCG(s) execution, but both SHR and SPR are triggered (e.g., both T310 in PCell and T310 in PSCell exceed a configured threshold), UE can store two successful report (SHR and SPR) separately. Considering that the two reports are related to the same one handover procedure, RAN2 needs to consider how to associate the two reports to network.

Proposal 12: RAN2 to consider how to associate the SHR and SPR for CHO with candidate SCG(s) execution:

- Option 1: one report to include both SHR and SPR information;

- Option 2: two reports for SHR and SPR separately, and the association information is added in the two reports.

3. Conclusion

According to the discussion in section 2, we propose:

LTM

Proposal 1: Include TCI state ID from LTM cell switch command in RLF report for RACH-less LTM failure.

Proposal 2: The legacy SHR trigger conditions can also be applied to LTM, i.e., T310 threshold, T312 threshold, and T304 threshold.

Proposal 3: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

Proposal 4: Enhance raPurpose in RACH report to support successful RACH-based LTM execution. The detail can be discussed further, e.g., whether to reuse legacy raPurpose or introduce a new raPurpose.

Proposal 5: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Proposal 6: RAN2 to discuss how to differentiate RACH information logging of network-based early TA acquisition from the legacy RACH information logging (e.g., RACH information for handover):

- Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

- Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

- Option 3: log the RACH information of all network-based early TA acquisition in a report other than RACH report, e.g., a new report or a current existing report.

Proposal 7: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

CHO with candidate SCGs

Proposal 8: Enhance RLF report for CHO with candidate SCGs to include the CPAC configuration information, i.e., CPAC candidate cells and CPAC execution conditions.

Proposal 9: Enhance RLF report for CHO with candidate SCGs to include the information for each candidate PSCell, i.e., first fulfilled event and time duration between two events fulfilled, if any.

Proposal 10: Enhance RLF report for CHO with candidate SCGs to include the association between CHO and CPAC.

Proposal 11: A new trigger condition that “the time difference between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold” is introduced in SHR for successful CHO with candidate SCGs execution.

Proposal 12: RAN2 to consider how to associate the SHR and SPR for CHO with candidate SCG(s) execution:

- Option 1: one report to include both SHR and SPR information;

- Option 2: two reports for SHR and SPR separately, and the association information is added in the two reports.

4. Reference

- Draft_RAN2_127b_Meeting_Report_v1.docx