3GPP TSG-RAN WG2 Meeting #128                                          R2-2409654

Orlando, USA, 18th – 22nd Nov, 2024

Source:     CATT

Title:       Discussion on UE capability on less than 5MHz (LS R4-2417119)

Agenda Item:	7.8.1

Document for:	Discussion and Decision

Introduction

According to RAN4 LS [1], RAN4 asks RAN2 to take into consideration RAN4 agreements and introduce necessary UE capability signaling updates to enable NR CA/DC operation with less than 5 MHz channel bandwidth. The relevant issues have been discussed in [POST127bis][011], in this paper we give our further views on how to address this RAN4 LS.

Discussion

According to RAN4 LS [1], UE needs to be able to indicate for each carrier in the CA/DC band combination whether it supports less than 5 MHz channel bandwidth operation for DL/UL.

The content of RAN4 LS [1] is as follows:

- UE needs to be able to indicate for each carrier in the CA/DC band combination whether it supports less than 5 MHz channel bandwidth operation for DL/UL - To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination UE needs to indicate UE capabilities defined for NR < 5MHz channel bandwidth in Rel-18 and the existing UE capability applicability restriction to single carrier needs to be removed. - UE also indicates BCS including support of less than 5MHz channel bandwidth - For BCS signalling the following framework is used: - Traditional BCSs (i.e., not BCS4 or BCS5): UE is expected to support 3MHz channel bandwidth in the band in the band combination if it indicates a corresponding BCS, which includes 3MHz channel bandwidth in RAN4 specifications. No signalling changes are expected by RAN4. - BCS4: if UE indicates support for BCS4 including a band that it also supports 3 MHz channel bandwidth for, then it shall support 3 MHz for CA for that band in the band combination. No signalling changes are expected by RAN4. - BCS5: if UE indicates support for BCS5 and minimum channel bandwidth of 3MHz as a part of BCS signalling, then it shall support 3 MHz for CA for that band in the combination. The existing BCS5 signalling framework needs to be modified to allow indication of 3 MHz as minimum channel bandwidth. - It should be possible for UE to indicate support of CA/DC with less than 5MHz channel bandwidth starting from Rel-18 (i.e., allow early implementation from Rel-18) RAN4 has not identified a need for new signaling element. The signalling details are up to RAN2. The specific UE feature groups for NR less than 5 MHz channel bandwidth, which have single carrier restriction are listed below: - support12PRB-CORESET0-r18 - support3MHz-ChannelBW-Asymmetric-r18 - support3MHz-ChannelBW-Symmetric-r18 - support5MHz-ChannelBW-20PRB-CORESET0-r18 - support12PRB-CORESET0-GSCN-41637-r18

Base on the LS, UE needs to indicate the support of less than 5 MHz bandwidth operation in the CA/DC band combination, and indicate BCS including support of less than 5MHz channel bandwidth. For BCS aspect, the RAN2 Postemail discussion [127bis][011] has achieved a common understanding to extend supportedMinBandwidthDL/UL-r17 for BCS 5 to include the 3MHz value. But, for the support of less than 5 MHz bandwidth in the CA/DC band combination, we think more discussion is needed.

Generally, existing capabilities for less than 5MHz is for single CC, while the desired new capability is FSPC (Feature Set Per CC) Level for supporting the operation in the CA/DC band combination. In the post email, three methods are proposed:

- Method 1: Extend the supportedBandwidthDL/UL

- Method 2: Add an indication for a 3M bandwidth (e.g. similar to the channelBW-90mhz)

- Method 3: The 3M would be determined based on the BCS of each BC for the CA/DC together with (support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18), for the single CC, it would be determined by the support3MHz-ChannelBW-Asymmetric-r18/ support3MHz-ChannelBW-Symmetric-r18. For the BCS5, extend the supportedMinBandwidthDL/UL-r17 to include 3M.

According to the companies’ views in this offline discussion, the majority view is to go with Method 1, i.e., to extend the supportedBandwidthDL/UL to indicate the 3MHz. But this capability is used to define the maximum bandwidth, if the maximum bandwidth is larger than 3M, UE will report supportedBandwidthDL/UL(-r18xy) with the other (larger) value, the NW has to determine whether the 3MHz is supported or not based on the per band and/or BCS capabilities. In this case we see two issues. The first one is that it’s not clear whether a UE supporting per band 3MHz can also support 3MHz in a BC concerning the same band.

Observation 1: it’s not clear whether a UE supporting 3MHz in a band can also support 3MHz in a BC concerning the same band.

The other issue is that it’s not clear whether the UE capability asymmetricBandwidthCombinationSet can be applied for a carrier in a band within a CA/DC band combination. As indicated in RAN1 feature list, there are two independent UE capabilities for 3MHz, i.e., symmetric and asymmetric UE capabilities for 3MHz. And it’s free for UE to indicate which to support or both are supported. It means in a FSPC level, this flexibility should be kept. For asymmetric 3MHz, if it can only rely on existing per band UE capability asymmetricBandwidthCombinationSet, we also need to check with RAN1/4 that if asymmetricBandwidthCombinationSet can be applied for a carrier in this band within a CA/DC band combination. If it cannot be referred to, it seems that a UE supporting symmetric 3MHz also supports asymmestric 3MHz considering 3MHz is added in supportedBandwidthUL.

Observation 2: it’s not clear whether the per band UE capability asymmetricBandwidthCombinationSet can be applied for a carrier in this band within a CA/DC band combination.

Instead, we prefer a concise method to support the 3MHz bandwidth in a per CC level in CA/DC band combination. Considering RAN2 already made the following agreement:

Agreements

1: 	For 3M channel bandwidth, both the CA/DC and single CC case would be considered from the Rel18.

The existing per band UE capability elements for 3MHz can be kept and the corresponding capability elements should be added in FSPC signaling, i.e.,

- Add support3MHz-ChannelBW-Symmetric-r18 and support12PRB-CORESET0-r18 in FeatureSetDownlinkPerCC

- Add support3MHz-ChannelBW-Asymmetric-r18 in FeatureSetUplinkPerCC

In this way it is simple and sufficient for network to understand whether the 3MHz bandwidth is supported or not for each carrier in CA/DC band combination, and this is also aligned with per band UE capability signaling for 3MHz while the flexibility of symmetric and asymmetric 3MHz UE capabilities are maintained.

Proposal 1: To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination,  make the following updates for UE capability signaling:

- Add support3MHz-ChannelBW-Symmetric-r18 and support12PRB-CORESET0-r18 in FeatureSetDownlinkPerCC.

- Add support3MHz-ChannelBW-Asymmetric-r18 in FeatureSetUplinkPerCC.

For other R18 capabilities mentioned by RAN4 LS for less than 5MHz bandwidth, i.e., support5MHz-ChannelBW-20PRB-CORESET0-r18 and support12PRB-CORESET0-GSCN-41637-r18, they are per UE capabilities and only applicable to n100. Since the corresponding FSPC UE capabilities for the support of 3MHz can be indicated seperately, the description on single CC restriction could be removed directly without misunderstanding between UE and network, so that these capabilities could be applied for CA/DC.

Proposal 2: Remove the single carrier restriction in the field description of support5MHz-ChannelBW-20PRB-CORESET0-r18 and support12PRB-CORESET0-GSCN-41637-r18.

Conclusion

According to the analysis in section 2, we have the following observations:

Observation 1: it’s not clear whether a UE supporting 3MHz in a band can also support 3MHz in a BC concerning the same band.

Observation 2: it’s not clear whether the per band UE capability asymmetricBandwidthCombinationSet can be applied for a carrier in this band within a CA/DC band combination.

And we propose:

Proposal 1: To declare the support of less than 5 MHz bandwidth operation in the CA/DC band combination,  make the following updates for UE capability signaling:

- Add support3MHz-ChannelBW-Symmetric-r18 and support12PRB-CORESET0-r18 in FeatureSetDownlinkPerCC.

- Add support3MHz-ChannelBW-Asymmetric-r18 in FeatureSetUplinkPerCC.

Proposal 2: Remove the single carrier restriction in the field description of support5MHz-ChannelBW-20PRB-CORESET0-r18 and support12PRB-CORESET0-GSCN-41637-r18.

Reference

- R4-2417119	LS on NR channel BW less than 5MHz