3GPP TSG-RAN WG2 Meeting #128	R2-2409670

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.8.4

Source: 	CATT, CBN

Title: 	Further discussion on support of broadcast service in NR NTN

Document for: 	Discussion and Decision

Introduction

In RAN2#127bis [1], the following agreement was made:

Agreements:

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this contribution, several remaining issues will be further discussed.

Discussion

Where to include the service areas: MCCH vs SIB

RAN2 agreed the service area IDs are included in MCCH, but where to contain the detailed service areas is still pending. The candidate options to contain service areas are MCCH, new SIB and existing SIB. Detailed analysis on these candidate solutions are provided as follows.

Why SIB is not suitable

Firstly, we would like to have a clarification on the cases of MBS service area update. From the perspective of MBS service level, the modification of MBS session’s service area is not likely to happen individually , i.e., once the MBS session starts, its service area is typically unchanged. Once the CN/AF initiates an MBS session, the corresponding intended service area is first determined and permitted by CN/AF, and is typically not changed during the whole MBS service duration afterward. This is to say, the modification of service area for an on-going MBS session already initiated by CN is unlikely to happen, at least a corner case to consider (if really any).

In the case of NTN MBS, the cause for service area update may only be the addition or removal of the MBS session(s) of this cell. In this situation, the UE still needs to re-acquire the MBS session configuration in the updated MCCH.

Observation 1: The service area of an MBS session will not be changed for an on-going MBS service already initiated (at least a corner case, if any). Service area update of an NTN cell is only likely to happen for MBS session addition/removal, in which case the UE still needs to re-acquire the updated MCCH.

Based on observation 1, containing the service areas themselves in the SIB has very limited benefit, i.e., only in case the UE is not in coverage of any service area broadcast in SIB upon MCCH update notification, can it avoid re-acquire the updated MCCH. Otherwise, as long as the UE is in coverage of any service area broadcast by the cell, regardless of whether the service area is its intended service area or not, it anyway needs to re-acquire the MCCH when it receives MCCH update notification. This is because it cannot tell whether it is within the intended service area for any of its interest MBS session, until it obtains the mapping between service area ID and TMGI now agreed to be included in MCCH.

Observation 2: Due to last-meeting agreement (i.e. including service area ID in MCCH), if service areas were contained in (new or existing) SIB, the UE should inevitably re-acquire MCCH upon MCCH update notification, as long as it is within any service area (regardless for its intended service or not) broadcast by the SIB. This brings rather marginal benefit for UE implementation simplification upon MCCH update.

From the message size perspective, SIB is also not suitable to contain the service areas. In the last meeting, contribution in [2] provides a rough calculation on the size of service area and left space in SIB20. It was justified that there is not enough space in SIB20 to contain the service area. Even if a new SIB is defined for that purpose, there is still SI size limitation issue to contain the service areas. That is mainly because we agreed to support polygon format for service area, and each polygon may need space up to 720 bits (with the calculation given in Annex A). The upper bound of SI message, i.e., 2976 bits [3], is too limited with considering the size of polygon format.

NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

Observation 3: Especially when the service area is in the format of polygon, including service areas in (new or existing) SIB faces SI size limitation issue, which results in the consequence that not sufficient number of MBS services can be supported in an NR NTN cell.

Additionally, if the mapping between service area IDs and MBS services is included in MCCH while the service areas themselves are contained in SIB, update misalignment between MCCH and SIB may occur.

Figure 1: BCCH modification period and MCCH modification period

Take Figure 1 as an example, when the UE has already acquired the updated SI which includes the updated service area (e.g. in BCCH modification period n+1) but the MCCH modification period (e.g. m+2) has not arrived, there is a mismatch duration (e.g. from the start of the BCCH modification period n+1 to the start of the MCCH modification period m+2), where the UE will use the new service area to associate with each MCCH session, thus not following the intention of the network.

Company may propose to align the BCCH modification period with the MCCH modification period. However, this is difficult to achieve: as a reference to TS 38.331 [3], the MCCH modification period ranges from 2 to 65536 radio frames, while the BCCH modification period ranges from 64 to 4096 radio frames. The MCCH modification period has a more flexible value range than the BCCH, in order to adapt MBS service requirement. From NW perspective, it is thus not acceptable to align the MCCH modification period with BCCH without considering the MBS service requirement. Even if BCCH and MCCH can be updated at the same time by network implementation, there may not be benefit for UE to obtain updated BCCH and MCCH at the same time (which may lead to more complication for UE implementation), especially considering the different occasions in frequency domain for BCCH reception and MCCH reception.

mcch-ModificationPeriod-r17          ENUMERATED {rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256,

rf512, rf1024, rf2048, rf4096, rf8192, rf16384, rf32768, rf65536}

modificationPeriodCoeff         ENUMERATED {n2, n4, n8, n16},

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

Another potential method which does not need such modification period alignment is to specify that the UE uses the updated MCCH and SIB, only if it acquires both updated MCCH and SIB. But again, this method introduces additional specified UE behaviours, which we do not recommend.

Observation 4: If the service areas are contained in (new or existing) SIB, misalignment between MCCH and SIB update may occur. This may require additional complexity on network implementation and/or UE behaviours to resolve this misalignment.

Contain the service areas in MCCH

As per above analyses, there is marginal benefit if the service areas is contained in (new or existing) SIB, but many additional problems need to be solved. By contrast, if the service area is contained in MCCH, the design is not only more consistent with the logic of MBS design, but able to avoid extra problems to be caused as well. Hence, we propose to contain the service areas in the message MBSBroadcastConfiguration on the MCCH

Proposal 1: The service areas are provided via MBSBroadcastConfiguration message on the MCCH.

How to avoid MCCH acquisition when out of service area

As mentioned by observation 1, the service area of a certain MBS session is not likely to change when the MBS service in on-going. Hence, once the UE acquires the MBS session ID and corresponding intended service area, it could store the information for future use, e.g., to determine whether it needs to re-acquire updated MCCH, when receiving MCCH change notification.

This approach is similar with what has been specified for MBS in TN cells. In legacy MBS mechanism, according to TS 38.331 [3] (marked in yellow in the box below), if UE interested to receive any MBS broadcast service and the cell is providing SIB20 for any MBS broadcast service, the UE shall acquire MCCH upon entering the cell. The intention is that the UE could determine whether the cell is broadcasting the service it is interested in, by checking whether the TMGI of this service is broadcast in MCCH. One may wonder whether the UE could use the stored service area in USD to avoid MCCH acquisition, when it is outside the service area of the MBS service it interested in. In TS 38.331 [3], only one NOTE is added for that (marked in pink in the box below). That is, in legacy MBS broadcast mechanism, how to enable the UE to avoid (re-)acquiring the MCCH outside the intended service area is up to UE implementation relying on the service area in USD, rather than depending on any specified UE behaviour in normative text.

#### 5.9.2.2 Initiation A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s). NOTE 1: It is up to UE implementation how to address a possibility of the UE missing an MCCH change notification. NOTE 2: It is up to UE implementation to use the cell/tracking area list in the USD to avoid acquiring the MCCH when the UE is outside the MBS service area of the MBS broadcast service. Unless explicitly stated otherwise in the procedural specification, the MCCH information acquisition procedure overwrites any stored MCCH information, i.e. delta configuration is not applicable for MCCH information and the UE discontinues using a field if it is absent in MCCH information.

Observation 5: According to TS 38.331, the problem of acquiring unnecessary MCCH when outside the service area also existed in legacy MBS broadcast mechanism, and how to address/alleviate this problem is left to UE implementation relying on the service area in USD, but not depending on any specified UE behaviour in normative texts.

For the MBS broadcast service in NTN scenario, the only different is that the service area now is acquired and stored by reading MCCH upon entering the cell, rather than being acquired via higher layer signalling and stored in USD. More specifically, when UE enters an NTN cell, it shall read the MCCH according to the current Spec and acquire the service areas of MBS sessions. Once the UE acquires the service area info in MCCH, it could store this information to determine whether to re-acquire up-to-date MCCH, when receiving MCCH change notification later. That says, the UE is not required to acquire the MCCH when it is outside any service area of its interested services as determined based on its stored MCCH information. How to use the stored service area to avoid re-acquiring MCCH, as illustrated above, can be up to UE implementation, and this does not need to be specified in normative text.

Proposal 2: It is up to UE implementation how to use the stored service area info acquired from the MCCH to avoid acquiring the MCCH when the UE is outside the intended service area of the MBS broadcast service.

Service continuity enhancements

In RAN2#127bis, MBS service continuity enhancements in the context of NTN were also discussed. From our perspective, these enhancements are not essential and have limited benefits.

Potential enhancements or not on service area for FSAI

Companies proposed to introduce service area for FSAI, so that the UE could handle the frequency priority with the consideration of whether it is within the service area of neighbour frequencies/cells. However, it is hard to introduce the service area for FSAI into SIB21 based on the current signalling structure.

According to TS 38.331 [3], the FSAI list in SIB21 is configured in a per frequency manner.

SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 }

In addition, according to TS 29.571 [4], one MBS session is associated with a list of FSAI, and one FSAI is allowed to be associated with multiple MBS session.

- 5.9.4.6 Type: MbsSession Table 5.9.4.6-1: Definition of type MbsSession Attribute name; Data type; P; Cardinality; Description. mbsSessionId; MbsSessionId; C; 0..1; MBS session identifier (TMGI and/or SSM, and NID for an SNPN) (NOTE 1). [unrelated part is omitted]. mbsFsaIdList; array(MbsFsaId); O; 1..N; List of MBS Frequency Selection Area Identifiers, for a broadcast MBS session. This attribute may be present if the "serviceType" attribute indicates a broadcast MBS session..

Above evidence implies that, if service areas were to be introduced specifically for FSAI in SIB21, the service area should be provided per FSAI and per MBS session. Still due to the SI size limitation, this brings excessively huge singling overhead for the system information, leading such signalling design to be unacceptable.

Observation 6: If service areas were to be introduced specifically for FSAI in SIB21, they should be provided per FSAI and per MBS session. This brings excessively huge signalling overhead for the system information, and the signalling design is unacceptable still due to the SI size limitation concern.

Having checked existing FSAI mechanisms, we found that the intention raised by company might have already been realized, based on current USD info and the service area configuration to be introduced in MCCH. Specifically:

- For a certain MBS session, by upper layer procedure the UE acquires the FSAI list of an MBS session ID [4][5], and by SIB21 the UE acquires related frequencies of FSAI. This means, the UE, from current USD and SIB21, can know the frequency applicable for each MBS session.

- From the service area configuration to be introduced in MCCH, the UE can know the intended service area for each MBS session

Then combing the above two aspects, the UE could determine whether it may prioritize the frequency(ies) associated with its intended service area (as per bullet "i"), based on whether it is still located within the its intended service as determined by its current GNSS location and bullet ii. That is, the UE may further prioritize a frequency(ies) in SIB21 for its interested service, if it finds itself located in the intended service area. The information in the exiting USD and the service area info to be introduced in MCCH are already sufficient, and additional ASN.1 impact is not needed.

Proposal 3: Do not introduce service area information specific for FSAI in SIB21.

Also, since anyway this MBS specific frequency prioritization behaviour is up to UE implementation (with "... the UE may consider that frequency to be the highest priority during the MBS broadcast session…" in TS38.304), we think RAN2 can also confirm that the UE can still considers whether to prioritize a frequency providing its interested MBS service (based on existing SIB21 and USD), as per the current procedure in TS 38.304, and discuss whether any informative text is needed to further clarify that the UE may also take into account whether it is within the intended service area based on the service area information in MCCH.

Proposal 3a: The UE can still determine whether to prioritize the frequency in SIB21 for its interested MBS service(s) (based on the existing SIB21 and USD) as per the current procedure in TS 38.304. RAN2 discusses whether any informative text is needed in the Spec to further clarify that UE may take into account whether it is located in the intended service area based on service area information in MCCH (if provided in NR NTN).

Introduce service area for neighbour cell list in MCCH

Companies proposed to introduce service area for neighbour cells in MCCH, so that the UE could determine whether to request unicast before reselecting to a neighbour cell.

However, introducing service area for the neighbour cell will increase the signalling overhead, especially if the service areas are contained in SIB. The intention raised by company can also be realized by implementation without ASN.1 impact. In detail, for a certain MBS session ID, the intended service area informed by the network is not limited to the intended service area within the serving cell, but may also include the service areas involved by a number of cells. The UE could determine whether a neighbour cell is providing this MBS session according to the service area configurations and its GNSS location. If the UE is still within the service area and the reselected cell is within the neighbour cell list provided in MCCH, the UE considers the neighbour cell providing this MBS session. No need to further introduce per neighbour cell service area distinction, and thus no additional ASN.1 impact is needed.

Observation 7: For a certain MBS session, the intended service area informed by the network in MCCH is not limited to the coverage within the serving cell, but can also involve those within the neighbour cells.

Also, we think whether/how the UE considers the service area related configuration in MCCH in addition to the neighbour cell list in MCCH can still be left to UE implementation, when it determines whether the reselected cell providing its interested MBS session at its location.

Proposal 4: Do not introduce service area per neighbour cell in MCCH.

Proposal 4a: It can be left to UE implementation whether/how the UE considers the service area related configured in MCCH for the MBS session, in addition to the neighbour cell list provided in MCCH, when the UE determines whether the reselected cell is providing its interested MBS session at its location. RAN2 discusses whether any informative text is needed in the Spec.

Conclusion

In this contribution, supportive of broadcast service via NR NTN is discussed with giving corresponding proposals:

Observation 1: The service area of an MBS session will not be changed for an on-going MBS service already initiated (at least a corner case, if any). Service area update of an NTN cell is only likely to happen for MBS session addition/removal, in which case the UE still needs to re-acquire the updated MCCH.

Observation 2: Due to last-meeting agreement (i.e. including service area ID in MCCH), if service areas were contained in (new or existing) SIB, the UE should inevitably re-acquire MCCH upon MCCH update notification, as long as it is within any service area (regardless for its intended service or not) broadcast by the SIB. This brings rather marginal benefit for UE implementation simplification upon MCCH update.

Observation 3: Especially when the service area is in the format of polygon, including service areas in (new or existing) SIB faces SI size limitation issue, which results in the consequence that not sufficient number of MBS services can be supported in an NR NTN cell.

Observation 4: If the service areas are contained in (new or existing) SIB, misalignment between MCCH and SIB update may occur. This may require additional complexity on network implementation and/or UE behaviours to resolve this misalignment.

Observation 5: According to TS 38.331, the problem of acquiring unnecessary MCCH when outside the service area also existed in legacy MBS broadcast mechanism, and how to address/alleviate this problem is left to UE implementation relying on the service area in USD, but not depending on any specified UE behaviour in normative texts.

Observation 6: If service areas were to be introduced specifically for FSAI in SIB21, they should be provided per FSAI and per MBS session. This brings excessively huge signalling overhead for the system information, and the signalling design is unacceptable still due to the SI size limitation concern.

Observation 7: For a certain MBS session, the intended service area informed by the network in MCCH is not limited to the coverage within the serving cell, but can also involve those within the neighbour cells.

Proposal 1: The service areas are provided via MBSBroadcastConfiguration message on the MCCH.

Proposal 2: It is up to UE implementation how to use the stored service area info acquired from the MCCH to avoid acquiring the MCCH when the UE is outside the intended service area of the MBS broadcast service.

Proposal 3: Do not introduce service area information specific for FSAI in SIB21.

Proposal 3a: The UE can still determine whether to prioritize the frequency in SIB21 for its interested MBS service(s) (based on the existing SIB21 and USD) as per the current procedure in TS 38.304. RAN2 discusses whether any informative text is needed in the Spec to further clarify that UE may take into account whether it is located in the intended service area based on service area information in MCCH (if provided in NR NTN).

Proposal 4: Do not introduce service area per neighbour cell in MCCH.

Proposal 4a: It can be left to UE implementation whether/how the UE considers the service area related configured in MCCH for the MBS session, in addition to the neighbour cell list provided in MCCH, when the UE determines whether the reselected cell is providing its interested MBS session at its location. RAN2 discusses whether any informative text is needed in the Spec.

Reference

- R2-2409213	Report from Break-out session on NR-NTN and IoT-NTN	Session Chair (ZTE Corporation)

- R2-2407961	Discussion on support of broadcast service in NR NTN	CATT	discussion	NR_NTN_Ph3-Core

- 3GPP TS 38.331: "NR: Radio Resource Control (RRC) protocol specification".

- 3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces".

- 3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

- Annex A: IE size of service area

1) Reference location + radius (Refer to TS 38.331 and TS 37.355, take format of TN area as example)

CoverageAreaInfo-r18 ::=       SEQUENCE {

tn-AreaId-r18                  TN-AreaId-r18,

tn-ReferenceLocation-r18       ReferenceLocation-r17,  	--	48 bits

tn-DistanceRadius-r18          INTEGER(0..65535)    		--	16 bits

}

48+16=64 bits

#### –	ReferenceLocation

The IE ReferenceLocation contains location information used as a reference location. The value of the field is same as Ellipsoid-Point defined in TS37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

ReferenceLocation information element

-- ASN1START

-- TAG-REFERENCELOCATION-START

ReferenceLocation-r17 ::= OCTET STRING

-- TAG-REFERENCELOCATION-STOP

-- ASN1STOP

#### –	Ellipsoid-Point

The IE Ellipsoid-Point is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

2) Polygon (Refer to TS 37.355)

#### –	Polygon

The IE Polygon is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints

PolygonPoints ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

48*(3~15)=144~720 bits