3GPP TSG-RAN WG2 Meeting #128	R2-2409671

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Further discussion on downlink coverage enhancements

Document for: 	Discussion and Decision

Introduction

This contribution will discuss several leftover issues on downlink coverage enhancement.

Discussion

2.1	Impacts or not on SMTC/gap due to beam hopping/largerer SSB periodicity

According to the current Spec, a maximum of 4 SMTC windows per frequency can be configured for NTN deployments [1]. In the previous meetings, there were proposals to further extend the SMTC number of one frequency layer for DL coverage enhancement scenario; finally the following FFS point was recorded in RAN2#127 meeting [2]:

3.	RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity

The main argument was that a cell has 6 neighbour cells typically, and a separate SMTC may be needed for each of the neighbour cell, because the offsets of neighbour cells' SMTCs may be different from each other, along with the beam hopping.

However, if the scenario that one frequency serves 7 adjacent footprints exists, this issue can be solved by NW implementation from our perspective. For example, the NW can transmit the SSBs for more than one cells which are not adjacent at the same time, even if those cells are served by different beam footprints. Then, from the SMTC configuration perspective, these neighbour cells share the aligned SMTC offset. An example is given in the following figure: for the SMTC with the offset being 5 ms, the beams for the three cells in blue can be provided by the satellite at the same time; similarly, for the SMTC with the offset being 25 ms, the beams for the three cells in pink can also be provided at the same time. In this instance, for the UE within the serving cell at the centre, the SSBs of its neighbour cells can be covered by two SMTC windows with offset 5 ms and 25 ms separately, in which case 2 SMTC configurations are already sufficient.

Even if these neighbour cells belong to different satellites and the offsets are not coordinated among satellites, 4 SMTC windows per frequency seem sufficient as well. If the SMTC number and duration do not need to be extended, the gap configuration does not need to be enhanced, either.

Observation 1: For the DL coverage enhancement scenario with beam hopping, it is up to NW implementation to ensure that up to 4 SMTCs and 4 measurement gaps as in current specification are sufficient, e.g. providing the SSBs of more than one neighbour cells at the same time within the same SMTC window.

Proposal 1: Do not extend the number and duration of SMTC/measurement gap due to DL coverage enhancement feature.

2.2	Cell level or beam level DRX/DTX

In previous meetings, companies proposed to introduce DRX/DTX like mechanism to indicate the activated time of a cell/beam. RAN2 has received a reply LS from RAN1 [3].

RAN1 thanks RAN2 for their questions on the DL coverage enhancements and would like to provide the responses to the following questions. RAN1 will try to provide a further response at a later time. Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values?. Answer to Q1: According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion. Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE.. Answer to Q4: Beam footprint status in terms of "off", "common messages only" and "active traffic” in one of the RAN1 agreements is only defined for the sake of system-level evaluation methodology.

According to RAN1’s answer on Q4, the beam footprint status in terms of "off", "common messages only" and "active traffic” is only defined for evaluation. Such beam footprints mentioned in RAN1 LS could be physical beams not visible to UE, not necessarily the NR beams, and thus RAN2 should not make any cell/beam level DRX/DTX mechanism design depending on such beam status. We propose to postpone the discussion on cell level or beam level DTX/DRX until receiving RAN1 reply on SSB periodicity and beam status design for the UE-visible beams (i.e. NR beams).

Proposal 2: Postpone the discussion on cell level or beam level DTX/DRX until receiving further RAN1 reply on SSB periodicity and beam status design for UE-visible beams (i.e. NR beams).

2.3	Whether to support enhancement to down-prioritize cells operating with DL CE

In RAN#127bis, a new barring bit specific to Rel-19 NTN DL CE was agreed. Based on that, companies having proposed this enhancement intended to prevent legacy NR NTN UEs from performing cell re-selection to the cell operating with DL CE.

Agreements:

1.	If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE)

2.	If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit)

3.	(also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

Since RAN2 already agreed in RAN2#127 [2] that the existing NTN barring bits can be relied on, if there is a need to bar legacy UEs' access attempt, it might already be sufficient and no other optimization needs further pursuing on measurements part from our perspective.

Proposal 3: Do not introduce enhancement to allow UEs not supporting DL CE to down-prioritize/prevent re-selection to cells operating with DL CE.

Conclusion

In this contribution, DL coverage enhancement is further discussed with the corresponding observations and proposals listed below:

Observation 1: For the DL coverage enhancement scenario with beam hopping, it is up to NW implementation to ensure that up to 4 SMTCs and 4 measurement gaps as in current specification are sufficient, e.g. providing the SSBs of more than one neighbour cells at the same time within the same SMTC window.

Proposal 1: Do not extend the number and duration of SMTC/measurement gap due to DL coverage enhancement feature.

Proposal 2: Postpone the discussion on cell level or beam level DTX/DRX until receiving further RAN1 reply on SSB periodicity and beam status design for UE-visible beams (i.e. NR beams).

Proposal 3: Do not introduce enhancement to allow UEs not supporting DL CE to down-prioritize/prevent re-selection to cells operating with DL CE.

Reference

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

- R2-2407573	Report from Break-out session on NR-NTN and IoT-NTN

- R1-2407538	Reply LS on DL coverage enhancements.