3GPP TSG-RAN WG2 Meeting #128	R2-2409674

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.9.2

Source: 	CATT

Title: 	Discussion on RAN2 impacts due to the satellite ID list from MME in S&F operation

Document for: 	Discussion and Decision

Introduction

This contribution will discuss what information the eNB needs to provide in SIB with respect to the indicated satellite IDs by MME, and potential impacts on the cell (re-)selection procedure.

Discussion

2.1	 Progress of SA2

Based on the conclusion in TR 23700-29 [1], for both split MME architecture and full CN on-board architecture, SA2 agreed that MME can provide a list of Satellite IDs which identify the satellites the UE needs to communicate with. The detailed impacts agreed by SA2 are list as follows:

- For split MME architecture

Under this architecture, Attach/TAU procedure cannot be completed during the coverage of a single satellite. The MME can provide a list of satellite IDs, identifying a series of subsequent satellites over which the UE may resume the Attach/TAU procedure based on the RRC connections. The UE re-selects to the cell which is served by the satellite identified by the Satellite ID indicated by MME to re-attempt the Attach/TAU procedure. Related texts in the Conclusion of TR 23700-29 [1] are listed below, and these impacts are already specified in TS 23.401 by the agreed CRs [2][3].

With the following normative impacts: [unrelated part is omitted] 2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB. [unrelated part is omitted] 5) When the wait timer has expired given to the UE in step 2, if the UE has not successfully attached to another PLMN and the UE finds the cell which broadcast the Satellite ID valid to re-attempt the attach procedure, the UE re-sends the Attach or TAU Request message. 6) During the Attach or TAU procedure with the UE, the MME may also provide a list of Satellite IDs over which the UE may exchange the signalling and data, and a wait-timer that indicates to the UE the time it should wait before attempting signalling and data exchanges in those satellites. [unrelated part is omitted]

- For full CN on-board architecture

The MME can provide a list of satellite IDs over which the UE could perform MO/MT data/singling transmission with the CN. The UE attaches on, transfers data with and detaches from each satellite as required and as determined by the monitoring list. The UE uses the satellites in the list for MO/MT data/signalling transfer with the CN. Related texts in the Conclusion of TR 23700-29 [1] are listed below, and these impacts are also already specified in TS 23.401 by the agreed CRs [2][3].

The following option is agreed for supporting Store and Forward operation with a full CN onboard the satellite with the following (informative) principles: - The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. - The implementation of the proxies and the interface between them is out of 3GPP scope. - The UE attaches, transfers data (e.g. SMS, MO and MT data, etc.) and detaches from each satellite as required and as determined by the monitoring list. [unrelated part is omitted] - Depending on the deployment and implementation (i.e. outside the scope of 3GPP in this release), the HSSs on the satellites may be populated with subscription data either for only the UEs that may access satellite or all UEs that may access the satellite. [unrelated part is omitted] With the following normative impacts: - Store and forward is only supported by EPS. - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data. - The UE needs to be aware that a satellite supports S&F mode. NOTE 9: How the UE is aware that a satellite supports S&F mode of operation depends on RAN. - A UE may be rejected if the satellite cannot support the UE at this time. The attach reject may provide a timer for the time the UE should wait before reattempting and S&F monitoring list which the UE can attempt attach again.

For both of these two architectures, MME can provide a list of Satellite IDs to UE for monitor or access. The signalling format of the Satellite ID aligns with the satellite ID indicated in the system information.

Observation 1: For both split MME architecture and full CN on-board architecture, SA2 agreed that MME can provide a list of Satellite IDs for UE to monitor and access, and the format of the Satellite ID aligns with the satellite ID indicated in the system information.

The reason for CN to signal a satellite ID list for the UE is that the CN can only transfer UE context and/or MT data/singling on these indicated satellites, and only if the UE connects with these satellites, can it finish the Attach/TAU procedure and avoid missing MT data/singling.

Observation 2: The reason for CN to signal a satellite ID list for the UE is that the CN can only transfer UE context and/or MT data/singling on these indicated satellites; only if the UE connects with these satellites, can it finish the Attach/TAU procedure and avoid missing MT data/singling.

Based on the progress of SA2, in the rest of this contribution, we will analyse potential impacts on RAN2 specifications, including what information the eNB provides in SIB with respect to the indicated satellite IDs by MME and the corresponding UE behaviours.

2.2	 How to use assistance info of satellite ID list from MME in AS

Considering the use case of the S&F operation, it is highly likely that the satellites indicated by MME are not the satellites which provide neighbour cell coverage (i.e. not the next satellite that provides the immediate neighbour cell coverage). Similar to the discontinuous coverage mechanism, if the UE could start/stop idle mode tasks related to S&F operation upon the indicated satellite coming and leaving, it is beneficial for UE power saving. In more detail, if the UE determines that it is out of coverage of any target satellite indicated by MME, the UE operating in S&F mode need not perform idle mode tasks (e.g. cell (re)selection, paging monitoring, etc.) related to S&F operation. Only when it is in coverage of a target satellite indicated by MME, shall the UE perform idle mode tasks related to S&F operation. How the UE determines whether it is in coverage or out of coverage of the indicated satellite(s) using satellite assistance information can be left up to UE implementation (similar to legacy discontinuous coverage mechinism).

Proposal 1a: If the UE determines that it is out of coverage of all target satellite(s) indicated by MME, the UE operating in S&F mode need not perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.). The determination of "in coverage/out of coverage" of a target satellite is up to UE implementation.

Proposal 1b: Once the UE determines that it is in coverage of a target satellite indicated by MME, the UE shall perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.).

2.3	 How to signal assistance info associated with the satellite ID list from MME in Uu signalling

It is noticed from SA2 design that the MME only provides Satellite IDs to UE for monitor and access without the related satellite assistance information. So UE still needs to acquire the satellite assistance information corresponding to these MME-indicated satellites from SIB broadcast by eNB. RAN2 needs to discuss what specific assistance information the eNB should provide to UE for the MME-indicated Satellite IDs.

Obviously, it would be helpful if eNB provides information for UE to determine the coverage time of these indicated satellites by MME, so that unnecessary power consumption can be avoided. In Rel-17, SIB32 was introduced for the prediction of discontinuous coverage, where ephemeris info and footprint info are provided [4]. As mentioned above, since the satellites indicated by the MME to provide the subsequent service may not be the next satellite which provides the same coverage as the current satellite, the satellite assistance information needed for S&F operation is logically similar with that used for discontinuous coverage, so that similar signalling format can be reused.

SatelliteInfo-r17 ::= SEQUENCE { satelliteId-r17 INTEGER (0..255), serviceInfo-r17 SEQUENCE { tle-EphemerisParameters-r17 TLE-EphemerisParameters-r17 OPTIONAL, -- Need OR t-ServiceStart-r17 TimeOffsetUTC-r17 OPTIONAL -- Need OR }, footprintInfo-r17 SEQUENCE { referencePoint-r17 SEQUENCE { longitude-r17 INTEGER (-131072..131071), latitude-r17 INTEGER (-131072..131071) } OPTIONAL, -- Need OR elevationAngles-r17 SEQUENCE { elevationAngleRight-r17 INTEGER (-14..14), elevationAngleLeft-r17 INTEGER (-14..14) OPTIONAL -- Need OP } OPTIONAL, -- Need OR radius-r17 INTEGER (1..256) OPTIONAL -- Need OR } }

However, it is better to introduce a separate list specifically for S&F usage, instead of reusing the existing list in SIB32 directly. That is because discontinuous coverage and S&F are corresponding to different functionalities and serving different purposes. For discontinuous coverage, the NW tends to indicate the first upcoming satellite which will provide same coverage as the current cell in future. It only cares about when the satellites could provide RRC connection with Uu coverage to UE. But for S&F, the satellite indicated by MME is the satellite which may provide service (e.g. attach procedure) and CN level connectivity for the specific UE, and is designated by the CN; so it may not be exactly the next upcoming satellite which will provide coverage for the UE.

Besides, the UE behaviour for the operation towards these two kinds of satellites is different either. While in coverage of any satellites in SIB32 for discontinuous coverage purpose, the UE shall perform idle mode tasks related to NTN. However, for the satellites indicated by MME for the S&F purpose, the UE needs to search for the specific satellites. Only in coverage of these specific satellites, does the UE need to perform related tasks. Based on the current procedure in [5], if current SIB32 were to be reused for S&F use, the UE which has indicated satellite ID list from MME might still perform discontinuous coverage mechanism besides S&F operation, i.e., performing idle mode tasks related to NTN for all the satellites broadcast in SIB32, even if the satellite is not belonging to the Satellite ID list provided by MME. This will bring unnecessary power consumption.

If SystemInformationBlockType32 has been received and if the UE has determined that it is out of coverage using available satellite assistance information (e.g. ephemeris parameters and coverage parameters in current or previously received SystemInformationBlockType32, SystemInformationBlockType31, t-Service in SystemInformationBlockType3 or other parameters), the AS configuration (e.g. priorities provided by dedicated signalling and logged measurements) is kept, but the UE need not perform any idle mode tasks related to NTN. It is up to UE implementation to handle running timers. The detection of out of coverage using satellite assistance information is up to UE implementation and once in NTN coverage the UE shall perform all idle mode tasks related to NTN. If SystemInformationBlockType32 includes carrierFreqList the UE may store and use this information for the cell selection process when UE resumes the idle mode tasks related to NTN once in NTN coverage.

Furthermore, the maximum number of the satellites that can be indicated in the satellite list for discontinuous coverage in SIB32 is 4. If this satellite list were directly reused to provide IDs of several satellites not limited to the next hop, this maximum number would be too limited.

Observation 3: If the SatelliteInfo-r17 in SIB32 were directly reused for S&F operation, the number of the satellites may be limited, and the configured satellites might not meet the S&F requirement of the UE, thus bringing unnecessary UE power consumption.

Hence, it is better to introduce a separate list to indicate the assistance information of the satellite(s) indicated by MME for S&F purpose. But the format of SatelliteInfo-r17 in SIB32 can be taken as the baseline for the signalling design.

Proposal 2: Introduce a separate list in system information to provide the satellite assistance information for S&F operation, i.e., used for UE to predict the coverage time of the satellites indicated by MME.

Proposal 3: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

2.4	 How cell (re)selection procedure is impacted by the satellites indicated by MME

As mentioned by observation 2, the reason for CN to signal satellite IDs for the UE is that the CN can only transfer UE context and/or MT data/singling on these indicated satellites. In this situation, the UE should camp on the cells of these satellites to ensure the completion of the signalling procedure and avoid missing any import DL data.

The UE behaviours under these two network architectures will be as follows respectively:

- For split MME architecture, the UE should camp on the cells served by these indicated satellites. When camping on these cells, the UE performs paging monitoring for possible DL data/signalling, initiates RRC connection setup/resume procedure for possible UL data/signalling, etc.

- For full CN on-board architecture, the UE should also camp on the cells served by these indicated satellites. When camping on these cells, the UE may perform attach procedure according to the indication from upper layer at the first step. Then the UL/DL signalling/data transmission can be performed normally.

For both of these two architectures, a common issue is how the UE selects and camps on the cells belonging to the indicated satellites by the MME. However, even when the UE initiates cell (re)selection measurement based on the satellite coverage time, it is also possible that the UE detects satellites other than the ones indicated by MME. In this case, if the UE relies on the existing cell (re-)selection mechanism based on RSRP/RSRQ, it may select a cell which is not served by a satellite indicated by the MME to camp on and thus miss the MT data or signalling from the indicated satellites.

Observation 4: To avoid missing MT data and signalling, UE should camp on the cells belonging to the satellites indicated by MME (if detected). However, existing cell (re-)selection mechanism based on RSRP/RSRQ cannot ensure that.

RAN2 should consider some enhancements on cell (re-)selection with the consideration of the MME-indicated satellite IDs introduced by SA2. For example, UE considers the frequencies served by the indicated satellites as highest priority. As another example, the UE should only select one of the cells which belong to the indicated satellite by MME to camp on, taking precedence over the RSRP/RSRQ ranking.

Proposal 4: Enhance cell (re-)selection mechanism to ensure that UE operating in S&F mode (re-)selects a cell belonging to the satellites indicated by MME to camp on (e.g., prioritizing the frequencies served by the indicated satellites, selecting the certain cell which belongs to the indicated satellite to camp on, etc.).

Conclusion

In this contribution, some potential issues on supporting S&F operation are discussed with corresponding observations and proposals listed as follows:

Observation 1: For both split MME architecture and full CN on-board architecture, SA2 agreed that MME can provide a list of Satellite IDs for UE to monitor and access, and the format of the Satellite ID aligns with the satellite ID indicated in the system information.

Observation 2: The reason for CN to signal a satellite ID list for the UE is that the CN can only transfer UE context and/or MT data/singling on these indicated satellites; only if the UE connects with these satellites, can it finish the Attach/TAU procedure and avoid missing MT data/singling.

Observation 3: If the SatelliteInfo-r17 in SIB32 were directly reused for S&F operation, the number of the satellites may be limited, and the configured satellites might not meet the S&F requirement of the UE, thus bringing unnecessary UE power consumption.

Observation 4: To avoid missing MT data and signalling, UE should camp on the cells belonging to the satellites indicated by MME (if detected). However, existing cell (re-)selection mechanism based on RSRP/RSRQ cannot ensure that.

Proposal 1a: If the UE determines that it is out of coverage of all target satellite(s) indicated by MME, the UE operating in S&F mode need not perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.). The determination of "in coverage/out of coverage" of a target satellite is up to UE implementation.

Proposal 1b: Once the UE determines that it is in coverage of a target satellite indicated by MME, the UE shall perform idle mode tasks related to S&F operation (e.g. cell (re)selection, paging monitoring, etc.).

Proposal 2: Introduce a separate list in system information to provide the satellite assistance information for S&F operation, i.e., used for UE to predict the coverage time of the satellites indicated by MME.

Proposal 3: Take signalling format of SatelliteInfo-r17 in SIB32 as the baseline for assistance information of the satellite(s) indicated by MME for S&F operation.

Proposal 4: Enhance cell (re-)selection mechanism to ensure that UE operating in S&F mode (re-)selects a cell belonging to the satellites indicated by MME to camp on (e.g., prioritizing the frequencies served by the indicated satellites, selecting the certain cell which belongs to the indicated satellite to camp on, etc.).

Reference

- 3GPP TR 23700-29: "Study on integration of satellite components in the 5G architecture ".

- S2-2410990 Introduction to Split MME architecture.

- S2-2410991 Support of Store and Forward Satellite Operation.

- TS 36.331 Radio Resource Control (RRC).

- TS 36.304 User Equipment (UE) procedures in idle mode.