3GPP TSG-RAN WG2 Meeting #128                                          	R2-2409676

Orlando, 18.-22.11.2024

Source:	Telit Communications S.p.A., Novamint , Sateliot, Thales

Title:	Further considerations on S&F operation from device perspective

Agenda Item:	8.9.2

Document for:	Discussion and Decision

# Introduction

The present document is an update of R2-2408244 which was partially discussed in the last meeting. The updated document reflects the agreements made in the last meeting and captured for convenience below. However, several of the raised points of R2-2408244 need still to be discussed and decided.

In the last RAN2 meeting following agreements were made related to S&F Satellite operation.

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

Related to the discussion in the present document, it’s also relevant to consider the progress made in last SA2 meeting. In particular, the functional description of S&F satellite operation to be included in TS 23.401 is addressed in  23.401 CR3801 (S2-2410989). While the CR3801 was postponed because of still some controversial issues with respect to the handling of the “S&F Monitoring List”, there are some elements that matter to this discussion that were not considered controversial and are reflected in the following:

When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the onboard eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5] which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode, and if a UE is Enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update. NOTE 2: Some deployments cannot support all system features, for example they cannot support user plane establishment and user plane data transfer, when operating in S&F Mode in the split MME deployment described in Annex X. […] The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation. Editor’s Note: following text from the TR conclusions needs to be clarified: The estimated delivery time is the estimated time to send data from the UE to Gateway […] The EPS may expose whether a UE is in S&F Mode, and provide to the SCS/AS related timing information to guide the SCS/AS decision when to try to contact the UE. This is further described in clause 5.6.3.x of TS 23.682 [74].

# Discussion

A. Possible combinations of cellBarred-NTN flag and “S&F operation” indication

The indication for S&F called “S&F operation” was clarified in the last meeting and the following agreements were made with respect to legacy barring procedure and the use of the “S&F operation” indication:

Agreements:

….

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

These agreements result in several combinations of using the legacy cellBarred-NTN setting and the “S&F operation” indication.

Below table summarises the possible combinations and settings:

| cellBarred-NTN | “S&F operation” indication | Combination |
| --- | --- | --- |
| True | Not broadcasted | Combination (1) The cell operates in normal/default mode. The cell is barred for all NTN devices |
| False | Not broadcasted | Combination (2) The cell operates in normal/default mode. The cell is not barred |
| True | Present with setting “1” | Combination (3) The cell operates in S&F mode. The cell is barred for all NTN devices except for S&F-capable devices. |
| True | Present with setting “0”. | Combination (4) The cell operates in S&F mode. The cell is barred for all NTN devices and S&F-capable devices except for S&F-capable devices being already in connected mode. |
| False | Present with setting “1” | Combination (5) The cell operates in S&F mode. The cell is not barred. It’s up to network implementation/operator policy how to handle non-S&F-capable UEs. |
| False | Present with setting “0” | Combination (6) The cell operates in S&F mode. The cell is not barred but S&F-capable devices in idle mode should not use it. It’s up to network implementation/operator policy how to handle non-S&F-capable UEs. |

Combinations (1) and (2) correspond to the legacy behavior of the cellBarred-NTN flag when an NTN cell is not operating in S&F mode, which remains unchanged for Rel-19 UEs. These two combinations would also cope for scenarios where a Rel-19 UE would camp on a Rel-18 NTN cell or where a Rel-19 NTN cell simply would not support S&F operation mode for whatever reason.

Combinations (3) and (4) correspond to the case of a Rel-19 cell that operates in S&F mode and access to this cell it is restricted to only S&F-capable devices.

Combinations (5) and (6) correspond to the case of a of a Rel-19 cell that operates in S&F mode and access to this cell it’s not restricted to only S&F-capable devices, but non-S&F-capable devices can still camp on this cell and be served using legacy mechanisms.

Obsersavation1: Agreements in RAN2 allow for cover all possible combinations of “S&F operation” indication and legacy barring, hence no need to further discuss barring or S&F indication itself.

B. Mode switching

In case of switching the operation mode between RT and S&F or vice-versa the handling of ongoing or started service needs to be further discussed, i.e. whether the mode transition should keep in-order delivery of data packets for a started session or how to handle started TCP/IP sessions.

Scenarios to be clarified, i.e. when a session was started in S&F and the satellite switches to RT, is the in-order packet delivery ensured, or when a TCP/IP session was started in RT but not finalized until switching point is reached, how behavior is considered in those scenarios.

Of note is that the switching point between the S&F and RT modes should not necessarily coincide with the time that the feeder link gets activated or deactivated. For instance, in case of switching between S&F and RT mode, at the time that the feeder link gets activated, the satellite may download all previous stored data before switching to RT operation.

Proposal 1: RAN2 to further discuss expected behavior for ongoing sessions during mode transition.

In addition, it was discussed in the last meeting whether switching times from real-time support to S&F support and vice versa should be also indicated by the satellite. It could be important for the UE to know, and especially for the application on the UE, which of the two modes, i.e. S&F mode or RT mode, is available at a given time. The knowledge on S&F is important also for applications on the device, e.g. whether to use TCP/IP or UDP protocol for the transfer.

Depending on the actual scenario it may be beneficial or not, if such information is provided. For instance, if next switching time from S&F to RT is provided, a UE connected to a NTN cell with S&F mode activated may refrain from transmission during the S&F period and wait for RT operation to become available. In the same way, if next switching time from RT to S&F is provided, a UE involved in bi-directional data exchange session (e.g. a TCP session) may decide to smoothly tore down the session before S&F operation starts.

On the other hand, given that an indication on mode switching time has no meaning for devices not supporting Rel-10 S&F operation, those devices may be impacted as the devices supporting both modes may adapt their behavior and hence are also impacted. In addition such an indication is only meaningful, when the service switching takes place while the UE is still in coverage of said satellite/NTNcell. (E.g. smaller than tservice for earth fixed cells.)

Proposal 2: It is up to the network to provide an indication on operation mode switching times.

Proposal 3: If network operation switching times are provided, UE behavior is up to UE implementation and application needs.

C. Additional indications/information for S&F Satellite operation: Estimated delivery time and MO data transfer barring

S&F is especially suited for delay-tolerant traffic, however additional indication on expected delivery delay would be important for the device.

Delivery delay is at least the time it requires to reach the next feeder link connection opportunity but may even be larger if not all data can be offloaded during visibility of the feeder link from the satellite. Hence an upper boundary indication of S&F expected delivery time is important in general for the device.

The UE could also use the expected delivery time to evaluate whether the data transfer service using S&F mode is suitable or not for the service needs.

Proposal 4: For S&F Satellite operation expected maximum delivery time should be indicated to the UE. FFS if an indication of the expected maximum delivery time can be provided via system information or signaling via NAS suffices.

A SIB indication could be a coarse indication informing on minimum delivery delay for a UE acquiring the S&F service, the NAS indication is also considering specific UE aspects. The tradeoffs should be further discussed by RAN2.

FFS to consider a situation in which the satellite data storage is full (i.e. the satellite cannot accept more MO data from UEs) but the satellite can still perform control plane procedures (e.g. Attach, TAU) and MT traffic delivery.

Proposal 5: In the case that the satellite data storage is full (i.e. the satellite cannot accept more MO data from UEs) but the satellite can still perform control plane procedures (e.g.; Attach, TAU) and MT traffic delivery, an indication can be provided for UEs to refrain from sending MO traffic. FFS is the indication on maximum delivery time can be used also for this purpose.

D. Proposal for an Information Element

In light of recent agreements and the discussions above, a possible approach to provide the S&F operation mode indication along with additional information on switching times, maximum delivery time and MO data not allowed could be in the form of a new optional IE to be included in SIB31 (TBC):

- SIB31 (TBC)

  - (Optional) S&F_Operation_Mode:

    - S&F Operation mode: TRUE: S&F Activated, FALSE: S&F activated but barred for idle mode UEs)

    - (Optional) S&F Switching Time: Signals the time instant at which the S&F mode will be deactivated.

    - (Optional) S&F Maximum Delivery Time

    - (Optional) S&F MO Data Barred Flag: TRUE: MO data is barred, not allowed, FALSE or IE not present: MO is allowed.

NOTE: Based on RAN2 decision, above signaling or signaling in general does not cover the case of providing the switching time between normal mode and S&F mode as not being present in normal mode.

Proposal 6: Indication of S&F operational status can be an optional IE broadcast in SIB31 and include the following information:

- SIB31 (TBC)

  - (Optional) S&F_Operation_Mode:

    - S&F Operation mode: TRUE: S&F Activated, FALSE: S&F activated but barred for idle mode UEs)

    - (Optional) S&F Switching Time: Signals the time instant at which the S&F mode will be deactivated.

    - (Optional) S&F Maximum Delivery Time

    - (Optional) S&F MO Data Barred Flag: TRUE: MO data is barred, not allowed, FALSE or IE not present: MO is allowed.

NOTE: Based on RAN2 decision, above signaling or signaling in general does not cover the case of providing the switching time between normal mode and S&F mode as not being present in normal mode.

# Conclusion

Obsersavation1: Agreements in RAN2 allow for cover all possible combinations of “S&F operation” indication and legacy barring, hence no need to further discuss barring or S&F indication itself.

Proposal 1: RAN2 to further discuss expected behavior for ongoing sessions during mode transition.

Proposal 2: It is up to the network to provide an indication on operation mode switching times.

Proposal 3: If network operation switching times are provided, UE behavior is up to UE implementation and application needs.

Proposal 4: For S&F Satellite operation expected maximum delivery time should be indicated to the UE. FFS if an indication of the expected maximum delivery time can be provided via system information or signaling via NAS suffices.

Proposal 5: In the case that the satellite data storage is full (i.e. the satellite cannot accept more MO data from UEs) but the satellite can still perform control plane procedures (e.g.; Attach, TAU) and MT traffic delivery, an indication can be provided for UEs to refrain from sending MO traffic. FFS is the indication on maximum delivery time can be used also for this purpose.

Proposal 6: Indication of S&F operational status can be an optional IE broadcast in SIB31 and include the following information:

- SIB31 (TBC)

  - (Optional) S&F_Operation_Mode:

    - S&F Operation mode: TRUE: S&F Activated, FALSE: S&F activated but barred for idle mode UEs)

    - (Optional) S&F Switching Time: Signals the time instant at which the S&F mode will be deactivated.

    - (Optional) S&F Maximum Delivery Time

    - (Optional) S&F MO Data Barred Flag: TRUE: MO data is barred, not allowed, FALSE or IE not present: MO is allowed.

NOTE: Based on RAN2 decision, above signaling or signaling in general does not cover the case of providing the switching time between normal mode and S&F mode as not being present in normal mode.

# Reference

[1] R2-240XXX Report from Break-Out Session on NR NTN and IoT NTN

[2] TR 23.700-29 v1.0.0 [FS_5GSAT_Ph3_Arch]