3GPP TSG-RAN WG2 Meeting #128	R2-2409679

Orlando, United States, 18th – 22nd November, 2024

Agenda Item:	8.5.4

Source: 	OPPO

Title:  	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and Decision

# Introduction

In Rel-19 NES WID [1], the objective on adaptation of common signal/channel transmissions is captured as follows.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

This contribution would like to provide our considerations on this objective of Rel-19 NES.

# Discussion

# Adaptation of SSB in time domain

The SSB configuration in time domain includes SSB periodicity and time domain positions of the transmitted SSB in an SSB burst. For legacy RRC_IDLE/ RRC_INACITVE UE, CD-SSB is configured by the parameters ssb-periodicityServingCell and ssb-PositionsInBurst in ServingCellConfigCommonSIB in SIB1. Network may adapt SSB in time domain relying on system information modification, and a modification period is used as effective time. For legacy RRC_CONNECTED UE, both CD-SSB and NCD-SSB could be configured and adapted by network via dedicated RRC signalling.

Regarding the applicable scenarios of SSB adaptation in time domain, R1 has made the following agreements.

R1#118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

R1#118bis Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

For RRC_CONNECTED UEs, SSB adaptation in SCells is supported, but it is still FFS on SSB adaptation in PCell. For RRC_IDLE UEs, the adaptation of SSB in time domain has been ruled out in Rel-19 due to no consensus in R1. In our understanding, this situation of RRC_INACTIVE UEs is also similar, i.e., R1 has also no consensus on the support of SSB adaptation for RRC_INACTIVE UEs in Rel-19. From R2 perspective, it is necessary to confirm that SSB adaptation in time domain for RRC_INACTIVE UEs is also not supported.

- At least SSB adaptation in SCells is supported as concluded by R1, but it is still FFS on SSB adaptation of PCell for RRC_CONNECTED UEs.

- R1 has ruled out the support of adaptation of SSB for RRC_IDLE UEs in Rel-19.

- R2 confirms that SSB adaptation in time domain for RRC_IDLE and RRC_INACTIVE UEs is not supported in Rel-19.

In case that the applicable scenarios are narrowed down to RRC_CONNECTED UEs, relying on MAC CE to indicate SSB adaptation in time domain would be sufficient. Note that in NES O1, i.e., on-demand SSB SCell supporting, a new MAC CE has already been introduced to indicate on-demand SSB transmission in SCells, which supports SSB periodicity change in RRC_CONNECTED mode. And for the new MAC CE, it is not necessary to restrict it only for SCells. Therefore, it would be possible to have a unified MAC CE design for both on-demand SSB transmission in SCells (i.e., NES O1) and SSB adaptation in time domain (i.e., NES O3).

- For on-demand SSB SCell supporting (i.e., NES O1), a new MAC CE will be introduced to indicate on-demand SSB transmission in SCells, which supports SSB periodicity change in RRC_CONNECTED mode.

- R2 adopts a unified MAC CE design for both on-demand SSB transmission in SCells (i.e., NES O1) and SSB adaptation in time domain (i.e., NES O3), and wait for R1 for the applicability to PCell.

Another issue is L3 measurement impact due to SSB adaptation which is also raised by R4 LS R4-2416911. In legacy RRM, UE performs SSB measurement based on SMTC window. In case that the SSB transmission could be dynamically adapted with different periodicity, some SSBs would be out of the SMTC window configured by RRC signalling, the mismatch between adaptive SSB and legacy static SMTC window would cause UE RRM measurement to fail.

In NES O1, L3 RRM measurement issue due to SSB periodicity change in connected mode SCell is also being discussed, therefore a unified solution would be straightforward. We propose to allow L3 RRM measurement based on SSB pattern without SMTC in case of SSB adaptation, and no need to introduce new SMTC configuration. Please note that since in legacy RLM measurement is already based on SSB periodicity rather than SMTC, so that only RRM measurement needs to be considered.

- R2 adopts a unified L3 RRM measurement design for both on-demand SSB transmission in SCells (i.e., NES O1) and SSB adaptation in time domain (i.e., NES O3).

# Adaptation of RACH in time domain

In legacy, the time domain configuration of RACH occasion (RO) is configured using

- An index via SIB or dedicated RRC signalling, i.e., by prach-ConfigurationIndex, where the periodicity and time domain positions are mapped in TS 38.211.

- Besides, the parameters PRACH mask indexes (i.e., ra-ssb-OccasionMaskIndex) are also used to define RO associated with an SSB in which the MAC entity may transmit a Random-Access Preamble, and the mapping table is defined in TS 38.321.

Based on R1 minutes shown, R1 has made some agreements on this.

R1#118 Agreement For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE), - At least DCI based adaptation is supported. No introduction of new DCI format.

For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation, which is fully up to R1 to further specify the detailed design. No R2 work is foreseen.

- For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation.

Another leftover issue raised by companies in the email discussion is RACH bundling, i.e., given the PO bundles, whether to also adapt PRACH resources so that RO can be bundled with PO bundles. In this case, from network perspective, network can perform both paging transmission and RACH reception in a shorter time range to obtain additional power saving gain.

Currently, it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not. Further check and confirmation are needed. In our understanding, if there is a need to go for RACH bundling, it is preferred that R1 lead the work.

- Currently it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not.

- Up to R1 to evaluate and lead the work for RACH bundling if needed.

Most work on the RACH enhancement is within R1 scope. From R2 perspective, what we need to discuss includes whether the RACH adaptation mechanism applies to different types of RACH resources (e.g., CBRA/CFRA, 2/4-step RA type, feature/feature combination-specific RACH resources).

For CBRA/CFRA, we understand that it is not justified on the motivation to apply additional RACH resources to CFRA, since CFRA can rely on dedicated signalling of networks to assign specific RACH resources. So we propose to only apply the RACH adaptation mechanism to CBRA.

- It is not motivated to apply time-domain RACH adaptation for CFRA, since CFRA can rely on dedicated signalling of networks to assign specific RACH resources.

- Adaptation mechanism for additional RACH resources only applies to CBRA (i.e., not apply to CFRA).

In the current RACH framework, networks can configure cell-specific CBRA resources via BWP-UplinkCommon in SIB1, as highlighted below, including

- Rel-15 4-step RACH resources (i.e., rach-configCommon),

- Rel-16 2-step RACH resources (i.e., msgA-ConfigCommon-r16),

- IAB RACH resources (i.e., rach-ConfigCommonIAB-r16) introduced for IAB-MT in Rel-16 IAB,

- Additional RACH resources (i.e., additionalRACH-ConfigList-r17) introduced in Rel-17 for feature/feature combination-specific RACH partitioning framework, and

- RACH resources for the additional PCIs (i.e., additionalRACH-perPCI-ToAddModList-r18) introduced in Rel-18 MIMO.

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                            OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                           OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                           OPTIONAL,   -- Need M

...,

[[

rach-ConfigCommonIAB-r16            SetupRelease { RACH-ConfigCommon }                            OPTIONAL,   -- Need M

useInterlacePUCCH-PUSCH-r16         ENUMERATED {enabled}                                          OPTIONAL,   -- Need R

msgA-ConfigCommon-r16               SetupRelease { MsgA-ConfigCommon-r16 }             OPTIONAL    -- Cond SpCellOnly2

]],

[[

enableRA-PrioritizationForSlicing-r17 BOOLEAN                                          OPTIONAL, -- Cond RA-PrioSliceAI

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }     OPTIONAL, -- Cond SpCellOnly2

rsrp-ThresholdMsg3-r17              RSRP-Range                                         OPTIONAL, -- Need R

numberOfMsg3-RepetitionsList-r17    SEQUENCE (SIZE (4)) OF NumberOfMsg3-Repetitions-r17      OPTIONAL,  -- Cond Msg3Rep

mcs-Msg3-Repetitions-r17            SEQUENCE (SIZE (8)) OF INTEGER (0..31)                   OPTIONAL   -- Cond Msg3Rep

]],

[[

additionalRACH-perPCI-ToAddModList-r18  SEQUENCE (SIZE (1.. maxNrofAdditionalPRACHConfigs-r18)) OF RACH-ConfigTwoTA-r18

OPTIONAL, -- Cond 2TA-Only

(omit the irrelevant IEs)

]]

}

- The current cell-specific CBRA resource configuration includes: 1) Rel-15 4-step RACH resources, 2) Rel-16 2-step RACH resources, 3) IAB RACH resources, 4)	additional RACH resources for RACH partitioning, and 5) RACH resources for the additional PCIs.

As baseline, at least 4-step RA type needs to be supported. And we are open to also support 2-step RA type from R2 perspective, since no additional workload in R2 is foreseen. Considering that the support of 2-step RA may have impact on DCI design in R1, e.g., indicating 2-step and/or 4-step RACH type in DCI. Therefore, it is good to send LS to R1 to further check the support of 2-stpe RA.

- From R2 perspective, at least apply time-domain RACH adaptation to 4-step RA. Send LS (as in Annex) to R1 to check whether to apply to 2-step RA or not.

For IAB RACH resources, RACH resources for the additional PCIs, and additional RACH resources for feature/feature combination-specific RACH partitioning, we understand the simplest way forward is that we do not consider to apply the RACH adaptation mechanism for them. Otherwise, the impact and complexity on R1-designed DCI-based solution needs to be evaluated by R1. So it would be safer to also check with R1.

- From R2 perspective, not apply time-domain RACH adaptation for IAB RACH resources, RACH resources for the additional PCIs, or additional RACH resources for feature(-combination)-specific RACH partitioning. Send LS (as in Annex) to R1 to ask if any concern.

Another point is that within the Rel-15/16 4/2-step RACH resource configurations, it can also allow feature/feature combination-specific RACH partitioning, since featureCombinationPreamblesList is optionally present in RACH-ConfigCommon/RACH-ConfigCommonTwoStepRA-r16 for Rel-15/16 4/2-step RACH resource configurations respectively. We also hold the same position on this, i.e., not to consider it for the RACH adaptation mechanism.

- From R2 perspective, not pursue feature(-combination)-specific RACH partitioning for time-domain RACH adaptation. Send LS (as in Annex) to R1 to ask if any concern.

For Rel-19 UEs, both legacy RACH resources and additional RACH resources are available. Another issue is that whether to specify UE behaviours for Rel-19 UEs to choose between additional RACH resources and legacy RACH resources. For example, Rel-19 UEs prioritize to choose additional RACH resources, considering legacy RACH resources can only be used by legacy UE. We understand that which resources to choose from UE perspective depends on different purposes, e.g., RACH collision avoidance, RACH latency minimization, so it would be various rules and hard to specify the related UE behaviour. We propose to leave it to UE implementation.

- Up to UE implementation on the selection between R19 DCI-indicated additional RACH resources and legacy RACH resources, i.e., no need to specify the UE behaviours on this.

# Adaptation of Paging

In legacy, the PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N), Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns,

where T denotes DRX cycle of the UE, N denotes the number of total paging frames in T, Ns denotes the number of paging occasions for a PF, PF_offset is the offset used for PF determination, and UE_ID is derived based on 5G-S-TMSI. The legacy PF/PO determination mechanism has already supported to confine the PF/PO in the time domain based on the adjustment of parameters N/Ns.

In R2#127bis meeting, the following agreements on paging adaptation were made.

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

Here we further discuss the Ns value considering signalling overhead minimization. Considering the values of N are extended (e.g., T/64, T/128 ...) to have increased interval between PFs, the values of Ns also need to be extended to compensate the decrease in number of PFs. In our understanding, from the perspective of network power saving, the benefit of paging adaption is mostly achieved in the scenario with small number of UEs. In this low loading scenario, paging overload would be not an essential issue. Therefore, it would be not necessary to fully compensate the decrease in number of PFs by introduce larger values of Ns. Considering the trade-off between signalling overhead and paging capacity, we propose to only introduce smaller Ns values, i.e., Ns = 8, 16.

- From the perspective of network power saving, the benefit of paging adaption is mostly achieved in the scenario with small number of UEs, where paging overload would be not an essential issue.

- Considering signalling overhead minimization, only smaller Ns values are introduced, i.e., Ns = 8, 16.

Another issue needed to be started to discuss is the capability for paging adaptation enhancement. The paging adaptation enhancement introduced in Rel-19 NES should be an optional feature for a Rel-19 UE to support, thus needing an optional capability reported to network to keep a common understanding between UE and network. R2 can confirm the introduction of the optional capability with signalling.

However, it is unclear from R2 perspective whether the capabilities of paging adaptation enhancement for each frequency band of UE are coincident or not. It is worth referring that for PEI enhancement in Rel-17, per-band capability (i.e., the band list supporting PEI) is introduced. R2 may further discuss the granularity of this UE capability (e.g., per UE, per band, etc.). In case that R2 is not confident to make the decision, R1/4 may need to be involved on this issue.

- Introduce a UE AS capability on whether the UE supports paging adaptation enhancement for Rel-19 NES, which is an optional capability with signalling.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Adaptation of SSB in time domain

- At least SSB adaptation in SCells is supported as concluded by R1, but it is still FFS on SSB adaptation of PCell for RRC_CONNECTED UEs.

- R1 has ruled out the support of adaptation of SSB for RRC_IDLE UEs in Rel-19.

- R2 confirms that SSB adaptation in time domain for RRC_IDLE and RRC_INACTIVE UEs is not supported in Rel-19.

- For on-demand SSB SCell supporting (i.e., NES O1), a new MAC CE will be introduced to indicate on-demand SSB transmission in SCells, which supports SSB periodicity change in RRC_CONNECTED mode.

- R2 adopts a unified MAC CE design for both on-demand SSB transmission in SCells (i.e., NES O1) and SSB adaptation in time domain (i.e., NES O3), and wait for R1 for the applicability to PCell.

- R2 adopts a unified L3 RRM measurement design for both on-demand SSB transmission in SCells (i.e., NES O1) and SSB adaptation in time domain (i.e., NES O3).

Adaptation of RACH in time domain

- For the adaptation mechanism of RACH in time domain, R1 has already decided to support DCI-based adaptation.

- Currently it is still not clear whether the resulted R1 conclusions for DCI-based RACH adaptation can support this kind of RACH bundling or not.

- Up to R1 to evaluate and lead the work for RACH bundling if needed.

- It is not motivated to apply time-domain RACH adaptation for CFRA, since CFRA can rely on dedicated signalling of networks to assign specific RACH resources.

- Adaptation mechanism for additional RACH resources only applies to CBRA (i.e., not apply to CFRA).

- The current cell-specific CBRA resource configuration includes: 1) Rel-15 4-step RACH resources, 2) Rel-16 2-step RACH resources, 3) IAB RACH resources, 4)	additional RACH resources for RACH partitioning, and 5) RACH resources for the additional PCIs.

- From R2 perspective, at least apply time-domain RACH adaptation to 4-step RA. Send LS (as in Annex) to R1 to check whether to apply to 2-step RA or not.

- From R2 perspective, not apply time-domain RACH adaptation for IAB RACH resources, RACH resources for the additional PCIs, or additional RACH resources for feature(-combination)-specific RACH partitioning. Send LS (as in Annex) to R1 to ask if any concern.

- From R2 perspective, not pursue feature(-combination)-specific RACH partitioning for time-domain RACH adaptation. Send LS (as in Annex) to R1 to ask if any concern.

- Up to UE implementation on the selection between R19 DCI-indicated additional RACH resources and legacy RACH resources, i.e., no need to specify the UE behaviours on this.

Adaptation of Paging

- From the perspective of network power saving, the benefit of paging adaption is mostly achieved in the scenario with small number of UEs, where paging overload would be not an essential issue.

- Considering signalling overhead minimization, only smaller Ns values are introduced, i.e., Ns = 8, 16.

- Introduce a UE AS capability on whether the UE supports paging adaptation enhancement for Rel-19 NES, which is an optional capability with signalling.

# 4. Reference

[1] RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

# 5. Annex: LS to R1 on Adaptive RACH

3GPP TSG-RAN WG2 Meeting #128	R2-240xxxx

Orlando, United States, 18th – 22nd November, 2024

Title:	[Draft] LS on Applicability of Time-Domain RACH Adaptation

Response to: 	-

Release:	Rel-19

Work Item:	Netw_Energy_NR_enh-Core

Source:	[to be RAN2]

To:	RAN1

Cc:	-

Contact Person:

#### Name:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 had discussed the issue of applicability of time-domain RACH adaptation w.r.t the following RACH related features:

- Rel-15 4-step RACH resources (i.e., rach-configCommon), with and without feature(-combination)-based RACH partition;

- Rel-16 2-step RACH resources (i.e., msgA-ConfigCommon-r16), with and without feature(-combination)-based RACH partition;

- IAB RACH resources (i.e., rach-ConfigCommonIAB-r16) introduced for IAB-MT in Rel-16 IAB,

- Additional RACH resources (i.e., additionalRACH-ConfigList-r17) introduced in Rel-17 for feature/feature combination-specific RACH partitioning framework, and

- RACH resources for the additional PCIs (i.e., additionalRACH-perPCI-ToAddModList-r18) introduced in Rel-18 MIMO.

And reached the following conclusion from R2 perspective:

[…]

2. Actions:

To RAN1:

ACTION: 	RAN2 kindly asks RAN1 to provide feedback if any concern on the R2 conclusion above.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#129		2025-02-17~2025-02-21 Athens, GR

RAN2#129-bis	2025-04-07~2025-04-11 China, CN