3GPP TSG-RAN WG2 Meeting #128                               		R2-2409680

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:		8.11.2

Source:				Charter Communications

Title:					SBFD RACH configuration for initial random access

Document for:		Discussion and decision

# Introduction

Sub-Band Full duplex (SBFD) work item described in the revised WID [1] highlight the objectives of specifying SBFD operation at the gNB. One of the core objectives is to specify enhancements for random access for SBFD-aware UE:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In RAN2#127-bis, RAN2 made progress and reached the following agreements on SBFD RACH configuration signaling to support random access procedure using configured resources in SBFD symbols:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

The following two RACH configuration options are considered for SBFD based random access:

    - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

    - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

The RACH configuration for SBFD is transmitted via SIB1.

FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

From the above RAN2 agreements, RAN2 agreed to support two configuration options for SBFD-based random access where Option 1 is to use one single RACH configuration based on the existing parameters and provide extension to existing parameters if needed. Option 2 uses two separate RACH configurations – one legacy RACH configuration and one additional RACH configuration.

In this contribution, we focus on supporting random access in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode and provide analysis of these two configurations options agreed in RAN1 and subsequently in RAN2, and then discuss the possible approaches to convey the RACH configurations for initial random access in SBFD symbols.

# Discussion

Option 1 uses single RACH configuration based on the existing RACH configuration parameters such as msg1-FrequencyStart and msg1-FDM etc. Configuring single RACH configuration for both SBFD and non-SBFD UEs for initial access could lead to invalid ROs (those ROs that fall outside of the SBFD UL symbol) due to msg1-FrequencyStart offset configured for legacy RACH without considering the SBFD-aware UEs whose the FDMed ROs may overlap with DL symbol, thereby causing invalid ROs as shown in Figure 1. Therefore, if using Option 1, existing RACH configuration parameters (msg1-FrequencyStart and msg1-FDM) must be configured to prevent invalid ROs.

UL

SSB2

RO2

SSB1

RO1

RO4

SSB4

RO3

SSB3

UL Initial BWP

X

UL

DL

DL

DL

X

DL

DL

time

frequency

UL

Invalid ROs for RACH in SBFD symbol.

RACH resource offset indicated by

msg1-FrequencyStart

for both SBFD and non-SBFD legacy RACH. The ROs may be invalid for SBFD UEs.

Figure 1. Illustration of RACH configuration signaling with Option 1 Alt 1

If the existing RACH parameters must be used for single RACH configuration, invalid ROs are inevitable. Therefore, provide a fallback mechanism for SBFD UEs to use non-SBFD RACH resource when RACH fails on SBFD UL symbols.

Observation 1: Option 1 with existing RACH configuration parameters may lead to invalid ROs in SBFD symbols.

Proposal 1: RAN2 to discuss handling of invalid ROs when using the single RACH configuration option.

If SBFD-aware UEs may find invalid ROs in SBFD symbols, they are allowed to use the ROs in non-SBFD symbols for the random access without constraints on the number re-attempt in SBFD symbols. If UE find invalid ROs in SBFD symbols, re-attempting RACH on the same SBFD ROs may increase interference to other UEs and RACH preamble collisions. Therefore, an SBFD UE may not be constrained to a number of retries on symbols with invalid ROs before trying on non-SBFD ROs.

Proposal 2: On finding invalid ROs in SBFD symbols, SBFD-UE switches to legacy ROs without re-attempt on the same SBFD ROs.

Alternatively, if Option 1 does not confine the valid ROs to the UL usable PRBs, possible solution is to limit the value of msg1-FDM to keep the frequency offset of the lowest ROs within the usable UL SBFD PRBs. This solution could have impact on specification since legacy RACH configuration uses the same msg1-FDM RACH configuration parameter.

Proposal 3: RAN2 discuss whether to limit the number of PRACH transmission occasions FDMed to confine ROs within UL SBFD usable PRBs.

Option 2 will support additional RACH configuration in addition to the legacy RACH configuration. For the determination of the lowest RO of additional-ROs in SBFD symbols, RAN1 agreed in RAN1#118b that the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs).

To reduce the overhead on the UE-side by allowing the network (NW) to configure the scaling factor but it is upto RAN2 to decide if UE implementation should calculate the scaling factor to determine the frequency offset of the lowest RO. Alternatively, a new parameter (such as ra-RO-ScalingFactorSBFD-r19) maybe introduced by RAN2 for the NW to signal the value of the scaling factor along with other parameters in the RACH configuration such as the rach-ConfigGeneric of RACH-ConfigCommon. This is a possible option since the NW knows the UL usable PRB and initial UL BWP sizes a priori.

Proposal 4: RAN2 to discuss if calculation of the scaling factor is upto UE implementation.

On whether UE first selects legacy-ROs or additional-ROs in Option 2, RAN2 agreed in RAN2#127-bis that this selection depends on certain specified/configured conditions/prioritizations. We believe this selection should be based on some SBFD UE capability indication. If the UE has some preconfigured SBFD capability indicator, it may naturally select additional-ROs (SBFD RACH resources) first and fallback to the legacy-ROs if RACH failed on a number of attempts on the RACH resources configured in SBFD symbols. Using a preconfigured SBFD capability indicator in UE implementation prevents additional signaling of configured conditions/prioritization from the NW.

Proposal 5: RAN2 to discuss if UE selects additional-ROs first based on some SBFD UE capability indicator.

To fallback to legacy-ROs after some attempts on the additional-ROs, RAN2 needs to define the maximum number of retries allowed, and study the fallback mechanism and analyze its impacts on RACH process delay and performance as well as interference and RACH collision that may be caused to other neighboring UEs.

Proposal 6: RAN2 to discuss the maximum number of retries allowed on additional-ROs before triggering fallback to legacy-ROs.

Proposal 7: RAN2 to study the impacts of fallback mechanism on RACH latency, collision, and interference to neighboring UEs.

# Conclusion

Consequently, we have made the following observation and proposals for discussion and decision in RAN2:

Observation 1: Option 1 with existing RACH configuration parameters may lead to invalid ROs in SBFD symbols.

Proposal 1: RAN2 to discuss handling of invalid ROs when using the single RACH configuration option.

Proposal 2: On finding invalid ROs in SBFD symbols, SBFD-UE switches to legacy ROs without re-attempt on the same SBFD ROs.

Proposal 3: RAN2 discuss whether to limit the number of PRACH transmission occasions FDMed to confine ROs within UL SBFD usable PRBs.

Proposal 4: RAN2 to discuss if calculation of the scaling factor is upto UE implementation.

Proposal 5: RAN2 to discuss if UE selects additional-ROs first based on some SBFD UE capability indicator.

Proposal 6: RAN2 to discuss the maximum number of retries allowed on additional-ROs before triggering fallback to legacy-ROs.

Proposal 7: RAN2 to study the impacts of fallback mechanism on RACH latency, collision, and interference to neighboring UEs.

# References

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)