3GPP TSG-RAN WG2 Meeting #128                               R2-2409681

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:		8.11.3

Source:				Charter Communications

Title:					SBFD UE Capability indication for SBFD operations

Document for:		Discussion and decision

# Introduction

The WID in [1] describes the work item on the evolution of NR duplex operation – Subband Full Duplex (SBFD) with the primary objective of specifying SBFD operation for TDD carrier. While the objectives are to specify the semi-static indication of time and frequency domain resources for UEs that are SBFD-aware. Also, this WI will specify enhancements for cross link interference (CLI) handling, and UE transmission, reception, and measurement procedures in SBFD symbols. In RAN2#127-bis, RAN2 made the following agreements related to signaling for SBFD operations:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

The following two RACH configuration options are considered for SBFD based random access:

    - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

    - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

The RACH configuration for SBFD is transmitted via SIB1.

FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

In this contribution, we will like to discuss an important signaling configuration related to the UE capability and SBFD operations.

# Discussion

Our understanding of the WI in [1] and the SR in [2] is that SBFD operation shall be supported for UEs in RRC_IDLE/INACTIVE and RRC_CONNECTED states etc.:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

For initial access from RRC_IDLE/INACTIBE mode, UE needs to know if it supports SBFD operations to use the SBFD symbols and/or RACH occasions for initial access. RAN2 agreed in RAN2#127-bis that RACH configuration for SBFD will be transmitted through SIB1. Via SIB1, UE may receive the RACH resources configured in SBFD symbols. However, if the UE is not aware of its SBFD capability, it may not known that the additional ROs (or SBFD RACH occasions) could be used for transmitting Msg1.

Observation 1: UE that is unaware of its SBFD capability, may not know it could use additional ROs or any SBFD specific resources.

As a result of Observation 1, RAN2 should discuss if there is need to include SBFD capability information element in the UE NR capability information list, which helps the UE know that it is SBFD capable and can use SBFD resources in SBFD symbols if configured.

Proposal 1: RAN2 to discuss configuring SBFD capability indicator in UE implementation.

Similarly, prior to the UE attaching to the gNB via random access, the gNB has no knowledge of the capabilities supported by the UE. Hence, the gNB is completely unaware if the UE supports SBFD operations or not, and may not configured UL resources in SBFD symbols for such UE. The advantage of having such indicator implemented in the UE and signaled to the gNB is that, NW will configure SBFD symbols indicated via TDD-UL-DL-ConfigCommon knowing fully that there are SBFD-aware UEs to use those SBFD resources. UEs may benefit from utilizing those SBFD resources if they are self-aware (via capability indicator) of their SBFD capability.

Observation 2: gNB may not know by default those UEs in the serving cell that are SBFD-aware.

Figure 1: UE Capabilities Enquiry [TS 38.331, 5.6.1.3]

Consequently, we identify two approaches to Observation 2, first, that the gNB needs to be informed of SBFD UE capability via some UE capability enquiry and information as shown in Figure 1 from TS 38.331. Second option is to allow the gNB inferred UE’s SBFD capability from the type of RACH occasions (legacy or additional ROs) used by the UE for transmitting Msg1. That is, if a UE uses additional ROs (SBFD RACH resources), gNB shall identify that UE as SBFD-aware UE when in RRC_CONNECTED mode and configure future UL resources (via TDD-UL-DL-ConfigCommon) in SBFD in symbols whenever possible for that UE.

Proposal 2: RAN2 to discuss signaling information for UE to indicate SBFD capability to the gNB.

Proposal 3: If UE uses additional-ROs for RACH, gNB can infer that UE is SBFD capable for further UL resource configuration in SBFD symbols while in RRC_CONNECTED mode.

# Conclusions

We have discussed our observations regarding SBFD capability indication from UE to the gNB and make the following proposal for RAN2’s discussion and decision:

Observation 1: UE that is unaware of its SBFD capability, may not know it could use additional ROs or any SBFD specific resources.

Observation 2: gNB may not know by default those UEs in the serving cell that are SBFD-aware.

Proposal 1: RAN2 to discuss configuring SBFD capability indicator in UE implementation.

Proposal 2: RAN2 to discuss signaling information for UE to indicate SBFD capability to the gNB.

Proposal 3: If UE uses additional-ROs for RACH, gNB can infer that UE is SBFD capable for further UL resource configuration in SBFD symbols while in RRC_CONNECTED mode.

# References

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- RP-242034, “Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Status report, Huawei, Samsung