3GPP TSG-RAN WG2 Meeting #128						R2-2409697

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.5.4

Source:	Samsung

Title:	Adaptation of common signal transmissions

Document for:	Discussion and Decision

# Introduction

RAN2 discussed the adaptation of paging occasions and made the following agreements:

RAN2#126

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study

  - bundle paging frames and

  - extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

  - Option 1: Prevent the access of legacy UE via barring;

  - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

RAN2#127bis

- Paging adaptation method

  - Select option-b as baseline for R19 NES paging enhancement.

  - R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Whether allow Rel-19 PF overlapping with legacy PF

  - Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Whether to bar legacy UEs?

  - Legacy UE is not barred.

- Whether Rel-19 UE only monitors Rel-19 configured PO?

  - Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

# Discussion

2.1 Adaptation of SSB in time domain

RAN1 has discussed adaptation of SSB in time domain and agreed the following:

- For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

When pagingSearchSpace is zero and SS/PBCH block and CORESET multiplexing pattern is 2 or 3 , PMOs for paging are same as SSB occasions, PF is the radio frame in which SSBs are transmitted. This is ensured by configuring nAndPagingFrameOffset based on period/offset of SSB transmission. Figure 1 is an example illustration of this for SSB periodicity = 80ms, N = T/8 and PF_Offset = offset.

Figure 1

For pagingSearchSpace is zero and SS/PBCH block and CORESET multiplexing pattern is 2 or 3:

- In case SSB periodicity is changed, paging configuration needs to be changed to ensure that PF configured by paging configuration is the radio frame in which SSBs are transmitted as per new SSB periodicity. As shown in figure 2, PF in red circle is not a valid PF for SSB periodicity of 160ms.

  - Option 1: When SSB periodicity is changed, SIB1 can be updated by network to include new paging configuration.

  - Option 2: Two paging configuration can be signalled in SIB1, one corresponding to each SSB periodicity/SSB configuration. UE applies the configuration according to active SSB periodicity/SSB configuration.

For dynamic adaptation of SSB periodicity, option 2 seems more efficient.

SSB periodicity 1 = 80ms

Figure 2

Proposal 1: Two paging configurations can be signalled in SIB1, one corresponding to each SSB periodicity/SSB configuration. UE applies the configuration according to active SSB periodicity/SSB configuration.

2.2. Paging clustering/bundling

RAN2 has agreed option-b (i.e. Extension of N and Ns) as baseline for R19 NES paging enhancement for paging clustering/bundling.

In option-b POs are clustered at the beginning of DRX cycle by extending the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF. For example, let’s say 32 POs are needed in a DRX cycle to meet the paging load in a cell. Network can configure 1 PF in DRX cycle and 32 POs per PF.

If cell supports paging clustering/bundling,

- it configures new parameters N’ and Ns’. N’ and Ns’ configures extended values of number of paging frames and number of POs.

- Starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO is also configured separately for paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

- PF_Offset’ is also configured separately for paging clustering/bundling for flexible allocation of PF with paging clustering/bundling and legacy PF.

Legacy UEs in the cell will apply legacy parameters N, Ns, PF_Offset and firstPDCCH-MonitoringOccasionOfPO (if configured) whereas NES UEs supporting paging clustering/bundling applies N’, Ns’. PF_Offset’ and firstPDCCH-MonitoringOccasionOfPO’.

Proposal 2: If cell supports paging clustering/bundling,

- it configures N’ and Ns’ for indicating number of paging frames and number of POs for UEs supporting paging clustering/bundling.

- if may configure starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO for UEs supporting paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

- it configures new parameter PF_Offset’ for UEs supporting paging clustering/bundling.

UE specific DRX cycle can be smaller or larger than default DRX cycle. Due to clustering of POs at the beginning of DRX cycle (i.e. default DRX cycle), DRX cycle T used by UE for PF/PO determination should be max (default DRX cycle, UE specific DRX cycle) or equal to default DRX cycle.

Proposal 3: For paging clustering/bundling, DRX cycle T used by UE for PF/PO determination should be max (default DRX cycle, UE specific DRX cycle) or equal to default DRX cycle.

# Conclusion

Proposal 1: Two paging configurations can be signalled in SIB1, one corresponding to each SSB periodicity/SSB configuration. UE applies the configuration according to active SSB periodicity/SSB configuration.

Proposal 2: If cell supports paging clustering/bundling,

- it configures N’ and Ns’ for indicating number of paging frames and number of POs for UEs supporting paging clustering/bundling.

- if may configure starting PDCCH monitoring occasion number (firstPDCCH-MonitoringOccasionOfPO’) of PO for UEs supporting paging clustering/bundling. Starting PDCCH monitoring occasion number of PO is optional as in legacy.

- it configures new parameter PF_Offset’ for UEs supporting paging clustering/bundling.

Proposal 3: For paging clustering/bundling, DRX cycle T used by UE for PF/PO determination should be max (default DRX cycle, UE specific DRX cycle) or equal to default DRX cycle.

# References