3GPP TSG-RAN WG2#128	R2-2409700

Orlando, Florida, USA, 18-22 November 2024

Agenda Item:	8.2.5

Source: 	MediaTek Inc.

Title:  	Device support of ambient IoT random access procedures

Document for:	Discussion, decision

# 1	Introduction

This document is a brief note on the support of different random access procedures by AIoT devices.

# 2	Discussion

## 2.1	CBRA and CFRA

From a service point of view, CBRA and CFRA are complementary.  A service request to inventory one device, or a few specific devices, might enable the reader to trigger CFRA for optimal completion of a quick procedure without using a lot of radio resources; a request to inventory a large group of devices may require CBRA, since it could be impractical to allocate separate access resources for every device.

Proposal 1: All AIoT devices support both CBRA and CFRA, with the reader taking the decision on which one is triggered for a particular service request.

As suggested in [1], it may be possible to unify CBRA and CFRA from the device point of view, which seems like a worthwhile effort for device complexity reasons.

Proposal 2: From the device perspective, CBRA and CFRA procedures should be unified by defining a single resource-selection algorithm.

## 2.2	2-step and 3-step

The situation with 2-step and 3-step random access is less tractable.  The procedures are inherently divergent, with different contents in Msg1 and different behaviour to send or not send Msg3.  It is not clear if there are “2-step-only” and/or “3-step-only” service use cases—simple inventory, for example, works well with either procedure, and the distinction between the two may have more to do with coverage conditions than service cases.

It has been suggested in discussion that there could be devices supporting only one procedure.  We consider this possibility to be conceivable, but it should not open the door to device capability signalling, which would be an invitation to complexity in a system that needs to prioritise simplicity.

Proposal 3: There is no device capability signalling to the reader to indicate random access procedure support.

(We would be open to generalising proposal 3 to have no capability signalling at all, but not every case of every procedure has been analysed, and from this paper’s perspective we make the proposal only for the random access procedures.)

We consider that there are three options:

- All devices support both 2-step and 3-step random access

- There may be 2-step-only or 3-step-only devices (without capability signalling)

- Downselect between 2-step and 3-step procedures for normative work

Options 1 and 3 are self-explanatory (in option 3, we assume that the downselection should be to 3-step, to avoid coverage problems due to only using 2-step), but option 2 may require some exploration to understand the consequences.  One possibility is that the service layer knows what the devices support, so that some deployments may cater only to 3-step-capable devices, e.g.  However, such a situation would need to be handled carefully across the different layers to avoid creating compatibility problems where a device “inexplicably” does not work in a particular deployment due to supporting a different access procedure.

The obvious procedure in which the reader requests a particular access procedure, and devices that do not support it simply do not respond, may work in some situations, depending on which procedures are supported by which devices.  In a tightly managed deployment that guarantees that all devices are selected by the operator or a specific customer (e.g., warehouse owner), it should be possible to mandate specific random access support at the devices and align the expectations of the reader(s).  In a more free deployment where new devices may emerge (e.g., bought off the shelf by a consumer), it should be possible to ensure that all devices work, which would be difficult if both random access procedures were optional.  If one random access procedure is mandatory and the other optional (most likely 3-step would be the mandatory one), the deployment can work, but it probably means that readers would use only the mandatory procedure.

Finally, it seems possible that the reader could always allocate enough Msg1 resources to support 2-step random access, and devices that can only respond with 3-step would intentionally underflow the resource allocation.  The result is some waste of radio resources, but it allows option 2 to work without prior knowledge at the reader of what the device supports.

To summarise option 2, we consider that it may be viable if 3-step random access is mandatory and 2-step is optional, but to avoid capability signalling while allowing deployments that are open to a variety of devices, the system needs to take some measures such as always using the mandatory procedure in some types of deployments or over-allocating resources for Msg1.

Proposal 4: Consider the following possibilities for support of random access procedures:

- All devices support both 2-step and 3-step random access

- There may be 3-step-only or 2-step-only devices, without capability signalling, and the reader and network implementations are responsible for ensuring that devices can perform random access successfully

- Downselect to 3-step random access only for normative work

# 3	Conclusion

This document promulgated the following proposals:

Proposal 1: All AIoT devices support both CBRA and CFRA, with the reader taking the decision on which one is triggered for a particular service request.

Proposal 2: From the device perspective, CBRA and CFRA procedures should be unified by defining a single resource-selection algorithm.

Proposal 3: There is no device capability signalling to the reader to indicate random access procedure support.

Proposal 4: Consider the following possibilities for support of random access procedures:

- All devices support both 2-step and 3-step random access

- There may be 3-step-only or 2-step-only devices, without capability signalling, and the reader and network implementations are responsible for ensuring that devices can perform random access successfully

- Downselect to 3-step random access only for normative work

# 4	References

[1]	R2-2408536: “Unified random-access procedure for A-IoT”, ZTE, RAN2#127bis